| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0308-v1 |
| --- | --- |

| Title: | AHG9: Leading pictures and the SPTI SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson-Allendes Sachin Deshpande | Tel: Email: | +46-721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Abstract

This input contribution relates to the Source Picture Timing Information (SPTI) SEI message which is included in the Additional SEI messages for VSEI version 4. The proposal in this document is intended to address a problem that is highlighted in JVET-AJ0252 related to the combination of SPTI and temporal layering and leading pictures. It is asserted that the proposal in this input contribution solves the problem in a manner that is robust to temporal scalability and in alignment with the current design of the SPTI.

# Introduction

JVET-AJ0252 [1] highlights a problem with the combination of SPTI and temporal layering and leading pictures. In the current design of the SPTI SEI message, it is not possible to provide information about source picture interval for leading pictures (i.e. pictures that are decoded after the first IRAP picture but output before the first IRAP picture) with TemporalId greater than 0. This is because the semantics of the SPTI SEI message states that the source picture interval is expressed “relative to the previous output picture with TemporalId less than or equal to i”. However, for leading pictures (as well as for the first IRAP picture) there might be no previous output picture with the same or lower TemporalId.

Figure 1. Example of leading pictures in different temporal layers

# Discussion

One way of solving the problem with leading pictures in temporal layers greater than 0 is to reverse the direction in which the source picture intervals are defined. That is to say “relative to the next output picture with TemporalId less than or equal to i“. This would solve the problem for leading pictures, but it would create a corresponding problem for trailing pictures whenever a sequence doesn’t end with a picture with TemporalId equal to 0.

Figure 1. Example of trailing pictures in different temporal layers

# Proposal

It is proposed to add a flag to the SPTI SEI message to indicate if the source picture interval is indicated in forward or backward direction, i.e. if the source picture interval is relative to the previous or to the next output picture with TemporalId less than or equal to i. The following changes are proposed:

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_source_type_present_flag | u(1) |
| if( spti_source_type_present_flag ) |  |
| spti_source_type | u(16) |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_interval | u(32) |
| spti_direction_flag | u(1) |
| if( spti_persistence_flag ) |  |
| spti_max_sublayers_minus1 | u(3) |
| for( i = sptiMinTemporalSublayer; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

spti_direction_flag indicates the direction of the signalled source picture intervals. spti_direction_flag equal to 0 indicates that source picture intervals are expressed relative to previous output pictures. spti_direction_flag equal to 1 indicates that source picture intervals are expressed relative to future output pictures.

…

spti_sublayer_interval_scale_factor[ i ], when present, specifies a scale factor used in determining the interval time between the source picture associated with the current picture and the source picture associated with the previous (if spti_direction_flag is equal to 0) or next (if spti_direction_flag is equal to 1) output picture with TemporalId less than or equal to i. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture with TemporalId less than or equal to i. The value of spti_sublayer_interval_scale_factor[ I ] shall be in the range of 0 to 232 − 2, inclusive.

The indicated source picture interval associated with an output picture having TemporalId equal to i, relative to the previous output picture with TemporalId less than or equal to i, denoted by the variable SourcePictureInterval[ i ], in units of seconds, is derived as follows:

SourcePictureInterval[ i ] = ElementalSourcePictureInterval * spti_sublayer_interval_scale_factor[ i ] *
( 1 − 2 * temporalReversalFlag )		(8-X)

If spti_direction_flag is equal to 0, the indicated source picture interval is relative to the previous output picture with TemporalId less than or equal to i. Otherwise (spti_direction_flag is equal to 1), the indicated source picture interval is relative to the next output picture with TemporalId less than or equal to i.

If spti_source_type_present_flag is equal to 1, the variable temporalReversalFlag is equal to ( spti_source_type & 0x10 )? 1 : 0. Otherwise (i.e. if spti_source_type_present_flag is equal to 0), the variable temporalReversalFlag is equal to 0.

# References

- JVET-AJ0252, AHG9: On SPTI SEI message, Y. Sanchez, T. M. Borges, R. Skupin, C. Hellge, T. Schierl (HHI), https://jvet-experts.org/doc_end_user/current_document.php?id=14859

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).