| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0318-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0090 (AHG12: temporal ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhenyu Dai | Tel: Email: | daizhenyu.dzy@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

Abstract

This contribution reports the crosscheck results of JVET-AJ0090[1], which proposes an adaptive loop filter that uses temporal reconstructed information as the input to a filter whose output will enhance the current picture. It is confirmed that the available simulation results are matched with those provided in JVET-AJ0090 except some RA class A and B sequences. The difference should be due to the ECM-14.0 issue on different test environments.

- Simulation Results

The proposed method of JVET-AJ0090 was implemented on top of the ECM-14.0[2]. Simulations were performed following the common test conditions[3]. More detailed results could be found in the attached excel sheets. The running time may contain some fluctuations due to the cluster busyness.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.19% | 0.29% | 0.40% | 101.0% | 103.0% | 100.5% | 100.3% |
| Class C | -0.08% | 0.22% | 0.29% | 101.3% | 102.8% | 100.6% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.08% | 0.25% | 0.31% | 101.0% | 101.8% | 100.6% | 100.0% |
| Class F | -0.26% | 0.41% | 0.37% | 103.0% | 104.5% | 101.5% | 100.4% |
| Class TGM |  |  |  |  |  |  |  |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.16% | 0.23% | -0.07% | 100.9% | 100.9% | 100.5% | 100.1% |
| Class E | -0.08% | 1.04% | 0.76% | 104.4% | 103.8% | 98.4% | 100.1% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.10% | 0.38% | 0.54% | 100.7% | 101.3% | 101.0% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

- References

- L. Xu, Y. Yu, H. Yu, D. Wang (OPPO), “AHG12: temporal ALF”, JVET-AJ0090, TR, November 2024.

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, JP, July 2024.

- Patent rights declaration(s)

Alibaba group does not have any current or pending patent rights relating to the technology described in this contribution.