| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0319-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0077 (EE2-2.1: Matrix-based position dependent intra prediction for TIMD) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zexing Sun | Email: | sunzexing@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

The contribution reports the results of the crosscheck for JVET-AJ0077 which proposes matrix-based position dependent intra prediction for TIMD. The crosschecking is conducted on top of ECM-14.0. It is confirmed that the crosscheck results are matched with those provided by the proponent.

Introduction

In JVET-AJ0077 [1], matrix-based position-dependent intra-prediction for TIMD is proposed. i.e., the conventional IPMs are replaced with PDP in TIMD. In addition, PDP is employed in template cost calculation for TIMD. Furthermore, the proposed method is leveraged to the inter-prediction tools using TIMD, such as CIIP and GPM intra. Details of the proposed method are as follows:

-	Signalling is kept as the conventional TIMD.

-	Angular modes replaced by PDP in TIMD are different from those in the regular intra-prediction since the number of IPMs in TIMD is double that of regular intra-prediction. Specifically, PDP is only used for modes 0, 1, and (2+4*k) for coding blocks with width and height up to 16 samples; otherwise, PDP is only used for modes 0, 1, and (2+8*k).

-	Template cost for TIMD is calculated by applying PDP to reference samples of the template, as shown in Figure 1.

-	PDPC is not applied to the PDP samples in TIMD, just as PDPC is not applied to the PDP samples in regular intra-prediction.

Figure 1 PDP (left) and PDP for template cost calculation in TIMD (right)

Experimental results

The software and related configuration file provided by the proponent were checked and it was exactly matched with the description given in the proponent’s contribution. The experiments were performed on top of ECM-14.0 under the EE2 description [2] and under the JVET common test conditions [3]. The BD-rate results are summarized in Table 1, and it is matched exactly with those provided by the proponent. Also, the runtime and peak memory measurements are close to the ones reported by the proponent. And, the detailed results can be found in the accompanied spreadsheet.

Table 1. The crosscheck results

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.05% | -0.12% | 100.7% | 101.4% | 100.7% | 100.1% |
| Class A2 | -0.05% | -0.14% | -0.07% | 100.6% | 102.3% | 100.0% | 100.0% |
| Class B | -0.05% | -0.09% | -0.05% | 100.1% | 103.0% | 100.0% | 100.1% |
| Class C | -0.04% | -0.15% | -0.09% | 99.7% | 103.7% | 99.2% | 100.0% |
| Class E | -0.08% | 0.05% | 0.14% | 100.2% | 103.9% | 100.7% | 100.0% |
| Overall | -0.06% | -0.06% | -0.04% | 100.2% | 102.9% | 100.1% | 100.0% |
| Class D | -0.05% | -0.03% | -0.14% | 99.7% | 105.3% | 100.2% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Reference

- Y. Kidani and et al., “EE2-2.1: Matrix-based position dependent intra prediction for TIMD”, JVET-AJ0077, Nov. 2024.

- V. Seregin and et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2),” JVET-AI2024, July 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, July 2024.

Patent rights declaration(s)

OPPO does not have any current or pending patent rights relating to the technology described in this contribution.