| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0321-v1 |
| --- | --- |

| Title: | AHG12: ECM complexity trade-offs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Chun-Chi Chen, Muhammed Coban, Kai Cui, Patrick Garus, Nan Hu, Han Huang, Marta Karczewicz, Jian-Liang Lin, Po-Han Lin, Pavel Nikitin, Bappaditya Ray, Vadim Seregin, Gleb Verba, Hongtao Wang, Emma Ye, Ruoyang Yu, Yan Zhang, Zhi Zhang | Tel: Email: |  |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents two sets of the results (a) ECM-14.0 without inter template matching, simplifications of other mainly decoder side ECM tools, and encoder optimizations; and (b) encoder only optimizations for ECM-14.0 keeping all tools enabled and unmodified.

Test results comparing to VTM-11.0ecm14.0 anchor are summarized as follows.

Test (a) RA: -20.78%, -20.43%, -23.25%; 412.4%, 606.1% (YUV, enc, dec)

# Introduction

In ECM, there are tools which have challenges in hardware implementation. One of the largest categories is the inter template matching including template based MV candidates reordering. The main difficulty is that MV candidate is not known upfront, so a reasonable memory prefetching is not possible with having a reasonable cache size. However, this category has significant runtime increase in ECM.

In this contribution, inter template matching was disabled, and some other tools were simplified with paying attention to the decoder side methods, encoder optimizations were also performed. It includes software optimization for DMVR and BDOF, restructuring affine motion compensation, reducing the number of candidates in GPM MMVD, skipping complex processes in iterative BDOF, LIC, affine DMVR process, OBMC, and cross-component prediction in inter prediction.

In the second part of contribution, encoder only optimizations (not the same as in the first test) were performed keeping all tools enabled.

Those modifications were implemented on top of ECM-14.0 and test results are summarized below using ECM-14.0 and VTM-11.0ecm14.0 as anchors.

# More hardware friendly design

The main motivation of this part of the contribution is to demonstrate that over 20% compression efficiency in RA over VTM can be achieved with more hardware friendly design and complexity reduction.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -22.16% | -17.56% | -30.38% | 499.8% | 656.2% |  |  |
| Class A2 | -23.59% | -26.33% | -31.41% | 480.1% | 697.9% |  |  |
| Class B | -18.93% | -24.46% | -21.66% | 372.5% | 585.6% |  |  |
| Class C | -19.94% | -13.11% | -13.75% | 361.8% | 536.5% |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall | -20.78% | -20.43% | -23.25% | 412.4% | 606.1% |  |  |
| Class D | -21.27% | -15.50% | -15.75% | 344.2% | 560.0% |  |  |
| Class F | -26.99% | -27.56% | -28.36% | 435.5% | 497.7% |  |  |
| Class TGM | -35.72% | -40.65% | -40.33% | 451.6% | 513.8% |  |  |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 5.67% | 6.32% | 7.19% | 47.6% | 65.1% | 67.9% | 68.5% |
| Class A2 | 8.23% | 9.65% | 11.16% | 48.0% | 58.5% | 67.7% | 67.8% |
| Class B | 6.39% | 9.37% | 9.01% | 43.8% | 58.1% | 72.3% | 76.0% |
| Class C | 7.52% | 10.02% | 10.50% | 40.7% | 49.8% | 73.9% | 85.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | 6.92% | 8.99% | 9.47% | 44.5% | 57.1% | 70.9% | 75.1% |
| Class D | 6.96% | 7.99% | 9.05% | 40.8% | 47.9% | 74.6% | 89.5% |
| Class F | 7.43% | 10.32% | 10.48% | 62.5% | 76.8% | 78.5% | 82.5% |
| Class TGM | 10.23% | 13.01% | 13.25% | 69.4% | 89.7% | 82.9% | 79.4% |

# Encoder only optimization

In the second set of results, only encoder optimization (not the same as in the first test) was applied to ECM-14.0 keeping all tools enabled.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -22.82% | -28.83% | -26.14% | 419.1% | 965.3% |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -24.79% | -18.14% | -18.62% | 414.5% | 1131.9% |  |  |
| Class F | -29.71% | -31.26% | -31.96% | 461.3% | 659.4% |  |  |
| Class TGM | -37.66% | -43.47% | -43.19% | 488.6% | 605.3% |  |  |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 1.24% | 2.83% | 2.65% | 49.3% | 95.8% | 100.1% | 100.0% |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 2.17% | 4.58% | 5.25% | 49.1% | 96.8% | 100.4% | 100.2% |
| Class F | 3.49% | 4.48% | 4.68% | 66.2% | 101.8% | 101.0% | 100.1% |
| Class TGM | 6.58% | 6.99% | 6.99% | 75.1% | 105.6% | 100.4% | 100.4% |