| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0341 |
| --- | --- |

| Title: | EE2-2.8i: Combination of EE2-2.5 and EE2-2.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli, Ramin G. Youvalari, Frank Plowman, Alexandre Tissier, Yao-Jen Chang, Vadim Seregin, Marta Karczewicz, Jiaye Fu, Jiaqi Zhang, Shanshe Wang Siwei Ma, Yoshitaka Kidani, Haruhisa Kato, Kei Kawaura | Email: | mabdoli@xiaomi.com yjchang@qti.qualcomm.com jyfu@stu.pku.edu.cn yo-kidani@kddi.com |
| Source: | Xiaomi Technology, Qualcomm Incorporated, Peking Univ, KDDI Corporation |  |  |

_____________________________

- Abstract

This contribution reports the results of EE2-2.8i which is the combinations of tests EE2-2.5 and EE2-2.1.

In the test EE2-2.5, the TIMD-merge mode is proposed, where an algorithm first creates a merge list based on the neighboring TIMD blocks. This list is then sorted based on the template cost and one candidate is finally kept in the list. A CU-level flag then indicates whether a TIMD-merge candidate is used for block prediction.

In test EE2-2.1, PDP for TIMD is proposed, which replaces part of the conventional intra prediction modes in TIMD with PDP. In this test, PDP is also employed in template cost calculation for TIMD and is applied to the inter prediction tools using TIMD, such as CIIP and GPM intra.

Here is a summary of the Test EE2-2.8i (EE2-2.5 + EE2-2.1) results:

AI { -0.13%, -0.03%, -0.05%, 100.5%, 102.9%}, RA { x.xx%, x.xx%, x.xx%, x.xx%, x.xx%}

Results

Table 1 Results for the combination of EE2-2.5 and EE2-2.1 over ECM-14.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.12% | 0.06% | -0.06% | 100.7% | 101.5% | 99.3% | 100.2% |
| Class A2 | -0.14% | -0.10% | -0.10% | 100.7% | 102.5% | 100.2% | 100.2% |
| Class B | -0.14% | -0.09% | -0.02% | 100.7% | 103.0% | 101.2% | 100.2% |
| Class C | -0.11% | -0.07% | -0.21% | 100.4% | 103.4% | 100.2% | 100.0% |
| Class E | -0.15% | 0.08% | 0.16% | 100.0% | 104.0% | 101.2% | 100.1% |
| Overall | -0.13% | -0.03% | -0.05% | 100.5% | 102.9% | 100.5% | 100.1% |
| Class D | -0.11% | -0.02% | -0.10% | 100.3% | 102.3% | 100.1% | 100.1% |
| Class F | -0.11% | -0.01% | -0.02% | 98.9% | 101.9% | 99.3% | 100.1% |
| Class TGM | -0.02% | -0.02% | -0.08% | 98.4% | 100.0% | 101.4% | 100.0% |

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |  |
|  | Y | U | V |  | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |
| Class C | -0.10% | -0.06% | 0.11% |  | 100.1% | 100.6% | 100.1% | 100.2% |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D | -0.06% | -0.21% |  | -0.10% | 99.9% | 100.8% | 100.1% | 100.1% |
| Class F |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |

Patent rights declaration(s)

Xiaomi Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking Univ Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).