| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1 – 8 November 2024 | Document: JVET-AJ0349 |
| --- | --- |

| Title: | EE2-5.1 related: Annealing coefficient derivation for CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichiro Takada, Sachin Deshpande | Email: | takadak@sharplabs.com sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to apply the annealing coefficient derivation and Huffman coding, similar to that in JVET-AJ0138 [1], to CCALF. The proposal is implemented on the top of ECM-14.0. The BD-rate PSNR of proposed approach compared to ECM-14.0 is

- {0.00%, -0.13%, -0.10%} for {Y, Cb, Cr} on AI condition,

- {0.01%, -0.19%, -0.09%} for {Y, Cb, Cr} on RA condition, and

# Introduction

In JVET-AJ0138, BD-Rate savings are achieved by applying coefficient value restriction, annealing coefficient derivation and Huffman coding to ALF. We propose the annealing coefficient derivation and Huffman coding for CCALF. Since CCALF coefficients are already restricted by a power of 2, we leave the coefficient restriction as it is in ECM-14.

The procedure of annealing coefficient derivation is modified as follows compared to that in JVET-AJ0138. In this contribution, if there are no newly accepted solutions for n+1 iterations in a row, the simulated annealing procedure is stopped, where n is the number of CCALF coefficients.

As for Huffman coding, each bit string of the coefficient is derived and pre-defined from the probability of occurrence of CCALF coefficients when ECM-14.0 encodes the CTC sequences. In practice, the length of the bit string of each coefficient is set { 8, 8, 6, 5, 4, 4, 3, 2, 3, 3, 4, 4, 5, 5, 7} for {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}.

It is asserted that the proposed approach is complementary to the approach in JVET-AJ0138 for ALF.

# Experimental results

The proposal is implemented on the top of ECM-14.0 and evaluated under the CTC by comparing to performance of the original ECM-14.0. Table 1 and 2 show the experimental results under AI and RA conditions, respectively. As for the experimental environment, the sequences of QP22 are computed on Intel(R) Xeon(R) Gold 6346, QP27, 32, 37 are computed on AMD EPYC 7763.

Table 1: Test result of CTC compared to ECM-14.0 under AI condition

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ecm140_anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.10% | -0.03% | 99.9% | 100.6% | 102.4% | 100.8% |
| Class A2 | 0.00% | -0.05% | -0.02% | 99.5% | 100.7% | 101.9% | 101.0% |
| Class B | -0.01% | -0.12% | -0.11% | 100.3% | 100.8% | 101.0% | 100.4% |
| Class C | 0.00% | -0.19% | -0.16% | 100.2% | 100.7% | 100.7% | 100.0% |
| Class E | -0.01% | -0.17% | -0.15% | 99.5% | 101.1% | 100.3% | 100.1% |
| Overall | 0.00% | -0.13% | -0.10% | 100.0% | 100.8% | 101.2% | 100.4% |
| Class D | 0.02% | -0.55% | -0.45% | 100.2% | 101.5% | 100.1% | 100.0% |
| Class F | 0.00% | -0.22% | -0.16% | 100.6% | 101.2% | 101.5% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 2: Test result of CTC compared to ECM-14.0 under RA condition

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ecm140_anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.04% | -0.17% | 0.02% | 97.1% | 88.4% | 100.2% | 102.2% |
| Class A2 | -0.01% | -0.05% | 0.01% | 94.2% | 85.6% | 100.3% | 101.5% |
| Class B | -0.01% | -0.26% | -0.22% | 99.6% | 100.0% | 100.3% | 101.3% |
| Class C | 0.02% | -0.24% | -0.07% | 98.9% | 99.4% | 100.1% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.19% | -0.09% | 97.8% | 94.4% | 100.2% | 101.2% |
| Class D | 0.00% | -0.31% | -0.15% | 98.5% | 101.0% | 100.3% | 99.9% |
| Class F | 0.03% | -0.17% | -0.06% | 99.3% | 99.8% | 100.3% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

The experimental results of LDB are not yet available, as their simulations are still in progress.

# Conclusion

This contribution shows that the BD-Rate of chroma is improved by applying an annealing process to CCALF. We recommend that this proposal be considered to study in EE2.

# References

- “EE2-5.1: Adaptive Mantissa Precision for ALF Coefficients,” V. Shchukin, P. Wennersten, J. Ström (Ericsson), JVET-AJ0138, 36th Meeting, Kemer, TR, 1–8 November 2024.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).