| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0350-v2 |
| --- | --- |

| Title: | Non-EE2: Improved LFNST/NSPT kernel set selection for SGPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saurabh Puri Charles Bonnineau Karam Naser Fabrice Le Léannec | Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In ECM, SGPM mode uses the split direction to determine the transform kernel set for coding of the predicted residual signal. In this proposal, it is proposed to apply DIMD process to the prediction signal of the SGPM to compute the VIPM, and to determine the transform kernel set. We compare VIPM and the true intra modes in SGPM using a threshold.

The following results are obtained compared to ECM-14.0:

AI {-0.03% Y, -0.01% U, -0.02% V, 100.0% EncT, 100.2 % DecT}

# Introduction

SGPM (Spatial Geometric Partitioning Mode) uses the split direction to determine the LFNST transform kernel set for coding of the predicted residual signal.

In ECM, DIMD process is applied to the predicted samples of the current block for MIP, EIP modes to derive a virtual intra prediction mode (VIPM), which can be used to determine the LFNST/NSPT kernel set.

Regarding SGPM, the LFNST transform kernel is determined by the direction of its partitioning mode.

# Proposed method

In this proposal, it is proposed to compute a VIPM by applying DIMD process to the prediction signal of SGPM to derive the LFNST/NSPT kernel set. The computed VIPM is compared with the true intra modes in SGPM using a threshold. If the value of the VIPM is close to at least one of the intra modes in SGPM, the VIPM is used. Otherwise, the direction of the partitioning mode is used to derive the LFNST/NSPT kernel set as in ECM-14.0.

# Experimental results

The proposed method is implemented on top of the ECM-14.0 [1]. The simulations are performed following the common test conditions [2] and are summarized in Table 1.

Table 1: Proposed method vs ECM-14.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.03% | -0.09% | 99.9% | 100.7% | #NUM! | #NUM! |
| Class A2 | -0.01% | -0.02% | 0.00% | 99.5% | 99.7% | #NUM! | #NUM! |
| Class B | -0.02% | 0.06% | 0.05% | 100.2% | 100.4% | #NUM! | #NUM! |
| Class C | -0.03% | -0.08% | -0.13% | 100.4% | 100.2% | #NUM! | #NUM! |
| Class E | -0.04% | -0.05% | 0.04% | 100.1% | 100.1% | #NUM! | #NUM! |
| Overall | -0.03% | -0.01% | -0.02% | 100.0% | 100.2% | #NUM! | #NUM! |
| Class D | -0.03% | 0.00% | -0.04% | 99.9% | 99.8% | #NUM! | #NUM! |
| Class F | -0.07% | -0.02% | -0.08% | 100.2% | 100.1% | #NUM! | #NUM! |

# Conclusion

We propose investigating the proposed approach in the next round of EE2.

# References

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AH2017, October 2023

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).