| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0351-v0 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0231(EE2-5.5: Reuse of ALF Control Information) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Wenbin Yin1 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | yinwenbin.hit@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This contribution reports the crosscheck results of JVET-AJ0231[1] (EE2-5.5). In EE2-5.5, the ALF control information (including on/off flag and filter usage information) is reused at both picture and CTU levels from previously encoded picture. The signaling of ALF control information will be skipped if the reuse method is enabled. At encoder side, the ALF online filters are derived for a group of pictures jointly.

There are 2 subtests included inside EE2-5.5, which are 5.5a and 5.5b. The ALF joint optimization (encoder only) is tested in 5.5a while ALF joint optimization with reuse of ALF control information is tested in 5.5b.

The available simulation results are matched with those provided in JVET-AJ0231 except some RA class A and B sequences. The difference should be due to the ECM-14.0 issue on different test environments.

- Simulation Results

The proposed method of EE2-5.5 was implemented on top of the ECM-14.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The running time may contain some fluctuations due to the cluster busyness.

Table 1 Performance of the Proposed EE2-5.5a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% | 99.2% | 98.2% | 100.2% | 99.3% |
| Class E | 0.00% | 0.00% | 0.00% | 100.0% | 99.3% | 97.3% | 98.5% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 97.0% | 94.7% | 99.9% | 99.7% |
| Class F |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.15% | -0.08% | 100.3% | 101.8% | 111.3% | 96.9% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | -0.23% | -0.13% | 99.8% | 102.3% | 107.3% | 98.2% |
| Class C | -0.06% | -0.15% | -0.08% | 99.8% | 103.5% | 110.3% | 99.4% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.16% | -0.12% | -0.24% | 98.6% | 103.3% | 104.2% | 99.7% |
| Class F | 0.02% | 0.02% | 0.01% | 99.9% | 104.0% | 110.1% | 98.6% |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 101.0% | 101.5% | 100.0% | 99.7% |
| Class F |  |  |  |  |  |  |  |

Table 2 Performance of the Proposed EE2-5.5b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.01% | 0.00% | 0.00% | 100.6% | 98.8% | 99.3% | 99.3% |
| Class E | 0.00% | 0.00% | 0.00% | 100.3% | 97.9% | 99.5% | 98.6% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 97.6% | 97.7% | 99.8% | 99.8% |
| Class F |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.13% | -0.50% | -0.56% | 100.6% | 101.0% | 110.7% | 97.2% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.13% | -0.76% | -0.71% | 100.2% | 102.5% | 106.8% | 98.4% |
| Class C | -0.13% | -0.78% | -0.86% | 99.8% | 103.5% | 109.8% | 99.4% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.31% | -0.72% | -0.81% | 97.5% | 103.0% | 104.0% | 99.8% |
| Class F | -0.02% | -0.53% | -0.60% | 101.3% | 110.5% | 109.5% | 98.5% |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | 0.01% | 0.01% | 100.8% | 100.1% | 100.1% | 99.7% |
| Class F |  |  |  |  |  |  |  |

- Reference

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, JP, July 2024.

- M. Karczewicz, H. Wang, N. Hu and V. Seregin, “EE2-5.5: Reuse of ALF Control Information,” Document of Joint Video Experts Team, JVET-AJ0231, TR, November 2024.