| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0359-v3 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0099 (EE1-3.1: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Luka Murn Nokia Technologies, London, United Kingdom | Email: | luka.murn@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-AJ0099 EE1-3.1 [1]. JVET-AJ0099 integrates a proposed RA/LDB unified reference frame synthesis for VVC inter coding enhancement named URFS from JVET-AI0089 [2] in the latest NNVC-v10rc anchor. Almost identical results are reported for the inference crosscheck of the trained URFS model over NNVC-v10rc for the RA and LDB configurations of the proponents.

During the inference crosscheck process, a bug was found in the code implementation of the RA configuration and was reported to the proponents and software coordinators. The proponents are trying to fix the bug. Currently, the RA encoded bitstream is not compliant with reference pictures having to be in a temporal layer lower than the picture to be coded.

The URFS implementation in NNVC software is based on JVET-AC0114 [3] and JVET-AD0160 [4], which utilizes an embedded Python interpreter in C++ for a floating-point model.

The difference in the BD-rate performance of URFS under RA and LDB configurations provided in JVET-AJ0099 and ours (only encoder, there is a mismatch on the decoder side due to the bug) is as follows:

RA: {0.01%, 0.00%, 0.01%} BD-rate difference for {Y, U, V} components over classes B, C and D.

LDB: {-0.01%, -0.06%, -0.06%} BD-rate difference for {Y, U, V} components over classes C, D and E.

# Introduction

In JVET-AI0089 [2], an RA/LDB unified reference frame synthesis method named URFS was proposed. URFS takes two previously decoded frames from the decoded picture buffer (DPB) as input and learns to interpolate (i.e., RA configuration) or extrapolate (i.e., LDB configuration) a reference frame. The reference frame is inserted into RPL for the subsequent inter prediction. For RA and LDB configurations, URFS adopts the same network architecture that is optimized end-to-end.

In JVET-AJ0099 [1], the URFS model is integrated in the latest NNVC-v10rc anchor. In that contribution, two tests were performed and reported:

Test 3.1.1: Trained on Vimeo-90K triplet (compressed data by VTM-11.0-NNVC-4.0).

Test 3.1.2: Trained on Vimeo-90K triplet (compressed data by VTM-11.0-NNVC-9.0).

The inference code is available at [5] and is provided on top of NNVC-4.0 (Test 3.1.1) and NNVC-v10rc (Test 3.1.2). The URFS implementation in NNVC software is based on JVET-AC0114 [3] and JVET-AD0160 [4], which utilizes an embedded Python interpreter in C++ for a floating-point model.

The provided code contained bugs in both implementations. Most were quickly fixed, apart from a bug in Slice.cpp for Test 3.1.2, present during inference under the RA configuration. Therefore, the encoded bitstream for Test 3.1.2 under RA configuration is not compliant with reference pictures having to be in a temporal layer lower than the picture to be coded.

We were not able to run Test 3.1.1 code on the cluster due to persistent issues with CMake compilation under the provided Python 3.8 virtual environment. Therefore, the results presented in the following section are for Test 3.1.2, encoder side.

# Results

The following tables show our experimental results for the inference cross-check. The encoding and decoding utilise the URFS RA and LDB fp32 models trained and provided by the proponents of JVET-AJ0099. The results were generated using a Python 3.8 environment with Torch 2.1.1. In JVET-AJ0099, proponents report using Torch 2.0 but have provided a virtual environment with Torch 2.1.1 for the crosscheck.

The results presented are for Test 3.1.2, encoder side – URFS over NNVC-v10rc.

### Crosscheck results on EE1-3.1 Test 3.1.2

Table 1 BD-rate performance of the proposed URFS over VTM-11.0_NNVC-10.0 (NN tools on)

under RA configuration

| Class | BD-rate Over VTM-11.0_NNVC-10.0 (Random access Main10) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -2.95% | -2.87% | -2.84% | -2.96% | -3.78% | -4.80% |
| Class C | -4.98% | -6.65% | -7.20% | -5.48% | -6.61% | -8.47% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | -6.88% | -6.82% | -6.86% | -5.62% | -6.55% | -6.65% |

Table 2 BD-rate performance of the proposed URFS over VTM-11.0_NNVC-10.0 (NN tools on)

under LDB configuration

| Class | BD-rate Over VTM-11.0_NNVC-10.0 (Low delay B Main10) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class B |  |  |  |  |  |  |
| Class C | -4.38% | -7.60% | -5.98% | -4.46% | -6.87% | -5.94% |
| Class E | -3.35% | -2.95% | 1.38% | -2.63% | -3.96% | -1.90% |
| Overall |  |  |  |  |  |  |
| Class D | -4.05% | -3.81% | -1.16% | -3.22% | -2.82% | 5.37% |

### Difference between crosscheck result and EE1-3.1 (JVET-AJ0099) result

Table 3 Difference in BD-rate performance between crosscheck and JVET-AJ0099

under RA configuration

| Class | BD-rate Over JVET-AJ0099 (Random access Main10) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.01% | 0.01% | 0.04% | 0.06% | 0.03% | -0.10% |
| Class C | 0.02% | 0.02% | 0.00% | 0.03% | 0.00% | -0.14% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D | -0.01% | -0.03% | -0.01% | 0.04% | -0.08% | -0.09% |

Table 4 Difference in BD-rate performance between crosscheck and JVET-AJ0099

under LDB configuration

| Class | BD-rate Over VTM-11.0_NNVC-10.0 (Low delay B Main10) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class B |  |  |  |  |  |  |
| Class C | -0.04% | -0.23% | -0.23% | 0.01% | -0.42% | -0.07% |
| Class E | 0.01% | -0.17% | 0.30% | 0.07% | 0.16% | -0.50% |
| Overall |  |  |  |  |  |  |
| Class D | -0.01% | 0.20% | -0.16% | -0.19% | 0.33% | -0.35% |

## Remarks

Overall, results for the RA and LDB configurations are promising. Additionally, the bug present in Slice.cpp has been identified and proponents are working on correcting it.

Compared to the proponents’ results: performance of RA and LDB configurations is almost identical.

# References

- Q. Qin and C. Jung. EE1-3.1: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding. JVET-AJ0099. October 2024.

- Q. Qin and C. Jung. AHG11: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding. JVET-AI0089, 25th Meeting, Sapporo, JP, 12–19 July 2024.

- J. Jia, Y. Zhang, H. Zhu, Z. Chen, Z. Liu, X. Xu, S. Liu. AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AC0114, 29th Meeting, by teleconference, 11–20 January 2023.

- J. Jia, Y. Zhang, H. Zhu, Z. Chen, Z. Liu, X. Xu, S. Liu. EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AD0160, 30th Meeting, Antalya, TR, 21–28 April 2023.

- https://vcgit.hhi.fraunhofer.de/jvet-ai-ee1/VVCSoftware_VTM/-/tree/EE1-3.1

# Patent rights declaration(s)

Nokia does not have any current or pending patent rights relating to the technology described in this contribution.