| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0361 |
| --- | --- |

| Title: | AhG11/AhG14: Current NNVC models analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Informative |  |  |
| Author(s) or Contact(s): | F. Galpin E. Alshina | Email: | franck.galpin@interdigital.com elena.alshina@huawei.com |
| Source: |  |  |  |

_____________________________

# Abstract

This report summarizes some aspects of Neural Network models integrated in NNVC software. Especially, the depth of the kernel size or the depth of the input latent when relevant is reported to assess the hardware friendliness of the model’s architecture.

# Introduction

In NNVC software, several models were integrated. The architecture of these models was decided on a combination of performance, complexity and hardware friendliness constraints.

For reference, we assess the current properties of these models. For simplicity purpose, the assessment is performed on the following layers:

- Addition (simple addition or bias addition): the size of the input latent is reported

- Conv2D: the size of the kernel depth or the size of the latent depth are reported. In current SADL implementation, most of the convolution operations are accelerated based on the kernel depth. In some cases, the acceleration is based on the latent depth. Current implementation does not necessarily choose the best option.

- Matrix multiplication: the size of the input vectors is reported.

The hardware friendliness is highly dependent of the implementation and data layout. In the following, we report the friendliness based on current implementation and data layout (NHWC). Alternative assessment might be possible for other implementation choices.

# Hardware friendliness assessment

In the following, we report the relevant size (size) and the number of times the layer is used (count).

## VLOP2

Figure 1. LOP and VLOP filters in NNVC-10

| Add |  |  | Conv2D |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| size | count |  | size IN | size OUT | count |  |
| 2 | 1 |  | 1 | 1 | 2 | input |
| 4 | 1 |  | 4 | 1 | 1 | input |
| 8 | 2 |  | 6 | 2 | 1 | input |
| 16 | 8 |  | 6 | 4 | 1 | input |
| 32 | 24 |  | 6 | 8 | 1 | input |
| 48 | 1 |  | 16 | 8 | 1 | output |
| 64 | 3 |  | 16 | 32 | 2 |  |
| 96 | 7 |  | 16 | 16 | 5 |  |
| 20736 | 3 |  | 17 | 64 | 1 | comb inputs |
| 165888 | 1 |  | 32 | 32 | 15 |  |
| 497664 | 6 |  | 32 | 16 | 3 |  |
|  |  |  | 32 | 96 | 7 |  |
|  |  |  | 64 | 48 | 1 |  |
|  |  |  | 64 | 64 | 2 |  |
|  |  |  | 96 | 32 | 7 |  |

Most of the non-friendly operations are on the input’s tensors and output tensors. The total MAC count for these layers is negligible compared to other part of the model.

## LOP3

Figure 1. LOP and VLOP filters in NNVC-10

| Add |  |  | Conv2D |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| size | count |  | size IN | size OUT | count |  |
| 2 | 3 |  | 1 | 2 | 2 | input |
| 4 | 1 |  | 4 | 2 | 1 | input |
| 8 | 2 |  | 6 | 16 | 1 | input |
| 16 | 2 |  | 6 | 4 | 1 | input |
| 32 | 62 |  | 6 | 8 | 1 | input |
| 64 | 4 |  | 32 | 16 | 1 |  |
| 128 | 5 |  | 32 | 8 | 1 | output |
| 144 | 15 |  | 32 | 144 | 15 |  |
| 663552 | 4 |  | 32 | 32 | 42 |  |
| 746496 | 14 |  | 32 | 128 | 5 |  |
|  |  |  | 34 | 64 | 1 |  |
|  |  |  | 64 | 64 | 3 |  |
|  |  |  | 128 | 32 | 5 |  |
|  |  |  | 144 | 32 | 15 |  |

The non-friendly operations are on the input’s tensors and output tensors. The total MAC count for these layers is negligible compared to other part of the model.

## HOP5

Figure 4 HOP filter architecture with attention block

| Add |  |  | Conv2D |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| size | count |  | size IN | size OUT | count |  |
| 6 | 1 |  | 1 | 16 | 3 | input |
| 16 | 4 |  | 3 | 16 | 1 | input |
| 32 | 24 |  | 3 | 192 | 1 | input |
| 48 | 24 |  | 3 | 32 | 1 | input |
| 64 | 48 |  | 32 | 80 | 23 |  |
| 80 | 23 |  | 48 | 64 | 24 |  |
| 192 | 24 |  | 64 | 6 | 1 | output |
| 262144 | 25 |  | 64 | 64 | 15 |  |
|  |  |  | 64 | 192 | 23 |  |
|  |  |  | 64 | 32 | 23 |  |
|  |  |  | 64 | 48 | 23 |  |
|  |  |  | 272 | 64 | 23 |  |
|  |  |  | 288 | 48 | 1 |  |

The non-friendly operations are on the input’s tensors and output tensors. The total MAC count for these layers is negligible compared to other part of the model.

## Intra NN

The table shows the matrices multiplication sizes for each model. All models use sizes multiple of at least 16. However, these sizes are the ones of the dense model. For the sparse layers, the matrix/vector multiplication is automatically aligned with 16 length chunks in order to be hardware friendly.

## Super resolution models

| Add |  |  |  |
| --- | --- | --- | --- |
| size | count |  |  |
| 2 | 1 |  |  |
| 4 | 1 |  |  |
| 8 | 3 |  |  |
| 16 | 61 |  |  |
| 32 | 10 |  |  |
| 82944 | 12 |  |  |
| Conv2D |  |  |  |
| size IN | size OUT | count |  |
| 1 | 8 | 3 | input |
| 3 | 16 | 1 | input |
| 16 | 2 | 1 | output |
| 16 | 4 | 1 | output |
| 16 | 16 | 51 |  |
| 16 | 32 | 9 |  |
| 32 | 32 | 1 |  |
| 32 | 16 | 8 |  |
| 40 | 16 | 1 | comb. Inputs |

The non-friendly operations are on the input’s tensors and output tensors. The total MAC count for these layers is negligible compared to other part of the model.

# Notes

The analysis was performed using the “debug_model” software provided in the SADL repository, parsing the [ANALYZE] logs of the software.

The software is available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl/-/merge_requests/115