| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0363-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0134 (Non-EE2: Enhanced derivation of affine merge candidates) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yan Zhang 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | yzh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of JVET-AJ0134. The simulations were carried out based on the software provided by the proponents under common test condition. The partial results match with the result provided by the proponents.

# Introduction

Two modification aspects are included in the proposal:

The first aspect is with respect to temporal affine merge candidate derivation. In ECM-14.0, a temporal affine merge candidate derivation method is adopted. However, when scaling the CPMV from the collocated picture to the current reference picture, the default selected reference picture is solely used as the target reference picture. In the proposed method, the reference picture with reference index equals to 0 will always be used to derive the temporal affine merge candidate if the default reference index is not 0.

In the second aspect, after ARMC reordering of the sub-block merge list, up to 5 synthetic affine merge candidates can be added to the sub-block merge list. The synthetic affine merge candidates are generated by combining the motion of the first few affine merge candidates in the sub-block merge candidate list with each of the selected affine merge candidate providing motions of one reference picture list if available. The synthetic candidates are always bi.

The above two aspects are only added to non-low-delay B frames. Therefore, there’s no performance on LDB configurations.

# Simulation Results

The proposed method was implemented on top of ECM-14.0 and tested following common test condition described in JVET-AI2017.

Table 1. Crosscheck results of JVET-AJ0134

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0.10% | -0.08% | 0.12% | 101.0% | 102.0% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.07% | -0.17% | 0.04% | 100.0% | 102.1% | 100.3% | 101.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |