| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0365-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0237 (AHG12: 12-bit internal bit depth for ECM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao 1No. 27 North Third Ring West Road, Haidian District, Beijing Fangheng Fashion Center, 100098, Beijing, China | Tel: Email: | zhaolei.0324@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This contribution reports crosscheck results of JVET-AJ0237. It is confirmed that the tests results are matched, also the code change conforms with the descriptions.

Introduction

In JVET-AI0237, modifications on some identified tools are proposed to achieve better operation at 12-bit internal bit depth. A high-level summary of modifications are as follows:

- BIF:

For MAD based scale factor look-up, the MAD values are scaled according to internal bit-depth. For BIF offset derivation, its input sample value difference as well as the derived BIF offsets are scaled according to internal bit-depth.

- CCSAO:

For CCSAO edge-based classification, thresholds are scaled according to internal bit-depth. The CCSAO edge offsets and band offsets are scaled according to internal bit-depth before applying to the reconstructed sample values.

- ALF:

For variance calculation in ALF classification, the variance is normalized according to internal bit-depth.

For ALF residual scaling, the internal bit-depth is considered in residual correction calculation, as well as in the setting of min/max classifier index.

- Diversity based ARMC:

The lambda values used in the diversity based ARMC are scaled according to internal bit-depth.

- Motion compensated picture boundary padding:

The clipping value (used after DC value correction) is set according to the internal bit-depth instead of a hard-coded value.

- DMVR:

In the current DMVR, the precision of intermediate interpolated sample values is kept at 10-bit regardless of internal bit depth. It is proposed to make the precision of intermediate interpolated sample values to be the same as the internal bit depth (with the maximum possible precision being capped at 14-bit).

- CCCM and tools which uses CCCM-like model

The precision for covariance matrices is increased to 32-bit and the precision for decimals is increased to 22-bit when the internal bit depth is greater than 10.

- Coefficient sign prediction

The precision for reconstructed residual template is increased (from 8-bit to 16-bit).

Cross-check results

The crosscheck results on ECM-14.0 are illustrated as follows. It is reported that partial crosscheck results are matched with the simulation results from the proponent.

Table 1: Coding performance of JVET-AJ0237

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.17% | 0.56% | 0.41% | 99.5% | 99.9% |
| Class E | -0.30% | 0.11% | 0.25% | 100.3% | 101.1% |
| Overall |  |  |  |  |  |
| Class D | -0.13% | 0.65% | 0.50% | 99.9% | 100.9% |
| Class F |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.31% | 1.03% | 0.85% | 100.9% | 100.0% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.21% | 0.52% | 0.42% | 100.5% | 100.4% |
| Class F | -0.19% | 0.03% | 0.42% | 102.8% | 101.2% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.33% | 0.61% | -0.06% | 102.5% | 101.5% |
| Class E | -0.52% | -0.73% | -2.35% | 102.9% | 98.6% |
| Overall |  |  |  |  |  |
| Class D | -0.18% | 1.11% | 0.88% | 103.4% | 100.7% |
| Class F |  |  |  |  |  |

Patent rights declaration(s)

Bytedance Inc. do not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).