| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0369 |
| --- | --- |

| Title: | AHG7: On memory footprint of Matrix-based prediction, Transform and ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Naser F. Le Léannec F. Galpin E. François | Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution provides details of the ECM-14 memory footprint of some coding tools that are associated with large memory requirements. The provided details can be used in the ongoing discussion in AHG7 for establishing a fifth group of tools focusing on memory requirements. The ECM-14 memory computation is reported:

| Tool | Memory (KByte) |
| --- | --- |
| PDP | 7672.232 |
| ALF | 1949.696 |
| NSPT | 3601.92 |
| LFNST | 564.577 |
| MTS | 2229.267 |

In addition, compared with VTM-23.4 is made, the following is reported:

| Tool | Memory (KBytes) in VTM-23.4 | Memory (KBytes) ECM-14 | multiplicative factor |
| --- | --- | --- | --- |
| Matrix-based prediction | 4.7 | 7672.2 | 1632 |
| ALF | 11.5 | 1949.7 | 170 |
| Non-separable transform | 8.23 | 9621.9 | 1169 |
| MTS | 32.7 | 2115.9 | 65 |

This contribution proposes adding the above listed tools to the fifth group of tools with large memory footprint considered in AHG7.

# Introduction

During the discussion of AHG7 report, it was suggested to study the possibility of adding a new group of tools to be tested. The meeting notes stated:

After discussion in JVET, it was concluded the group 4 might not be appropriate for PDP. It was recommended to study potential for defining another group which would include tools with larger need for local memory bandwidth (e.g. also trained transforms, fixed filters might belong to such a category).

Among the different coding tools in ECM-14, it is found that the following ones have the highest memory footprint:

- PDP

- ALF

- NSPT

- LFNST

- MTS

To compute the memory footprint of each tool, the associated variables are examined and the respective size is computed. The overall memory is the sum of the individual variables of each tool. In the following sections, the memory details of each tool are provided. An excel sheet with the details is also provided in this contribution for further check.

# Memory Footprint of selected tools in ECM-14

## PDP

PDP is a Matrix-based position dependent intra prediction. In ECM-14, it replaces some of the conventional and MIP prediction modes. The memory calculation is based on ECM-14 files: RomMIPFilters.h and RomMIPFilters2.h. The first one corresponds to PDP replacement of conventional intra prediction and the second one corresponds to PDP replacement of MIP prediction. The following memory requirement is calculated:

| PDP conventional prediction |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| type | variable | dimension |  |  | size (Kbytes) |
| int16_t | g_weights4x4 | 11 | 16 | 36 | 12.672 |
| int16_t | g_weightsShort4x4 | 8 | 16 | 20 | 5.12 |
| int16_t | g_weights4x8 | 11 | 32 | 52 | 36.608 |
| int16_t | g_weightsShort4x8 | 8 | 32 | 28 | 14.336 |
| int16_t | g_weights8x4 | 11 | 32 | 52 | 36.608 |
| int16_t | g_weightsShort8x4 | 8 | 32 | 28 | 14.336 |
| int16_t | g_weights8x8 | 11 | 64 | 68 | 95.744 |
| int16_t | g_weightsShort8x8 | 8 | 64 | 36 | 36.864 |
| int16_t | g_weights4x16 | 11 | 64 | 84 | 118.272 |
| int16_t | g_weightsShort4x16 | 8 | 64 | 44 | 45.056 |
| int16_t | g_weights16x4 | 11 | 64 | 84 | 118.272 |
| int16_t | g_weightsShort16x4 | 8 | 64 | 44 | 45.056 |
| int16_t | g_weights8x16 | 11 | 128 | 100 | 281.6 |
| int16_t | g_weightsShort8x16 | 8 | 128 | 52 | 106.496 |
| int16_t | g_weights16x8 | 11 | 128 | 100 | 281.6 |
| int16_t | g_weightsShort16x8 | 8 | 128 | 52 | 106.496 |
| int16_t | g_weights16x16 | 11 | 256 | 132 | 743.424 |
| int16_t | g_weightsShort16x16 | 8 | 256 | 68 | 278.528 |
| int16_t | g_weights16x32 | 7 | 256 | 97 | 347.648 |
| int16_t | g_weightsShort16x32 | 4 | 256 | 49 | 100.352 |
| int16_t | g_weights32x16 | 7 | 256 | 97 | 347.648 |
| int16_t | g_weightsShort32x16 | 4 | 256 | 49 | 100.352 |
| int16_t | g_weights32x32 | 7 | 256 | 129 | 462.336 |
| int16_t | g_weightsShort32x32 | 4 | 256 | 65 | 133.12 |
| sum |  |  |  |  | 3868.544 |

| PDP-MIP |  |  |  |
| --- | --- | --- | --- |
| type | variable | # parameters | size (Kbytes) |
| int16_t | g_filterDataPdpMip | 1901844 | 3803.688 |

The total memory footprint for PDP is 7672.232 (3868.544+3803.688).

## ALF

This section is restricted to ALF fixed filters, defined in alfFixedFilters.h. The following memory requirement is calculated:

| ALF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| type | variable | dimension |  |  | size (Kbytes) |
| short | m_classIdnFixedFilter9Db9 | 8 | 7168 | 1 | 114.688 |
| short | m_filterCoeffFixed9Db9 | 8 | 512 | 41 | 335.872 |
| short | m_clippingFixed9Db9 | 8 | 512 | 41 | 335.872 |
| short | m_classIdnFixedFilter13Db9 | 8 | 7168 | 1 | 114.688 |
| short | m_filterCoeffFixed13Db9 | 8 | 512 | 64 | 524.288 |
| short | m_clippingFixed13Db9 | 8 | 512 | 64 | 524.288 |
| sum |  |  |  |  | 1949.696 |

## NSTP

The NSPT variables are defined romNSPT.h. The following memory requirement is calculated:

| NSPT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| type | variable | dimension |  |  |  | size (Kbytes) |
| int8_t | g_nspt4x4 | 35 | 3 | 16 | 16 | 26.88 |
| int8_t | g_nspt4x8 | 35 | 3 | 20 | 32 | 67.2 |
| int8_t | g_nspt4x16 | 35 | 3 | 24 | 64 | 161.28 |
| int8_t | g_nspt8x4 | 35 | 3 | 20 | 32 | 67.2 |
| int8_t | g_nspt8x8 | 35 | 3 | 32 | 64 | 215.04 |
| int8_t | g_nspt8x16 | 35 | 3 | 40 | 128 | 537.6 |
| int8_t | g_nspt16x4 | 35 | 3 | 24 | 64 | 161.28 |
| int8_t | g_nspt16x8 | 35 | 3 | 40 | 128 | 537.6 |
| int8_t | g_nspt4x32 | 35 | 3 | 20 | 128 | 268.8 |
| int8_t | g_nspt8x32 | 35 | 3 | 24 | 256 | 645.12 |
| int8_t | g_nspt32x4 | 35 | 3 | 20 | 128 | 268.8 |
| int8_t | g_nspt32x8 | 35 | 3 | 24 | 256 | 645.12 |
| sum |  |  |  |  |  | 3601.92 |

## LFNST

The LFNST variables are defined romLFNST.h. The following memory requirement is calculated:

| LFNST |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | variable | dimension |  |  |  | size (Kbytes) |
| uint8_t | g_lfnstLut | 97 | 1 | 1 | 1 | 0.097 |
| int8_t | g_lfnst4x4 | 35 | 3 | 16 | 16 | 26.88 |
| int8_t | g_lfnst8x8 | 35 | 3 | 32 | 64 | 215.04 |
| int8_t | g_lfnst16x16 | 35 | 3 | 32 | 96 | 322.56 |
| Sum |  |  |  |  |  | 564.577 |

## MTS

The MTS variables are defined in rom.h. Compared to other tools, these variables are not fully prestored, where some are calculated using the DST/DCT formulas. The following memory requirement is calculated:

| MTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| type | size(type) | variable | dimension |  |  | size (Kbytes) |
| TMatrixCoeff | 2 | g_trCoreDCT2P2 | 2 | 2 | 2 | 0.016 |
| TMatrixCoeff | 2 | g_trCoreDCT2P4 | 2 | 4 | 4 | 0.064 |
| TMatrixCoeff | 2 | g_trCoreDCT2P8 | 2 | 8 | 8 | 0.256 |
| TMatrixCoeff | 2 | g_trCoreDCT2P16 | 2 | 16 | 16 | 1.024 |
| TMatrixCoeff | 2 | g_trCoreDCT2P32 | 2 | 32 | 32 | 4.096 |
| TMatrixCoeff | 2 | g_trCoreDCT2P64 | 2 | 64 | 64 | 16.384 |
| TMatrixCoeff | 2 | g_trCoreDCT2P128 | 2 | 128 | 128 | 65.536 |
| TMatrixCoeff | 2 | g_trCoreDCT8P4 | 2 | 4 | 4 | 0.064 |
| TMatrixCoeff | 2 | g_trCoreDCT8P8 | 2 | 8 | 8 | 0.256 |
| TMatrixCoeff | 2 | g_trCoreDCT8P16 | 2 | 16 | 16 | 1.024 |
| TMatrixCoeff | 2 | g_trCoreDCT8P32 | 2 | 32 | 32 | 4.096 |
| TMatrixCoeff | 2 | g_trCoreDCT8P64 | 2 | 64 | 64 | 16.384 |
| TMatrixCoeff | 2 | g_trCoreDCT8P128 | 2 | 128 | 128 | 65.536 |
| TMatrixCoeff | 2 | g_trCoreDST7P4 | 2 | 4 | 4 | 0.064 |
| TMatrixCoeff | 2 | g_trCoreDST7P8 | 2 | 8 | 8 | 0.256 |
| TMatrixCoeff | 2 | g_trCoreDST7P16 | 2 | 16 | 16 | 1.024 |
| TMatrixCoeff | 2 | g_trCoreDST7P32 | 2 | 32 | 32 | 4.096 |
| TMatrixCoeff | 2 | g_trCoreDST7P64 | 2 | 64 | 64 | 16.384 |
| TMatrixCoeff | 2 | g_trCoreDST7P128 | 2 | 128 | 128 | 65.536 |
| TMatrixCoeff | 2 | g_trCoreDCT2P256 | 256 | 256 | 1 | 131.072 |
| TMatrixCoeff | 2 | g_trCoreDCT8P256 | 256 | 256 | 1 | 131.072 |
| TMatrixCoeff | 2 | g_trCoreDST7P256 | 256 | 256 | 1 | 131.072 |
| TMatrixCoeff | 2 | g_aiTr2 | 9 | 2 | 2 | 0.072 |
| TMatrixCoeff | 2 | g_aiTr4 | 9 | 4 | 4 | 0.288 |
| TMatrixCoeff | 2 | g_aiTr8 | 9 | 8 | 8 | 1.152 |
| TMatrixCoeff | 2 | g_aiTr16 | 9 | 16 | 16 | 4.608 |
| TMatrixCoeff | 2 | g_aiTr32 | 9 | 32 | 32 | 18.432 |
| TMatrixCoeff | 2 | g_aiTr64 | 9 | 64 | 64 | 73.728 |
| TMatrixCoeff | 2 | g_aiTr128 | 9 | 128 | 128 | 294.912 |
| TMatrixCoeff | 2 | g_aiTr256 | 9 | 256 | 256 | 1179.648 |
| uint8_t | 1 | g_aucIpmToTrSet | 16 | 36 | 1 | 0.576 |
| uint8_t | 1 | g_aucTrSet | 80 | 6 | 1 | 0.48 |
| int8_t | 1 | g_aiIdLut | 3 | 3 | 1 | 0.009 |
| uint8_t | 1 | g_aucTrIdxToTr | 25 | 2 | 1 | 0.05 |
| sum |  |  |  |  |  | 2229.267 |

## Summary

The following summary table is obtained from each tool:

| Tool | Memory (KByte) |
| --- | --- |
| PDP | 7672.232 |
| ALF | 1949.696 |
| NSPT | 3601.92 |
| LFNST | 564.577 |
| MTS | 2229.267 |

# Comparison to VTM

## MIP

The memory calculation is based on VTM-23.4 file: MipData.h. it is noted that the same data are also kept in ECM-14 for special modes that don’t use PDP. The following memory requirement is calculated:

| MIP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| type | variable | dimension |  |  | size (Kbytes) |
| uint8_t | mipMatrix4x4 | 16 | 16 | 4 | 1.024 |
| uint8_t | mipMatrix8x8 | 8 | 16 | 8 | 1.024 |
| uint8_t | mipMatrix16x16 | 6 | 64 | 7 | 2.688 |
| sum |  |  |  |  | 4.736 |

The amount of increase in matrix-based prediction (MIP and PDP) is from 4.7Kbytes to 7672.2Kbytes 7672Kbytes (=7672.2+4.7). That is, the amount of data is multiplied by a factor of 1632.

## ALF

This section is restricted to ALF fixed filters, defined in AdaptiveLoopFilter.cpp. The following memory requirement is calculated:

| ALF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| type | size(type) | variable | dimension |  | size (Kbytes) |
| alfCoeff | 2 | m_fixedFilterSetCoeff | 64 | 13 | 3.328 |
| int | 4 | m_classToFilterMapping | 16 | 64 | 8.192 |
| sum |  |  |  |  | 11.52 |

The amount of increase in ALF fixed filters is from 11.5Kbytes to 1949.7Kbytes. That is, the amount of data is multiplied by a factor of 170.

## LFNST

The LFNST variables are defined romlfnst.cpp. The following memory requirement is calculated:

| LFNST |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| type | variable | dimension |  |  |  | size (Kbytes) |
| uint8_t | g_lfnstLut | 97 | 1 | 1 | 1 | 0.097 |
| uint8_t | g_lfnst8x8 | 4 | 2 | 16 | 48 | 6.144 |
| uint8_t | g_lfnst4x4 | 4 | 2 | 16 | 16 | 2.048 |
| sum |  |  |  |  |  | 8.289 |

The amount of increase in non-separable transforms, both LFNST and NSPT, is from 8.3Kbytes to 4166.5Kbytes (3601.92+564.57). That is, the amount of data is multiplied by a factor of 1169.

## MTS

The MTS variables are defined rom.h. Compared to other tools, these variables are not fully prestored, where some are calculated using the DST/DCT formulas. It should be noted that some coefficients are shared between ECM and VTM as they correspond to the same transform. The following memory requirement is calculated:

| MTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| type | size(type) | variable | dimension |  |  | size (Kbytes) |
| TMatrixCoeff | 2 | g_trCoreDCT2P2 | 2 | 2 | 2 | 0.016 |
| TMatrixCoeff | 2 | g_trCoreDCT2P4 | 2 | 4 | 4 | 0.064 |
| TMatrixCoeff | 2 | g_trCoreDCT2P8 | 2 | 8 | 8 | 0.256 |
| TMatrixCoeff | 2 | g_trCoreDCT2P16 | 2 | 16 | 16 | 1.024 |
| TMatrixCoeff | 2 | g_trCoreDCT2P32 | 2 | 32 | 32 | 4.096 |
| TMatrixCoeff | 2 | g_trCoreDCT2P64 | 2 | 64 | 64 | 16.384 |
| TMatrixCoeff | 2 | g_trCoreDCT8P4 | 2 | 4 | 4 | 0.064 |
| TMatrixCoeff | 2 | g_trCoreDCT8P8 | 2 | 8 | 8 | 0.256 |
| TMatrixCoeff | 2 | g_trCoreDCT8P16 | 2 | 16 | 16 | 1.024 |
| TMatrixCoeff | 2 | g_trCoreDCT8P32 | 2 | 32 | 32 | 4.096 |
| TMatrixCoeff | 2 | g_trCoreDST7P4 | 2 | 4 | 4 | 0.064 |
| TMatrixCoeff | 2 | g_trCoreDST7P8 | 2 | 8 | 8 | 0.256 |
| TMatrixCoeff | 2 | g_trCoreDST7P16 | 2 | 16 | 16 | 1.024 |
| TMatrixCoeff | 2 | g_trCoreDST7P32 | 2 | 32 | 32 | 4.096 |
| sum |  |  |  |  |  | 32.72 |

The new added transforms are those corresponding to 128x128 kernels, 256x256 kernels, additional transforms (DCT5, DST1, etc..). Specifically, these are the added transforms in ECM compared to VTM:

| MTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| type | size(type) | variable | dimension |  |  | size (Kbytes) |
| TMatrixCoeff | 2 | g_trCoreDCT2P128 | 2 | 128 | 128 | 65.536 |
| TMatrixCoeff | 2 | g_trCoreDCT8P64 | 2 | 64 | 64 | 16.384 |
| TMatrixCoeff | 2 | g_trCoreDCT8P128 | 2 | 128 | 128 | 65.536 |
| TMatrixCoeff | 2 | g_trCoreDST7P4 | 2 | 4 | 4 | 0.064 |
| TMatrixCoeff | 2 | g_trCoreDST7P8 | 2 | 8 | 8 | 0.256 |
| TMatrixCoeff | 2 | g_trCoreDST7P16 | 2 | 16 | 16 | 1.024 |
| TMatrixCoeff | 2 | g_trCoreDCT2P256 | 256 | 256 | 1 | 131.072 |
| TMatrixCoeff | 2 | g_trCoreDCT8P256 | 256 | 256 | 1 | 131.072 |
| TMatrixCoeff | 2 | g_trCoreDST7P256 | 256 | 256 | 1 | 131.072 |
| TMatrixCoeff | 2 | g_aiTr2 | 9 | 2 | 2 | 0.072 |
| TMatrixCoeff | 2 | g_aiTr4 | 9 | 4 | 4 | 0.288 |
| TMatrixCoeff | 2 | g_aiTr8 | 9 | 8 | 8 | 1.152 |
| TMatrixCoeff | 2 | g_aiTr16 | 9 | 16 | 16 | 4.608 |
| TMatrixCoeff | 2 | g_aiTr32 | 9 | 32 | 32 | 18.432 |
| TMatrixCoeff | 2 | g_aiTr64 | 9 | 64 | 64 | 73.728 |
| TMatrixCoeff | 2 | g_aiTr128 | 9 | 128 | 128 | 294.912 |
| TMatrixCoeff | 2 | g_aiTr256 | 9 | 256 | 256 | 1179.648 |
| uint8_t | 1 | g_aucIpmToTrSet | 16 | 36 | 1 | 0.576 |
| uint8_t | 1 | g_aucTrSet | 80 | 6 | 1 | 0.48 |
| int8_t | 1 | g_aiIdLut | 3 | 3 | 1 | 0.009 |
| uint8_t | 1 | g_aucTrIdxToTr | 25 | 2 | 1 | 0.05 |
| sum |  |  |  |  |  | 2115.971 |

That is, the amount of data is multiplied by a factor of 65.

## Overall comparison

The overall memory comparison of the studied tools compared to VTM is provided in the table below:

| Tool | Memory (KBytes) in VTM-23.4 | Memory (KBytes) ECM-14 | multiplicative factor |
| --- | --- | --- | --- |
| Matrix-based prediction | 4.7 | 7672.2 | 1632 |
| ALF | 11.5 | 1949.7 | 170 |
| Non-separable transform | 8.23 | 9621.9 | 1169 |
| MTS | 32.7 | 2115.9 | 65 |

Although the memory increase from VTM to ECM appears to be large, it is quite typical in the face of development. It is also typical that the memory increases from one standard to the next one, which is true if VVC is compared to HEVC.

# Conclusion

This contribution studies the memory footprint of PDP, ALF, NSPT, LFNST and MTS. It is observed that these coding tools require larger memory compared to others. It is suggested to include these tools under the new group (group #5) of tools having large memory footprint

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.