| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0369-v2 |
| --- | --- |

| Title: | AHG7: Information on memory footprint in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | K. Naser F. Le Léannec F. Galpin E. François | Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution provides non-exhaustive informational data on ECM-14 memory footprint of some coding tools, within the scope of defining new Ahg7 category of tools associated with large memory requirements. The provided details can be used in the ongoing discussion in AHG7 for establishing a fifth group of tools focusing on memory requirements. The ECM-14 memory computation is reported:

| Tool | Memory (KByte) |
| --- | --- |
| PDP | 7672.232 |
| ALF | 1949.696 |
| NSPT | 3601.92 |
| LFNST | 564.577 |
| MTS | 2229.267 |

It is stated that this contribution only aims at providing AHG7 with useful information. However, it does not express any opinion towards the cited tools. It is also stated that since ECM is intended to exploratory work towards video compression performance beyond VVC, it is natural that the memory footprint of developed tools is not optimized at all at this stage.

# Introduction

During the discussion of AHG7 report, it was suggested to study the possibility of adding a new group of tools to be tested. The meeting notes stated:

After discussion in JVET, it was concluded the group 4 might not be appropriate for PDP. It was recommended to study potential for defining another group which would include tools with larger need for local memory bandwidth (e.g. also trained transforms, fixed filters might belong to such a category).

Among the different coding tools in ECM-14, the memory footprint of following tools is reported.

- PDP

- ALF

- NSPT

- LFNST

- MTS

To compute the memory footprint of each tool, the associated variables are examined, and the respective size is computed. The overall memory is the sum of the individual variables of each tool. In the following sections, the memory details of each tool are provided. An excel sheet with the details is also provided in this contribution for further check.

# Memory Footprint of selected tools in ECM-14

## PDP

PDP is a Matrix-based position dependent intra prediction. In ECM-14, it replaces some of the conventional and MIP prediction modes. The memory calculation is based on ECM-14 files: RomMIPFilters.h and RomMIPFilters2.h. The first one corresponds to PDP replacement of conventional intra prediction, and the second one corresponds to PDP replacement of MIP prediction. The following memory requirement is calculated:

| PDP conventional prediction |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| type | variable | dimension |  |  | size (Kbytes) |
| int16_t | g_weights4x4 | 11 | 16 | 36 | 12.672 |
| int16_t | g_weightsShort4x4 | 8 | 16 | 20 | 5.12 |
| int16_t | g_weights4x8 | 11 | 32 | 52 | 36.608 |
| int16_t | g_weightsShort4x8 | 8 | 32 | 28 | 14.336 |
| int16_t | g_weights8x4 | 11 | 32 | 52 | 36.608 |
| int16_t | g_weightsShort8x4 | 8 | 32 | 28 | 14.336 |
| int16_t | g_weights8x8 | 11 | 64 | 68 | 95.744 |
| int16_t | g_weightsShort8x8 | 8 | 64 | 36 | 36.864 |
| int16_t | g_weights4x16 | 11 | 64 | 84 | 118.272 |
| int16_t | g_weightsShort4x16 | 8 | 64 | 44 | 45.056 |
| int16_t | g_weights16x4 | 11 | 64 | 84 | 118.272 |
| int16_t | g_weightsShort16x4 | 8 | 64 | 44 | 45.056 |
| int16_t | g_weights8x16 | 11 | 128 | 100 | 281.6 |
| int16_t | g_weightsShort8x16 | 8 | 128 | 52 | 106.496 |
| int16_t | g_weights16x8 | 11 | 128 | 100 | 281.6 |
| int16_t | g_weightsShort16x8 | 8 | 128 | 52 | 106.496 |
| int16_t | g_weights16x16 | 11 | 256 | 132 | 743.424 |
| int16_t | g_weightsShort16x16 | 8 | 256 | 68 | 278.528 |
| int16_t | g_weights16x32 | 7 | 256 | 97 | 347.648 |
| int16_t | g_weightsShort16x32 | 4 | 256 | 49 | 100.352 |
| int16_t | g_weights32x16 | 7 | 256 | 97 | 347.648 |
| int16_t | g_weightsShort32x16 | 4 | 256 | 49 | 100.352 |
| int16_t | g_weights32x32 | 7 | 256 | 129 | 462.336 |
| int16_t | g_weightsShort32x32 | 4 | 256 | 65 | 133.12 |
| sum |  |  |  |  | 3868.544 |

| PDP-MIP |  |  |  |
| --- | --- | --- | --- |
| type | variable | # parameters | size (Kbytes) |
| int16_t | g_filterDataPdpMip | 1901844 | 3803.688 |

The total memory footprint for PDP is 7672.232 (3868.544+3803.688).

## ALF

This section is restricted to ALF fixed filters, defined in alfFixedFilters.h. The following memory requirement is calculated:

| ALF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| type | variable | dimension |  |  | size (Kbytes) |
| short | m_classIdnFixedFilter9Db9 | 8 | 7168 | 1 | 114.688 |
| short | m_filterCoeffFixed9Db9 | 8 | 512 | 41 | 335.872 |
| short | m_clippingFixed9Db9 | 8 | 512 | 41 | 335.872 |
| short | m_classIdnFixedFilter13Db9 | 8 | 7168 | 1 | 114.688 |
| short | m_filterCoeffFixed13Db9 | 8 | 512 | 64 | 524.288 |
| short | m_clippingFixed13Db9 | 8 | 512 | 64 | 524.288 |
| sum |  |  |  |  | 1949.696 |

## NSTP

The NSPT variables are defined romNSPT.h. The following memory requirement is calculated:

| NSPT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| type | variable | dimension |  |  |  | size (Kbytes) |
| int8_t | g_nspt4x4 | 35 | 3 | 16 | 16 | 26.88 |
| int8_t | g_nspt4x8 | 35 | 3 | 20 | 32 | 67.2 |
| int8_t | g_nspt4x16 | 35 | 3 | 24 | 64 | 161.28 |
| int8_t | g_nspt8x4 | 35 | 3 | 20 | 32 | 67.2 |
| int8_t | g_nspt8x8 | 35 | 3 | 32 | 64 | 215.04 |
| int8_t | g_nspt8x16 | 35 | 3 | 40 | 128 | 537.6 |
| int8_t | g_nspt16x4 | 35 | 3 | 24 | 64 | 161.28 |
| int8_t | g_nspt16x8 | 35 | 3 | 40 | 128 | 537.6 |
| int8_t | g_nspt4x32 | 35 | 3 | 20 | 128 | 268.8 |
| int8_t | g_nspt8x32 | 35 | 3 | 24 | 256 | 645.12 |
| int8_t | g_nspt32x4 | 35 | 3 | 20 | 128 | 268.8 |
| int8_t | g_nspt32x8 | 35 | 3 | 24 | 256 | 645.12 |
| sum |  |  |  |  |  | 3601.92 |

## LFNST

The LFNST variables are defined romLFNST.h. The following memory requirement is calculated:

| LFNST |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | variable | dimension |  |  |  | size (Kbytes) |
| uint8_t | g_lfnstLut | 97 | 1 | 1 | 1 | 0.097 |
| int8_t | g_lfnst4x4 | 35 | 3 | 16 | 16 | 26.88 |
| int8_t | g_lfnst8x8 | 35 | 3 | 32 | 64 | 215.04 |
| int8_t | g_lfnst16x16 | 35 | 3 | 32 | 96 | 322.56 |
| Sum |  |  |  |  |  | 564.577 |

## MTS

The MTS variables are defined in rom.h. Compared to other tools, these variables are not fully prestored, where some are calculated using the DST/DCT formulas. The following memory requirement is calculated:

| MTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| type | size(type) | variable | dimension |  |  | size (Kbytes) |
| TMatrixCoeff | 2 | g_trCoreDCT2P2 | 2 | 2 | 2 | 0.016 |
| TMatrixCoeff | 2 | g_trCoreDCT2P4 | 2 | 4 | 4 | 0.064 |
| TMatrixCoeff | 2 | g_trCoreDCT2P8 | 2 | 8 | 8 | 0.256 |
| TMatrixCoeff | 2 | g_trCoreDCT2P16 | 2 | 16 | 16 | 1.024 |
| TMatrixCoeff | 2 | g_trCoreDCT2P32 | 2 | 32 | 32 | 4.096 |
| TMatrixCoeff | 2 | g_trCoreDCT2P64 | 2 | 64 | 64 | 16.384 |
| TMatrixCoeff | 2 | g_trCoreDCT2P128 | 2 | 128 | 128 | 65.536 |
| TMatrixCoeff | 2 | g_trCoreDCT8P4 | 2 | 4 | 4 | 0.064 |
| TMatrixCoeff | 2 | g_trCoreDCT8P8 | 2 | 8 | 8 | 0.256 |
| TMatrixCoeff | 2 | g_trCoreDCT8P16 | 2 | 16 | 16 | 1.024 |
| TMatrixCoeff | 2 | g_trCoreDCT8P32 | 2 | 32 | 32 | 4.096 |
| TMatrixCoeff | 2 | g_trCoreDCT8P64 | 2 | 64 | 64 | 16.384 |
| TMatrixCoeff | 2 | g_trCoreDCT8P128 | 2 | 128 | 128 | 65.536 |
| TMatrixCoeff | 2 | g_trCoreDST7P4 | 2 | 4 | 4 | 0.064 |
| TMatrixCoeff | 2 | g_trCoreDST7P8 | 2 | 8 | 8 | 0.256 |
| TMatrixCoeff | 2 | g_trCoreDST7P16 | 2 | 16 | 16 | 1.024 |
| TMatrixCoeff | 2 | g_trCoreDST7P32 | 2 | 32 | 32 | 4.096 |
| TMatrixCoeff | 2 | g_trCoreDST7P64 | 2 | 64 | 64 | 16.384 |
| TMatrixCoeff | 2 | g_trCoreDST7P128 | 2 | 128 | 128 | 65.536 |
| TMatrixCoeff | 2 | g_trCoreDCT2P256 | 256 | 256 | 1 | 131.072 |
| TMatrixCoeff | 2 | g_trCoreDCT8P256 | 256 | 256 | 1 | 131.072 |
| TMatrixCoeff | 2 | g_trCoreDST7P256 | 256 | 256 | 1 | 131.072 |
| TMatrixCoeff | 2 | g_aiTr2 | 9 | 2 | 2 | 0.072 |
| TMatrixCoeff | 2 | g_aiTr4 | 9 | 4 | 4 | 0.288 |
| TMatrixCoeff | 2 | g_aiTr8 | 9 | 8 | 8 | 1.152 |
| TMatrixCoeff | 2 | g_aiTr16 | 9 | 16 | 16 | 4.608 |
| TMatrixCoeff | 2 | g_aiTr32 | 9 | 32 | 32 | 18.432 |
| TMatrixCoeff | 2 | g_aiTr64 | 9 | 64 | 64 | 73.728 |
| TMatrixCoeff | 2 | g_aiTr128 | 9 | 128 | 128 | 294.912 |
| TMatrixCoeff | 2 | g_aiTr256 | 9 | 256 | 256 | 1179.648 |
| uint8_t | 1 | g_aucIpmToTrSet | 16 | 36 | 1 | 0.576 |
| uint8_t | 1 | g_aucTrSet | 80 | 6 | 1 | 0.48 |
| int8_t | 1 | g_aiIdLut | 3 | 3 | 1 | 0.009 |
| uint8_t | 1 | g_aucTrIdxToTr | 25 | 2 | 1 | 0.05 |
| sum |  |  |  |  |  | 2229.267 |

## Summary

The following summary table is obtained from each tool:

| Tool | Memory (KByte) |
| --- | --- |
| PDP | 7672.232 |
| ALF | 1949.696 |
| NSPT | 3601.92 |
| LFNST | 564.577 |
| MTS | 2229.267 |

# Conclusion

This contribution reports the memory footprint of PDP, ALF, NSPT, LFNST and MTS.

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.