| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting: Kemer, TR, 1–8 November 2024 | Document: JVET-AJ1006-v1 |
| --- | --- |

| Title: | HEVC extensions and corrections (draft 1) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Benjamin Bross Sachin Deshpande Gary J. Sullivan Alexis M. Tourapis | Email: | yekui.wang@bytedance.com benjamin.bross@hhi.fraunhofer.de sdeshpande@sharplabs.com gary.sullivan@dolby.com atourapis@apple.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for changes to the High Efficiency Video Coding (HEVC) standard (Rec. ITU-T H.265 | ISO/IEC 23008-2). The changes include the support of a new profile, namely the Multiview Main 10 profile.

Changes that have been integrated for the 36th JVET meeting in November 2024:

- JVET-AJ0213 On Multiview Main 10 profiles

- Bug fix: The variable subBitstream was used without being defined for the Multiview Extended or Multiview Extended 10 profiles.

- Bug fix: Change the general_profile_idc value for the multiview format range extension profiles from 10 to 14, to differ from that for the multiview format range extension profiles.

Changes to the specification text:

# Subclause F.11.2

Replace Table F.3 with the following:

Table F.3 — Specification of CompatibleProfileList

| Profile to which the decoder conforms | Profiles that the decoder shall support CompatibleProfileList |
| --- | --- |
| Scalable Main | Scalable Main, Main, Main Still Picture |
| Scalable Main 10 | Scalable Main 10, Main, Main Still Picture, Main 10, Scalable Main |
| Scalable Monochrome | The compatible format range extensions profiles of the Monochrome profile, and the compatible scalable format range extensions profiles of the Scalable Monochrome profile |
| Scalable Monochrome 12 | The compatible format range extensions profiles of the Monochrome 12 profile, and the compatible scalable format range extensions profiles of the Scalable Monochrome 12 profile |
| Scalable Monochrome 16 | The compatible format range extensions profiles of the Monochrome 16 profile, and the compatible scalable format range extensions profiles of the Scalable Monochrome 16 profile |
| Scalable Main 4:4:4 | Scalable Main, Main, Main Still Picture, the compatible format range extensions profiles of the Main 4:4:4 profile, and the compatible scalable format range extensions profiles of the Scalable Main 4:4:4 profile |
| Multiview Main | Multiview Main, Main, Main Still Picture |
| Multiview Main 10 | Multiview Main 10, Multiview Main, Main 10, Main, Main Still Picture |
| Multiview Extended | Multiview Extended, Multiview Main, Main, Main Still Picture |
| Multiview Extended 10 | Multiview Extended 10, Main, Main Still Picture, Main 10, Multiview Main, Multiview Extended |
| Multiview Monochrome | Multiview Monochrome, Monochrome |
| Multiview Monochrome 10 | Multiview Monochrome 10, Multiview Monochrome, Monochrome 10, Monochrome |
| Multiview Monochrome 12 | Multiview Monochrome 12, Multiview Monochrome 10, Multiview Monochrome, Monochrome 12, Monochrome 10, Monochrome |
| Multiview Monochrome 16 | Multiview Monochrome 16, Multiview Monochrome 12, Multiview Monochrome 10, Multiview Monochrome, Monochrome 16, Monochrome 12, Monochrome 10, Monochrome |
| 3D Main | 3D Main, Multiview Main, Main, Main Still Picture |

# Subclause G.11

Make the following changes:

  - Profiles, tiers, and levels

    - Profiles

      - Multiview Main and Multiview Main 10 profiles

For a layer in an output operation point associated with an OLS in a bitstream, the layer conforming to the Multiview Main or Multiview Main 10 profile, the following applies:

—	Let olsIdx be the OLS index of the OLS, the sub-bitstream subBitstream and the base layer sub-bitstream baseBitstream are derived as specified in F.11.3.

When vps_base_layer_internal_flag is equal to 1, the base layer sub-bitstream baseBitstream shall obey the following constraints:

—	When the layer conforms to the Multiview Main profile, the base layer sub-bitstream baseBitstream shall be indicated to conform to the Main profile.

—	When the layer conforms to the Multiview Main 10 profile, the base layer sub-bitstream baseBitstream shall be indicated to conform to the Main 10 or Main profile.

When the layer conforms to the Multiview Main or Multiview Main 10 profile, the sub-bitstream subBitstream shall obey the following constraints:

—	All active VPSs shall have vps_num_rep_formats_minus1 in the range of 0 to 15, inclusive.

—	All active SPSs for layers in the sub-bitstream subBitstream shall have chroma_format_idc equal to 1 only.

—	All active SPSs for layers in the sub-bitstream subBitstream shall have transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, persistent_rice_adaptation_enabled_flag, and cabac_bypass_alignment_enabled_flag, when present, equal to 0 only.

—	CtbLog2SizeY derived from all active SPSs for layers in the sub-bitstream subBitstream shall be in the range of 4 to 6, inclusive.

—	All active PPSs for layers in the sub-bitstream subBitstream shall have log2_max_transform_skip_block_size_minus2 and chroma_qp_offset_list_enabled_flag, when present, equal to 0 only.

—	ScalabilityId[ j ][ smIdx ] derived according to any active VPS shall be equal to 0 for any smIdx value not equal to 1 or 3 and for any value of j such that layer_id_in_nuh[ j ] is among layerIdListTarget that was used to derive the sub-bitstream subBitstream.

—	When NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ] is equal to 2, output_layer_flag[ olsIdx ][ j ] derived according to any active VPS shall be equal to 1 for j in the range of 0 to 1, inclusive, for the sub-bitstream subBitstream.

—	All active VPSs shall have alt_output_layer_flag[ olsIdx ] equal to 0 only.

—	When ViewOrderIdx[ i ] derived according to any active VPS is equal to 1 for the layer with nuh_layer_id equal to i in the sub-bitstream subBitstream, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_multilayer_extension( ) syntax structure in each active SPS for that layer.

—	When ViewOrderIdx[ i ] derived according to any active VPS is greater than 0 for the layer with nuh_layer_id equal to i in the sub-bitstream subBitstream, num_ref_loc_offsets shall be equal to 0 in each active PPS for that layer.

—	When ViewOrderIdx[ i ] derived according to any active VPS is greater than 0 for the layer with nuh_layer_id equal to i in the sub-bitstream subBitstream, the values of pic_width_in_luma_samples and pic_height_in_luma_samples in each active SPS for that layer shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in each active SPS for all reference layers of that layer.

—	For a layer with nuh_layer_id iNuhLId equal to any value included in layerIdListTarget that was used to derive the sub-bitstream subBitstream, the value of NumRefLayers[ iNuhLId ], which specifies the total number of direct and indirect reference layers and is derived as specified in F.7.4.3.1, shall be less than or equal to 4.

—	All active SPSs for layers in the sub-bitstream subBitstream shall have sps_range_extension_flag and sps_scc_extension_flag equal to 0 only.

—	All active PPSs for layers in the sub-bitstream subBitstream shall have pps_range_extension_flag and pps_scc_extension_flag equal to 0 only.

—	All active PPSs for layers in the sub-bitstream subBitstream shall have colour_mapping_enabled_flag equal to 0 only.

—	When an active PPS for any layer in the sub-bitstream subBitstream has tiles_enabled_flag equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

—	When an active PPS for any layer in the sub-bitstream subBitstream has tiles_enabled_flag equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

—	The number of times read_bits( 1 ) is called in 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any CTU shall be less than or equal to 5 * RawCtuBits / 3.

—	The tier and level constraints specified for the Multiview Main profile in G.11.2 shall be fulfilled.

—	For any active VPS, ViewOrderIdx[ i ] shall be greater than ViewOrderIdx[ j ] for any values of i and j among layerIdListTarget that was used to derive the sub-bitstream subBitstream such that AuxId[ i ] is equal to AuxId[ j ] and i is greater than j.

When the layer conforms to the Multiview Main profile, the sub-bitstream subBitstream shall obey the following constraints:

—	All active SPSs for layers in the sub-bitstream subBitstream shall have bit_depth_luma_minus8 equal to 0 only.

—	All active SPSs for layers in the sub-bitstream subBitstream shall have bit_depth_chroma_minus8 equal to 0 only.

—	The tier and level constraints specified for the Multiview Main profile in G.11.2 shall be fulfilled.

When the layer conforms to the Multiview Main 10 profile, the sub-bitstream subBitstream shall obey the following constraints:

—	All active SPSs for layers in subBitstream shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

—	All active SPSs for layers in subBitstream shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

—	For a layer layerA with nuh_layer_id equal to i in the sub-bitstream subBitstream and another layer layerB with nuh_layer_id equal to j, when layerB is a reference layer of layerA, the value of bit_depth_luma_minus8 for layerA and the value of bit_depth_luma_minus8 for layerB shall be identical, and the value of bit_depth_chroma_minus8 for layerA and the value of bit_depth_chroma_minus8 for layerB shall be identical.

—	The tier and level constraints specified for the Multiview Main 10 profile in G.11.2 shall be fulfilled.

In the remainder of this subclause, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Multiview Main profile is indicated as follows:

—	If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by having both of the following conditions satisfied:

—	general_profile_idc being equal to 6 or general_profile_compatibility_flag[ 6 ] being equal to 1

—	general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 1, general_max_422chroma_constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_constraint_flag being equal to 0, general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1

—	Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by having both of the following conditions satisfied:

—	sub_layer_profile_idc[ OpTid ] being equal to 6 or sub_layer_profile_compatibility_flag[ OpTid ][ 6 ] being equal to 1

—	sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0, and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Multiview Main 10 profile is indicated as follows:

—	If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by having both of the following conditions satisfied:

—	general_profile_idc being equal to 6 or general_profile_compatibility_flag[ 6 ] being equal to 1,

—	general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 0, general_max_422chroma_constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_constraint_flag being equal to 0, general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1.

—	Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by having both of the following conditions satisfied:

—	sub_layer_profile_idc[ OpTid ] being equal to 6 or sub_layer_profile_compatibility_flag[ OpTid ][ 6 ] being equal to 1,

—	sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 0, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0, and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1.

      - Multiview Extended and Multiview Extended 10 profiles

For a layer in an output operation point associated with an OLS in a bitstream, the layer conforming to the Multiview Extended or Multiview Extended 10 profile, the following applies:

—	Let olsIdx be the OLS index of the OLS, the sub-bitstream subBitstream is derived as specified in F.11.3.

When the layer (referred to as the current layer below) conforms to the Multiview Extended or Multiview Extended 10 profile, the following constraints apply:

- All active VPSs for the sub-bitstream subBitstream shall have vps_num_rep_formats_minus1 in the range of 0 to 15, inclusive.

- All active SPSs for the current layer and its reference layers shall have chroma_format_idc equal to 1 only.

—	All active SPSs for the current layer and its reference layers shall have transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_‌enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, persistent_rice_adaptation_‌enabled_flag, and cabac_bypass_alignment_enabled_flag, when present, equal to 0 only.

- CtbLog2SizeY derived from all active SPSs for the current layer and its reference layers shall be in the range of 4 to 6, inclusive.

- All active PPSs for the current layer and its reference layers shall have log2_max_transform_skip_block_size_minus2 and chroma_qp_offset_list_enabled_flag, when present, equal to 0 only.

- When NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ] is equal to 2, output_layer_flag[ olsIdx ][ j ] derived according to any active VPS for the sub-bitstream subBitstream shall be equal to 1 for j in the range of 0 to 1, inclusive.

- All active VPSs for the sub-bitstream subBitstream shall have alt_output_layer_flag[ olsIdx ] equal to 0 only.

- When ViewOrderIdx[ i ] derived according to any active VPS for the sub-bitstream subBitstream is equal to 1 for the layer, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_multilayer_extension( ) syntax structure in each active SPS for the layer.

—	When ViewOrderIdx[ i ] derived according to any active VPS for the sub-bitstream subBitstream is greater than 0 for the layer, num_ref_loc_offsets shall be equal to 0 in each active PPS for the layer.

- When ViewOrderIdx[ i ] derived according to any active VPS for the sub-bitstream subBitstream is greater than 0 for the layer, the values of pic_width_in_luma_samples and pic_height_in_luma_samples in each active SPS for the layer shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in each active SPS for all reference layers of the layer.

- The value of NumRefLayers[ iId ] for the current layer or any of its reference layers, which specifies the total number of direct and indirect reference layers and is derived as specified in F.7.4.3.1, shall be less than or equal to 4.

- All active SPSs for the current layer and its reference layers shall have sps_range_extension_flag and sps_scc_extension_flag equal to 0 only.

- All active PPSs for the current layer and its reference layers shall have pps_range_extension_flag and pps_scc_extension_flag equal to 0 only.

- When an active PPS for the current layer or any of its reference layers has tiles_enabled_flag equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When an active PPS for the current layer or any of its reference layers has tiles_enabled_flag equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- For the current layer or any of its reference layers, the number of times read_bits( 1 ) is called in 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any CTU shall be less than or equal to 5 * RawCtuBits / 3.

- For any active VPS for the sub-bitstream subBitstream, ViewOrderIdx[ i ] shall be greater than ViewOrderIdx[ j ] for any values of i and j among layerIdListTarget that was used to derive the sub-bitstream subBitstream such that one of these two layers is a reference layer of the other layer, AuxId[ i ] is equal to AuxId[ j ], and i is greater than j.

- All active PPSs for the current layer and its reference layers shall have colour_mapping_enabled_flag equal to 0 only.

- For the current layer, referred to as layerA and having nuh_layer_id equal to i, and another layer layerB with nuh_layer_id equal to j, when layerB is a reference layer of layerA, the value of bit_depth_luma_minus8 for layerA and the value of bit_depth_luma_minus8 for layerB shall be identical, and the value of bit_depth_chroma_minus8 for layerA and the value of bit_depth_chroma_minus8 for layerB shall be identical.

When the layer conforms to the Multiview Extended profile, the following constraints apply:

- All active SPSs for the layer shall have bit_depth_luma_minus8 equal to 0 only.

- All active SPSs for the layer shall have bit_depth_chroma_minus8 equal to 0 only.

- The tier and level constraints specified for the Multiview Extended profile in G.11.2 shall be fulfilled.

When the layer conforms to the Multiview Extended 10 profile, the following constraints apply:

- All active SPSs for the layer shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

- All active SPSs for the layer shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

- The tier and level constraints specified for the Multiview Extended 10 profile in G.11.2 shall be fulfilled.

In the remainder of this subclause and G.11.2.1, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.In the remainder of this subclause, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Multiview Main profile is indicated as follows:

—	If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by having both of the following conditions satisfied:

—	general_profile_idc being equal to 6 or general_profile_compatibility_flag[ 6 ] being equal to 1

—	general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 1, general_max_422chroma_constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_constraint_flag being equal to 0, general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1

—	Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by having both of the following conditions satisfied:

—	sub_layer_profile_idc[ OpTid ] being equal to 6 or sub_layer_profile_compatibility_flag[ OpTid ][ 6 ] being equal to 1

—	sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0, and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Multiview Extended profile is indicated as follows:

- If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by having both of the following conditions satisfied:

  - general_profile_idc being equal to 12 or general_profile_compatibility_flag[ 12 ] being equal to 1,

  - general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 1, general_max_422chroma_‌constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_constraint_flag being equal to 0, general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1.

- Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by having both of the following conditions satisfied:

  - sub_layer_profile_idc[ OpTid ] being equal to 12 or sub_layer_profile_‌compatibility_flag[ OpTid ][ 12 ] being equal to 1, and

  - sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_‌constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_‌monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0 and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1.

Conformance of a layer in an output operation point associated with an OLS in a bitstream to the Multiview Extended 10 profile is indicated as follows:

- If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by having both of the following conditions satisfied:

  - general_profile_idc being equal to 13 or general_profile_compatibility_flag[ 13 ] being equal to 1,

  - general_max_12bit_constraint_flag being equal to 1, general_max_10bit_constraint_flag being equal to 1, general_max_8bit_constraint_flag being equal to 0, general_max_‌422chroma_constraint_flag being equal to 1, general_max_420chroma_constraint_flag being equal to 1, general_max_monochrome_constraint_flag being equal to 0, general_intra_‌constraint_flag being equal to 0, general_one_picture_only_constraint_flag being equal to 0, and general_lower_bit_rate_constraint_flag being equal to 1.

- Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by having both of the following conditions satisfied:

  - sub_layer_profile_idc[ OpTid ] being equal to 13 or sub_layer_profile_‌compatibility_flag[ OpTid ][ 13 ] being equal to 1, and

  - sub_layer_max_12bit_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_10bit_‌constraint_flag[ OpTid ] being equal to 1, sub_layer_max_8bit_constraint_flag[ OpTid ] being equal to 0, sub_layer_max_422chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_420chroma_constraint_flag[ OpTid ] being equal to 1, sub_layer_max_‌monochrome_constraint_flag[ OpTid ] being equal to 0, sub_layer_intra_constraint_flag[ OpTid ] being equal to 0, sub_layer_one_picture_only_constraint_flag[ OpTid ] being equal to 0 and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] being equal to 1.

      - Multiview format range extensions profiles

The following profiles, collectively referred to as the multiview format range extensions profiles, are specified in this subclause:

- the Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles.

For a layer in an output operation point associated with an OLS in a bitstream, the layer conforming to the Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, or Multiview Monochrome 16 profile, the following applies:

- Let olsIdx be the OLS index of the OLS, the sub-bitstream subBitstream is derived as specified in F.11.3.

The sub-bitstream subBitstream and the layer (referred to as the current layer below) shall obey the following constraints:

- All active VPSs shall have vps_num_rep_formats_minus1 in the range of 0 to 15, inclusive.

- All active SPSs for the current layer and its reference layers shall have chroma_format_idc equal to 0 only.

- All active SPSs for the current layer and its reference layers shall have separate_colour_plane_flag, cabac_bypass_alignment_enabled_flag, when present, equal to 0 only.

- CtbLog2SizeY derived from all active SPSs for the current layer and its reference layers shall be in the range of 4 to 6, inclusive.

- The constraints specified in Table G.1 shall apply for all active SPSs and PPSs for the current layer and its reference layers.

- For the current layer, referred to as layerA and having nuh_layer_id equal to i, and another layer layerB with nuh_layer_id equal to j, when layerB is a reference layer of layerA, the value of chroma_format_idc for layerA and the value of chroma_format_idc for layerB shall be identical, the value of bit_depth_luma_minus8 for layerA and the value of bit_depth_luma_minus8 for layerB shall be identical, the value of bit_depth_chroma_minus8 for layerA and the value of bit_depth_chroma_minus8 for layerB shall be identical, and AuxId[ i ] shall be equal to AuxId[ j ].

- The values of pic_width_in_luma_samples and pic_height_in_luma_samples in each active SPS for the layer shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in each active SPS for all reference layers of that layer.

- All active SPSs for the current layer and its reference layers shall have the same value of chroma_format_idc.

- All active SPSs for the current layer and its reference layers shall have sps_scc_extension_flag equal to 0 only.

- All active PPSs for the current layer and its reference layers shall have pps_scc_extension_flag equal to 0 only.

- When an active PPS for the current layer or any of its reference layers has tiles_enabled_flag equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When an active PPS for the current layer or any of its reference layers has tiles_enabled_flag equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- For the current layer or any of its reference layers, the number of times read_bits( 1 ) is called in 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any CTU shall be less than or equal to 5 * RawCtuBits / 3.

- For any active VPS, ViewOrderIdx[ i ] shall be greater than ViewOrderIdx[ j ] for any values of i and j among layerIdListTarget that was used to derive the sub-bitstream subBitstream such that one of these two layers is a reference layer of the other layer, AuxId[ i ] is equal to AuxId[ j ], and i is greater than j.

- The tier and level constraints specified for the Multiview Monochrome, Multiview Monochrome 10, and Multiview Monochrome 12 profiles in G.11.2, as applicable, shall be fulfilled.

Table G.1 — Allowed values for syntax elements in the multiview format range extensions profiles

| Profile for which constraint is specified | chroma_format_idc | bit_depth_luma_minus8 and bit_depth_chroma_minus8 | transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, intra_smoothing_disabled_flag, persistent_rice_adaptation_enabled_flag, and log2_max_transform_skip_block_size_minus2 | extended_precision_processing_flag | chroma_qp_offset_list_enabled_flag |
| --- | --- | --- | --- | --- | --- |
| Multiview Monochrome | 0 | 0 | 0 | 0 | 0 |
| Multiview Monochrome 10 | 0 | 0..2 | 0 | 0 | 0 |
| Multiview Monochrome 12 | 0 | 0..4 | 0 | 0 | 0 |
| Multiview Monochrome 16 | 0 | 0..8 | 0 | 0 | 0 |

In the remainder of this clause and G.11.2.1, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.

Conformance of a layer in an output operation point associated with an OLS in a bitstream for the multiview format range extensions profiles is indicated as follows:

- If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by general_profile_idc being equal to 14 or general_profile_compatibility_flag[ 14 ] being equal to 1, with the additional indications specified in Table G.2 for the general constraint flags.

- Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by sub_layer_profile_idc[ OpTid ] being equal to 10 or sub_layer_profile_‌compatibility_flag[ OpTid ][ 10 ] being equal to 1, with the additional indications specified in Table H.4 for the flags associated with the index OpTid.

All other combinations of general_max_14bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_8bit_constraint_flag, general_max_422chroma_‌constraint_flag, general_max_420chroma_constraint_flag, general_max_monochrome_constraint_flag, general_intra_constraint_flag, general_one_picture_only_constraint_flag, and general_lower_bit_rate_‌constraint_flag with general_profile_idc equal to 14 or general_profile_compatibility_flag[ 14 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. All other combinations of sub_layer_‌max_14bit_constraint_flag[ OpTid ], sub_layer_max_12bit_constraint_flag[ OpTid ], sub_layer_‌max_10bit_constraint_flag[ OpTid ], sub_layer_max_8bit_constraint_flag[ OpTid ], sub_layer_‌max_422chroma_constraint_flag[ OpTid ], sub_layer_max_420chroma_constraint_flag[ OpTid ], sub_layer_max_‌monochrome_constraint_flag[ OpTid ], sub_layer_intra_constraint_flag[ OpTid ], sub_layer_one_‌picture_only_constraint_flag[ OpTid ], and sub_layer_lower_bit_rate_‌constraint_flag[ OpTid ] with sub_layer_profile_idc[ OpTid ] equal to 10 or sub_layer_profile_‌compatibility_flag[ OpTid ][ 10 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. Such combinations shall not be present in bitstreams conforming to this document. However, decoders conforming to the multiview format range extensions profiles shall allow other combinations as specified below in this clause to occur in the bitstream.

Table G.2 — Bitstream indications for conformance to multiview range extensions profiles

| Profile for which the bitstream indicates conformance | general_max_14bit_constraint_flag or sub_layer_max_14bit_constraint_‌flag[ OpTid ] | general_max_12bit_constraint_flag or sub_layer_max_12bit_constraint_‌flag[ OpTid ] | general_max_10bit_constraint_flag or sub_layer_max_10bit_constraint_‌flag[ OpTid ] | general_max_8bit_constraint_flag or sub_layer_max_8bit_constraint_‌flag[ OpTid ] | general_max_422chroma_constraint_flag or sub_layer_max_422chroma_constraint_‌flag[ OpTid ] | general_max_420chroma_constraint_flag or sub_layer_max_420chroma_constraint_‌flag[ OpTid ] | general_max_monochrome_constraint_flag or sub_layer_max_monochrome_constraint_‌flag[ OpTid ] | general_intra_constraint_flag or sub_layer_intra_constraint_‌flag[ OpTid ] | general_one_picture_only_constraint_flag or sub_layer_one_picture_only_constraint_‌flag[ OpTid ] | general_lower_bit_rate_constraint_flag or sub_layer_lower_bit_rate_constraint_‌flag[ OpTid ] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Multiview Monochrome | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| Multiview Monochrome 10 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| Multiview Monochrome 12 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| Multiview Monochrome 16 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |

    - Tiers and levels

      - General tier and level limits

In this subclause, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.

For purposes of comparison of tier capabilities, the tier with general_tier_flag or sub_layer_tier_flag[ i ] equal to 0 is considered to be a lower tier than the tier with general_tier_flag or sub_layer_tier_flag[ i ] equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc or sub_layer_level_idc[ i ] of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this subclause and G.11.2.2:

—	For the Multiview Main and Multiview Extended profiles, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.8 for the Main profile.

—	For the Multiview Main 10 and Multiview Extended 10 profiles, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor is the same as that specified in Table A.10 for the Main 10 profile.

—	For the Multiview Monochrome profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome profile.

—	For the Multiview Monochrome 10 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome 10 profile.

—	For the Multiview Monochrome 12 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome 12 profile.

—	For the Multiview Monochrome 16 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome 16 profile.

—	Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0 (i.e. the 0-th access unit).

—	Let the variable fR be set as follows:

—	If the bitstream or sub-layer representation is indicated to conform to the Main tier or is indicated to conform to a level that is lower than level 7.0, fR is set equal to 1 ÷ 300.

—	Otherwise, fR is set equal to 1 ÷ 960.

—	Let the variable olsIdx be the index of the OLS.

—	For each layer with nuh_layer_id equal to currLayerId, let the variable layerSizeInSamplesY be derived as follows:

layerSizeInSamplesY = pic_width_vps_in_luma_samples * pic_height_vps_in_luma_samples	(G-2)

where pic_width_vps_in_luma_samples and pic_height_vps_in_luma_samples are found in the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS.

When the specified level is not level 8.5, each layer with nuh_layer_id equal to currLayerId conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in F.13, where “access unit” is used to denote the picture unit in the layer and the CPB is understood to be the BPB:

a)	The value of layerSizeInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.8 for the tier and level of the layer.

b)	The value of pic_width_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

c)	The value of pic_height_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

d)	The value of max_vps_dec_pic_buffering_minus1[ olsIdx ][ LayerIdxInVps[ currLayerId ] ][ HighestTid ] shall be less than or equal to MaxDpbSize as derived by Formula (A-2), with PicSizeInSamplesY being replaced with layerSizeInSamplesY, for the tier and level of the layer.

e)	For level 5 and higher levels, the value of CtbSizeY for the layer shall be equal to 32 or 64.

f)	The value of NumPicTotalCurr for each picture in the layer shall be less than or equal to 8.

g)	When decoding each coded picture in the layer, the value of num_tile_columns_minus1 shall be less than MaxTileCols and num_tile_rows_minus1 shall be less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.8 for the tier and level of the layer.

h)	For the VCL HRD parameters of the layer, CpbSize[ i ] shall be less than or equal to CpbVclFactor * MaxCPB for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where CpbSize[ i ] is specified in F.13.1 and MaxCPB is specified in Table A.8 for the tier and level of the layer in units of CpbVclFactor bits.

i)	For the NAL HRD parameters of the layer, CpbSize[ i ] shall be less than or equal to CpbNalFactor * MaxCPB for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where CpbSize[ i ] is specified in F.13.1 and MaxCPB is specified in Table A.8 for the tier and level of the layer in units of CpbNalFactor bits.

j)	For each coded picture, the value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY ) ÷ 32.

Table A.8 specifies the limits for each level of each tier for levels other than level 8.5.

NOTE	Since there are no limits specified by Table A.8 for level 8.5, it is not possible in general for a practical decoder to be assured of being able to decode all bitstreams that conform to this level. The purpose of the definition of level 8.5 is to provide a suitable label for bitstreams that can exceed the limits of all other specified levels. When the bitstream is indicated to conform to level 8.5, a decoder is expected to examine the characteristics of the bitstream during its operation in order to determine whether it is capable of decoding the bitstream.

A tier and level to which a layer in an output operation point associated with an OLS in a bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc if OpTid of the output layer set is equal to vps_max_sub_layer_minus1, and by the syntax elements sub_layer_tier_flag[ OpTid ] and sub_layer_level_idc[ OpTid ] otherwise, as follows:

—	If the specified level is not level 8.5, general_tier_flag or sub_layer_tier_flag[ OpTid ] equal to 0 indicates conformance to the Main tier, and general_tier_flag or sub_layer_tier_flag[ OpTid ] equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A.8, and general_tier_flag and sub_layer_tier_flag[ OpTid ] shall be equal to 0 for levels below level 4 (corresponding to the entries in Table A.8 marked with "-"). Otherwise (the specified level is level 8.5), it is a requirement of bitstream conformance that general_tier_flag and sub_layer_tier_flag[ OpTid ] shall be equal to 1 and the value 0 for general_tier_flag and sub_layer_tier_flag[ OpTid ] is reserved for future use by ITU-T | ISO/IEC, and decoders shall ignore the value of general_tier_flag and sub_layer_tier_flag[ OpTid ].

—	general_level_idc and sub_layer_level_idc[ OpTid ] shall be set equal to a value of 30 times the level number specified in Table A.8.

      - Profile-specific tier and level limits for the Multiview Main, Multiview Extended, Multiview Extended 10, and multiview format range extensions profiles

The following is specified for expressing the constraints in this subclause:

—	The variable HbrFactor is set equal to 1.

—	The variable BrVclFactor is set equal to CpbVclFactor * HbrFactor.

—	The variable BrNalFactor is set equal to CpbNalFactor * HbrFactor.

—	The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor, where MinCrBase is specified in Table A.9.

When the specified level is not level 8.5, each layer conforming to the Multiview Main, Multiview Extended, or Multiview Extended 10 profile, or a multiview format range extensions profile at a specified tier and level shall obey the following constraints for each conformance test as specified in F.13, where "access unit" is used to denote the picture unit in the layer, and the CPB is understood to be the BPB:

a)	The nominal removal time of access unit n (with n greater than 0) from the CPB, as specified in F.13.2.3, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( layerSizeInSamplesY ÷ MaxLumaSr, fR ), where layerSizeInSamplesY is the value of layerSizeInSamplesY for access unit n − 1 and MaxLumaSr is the value specified in Table A.9 that applies to access unit n − 1 for the tier and level of the layer.

b)	The difference between consecutive output times of pictures in different access units, as specified in F.13.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( layerSizeInSamplesY ÷ MaxLumaSr, fR ), where layerSizeInSamplesY is the value of layerSizeInSamplesY of access unit n and MaxLumaSr is the value specified in Table A.9 for access unit n for the tier and level of the layer, provided that access unit n is an access unit that has a picture that is output and is not the last of such access units.

c)	The removal time of access unit 0 shall satisfy the constraint that the number of coded slice segments in access unit 0 is less than or equal to Min( Max( 1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSliceSegmentsPerPicture * layerSizeInSamplesY / MaxLumaPs ), MaxSliceSegmentsPerPicture ), for the value of layerSizeInSamplesY of access unit 0, where MaxSliceSegmentsPerPicture, MaxLumaPs, and MaxLumaSr are the values specified in Table A.8 and Table A.9 for the tier and level of the layer.

d)	The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slice segments in access unit n is less than or equal to Min( ( Max( 1, MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSliceSegmentsPerPicture ), where MaxSliceSegmentsPerPicture, MaxLumaPs, and MaxLumaSr are the values specified in Table A.8 and Table A.9 that apply to access unit n for the tier and level of the layer.

e)	For the VCL HRD parameters for the layer, BitRate[ i ] shall be less than or equal to BrVclFactor * MaxBR for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where BitRate[ i ] is specified in F.13.1 and MaxBR is specified in Table A.9 in units of BrVclFactor bits/s for the tier and level of the layer.

f)	For the NAL HRD parameters for the layer, BitRate[ i ] shall be less than or equal to BrNalFactor * MaxBR for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where BitRate[ i ] is specified in F.13.1 and MaxBR is specified in Table A.9 in units of BrNalFactor bits/s for the tier and level of the layer.

g)	The sum of the NumBytesInNalUnit variables for access unit 0 shall be less than or equal to FormatCapabilityFactor * ( Max( layerSizeInSamplesY, fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr for the value of layerSizeInSamplesY of access unit 0, where MaxLumaSr is specified in Table A.9, and both MaxLumaSr and FormatCapabilityFactor are the values that apply to access unit 0 for the tier and level of the layer.

h)	The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr is specified in Table A.9, and both MaxLumaSr and FormatCapabilityFactor are the values that apply to access unit n for the tier and level of the layer.

i)	The removal time of access unit 0 shall satisfy the constraint that the number of tiles in coded pictures in access unit 0 is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * PicSizeInSamplesY / MaxLumaPs ), MaxTileCols * MaxTileRows ), for the value of layerSizeInSamplesY of access unit 0, where MaxTileCols and MaxTileRows are the values specified in Table A.8 that apply to access unit 0 for the tier and level of the layer.

j)	The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in coded pictures in access unit n is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Table A.8 that apply to access unit n for the tier and level of the layer.

    - Decoder capabilities

When a decoder conforms to any profile specified in Annex G, it shall also have the INBLD capability specified in F.11.1.

F.11.2 specifies requirements for a decoder conforming to any profile specified in Annex G.