| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting: Kemer, TR, 1–8 November 2024 | Document: JVET-AJ2005-v2 |
| --- | --- |

| Title: | Additions and corrections for VVC version 4 (Draft 10) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Benjamin Bross Miska M. Hannuksela Ye-Kui Wang | Email: | gary.sullivan@dolby.com benjamin.bross@hhi.fraunhofer.de miska.hannuksela@nokia.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for changes to the Versatile Video Coding (VVC) standard (Rec. ITU-T H.266 | ISO/IEC 23090-3).

Changes to be integrated or checked:

- Elements from JVET-AG1004 (errata), and bug fixes from JVET-AH0002 for tickets #1609 (NoBackwardPredFlag derivation ambiguity), #1617 (Not initialized NumCtusInSlice[ 0 ] to 0), #1624 (Incorrect indexing in computation of motion vector offset), #1628 (Derivation of ModeTypeCondition should say “one or more”), #1630 (Missing equations for applying AmvrShift), #1631 (Should “Motion vector storing process for geometric partitioning mode” store HpelIfIdx? BBross: HpekIfIdx should always be written since it might be accessed later so it is definitely missing in "Motion vector storing process for geometric partitioning mode".), #1632 (Incorrect indexing used for choosing matrix intra sample prediction), #1634 (Matrices QStateTransTable,levelScale,AlfFixFiltCoeff,AlfClassToFiltMap are incorrectly transposed).

Changes that have been integrated for the 36th JVET meeting in November 2024:

- JVET-AJ0048 On SeiProcessingOrderSeiList and SpoProcessSeiList

- JVET-AJ0049 NNPFC and NNPFA interface text in VVC

- JVET-AJ0129 Miscellaneous comments on the SPO SEI message

- JVET-AJ0151 Move digitally signed content SEI messages to VSEI draft

- JVET-AJ0207 Move GFV and GFVE to VSEI draft

- JVET-AJ0114 On signalling input pictures shift in NNPFA SEI message (the aspect regarding the establishment of input picture lists for NNPF)

Changes that have been integrated for the 35th JVET meeting in July 2024:

- JVET-AI0146 Move the SPO and PON SEI messages from VVC to VSEI

- Added planned VSEI messages to D.2.1 syntax table with payloadType value assignments

- Added subclause for use of the source picture timing SEI message interface

Changes that have been integrated for the 34th JVET meeting in April 2024:

- (JVET-AH0121) Agreed to allow RWP as a process, and generalized cubemap and ERP as properties in SPO. A list of “property SEIs” should be generated.

- (JVET-AH0121) Item 3: Agreed to remove “or alternative processes that can be applied” from the spec. text.

- (JVET-AH0123) Decision: For now, we should specify that all processes and properties in a processing chain that follow an NNPF in an SPO chain are applied to all pictures produced by the NNPF.

- (JVET-AH0132) Editor action item: The editors are asked to check if the existing text could provide further clarification in the identified situation.

- (JVET-AH0159) Decision: Adopt Item 2 Option 2, with possible editorial improvement by the editors.

- (JVET-AH0352) Decision: Adopt JVET-AH0352v2 (resolves item #1 from JVET-AH0159)

Changes to the specification text:

# Replace subclause D.2.1 with the following:

D.2.1	General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| SeiExtensionBitsPresentFlag = 0 |  |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| else if( payloadType = = 4 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 19 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 45 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 56 ) /* Specified in ISO/IEC 23001-11 */ |  |
| green_metadata( payloadsize ) |  |
| else if( payloadType = = 129 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| parameter_sets_inclusion_indication( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 137 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| mastering_display_colour_volume( payloadSize ) |  |
| else if( payloadType = = 142 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| colour_transform_info( payloadSize ) |  |
| else if( payloadType = = 144 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| content_light_level_info( payloadSize ) |  |
| else if( payloadType = = 145 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| dependent_rap_indication( payloadSize ) |  |
| else if( payloadType = = 147 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| alternative_transfer_characteristics( payloadSize ) |  |
| else if( payloadType = = 148 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| ambient_viewing_environment( payloadSize ) |  |
| else if( payloadType = = 149 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| content_colour_volume( payloadSize ) |  |
| else if( payloadType = = 150 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| equirectangular_projection( payloadSize ) |  |
| else if( payloadType = = 153 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| generalized_cubemap_projection( payloadSize ) |  |
| else if( payloadType = = 154 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sphere_rotation( payloadSize ) |  |
| else if( payloadType = = 155 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| regionwise_packing( payloadSize ) |  |
| else if( payloadType = = 156 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| omni_viewport( payloadSize ) |  |
| else if( payloadType = = 165 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| alpha_channel_info( payloadSize ) |  |
| else if( payloadType = = 168 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_field_info( payloadSize ) |  |
| else if( payloadType = = 177 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| depth_representation_info( payloadSize ) |  |
| else if( payloadType = = 179 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| multiview_acquisition_info( payloadSize ) |  |
| else if( payloadType = = 180 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| multiview_view_position( payloadSize ) |  |
| else if( payloadType = = 200 ) |  |
| sei_manifest( payloadSize ) |  |
| else if( payloadType = = 201 ) |  |
| sei_prefix_indication( payloadSize ) |  |
| else if( payloadType = = 202 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| annotated_regions( payloadSize ) |  |
| else if( payloadType = = 203 ) |  |
| subpic_level_info( payloadSize ) |  |
| else if( payloadType = = 204 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sample_aspect_ratio_info( payloadSize ) |  |
| else if( payloadType = = 205 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| shutter_interval_info( payloadSize ) |  |
| else if( payloadType = = 206 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| extended_drap_indication( payloadSize ) |  |
| else if( payloadType = = 207 ) |  |
| constrained_rasl_encoding_indication( payloadSize ) |  |
| else if( payloadType = = 208 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| scalability_dimension_info( payloadSize ) |  |
| else if( payloadType = = 209 ) /* Specified in ISO/IEC 23090-13 */ |  |
| vdi_sei_envelope( payloadsize ) |  |
| else if( payloadType = = 210 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_characteristics( payloadSize ) |  |
| else if( payloadType = = 211 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_activation( payloadSize ) |  |
| else if( payloadType = = 212 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| phase_indication( payloadSize ) |  |
| else if( payloadType = = 213 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sei_processing_order( payloadSize ) |  |
| else if( payloadType = = 214 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| processing_order_nesting( payloadSize ) |  |
| else if( payloadType = = 215 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| encoder_optimization_info( payloadSize ) |  |
| else if( payloadType = = 216 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| source_picture_timing_info( payloadSize ) |  |
| else if( payloadType = = 217 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| object_mask_info( payloadSize ) |  |
| else if( payloadType = = 218 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| modality_info( payloadSize ) |  |
| else if( payloadType = = 219 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| text_description( payloadSize ) |  |
| else if( payloadType = = 220 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| digitally_signed_content_initialization( payloadSize ) |  |
| else if( payloadType = = 221 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| digitally_signed_content_selection( payloadSize ) |  |
| else if( payloadType = = 223 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| generative_face_video( payloadSize ) |  |
| else if( payloadType = = 224 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| generative_face_video_enhancement( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 132 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 214 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| processing_order_nesting( payloadSize ) |  |
| else if( payloadType = = 215 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| encoder_optimization_info( payloadSize ) |  |
| else if( payloadType = = 217 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| object_mask_info( payloadSize ) |  |
| else if( payloadType = = 222 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| digitally_signed_content_verification( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| if( SeiExtensionBitsPresentFlag ¦ ¦ more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| sei_reserved_payload_extension_data | u(v) |
| sei_payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| sei_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

# In subclause D.2.2, make the following changes:

...

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Buffering period | The remainder of the bitstream |
| Picture timing | The AU containing the SEI message |
| DU information | The AU containing the SEI message |
| Scalable nesting | Depending on the scalable-nested SEI messages. Each scalable-nested SEI message has the same persistence scope as if the SEI message was not scalable-nested |
| SEI manifest | The CVS containing the SEI message |
| SEI prefix indication | The CVS containing the SEI message |
| Subpicture level information | The CVS containing the SLI SEI message and up to but not including the next CVS, in decoding order, that contains an SLI SEI message with different content |
| Constrained RASL encoding indication | The CVS containing the SEI message |

...

The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 47, 129, 132, 137, 142, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 165, 168, 177, 179, 180, 200 to 202, inclusive, and 204 to 214, inclusive.

...

# Replace subclause D.12.2 with the following (where highlighted text indicates the changes):

### D.12.2 Use of the film grain characteristics SEI message

For purposes of interpretation of the film grain characteristics SEI message, the following variables are specified:

If the film grain characteristics SEI message is not applied as part of a processing chain indicated by an SEI processing order SEI message or is applied as the first processing step of the processing chain indicated by an SEI processing order SEI message, the following applies:

–	PicWidthInLumaSamples and PicHeightInLumaSamples are set equal to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, respectively.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

Otherwise (the film grain characteristics SEI message is applied as the second or a later processing step of the processing chain indicated by an SEI processing order SEI message), prevPic is a picture resulting from the previous processing step of the processing chain indicated by the SEI processing order SEI message, and the following applies:

–	PicWidthInLumaSamples and PicHeightInLumaSamples are set equal to the picture width and picture height of prevPic, respectively.

–	ChromaFormatIdc is set equal to the picture chroma format indicator of prevPic.

–	BitDepthY and BitDepthC are set equal to the luma bit depth and chroma bit depth of prevPic, respectively.

# Replace subclause D.12.11 with the following:

### D.12.11	Use of the neural network post-filter characteristics SEI message and the neural network post-filter activation SEI message

Let currPic be the cropped decoded output picture for which the neural-network post-processing filter (NNPF) defined by the neural-network post-filter characteristics (NNPFC) SEI message is activated by a neural-network post-filter activation (NNPFA) SEI message and currLayerId be the nuh_layer_id value of currPic.

It is a requirement of bitstream conformance that when a picture unit contains an NNPFA SEI message, the value of ph_pic_output_flag in the picture header contained in that picture unit shall be equal to 1.

NOTE – Since only cropped decoded output pictures are used as input pictures of the NNPF, the value of ph_pic_output_flag in the picture header of the coded picture corresponding to each input picture of the NNPF is equal to 1.

The variable pictureRateUpsamplingFlag is set equal to ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0.

The variable numInputPics is set equal to nnpfc_num_input_pics_minus1 + 1.

The variable numInferences is derived as follows:

–	If all of the following conditions are true, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0 and the variable numInferences is set equal to 1 + numPostRoll:

–	nnpfc_purpose is equal to 8 (i.e., the only purpose for the NNPF is picture rate upsampling).

–	nnpfa_persistence_flag is equal to 1.

–	nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0.

–	Either of the following conditions is true:

–	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

–	currPic is the last picture in the CLVS in output order and nnpfa_no_foll_clvs_flag is equal to 1.

–	Otherwise, if all of the following conditions are true, the variable numPostRoll is set equal to InpIdx[ i ] for the value of i such that nnpfa_output_flag[ i ] is equal to 1, and the variable numInferences is set equal to 1 + numPostRoll:

–	pictureRateUpsamplingFlag is equal to 0.

–	numInputPics is greater than 1.

–	nnpfa_persistence_flag is equal to 1.

–	nnpfa_output_flag[ idx ] is equal to 1 for a single value of idx in the range of 0 to NumInpPicsInOutputTensor − 1, inclusive, and for that single value of idx, InpIdx[ idx ] is greater than 0.

–	Either of the following conditions is true:

–	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

–	currPic is the last picture in the CLVS in output order and nnpfa_no_foll_clvs_flag is equal to 1.

–	Otherwise, the variable numInferences is set equal to 1.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The variable numCandInputPics, which indicates the number of candidate input pictures to the NNPF, is derived as follows:

numCandInputPics = numInputPics + nnpfa_num_input_pic_shift	(xx)

–	The arrays candInputPic[ i ] and candInputPresentFlag[ i ] for i in the range of 0 to numCandInputPics − 1, inclusive, representing all the candidate input pictures and the presence of candidate input pictures, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, candInputPic[ k ] is set to be currPic and candInputPresentFlag[ k ] is set equal to 0.

–	The j-th candidate input picture, candInputPic[ j ], is set to be currPic and candInputPresentFlag[ j ] is set equal to 1.

–	When numCandInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numCandInputPics − 1, inclusive, in increasing order of i:

–	If both of the following conditions are true, candInputPic[ i ] is set to be prevPic and candInputPresentFlag[ i ] is set equal to 1:

–	Either of the following conditions is true:

–	pictureRateUpsamplingFlag is equal to 1 and currPic is associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId, precede candInputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

–	pictureRateUpsamplingFlag is equal to 0 or currPic is not associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede candInputPic[ i − 1 ] in output order.

–	nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same CLVS.

–	Otherwise, the following applies:

–	candInputPic[ i ] is set to be the same picture as candInputPic[ i − 1 ] and candInputPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that, when pictureRateUpsamplingFlag is equal to 1, nnpfc_interpolated_pics[ i − 1 ] shall be equal to 0.

–	It is a requirement of bitstream conformance that when inputPresentFlag[ i ] is equal to 0 and nnpfc_input_pic_output_flag[ i ] is equal to 1, the value of nnpfa_output_flag[ idx ] shall be equal to 0 for the value of idx such that InpIdx[ idx ] is equal to i.

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures, respectively, are specified as follows:

for( i = 0, candIdx = nnpfa_num_input_pic_shift; i  <=  nnpfc_num_input_pics_minus1; i++, candIdx++ ) {
	inputPic[ i ] = candInputPic[ candIdx ]			(xx)
	inputPresentFlag[ i ] = candInputPresentFlag[ candIdx ]
}

–	For each value of i in the range 0 to numInputPics − 1, inclusive, it is a requirement of bitstream conformance that when inputPresentFlag[ i ] is equal to 0 and nnpfc_input_pic_output_flag[ i ] is equal to 1, the value of nnpfa_output_flag[ idx ] shall be equal to 0 for the value of idx such that InpIdx[ idx ] is equal to i.

–	For purposes of interpretation of the NNPFC SEI message, the following variables are specified:

–	CroppedWidth is set equal to the value of pps_pic_width_in_luma_samples − ‌SubWidthC * ‌ ( pps_conf_win_left_offset + pps_conf_win_right_offset ) for currPic.

–	CroppedHeight is set equal to the value of pps_pic_height_in_luma_samples − ‌SubHeightC * ‌ ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) for currPic.

–	The luma sample arrays CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are derived as follows for each value of i in the range of 0 to numInputPics − 1, inclusive:

–	The variable sourcePic is derived as follows:

–	If inputPresentFlag[ i ] is equal to 1 or nnpfc_absent_input_pic_zero_flag is equal to 0, sourcePic is set to be inputPic[ i ].

–	Otherwise(inputPresentFlag[ i ] is equal to 0 and nnpfc_absent_input_pic_zero_flag is equal to 1), sourcePic is set to be a picture with a luma sample array of CroppedWidth × CroppedHeight samples equal to 0 and Cb and Cr sample arrays of ( CroppedWidth / SubWidthC ) × ( CroppedHeight / SubHeightC ) samples equal to 0.

–

The luma sample array CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are set to be the 2-dimensional arrays of decoded sample values of the Y, Cb and Cr components, respectively, of sourcePic.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	The array StrengthControlVal[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, specifying the filtering strength control value for the input pictures for the NNPF, is derived as follows:

–	StrengthControlVal[ i ] is set equal to the value of ( firstSliceQpY + QpBdOffset ) ÷ ( 63 + QpBdOffset ), where firstSliceQpY is equal to SliceQpY of the first slice of inputPic[ i ].

There shall not be more than two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id. When there are two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id, these SEI messages shall have different content. When two NNPFC SEI messages with the same nnpfc_id and different content are present in the same picture unit, both of these NNPFC SEI messages shall be in the same SEI NAL unit.

# Add subclause D.12.13 as follows:

### D.12.13 Use of the SEI processing order and processing order nesting SEI messages

For purposes of interpretation of the SEI processing order and processing order nesting (PON) SEI messages, the following are specified:

–	The list SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 45, 47, 137, 142, 144, 147, 148, 149, 150, 153, 155, 165, 177, 202, 204, 210, 211, 217, and 218.

–	The list SpoProcessSeiList is set to consist of the payloadType values 19, 47, 142, 155, and 211.

–	The syntax structrure sei_pon_nested_message( ), the container of SEI messages, is set to be identical to the syntax structure sei_message( ).

The SEI messages contained in a PON SEI message are referred to as PON-nested SEI messages. When a generalized cubemap project SEI message is present with gcmp_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated generalized cubemap project SEI message in the same CLVS that is a PON-nested SEI message.

# Add subclause D.12.14 as follows:

### D.12.14 Use of the source picture timing SEI message

For purposes of interpretation of the source picture timing SEI message, the following variable is specified:

–	TemporalId is set equal to TemporalId.

# Add subclause D.12.15 as follows:

### D.12.15 Use of the digitally signed content initialization, digitally signed content selection, and digitally signed content verification SEI messages

For purposes of interpretation of the digitally signed content initialization, digitally signed content selection, and digitally signed content verification SEI messages, the list nonVclDigitallySignedNalUnitsList is set to consist of the NAL units types with nal_unit_type values 15, 16, 17, 18, and 19.