| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting: Kemer, TR, 1–8 November 2024 | Document: JVET-AJ2024-v2 |
| --- | --- |

| Title: | Exploration Experiment on Enhanced Compression beyond VVC capability (EE2) |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | EE description |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Roman Chernyak Karam Naser Jacob Ström Fan Wang Martin Winken Xiaoyu Xiu Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com chernyak@tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com wangfan6@oppo.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document describes Exploration Experiments (EEs) planned to be performed between 36th and 37th JVET meetings to evaluate enhanced compression tools beyond VVC capability.

# Introduction

EE focus is to evaluate individual coding technologies and their combinations. Contributions improving compression efficiency further is highly encouraged.

EE related discussions shall happen on JVET reflector.

EE tests should be implemented on top the ECM software, ECM-15.0 is used as an anchor in the tests.

Tests shall be performed according to the CTC described in JVET-AI2017.

TGM class tests is required for SCC tests and is optional otherwise.

Palette mode shall be enabled for classes F and TGM in both ECM anchor and EE tests.

For RPR tests, in addition to ECM CTC the tests are performed following JVET-Q2015, where only LB configuration is mandatory, and the sequences length is reduced to 5 seconds for all classes.

AI and RA test configurations are required for intra tool testing, while RA and LB test configurations are required for inter tool testing. LP configuration is optional. In LB and LP configurations, the sequences length is reduced to 5 seconds for all classes.

If encoder modification is included in EE tests, such encoder optimization, if applicable, introduced to the anchor should be tested.

# Timeline

T1 = 3 weeks (November 29, 2024) after JVET meeting: ECM is released

T2 = T1 + 1 week (December 6, 2024): EE description is finalized

T3 = T2 + 2 weeks (December 20, 2024): Initial software release for EE tests

T4 = JVET meeting start – 3 weeks (December 24, 2024): Software in EE branches is frozen

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1a | Extended OBMC for IBC/IntraTMP coded blocks | Y. Kidani (KDDI) X. Li (Alibaba) |  |
| 1.1b | Test 1.1a + extended OBMC for CCP coded blocks | X. Li (Alibaba) |  |
| 1.2a | Improvement on non-MPM | G. Wang (vivo) |  |
| 1.2b | Most dominant intra prediction mode | M. Hong (LGE) |  |
| 1.2c | Test 1.2b + excluding improbable intra modes | M. Hong (LGE) |  |
| 1.2d | Test 1.2a + Test 1.2c | G. Wang (vivo), M. Hong (LGE) |  |
| 1.3 | IntraTMP merge candidates enrichment | M. Radosavljević (InterDigital) |  |
| 1.4 | IntraTMP signalling improvement | F. Le Léannec (InterDigital) |  |
| 1.5 | Test 1.3 + Test 1.4 | F. Le Léannec (InterDigital) |  |
| 1.6 | Extended IntraTMP merge candidate list with an improved fusion mode | D. Ruiz Coll (Ofinno) |  |
| 1.7 | Block vector guided DIMD | L. Zhang (OPPO) |  |
| 1.8 | Non-adjacent DIMD fusion modes for TMRL intra mode candidate list construction | V. Rufitskiy, A. Filippov (TCL) |  |
| 1.9 | Weighted OBIC | P. Andrivon (Ofinno) |  |
| 1.10a | Bilateral filtering for intra prediction | W. Yin (Bytedance) |  |
| 1.10b | Test 1.10a + 6-tap cubic interpolation filter for TIMD | W. Yin (Bytedance) |  |
| 1.11 | 8-tap interpolation filter for angular intra prediction | T. Dong (TCL) |  |
| 1.12 | On MPM with matrix-based position dependent intra prediction | Z. Sun (OPPO) |  |
| 1.13 | Multiple filter taps for EIP | C. Zhou (vivo) |  |
| 1.14 | Block vector guided EIP | Z. Xie (OPPO) |  |
| 1.15 | Test 1.13 + Test 1.14 | C. Zhou (vivo) Z. Xie (OPPO) |  |
| 1.16a | Adaptive intra filter for template prediction | Z. Zhang (Alibaba) C. Zhou (vivo) |  |
| 1.16b | Template-based filter selection for intra prediction | Z. Zhang (Alibaba) C. Zhou (vivo) |  |
| 1.16c | Test 1.16a+Test 1.16b | Z. Zhang (Alibaba) C. Zhou (vivo) |  |
| 2 Inter prediction |  |  |  |
| 2.1a | OBMC modification for GPM blocks | R. Yu (Qualcomm) |  |
| 2.1b | OBMC operation order for block and subblock boundaries | R. Yu (Qualcomm) |  |
| 2.1c | Test 2.1a + Test 2.1b | R. Yu (Qualcomm) |  |
| 2.2a | Additional temporal affine candidates | L. Zhao (Bytedance) |  |
| 2.2b | Additional synthetic affine candidates | L. Zhao (Bytedance) |  |
| 2.2c | Test 2.2a + Test 2.2b | L. Zhao (Bytedance) |  |
| 2.3 | Regression-based GPM with intra and inter prediction | K. Jia (Alibaba) |  |
| 2.4 | GPM extension | H. Zhang (OPPO) |  |
| 2.5 | TMVP candidate selection | S. Hong (Nokia) |  |
| 2.6 | Template matching padding | N. Neumann (RWTH) |  |
| 3 Transform and coefficients coding |  |  |  |
| 3.1 | Multiple kernel set selection for intra LFNST/NSPT with subsampled DIMD for transform set selection | M. Coban (Qualcomm), F. Wang (OPPO), C. Bonnineau (Interdigital), L. Zhao (Bytedance) |  |
| 3.2 | LFNST/NSPT set derivation for CCP coded block | H. Huang (OPPO) |  |
| 3.3a | Advanced SBT | G. Laroche (Canon) |  |
| 3.3b | Test 3.1a with position granularity set to 4 instead to 1 | G. Laroche (Canon) |  |
| 3.3c | Test 3.1a not applied after SBT | G. Laroche (Canon) |  |
| 3.3d | Test 3.1a with only the position is inferred | G. Laroche (Canon) |  |
| 3.4 | Last significant coefficient position signalling with secondary prefix | F. Le Léannec (InterDigital) |  |
| 3.5a | Coefficient significant and gtX flags coding with probability adaptation | M. Balcilar (InterDigital) |  |
| 3.5b | Coefficient significant and gtX flags coding with increased CABAC engine’s range resolution | M. Balcilar (InterDigital) |  |
| 3.6a | Test 3.4 + Test 3.5a | F. Le Léannec (InterDigital) |  |
| 3.6b | Test 3.4 + Test 3.5b | F. Le Léannec (InterDigital) |  |
| 3.7 | Improved LFNST/NSPT kernel set selection for SGPM | S. Puri (InterDigital) |  |
| 3.8a | Implicit MTS extension | M. Karczewicz (Qualcomm) |  |
| 3.8b | Test 3.8a reusing existing LUT | M. Karczewicz (Qualcomm) |  |
| 4 In-loop filtering |  |  |  |
| 4.1 | Restrictions on ALF coefficient values | V. Shchukin (Ericsson), P. Bordes (InterDigital) |  |
| 4.2 | Using Laplacian information in ALF | I. Jumakulyyev (Nokia) |  |
| 4.3a | Boundary-aware offset refinement for ALF | W. Yin (Bytedance) |  |
| 4.3b | Test 4.3a + boundary-aware offset refinement for DBF | W. Yin (Bytedance) |  |
| 4.4 | Improvement to ALF scaling factors | N. Hu (Qualcomm) |  |
| 4.5a | ALF joint optimization (encoder only) | H. Wang (Qualcomm) |  |
| 4.5b | Encoder only ALF joint optimization tested in VTM | H. Wang (Qualcomm) |  |
| 4.5c | ALF joint optimization with reuse of ALF control information | H. Wang (Qualcomm) |  |
| 4.6a | Huffman coding for CCALF + simulated annealing for CCALF | K.Takada (Sharp) |  |
| 4.6b | Test 4.1 + Test 4.6a | K. Takada (Sharp), V. Shchukin (Ericsson) |  |
| 4.7 | Temporal ALF | L. Xu (OPPO) |  |
| 4.8a | NN-based ILF with ALF | D. Rusanovsky (Qualcomm) |  |
| 4.8b | NN-based ILF in parallel to the deblocking filter | D. Rusanovsky (Qualcomm) |  |
| 4.9a | ALF-CCCM | P. Astola (Nokia) |  |
| 4.9b | ALF-CCCM without the use of reference pictures | P. Astola (Nokia) |  |

# Tools description

## 1. Intra prediction

## Test 1.1: Extended OBMC for non-inter blocks (JVET-AJ0078, JVET-AJ0113)

This test studies extended OBMC methods for non-inter blocks. Test 1.1a extends OBMC to IBC/IntraTMP coded blocks. The extended OBMC method is applied to sub-blocks on top and left boundaries of the current block. A predictor is generated using the neighbouring block information and blended with the prediction samples of the current block. If a neighbouring block is intra mode (neither IBC nor IntraTMP) coded, an intra prediction mode derived by applying DIMD-liked method on the adjacent reconstructed samples of the current block is used to generate the predictor. Test 1.1b additionally extends OBMC to CCP coded blocks.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.1a | Extended OBMC for IBC/IntraTMP coded blocks | Y. Kidani (KDDI) X. Li (Alibaba) |
| 1.1b | Test 1.1a + extended OBMC for CCP coded blocks | X. Li (Alibaba) |

## Test 1.2: Improvement on non-MPM and HoG based intra mode coding (JVET-AJ0169, JVET-AJ0102)

In this test, an intra mode coding modification utilizing a histogram of gradients (HoG) extracted from neighbouring templates is investigated. To further enhance the non-MPM mode coding, modified construction method for non-MPM list with the pre-determined way considering PDP mode condition and improbable intra mode removal based on HoG are also studied.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.2a | Improvement on non-MPM | G. Wang (vivo) |
| 1.2b | Most dominant intra prediction mode | M. Hong (LGE) |
| 1.2c | Test 1.2b + excluding improbable intra modes | M. Hong (LGE) |
| 1.2d | Test 1.2a + Test 1.2c | G. Wang (vivo), M. Hong (LGE) |

## Test 1.3: IntraTMP merge candidates enrichment (JVET-AJ0299)

The IntraTMP merge mechanism is enriched with the use of new merge BV candidates. New merge BV candidates include block vector candidates from HMVP list and block vector candidates fetched from PUs that are previously not tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.3 | IntraTMP merge candidates enrichment | M. Radosavljević (InterDigital) |

## Test 1.4: IntraTMP signalling improvement (JVET-AJ0299)

Extended context modelling is used to signal IntraTMP LIC flag. The context used to signal LIC flag is chosen among several ones, according to some previously signalled information, like the IntraTMP fusion flag.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.4 | IntraTMP signalling improvement | F. Le Léannec (InterDigital) |

## Test 1.5: Combination of tests 1.3 and 1.4

Combines two tests.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.5 | Test 1.3 + Test 1.4 | F. Le Léannec (InterDigital) |

Test 1.6: Extended IntraTMP merge candidate list with an improved fusion mode (JVET-AJ0194)

Extending the IntraTMP merge list and the ARBVP list by including up to five fusion candidates based on the retrieved weighting parameters and adding up to three non-fusion BVs with lower template costs. In addition, the fusion model for IntraTMP groups 1 and 2 is derived on the decoder side according to the template cost. According to the expert's suggestions, the encoder and decoder complexity reduction mechanism will also be studied.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.6 | Extended IntraTMP merge candidate list with an improved fusion mode | D. Ruiz Coll (Ofinno) |

## Test 1.7: Block vector guided DIMD (JVET-AJ0087)

In this test, block-vector guided DIMD method is tested. This method uses block vectors to locate reference areas, construct a histogram of gradients for the samples in the reference areas, and derive intra modes based on the histogram. The prediction of the current block is obtained by using these intra modes

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.6 | Block vector guided DIMD | L. Zhang (OPPO) |

## Test 1.8: Non-adjacent DIMD fusion modes for TMRL intra mode candidate list construction (JVET-AJ0229)

In the test, DIMD processing is reused to construct a TMRL candidate list. For this purpose, DIMD is applied to non-adjacent reference lines(i.e. HoG is calculated in the area determined by a reference line) to extend the TMRL candidate list. These additional candidates can include both single modes and sets of modes which are combined when generating a block prediction (see Figure 1).

Figure 1. TMRL candidate list comprising non-adjacent DIMD fusion candidates

DC mode might be replaced with other modes in the TMRL candidate list. In addition, a similarity check can be performed for a pair of candidate predictors to discard similar TMRL candidates.

| # | Test | Tester |
| --- | --- | --- |
| 1.7 | Non-adjacent DIMD fusion modes for TMRL intra mode candidate list construction | V. Rufitskiy, A. Filippov (TCL) |

## Test 1.9: Weighted OBIC (JVET-AJ0171)

This test proposes to study the weighting of the occurrence value of neighbouring blocks coded with multiple IPMs (e.g. DIMD, TIMD...) in the construction of OBIC HoC. Weights and location dependency states are inherited from information used in the blending of the neighbouring predictors. A distance scaling factor is also considered.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.9 | Weighted OBIC | P. Andrivon (Ofinno) |

## Test 1.10: Bilateral filtering for intra prediction (JVET-AJ0223)

In test 1.10a, the generated intra prediction samples are further filtered by Bilateral Filtering (BF) to reduce the noise level of the prediction samples. The BF used in this test is kept the same as the BF used in the loop-filtering stage. The available above and left neighbouring reconstructed samples are used in BF filtering for the current block. The order of BF and PDPC is aligned in planar and angular mode. On top of test 1.10a, the 6-tap cubic interpolation filter is always selected for the blocks with TIMD mode in test 1.10b.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.10a | Bilateral filtering for intra prediction | W. Yin (Bytedance) |
| 1.10b | Test 1.10a + 6-tap cubic interpolation filter for TIMD | W. Yin (Bytedance) |

## Test 1.11: 8-tap interpolation filter for angular intra prediction (JVET-AJ0156)

In this test, a new 8-taps interpolation filter is introduced and an adaptive interpolation filter selection is implemented on the top of existing 6-taps interpolation filter in intra prediction angular mode. And this 8-taps interpolation filter is used if the following conditions are met:

Condition 1 determines whether the current prediction angle has sufficient angular distance from horizontal or vertical modes. The threshold to determine that is dependent on the block size. The threshold is different for normal (66 modes) and high (131 modes) angular mode precision.

Condition 2 determines whether a current block is smaller than a given threshold. The block size threshold is different for normal (66 modes) and high (131 modes) angular mode precision.

Otherwise, the existing 6-taps interpolation filter is in use.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.11a | 8-tap interpolation filter for angular intra prediction | T. Dong (TCL) |
| 1.11b | 8-tap interpolation filter for angular intra prediction with reduced precision of filter coefficients |  |

## Test 1.12: On MPM with matrix-based position dependent intra prediction (JVET-AJ0088)

This test studies the construction of the most probable mode (MPM) list relating to matrix-based position-dependent intra-prediction (MPDIP). More specifically, some non-MPDIP modes in the primary MPM may be converted to the MPDIP modes, and the MPDIP prediction instead of the conventional intra prediction will be used to calculate the template matching cost for primary MPM sorting.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.12a | Converting non-MPDIP to MPDIP in the primary MPM | Z. Sun (OPPO) |
| 1.12b | Using MPDIP prediction to calculate the template matching for primary MPM sorting | Z. Sun (OPPO) |
| 1.12c | Test 1.12a + Test 1.12b | Z. Sun (OPPO) |

## Test 1.13: Multiple filter taps for EIP (JVET-AJ0218)

In ECM, the EIP mode uses a 15-tap filter for prediction.

In this test, two filter taps are supported: 15-tap and 9-tap. An index is signalled to indicate which filter tap is used.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.13 | Multiple filter taps for EIP | C. zhou (vivo) |

## Test 1.14: Block vector guided EIP

The block vector guided EIP method uses a block vector to determine the reference area instead of directly using the adjacent spatial reference area for calculating the EIP filter parameters. Then the current coding unit utilizes the calculated EIP filter parameters and neighbouring reconstructed samples to form the prediction.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.14 | Block vector guided EIP | Z. Xie (OPPO) |

## Test 1.15: combination of Test1.13 and Test1.14

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.15 | Test1.13 + Test1.14 | C. Zhou (vivo) Z. Xie (OPPO) |

## Test 1.16: On intra interpolation filter (JVET-AJ0216, JVET-AJ0116 and JVET-AJ0217)

There are multiple filters used in ECM-14.0 for intra prediction, including [1 2 1]/4 reference sample filter, 4-tap/6-tap smooth interpolation filter and 6-tap DCT-based interpolation filter. In this test, the selection of the intra filters is improved. Three tests are conducted.

In Test 1.16a, instead of using a fixed intra filter, an adaptive selection method of the intra filters is used to determine the intra filter for intra prediction of the template area. (e.g., in the TIMD, MPM reordering, SGPM and etc.).

In Test 1.16b, the template-based intra filter selection method is used. That is, based on the TM cost, it is decided which intra filter to be used in the intra prediction of the current block.

Test 1.16c is a combined test of Test 1.16a and Test 1.16b.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.16a | Adaptive intra filter for template prediction | Z. Zhang (Alibaba) C. Zzhou (vivo) |
| 1.16b | Template-based filter selection for intra prediction | Z. Zhang (Alibaba) C. Zhou (vivo) |
| 1.16c | Test 1.16a + Test 1.16b | Z. Zhang (Alibaba) C. Zhou (vivo) |

## 2.Inter prediction

## Test 2.1: OBMC modifications (JVET-AJ0238)

Test 2.1a is about OBMC modifications for GPM blocks. It is proposed to apply OBMC separately on top of each GPM partition and then apply blending on top of OBMC modified samples of the two partitions. If the corresponding partition contains subblock motion, OBMC is applied on those subblock boundaries as well.

Test 2.1b is about operation order of applying OBMC on block and subblock boundaries. It is proposed to first apply OBMC on subblock boundaries (if they are present within the CU), and then apply OBMC on block boundaries.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.1a | OBMC modification for GPM blocks | R. Yu (Qualcomm) |
| 2.1b | OBMC operation order for block and subblock boundaries | R. Yu (Qualcomm) |
| 2.1c | Test 2.1a + Test 2.1b | R. Yu (Qualcomm) |

## Test 2.2: Enhanced derivation of affine merge candidates (JVET-AJ0134)

In Test 2.2a, additional affine temporal candidates can be derived by scaling the collocated CPMVs to the reference picture with index 0, and the collocated CPMVs referring to both lists can be utilized to obtain the temporal affine candidates.

In Test 2.2b, additional synthetic affine candidates are introduced by combining the motion of two existing candidates.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.2a | Additional temporal affine candidates | L. Zhao (Bytedance) |
| 2.2b | Additional synthetic affine candidates | L. Zhao (Bytedance) |
| 2.2c | Test 2.2a + Test 2.2b | L. Zhao (Bytedance) |

## Test 2.3: Regression-based GPM with intra and inter prediction (JVET-AJ0117)

In this test, the regression-based GPM is extended by adding a new mode in which the partitioning is derived as the current regression-based GPM but two partitions are respectively use intra prediction and inter prediction. A CU level flag is introduced to indicate this new regression-based GPM mode and if the flag is true, the intra prediction mode for the intra predicted partition and the motion information of the inter predicted partition are signalled.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.3 | Regression-based GPM with intra and inter prediction | K. Jia (Alibaba) |

## Test 2.4: GPM extension (JVET-AJ0089)

In ECM, the CU size restriction of Geometric Partition Mode (GPM) is currently specified as “cbWidth >= 8 && cbHeight >= 8 && cbWidth <= 64 && cbHeight <= 64 && cbHeight < 8 * cbWidth && cbWidth < 8 * cbHeight”.

The test extends the usage of GPM. The block size restriction is modified to allow more CUs applicable to GPM. The amount of usage of the different additional block sizes will be reported in the test.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.4 | GPM extension | H. Zhang (OPPO) |

## Test 2.5: TMVP candidate selection(JVET-AJ0261)

This test studies TMVP candidate selection. In the current ECM design, TMVP candidates are selected from either positions C0 or C1 of pairs. This test checks both positions C0 and C1, and includes motion information from both positions in the candidate list, provided there are still available slots for TMVP candidates.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.5 | TMVP Candidate Selection | S. Hong (Nokia) |

## Test 2.6: Template matching padding (JVET-AJ0043)

This test introduces template matching padding for reference picture boundary handling. Template matching structure prediction is used to generate the outside samples of the reference picture from content inside the reference picture. For this purpose, the outside area is partitioned into so-called target blocks. Then a template matching search is performed for each target block and the search results are used to fill the target block.

The test uses TM-padding and thereby replaces motion compensated padding (JVET-AA0096) for low delay reference pictures. For random access reference pictures, the padding method of ECM remains unchanged.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.6 | Template matching padding | N. Neumann (RWTH) |

## 3. Transform and coefficients coding

## Test 3.1 Multiple transform set selection for intra LFNST/NSPT with subsampled DIMD for transform set selection (JVET-AJ0255)

In this test, the multiple transform set selection method for intra blocks coded with LFNST/NSPT is investigated. In the method, CUs coded with DIMD/OBIC/TIMD/SGPM/MIP/EIP/ITMP modes can choose an alternative LFNST/NSPT transform set. Subsampled DIMD is used for intra prediction mode derivation for transform set selection.  The transform set selection is signalled in the bitstream.

The highest possible gain as well as the compression vs. complexity trade-off are investigated.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.1a | Multiple kernel set selection for intra LFNST/NSPT with subsampled DIMD for transform set selection | M. Coban (Qualcomm), F. Wang (OPPO), C. Bonnineau (Interdigital), L. Zhao (Bytedance) |
| 3.1b | Test 3.1a with different types of transform sets | C. Bonnineau (InterDigital) |

## Test 3.2 LFNST/NSPT set derivation for CCP coded block (JVET-AJ0086)

In this test, a virtual intra prediction mode is derived by using DIMD based on prediction samples for a CCP coded block, and the LFNST/NSPT transform set is then determined according to the derived intra mode.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.2 | LFNST/NSPT set derivation for CCP coded block | H. Huang (OPPO) |

## Test 3.3 Advanced SBT with position and direction inference (JVET-AJ0164)

In these tests, the advanced SBT mode is applied at TU level. For this mode, the position and the direction of the transform sub blocks are determined based on the corresponding Inter predictor block. Alternatively, depending on the following tests, only the position is determined, or the number of possible positions is restricted, or the proposed mode is not applied after the regular SBT.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.1a | Advanced SBT | G. Laroche (Canon) |
| 3.1b | Test 3.1a with position granularity set to 4 instead to 1. | G. Laroche (Canon) |
| 3.1c | Test 3.1a not applied after SBT | G. Laroche (Canon) |
| 3.1d | Test 3.1a with only the position is inferred | G. Laroche (Canon) |

## Test 3.4 Last significant coefficient position signalling with secondary prefix (JVET-AJ0198)

In this test, last XY signalling of ECM is modified as shown by following table. It consists in the following. Prefix is signalled as in ECM-14.0. For some transform blocks (e.g. at least 32) from some prefix value (e.g. the maximum allowed value), a secondary prefix flag is signalled to constrain the suffix part signalling. As an example, it may indicate that suffix is equal to maximum allowed value, given the TB size and possible zeroed out transform coefficients.

If secondary prefix flag is signalled and is equal to false, then suffix is signalled as in ECM-14.0. Otherwise, it is coded conditionally to the information given by the secondary prefix.

Table 1: proposed signalling for last significant coefficient X/Y position

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.4 | Last significant coefficient position signalling with secondary prefix | F. Le Léannec (InterDigital) |

## Test 3.5 Coefficient significant and gtX flags coding with probability adaptation

In this test, when the 15-bit CABAC probability associated to significant flag or of these flags are very low (e.g. below 48), the true probability is assumed to be 0.

Conversely, when 15-bit predicted probability is higher than some certain threshold (e.g. 32720), the probability is assumed to be equal to 1.

In both above cases, the bin is omitted from the bit-stream and its most probable value is inferred at decoder side. The CABAC probability updating is applied as in ECM, including in the two above cases.

In an alternative method, the resolution of the range of the CABAC engine may be increased compared to ECM-14.0.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.5a | Coefficient significant and gtX flags coding with probability adaptation | M. Balcilar (InterDigital) |
| 3.5b | Coefficient significant and gtX flags coding with increased CABAC engine’s range resolution | M. Balcilar (InterDigital) |
| 3.6b | Test 3.4 + Test 3.5b | F. Le Léannec (InterDigital) |

## Test 3.6: Combination of tests 3.4 and 3.5

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.6a | Test 3.4 + Test 3.5a | F. Le Léannec (InterDigital) |
| 3.6b | Test 3.4 + Test 3.5b | F. Le Léannec (InterDigital) |

## Test 3.7: Combination of tests 3.4 and 3.5

In this test, DIMD process is applied to the prediction signal of the SGPM to compute a virtual intra prediction mode and to determine the transform kernel set.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.7 | Improved LFNST/NSPT kernel set selection for SGPM | S. Puri (InterDigital) |

## Test 3.8: Implicit MTS extension (JVET-AJ0259)

In this test, an improved implicit MTS scheme is applied by considering in addition to block shape, intra mode type and direction as in existing LUT, also the difference between two dominant directions of the predicted block. In an additional test, the existing LUT is reused if applicable.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.8a | Implicit MTS extension | M. Karczewicz (Qualcomm) |
| 3.8b | Test 3.8a reusing existing LUT | M. Karczewicz (Qualcomm) |

## 4. In-loop filtering

## Test 4.1: Restrictions on ALF coefficient values (JVET-AJ0140)

In this test, allowed values of ALF coefficients are restricted to the elements of set {m*2^b}, where m lies in {-1, 0, 1} or {-3, -2, -1, 0, 1, 2, 3}, and b is a non-negative integer. The coefficients are derived using a simulated annealing approach and signalled into APS using Huffman coding. The coefficients are supplemented by a scale factor signalled in APS. The scale factor may be updated in the slice header.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.1 | Restrictions on ALF coefficient values | V. Shchukin (Ericsson), P. Bordes (InterDigital) |

## Test 4.2: Using Laplacian information in ALF (JVET-AJ0174)

In this test, ALF online coefficients are extended by adding new source of information, Laplacian of post-SAO buffer for additional filter coefficients derivation.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.2 | Using Laplacian information in ALF | I. Jumakulyyev (Nokia) |

## Test 4.3: Boundary-aware offset refinement for in-loop-filters (JVET-AJ0224)

In this test, a method of boundary-aware offset refinement is proposed. The offset between the input and the output sample of one in-loop-filter is further refined by a non-linear function, depending on the boundary information. The partitioning information is utilized to determine the boundary level for each position. A flag is signalled in the slice header to achieve on/off control.

In Test 4.3a, the proposed is applied to ALF. On top of Test 4.3a, the proposed method is further applied to DBF in test 4.3b.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.3a | Boundary-aware offset refinement for ALF | W. Yin (Bytedance) |
| 4.3b | Test 4.3a + boundary-aware offset refinement for DBF | W. Yin (Bytedance) |

## Test 4.4: Improvement to ALF scaling factors (JVET-AJ0276)

ECM signals a scaling factor for each ALF class. A scaling factor is applied to ALF correction residue of a sample. In this test, for each ALF class, multiple scaling factors can be signalled. For each ALF class, a boundary-based sub-classifier further classifies the samples into sub-classes. A flag is signalled to indicate whether a separated scaling factor is signalled for each sub-class. In the test, the sub-classifier will be adjusted according to the adopted coding information based classification in JVET-AJ0188.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.4 | Improvement to ALF scaling factors | N. Hu (Qualcomm) |

## Test 4.5: Reuse of ALF control information (JVET-AJ0231)

ECM signals ALF control information, including on/off flags and filter usage information, at both picture and CTU levels. In this test, ALF control information is reused at both picture and CTU levels from the previously encoded picture, and if it is reused then the signalling of that information is skipped. At encoder, ALF filter may be derived for a group of pictures jointly.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.5a | ALF joint optimization (encoder only) | H. Wang (Qualcomm) |
| 4.5b | Encoder only ALF joint optimization tested in VTM | H. Wang (Qualcomm) |
| 4.5c | ALF joint optimization with reuse of ALF control information | H. Wang (Qualcomm) |

## Test 4.6: Huffman coding and annealing coefficient derivation for CCALF (JVET-AJ0349)

In this test, Huffman coding and the annealing coefficient derivation are applied to CCALF. The combination test with test 4.1 is planned to be conducted as well.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.6a | Huffman coding for CCALF + simulated annealing for CCALF | K.Takada (Sharp) |
| 4.6b | Test 4.1 + Test 4.6a | K.Takada (Sharp), V. Shchukin (Ericsson) |

## Test 4.7: Temporal ALF (JVET-AJ0090)

The test studies the temporal ALF with uni-directional and bi-directional filtering modes. The inputs for the filter are the reconstructed samples from the reference pictures. The coefficient signalling is moved to the APS in this test.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.7 | Temporal ALF | L.Xu (OPPO) |

## Test 4.8: Integration of NN-based ILF in ALF (JVET-AJ0210)

In Test 4.8a an extension of the ECM-ALF with NN-based ILF as described in JVET-AJ0210 is tested. Tested NN model is derived from NNVC VLOP (JVET-AH0051) and modified to achieve better performance/complexity trade-off.

In Test 4.8b the respective NNVC VLOP model to be tested in the place parallel to the deblocking filter following NNVC integration.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.8a | NN-based ILF with ALF | D. Rusanovsky (Qualcomm) |
| 4.8b | NN-based ILF in parallel to the deblocking filter | D. Rusanovsky (Qualcomm) |

## Test 4.9: ALF-CCCM (JVET-AJ0173)

In Test 4.9a an additional chroma in-loop filter (ALF-CCCM) is tested as described in JVET-AJ0173.

In Test 4.9b ALF-CCCM is tested without the use of reference pictures for inheriting the control information.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.9a | ALF-CCCM | P. Astola (Nokia) |
| 4.9b | ALF-CCCM without the use reference pictures | P. Astola (Nokia) |