| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ2026-v1 |
| --- | --- |

| Title: | Testing of video coding technology beyond conditions of exploration experiments |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Wien, Y. Ye V. Baroncini E. Alshina | Tel: Email: | wien@lfb.rwth-aachen.de yan.ye@alibaba-inc.com baroncini@gmx.com elena.alshina@gmail.com |
| Source: | Editors |  |  |

_____________________________

# Introduction

In an effort to perform testing of video coding technology beyond conditions of exploration experiments, JVET collects candidate test sequences for assessing the compression performance of video coding tools extending beyond VVC. This document presents a collection of test sequences and associated encoder configuration settings for this purpose.

# Test sequences

The following categories of test sequences are considered. Each category name includes a characterization of the content type, the suggested encoder configuration and an indication of the resolution.

- SDR RA UHD/4K: Representing the use case of distribution of standard dynamic range UHD/4K video content e.g. in a streaming scenario, using a random-access configuration. This category is represented in the JVET CTC and is used for evaluation of the ECM. Here, a new set of test sequences is considered, including natural scenes as well as computer generated content.

- SDR RA HD: Representing the use case of distribution of standard dynamic range HD video content e.g. in a streaming scenario, using a random-access configuration. This category is represented in the JVET CTC and is used for evaluation of the ECM. Here, a new set of test sequences is considered.

- SDR LB HD: Representing the use case of conversational and other low delay applications at HD resolution, correspondingly using a low-delay configuration. This category is represented in the JVET CTC by test sequences at 720p resolution and is used for evaluation of the ECM. Here, a new set of test sequences at 1080p resolution is considered.

- HDR RA 4K: Representing the use case of distribution of high dynamic range UHD/4K video content e.g. in a streaming scenario, using a random-access configuration. This category so far is not represented JVET CTC for ECM evaluation.

- HDR RA Cropped 8K: Representing the use case of distribution of high dynamic range 8K video content e.g. in a streaming scenario, using a random-access configuration. This category so far is not represented JVET CTC for ECM evaluation. In order to reduce the encoding workload for assessment of this category, cropped regions of 3840×2160 resolution are used.

- Gaming LB HD: Representing the use case of online gaming with a low-delay configuration. The sequences include candidates from the new JVET CTC for gaming applications. Those sequences were not used for ECM development.

- UGC RA: Representing the use case of user generated content in portrait format at 1080×1920 resolution using a random-access configuration. This category so far is not represented JVET CTC for ECM evaluation.

The test sequences under consideration are listed in the table below. Availability of the test sequences as well as further information is provided in the accompanying spreadsheet. The list may be subject to further refinements. Further potential candidate sequences may be added to the list of test sequences under consideration.

| Category | Test sequence |
| --- | --- |
| SDR RA UHD/4K | DrivingPOV3 |
|  | Marathon2 |
|  | MountainBay2 |
|  | NeptuneFountain3 |
|  | TallBuildings2 |
|  | CrowdRun |
|  | ToddlerFountain2 |
|  | TrafficFlow |
|  | BaoleiYard |
| SDR RA HD | BarScene |
|  | DrivingPOV |
|  | Meridian2 |
|  | Metro |
| SDR LB HD | Beatriz |
|  | OfficeWalkAtWall |
|  | OfficeWalkCeiling |
| HDR RA 4K | NeptuneFountain3, downsample to 4K (PQ) |
|  | SubwayTree, downsample to 4K (PQ) |
|  | Meridian HDR2 (Car) (PQ) |
|  | SparksDirtLot (PQ) |
|  | SparksWelding (PQ) |
|  | H3_AMS05_3840x2160_10bit_420_HLG |
|  | H3_AMS06_3840x2160_10bit_420_HLG (CanalWithTrees) |
|  | H3_AMS01_3840x2160_10bit_420_HLG (TramBridge) |
|  | RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 |
| HDR RA Cropped 8K | BodeMuseum |
|  | NeptuneFountain3 |
|  | SubwayTree |
|  | JVET-AJ0176: Shells |
|  | JVET-AJ0176: Chandelier |
|  | JVET-AJ0176: Flowers |
| Gaming LB HD | Darktree |
|  | Level1 |
|  | ARPG2 |
|  | GTAV |
|  | JianlingTemple |
|  | Dota2 |
|  | EuroTruckSimulator2 |
|  | Minecraft |
| UGC RA | Animal_6v9a_pheasant |
|  | Animal_8m90_cat |
|  | CoverSong_1b0c_outdoor |
|  | CoverSong_3499_bar |
|  | Dance_i8b9_lady |
|  | Dance_l37s_street |
|  | Hobby_1fc7_park |
|  | Lecture_3ce0_writing |
|  | MusicVideo_7f2e_guitar |
|  | Sports_76a2_iceball |
|  | Sports_aicl_basketball |
|  | Sports_vnri_scooter |
|  | TelevisionClip_3758_cafe |
|  | VerticalVideo_4b92_park |
|  | Vlog_5904_dogs |

# Encoding task

For the assessment of the compression performance, the test sequences are to be encoded using the VTM-23.5 and ECM-14.1 software versions. For both, VTM and ECM, the CTC RA and LDB configuration files for the corresponding resolutions and adapted to the listed test sequences are to be used. In order to allow for a wide range performance assessment, the following QP values are defined in the format of “lowest QP:QP step size:highest QP”:

- VTM

  - HD: QP 20:2:50

  - UHD: QP 26:2:50

- ECM

  - HD: QP 22:4:50

  - UHD: QP 28:4:52

The encoding task includes the following steps:

- Generation of encoded bitstreams for the test sequences under consideration and collection of objective rate distortion data.

- Pre-viewing of the reconstructed video sequences, determining candidate QPs for lowest and highest rate point in a viewing test

  - Goal: Define a rate range for each sequence where the quality range for the VTM is such that significantly better candidate coding schemes can be distinguished.

    - At the highest rate point, a quality close to transparency should not be reached by the VTM (and should also not be fully reached by the ECM)

    - In the lowest rate point, the quality of candidate rate points should be at “annoying / severely annoying”

It is planned to perform a first review and potential selection of candidate sequences during the AhG phase until the 38th JVET meeting.

## Volunteering organizations

The following table lists the volunteering organizations with their contact persons for performing the encoding task using the VTM and ECM software.

| Company | Email |
| --- | --- |
| Alibaba | ruling.lrl@alibaba-inc.com |
| Bytedance | zhipin.deng@bytedance.com |
| Dolby | Fangjun.Pu@dolby.com |
| Ericsson | kenneth.r.andersson@ericsson.com |
| Fraunhofer HHI | adam.wieckowski@hhi.fraunhofer.de |
| Google | xlxiangli@google.com |
| Huawei | elena.alshina@huawei.com |
| InterDigital | Edouard.Francois@InterDigital.com |
| KDDI | yo-kidani@kddi.com |
| Kwai | xiaoyuxiu@kwai.com |
| Nokia | done.bugdayci_sansli@nokia.com |
| OPPO | wangfan6@oppo.com |
| Panasonic | hanboon.teo@sg.panasonic.com |
| Qualcomm | vseregin@qti.qualcomm.com |
| Tencent | chernyak@global.tencent.com |
| Transsion | yongkai.huo@transsion.com |
| vivo | zhuoyi.lv@vivo.com |

The assignment of encoding tasks is documented in the accompanying spreadsheet, with two companies assigned for each sequence for load-sharing and crosschecking. For this purpose for both, VTM and ECM, the target QP range is split in to two QP sets with an overlap for one QP value. The QP sets are listed in the table below.

| Resolution | QP set Name | Values |
| --- | --- | --- |
| HD | VTM QP set 1 | QP=20:4:48 |
| HD | VTM QP set 2 | QP=22:4:50, 36 |
| HD | ECM QP set 1 | QP=22:8:46 |
| HD | ECM QP set 2 | QP=26:8:50, 38 |
| UHD | VTM QP set 1 | QP=26:4:50 |
| UHD | VTM QP set 2 | QP=28:4:52, 38 |
| UHD | ECM QP set 1 | QP=28:8:52 |
| UHD | ECM QP set 2 | QP:32:8:48, 36 |