| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 43rd Meeting, Geneva, CH, 7–15 July 2026 | Document: JVET-AQ_notes_dF |
| --- | --- |

| Title: | Meeting Report of the 43rd Meeting of the Joint Video Experts Team (JVET), Geneva, CH, 7–15 July 2026 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contacts): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its forty-third meeting during 7 – 15 July 2026 at the ITU headquarters facilities in Geneva, Switzerland. The meeting was held as a physical meeting with remote participation, provided on best-effort basis for experts who were unable to travel.

For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the twenty-fourth meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.16 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and preparations towards future standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 0900 CEST (0700 UTC) on Tuesday 7 July 2026. Meeting sessions were held on all days including the weekend days of Saturday and Sunday 11 and 12 July 2026, until the meeting was closed at approximately 1400 hours CEST on Wednesday 15 July 2026. Approximately 486 people attended the JVET meeting (200 in person and 286 remotely), and approximately 166 input documents (not counting crosschecks, reports, and summary documents), 18 JVET AHG reports, 1 report of the Joint AHG on Gaussian splat coding, one EE summary report, 0 BoG report(s), and 2 incoming liaison document(s) were discussed. The meeting took place in a collocated fashion with a meeting of SG21 – one of the two parent bodies of the JVET, under whose auspices this JVET meeting was held. Various SC29 Working Groups and Advisory Groups were also meeting in Geneva with partial temporal overlap – where WG 5 is representing the Joint Video Experts Team and its activities from the perspective of the SC 29 parent body.  The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the Exploration Experiment (EE) on Neural Network-based Video Coding, of the joint EE on Gaussian splat coding, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development. In the latter context, the final development of the Joint Call for Proposals on video compression with capability beyond VVC and of related documents for preparation of submissions was a major activity at his meeting.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the forty-second JVET meeting in producing the following documents:

a) JVET documents

JVET-AP1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AP1005 Future CICP extensions (Draft 3)

JVET-AP1006 HEVC additional profiles (Draft 1)

JVET-AP1008 Conformance testing for HEVC multiview extended and monochrome profiles (Draft 2)

JVET-AP1016 AVC with extensions and corrections (Draft 6), also forwarded for consent as WP3/21 TD 278-WP3 ITU-T H.264 (V16) "Advanced Video Coding" (Rev.)

JVET-AP2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video

JVET-AP2016 Common test conditions and evaluation procedures for neural network-based video coding technology

JVET-AP2019 Description of algorithms version 15 and software version 17 in neural network-based video coding (NNVC)

JVET-AP2020 Film grain synthesis technology for video applications ed. 2 (Draft 5)

JVET-AP2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AP2025 Algorithm description of Enhanced Compression Model 20 (ECM 20)

JVET-AP2026 Final draft Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 400

JVET-AP2028 Common test conditions on Gaussian splat coding

JVET-AP2032 Technologies under consideration for future extensions of VSEI (version 12)

JVET-AP2033 Report of verification tests on VVC multi-layer coding

b) documents produced as WG 5 documents only:

WG 5 N 397 Disposition of comments received on ISO/IEC 14496-10:2025/DAM 1

WG 5 N 398 Disposition of comments received on DIS ISO/IEC 23090-15:202 Text of ISO/IEC FDIS 14496-10:202x Advanced video coding (12th ed.)

WG 5 N 399 Request for ISO/IEC TR 23002-9 Film grain synthesis technology for video applications (2nd ed.)

WG 5 N 401 Disposition of comments received on ISO/IEC DTR 23888-3.2

WG 5 N 403 Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

As main results, the JVET produced 13 output documents from the current meeting:

JVET-AQ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AQ1006 HEVC additional profiles (Draft 1), also submitted for ballot as WG 5 CDAM N 411

JVET-AQ1008 Conformance testing for HEVC multiview extended and monochrome profiles (Draft 3)

JVET-AQ1012 Overview of IT systems used in JVET

JVET-AQ1017 Updates of HEVC reference software (draft 1)

JVET-AQ2006 Additional SEI messages for VSEI version 5 (Draft 1)

JVET-AQ2019 Description of algorithms version 16 and software version 18 in neural network-based video coding (NNVC)

JVET-AQ2020 Film grain synthesis technology for video applications ed. 2 (Draft 6), also submitted for ballot as WG 5 CDTR N 409

JVET-AQ2021 Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 415 and SG21 TD348/Plen

JVET-AQ2022 Draft template for proposal description documents of responses to the Joint CfP on video compression with capability beyond VVC

JVET-AQ2028 Common test conditions on Gaussian splat coding, also issued as WG 5 N 420

JVET-AQ2032 Technologies under consideration for future extensions of VSEI (version 13)

JVET-AQ2040 Description of SEI-related exploration experiments on Gaussian splat coding (Excerpt from WG 5 N 430 and N 432)

- The following 17 documents were produced as WG 5 documents only, without a corresponding JVET output document or direct repetition of their content in this meeting report:

WG 5 N 410 Request for ISO/IEC 23008-2:202x/Amd.1 Additional HEVC profiles

WG 5 N 412 Request for ISO/IEC 23008-5:202x Reference software for HEVC (3rd edition)

WG 5 N 413 Text of ISO/IEC CD 23008-5:202x Reference software for HEVC (3rd edition)

WG 5 N 414 Request for ISO/IEC 23008-8:202x Conformance specification for HEVC (3rd edition)

WG 5 N 415 Text of ISO/IEC 23008-8:202x Conformance specification for HEVC (3rd edition)

WG 5 N 421 Draft CfP of Gaussian splat coding

WG 5 N 422 Call for content for Gaussian splat coding

WG 5 N 423 FAQ on Gaussian splat coding

WG 5 N 424 JEE 6.1 on data preparation

WG 5 N 425 JEE 6.2 on anchor generation

WG 5 N 426 JEE 6.3 on new coding technologies

WG 5 N 427 JEE 6.4 on collecting requirements and use cases for Gaussian splat coding

WG 5 N 428 JEE 6.5 on 3DGS software

WG 5 N 429 JEE 6.6 on coordinating the 1F-geo Track

WG 5 N 430 JEE 6.7 on video-based coding technologies

WG 5 N 431 JEE 6.8 on Gaussian Splat Coding evaluation and CTC

WG 5 N 432 JEE 6.9 on evaluation of encapsulation methods for Gaussian Splats

- The following draft ITU-T Supplement was forwarded by JVET and Q6/21 for ITU-T Agreement:

SG21 TD326/Plen H-Series Supplement 22 (ex. H.Sup-MACVC) Optimization of encoders and receiving systems for machine analysis of coded video content

- The following liaison communications were forwarded by JVET and Q6/21 for handling by ITU-T:

Liaison to JPEG on JPEG AI and explorations on video coding, included as item 1 in SG21 TD354/WP3

Liaison to 3GPP SA4 on HEVC multi-layer profiles, chroma subsampling and bit depth combinations, included as item 2 in SG21 TD354/WP3

For the organization and planning of its future work, the JVET established 17 “ad hoc groups” (AHGs) to progress the work on particular subject areas. Another Joint AHG on Gaussian splat coding was established together with WG 4 and WG 7. At this meeting, several Joint EEs on Gaussian splat coding were defined together with WG 4 and WG 7, where JEE 6.7 and JEE 6.9 are related to topis of using SEI messages for that purpose. The next ten JVET meetings were planned for 17 – 23 October 2026 under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN; during 13 – 22 January 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU; during 17 – 28 April 2027 under ITU-T SG21 auspices in Geneva, CH; during 7 – 16 July 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI; during 20 – 29 October 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN; during 13 – 22 January 2028 under ISO/IEC JTC 1/‌SC 29 auspices in Thessaloniki, GR; during 25 March – 5 April 2028 under ITU-T SG21 auspices in Geneva, CH; during 19 – 28 July 2028 under ISO/IEC JTC 1/‌SC 29 auspices in Fukuoka, JP; during October 2028 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; and during January 2029 under ITU-T SG21 auspices, date and location t.b.d.

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. The most recent versions of JCT-VC and JCT-3V documents can now be accessed directly via the JVET site, whereas all uploaded versions are also available from http://wftp3.itu.int/av-arch/jctvc-site/ and http://wftp3.itu.int/av-arch/jct3v-site/, respectively. JVET documents are also provided via http://wftp3.itu.int/av-arch/jvet-site/, but as this is currently not maintained as a mirror with the jvet-experts.org site, documents are typically only made available with a delay of two or three meeting cycles, to avoid that late uploads of output documents would be missing.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 21 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its forty-third meeting during 7 – 15 July 2026 at the ITU headquarters facilities in Geneva, Switzerland. The meeting was held as a physical meeting with remote participation, provided on best-effort basis for experts who were unable to travel.

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding

- Exploration on Enhanced Compression beyond VVC capability (EE discontinued by 42nd meeting)

- Joint exploration (with WG 4 and WG 7 of SC 29) on Gaussian splat coding

JVET had been tasked by the parent bodies to develop a call for proposals on video compression with capability beyond existing standards, with plan to issue a first draft call from the current meeting.

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants, consisting of two parts, (B1) in-person attendees as recorded by a sign-in sheet circulated in meeting rooms, (B2) remote attendees as recorded by the teleconferencing tool used for the meeting

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2026_07_AQ_Geneva/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the forty-second JVET meeting in producing the following output documents:

JVET-AP1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AP1005 Future CICP extensions (Draft 3)

JVET-AP1006 HEVC additional profiles (Draft 1)

JVET-AP1008 Conformance testing for HEVC multiview extended and monochrome profiles (Draft 2)

JVET-AP1016 AVC with extensions and corrections (Draft 6), also forwarded for consent as WP3/21 TD 278-WP3 ITU-T H.264 (V16) "Advanced Video Coding" (Rev.)

JVET-AP2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video

JVET-AP2016 Common test conditions and evaluation procedures for neural network-based video coding technology

JVET-AP2019 Description of algorithms version 15 and software version 17 in neural network-based video coding (NNVC)

JVET-AP2020 Film grain synthesis technology for video applications ed. 2 (Draft 5)

JVET-AP2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AP2025 Algorithm description of Enhanced Compression Model 20 (ECM 20)

JVET-AP2026 Final draft Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 400

JVET-AP2028 Common test conditions on Gaussian splat coding

JVET-AP2032 Technologies under consideration for future extensions of VSEI (version 12)

JVET-AP2033 Report of verification tests on VVC multi-layer coding

b) documents produced as WG 5 documents only:

WG 5 N 397 Disposition of comments received on ISO/IEC 14496-10:2025/DAM 1

WG 5 N 398 Disposition of comments received on DIS ISO/IEC 23090-15:202 Text of ISO/IEC FDIS 14496-10:202x Advanced video coding (12th ed.)

WG 5 N 399 Request for ISO/IEC TR 23002-9 Film grain synthesis technology for video applications (2nd ed.)

WG 5 N 401 Disposition of comments received on ISO/IEC DTR 23888-3.2

WG 5 N 403 Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

Further important goals were reviewing the results of the EE on Neural Network-based Video Coding, of the Joint EE on Gaussian Splat Coding, of other technical input on novel aspects of video coding technology, and preparing next steps for investigation of candidate technology towards further standard development by issuing the Joint Call for Proposals on video compression with capability beyond VVC.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the UTC time, except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Other parenthetical comments may be used for describing the impact or motivation of a decision. Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for work items under study, non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair, or other experts that were appointed to chair parallel sessions. It is indicated who took the notes in review of a given category (or individual documents în a category), and by which day and time the review was conducted. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 30 June 2026. Any documents uploaded after 1159 hours Paris/Geneva time on Wednesday 1 July 2026 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AQ0192 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meeting, or documents which were requested to be produced for the purpose of improving specification text, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AQ0172 (a proposal on VLOP4 Modification), uploaded 07-04,

- JVET-AQ0194 (a proposal on NextSoftware2 implementation), uploaded 07-01,

- JVET-AQ0196 (a proposal on Loss concealment purpose for NNPFC), uploaded 07-01,

- JVET-AQ0200 (a proposal on Training-Time Optimization for LOP7), uploaded 07-04,

- JVET-AQ0219 (a proposal on implementation of an GS SEI message), uploaded 07-06,

- JVET-AQ0221 (a proposal on an avatar representation SEI), uploaded 07-08,

- JVET-AQ0222 (a proposal on updates to the TR on film grain synthesis), uploaded 07-07,

- JVET-AQ0226 (a proposal on OMI and PSI SEI messages), uploaded 07-09,

- JVET-AQ0227 (a proposal on optimization of the GSC metric software), uploaded 07-09,

- JVET-AQ0228 (a proposal on DSC SEI message extensions), uploaded 07-10,

- JVET-AQ0229 (a proposal on PSI SEI message), uploaded 07-10,

- JVET-AQ0230 (a proposal on interface for VSEI SEI messages), uploaded 07-11,

- JVET-AQ0235 (a proposal on extensions of GSI SEI), uploaded 07-12,

- JVET-AQ0239 (a proposal on scalable syntax in GSI SEI), uploaded 07-13,

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AQ0043 (a contribution on placement of metadata), uploaded 07-08,

- JVET-AQ0156 (a contribution on a showcase environment), uploaded 07-10,

- JVET-AQ0180 (a contribution on showcase for IR camera adaptation SEI), uploaded 07-11,

- JVET-AQ0181 (a contribution on Showcase for ASAI SEI), uploaded 07-12,

- JVET-AQ0185 (a contribution on RDO selection count restriction), uploaded 07-05,

- JVET-AQ0190 (a contribution on a prototype VVC decoder), uploaded 07-09,

- JVET-AQ0191 (a contribution on a browser-based VVC player), uploaded 07-09,

- JVET-AQ0198 (a contribution on ECM Progress), uploaded 07-05,

- JVET-AQ0199 (a contribution on reference preservation for ULL), uploaded 07-06,

- JVET-AQ0204 (a contribution on average PSNR calculation), uploaded 07-04,

- JVET-AQ0211 (a contribution on Film grain technology support in HDRTools), uploaded 07-06,

- JVET-AQ0220 (a contribution on editorial changes to CfP), uploaded 07-06,

- JVET-AQ0238 (a contribution on software availability and CTC results in GSC), uploaded 07-13,

Almost all cross-verification reports at this meeting (except for JVET-AQ0134, JVET-AQ0137, JVET-AQ0178, and JVET-AQ0207) were registered late, and/or uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AQ0077, JVET-AQ0120, JVET-AQ0121, JVET-AQ0133, JVET-AQ0195, JVET-AQ0197, JVET-AQ0231, JVET-AQ0233, and JVET-AQ0267.

The following cross-verification report(s) were still missing by the end of the meeting, but were uploaded later: JVET-AQ0XXX, …. The following report(s) had not become available yet three weeks after the end of the meeting: JVET-AQ0XXX, … . These were marked as withdrawn by the JVET chair, assuming the registration had become obsolete (paragraph kept for future use).

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation did not apply to any contributions. (did apply to document(s) JVET-AQ0XXX, being initially empty of results and flagged as late in the list above, based on the time of the first reasonable document upload | sentence kept for future use).

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AP1000, the Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP JVET-AP1004, the Future CICP extensions (Draft 3) JVET-AP1005, the HEVC additional profiles (Draft 1) JVET-AP1006, the Conformance testing for HEVC multiview extended and monochrome profiles (Draft 2) JVET-AP1008, the AVC with extensions and corrections (draft 6) JVET-AP1016, the VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video JVET-AP2010, the Common test conditions and evaluation procedures for neural network-based video coding technology JVET-AP2016, the Description of algorithms version 15 and software version 17 in neural network-based video coding (NNVC) JVET-AP2019, the Film grain synthesis technology for video applications ed. 2 (Draft 5) JVET-AP2020, the Description of the EE on Neural Network-based Video Coding JVET-AP2023, the Algorithm description of Enhanced Compression Model 20 (ECM 20) JVET-AP2025, the Final draft Joint Call for Proposals on video compression with capability beyond VVC JVET-AP2026, the Common test conditions on Gaussian splat coding JVET-AP2028, the Technologies under consideration for future extensions of VSEI (version 12) JVET-AP2032, and the Report of verification tests on VVC multi-layer coding JVET-AP2033, had been completed and those which were available were approved. In a few cases, the corresponding WG 5 N-numbered documents had not yet been uploaded, and this was requested to be done as soon as possible (kept for future use). The software implementations of (VTM version 24.x, - retained for future use) ECM version 20.0, and NNVC version 17.0 and 17.1 were also approved.

It was further noted that some editorial modifications were necessary on the WG 5 versions of standard deliverables, to be compliant with the ISO/IEC rules.

Only minor editorial issues were found in the meeting report JVET-AP1000; no need to produce an update was identified (see section 2.14 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annexes B1 and B2 of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/21 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts, and through the Q6/21 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- According to rules of JVET’s parent bodies, any recording of meeting session (regardless if conducted in the meeting room or from the zoom screen) is not permitted. Participants who are identified to perform recording will be removed from the session unless they stop recording upon request.

- Usage of transcription tools in zoom sessions is strongly discouraged, as they often give erroneous results and lead to misunderstanding of verbal expressions by other experts.

- Generally, do not use video for the teleconferencing calls in order to avoid overloading internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which shared screen they want to see.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Approval of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities, and Joint AHG on Gaussian splat coding

Report of exploration experiment on neural-network-based video coding (EE1)

Report of joint EE on Gaussian splat coding (JEE 6.7 and 6.9)

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development

Consideration of contributions on software development in exploration activities

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology

Consideration of contributions on gaming content compression

Consideration of contributions on optimization of encoders and receiving systems for machine analysis of coded video content

Consideration of contributions on assessment of implementation complexity of video coding technology

Consideration of contributions on aspects of ultra-low latency and packet loss resilience in the context of video compression

Consideration of contributions and planning of next steps towards a future video coding standard, finalization of a Joint Call for Proposals, and definition of criteria for technology assessment

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions on technical reports relating to standards and exploration study activities in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The times of meeting sessions followed the needs of the face-to-face meeting, with highest priority given to the aim of achieving the goals of the meeting (see under section 2.15). Typical meeting hours were expected to be 0900-2000 CEST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were anticipated to be necessary. Sessions were announced in the online JVET calendar in advance as far as possible, but it was agreed that some activities (such as breakout sessions) could be held at short notice.

## ISO, IEC and ITU-T Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100011.html.

https://www.iec.ch/basecamp/iec-code-conduct-technical-work

These include points relating to:

- Behave ethically

- Promote and enable all voices to be heard

- Engage constructively in ISO and IEC activities

- Respect others (in meetings, when writing, on social media)

- Declare actual and potential conflicts of interest and manage them appropriately

- Protect confidential information

- Protect ISO and IEC assets

- Avoid and prevent any form of bribery or corruption

- Uphold the consensus process

- When disputes arise, escalate, resolve and uphold agreed resolution

Participants were also reminded of the UN Codes of Conduct, applying to work under ITU-T, found at https://www.un.org/management/sites/www.un.org.management/files/un-system-model-code-conduct.pdf

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11 of June/July 2009. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. The site contains links for creating a user account for document uploading and for contacting an administrator – e.g., to update the email address associated with an account, and experts were reminded to keep this information up to date. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

It was noted that also the most recent versions of JCT-VC and JCT-3V documents can be accessed directly via the JVET site. A mirror for JCT-VC, JCT-3V and JVET document access (including all versions with original upload dates) has also been provided via the ITU ftp site, providing a “documents” subfolder in the directory of each respective meeting.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1386 (as of 6 July 2026). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

It is further emphasized that the document JVET-AJ1012 (note that an update JVET-AQ1012 is an output from the current meeting) gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials. It was suggested to produce an updated version of this document, including refined guidance for the usage of Zoom functionality in alignment with ISO directives and policies as a mandate of an AHG.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – video coding standard developed by JVT, formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BIF: Bilateral filter

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardization

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted for study of coding technology

- CfE: Call for Evidence

- CfP: Call for Proposals

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CICP: Coding-independent code points (Rec. ITU-T H.273 | ISO/IEC 23091-2)

- CIIP: Combined inter/intra prediction

- CL-RAS: Cross-layer random-access skip

- CNN: Convolutional neural network

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DRF: Deep reference frame (for NNVC inter prediction)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EE: Exploration experiment – a coordinated experiment conducted for study of coding technology prior during exploration work prior to establishment of a formal project (EEn for some number n is exploration experiment number n)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase and test model algorithm for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EIP: Extrapolation based intra prediction

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FGRC: Film grain regions characteristics

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GFV: Generative face video

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- GS/GSC: Gaussian splatting / Gaussian splat coding

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a software codebase and video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HOP: High-complexity operating point for NNLF.

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC 3D video extensions)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (originally joint video exploration team, initially for VVC, later expanded in scope)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- kMAC: thousands of multiply-accumulate operations – used as a complexity metric

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LB-CCP: Local-boosting CCP

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LOP: Low-complexity operating point for NNLF.

- LP or LDP: Low delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MM-CCCM: Multi-model CCCM

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MOS: Mean opinion score (a metric for subjective quality evaluation)

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MSE: Mean-squared error, a distortion metric

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNLF: Neural network loop filter

- NNPF: Neural network post filter

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSPT: Non-separable primary transform

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display)

- ONNX: Open Neural Network Exchange – a format used to convert code from common neural network software packages into SADL code

- operation point: A temporal subset of an OLS

- PDPC: Position-dependent (intra) prediction combination

- PERP: Padded equirectangular projection (a 360° projection format)

- PH: Picture header

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format)

- PMMVD: Pattern-matched motion vector derivation

- POC: Picture order count

- PoR: Plan of record

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set

- PSNR: Peak signal-to-noise ratio, a fidelity metric derived from MSE

- PTL: Profile/tier/level combination

- QM: Quantization matrix

- QP: Quantization parameter (sometimes conflated with quantization step size)

- QT: Quadtree

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD)

- RADL: Random-access decodable leading (type of picture)

- RASL: Random-access skipped leading (type of picture)

- R-D: Rate-distortion

- RDO: Rate-distortion optimization

- RDOQ: Rate-distortion optimized quantization

- RDPCM: Residual DPCM

- ROI: Region of interest

- ROT: Rotation operation for low-frequency transform coefficients

- RPL: Reference picture list

- RPLM: Reference picture list modification

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC

- RPS: Reference picture set

- RQT: Residual quadtree

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q)

- RVM: Rate variation measure

- SAD: Sum of abolute differences, a measurement of distortion

- SADL: Small adhoc deep learning librar

- SAO: Sample-adaptive offset

- SBT: Subblock transform

- SbTMVP: Subblock based temporal motion vector prediction

- SCIPU: Smallest chroma intra prediction unit

- SCM: Screen content model – a software codebase and test model algorithm for the HEVC screen content coding extensions

- SD: Slice data; alternatively, standard-definition

- SDH: Sign data hiding

- SDT: Signal-dependent transform

- SE: Syntax element

- SEI: Supplemental enhancement information

- SGPM: Spatial geometric partition mode

- SH: Slice header

- SHM: Scalable HM – a software codebase and test model algorithm for SHVC

- SHVC: Scalable high efficiency video coding

- SIF: Switchable (motion) interpolation filter

- SIMD: Single instruction, multiple data

- SMVD: Symmetric MVD

- SPO: SEI processing order

- SPS: Sequence parameter set

- SR: Super-resolution

- STMVP: Spatial-temporal motion vector prediction

- STRP: Short-term reference picture

- STSA: Step-wise temporal sublayer access

- TALF: Temporal adaptive loop filter

- TBA/TBD/TBP: To be announced/determined/presented/public

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content

- TIMD: Template-based intra mode derivation

- TM: Template matching

- TMRL: Template-based multiple reference line prediction

- TMVP: Temporal motion vector prediction

- TS: Transform skip

- TSRC: Transform skip residual coding

- TT: Ternary tree

- TuC: Technology under consideration

- UGC: User-generated content

- UWP: Unequal weight prediction

- VCEG: Visual coding experts group (ITU-T Q.6/21, the relevant rapporteur group in ITU-T WP3/21, which is one of the two parent bodies of the JVET)

- VCM: Video coding for machines

- VLOP: Very low-complexity operating point for NNLF

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy

- VQA: Visual quality assessment

- VT: Verification testing

- VTM: VVC Test Model – a software codebase and test model algorithm for the VVC standard

- VUI: Video usability information (part of an SPS, similar to an SEI message that applies to a video sequence)

- VVC: Versatile Video Coding, video coding standard developed by JVET, formalized by ITU-T as Rec. ITU-T H.266 and by ISO/IEC as ISO/IEC 29090-3

- WCG: Wide colour gamut

- WG: Working group, a group of technical experts (usually used to refer to the WGs of ISO/IEC JTC 1/SC 29)

- WPP: Wavefront parallel processing (usually synonymous with ECS)

- Block and unit names in MPEG-2 and AVC:

  - MB: Macroblock – a 16x16 array for the luma component and corresponding 8x8 arrays for the chroma components (basically the same as a CTU in later standards, but with a fixed size)

  - MB pair: A pair of vertically contiguous macroblocks in a frame that is coupled for use in macroblock-adaptive frame/field decoding

  - Sub-MB: One quarter of the samples of a macroblock, i.e., an 8x8 luma block and two corresponding chroma blocks of which one corner is located at a corner of the macroblock for a picture that has three sample arrays or an 8x8 luma block of which one corner is located at a corner of the macroblock for a monochrome picture or a picture that is coded using three separate colour planes.

  - Sub-MB partition: A block of luma samples and two corresponding blocks of chroma samples resulting from a partitioning of a sub-macroblock for inter prediction for a picture that has three sample arrays or a block of luma samples resulting from a partitioning of a sub-macroblock for inter prediction for a monochrome picture or a picture that is coded using three separate colour planes

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU)

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions)

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape

  - PB: Prediction block, a luma or chroma block of a PU

  - PU: Prediction unit, has the same size as a CU in the VVC context

  - TB: Transform block, a luma or chroma block of a TU

  - TU: Transform unit, has the same size as a CU in the VVC context

## Standards, TRs, supplements and technical papers approval and publication status

- MPEG-2 | H.262 (the video coding specification is common text and not publicly available without cost per ISO special request)

  - ITU-T H.262 V3 was approved in 2012-02; its Amd 1 was approved in 2013-03 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-09

  - ISO/IEC 13818-2:2013 (Ed. 3) FDIS ballot closed 2012-05-08; FDAM 1 ballot closed 2013-04-12 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-10

  - Conformance testing (not joint with ITU-T)

    - ISO/IEC 13818-4:2004 (Ed. 2) FDIS closed 2004-08-22, published 2004-12-12; it specifies conformance testing for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), and Part 7 (AAC) (confirmed freely available 2026-05)

    - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing (confirmed freely available 2026-05)

    - Cor 1:2007, Cor 2:2009, Cor 3:2012 (publication availability unclear, as these do not appear to be directly available from the ISO website) may also have video relevance

  - Reference software (not joint with ITU-T)

    - ISO/IEC TR 13818-5:2005 (Ed. 2) final ballot closed 2005-07-24, published 2005-10; it specifies reference software for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), Part 7 (AAC) and Part 11 (IPMP) (confirmed freely available 2026-05)

- AVC (twin text)

  - ITU-T H.264 V14 was Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13 (ITU work programme link)

  - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

  - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 IPT-PQ-C2 issued at 28th meeting 2022-10, third draft issued at 29th meeting 2023-01, fourth draft issued at 30th meeting 2023-04, formal project requested and CD of 11th edition issued at 31st meeting 2023-07, DIS issued at 32nd meeting 2023-10, DAM ballot closed 2024-04-15, ISO/IEC 14496-10:2025 (Ed. 11) FDIS issued at 34th meeting 2024-04, DIS approved for registration as FDIS 2024-09-17, FDIS ballot issued 2025-01-14, FDIS ballot closed 2025-03-23; published 2025-07-10, not yet available without cost

  - ITU-T H.264 V15 Consented 2024-04-26, last call began 2024-07-16, approved 2024-08-13, published 2024-11-13

  - ISO/IEC 14496-10:202x (Ed. 12), initiated as ISO/IEC 14496-10:2025/Amd.1 to support some SEI messages of VSEI v4, requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, closed 2025-12-31, summary of voting in m75350, preliminary DoCR issued at January meeting in WG 5 N 384, FDIS issued 2025-05-01, DIS approved for registration as FDIS 2026-07-09, pending text submission, public availability requested 2026-04 (ISO Projects link)

  - ITU-T H.264 V16 Consented by WP3/21 2026-05-01, last call began 2026-05-16, approved 2026-06-13 (ITU work programme link)

- Post-meeting update: Published 2026-07-28

  - Conformance testing (twin text)

    - ITU-T H.264.1 V6 Approved 2016-02-13, published 2016-06-17

    - Various amendments of ISO/IEC 14496-4:2004 (Ed. 2), including:

      - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 43:2015 3D-AVC conformance testing (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC (confirmed publicly available 2026-05)

  - Reference software (twin text)

    - ITU-T H.264.2 V6 Approved 2016-02-13, published 2016-05-30

    - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) have been published, including:

      - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC (confirmed publicly available 2026-05)

- HEVC (twin text)

  - ITU-T H.265 V7 approved 2019-11-29, published 2020-01-10

  - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

  - ITU-T H.265 V8 Consented at the 22nd meeting (with shutter interval information SEI message and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

  - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

  - ISO/IEC 23008-2:2023 (Ed. 5), initiated as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of 2022-04, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10, FDIS issued 29th meeting of 2023-01, FDIS ballot opened 2023-08-06, closed 2023-10-02, published 2023-10-30

  - ITU-T H.265 V9 Consented at 31st meeting 2023-07, approved 2023-09-13, and pre-published 2023-09, published 2023-11-24

  - Preliminary draft HEVC text for YCgCo-Re and YCgCo-Ro issued at 26th meeting 2022-04, second draft including SMPTE ST 2128 issued at 28th meeting 2022-10, third draft at 29th meeting 2023-01, fourth draft at 30th meeting 2023-04, formal work item requested and CDAM 1 issued 31st meeting 2023-07, DAM issued with new (multiview) profiles and SEI messages at 32nd meeting 2023-10, DAM ballot closed 2024-04-08

    - ITU-T H.265 V10 Consented 2024-04-26, last call opened 2024-07-01, closed 2024-07-28, approved 2024-07-29, pre-published 2024-08-06, published 2024-10-07

    - ISO/IEC 23008-2:2025 (Ed. 6) FDIS issued from 35th meeting 2024-04, DIS approved for registration as FDIS 2024-10-03, FDIS registered for formal approval 2024-10-13, FDIS ballot issued 2024-12-30, FDIS ballot closed 2025-02-24, published 2025-03-20

  - ISO/IEC 23008-2:2026 (Ed. 7), initiated as planned ISO/IEC 23008-2:2025/Amd.1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consultation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, FDIS issued from 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, FDIS registered for formal approval 2026-03-09, FDIS ballot issued 2026-06-08, public availability requested 2025-10 (ISO Projects link)

- Post-meeting update: FDIS ballot closed 2026-08-04, published 2026-09-27, not yet available without cost.

  - ITU-T H.265 V11 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19, published 2026-03-18 (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.265.1 V3 approved 2018-10-14, published 2019-01-15

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06, not yet available without cost

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15, not yet available without cost

  - Reference software (twin text)

    - ITU-T H.265.2 V3 approved 2016-12-22, published 2017-04-10

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, FDIS issued from 2016-02 meeting, published 2017-03-01, not yet available without cost

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, FDAM issued from 2017-04 meeting, FDAM ballot opened 2017-08-10, closed 2017-10-07, published 2017-11-09, not yet available without cost

- VVC (twin text)

  - ITU-T H.266 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

  - ITU-T H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

  - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, FDIS ballot opened 2022-06-29, closed 2022-08-24, published 2022-09-25

  - ISO/IEC 23090-3:2024 (Ed. 3), initiated as (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03, FDIS issued at WG level 2023-07, FDIS ballot opened 2024-05-11, closed 2024-06-26, published 2024-07-17

  - ITU-T H.266 V3 Consented 2023-07, approved 2023-09-29 and pre-published 2023-09, published 2023-11-29

    - H.266 (2023) Erratum 1 published 2025-05 to correct errors in a few figures

  - ISO/IEC 23090-3:2026 (Ed. 4), initiated as ISO/IEC 23090-3:2024/Amd.1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, FDIS registered for formal approval 2026-03-09, FDIS ballot initiated 2026-06-08, public availability requested 2025-10 (ISO Projects link)

- Post-meeting update: FDIS ballot closed 2026-08-04, published 2026-09-27, not yet available without cost.

  - ITU-T H.266 V4 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-22, published 2026-03-19 (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-15:2022 (Ed. 1) approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

    - ISO/IEC 23090-15:2024 (Ed. 2) began as Amd.1 Operation range extensions – CDAM 1 issued from 24th meeting 2021-10, DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15, consolidated into FDIS 2nd edition issued as an output of the 29th meeting in January 2023, ballot opened 2024-04-08, closed 2024-06-03, published 2024-07-04

    - ITU-T H.266.1 V2 Consented 2023-07, approved 2023-09-13 and pre-published 2023-09, published 2023-10-19

    - ISO/IEC 23090-15:2026 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, closed 2025-06-30, DIS issued at 39th meeting 2025-07, closed 2025-12-22, summary of voting in m75347, proceeded directly to publication at 41st meeting 2026-01, final text received 2026-04-27, pending proof review (ISO Projects link)

- Post-meeting update: Proof returned by secretariat 2026-08-03, published 2026-08-23, not yet available without cost.

    - ITU-T H.266.1 (V3) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19, published 2026-03-23 (ITU work prgramme link)

  - Reference software (twin text)

    - ITU-T H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to DIS approved for registration status in ISO Projects system 2022-04-21, FDIS registered for formal approval 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

    - ITU-T H.266.2 V2 Consented 2024-04-26, last call opened 2024-05-16, closed 2024-06-12, approved 2024-06-13, pre-published 2024-08-06, published 2024-10-09

    - ISO/IEC 23090-16:2025 (Ed. 2) Request & CD issued 2024-04, consultation deferred due to meeting timing, consultation initiated 2024-09-06, closed 2024-11-01, DIS issued at 36th meeting 2024-11, approved for registration as DIS 2024-12-18, DIS registered 2025-01-7, DIS ballot opened 2025-03-09, closed 2025-06-01, approved to proceed to publication without an FDIS at 29th meeting 2025-07, public availability requested 2025-07, published 2025-11-19, not yet available without cost. (Roughly corresponding to H.266.2 V2 already approved and published in ITU-T)

- VSEI (twin text)

  - ITU-T H.274 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

  - ITU-T H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

  - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

  - ISO/IEC 23002-7:2024 (Ed. 3) began as (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, DAM ballot closed 2022-04-06, FDIS 3rd edition issued 2023-07, FDIS ballot began 2024-07-25, closed 2024-09-20, published 2024-10-30

  - ITU-T H.274 V3 Consent 2023-07, approved 2023-09-29, pre-published 2023-10-11, published 2024-03-12

  - ISO/IEC 23002-7:2026 (Ed. 4) Request & CDAM 1 issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, FDIS registered for formal approval 2026-04-01, FDIS ballot initiated 2026-06-08, public availability requested 2025-10 (ISO Projects link)

- Post-meeting update: FDIS ballot closed 2026-08-04, published 2026-09-27, not yet available without cost.

  - ITU-T H.274 V4 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-02-04, published 2026-04-09 (ITU work programme link)

- CICP (twin text)

  - ITU-T H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

  - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

  - ITU-T H.273 V3 Consent 2023-07, approved 2023-09, not published due to waiting for publication of SMPTE ST 2128

  - ITU-T H.273 V4 Consented 2024-04-26, last call opened 2024-06-16, closed 2024-07-13, approved 2024-07-14, pre-published 2024-08-06, published 2024-10-07

  - ISO/IEC 23091-2:2025 (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, DIS ballot closed 2023-04-06, preliminary draft text for including SMPTE ST 2128 issued at 28th meeting, incorporated into preliminary FDIS at 30th meeting 2023-04, FDIS issued 2024-04 (after waiting for publication of SMPTE ST 2128, then proceeding), FDIS registered for formal approval 2024-10-24, FDIS ballot issued 2024-12-03, FDIS ballot closed 2025-01-28, published 2025-02-25, not yet available without cost

- Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics (twin text)

  - ITU-T H-Series Supplement H.Sup15 V1, approved 2017-01-27, published 2017-04-12

  - ISO/IEC TR 23008-14:2018 (Ed. 1) published 2018-08-06, not yet available without cost

- Signalling, backward compatibility and display adaptation for HDR/WCG video coding (twin text)

  - ITU-T H-Series Supplement H.Sup18 V1, approved 2017-10-27, published 2018-01-18

  - ISO/IEC TR 23008-15:2018 (Ed. 1) published 2018-08, not yet available without cost

- Usage of video signal type code points (twin text)

  - ITU-T H-Series Supplement H.Sup19 V3 approved 2021-04-30, published 2021-06-04

  - ISO/IEC TR 23091-4:2021 (Ed. 3) published 2021-05-23, not yet available without cost

- Working practices using objective metrics for evaluation of video coding efficiency experiments (twin text)

  - ITU-T H-Series Supplement HSTP-VID-WPOM V1 approved 2020-07-03, published 2020-11

  - ISO/IEC TR 23002-8:2021 (Ed. 1) published 2021-05-20, not yet available without cost

- Film grain synthesis technologies for video applications (twin text)

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting, CDTR issued at 29th meeting 2023-01, consultation period ended 2023-07-09, DTR text was issued from the 31st meeting in July 2023, put on hold by ISO staff editors, DTR ballot opened 2024-03-05, closed 2024-04-30, NB comments handled by ISO staff editor in consultation with project editors, published 2024-07-24

  - ITU-T H-Series Supplement H.Sup21 V1 (ex H.Sup-FGST), Agreement 2025-01-24 by ITU-T SG21, pre-publication 2025-03-26, published 2025-05-21

  - ISO/IEC TR 23002-9:202x (Ed. 2) request for new edition issued from 42nd meeting 2026-05-01, pending CDTR at current meeting

- Optimization of encoders and receiving systems for machine analysis of coded video content (twin text)

  - ISO/IEC TR 23888-3:2026 (Ed. 1) Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, closed 2025-11-03, summary of voting in m75361, second DTR ballot issued from 41st meeting 2026-01, ballot opened 2026-03-30, closed 2026-04-28, summary of voting in m76996, agreed to proceed to publication at 42nd meeting 2026-05-01, published 2026-06-14 (ISO Projects link), not yet available without cost

  - ITU-T provisional name H.Sup.MACVC, approval target date 2026-07, i.e., the current meeting (ITU work programme link)

- Freely available ISO/IEC standards (without cost) in the ISO Webstore and IEC Webstore include

  - ISO/IEC 13818-4:2004 Conformance testing for MPEG-2 (confirmed available 2025-10-31)

  - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing (confirmed available 2025-10-31)

  - ISO/IEC TR 13818-5:2005 Software simulation for MPEG-2 (confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-4:2004 specify conformance for AVC (base text confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) specify reference software for AVC (base text confirmed available 2025-10-31)

- Intended to be freely available (without cost) but unavailable (without cost) standards and technical reports include the following. (These should be checked for previously issued requests for free availability.)

  - ISO/IEC 14496-10:2025 (Ed. 11) AVC, published 2025-07-10 (public availability request to be confirmed, publication of Ed. 12 pending)

  - ISO/IEC 23002-7:2024 (Ed. 3) VSEI, published 2024-10-30 (public availability requested 2023-07, approved by SC 29 Res. 59/2023, publication of Ed. 4 pending)

  - ISO/IEC 23008-2:2025 (Ed. 6) HEVC, published 2025-03 (public availability requested 2024-04, publication of Ed. 7 pending)

  - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for HEVC, published 2017-03-01 (public availability requested 2016-02, reiterated 2025-10, approved by SC 29 Res. 24/2026)

  - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for HEVC screen content coding extensions, published 2017-11-09 (public availability not requested 2016-06, 2016-10, 2017-01, 2017-04, 2017-07, 2017-10, 2018-01, 2018-04, public availability requested 2025-10, approved by SC 29 Res. 24/2026)

  - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06 (public availability requested 2018-10, reiterated 2025-10, approved by SC 29 Res. 24/2026)

  - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15 (public availability requested 2025-10, approved by SC 29 Res. 24/2026)

  - ISO/IEC 23090-3:2024 (Ed. 3) VVC, published 2024-07-17 (public availabilty request unconfirmed, publication of Ed. 4 pending)

  - ISO/IEC 23090-15:2024 (Ed. 2) Conformance specification for VVC, published 2024-07-04 (public availability request unconfirmed, publication of Ed. 3 pending)

  - ISO/IEC 23090-16:2025 (Ed. 2) Reference software for VVC, published 2025-11-19 (public availability requested 2025-07, approved by SC 29 Res. 34/2025)

  - ISO/IEC 23091-2:2025 (Ed. 3) CICP, published 2025-02-25 (public availability requested 2024-04, reiterated 2025-10, approved by SC 29 Res. 24/2026)

  - ISO/IEC TR 23008-14:2018 (Ed. 1) Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics, published 2018-08-06 (public availability requested 2025-07, approved by SC 29 Res. 34/2025)

  - ISO/IEC TR 23008-15:2018 (Ed. 1) Signalling, backward compatibility and display adaptation for HDR/WCG video coding, published 2018-08 (public availability requested 2025-07, approved by SC 29 Res. 34/2025)

  - ISO/IEC 23090-16:2025 (Ed. 2) Reference software for VVC (public availability requested 2025-07, approved by  SC 29 Res. 34/2025)

  - ISO/IEC TR 23091-4:2021 (Ed. 3) Usage of video signal type code points, published 2021-05-23 (public availability requested 2025-07, approved by SC 29 Res. 34/2025)

  - ISO/IEC TR 23002-8:2021 (Ed. 1) Working practices using objective metrics for evaluation of video coding efficiency experiments, published 2021-05-20 (public availability requested 2025-07, approved by Res. 34/2025)

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Film grain synthesis technologies for video applications, published 2024-07-24 (not requested 2024-04, public availability requested 2025-07, approved by Res. 34/2025)

  - ISO/IEC TR 23888-3:2026 (Ed. 1) Optimization of encoders and receiving systems for machine analysis of coded video content (public availability requested 2026-05, not yet endorsed by SC 29)

## Draft standards progression status for active work items

- AVC ISO/IEC 14496-10: 202x (Ed. 12), initiated as ISO/IEC 14496-10:2025/Amd.1 to support some SEI messages of VSEI v4, requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, closed 2025-12-31, summary of voting in m75350, preliminary DoCR issued at January meeting in WG 5 N 384, FDIS issued 2025-05-01, DIS approved for registration as FDIS 2026-07-09, pending text submission, public availability requested 2026-04 (ISO Projects link)

  - ITU-T H.264 V16 Consented by WP3/21 2026-05-01, last call began 2026-05-16, approved 2026-06-13 (ITU work programme link)

- ISO/IEC 23008-2:2026 (Ed. 7), initated as planned ISO/IEC 23008-2:2025/Amd.1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consulation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, FDIS issued from 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, final text received or FDIS registered for formal approval 2026-03-09, FDIS ballot issued 2026-06-08, public availability requested 2025-10 (ISO Projects link)

- Post-meeting update: FDIS ballot closed 2026-08-04, published 2026-09-27, not yet available without cost.

  - ITU-T H.265 V11 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19, published 2026-03-18 (ITU work programme link)

- VVC ISO/IEC 23090-3:2026 (Ed. 4), initiated as ISO/IEC 23090-3:2024/Amd.1, Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, FDIS registered for formal approval 2026-03-09, initiated 2026-06-08, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.266 V4 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-22, published 2026-03-19 (ITU work programme link)

- VVC conformance ISO/IEC 23090-15:2026 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, closed 2025-06-30, DIS issued at 39th meeting 2025-07, closed 2025-12-22, summary of voting in m75347, proceeded directly to publication at 41st meeting 2026-01, final text received 2026-04-27, pending proof review (ISO Projects link)

- Post-meeting update: Proof returned by secretariat 2026-08-03, published 2026-08-23, not yet available without cost.

  - ITU-T H.266.1 (V3) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19, published 2026-03-23 (ITU work programme link)

- ISO/IEC 23002-7:2026 (Ed. 4) Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, FDIS registered for formal approval 2026-04-01, FDIS ballot initiated 2026-06-08, public availability requested 2025-10 (ISO Projects link)

- Post-meeting update: FDIS ballot closed 2026-08-04, published 2026-09-27, not yet available without cost.

  - ITU-T H.274 V4 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-02-04, published 2026-04-09 (ITU work programme link)

- ISO/IEC TR 23002-9:202x (Ed. 2) Film grain synthesis technology for video applications – request for new edition issued from 42nd meeting 2026-05-01, pending CDTR at current meeting

- H.Sup.MACVC | ISO/IEC TR 23888-3:2026 (Ed. 1) Optimization of encoders and receiving systems for machine analysis of coded video content – ISO/IEC TR 23888-3 Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, closed 2025-11-03, summary of voting in m75361, second DTR ballot issued from 41st meeting 2026-01, ballot opened 2026-03-30, closed 2026-04-28, summary of voting in m76996, agreed to proceed to publication at 42nd meeting 2026-05-01, published 2026-06-14 (ISO Projects link), not yet available without cost

  - ITU-T provisional name H.Sup.MACVC, approval target date 2026-07, i.e., the current meeting (ITU work programme link)

- A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations or in an SC 29 Resolution. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the ongoing work items when they become finalized.

## Opening remarks

Remarks during the opening session of the meeting Tuesday 7 July at 1000-1050 CEST were as follows.

- Timing and organization of the meeting and online access and calendar posting of session plans were reviewed

  - The initial number of documents was lower than in last meeting (approximately 165 vs. 200 by the time of opening), where an increase is again observed in SEI related contributions, whereas low-level coding tools largely have decreased in number (in particular, after EE2 was discontinued). Also, only few contributions were received on  CfP preparation, indicating the maturity of the final draft from the last meeting.

  - Parallel sessions were considered to be necessary (SEI and core coding aspects), starting Wednesday morning. For the first day of the meeting (Tuesday), JVET can only use the Popov room; starting from Wednesday, two rooms (Popov or C for main track, and additionally A for HLS track) will be available.

  - Aspects of common interest (final CfP preparation, non-SEI HLS, GS SEI, plans for VSEIv5) to be reviewed and discussed at plenary level

- Future non-virtual meetings will be held as hybrid meetings with best-effort remote access (when under ISO/IEC SC 29 auspices) or as physical meetings with remote participation (when under ITU-T SG21 auspices). In terms of practical arrangements, this is identical.

- Plans for subsequent non-virtual meetings were reviewed: October 2026 (Hangzhou), January 2027 (Brisbane), April 2027 (Geneva), July 2027 (Tampere), October 2027 (Shenzhen), January 2028 (Thessaloniki), April 2028 (Geneva), July 2028 (tbd).

- Depending on the status of preparing future standardization activities, it may be necessary to extend the duration of meetings, or plan for AHG meeting days prior to the regular meeting (e.g., for analysis of CfP submissions)

- It was mentioned that some minor problems which had occurred after the re-design of the jvet.org site were fixed. Generally, the site appears much more stable and generally improved relative to the old version. Unfortunately, some problems were recently observed considering accessibility of the ftp site in ITU.

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Access to the meeting was provided using Zoom. The meeting notes by the session chair were to be continually shared via zoom screen sharing

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

- The results of the previous meeting and the meeting report JVET-AP1000 were reviewed (which was finalized ten days before the meeting, but sufficiently mature drafts had been available before in the ITU ftp site). Only minor issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision.

- At the current meeting, on-site attendance will be recorded both via the traditional sign-in sheet, and a list of JVET ITU registrations with badge pickup. Participants were asked to correct their affiliation and email in the sign-in sheet in case these changed. In the previous meeting, a few cases were found (mostly for those mentioned in the previous bullet point) where the affiliation was outdated. Remote attendance (of those not present in person) will be recorded via the zoom records. It is therefore important to follow the conventions of naming as mentioned before. Participants who cannot be correctly identified will not appear in the attendance sheet.

- There were no objections voiced in the opening plenary to the consideration of late contributions.

- There were again a significant number of documents registered where authors’ given names were not abbreviated according to the JVET custom (which helps produce shorter headings in the JVET meeting report), and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions. As for now, the JVET chair took action of correcting those cases in the document registry. Several documents also have a “self constructed” header with wrong meeting dates July 7-14.

- Some general principles:

  - Experts were asked not to pick a specific JVET number for regular documents – this function is reserved for AHG reports, summary reports, and output docs. Reserving numbers without filling a precise title shall also be avoided – the chair may flag such documents as withdrawn, as they cannot be allocated to a certain category in the meeting notes.

  - Experts were asked to always register JVET documents via the “jvet-experts.org” site, not via the MPEG DMS site, as WG 5 docs. Only at the current meeting, an exception are documents related to the work of the joint AHG on Gaussian splat coding (due to the fact that JVET/WG 5 takes lead on that at the current meeting). All documents related to that (except those that are concerned with plain syntax/semantics improvements of GSC messages in TuC) shall be registered as WG 5, and those that are related to experimentation using SEI messages (JEE6.7) and new aspects of SEI messages (JEE6.9) shall have duplicate registrations, once as WG 5 (non-public) and once as JVET (public) document. Please refer to sections 6.5.3 and 6.5.4 where some cases are listed with missing duplicate registration.

  - Experts were asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes. Provisional titles such as “EEx contribution” shall be avoided.

- A joint AHG meeting on GSC was planned to be held in ITU during the weekend July 11/12. Non-JVET participants have the opportunity of separate registration to get access to the building, JVET participants can use the JVET badge. Discussions related to SEI messages will be conducted on Saturday (HLS should not meet in parallel). Documents in section 6.5.2 can be discussed before that.

- The following ballot result had become available through the SC 29 secretariat:

  - m77878  Summary of Voting on CIB on the 23rd WG 5 meeting  [SC 29 Secretariat], reporting that the 2nd edition of film grain TR was approved.

- The FDIS ballots for next editions of HEVC, VVC and VSEI will close around August 4 (all synchronized in timing).

- The FDIS of a new edition for 14496-10 was approved at the last meeting, but not yet finally edited. The FDIS ballot should be started as soon as possible, but an overlap in timing with the ballots on HEVC, VVC and VSEI is not possible anymore.

- Plans for generating next version/edition of HEVC conformance (regarding new multiview pofiles) need to be developed. Is it mature enough for starting the work item? See further discussion under AHG5.

- Plans for generating the next versions of reference software for AVC, HEVC and VVC (which would need to support new SEI messages from VSEIv4) need to be developed. Earliest target date for ITU-T consent would be at this meeting – would at least VVC be ready to go to consent in SG 21, and should requests for all be made in ISO/IEC? For AVC, it might further be useful to generate a new part of ISO/IEC 14496 containing the entire newest JM software at least, and cancel the corresponding outdated amendments of 14496-5. See further discussion under AHG3.

- The primary goals of the meeting were:

  - Issue Final CfP, and first version of the proposal description package, including complexity analysis to be provided

  - Issue H.Sup MAVC (Corresponding TR 23888-3 was submitted for publication from the last meeting)

  - Issue second TR on film grain? See further discussion under AHG13.

  - TuC on VSEI

  - Discussion on potential WD VSEI v5

  - Consideration of new SEI proposals

  - SEI show cases and software

  - Gaussian splats – work of Joint AhG (meets Saturday/Sunday), and planning of Joint EE. All documents on this activity will be handled by JVET / WG 5

  - Exploration Experiment on Neural network-based video coding

  - Enhanced compression beyond VVC: Information docs on ECM and other improved compression packages

  - VVC white paper update (remove VSEI, some update of the 2021 version)

- Needs for joint meetings were initially discussed (see section 7.3 on joint meetings that were later actually held).

- There are two contributions marked as “GSC related” which are somewhat off-topic about avatar coding using SEI messages – these might need joint consideration with WGs 3 and 7.

- New ISO/IEC directives will enter into force by 5 Oct., and also some rules in ITU-T regarding standards development are subject to change. Both may have impact on JVET mode of working. Presentation on consequences in October meeting would be desirable.

- Principles of standards development were discussed.

- Scheduling of sessions was discussed – see under sections 2.6 and 2.15.

## Scheduling of discussions

- The times of the meeting sessions followed the needs of the onsite meeting arrangements, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were in the range of 0900-2000 CEST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were noted to potentially be necessary. Sessions were announced in the JVET calendar as far as possible in advance, although it was acknowledged that some activities (such as breakout sessions) might be held at short notice.

Particular scheduling notes are shown below, although not necessarily 100% accurate or complete. Times are recorded in the local timezone of the meeting venue, except as otherwise noted:

- Tue. 7 July, 1st day

  - Morning session:

    - 0900–1050 Opening remarks, review of practices, agenda, IPR policy reminder, scheduling

    - 1050–1330 Reports of AHGs 1-12, 14, 17

  - Afternoon session:

    - 1500–1530 Reports of AHGs 13, 15, 16, 18

    - 1530–1840 Review 4.16 CfP

    - 1840–2000 6.3 Discuss VSEI v5 WD

- Wed. 8 July, 2nd day

  - Main track:

    - 0910–1300, Review 5.1 EE1 and related, NNVC, SADL, 5.2 Enhanced compression

    - 1430–2000 4.16 CfP

  - HLS track: 0900–1800 Review 6.1, 6.2.2, 6.2.4, 6.2.5, 6.2.6, 6.2.8, 6.2.9, 6.2.10 (chaired by J. Boyce / S. Deshpande)

- Thu. 9 July, 3rd day

  - Morning sessions:

    - Main track: No meeting, side activity on proposal description template

    - HLS track: 0900–1315 6.3 Requests for VSEIv5  (chaired by JRO)

  - Afternoon sessions:

    - Main track: 1400–1600 Planning/scheduling, review CfP proposal description template

    - HLS track: 1430–2000 Review 6.2.11, 6.2.12, 6.2.13, 6.2.14, 6.2.15, 6.2.16, 6.2.17, 6.2.20 (chaired by J. Boyce / S. Deshpande)

- Fri. 10 July, 4th day

  - Morning sessions:

    - Main track: 0900–1400 Review 4.x categories except 4.13/4.18, remaining document review 5.1/5.2

    - HLS track: 0900–1300 6.2.7, 6.2.3, 6.2.13, 6.2.18, 6.2.21, 6.2.19, 6.2.21 (chaired by J. Boyce / S. Deshpande)

  - Afternoon sessions:

    - Main track: 1530–1700 Report from side activity on CfP response template document, further updates CfP preparation

    - HLS track: 1400–2000 continue review of topics from morning session (chaired by J. Boyce / S. Deshpande)

- Sat. 11 July, 5th day

  - 0900–2000 Joint AHG on Gaussian splat coding

- Sun. 12 July, 6th day

  - 0900-1015 JVET plenary for coordination/planning, review of JVET-AQ0156

  - 1030–2000 Joint AHG on Gaussian splat coding

  - HLS track: 1030–2000 Revisits, 6.4, remaining 6.6, 6.5.2 (chaired by J. Boyce / S. Deshpande)

- Mon. 13 July, 7th day

  - 0900–1230 MPEG information sharing session (outside JVET in Crowne Plaza)

  - 0900–1230 Review CfP response document template (chaired by F. Bossen)

  - 0900–1600 HLS track: Revisits, 6.5.2, 6.5.4, Remaining docs TBP (chaired by J. Boyce / S. Deshpande)

  - Afternoon sessions:

    - 1330-1530 Plenary: Coordination/planning, review of 4.13/4.18, VSEI v5, output and AHG planning, liaison communication (if any), other business as appropriate

    - Main track: 1600–1800 Final review CfP and response document template, Revisits EE1 and film grain software

    - 1600-1840 Joint meeting on Gaussian splats

- Tue. 14 July, 8th day

  - Morning sessions:

    - 0830–0900 Joint meeting on CfP

    - 0915–1215 JVET session: Review 4.18, draft liaison, action points from HLS track and other revisits, any other business

  - Afternoon sessions:

    - 1530-1600 Joint meeting on SEI for avatars

    - 1600-1800 Joint meeting on Gaussian splats

- Wed. 15 July, 9th day

  - 0900–1000 MPEG information sharing session (outside JVET in Crowne Plaza)

  - 0900–1400 JVET closing plenary:

    - Report of JAHG JVET-AQ2019 and presentation of JEE6.7/6.9 summary report JVET-AQ0022

    - Establishment of AHGs

    - Establishment of Joint AHG on GSC, and announcement of two F2F meetings Wed./Thu.

    - Review of JVET-AQ0230

    - List of HLS adoptions

    - Approval of output docs (including review as needed)

    - Review of WG 5 meeting recommendations

    - Software database for SEI etc.

    - Software timeline

    - Future planning, a.o.b.

    - Closing of meeting

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (19) (section 3)

- Project development (section 4)

  - AHG1: Deployment and advertisement of standards (0)

  - AHG2: Text development and errata reporting (0)

  - AHG3: Software development (1)

  - AHG3: Test conditions and metrics (0)

  - AHG4: Subjective quality testing and verification testing (0)

  - AHG4: Test and training material (0)

  - AHG5: Conformance test development (1)

  - AHG7: Tool assessment (0)

  - AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (0)

  - AHG10: Encoding algorithm optimization (0)

  - AHG13: Film grain technology (2)

  - Implementation studies (2)

  - Profile/tier/level specification (1)

  - AHG15: Gaming content compression (0)

  - AHG16: Hardware implementation complexity (3)

  - AHG17: CfP on video coding technology beyond VVC (6)

  - AHG18: Ultra-low latency and error resilience (2)

  - Video CICP, generic aspects of metadata (1)

- Low-level tool technology proposals (section 5) with subtopics (number counts excluding BoG and summary reports)

  - AHG11/AHG14 and EE1: Neural network-based video coding (14) (section 5.1)

  - AHG6/AHG12: Enhanced compression beyond VVC capability (3) (section 5.2)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics

  - Aspects of SEI messages in VSEI, VVC, HEVC and AVC (3) (section 6.1)

  - Study SEI messages in TuC for VSEI (58) (section 6.2)

  - Requests for VSEI v5 working draft (13) (section 6.3)

  - Potential needs for SEI messages on other topics (11) (section 6.4)

  - Usage of SEI messages for Gaussian splatting (35) (section 6.5)

  - Usage of SEI messages for avatar coding (3) (section 6.5)

  - SEI software and showcases (7) (section 6.7)

  - Non-SEI HLS aspects (0) (section 6.8)

- Joint meetings, plenary discussions, BoG reports (0) liaison (2), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (19)

These reports were discussed during 1050–1330 and 1500–1530 on Tuesday 7 July 2026 (chaired by JRO).

JVET-AQ0001 JVET AHG report: Project Management (AHG1) [J.-R. Ohm, M. Wien (co-chairs), G. J. Sullivan (vice-chair)]

Goals and activity

The work of the JVET overall had proceeded well in the interim period with lower number of input documents (as compared to the previous meeting) submitted to the current meeting. Intense discussion had been carried out on the group email reflector, and output documents from the preceding meeting had been produced as planned.

Output documents from the preceding meeting had been made initially available at the JVET web site (https://jvet-experts.org/) or the ITU-based JVET ftp site (http://wftp3.itu.int/av-arch/jvet-site/2026_04_AP_SantaEularia/). JCT-VC and JCT-3V documents can also be accessed directly via the jvet-experts site. Previous meeting documents of JVT, JCT-VC, JCT-3V and JVET are also available from the ITU-based ftp site, where sub-folders of the different groups can be found under https://www.itu.int/wftp3/av-arch/, and sub-folders ‘./documents/’ were created in the folders of the respective meetings (it is noted that, for practicality reasons, for JVET this is only done after all meeting documents including output documents are completely available in all versions, such that there may be some delay in availability). All those files come with the original time stamps by which the original versions of documents were uploaded.

The list of output documents produced since the last meeting included the following:

JVET-AP1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Posted 2026-05-15]

JVET-AP1005 Future CICP extensions (Draft 3) [Posted 2026-06-11]

JVET-AP1006 HEVC additional profiles (Draft 1) [Posted 2026-06-10]

JVET-AP1008 Conformance testing for HEVC multiview extended and monochrome profiles (Draft 2) [Posted 2026-06-20]

JVET-AP1016 AVC with extensions and corrections (Draft 6) [Posted 2026-05-14]

JVET-AP2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [Posted 2026-05-28]

JVET-AP2016 Common test conditions and evaluation procedures for neural network-based video coding technology [Posted 2026-05-24]

JVET-AP2019 Description of algorithms version 15 and software version 17 in neural network-based video coding (NNVC) [Posted 2026-06-30]

JVET-AP2020 Film grain synthesis technology for video applications ed. 2 (Draft 5) [Posted 2026-07-06]

JVET-AP2023 Exploration experiment on neural network-based video coding (EE1) [Posted 2026-04-30, last update 2026-05-18]

JVET-AP2025 Algorithm description o Enhanced Compression Model 20 (ECM 20) [Posted 2026-06-19]

JVET-AP2026 Final draft Joint Call for Proposals on video compression with capability beyond VVC [Posted 2026-05-08]

JVET-AP2028 Common test conditions on Gaussian splat coding [Posted 2026-06-30]

JVET-AP2032 Technologies under consideration for future extensions of VSEI (version 12) [Posted 2026-06-11, last update 2026-06-29]

- JVET-AP2033 Report of verification tests on VVC multi-layer coding [Posted 2026-07-06]

The eighteen ad hoc groups had made progress, and reports from those activities had been submitted. A report from the Joint AHG on Gaussian splat coding (together with WG 4 and WG 7) is expected during the meeting; a meeting of that AHG will be conducted at the ITU headquarters during the weekend of 11 and 12 July. Numerous teleconferences had been held by AHGs. Furthermore, one exploration experiments (EE) on neural network-based video coding, as well as a joint EE on Gaussian splat coding (under the JAHG) were conducted.

The arrangements for the 43rd meeting had been announced in the JVET reflector, in the JVET logistics document https://www.itu.int/wftp3/av-arch/jvet-site/2026_07_AQ_Geneva/JVET-AQ_Logistics.docx, and in the WG 5 calling notice (N 393) and agenda (N 394) for the 23rd WG 5 meeting.

Software integration was finalized approximately according to the plan.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

Around 165 input contributions (not counting the AHG reports, summary reports and crosschecks) had been registered for consideration at the current meeting.

It is further noted that, starting from the twentieth JVET meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, maintenance, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

To retain a consistent numbering scheme, the number range of output documents starting from 1001 was reserved for the previous JCT-VC and JCT-3V topic items listed above, whereas the number range starting from 2001 was retained for VVC, VSEI and exploration activities.

A preliminary basis for the document subject allocation and meeting notes for the 43rd meeting had been made publicly available on the ITU-hosted ftp site as http://wftp3.itu.int/av-arch/jvet-site/2026_07_AQ_Geneva/JVET-AQ_notes_d0.docx.

Recommendations

- The AHG recommends its continuation.

JVET-AQ0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J. R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs)]

Output documents produced

JVET-AP1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, and Video CICP, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-AP meeting:

- For VSEI

  - All four VSEI errata items from JVET-AP0058.

  - All three VSEI errata items from JVET-AP0090 with update to item 1 as noted in the JVET-AP meeting minutes.

  - All VSEI errata items from JVET-AP0091.

  - VSEI bug ticket#2 (by Xiaozhong Xu): Change "nnpfc_num_metadata_extensions_bits" to "nnpfc_num_metadata_extension_bits".

  - VSEI bug ticket#3 (by Shuai Zhao):

    - Change "omi_mask_exact_tolerance_present_flag equal to 0 specifies" to "omi_tolerance_present_flag equal to 0 specifies".

    - Change "omi_mask_width_minus1[ i ][ j ] plus 1 and omi_mask_height_minus1[ i ][ j ] plus 1 specify" to "omi_mask_width_minus1[ i ][ j ] plus 1 and omi_mask_height_minus1[ i ][ j ] plus 1 specify".

    - Change "mi_mask_label[ i ][ j ] specifies" to "omi_mask_label[ i ][ j ] specifies".

- For VVC:

  - All four VVC errata items from JVET-AP0058.

- For HEVC

  - Both HEVC errata items from JVET-AP0058.

  - The HEVC errata item from JVET-AP0091 (add a NOTE).

  - In D.3.1, correct the index of the last NOTE.

- For AVC:

  - None added, as those items for AVC from JVET-AP0058, JVET-AP0091 and JVET-AP0217 have already been integrated into the ITU-T consent text for the new version of H.264.

JVET-AP1006 HEVC additional profiles (Draft 1)

This document contains the draft text for changes to the High Efficiency Video Coding (HEVC) standard (Rec. ITU T H.265 | ISO/IEC 23008 2). The changes include the support of two new profiles, namely the Multiview Main 4:4:4 10 and 12 profiles.

JVET-AP1016 AVC with extensions and corrections (Draft 6)

The 16th edition of Rec. ITU-T H.264, approved in 2026-05, adds the interface to support eleven new SEI messages, namely SEI processing order, processing order nesting, encoder optimization information, source picture timing information, modality information, digitally signed content initialization, digitally signed content selection, digitally signed content verification, generative face video, generative enhancement face video, and AI usage restrictions request, through referencing the Versatile Supplemental Enhancement Information (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), including a new mechanism that allows future extensions of the newly introduced SEI messages. It also adds support to interface extended versions of the film grain characteristics and neural network post-filter activation SEI messages through referencing the VSEI standard.

The changes in this edition also include corrections to various minor defects in the prior content of the Recommendation.

Related input contributions

Input contributions were noted as relevant to the work of this ad hoc group:

- JVET-AQ0061 AHG9: Errata report on VSEI and HEVC

- JVET-AQ0086 AHG2/AHG9: Initial proposed text for a potential WD of VSEI v5

- JVET-AQ0089 AHG2/AHG9: Errata on general SEI signalling mechanism for VVC, VSEI, HEVC, and AVC

- JVET-AQ0093 AHG2/AHG9: On the FPA and CTI SEI messages

Remaining VVC spec tickets

Closed since JVET-AP0002 was reported:

- (none)

Carried over (strikethough means addressed in the indicated errata report and can be closed):

- #1629 mtt_split_cu_vertical_flag context uses undefined variable chType. (Discussed in JVET-AI1004)

- #1650 Incorrect indexing for the h location component in ALF process. (Discussed in JVET-AJ1004)

- #1654 Typo in derivation process for GPM motion vectors.

New (since JVET-AP0002 was reported)

- (none)

Remaining HEVC spec tickets

Closed since JVET-AQ0002 was reported:

- (none)

Carried over:

- (none)

New (since JVET-AP0002 was reported):

- #1524 Typos in Annexes C and D

Recommendations

The AHG recommended to:

- Approve JVET-AP1004, JVET-AP1006, and JVET-AP1016 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review bug tickets, and other AHG2-related inputs and act on them if found to be necessary.

Potential inclusion in JVET-AQ1004.

JVET-AQ0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs)]

The software model versions prior to the start of the meeting were:

- VTM 24.0 (April 2026)

- VTM-22.2-TuC4.0 (Feb. 2025)

- HM-18.0 (Apr. 2023)

- HM-16.21+SCM-8.8 (Mar. 2020)

- SHM 12.4 (Jan. 2018)

- HTM 16.3 (Jul. 2018)

- JM 19.1 (Apr. 2023)

- JSVM 9.19.15

- JMVC 8.5

- 3DV ATM 15.0 (no version history)

- HDRTools 0.26 (August 2025)

Software for MFC and MFCD is only available as published by ITU-T and ISO/IEC. It is planned to create repositories with the latest versions available in ITU-T H.264.2 (02/2016). All development history is lost.

- Software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow are documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

Although the development process is described in the context of the VTM software, it can be applied to all other software projects hosted on the GitLab server as well.

  - Server availability

Short before the 42nd JVET meeting denial of service was observed on the Fraunhofer HHI web servers, which required blocking large IP spaces. This may also affect legitimate access.

Shortly after lifting the block, the usage pattern reappeared. Some improvements were made by limiting the access per IP address, but this was not permanent. It is observed that the server is being crawled from thousands of IP addresses, where each is just used for 60 to 80 accesses.

Sources were found in IP ranges that belonged to Chinese telecommunication providers.

Since short before the 43rd JVET meeting, additional traffic with the same pattern appeared from IP spaces associated with US based telecommunications providers.

The number of connection attempts to the server are so high, that legitimate access is extremely slow or may fail.

Looking at the access patters, it looks like web crawling, which used to be done by a limited number of search engines. While these were usually trying not to crash the servers that are being indexed, these new players seem to not care about their impact.

The example graph below shows CPU usage caused by excessive connections to the server around July 6 2026.

  - Possible solutions

There are different options, which we may consider implementing to improve access to the GitLab repositories:

- improve proxy access for better throughput and possible pattern analysis

- reduce the crawling surface by:

  - making all repositories "internal", except these that we need to share (e.g. for personal forks of VTM, ECM, etc)

  - making all repositories "internal" and set up a new server for public access, which only gets update once daily

- Move the GitLab server to a professional hosting service / CDN (requires sponsorship)

- VTM related activities

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

The software development continued using the GitLab server. VTM version 24.0 was tagged on April 8, 2026. No new version was tagged after the 42nd JVET meeting.

The following merge requests were merged on top of VTM 24.0. A new version will be tagged as necessary:

- WG03 N01464: Implementation of carriage and application of attenuation maps (green_metadata_type==2)

- Fix step length in paddingSimd

- Film grain analysis: fix coding style and cutoff estimation, update default parameters

- JVET-AP0070: AHG10: Additional random-access configuration with reduced delay

- Remove post filter hint SEI message

- Update download location of Docker images for CI pipeline

- Fix deblocking corner case involving palette and dual-tree

- fix memory growth due to unused DecLib::m_nalUnitInfo

The following merge requests were submitted and are pending:

- Fix transcoding functionality (DebugBitstream), including switching (SwitchPOC) by adapting LMCS and ALF APS handling

Additionally a branch cfp-2026 was created based on VTM 24.0 for preparation of software for generating anchors for the upcoming CfP. The following merge requests were merged into that branch:

- JVET-AP2026: Add configurations files for CfP

- Add LastValidFrame to all per-sequence cfg files

- add packed yuv conversion application

- Update howto for 2026 CfP

- Add qp/qpif settings

  - CTC Performance

There is currently no performance change expected on after VTM 24.0.

  - Issues in VTM affecting conformance

The following issues in VTM master branch may affect conformance:

- Missing HLS features (see sections below)

  - Status of implementation of proposals of previous JVET meetings

The following list contains all adoptions of the Q and R meetings that were not marked as merged (or submitted) or specification only change in the software coordinator tracking sheet:

- JVET-Q0112

- JVET-Q0154: Disallow mixing of GDR and IRAP (Disallow mixing of GDR with any non-GDR).

- JVET-Q0164

- JVET-Q0402

- JVET-R0178: Require that when no_aps_constraint_flag is equal to 1, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall be equal to 0

- JVET-R0221

- JVET-R0046: Change the description of the bitstream extraction process per the value of max_tid_il_ref_pics_plus1[ ][ ] (aspect 1.2 per JVET-R0046-v4).

- JVET-R0065: Specify that GDR AUs shall be complete – i.e., all of the layers in the CVS shall have a picture in the AU (as with IRAP AUs).

- JVET-R0191: Update the range value for num_ols_hrd_params_minus1.

- JVET-R0222 aspect 1: Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). The exact editorial expression is at the discretion of the editor.

- JVET-S0196 (JVET-S0144 item 17)

- JVET-S0227 (JVET-S0144 item 22)

- JVET-S0077 (JVET-S0139 item 5)

- JVET-S0174 aspect 2 (JVET-S0139 item 18.b)

- JVET-S0156 aspect 3 (JVET-S0139 item 21)

- JVET-S0139 item 26 (no source listed, text only?)

- JVET-S0188 aspect 1 (JVET-S0139 item 28)

- JVET-S0139 item 40 (item does not exist)

- JVET-S0042 (JVET-S0142 item 1.b)

- JVET-S0174 aspect 1 (JVET S0143 item 19)

- JVET-S0096 aspect 3 (JVET-S0140 item 10)

- JVET-S0096 aspect 4 (JVET-S0140 item 13)

- JVET-S0159 aspect 3 (JVET-S0140 item 16)

- JVET-S0171 (JVET-S0256)

- JVET-S0118 (JVET-S0141 item 7)

- JVET-S0102 (JVET-S0141 item 9.a)

- JVET-S0157 item 2 (JVET-S0141 item 13)

- JVET-S0157 item 4 (JVET-S0141 item 14)

- JVET-S0175 aspect 3 (JVET-S0141 item 16)

- JVET-S0175 aspect 1, 2 (JVET-S0141 item 17)

- JVET-S0175 aspects 4 and 5 (JVET-S0141 item 18)

- JVET-S0175 aspect 6 (JVET-S0141 item 19)

- JVET-S0198/ JVET-S0223 (JVET-S0141 item 24)

- JVET-S0173 aspect 2 (JVET-S0141 item 40.b)

- JVET-S0173 item 1 (JVET-S0141 item 51)

- JVET-S0173 item 3 (JVET-S0141 item 52)

- JVET-S0173 item 5 (JVET-S0141 item 53)

- JVET-S0173 item 6 (JVET-S0141 item 54)

- JVET-S0173 item 4 (JVET-S0141 item 56)

- JVET-S0176 item 4 (JVET-S0141 item 60)

- JVET-S0154 aspect 5 (JVET-S0141 item 68)

- JVET-S0154 aspect 6 (JVET-S0141 item 69)

- JVET-S0154 aspect 8 (JVET-S0141 item 71)

- JVET-S0095 aspect 5 (JVET-S0145 item 5)

- JVET-S0095 aspect 6 (JVET-S0145 item 6)

- JVET-S0100 aspect 1, depends on JVET-R0193 (JVET-S0147 item 2)

- FINB ballot comments

- Make high tier support up to 960.

  - SEI TuC software

Per the decision during the 32nd JVET meeting, an SEI TuC repository was created based on VTM-22.2. It was later updated to VTM-24.0. The repository is located at:

https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM

VTM-22.2-TuC5.0 was tagged on May 22nd . Changes since VTM-22.2-TuC4.0 include

- Fix build on macos arm

- JVET-AK0153: Photosensitive content SEI messages

- Remove Packed Regions SEI (moved to VTM)

- JVET-AJ0245: Multi-layer support for Constituent Rectangles SEI

- JVET-AG0328: FGR SEI message

- JVET-AK0142: Display Rectangles SEI

- Update build system for new build clients

- Update version to VTM 22.2-TuC5.0

VTM-24.0-TuC-5.0 was tagged on May 26th. Changes related to VTM-22.2-TuC5.0 include:

- Cleanup of conflicting code with VTM 24.0

- Merge VTM 24.0

The following merge requests were merged on top of VTM-24.0-TuC-5.0:

- JVET-AM0083: Auxiliary Sampling Alignment Information SEI

- JVET-AO0097 & JVET-AO0157 & JVET-AO0281: Update EOI SEI extension

- JVET-AK0155 & JVET-AL221: Implementation of luma adaptation in the EOI SEI message

- JVET-AP0073: Shutter Interval Information extension for rolling shutter

- JVET-AM0086: Photosensitive content additional information

The following merge requests were submitted and are pending:

- update FGR SEI from AP2032-v3

- Alignement of CMI SEI implementation with AP2032.

- Alignement of DOI SEI implementation with AP2032.

- Alignement of DR SEI implementation with AP2032.

- Alignement of QM SEI implementation with AP2032.

- JVET-AL0139: CR SEI update

- JVET-AJ0139: BRI SEI partition info

- Implementation of the danmu information SEI message specified in JVET-AP2032

- JVET-AL0064: PO SEI message extension for root process NNPFs complexity

- JVET-AP0084: Gain Map SEI

- JVET-AO0160: MI Extension for Multispectrum

- JVET-AP0076: Floating point in SEI

- JVET-AI0340: Implementation of AI-restrictions usage SEI message

Reasons for MR being pending are:

- no reply on questions

- no rebase after other merges

- missing review

- unavailability of the GitLab server (see above)

- HM related activities

There was no new HM version tagged during this meeting cycle.

The following MRs were merged:

- JVET-AE0101: implement phase indication SEI message

- Update auto-build from VTM build definition

- Update copyright headers to include 2025

- JVET-AK0107: Modality Information SEI

- JVET-AK0194: Digitally Signed Content SEI messages

- Fix building on ARM

- Enable build on macOS/arm

- Modify semantics of IntraPeriod to enable autosetting based on frame rate

- Fix yuv output when bitdepth is changed between sequence

- Add missing initialization in TComPPS and TComScalingList (fix asan/msan errors)

- Avoid reading SEI content when payloadSize = 0

- Modified CMake for ARM variations

- Update build pipeline from VTM

- JVET-AK2006:Add support for SPTI SEI message

- Remove output picture marked as non reference from the DPB

- Fix build after merging !82

- JVET-AL0059: Packed regions information SEI

- JVET-AL0061: Import encoder optimization information SEI Message from VTM

- JVET-AN0237: Film Grain Analysis

- JVET-AL0148: implementation of GFV and GFVE SEI messages

- JVET-AL0339: Spatial resolution for film grain characteristics SEI message.

- Add 2026 to copyright date range

- Fix erroneously skipped pictures when decoding

- Fix condition to reset PicOrderCntMsb (could happen on CRA)

- Mark all pictures as to not be output when encountering a CRA with noRaslOutputFlag = 1

- Fix bumping process (add conditions on PicLatencyCnt, fixes for EOS before CRA cases)

- JVET-AL0062: AI usage restrictions SEI message

- Add support for NNPF SEI messages

- Merge DSC SEI code from VTM up to VSEIv4 syntax

The following MRs are pending for the main branch.

- Aligning VTM and HM code - film grain analysis

A new version of HM is pending to be tagged . CTC need to be run to verify coding performance.

A new branch was created for 4:4:4/Multiview profile support:

https://vcgit.hhi.fraunhofer.de/jvet/HM/-/tree/dev-multiview444?ref_type=heads

The following MRs were merged into the dev-multiview444 branch:

- JVET-AN0292-0293: Multiview HEVC encoding and decoding including 4:4:4 profiles

- Remove restriction of MR target branch for CI

- MV-HEVC 4:4:4 multi-profile support and decoder compatibility fixes

- JVET-AO0227: Multivew 444 fixes

The following MRs are pending:

- HRD Initial CPB removal delay handling

- MV-HEVC range-extension profiles, SHVC support, and arm64 portability

The HM SCC (SCM) branch (HM-16.21+SCM-8.8) has not been updated for the recent HM versions. Updating SCM to, for example, HM-18.0+SCM-8.8 should be considered. It may though be helpful to move SCC related functionality into separate source files. Volunteer work towards merging the branches would be appreciated.

As reported in the previous reports, further information on lambda optimisation in HM would be appreciated, including comparison of allocation of bits within the GOP structures between HM and VTM.

Otherwise the HEVC bug tracker lists:

- 44 tickets for “HM”, (most of which were created before 2019; 1 created in last year),

- 1 ticket for “HM RExt” (created in 2020),

- 9 tickets for “HM SCC” (most of which were created before 2018),

- 1 ticket for “RExt Text” (created in 2015),

- 1 ticket for “SCC Text” (created in 2016),

- 8 tickets for text (6 created before 2021; 2 created in the last year),

- 2 tickets for encoder description (created in 2020 and 2014)

Help to address these tickets would be appreciated.

- 360Lib related activities

There had not been any further developments to 360Lib during this meeting cycle.

- SCM related activities

There had not been any further developments to SCC’s SCM during this meeting cycle.

- SHM related activities

An update to HM 18.0 and some smaller changes were merged. A new tag is planned.

- HTM related activities

There had not been any releases of HTM of MV-HEVC and 3D-HEVC.

One merge request is was merged:

- JVET-AE0295: MV Main 10 profile support (waiting for proponent response)

The next release will also include the following changes:

- JVET-Z0209: Early termination during calculating RDcost of depth

- HDRTools related activities

Considerable changes were recently made to the HDRTools package, all in the 0.27-dev branch. These include:

- Add FilmGrainTool for film grain analysis and synthesis supporting two grain models selectable via GrainModel parameter:

- AFGS1 (AOMedia Film Grain Synthesis 1): autoregressive model with per-pixel scaling LUTs, LFSR-based random grain, and overlap processing

- FGC (MPEG/ITU Film Grain Characteristics, SEI type 19): frequency-filtering model with DCT-based grain databases, intensity-interval component models (sigma, h_cutoff, v_cutoff), and block-based blending

- Synthesis-only mode auto-detects grain model from JSON metadata

- Add FilmGrainParams, FilmGrainAnalyzer, FilmGrainSynthesizer classes in common/ for extensible film grain processing

- Add FrameFilterBilateral edge-preserving bilateral filter

- Add ECFIConvert tool for Enhanced Colour Format conversion (4:4:4 <-> 4:2:0 + chroma enhancement files) with RGB 3-channel support and separate/vertical/horizontal packing modes

- Add PictureSegmentation tool for mask-based picture segmentation with 4 modes (mask background, retain specific, extract per-object, mask specific segments) supporting 4:4:4, 4:2:0, and monochrome inputs

- Add Json parser for metadata file support

- Add Frame::copyComponent for cross-component copy with bit-depth conversion

- Add Frame::stackComponent/unstackComponent for vertical and horizontal channel packing/unpacking

- Add Frame::maskWithSegmentMap for segment-based masking

- Add Frame::buildSegmentMap and SegmentMapConfig for PSI segment map construction from mask frames (NULL/IMPLICIT/FULL/SPARSE types)

- Fix compiler warnings (strncat, unused variables, indentation)

- Add InputAVI class for reading AVI files, enabling AVI-to-any-format conversion in HDRConvert. Parses RIFF/AVI headers and builds a frame index for direct seeking. Supports BI_RGB, I420, UYVY, YUY2, YVYU, v210, v410, r210, r10k, R12B, and R12L compressors.

- Add AVI 2.0 (OpenDML) reading support in InputAVI. Parses super index (indx) and standard index (ix##) chunks, LIST odml with dmlh for real total frame count, and RIFF AVIX continuation chunks for files >2GB.

- Fix missing m_avi initialization in IOFunctions AVI extension block

- Add AlphaComposite tool for alpha channel compositing per H.265/H.274

- Alpha Channel Information SEI, with per-frame JSON metadata support, standard/pre-multiplied/unspecified blending modes, and configurable alpha post-processing (increment, clip, threshold/range clamp)

- Add dithering support for bit-depth reduction (DitherMode parameter):

- RPDF, TPDF, and ordered Bayer 8x8 modes with reproducible xorshift32 PRNG seeding. Legacy EnableDither=1 maps to DitherMode=1.

- Fix Y4M header to always emit C tag for >8-bit and add X tag extensions for transfer function, colour primaries, and sample range metadata

- Fix Y4M header parameter handling: add warnings for unsupported parameters and fix output defaults

- Fix frame index computation for fractional frame rates

- Fix HDRConvertTIFF chroma upsampling for subsampled input

- Fix ScaleOnly bit-depth conversion producing all-zero samples

- Fix v210 48-pixel horizontal line alignment per Apple TN2162

- Fix chroma location handling: CL_FOUR/CL_FIVE vPhase corrections in

- Conv444to420Generic, Conv422to420Generic, and FrameScaleHalf; add chromaLocationHPhase/VPhase utilities to TypeDefs.H; add ChromaLocation parameter support in Conv420to422Generic

- Fix FrameScaleLanczosInt m_factorX/Y typos in chroma offsets, extract setChromaOffsets() helper, fix chroma filter usage and stale variable

- Fix FrameScaleHalf shift operator bugs (3 instances of * -> >>)

- Fix FrameScale integer offset inconsistency (add m_offsetX/Y to origX/Y)

- Consolidate FrameScale imgpel/uint16 duplicate pairs into C++ templates

- Add HDRConvScalerYUV FrameStoreIndex enum replacing magic number indices

- Add PrintDebugParams CLI parameter (replacing compile-time define)

- Add Google Test framework (unit + integration tests, 201 tests total) with BUILD_TESTING CMake option; bump C++ standard to C++14

- Add User Manual (doc/UserManual.md) consolidating all tool documentation

Given the number of changes, the AHG is considering whether a new version should be released soon.

- JM, JSVM, JMVM related activities

There had not been any further developments to JSVM or JMVM during this meeting cycle.

There was no new JM version tagged during this meeting cycle, although merge requests were submitted for SEI messages in VSEI.

The following MRs were merged:

- Update build servers aligned with HM

- JVET-AK0107: Modality Information SEI

- Update CI & CMake configuration to include current build setups

The following MRs are pending [with status indicated]:

- JVET-AK2006: NNPFC and NNPFA SEI message (in review)

- JVET-AL0062: AI usage restrictions SEI message (in review)

- JVET-AK2006: SPTI SEI Message (in review)

- JVET-AL0061: Encoder Optimization Information SEI Message (in review)

- JVET-AE0101: implement phase indication SEI message (in review)

- JVET-AL0148: Add support for Generative Face Video (GFV) and Generative Face Video Enhancement (GFVE) SEI messages(in review)

For the previous release of JM only basic testing was performed to confirm that the code still works, but performance testing is pending. Performance changes are unlikely but may occur due to bug fixes. A possible encoder/decoder mismatch was reported privately, but needs more investigation.

- Bug tracking

The creation of accounts to the Trac based bug trackers was closed due to excessive spam account registrations. It is planned to move to GitLab integrated bug trackers and keep the old ones only read only.

GitLab based tracking was tested for TuC and VSEI specification texts, which seems to work well.

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain.

https://hevc.hhi.fraunhofer.de/trac/hevc

New account creation was disabled due to a large amount of spam registration. Extensive maintenance of the Trac bug tracker would be required to keep it secure. It should be considered to move bug tracking to GitLab, although no automatic conversion of existing bugs is available.

Bug tracking for HDRTools is located at:

https://gitlab.com/standards/HDRTools/-/issues

Please file all issues related to the VVC reference software and HDRTools into the appropriate bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

- CTC alignment and merging

There are currently 8 JVET CTC documents:

JVET-AL2010		VTM/HM 4:2:0 test conditions

JVET-Z2011		VTM/HM HDR test conditions

JVET-AA2018	VTM/HM high bit depth test conditions (without spreadsheet)

JVET-T2013		VTM non-4:2:0 test conditions

JVET-AA1100	HM non-4:2:0 test conditions

JVET-U2012		VTM 360 video test conditions

JVET-AC1009	SHVC test conditions

JVET-AC1015	SCM test conditions

JVET-AE1013		3DV test conditions

Further merging of HM RExt CTC into the appropriate VVC CTC was investigated (non 4:2:0 chroma formats), but proper comparable HM configuration files were not yet available by the beginning of this meeting.

The HM HDR test conditions require additional post-processing stages, due to WPSNR, deltaE100 and PSNRL100 values not being generated directly by the HM encoder. Merging of respective functions from VTM is recommended to streamline the process.

- Guidelines for reference software development

Current versions of the software guidelines are:

- Guidelines for JM software development are included with the software

- JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

- JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

- Related proposals

| JVET-AQ0187 | m77479 | Consolidating the HEVC Reference Software: Adding SHVC Support to the dev-multiview444 Branch | A. M. Tourapis, D. Podborski, J. Kim, S. Paluri, S. Choi, W. Zia (Apple) |
| --- | --- | --- | --- |

- Recommendations

The AHG recommended to:

- Continue to develop reference software.

- Improve documentation, especially the software manual.

- Encourage people to test VTM and other reference software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bitstreams/test cases that trigger bugs in VTM and other reference software.

- Encourage people to submit non-normative changes that either reduce encoder run time without significantly sacrificing compression performance or improve compression performance without significantly increasing encoder run time.

- Design and add configuration files to the VTM software for testing of HLS features.

- Review VTM-related contributions and determine whether features should be added (or removed) from the software.

- Continue to investigate the merging of branches.

- Continue to investigate merging of CTC documents.

- Verify correctness of CTC documents and issue updates as appropriate

- Keep common test conditions aligned for the different standards.

Regarding the attacks to gitlab, it was agreed that the solution “making all repositories "internal" and set up a new server for public access, which only gets update once daily” would be preferred.

It was planned to issue new versions of H.266.2 and H.265.2 (reference SW of VVC and HEVC) by April 2027, and at the current meeting to issue requests and CDs for new editions in WG 5, with target of also go to FDIS in April 2027.

More implementation work and study is needed for AVC reference software before going to next version/edition.

JVET-AQ0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs)]

Test sequences

The set of JVET test sequences is hosted at https://vqa.lfb.rwth-aachen.de. A mirror of this site is available at https://datacloud.hhi.fraunhofer.de with the same login credentials. The directory structure of the previous ftp server has been maintained. The test sequences used for CfP/CTC are available in directory “/ctc”.

Due to copyright restrictions, the JVET database of test sequences is only available to accredited members of JVET (i.e., members of ISO/IEC MPEG and ITU-T VCEG). Members of JVET may contact the JVET chair for login information. The password for the site is updated after each meeting with the Zoom password for that meeting. The tentative schedule for the changes is as follows:

- After the April meeting, password changes on 1st of June

- After the July meeting, password changes on 1st of September

- After the October meeting, password changes on 1st of December.

- After the January meeting, password changes on 1st of March.

Related contributions

No input contributions specifically related to AHG4 were registered for this meeting.

Recommendations

The AHG recommended to:

- To continue to collect new test sequences available for JVET with licensing statement.

- To collect volunteers to conduct further verification tests and subjective quality tests.

- To collect volunteers to actively contribute to the verification test development.

It was commented that a verification test comparing new HEVC multiview profiles and corresponding VVC multiview would be interesting, but for that novel test material (stereo 10+ bit, UHD, HDR) would be needed.

JVET-AQ0005 JVET AHG report: Conformance testing (AHG5) [I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice-chairs)]

- Activities

The AHG communication is conducted through the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG5] in message headers. However, no correspondence marked as AHG5 was sent between the 42nd and 43rd meetings.

- Timeline

Progress on the conformance testing specification is proceeding according to the timeline below:

- VVCv1 conformance:

  - ISO/IEC FDIS 23090-15 issued from 2021-10 meeting, FDIS registered for formal approval 2022-07-11, FDIS ballot closed 2022-11-04, standard published 2022-11-24

  - H.266.1 V1 Consent 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, standard published 2022-07-12.

- VVCv2 conformance:

  - ISO/IEC 23090-15/Amd.1 CDAM: 2021-10

  - ISO/IEC 23090-15/Amd.1 DAM: 2022-01

  - DAM ballot closed 2022-11-15

  - ISO/IEC FDIS 23090-15:202x 2nd edition text output of 2023-01, preparation delayed to 2023-09, FDIS ballot opened 2024-04-08, FDIS ballot closed 2024-06-03, published 2024-07-04

  - H.266.1 V2 Consent 2023-07, Last Call began 2023-08-16, Approved 2023-09-13, pre-published 2023-10-06, published 2023-10-19

- VVCv3 conformance:

  - CD: 2025-04-15

  - DIS: 2025-07-25

  - DIS ballot closed 2025-12-22

  - FDIS: 2026-02-20

  - IS: 2026-06-30

  - H.266.1 V3 Consent 2025-10-17, Last Call began 2025-12-01, Last Call ended 2026-01-12, Approved 2026-01-16, pending prepublication.

- Status of bitstream submissions

The status at the time of preparation of this report is as follows:

- conformance bitstreams for VVC:

  - 104 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 283 total bitstreams have been provided, checked, and made available

  - No changes occurred between the 42nd and 43rd meetings.

- conformance bitstreams for VVC operation range extensions:

  - 57 bitstream categories have been identified

  - 128 bitstreams of 57 identified categories have been provided, cross-checked, and made available

  - No changes occurred between the 42nd and 43rd meetings.

- additional conformance bitstreams for VVC Multilayer:

  - 3 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 7 total bitstreams have been provided, cross-checked, and made available

  - No changes occurred between the 42nd and 43rd meetings.

- conformance bitstreams for new HEVC Multiview profiles:

  - 4 new HEVC Multiview Extended 10 bitstreams provided in JVET-AP0244 have been cross-checked and made available, resulting in a total of 6 HEVC Multiview Extended 10 bitstreams.

  - 2 new HEVC Multiview Extended bitstreams provided in JVET-AP0244 have been cross-checked and made available, resulting in a total of 4 Multiview Extended bitstreams.

  - 2 HEVC Multiview Main 10 bitstreams have been provided, cross-checked, and made available. There were no changes between the 42nd and 43rd meetings.

  - 4 new HEVC Multiview Monochrome bitstreams provided in JVET-AP0244  have been cross-checked and made available.

- Activities and Discussion

The AHG activities are on schedule relative to the preliminary timeline shown in section 3.

VVC activities:

The streams are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. There were no changes to the bitstreams or packages.

VVC operation range extensions activities:

The streams are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. There were no changes to the bitstreams or packages.

VVC Multilayer activities:

The streams are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. There were no changes to the bitstreams or packages.

HEVC Multiview activities supporting extended bit depth:

The 16 packages are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/HEVCMultiview/

The 10 HEVC Multiview streams provided in JVET-AP0244 at the 42nd meeting have been cross-checked using the HM dev-multiview444 branch (https://vcgit.hhi.fraunhofer.de/jvet/HM/-/tree/dev-multiview444) and made available. There was no progress on generating streams for the HEVC Multiview Monochrome 10, 12, and 16 between the 42nd and 43rd meetings.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list, but they may send email to it.

- Contributions

JVET-AQ0186 MV-HEVC Conformance Bitstreams for the Multiview Monochrome 10/12 and Multiview 4:4:4 10/12 Profiles [A. M. Tourapis, D. Podborski, J. Kim (Apple)].

- Ftp site information

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

- VVC:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

- VVC operation range extensions:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2

The ftp site for uploading bitstream file is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, the following configuration is suggested:

In the Filezilla Edit  Settings  Connection menu, it was previously necessary to set the minimum TLS level to 1.0. Since around January 2024, this configuration issue went away, and ITU disabled the use of TLS 1.0 and 1.1 on its servers.

- Recommendations

The AHG recommends the following:

- Review the related contributions.

- Maintain and update the conformance bitstream database and report problems in JVET document 1004.

- Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AP1008, and continue the generation, cross-checking, and documentation of the conformance streams for the HEVC Multiview profiles supporting extended bit depth, the HEVC Multiview Main 10 profile, and the HEVC Multiview monochrome profiles (Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles, collectively referred to as the multiview format range extensions profiles).

Streams for monochrome 16 are still missing. The software supports this, such that generation of those streams should be possible.

It was planned to issue new versions of H.265.1 (conformance of HEVC) by April 2027, and at the current meeting to issue request and CD for new edition in WG 5, with target of also go to FDIS in April 2027.

JVET-AQ0006 JVET AHG report: ECM software development (AHG6) [V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs)]

- Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following changes were integrated into ECM-20.0:

JVET-AP0180: Fix incorrect second HoG amplitude mode assignment in MDIP [MR 980]

JVET-AP0168: CCCM clipping operations with clipping range adjustment (Test 1.3) [MR 982]

JVET-AP0105: Fix on subsampling filter of CCLM/CCCM for HDR test sequences [MR 984]

JVET-AP0086: Modified GPM partition mode with refined offset direction derivation (Test 2.1a) [MR 985]

Bug fixes:

Fix encoder crash when inter-slice references I-Slice SplitPred beyond its Qt depth limit [MR 987]

ECM-20.0 was tagged on June 23, 2026.

  - CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-AI2017 are summarized.

ECM-20.0 performance over ECM-19.1 anchor is summarized in the tables below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.24% | -0.10% | 102.6% | 101.0% | 100.2% | 100.1% |
| Class A2 | -0.01% | -0.08% | -0.13% | 102.8% | 102.3% | 100.2% | 100.1% |
| Class B | -0.01% | -0.13% | -0.29% | 106.2% | 107.4% | 102.0% | 100.2% |
| Class C | -0.06% | -0.40% | -0.42% | 103.9% | 101.6% | 100.2% | 100.1% |
| Class E | -0.03% | -0.36% | -0.59% | 100.9% | 98.3% | 99.1% | 100.2% |
| Overall | -0.03% | -0.24% | -0.31% | 103.6% | 102.6% | 100.5% | 100.1% |
| Class D | -0.07% | -0.58% | -0.61% | 102.6% | 100.5% | 100.4% | 100.0% |
| Class F | -0.03% | -0.28% | -0.58% | 107.3% | 104.6% | 99.8% | 100.1% |
| Class TGM | -0.15% | -0.19% | -0.29% | 109.4% | 124.0% | 101.5% | 100.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-19.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | -0.24% | 0.02% | 101.2% | 99.0% | 100.2% | 100.5% |
| Class A2 | -0.10% | -0.10% | -0.30% | 101.0% | 98.5% | 100.3% | 100.2% |
| Class B | -0.07% | -0.32% | -0.34% | 97.6% | 97.3% | 100.3% | 100.3% |
| Class C | -0.16% | 0.17% | -0.40% | 97.9% | 96.6% | 100.2% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.10% | -0.13% | -0.28% | 99.0% | 97.7% | 100.2% | 100.3% |
| Class D | -0.08% | -0.84% | -0.53% | 99.5% | 96.5% | 100.3% | 100.1% |
| Class F | -0.07% | -0.02% | -0.01% | 105.0% | 105.3% | 99.9% | 100.3% |
| Class TGM | -0.05% | -0.07% | 0.01% | 103.5% | 104.8% | 99.0% | 99.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-19.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | 0.23% | -0.16% | 105.1% | 106.4% | 100.2% | 100.5% |
| Class C | 0.08% | -0.31% | -0.24% | 103.0% | 107.7% | 100.2% | 100.1% |
| Class E | 0.11% | 0.00% | -0.51% | 100.7% | 105.5% | 100.1% | 100.7% |
| Overall | 0.03% | -0.01% | -0.28% | 103.3% | 106.6% | 100.2% | 100.4% |
| Class D | -0.08% | 0.79% | 0.82% | 99.6% | 101.9% | 100.2% | 100.1% |
| Class F | 0.08% | -0.51% | 0.25% | 107.5% | 107.7% | 99.6% | 100.3% |
| Class TGM | -0.03% | 0.13% | -0.17% | 111.9% | 117.9% | 101.5% | 99.7% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-19.1 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.03% | -0.33% | -0.77% | 103.7% | 105.6% | 100.2% | 100.4% |
| Class C | -0.06% | -0.55% | -0.26% | 98.2% | 103.0% | 100.2% | 100.2% |
| Class E | -0.03% | -1.04% | -1.47% | 97.8% | 100.4% | 100.1% | 100.2% |
| Overall | -0.04% | -0.58% | -0.78% | 100.3% | 103.4% | 100.2% | 100.3% |
| Class D | 0.00% | 0.10% | -0.32% | 97.5% | 94.9% | 99.9% | 100.1% |
| Class F | -0.03% | -0.37% | -0.86% | 108.0% | 113.3% | 100.8% | 100.2% |
| Class TGM | -0.02% | -0.05% | -0.18% | 111.7% | 112.2% | 100.5% | 99.8% |

The below tables show ECM-20.0 performance comparing to VTM-11.0ecm19.0 anchor.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -14.90% | -16.51% | -27.38% | 1214.6% | 584.7% | 238.1% | 327.1% |
| Class A2 | -21.55% | -25.07% | -29.33% | 1148.4% | 632.4% | 239.4% | 326.7% |
| Class B | -15.12% | -23.14% | -21.14% | 1116.1% | 694.7% | 181.8% | 356.5% |
| Class C | -15.34% | -12.36% | -13.56% | 1070.7% | 660.2% | 122.5% | 395.1% |
| Class E | -19.71% | -23.60% | -21.83% | 1013.8% | 648.9% | 140.1% | 381.9% |
| Overall | -16.97% | -20.04% | -21.97% | 1109.0% | 649.6% | 174.7% | 358.4% |
| Class D | -13.10% | -9.15% | -10.04% | 1019.5% | 696.0% | 110.5% | 414.5% |
| Class F | -31.06% | -35.35% | -35.73% | 661.7% | 686.8% | 125.7% | 375.0% |
| Class TGM | -43.82% | -49.39% | -48.74% | 552.7% | 800.5% | 152.5% | 352.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -27.75% | -25.60% | -37.70% | 1240.3% | 1262.7% | 153.6% | 248.8% |
| Class A2 | -31.04% | -35.61% | -41.12% | 1192.9% | 1441.3% | 153.6% | 248.1% |
| Class B | -25.64% | -35.41% | -32.48% | 1004.1% | 1331.1% | 189.7% | 279.5% |
| Class C | -27.44% | -24.79% | -25.87% | 1130.3% | 1502.4% | 117.6% | 322.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -27.62% | -30.66% | -33.49% | 1119.0% | 1382.2% | 153.5% | 276.9% |
| Class D | -28.06% | -25.46% | -26.60% | 1078.4% | 1593.9% | 105.7% | 360.7% |
| Class F | -33.74% | -37.37% | -38.28% | 848.0% | 911.7% | 121.5% | 296.9% |
| Class TGM | -43.06% | -49.02% | -48.99% | 746.7% | 781.3% | 124.6% | 280.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -22.74% | -40.91% | -36.05% | 1102.3% | 1259.7% | 167.6% | 267.4% |
| Class C | -25.17% | -29.43% | -31.33% | 1046.9% | 1332.2% | 126.0% | 317.0% |
| Class E | -22.73% | -29.46% | -28.52% | 1040.4% | 771.9% | 140.4% | 297.6% |
| Overall | -23.55% | -34.22% | -32.60% | 1068.0% | 1135.5% | 145.8% | 290.7% |
| Class D | -26.73% | -30.24% | -31.92% | 1036.2% | 1442.9% | 111.3% | 353.0% |
| Class F | -31.42% | -42.14% | -42.29% | 879.9% | 921.6% | 129.6% | 295.7% |
| Class TGM | -41.38% | -51.81% | -51.63% | 791.4% | 758.6% | 122.6% | 281.8% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -20.63% | -49.43% | -44.98% | 956.1% | 1234.5% | 172.7% | 294.8% |
| Class C | -23.27% | -38.09% | -39.45% | 847.8% | 1207.8% | 127.4% | 345.6% |
| Class E | -20.99% | -38.83% | -37.96% | 914.6% | 740.8% | 144.3% | 318.9% |
| Overall | -21.60% | -43.00% | -41.38% | 908.4% | 1078.7% | 149.2% | 317.0% |
| Class D | -25.96% | -41.71% | -42.64% | 841.8% | 1311.8% | 111.5% | 374.9% |
| Class F | -29.53% | -47.46% | -48.11% | 826.4% | 899.0% | 131.5% | 319.2% |
| Class TGM | -39.49% | -54.09% | -53.61% | 826.3% | 749.5% | 127.5% | 294.2% |

- ECM memory consumption

ECM encoder memory consumption (VmPeak, GiB) is provided in ECM encoder log files and is summarized in the table below as maximum class-wise consumption rounded up to GiB.

|  | AI | RA | LB |
| --- | --- | --- | --- |
| class A (A1 & A2) | 9 | 16 |  |
| class B | 4 | 8 | 5 |
| class C | 2 | 3 | 3 |
| class D | 2 | 2 | 2 |
| class E | 3 |  | 3 |
| class F | 4 | 6 | 5 |
| class TGM | 4 | 8 | 7 |

It is encouraged to care about memory allocation when developing and integrating tools into ECM. In particular, it is strongly recommended to re-use already existed memory wherever possible, rather than systematically allocating new memory, and allocate only the required amount of memory if new memory is needed.

- Recommendations

The AHG recommended to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.

- Encourage people to continue working on ECM memory consumption reduction.

- Encourage people to continue working on speeding up ECM encoder to reduce the simulation time.

It was agreed to conduct remaining maintenance work on top of ECM20.0 in the context of AHG12.

JVET-AQ0007 JVET AHG report: Tool assessment (AHG7) [X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice-chairs)]

- Group off tests

  - Test settings and crosschecking

The following five groups of tools were defined.

- Group 1: Inter template matching tools

- Group 2: Coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process

- Group 3: Intra and IBC template matching (with search) related tools

- Group 4: Tools that require more processing on the neighbouring reconstructed samples than VVC

- Group 5: Tools with large memory access

The testers and crosscheckers are planed in the table below.

| Tests | Testers / Crosscheckers | Testers / Crosscheckers |
| --- | --- | --- |
| Group 1 off | Johan Pardo (johan.esprit.pardo1@huawei.com) | -- |
| Group 2 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Xinwei Li (sid.lxw@alibaba-inc.com) |
| Group 3 off | Ryo Ishimoto (ishimoto.ryo@mail.sharp) | Zhipin Deng (zhipin.deng@bytedance.com) |
| Group 4 off | Hong-Jheng Jhu (jhuhong-jheng@kwai.com) | Xiang Li (xlxiangli@google.com) |
| Group 5 off | Lien-Fei Chen (lienfei.chen@global.tencent.com) | Jani Lainema (jani.lainema@nokia.com) |
| Group 1-5 off | Xiang Li (xlxiangli@google.com) | Hongtao Wang (hongtaow@qti.qualcomm.com) |

ECM-20.0 was used in the AHG7 tool off tests. The cfg files used are included in the ECM software package.

  - Group 1 off

Group 1 includes inter template matching tools. The offgroup1.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98.8% | 99.2% | #DIV/0! | -14.91% | -16.52% | -27.38% | 1202.9% | 678.7% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.00% | -0.01% | -0.01% | 101.2% | 99.8% | #DIV/0! | -15.13% | -23.14% | -21.14% | 1159.1% | 752.1% |
| Class C | -0.02% | -0.01% | -0.01% | 99.1% | 97.6% | #DIV/0! | -15.36% | -12.37% | -13.57% | 1104.8% | 723.6% |
| Class E | -0.01% | -0.01% | -0.01% | 98.0% | 95.9% | #DIV/0! | -19.73% | -23.61% | -21.84% | 1082.0% | 775.0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.03% | -0.02% | -0.03% | 99.2% | 99.9% | #DIV/0! | -13.12% | -9.17% | -10.06% | 1073.8% | 718.8% |
| Class F | -0.01% | -0.01% | -0.01% | 101.9% | 100.3% | #DIV/0! | -31.07% | -35.36% | -35.74% | 742.8% | 764.1% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 88.9% | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 987.7% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 779.3% |
| Class B | 4.46% | 4.64% | 4.62% | 78.2% | 68.4% | #DIV/0! | -22.28% | -32.38% | -29.33% | 794.2% | 1044.8% |
| Class C | 5.01% | 4.95% | 5.34% | 71.4% | 58.5% | #DIV/0! | -23.76% | -21.07% | -21.90% | 840.1% | 970.7% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 955.3% |
| Class D | 4.15% | 4.18% | 3.99% | 73.0% | 58.4% | #DIV/0! | -25.01% | -22.28% | -23.62% | 807.7% | 1047.6% |
| Class F | 3.70% | 3.70% | 3.65% | 85.4% | 72.4% | #DIV/0! | -31.19% | -34.95% | -35.92% | 758.7% | 720.1% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | -18.38% | -37.61% | -32.10% | 764.1% | 822.2% |
| Class C | 5.58% | 6.10% | 6.23% | 68.7% | 52.5% | #DIV/0! | -20.99% | -25.16% | -27.03% | 751.0% | 767.8% |
| Class E | 5.88% | 5.88% | 5.43% | 75.1% | 64.4% | #DIV/0! | -18.18% | -25.39% | -24.65% | 764.5% | 670.2% |
| Overall (Ref) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | -19.20% | -30.41% | -28.55% | 759.8% | 763.6% |
| Class D | 5.20% | 3.98% | 4.59% | 67.3% | 51.0% | #DIV/0! | -22.93% | -27.53% | -29.15% | 735.0% | 837.9% |
| Class F | 5.58% | 4.55% | 3.83% | 82.2% | 64.3% | #DIV/0! | -27.51% | -39.32% | -39.90% | 723.9% | 649.8% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

  - Group 2 off

Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process. The offgroup2.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM20 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | 0.41% | 0.51% | 94.7% | 97.7% | 100% | -14.77% | -16.19% | -26.99% | 1104.7% | 549.9% |
| Class A2 | 0.23% | 0.44% | 0.59% | 94.1% | 97.6% | 100% | -21.37% | -24.77% | -28.96% | 1025.4% | 570.5% |
| Class B | 0.17% | 0.46% | 0.46% | 93.3% | 97.2% | 95% | -14.97% | -22.80% | -20.75% | 1014.7% | 598.8% |
| Class C | 0.13% | 0.32% | 0.46% | 91.6% | 96.9% | 99% | -15.23% | -12.09% | -13.16% | 1002.4% | 580.5% |
| Class E | 0.29% | 0.43% | 0.53% | 93.1% | 96.8% | 98% | -19.47% | -23.26% | -21.41% | 991.0% | 676.4% |
| Overall | 0.19% | 0.41% | 0.50% | 93.3% | 97.2% | 98% | -16.81% | -19.72% | -21.58% | 1024.1% | 593.5% |
| Class D | 0.12% | 0.29% | 0.47% | 92.9% | 96.7% | 99% | -12.99% | -8.88% | -9.62% | 963.8% | 655.7% |
| Class F | 0.48% | 0.63% | 0.66% | 92.9% | 97.5% | 102% | -30.75% | -34.96% | -35.28% | 613.1% | 627.3% |
| Class TGM | 0.33% | 0.42% | 0.40% | 93.3% | 99.7% | 99% | -43.62% | -49.17% | -48.53% | 470.4% | 609.6% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM20 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 2.44% | 1.70% | 2.28% | 84.0% | 96.5% | 100% | -25.99% | -24.27% | -36.30% | 969.7% | 1329.0% |
| Class A2 | 1.51% | 1.40% | 2.12% | 83.5% | 98.7% | 100% | -29.99% | -34.70% | -39.84% | 926.1% | 1609.3% |
| Class B | 1.60% | 1.57% | 1.74% | 85.2% | 97.0% | 100% | -24.44% | -34.36% | -31.26% | 817.5% | 1534.1% |
| Class C | 1.29% | 1.02% | 1.41% | 83.8% | 95.2% | 99% | -26.51% | -24.06% | -24.83% | 912.7% | 1685.6% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.67% | 1.42% | 1.84% | 84.2% | 96.8% | 100% | -26.41% | -29.66% | -32.27% | 893.1% | 1543.3% |
| Class D | 0.60% | 1.26% | 1.08% | 83.3% | 95.3% | 99% | -27.61% | -24.43% | -25.83% | 854.3% | 1806.3% |
| Class F | 2.06% | 2.11% | 1.94% | 88.1% | 96.4% | 98% | -32.36% | -36.00% | -37.01% | 726.6% | 1008.0% |
| Class TGM | 3.34% | 4.96% | 5.26% | 90.5% | 99.9% | 100% | -41.28% | -46.75% | -46.59% | 633.3% | 892.5% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM20 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 2.03% | 3.25% | 3.19% | 78.9% | 95.1% | 99% | -21.17% | -39.03% | -33.95% | 784.1% | 929.1% |
| Class C | 1.74% | 3.25% | 3.24% | 80.4% | 96.2% | 99% | -23.92% | -27.20% | -29.16% | 793.1% | 913.6% |
| Class E | 1.31% | 1.71% | 2.60% | 83.3% | 96.2% | 99% | -21.71% | -28.29% | -26.58% | 801.7% | 544.1% |
| Overall (Ref) | 1.75% | 2.86% | 3.06% | 80.5% | 95.7% | 99% | -22.22% | -32.40% | -30.51% | 791.5% | 808.2% |
| Class D | 0.91% | 0.77% | 1.73% | 77.5% | 96.1% | 98% | -26.05% | -29.61% | -30.89% | 835.9% | 1268.4% |
| Class F | 2.69% | 2.25% | 2.70% | 85.5% | 93.3% | 100% | -29.59% | -40.64% | -40.60% | 706.9% | 605.4% |
| Class TGM | 2.65% | 6.37% | 7.16% | 88.7% | 95.4% | 100% | -39.97% | -49.17% | -48.89% | 642.7% | 516.2% |

Group 3 off

Group 3 includes intra and IBC template matching (with search) related tools. The offgroup3.cfg was used in addition to ECM CTC settings

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.20% | 4.07% | 6.48% | 71.7% | 88.8% | 100% | -13.87% | -13.38% | -23.14% | 824.6% | 497.8% |
| Class A2 | 2.72% | 7.02% | 6.69% | 71.9% | 79.9% | 100% | -19.36% | -20.26% | -25.13% | 781.3% | 483.2% |
| Class B | 2.68% | 5.26% | 5.53% | 69.0% | 78.2% | 99% | -12.88% | -19.50% | -17.01% | 743.3% | 515.8% |
| Class C | 2.56% | 3.01% | 3.39% | 69.5% | 76.6% | 99% | -13.23% | -9.85% | -10.73% | 720.6% | 503.9% |
| Class E | 3.74% | 4.68% | 4.45% | 69.7% | 78.0% | 100% | -16.70% | -20.16% | -18.43% | 706.6% | 537.5% |
| Overall | 2.59% | 4.76% | 5.23% | 70.2% | 79.8% | 100% | -14.84% | -16.57% | -18.23% | 751.0% | 508.1% |
| Class D | 1.59% | 2.28% | 2.60% | 70.0% | 78.8% | 100% | -11.72% | -7.11% | -7.72% | 696.3% | 590.9% |
| Class F | 7.24% | 8.30% | 8.47% | 72.5% | 67.7% | 100% | -26.30% | -30.47% | -30.71% | 454.3% | 440.3% |
| Class TGM | 12.50% | 14.47% | 14.95% | 77.3% | 55.0% | 100% | -36.94% | -42.24% | -41.28% | 400.1% | 360.7% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.10% | 2.41% | 5.82% | 93.0% | 101.5% | 100% | -26.97% | -23.85% | -34.87% | 1169.6% | 1276.6% |
| Class A2 | 1.67% | 3.95% | 3.81% | 91.9% | 100.2% | 100% | -29.89% | -33.19% | -38.92% | 1094.3% | 1441.2% |
| Class B | 1.37% | 5.25% | 5.21% | 90.3% | 97.9% | 100% | -24.62% | -32.08% | -29.08% | 925.7% | 1448.3% |
| Class C | 1.05% | 1.31% | 2.06% | 92.1% | 98.5% | 100% | -26.67% | -23.81% | -24.27% | 1004.5% | 1549.0% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.29% | 3.37% | 4.21% | 91.6% | 99.2% | 100% | -26.69% | -28.45% | -30.92% | 1025.1% | 1436.3% |
| Class D | 0.81% | 2.17% | 2.42% | 92.5% | 98.2% | 100% | -27.47% | -23.77% | -24.77% | 910.5% | 1484.5% |
| Class F | 4.71% | 5.14% | 5.47% | 90.3% | 91.3% | 100% | -30.85% | -34.51% | -35.21% | 768.7% | 840.4% |
| Class TGM | 7.57% | 9.17% | 9.57% | 93.2% | 94.3% | 100% | -38.77% | -44.39% | -44.15% | 690.8% | 762.3% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.51% | 1.66% | 1.68% | 94.3% | 99.3% | 100% | -22.35% | -40.04% | -35.11% | 1011.8% | 1215.5% |
| Class C | 0.35% | 1.27% | 1.05% | 91.9% | 96.8% | 100% | -24.90% | -28.52% | -30.59% | 992.1% | 1462.7% |
| Class E | 0.75% | 2.37% | 1.88% | 94.8% | 98.6% | 100% | -22.15% | -27.88% | -27.04% | 953.6% | 889.4% |
| Overall (Ref) | 0.52% | 1.71% | 1.52% | 93.6% | 98.3% | 100% | -23.15% | -33.16% | -31.58% | 990.4% | 1195.7% |
| Class D | 0.36% | -0.40% | 0.37% | 94.2% | 96.6% | 100% | -26.48% | -30.51% | -31.92% | 936.4% | 1431.4% |
| Class F | 3.84% | 4.11% | 4.73% | 92.6% | 97.3% | 100% | -28.92% | -39.94% | -39.73% | 798.8% | 854.5% |
| Class TGM | 5.15% | 6.49% | 6.69% | 92.3% | 95.5% | 100% | -38.37% | -48.85% | -48.66% | 719.8% | 734.7% |

  - Group 4 off

Group 4 includes tools that require more processing on the neighbouring reconstructed samples than VVC.

The offgroup4.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.54% | 5.42% | 5.94% | 84.6% | 83.6% | 100% | -13.59% | -12.26% | -23.16% | 801.9% | 341.5% |
| Class A2 | 1.72% | 5.32% | 5.83% | 81.0% | 84.6% | 100% | -20.24% | -21.42% | -25.65% | 738.9% | 401.4% |
| Class B | 1.04% | 5.01% | 5.01% | 85.8% | 92.5% | 97% | -14.25% | -19.72% | -17.41% | 895.5% | 581.7% |
| Class C | 1.05% | 3.44% | 3.57% | 84.3% | 87.6% | 99% | -14.47% | -9.50% | -10.59% | 877.6% | 538.5% |
| Class E | 1.63% | 6.62% | 5.56% | 88.1% | 93.4% | 100% | -18.42% | -18.95% | -17.66% | 885.9% | 608.2% |
| Overall | 1.34% | 5.05% | 5.07% | 84.8% | 88.7% | 99% | -15.88% | -16.36% | -18.27% | 846.1% | 495.5% |
| Class D | 0.99% | 2.81% | 3.27% | 82.6% | 83.3% | 100% | -12.24% | -6.62% | -7.11% | 854.8% | 561.5% |
| Class F | 1.32% | 4.58% | 5.31% | 93.2% | 92.9% | 100% | -30.17% | -32.54% | -32.55% | 520.2% | 557.7% |
| Class TGM | 2.30% | 4.27% | 4.50% | 91.6% | 96.9% | 99% | -42.59% | -47.33% | -46.54% | 438.3% | 633.3% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.90% | 3.03% | 4.84% | 92.4% | 97.8% | 100% | -26.39% | -23.41% | -35.05% | 872.3% | 832.5% |
| Class A2 | 1.33% | 3.41% | 3.55% | 94.5% | 99.2% | 100% | -30.12% | -33.48% | -39.10% | 853.0% | 971.9% |
| Class B | 1.03% | 3.73% | 3.52% | 94.5% | 99.3% | 100% | -24.88% | -33.14% | -30.30% | 870.2% | 1154.3% |
| Class C | 1.18% | 1.42% | 2.03% | 95.5% | 98.9% | 100% | -26.59% | -23.75% | -24.35% | 1012.3% | 1270.2% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.30% | 2.91% | 3.39% | 94.3% | 98.9% | 100% | -26.69% | -28.76% | -31.42% | 902.8% | 1071.7% |
| Class D | 0.92% | 1.95% | 1.98% | 95.2% | 98.0% | 99% | -27.37% | -23.98% | -25.16% | 992.3% | 1394.1% |
| Class F | 1.26% | 2.40% | 2.92% | 96.5% | 99.8% | 100% | -32.97% | -35.96% | -36.54% | 789.0% | 765.5% |
| Class TGM | 0.88% | 1.64% | 1.57% | 96.5% | 100.0% | 100% | -42.59% | -48.20% | -48.21% | 678.4% | 666.6% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.09% | 1.04% | 1.22% | 97.4% | 99.8% | 100% | -21.90% | -40.41% | -35.37% | 990.2% | 1023.5% |
| Class C | 1.31% | 1.29% | 0.85% | 95.6% | 95.3% | 99% | -24.20% | -28.53% | -30.76% | 917.2% | 1099.0% |
| Class E | 1.05% | 3.38% | 2.02% | 97.9% | 96.3% | 100% | -21.92% | -27.20% | -26.91% | 935.4% | 680.9% |
| Overall (Ref) | 1.15% | 1.71% | 1.30% | 96.9% | 97.4% | 100% | -22.67% | -33.15% | -31.72% | 951.6% | 946.6% |
| Class D | 1.15% | -0.51% | -0.65% | 97.6% | 98.0% | 99% | -25.85% | -30.67% | -32.63% | 979.1% | 1199.2% |
| Class F | 1.39% | 1.12% | 0.97% | 97.2% | 95.3% | 101% | -30.47% | -41.41% | -41.76% | 768.4% | 680.1% |
| Class TGM | 0.44% | 1.07% | 1.19% | 97.0% | 97.2% | 99% | -41.11% | -51.25% | -51.08% | 729.6% | 628.7% |

  - Group 5 off

Group 5 includes tools which need large memory access.

The offgroup5.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-20.0 |  |  |  |  |  | Over VTM-11.0ecm19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 4.51% | 4.60% | 4.61% | 79.1% | 85.9% | 91% | -11.01% | -12.67% | -24.06% | 811.0% | 457.2% |
| Class A2 | 3.73% | 3.37% | 3.93% | 80.7% | 90.1% | 90% | -18.61% | -22.64% | -26.72% | 786.6% | 531.3% |
| Class B | 4.34% | 4.04% | 4.41% | 79.8% | 89.6% | 80% | -11.42% | -20.16% | -17.68% | 787.9% | 562.8% |
| Class C | 6.19% | 4.39% | 5.08% | 80.0% | 92.1% | 92% | -10.27% | -8.66% | -9.37% | 764.2% | 574.9% |
| Class E | 6.16% | 5.31% | 4.55% | 80.8% | 89.2% | 87% | -14.77% | -19.68% | -18.33% | 747.4% | 596.7% |
| Overall | 4.98% | 4.31% | 4.54% | 80.0% | 89.5% | 87% | -12.85% | -16.69% | -18.51% | 779.2% | 546.3% |
| Class D | 5.07% | 4.40% | 5.54% | 78.5% | 91.6% | 97% | -8.69% | -5.11% | -5.04% | 726.0% | 594.5% |
| Class F | 4.28% | 3.02% | 3.44% | 88.0% | 90.0% | 87% | -27.93% | -33.19% | -33.29% | 557.3% | 592.6% |
| Class TGM | 0.55% | 0.59% | 0.54% | 89.0% | 89.5% | 83% | -43.50% | -49.10% | -48.48% | 457.8% | 610.8% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20.0 |  |  |  |  |  | Over VTM-11.0ecm19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 3.88% | 2.72% | 3.37% | 93.5% | 92.7% | 96% | -24.97% | -23.50% | -35.79% | 1079.3% | 1117.1% |
| Class A2 | 3.41% | 2.17% | 2.80% | 94.5% | 92.9% | 97% | -28.71% | -34.24% | -39.48% | 1044.7% | 1344.7% |
| Class B | 4.15% | 2.75% | 3.40% | 92.7% | 93.4% | 98% | -22.57% | -33.69% | -30.25% | 860.8% | 1262.7% |
| Class C | 5.38% | 3.17% | 3.94% | 96.0% | 94.1% | 92% | -23.52% | -22.45% | -22.97% | 996.1% | 1468.8% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 4.27% | 2.74% | 3.42% | 94.1% | 93.4% | 95% | -24.53% | -28.76% | -31.26% | 973.4% | 1299.1% |
| Class D | 4.69% | 3.90% | 4.23% | 93.9% | 94.9% | 96% | -24.63% | -22.46% | -23.47% | 906.8% | 1575.9% |
| Class F | 3.15% | 2.01% | 2.17% | 88.1% | 92.2% | 89% | -31.50% | -35.95% | -36.75% | 717.2% | 803.4% |
| Class TGM | 0.94% | 0.85% | 0.84% | 87.1% | 84.4% | 88% | -42.51% | -48.60% | -48.56% | 608.8% | 619.6% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20.0 |  |  |  |  |  | Over VTM-11.0ecm19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 3.50% | 0.75% | 1.69% | 93.6% | 89.9% | 81% | -20.06% | -40.54% | -35.01% | 921.8% | 1077.4% |
| Class C | 4.90% | 1.92% | 2.27% | 95.5% | 91.2% | 90% | -21.53% | -28.12% | -29.80% | 906.6% | 1193.2% |
| Class E | 4.14% | 2.55% | 2.31% | 88.9% | 91.3% | 93% | -19.54% | -27.77% | -26.80% | 810.6% | 748.1% |
| Overall (Ref) | 4.13% | 1.59% | 2.04% | 93.0% | 90.7% | 87% | -20.42% | -33.21% | -31.22% | 887.7% | 1017.6% |
| Class D | 4.44% | 0.34% | 0.35% | 93.9% | 92.3% | 96% | -23.47% | -29.94% | -31.92% | 884.3% | 1350.7% |
| Class F | 3.32% | 0.66% | 0.98% | 87.2% | 89.6% | 87% | -29.05% | -41.62% | -41.68% | 710.9% | 783.4% |
| Class TGM | 0.96% | 0.92% | 0.89% | 86.4% | 86.3% | 85% | -40.79% | -51.36% | -51.23% | 613.3% | 620.8% |

  - Group 1-5 off

In this test, all the tools in the group 1-5 are switched off. The offgroup1-5.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 9.78% | 19.62% | 25.23% | 38.4% | 41.1% | 90% | -6.52% | -1.08% | -10.40% | 363.7% | 167.8% |
| Class A2 | 11.39% | 22.40% | 23.27% | 38.0% | 36.8% | 90% | -12.57% | -9.48% | -14.42% | 346.2% | 174.8% |
| Class B | 10.47% | 21.12% | 20.45% | 37.4% | 35.6% | 83% | -6.24% | -8.38% | -5.63% | 389.8% | 223.6% |
| Class C | 11.64% | 14.11% | 15.49% | 37.0% | 36.9% | 96% | -5.73% | -0.41% | -0.55% | 385.8% | 226.5% |
| Class E | 15.79% | 23.47% | 20.17% | 39.0% | 38.4% | 96% | -7.05% | -6.61% | -6.25% | 392.4% | 249.9% |
| Overall | 11.65% | 19.92% | 20.57% | 37.8% | 37.4% | 90% | -7.36% | -5.28% | -6.86% | 377.3% | 209.0% |
| Class D | 9.21% | 12.24% | 13.79% | 36.7% | 34.9% | 97% | -5.11% | 1.93% | 2.37% | 379.6% | 235.1% |
| Class F | 19.31% | 24.67% | 26.59% | 50.2% | 37.8% | 95% | -18.05% | -20.53% | -19.72% | 279.9% | 227.0% |
| Class TGM | 20.44% | 25.15% | 26.43% | 55.1% | 36.1% | 87% | -32.63% | -36.99% | -35.62% | 263.8% | 235.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 16.25% | 16.80% | 25.10% | 53.7% | 57.1% | 94% | -16.07% | -13.42% | -23.40% | 507.1% | 486.6% |
| Class A2 | 16.04% | 19.97% | 20.90% | 53.9% | 55.6% | 94% | -20.03% | -23.14% | -28.93% | 486.9% | 544.9% |
| Class B | 14.53% | 21.94% | 22.29% | 49.4% | 56.7% | 96% | -14.81% | -21.57% | -17.73% | 454.8% | 658.8% |
| Class C | 15.24% | 15.73% | 17.32% | 47.6% | 51.6% | 90% | -16.33% | -13.01% | -12.97% | 504.7% | 663.1% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 15.36% | 18.86% | 21.25% | 50.6% | 55.2% | 93% | -16.51% | -17.97% | -19.83% | 484.4% | 598.0% |
| Class D | 12.20% | 14.33% | 15.62% | 45.8% | 51.0% | 95% | -19.13% | -14.61% | -14.97% | 477.6% | 725.6% |
| Class F | 19.13% | 21.10% | 22.55% | 52.5% | 61.0% | 89% | -21.24% | -24.68% | -24.85% | 429.6% | 467.4% |
| Class TGM | 20.83% | 25.47% | 26.04% | 54.8% | 55.1% | 89% | -31.33% | -36.30% | -36.03% | 385.1% | 367.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-20 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 13.38% | 13.24% | 15.01% | 43.2% | 45.0% | 79% | -12.39% | -33.23% | -26.64% | 439.8% | 461.3% |
| Class C | 14.52% | 13.21% | 13.96% | 42.6% | 40.3% | 89% | -14.39% | -20.33% | -21.79% | 408.7% | 465.3% |
| Class E | 14.34% | 16.62% | 14.24% | 45.9% | 56.8% | 84% | -11.63% | -18.17% | -18.31% | 438.3% | 401.5% |
| Overall (Ref) | 14.00% | 14.08% | 14.47% | 43.7% | 46.0% | 83% | -12.87% | -25.16% | -22.94% | 428.8% | 446.8% |
| Class D | 12.26% | 8.61% | 11.92% | 41.1% | 41.6% | 95% | -17.69% | -24.13% | -23.94% | 412.4% | 509.7% |
| Class F | 18.28% | 17.09% | 18.05% | 49.8% | 52.7% | 87% | -18.94% | -32.39% | -32.02% | 393.9% | 376.0% |
| Class TGM | 19.38% | 25.26% | 26.77% | 55.5% | 46.2% | 85% | -30.06% | -40.05% | -39.56% | 417.7% | 298.7% |

- Issues

  - Open issues

- #91, MaxTU setting

- #80, Compilation Issue ECM14.0 when disabling GPM reordering

- #78, Encoder crash in RA with --LMChroma=0

- #71, Encode/decode mismatch when using single tree

- #66, Difference in encoding results of ECM-12.0 in AhG7 group 2 tool off test

- #65, Different encoding results of ECM12 in AHG7 group1-4 off tests

- #64, Encode/decode mismatch and decoder crash when inter-CCCM is disable

- #53, Decoding mismatch was observed when AMVR is off

- Input contributions

| JVET-AQ0152 | m77432 | Runtime and Energy Consumption Estimation for Neural Network-based In-loop Filters in NNVC | S. Caglar, Z. Chen, M. Y. Saritas, H. B. Dogaroglu, C. Eteke, E. Steinbach (TU Munich) |
| --- | --- | --- | --- |
| JVET-AQ0204 | m77546 | How to calculate average PSNR for a video sequence | J. Samuelsson-Allendes, S. Deshpande (Sharp) |

- Recommendations

- Resolve identified software issues related to the tool assessment

- Review all the input documents

JVET-AQ0008 JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) [S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs)]

Activities

The AHG 8 used the main JVET reflector, jvet@lists.rwth-aachen.de, for email discussion. Between the last and this meeting, there was no AHG8 email sent to the main reflector. There is no AHG8 input contribution to this meeting outside this report. The ISO/IEC TR 23888-3:2026 Information technology — Artificial intelligence for multimedia, Part 3: Optimization of encoders and receiving systems for machine analysis of coded video content (Edition 1) has been published. The corresponding ITU-T H.Sup twin text will be submitted to SG21 by July 15.

AHG can be closed.

JVET-AQ0009 JVET AHG report: SEI message studies (AHG9) [J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs)]

Related contributions

As of Thursday July 2, a total of 119 contributions (not including this AHG report) were identified relating to the mandates of AHG9. Some contributions relate to more than one mandate and/or SEI message.

Study the SEI messages in VSEI, VVC, HEVC and AVC (3)

JVET-AQ0061, AHG9: Errata report on VSEI and HEVC, Y. He, M. Karczewicz (Qualcomm), Y.-K. Wang (Bytedance)

JVET-AQ0089, AHG2/AHG9: Errata on general SEI signalling mechanism for VVC, VSEI, HEVC, and AVC, Y.-K. Wang, J. Xu (Bytedance)

JVET-AQ0093, AHG2/AHG9: On the FPA and CTI SEI messages, J. Xu, Y.-K. Wang (Bytedance)

Study JVET-AO2032 TuC for VSEI (57)

General / multiple SEI messages (4)

JVET-AQ0072, AHG9: Editorial improvements for TuC, X. Xu, S. Liu (Tencent)

JVET-AQ0075, AHG9: On video full range indication, X. Xu, S. Liu (Tencent)

JVET-AQ0105, AHG9: On miscellaneous aspects of SEI messages in TuC, C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)

JVET-AQ0163, AHG9: on SeiExtenionEnableFlag Interface for SII, PRI, EOI, S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

Alpha channel information SEI message (1)

JVET-AQ0129, AHG9: On the Alpha Channel Information SEI message, C.-H. Demarty, A. Ak, E. François, P. de Lagrange, F. Urban, N. Caramelli (InterDigital)

Neural-network post-filter (NNPF) SEI messages (6)

JVET-AQ0052, AHG9: On backward compatibility of NNPF alternative temporal extrapolation versions, M. M. Hannuksela, F. Cricri (Nokia)

JVET-AQ0053, AHG9: On multilayer NNPF, M. M. Hannuksela, J. Boyce (Nokia)

JVET-AQ0058, AHG9: On NNPF Multiple Inferences Information Signalling, S. Deshpande (Sharp)

JVET-AQ0059, AHG9: Bug Fixes for Neural-network Post-filter in TuC, S. Deshpande (Sharp)

JVET-AQ0155, AHG9: On NNPFC updates, T. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (HHI)

JVET-AQ0157, AHG9: On NNPFC framework formats and model parameter compression, R. Skupin, T. Borges, Y. Sanchez, C. Hellge, T. Schierl (HHI)

SEI processing order (SPO) SEI message (2)

JVET-AQ0099, AHG9: On VSEI version indication in SPO SEI message, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AQ0158, AHG9: On detection of SEI extensions in SPO, T. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (HHI)

Encoder optimization information (EOI) SEI message (1)

JVET-AQ0104, AHG9: On spatial quality optimization in EOI SEI message, C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)

Modality information (MI) SEI message (2)

JVET-AQ0106, AHG9: On multi-layer spectral representations in modality information SEI message, C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)

JVET-AQ0109, AHG9: On the modality information SEI message extension, J. Xu, Y.-K. Wang (Bytedance)

Digitally signed content (DSC) SEI messages (8)

JVET-AQ0044, AHG9: Proposed clarifying improvements to subpicture DSC, I. Sodagar (Dolby)

JVET-AQ0055, AHG9: On subpicture mapping of the DSCS SEI message, M. M. Hannuksela, J. Boyce (Nokia)

JVET-AQ0056, AHG9: On subpicture support of the DSC SEI messages, M. M. Hannuksela, J. Boyce (Nokia)

JVET-AQ0063, AHG9: On the DSC SEI message interface text, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AQ0114, AHG9: On DSCS SEI message in VSEI v4, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AQ0116, AHG9: On DSCI SEI message in TuC, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AQ0117, AHG9: On DSCS SEI message in TuC, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AQ0118, AHG9: On non-VCL NAL units signing in DSC SEI messages in TuC, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

Film grain regions characteristics (FGRC) SEI message (1)

JVET-AQ0092, AHG9: On the FGRC SEI message, Y.-K. Wang, J. Xu (Bytedance)

Constituent rectangles (CR) SEI message (1)

JVET-AQ0096, AHG9: On the CR SEI message, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

Quality metrics (QM) SEI message (6)

JVET-AQ0073, AHG9: On Quality Metric SEI message, X. Xu, S. Wenger, S. Liu (Tencent)

JVET-AQ0108, AHG9: On quality metrics SEI message, J. Nam, H. Tan, C. Kim, J. Lee, J. Lim, S. Kim (LGE)

JVET-AQ0123, AHG9: On reference and source in QM SEI, T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AQ0124, AHG9: On delayed transmission of QM SEI, T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AQ0125, AHG9: On QM-SEI and SVTA-MQA, T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AQ0130, AHG9: On the QM SEI message, C.-H. Demarty, A. Ak, R. James (InterDigital)

Display Overlays Information (DOI) SEI message (1)

JVET-AQ0097, AHG9: On the DOI SEI message, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

Display rectangles (DR) SEI message (2)

JVET-AQ0107, AHG9: On display rectangles SEI message, J. Nam, H. Tan, C. Kim, J. Lee, J. Lim, S. Kim (LGE)

JVET-AQ0127, AHG9: On miscellaneous aspects of display rectangles SEI message, H. Tan, J. Nam, J. Lee, C. Kim, J. Lim, S. Kim (LGE)

Picture segmentation information (PSI) SEI message (2)

JVET-AQ0076, AHG9: On PSI SEI message, X. Xu, S. Liu (Tencent)

JVET-AQ0161, AHG9: on PSI SEI, S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

Danmu information SEI message (3)

JVET-AQ0074, AHG9: On Danmu Information SEI message, X. Xu, S. Wenger, S. Liu (Tencent)

JVET-AQ0126, AHG9: On Danmu SEI, T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia), C. Kim, H. Tan (LGE)

JVET-AQ0144, AHG9: On Danmu Information SEI, S. Xie, Y. Gao, S. Wang, W. Niu, Y. Bai (ZTE)

Auxiliary sampling alignment information (ASAI) SEI message (1)

JVET-AQ0087, AHG9: On the ASAI SEI message, Y. He, M. Karczewicz (Qualcomm), J. Chen, Y. Ye (Alibaba)

Localization and mapping (LAM) SEI message (1)

JVET-AQ0119, AHG9: On LAM SEI message, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

Sample interleaving SEI message (1)

JVET-AQ0141, AHG9: Fixes to sample interleaving SEI message, J. Chen, Y. Ye (Alibaba)

Gain map (GM) SEI message (4)

JVET-AQ0077, AHG9: On Gain map SEI, S. Wang, S. Xie, Y. Gao, Y. Bai(ZTE)

JVET-AQ0078, AHG9: On Gain map SEI, S. Wang, S. Xie, Y. Gao, Y. Bai (ZTE)

JVET-AQ0110, AHG9: On the gain map information SEI message, J. Xu, Y.-K. Wang (Bytedance)

JVET-AQ0162, AHG9: on SDI and GM SEI, S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

Generalized alpha plane information SEI message (2)

JVET-AQ0098, AHG9: On the generalized alpha plane information SEI messages, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AQ0112, AHG9: On the generalized alpha plane information SEI message, J. Xu, Y.-K. Wang (Bytedance)

Generalized depth information SEI message (2)

JVET-AQ0111, AHG9: On the generalized depth information SEI message, J. Xu, Y.-K. Wang (Bytedance)

JVET-AQ0165, AHG9: on NADPI BitDepthY Variable, S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

VSEI RBSP (3)

JVET-AQ0057, AHG9: On indicating necessity of SEI message extensions, M. M. Hannuksela, J. Boyce (Nokia)

JVET-AQ0090, AHG9: On SEI position, payload type, and payload size signalling in the general SEI signalling mechanism in VSEI TuC, Y.-K. Wang, J. Xu (Bytedance)

JVET-AQ0091, AHG9: On SEI payload size semantics and its impacts in the general SEI signalling mechanism in VSEI TuC, Y.-K. Wang, J. Xu, K. Zhang, L. Zhang (Bytedance)

Collect software and showcase information for SEI messages (8)

JVET-AQ0046, AHG9: Implementation and showcase of the luma adaptation extension of the EOI SEI message, H. Zhang, M. M. Hannuksela (Nokia)

JVET-AQ0060, AHG9: Showcase for Photosensitive Content Information SEI, S. Deshpande (Sharp)

JVET-AQ0115, AHG9: Showcase for DSC SEI messages for subpicture-based signing in TuC, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AQ0128, AHG9: Software implementation of luma adaptation optimization in the EOI SEI message and showcase for display power adaptation, C.-H. Demarty, A. Ak, F. Aumont (InterDigital)

JVET-AQ0140, AHG9: A showcase for picture quality indication in EOI SEI message extension, B. Chen, J. Chen, Y. Ye (Alibaba), S. Yin, X. Ju, S. Wang (CityUHK)

JVET-AQ0156, AHG9: Proposal for a showcase playground, E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)

JVET-AQ0180, AHG9: Showcase for infrared camera adaptation parameters SEI, V. Zakharchenko (Nokia)

JVET-AQ0181, AHG9: Showcase for auxiliary sampling alignment information SEI message, V. Zakharchenko (Nokia)

Generation of VSEI v5 working draft (12)

JVET-AQ0051, AHG9: Encoder optimization information (EOI) SEI message extension for VSEI version 5, M. M. Hannuksela, J. Boyce (Nokia), J. Chen, Y. Ye (Alibaba), C.-H. Demarty, A. Ak, F. Aumont (InterDigital), X. Xu, S. Wenger, G. Teniou (Tencent)

JVET-AQ0054, AHG9: Neural-network post-filter (NNPF) extension for VSEI version 5, M. M. Hannuksela, F. Cricri, J. Boyce (Nokia)

JVET-AQ0062, AHG9: SPO SEI message extension for VSEI v5, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AQ0064, AHG9: proposing SII extension for VSEIv5, L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), J. Xu, Y.-K. Wang (Bytedance)

JVET-AQ0065, AHG9: proposing LAM for VSEIv5, L. Kerofsky, Y. He, Z. ZHao, M. Karczewicz (Qualcomm), G. Teniou (Tencent)

JVET-AQ0066, AHG9: proposing LOC for VSEIv5, L. Kerofsky, Y. He, Z. ZHao, M. Karczewicz (Qualcomm), G. Teniou (Tencent)

JVET-AQ0086, AHG2/AHG9: Initial proposed text for a potential WD of VSEI v5, Y.-K. Wang, J. Xu, L. Zhang, K. Zhang (Bytedance), S. Deshpande, J. Samuelsson-Allendes (Sharp), Y. He, L. Kerofsky (Qualcomm), A. M. Tourapis, D. Podborski, J. Kim (Apple), E. François, C.-H. Demarty, A. Ak (InterDigital), S. Wenger, G. Teniou (Tencent), H. Tan (LGE)

JVET-AQ0159, AHG9: MI Multispectrum Extension for VSEIv5, S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AQ0160, AHG9: GM SEI for VSEIv5, S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AQ0175, AHG9: Display overlays info (DOI) SEI message for VSEI v5 WD, J. Boyce (Nokia)

JVET-AQ0176, AHG9: Display rectangles (DR) SEI message for VSEI v5 WD, J. Boyce (Nokia)

JVET-AQ0177, AHG9: Bitdepth range info (BRI) SEI message for VSEI v5 WD, J. Boyce (Nokia)

Identify potential needs for additional SEI messages, including study of AVC and HEVC SEI messages for use in VVC (11)

JVET-AQ0122, AHG9: QR Code Information SEI, T. Biatek, S. He, D. Fortin, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AQ0135, AHG9: Support of regions of update and update priority SEI message, S. He, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AQ0136, AHG9: Composited Pictures Information SEI, S. He, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AQ0139, AHG9: Historical Information SEI message, J. Pardo, T. Solovyev, E. Alshina (Huawei), X. Xu, S. Liu (Tencent)

JVET-AQ0145, AHG9: IR camera adaptation parameters SEI, V. Zakharchenko, J. Boyce, M. Hannuksela (Nokia)

JVET-AQ0147, AHG9: Layer priority SEI message, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AQ0149, AHG9: Classification label info SEI, J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)

JVET-AQ0154, AHG9: Proposed GDR Info SEI message, S. Mate, M. M. Hannuksela, J. Boyce (Nokia)

JVET-AQ0166, AHG9: C2PA SEI Message, S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AQ0179, AHG9: Processing History Information SEI, C. Feldmann, T. Biatek, J. Boyce (Nokia)

JVET-AQ0188, Subpicture support in HEVC, A. M. Tourapis, D. Podborski, J. Kim, S. Paluri (Apple)

Coordinate with the joint AHG on Gaussian splat coding about aspects related to the usage of SEI messages (28)

JVET-AQ0068, AHG9: [GSC][JEE6.7] TV-GS: triplane video-based implicit Gaussian splatting representation compression, B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)

JVET-AQ0069, AHG9: [GSC] Coordinate-remapping in triplane video-based implicit Gaussian splatting framework, B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)

JVET-AQ0070, AHG9: [GSC] Improved SEI message for triplane video-based implicit Gaussian splatting representation, B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)

JVET-AQ0079, AHG9: Editorial changes for the GSI SEI message in VSEI TuC, Y.-K. Wang, Y. Xu, J. Xu, Q. Yang (Bytedance)

JVET-AQ0080, AHG9: On coding of some syntax elements in the GSI SEI message, Y.-K. Wang, J. Xu, Y. Xu, Q. Yang, K. Zhang (Bytedance)

JVET-AQ0081, AHG9: On persistence etc. and error resilience of the GSI SEI message, Y.-K. Wang, J. Xu, Y. Xu, Q. Yang, K. Zhang (Bytedance)

JVET-AQ0082, AHG9: On value ranges and decoder handling of reserved values for syntax elements in the GSI SEI message, Y.-K. Wang, J. Xu, Q. Yang, Y. Xu, K. Zhang (Bytedance)

JVET-AQ0083, AHG9: Non-uniform quantization for scale and opacity support in the GSI SEI message, Y. Xu, Q. Yang, Y.-K. Wang, J. Xu, K. Zhang, L. Zhang (Bytedance)

JVET-AQ0084, AHG9: Patch quantization offset and LoD information in the GSI SEI message, Y. Xu, Y.-K. Wang, J. Xu, K. Zhang, Q. Yang, L. Zhang (Bytedance)

JVET-AQ0085, AHG9: On semantics of some syntax elements in the GSI SEI message, Y. Xu, Q. Yang, J. Xu, Y.-K. Wang, K. Zhang, L. Zhang (Bytedance)

JVET-AQ0088, AHG9: On the GSI SEI message syntax and semantics, Y. He, J. Jung, R. Hooda, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AQ0094, AHG9: On SH non-uniform quantization signalling in the GSI SEI message, H. Lee, J. Do, G. Bang (ETRI)

JVET-AQ0095, AHG9: On unified signalling of SH transforms in the GSI SEI message, H. Lee, J. Do, G. Bang (ETRI)

JVET-AQ0100, AHG9: On the GSI SEI message for spatial random access, Y. He, J. Jung, Y. Shao, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AQ0102, AHG9: Syntax and semantics clean-up of the GSI SEI message, J. Do, H. Lee, G. Bang (ETRI)

JVET-AQ0113, AHG9: On Gaussian Splatting Information SEI message enhancements, Y. Hu, X. Ma (MLS Labs), Y. Xu, K. Yang (SJTU)

JVET-AQ0131, AHG9: On the GSI SEI message, S. Harribey, M. Pettersson, M. Damghanian, R. Sjöberg (Ericsson)

JVET-AQ0132, AHG9: On signalling of various parameters in the GSI SEI message, S. Harribey, M. Pettersson, M. Damghanian, R. Sjöberg (Ericsson)

JVET-AQ0142, AHG9: [GSC-related] An animation-based 3DGS Avatar Coding, Z. Zhang, S. Yin, X. Wu, S. Wang (CityUHK), B. Chen, J. Chen, R.-L. Liao, Y. Yan (Alibaba), A. Trioux, F. Yang (Xidian Univ), C. Wang (PCL), R. Wang (PKUSZ)

JVET-AQ0143, AHG9: [GSC-related] Avatar Generation Information SEI message, J. Chen, B. Chen, Y. Yan (Alibaba), S. Yin, Z. Zhang, S. Wang (CityUHK), A. Trioux, F. Yang (Xidian Univ.)

JVET-AQ0148, AHG9: Support for multiple bitstreams and alternative splat types in Gaussian splatting info SEI, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AQ0164, AHG9: Cross-platform determinism and bit-exact conformance of the GSC V-SEI framework, J. Ricard, G. Teniou, S. Wenger (Tencent)

JVET-AQ0167, AHG9: Integer-only GSC V-SEI decoder, J. Ricard, G. Teniou, S. Wenger (Tencent)

JVET-AQ0168, AHG9: High-level syntax modifications and bitstream specification for an integer-based GSC V-SEI decoder pipeline, J. Ricard, G. Teniou, S. Wenger (Tencent)

JVET-AQ0169, AHG9: Single-video GSC V-SEI framework, J. Ricard, G. Teniou, S. Wenger (Tencent)

JVET-AQ0170, AHG9: Multi-frame improvement of the GSC V-SEI framework, J. Ricard, G. Teniou, S. Wenger (Tencent), J. Jung (Qualcomm)

JVET-AQ0174, AHG9: On the Gaussian splatting information SEI message, R. Hooda, J. Jung, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AQ0184, AHG9: [GSC][JEE6.7][JEE6.9] On the Implicit GSI SEI message, Y. Sanchez, S. Sasse, T. M. Borges, S. Lee, Y. Berendsohn, R. Skupin, C. Hellge, T. Schierl

Activities

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de) with [AHG9] in message headers. No emails with [AHG9] were exchanged.

Git projects were set up for reporting and tracking of specification bugs for VSEI and the TuC, at the links below. It is encouraged to submit an issue to report any specification bugs.

- VSEI: https://vcgit.hhi.fraunhofer.de/jvet/publications/VSEI

- TuC: https://vcgit.hhi.fraunhofer.de/jvet/publications/VSEI-TuC

Recommendations

The AHG recommended to:

- Review all related contributions

- Output a working draft for a version 5 of VSEI from this meeting

- Continue SEI messages studies

J. Boyce and S. Deshpande to co-chair the HLS track, starting from Wednesday.

JVET-AQ0010 JVET AHG report: Encoding algorithm optimization (AHG10) [K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice-chairs)]

Related contributions

Following contribution was identified relating to AHG10 and summarized in the following section.

JVET-AQ0194: AHG10/AHG12: Clean-Slate NextSoftware2 implementation without future Video Coding Tools

This document proposes a software without new normative tools (Clean-Slate NextSoftware2) of the CfE proposal by Ericsson, Fraunhofer HHI and Nokia (JVET-AN0267). The proponents of this software indicate that in comparison to VTM this software have less restrictions. The BD-rate impact in comparison to VTM-23.14 is:

- All-intra (Y/U/V): 1,66%/-1,82%/-1,21% EncT 54,9% DecT 72,5%

- Random-Access (Y/U/V): 0,69%/-0,51%/-0,04% EncT 48,3% DecT 82,1%

- Low delay B (Y/U/V): 0,34%/2,43%/2,03%	EncT 52,8% DecT 80,0%

When also enabling BIM=2 (lambda-based BIM) and GOPBasedRPR on VTM the BD-rate impact is:

- Random-Access (Y/U/V): 2,50%/1,93%/2,44% EncT 50,4%	DecT 92,2%

- Low delay B (Y/U/V): 1,80%/0,81%/1,27%	EncT 52,9% DecT 81,9%

The document proposes to include the clean-slate version of the NextSoftware2 into the “JVET AHG Enhanced compression beyond VVC capability” repository for evaluation and future consideration.

Recommendations

The AHG recommends that the related input contribution is reviewed, and to further continue the study of encoding algorithm optimizations in JVET.

JVET-AQ0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien (vice-chairs)]

Teleconferences

The AHG conducted two joint teleconferences with AHG14 and EE1 during the interim period. The teleconferences were held on May 25th and June 9th. In those teleconferences, the following topics were discussed:

- NNVC17.0 software integration status and anchor performance

- EE1 tests status

- Clarification on NNVC CTC document

- Bit exact reproducibility, JVET-AP0234

- NNVC17.1 software release

- EE1 tests final design

- MPEG survey on AI hardware

It was agreed that NNVC17.1 should be a basis for EE1 tests, SW branches for EE1 test have been created, deadline for EE1 SW freeze was June 16.

Details are summarized in the AHG11 & AHG14 teleconference report JVET-AQ0042.

Common Test Conditions

Anchor for the NN-based video coding activity was made available though the Git repository used for the AHG activity:

https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/blob/master/Anchor%20performance/NNVC_anchor_performance_NNVC-17-VTM_vs_NNVC-17.xlsm

Anchor performance data also distributed by AhG14 in JVET-AO0014 and announced during intermediate AhG meeting.

Training data and NNVC CTC

Revised at JVET 42nd meeting list training sets has been made available as

https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/blob/master/NNVC_training_sets.xlsx?ref_type=heads

NNVC common test conditions document has been up-dated accordingly and uploaded as JVET-AP2016 on May 28.

JPEG AI data sets (training and validation) have been made available to JVET members at https://vqa.lfb.rwth-aachen.de/index.php/apps/files/files/1782294?dir=/jvet/ahg/trainingset/JVET-AP0228.

Bit-exact reproducibility

In case of overflow (which is not happening under NNVC CTC) NNVC tools might produce different results on different platforms since handling of overflow in not defined. In one of EE1 tests de-scaling shifts and clipping are explicitly introduced for NNIP (NN-based Intra prediction) and mathematical proof for no overflow of 32 bits register is provided. This way a bit-exact behavior is guaranteed (assuming the shifts and the clipping are described in the specification and implementors follow the specification). An impact to the performance and implementation details will be discussed in of EE1 report.

EE Coordination

The AHG finalized, conducted, and discussed the EE on NN based video coding. A summary report for the EE is available in JVET-AQ0023.

EE1 related contributions and cross-checks are summarized in EE1 summary report. Potential performance improvement provided by EE1 tests is 0.7% gain from new version of LOP NN-based in-loop filter and 0.5% gain from DRF improvements.

Performance Evaluation

The performance and complexity of NN-based tools available in NNVC SW is summarized in the table below. All test data provided by AhG14. Encoding and decoding run time is very dependent on cluster used for simulation. Run time data in this table are all from InterDigital, except content adaptive tools results that are from Nokia.

There were no changes in default configuration of NNVC SW. The main reason for decrease of decoding run time relatively to NNVC configured as VTM is SADL optimization.

Test results for tools combination though the history of NNVC tools development

| Test vs NNVC (configured as VTM) | Random Access cfg |  |  |  |  | kMAC/pxl |  |  |  | Param (Mprm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Total | Filter | Intra | SR or DRF or E2E | Total | Filter | Intra | SR or DRF or E2E |
| NN-Intra & LOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-17.1 (LOP7) | -8.2% | -15.1% | -13.9% | 1.2 | 20 | 21.3 | 16.5 | 4.8 | 0 | 1.5 | 0.24 | 1.3 | 0 |
| NNVC-16.0 (LOP7) | -8.2% | -15.1% | -13.9% | 1.2 | 23 | 21.3 | 16.5 | 4.8 | 0 | 1.5 | 0.24 | 1.3 | 0 |
| NNVC-15.0 (LOP6) | -8.0% | -14.9% | -13.5% | 1.2 | 39 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NNVC-14.0 (LOP6) | -8.0% | -14.9% | -13.5% | 1.2 | 27 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NN-Intra & HOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-14.0 (HOP5) | -14.1% | -19.8% | -20.1% | 3.4 | 1180 | 471 | 466 | 4.8 | 0 | 2.7 | 1.41 | 1.3 | 0 |
| NN-Intra & VLOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-16.0 (VLOP4) | -6.1% | -7.6% | -6.1% | 1.2 | 15 | 9.9 | 5.1 | 4.8 | 0 | 1.3 | 0.07 | 1.3 | 0 |
| External E2E AI coded picture (1 tool)* |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-16.0 (DCVC-RT) | -1.7% | -13.7% | -10.2% | 1.0 | 5.3 | 365 /23 | 0 | 0 | 365/ 23 | 36.4 | 0 | 0 | 36.4 |
| NN-Intra & LOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC 16.0 caLOP4 | -8.5% | -18.3% | -17.8% | 3.3 | 21 | 22 | 17.2 | 4.8 | 0 | 1.6 | 0.25 | 1.3 | 0 |
| NNVC 15.0 caLOP4 | -8.5% | -18.0% | -17.5% | 2.3 | 38 | 22 | 17.2 | 4.8 | 0 | 1.6 | 0.25 | 1.3 | 0 |
| NNVC 14.0 caLOP4-decp | -8.6% | -15.7% | -16.3% | 2.3 | 25 | 22 | 17.2 | 4.8 | 0 | 1.6 | 0.25 | 1.3 | 0 |
| NN-Intra & VLOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-16.0 caVLOP4 | -6.9% | -8.5% | -7.2% | 3.2 | 11 | 10.2 | 5.4 | 4.8 | 0 | 1.3 | 0.02 | 1.3 | 0 |
| NNVC-15.0 caVLOP4 | -6.9% | -8.5% | -7.2% | 2.3 | 18 | 10.2 | 5.4 | 4.8 | 0 | 1.3 | 0.02 | 1.3 | 0 |
| NN-Intra & LOP filter & adaptive resolution coding (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-16.0 NNSR | -8.8% | -13.7% | -12.8% | 1.1 | 18 | 26.1 | 16.8 | 4.8 | 4.7 | 1.6 | 0.05 | 1.3 | 0.2 |
| NNVC-14.0 NNSR | -8.6% | -13.7% | -12.5% | 1.1 | 24 | 26.1 | 16.8 | 4.8 | 4.7 | 1.6 | 0.05 | 1.3 | 0.2 |
| NNVC-16.0 RPR | -8.7% | -14.4% | -13.4% | 1.1 | 18 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NNVC-14.0 RPR | -8.6% | -13.7% | -12.6% | 1.1 | 24 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NN-Intra & LOP filter & DRF (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-17.1 LDRF | -9.6% | -14.8% | -15.2% | 1.3 | 230 | 83 | 16.6 | 4.8 | 62 | 5.2 | 0.25 | 1.3 | 3.6 |
| NNVC-16.0 LDRF | -9.6% | -14.8% | -15.2% | 1.7 | 278 | 83 | 16.6 | 4.8 | 62 | 5.2 | 0.25 | 1.3 | 3.6 |
| NNVC-16.0 HDRF | -10.6% | -16.9% | -15.1% | 1.7 | 869 | 477 | 16.6 | 4.8 | 420 | 7.7 | 0.25 | 1.3 | 6.1 |
| NNVC-15.0 DRF | -10.0% | -16.2% | -15.4% | 1.7 | 869 | 477 | 16.6 | 4.8 | 420 | 7.7 | 0.25 | 1.3 | 6.1 |
| NNVC-14.0 DRF | -9.8% | -15.9% | -14.2% | 1.7 | 604 | 477 | 16.6 | 4.8 | 420 | 7.7 | 0.25 | 1.3 | 6.1 |

*external E2E AI coded picture is inserted only once during Intra period, so kMAC/pxl is different for ‘all Intra ‘ and ‘random access’ coding scenario.

NNVC implementation aspects and complexity

Three contributions are focusing on implementation aspects.

One contribution JVET-AQ0173 demonstrates real time coding (1080p 30-60 fps) on consumer devices (encoding on MacBook Pro, 14-inch, 2023, equipped with an M2 Pro processor, and decoding was performed on both the same MacBook Pro and an iPhone 15 Pro) for end-to-end AI-based video codec, using built-in neural accelerations as well as CPU for entropy part.

Further study of power consumption for NN-based video coding algorithms, including LOP7 NN-based in-loop filter is provided in JVET-AQ0152.

New EE1-related proposals

There are two proposals: one JVET-AQ0146 is improving DRF by taking into account Chroma location (chroma gain 0.1 for LDRF and more than 1% for HDRF as demonstrated), another is targeting complexity reduction of VLOP.

E2E AI video coding

As a continuation of JVET-AP0190 demonstration of early implementation of E2E AI video codec is provided in JVET-AQ0173. No test results under NNVC CTC provided but encoding and decoding speed are very promising. Only low-delay coding scenario is supported.

List of related contributions

In total 21 NNVC related contributions submitted for this meeting. Among them 3 reports, 6 documents describing EE1 tests, 10 cross-checks, 3 contributions on implementation aspects (including one E2E AI video codec), 2 new proposals improving or modifying NNVC tools, 1 contribution on training.

| JVET- number | Title | Authors |
| --- | --- | --- |
| Reports (3) |  |  |
| JVET-AQ0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators) |
| JVET-AQ0042 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| EE1 tests (6) |  |  |
| JVET-AQ0047 | EE1-2.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement | T. Shu, X. Chen, W. Zhang, N. Fu, Z. Chen (Wuhan Univ.) |
| JVET-AQ0048 | EE1-2.2: Improved H-DRF with Weighted Fusion and Optimized YUV Processing | X. Chen, J. Zhang, Z. Chen (Wuhan Univ.) |
| JVET-AQ0049 | EE1-2.3: Deep Reference Frame Generation for Inter Prediction Enhancement with motion compensation | P. Bordes, F. Galpin, F. Lo-Bianco, M. Paquiry (InterDigital) |
| JVET-AQ0050 | EE1-2.4: Combination of test 2.3 (Deep Reference Frame Generation with motion compensation) and tests 2.1 and 2.2 | P. Bordes, F. Galpin, F. Lo-Bianco, M. Paquiry (InterDigital), X. Chen, T. Shu, J. Zhang, Z. Chen (Wuhan Univ.) |
| JVET-AQ0071 | EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering | J. Lee, M. Jeon, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| JVET-AQ0182 | EE1-4.1: Adaptive Quantization and Hardware Optimization for NNIP | A. Akhtar, S. Esenlik, Y. Matsuba, M. Karczewicz (Qualcomm) |
| New technical proposals (2) |  |  |
| JVET-AQ0146 | EE1-related: chroma location information handling in DRF (EE1-2.3 and EE1-2.4) | F. Galpin, P. Bordes (InterDigital) |
| JVET-AQ0172 | AHG11: VLOP4 Modification for Complexity Reduction with Coding Parameter-guided Feature Modulation | H. Cho, J. Kim, S. Bahk, T. Lee, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI) |
| Implementation aspects (3) |  |  |
| JVET-AQ0151 | AHG11] [AHG14] Huawei response to MPEG survey on AI hardware | E. Alshina, Y. Zhao (Huawei) |
| JVET-AQ0152 | Runtime and Energy Consumption Estimation for Neural Network-based In-loop Filters in NNVC | S. Caglar, Z. Chen, M. Y. Saritas, H. B. Dogaroglu, C. Eteke, E. Steinbach (TU Munich) |
| JVET-AQ0173 | [AHG11][AHG 14] Real-Time Implementation of an End-to-End AI Video Codec | S. Cizel, R. Mullakhmetov, A. Berk, C. Finlay, F. Galpin, E. François (InterDigital) |
| Training (1) |  |  |
| JVET-AQ0200 | [AHG11] Training-Time Optimization for LOP7 | J. Lee, M. Jeon, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| Cross-checks (10) |  |  |
| JVET-AQ0134 | Crosscheck of JVET-AQ0049 (EE1-2.3: Deep Reference Frame Generation for Inter Prediction Enhancement with motion compensation) | X. Chen, Z. Chen (Wuhan Univ.) |
| JVET-AQ0137 | Crosscheck of JVET-AQ0047 (EE1-2.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) and JVET-AQ0048 (EE1-2.2: Improved H-DRF with Weighted Fusion and Optimized YUV Processing) | P. Bordes, F. Galpin (InterDigital) |
| JVET-AQ0178 | Crosscheck of JVET-AQ0071 (EE1-1.1.1 and EE1-1.1.2: Dynamic convolution for LOP7 neural in-loop filtering) | V. Khamidullin, S. Ikonin, E. Alshina (Huawei) |
| JVET-AQ0193 | Crosscheck of JVET-AQ0071 (EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering) | N. J. Gadgil, M. N. Hosmane, R. N. Gadde, W. I. Choi, K. P. Choi (Samsung) |
| JVET-AQ0205 | Cross-check of JVET-AQ0182 (EE1-4.1: adaptive quantization and hardware optimization for NNIP) | T. Dumas (InterDigital) |
| JVET-AQ0206 | Crosscheck of JVET-AQ0048 (EE1-2.2.2: Improved L-DRF with Weighted Fusion and Optimized YUV Processing) | F. Kozhamkulova, N. Bhaskar, T. Solovyev , E. Alshina (Huawei) |
| JVET-AQ0213 | Crosscheck of JVET-AQ0050 (EE1-2.4: Combination of test 2.3 (Deep Reference Frame Generation with motion compensation) and tests 2.1 and 2.2) | F. Kozhamkulova, N. Bhaskar, T. Solovyev, E. Alshina (Huawei) |
| JVET-AQ0216 | Crosscheck of JVET-AQ0048 (EE1-2.2.1: Improved H-DRF with Weighted Fusion and Optimized YUV Processing) | N. Bhaskar, F. Kozhamkulova, T. Solovyev (Huawei) |
| JVET-AQ0217 | Crosscheck of JVET-AQ0047 (EE1-2.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) | N. Bhaskar, T. Solovyev (Huawei) |
| JVET-AQ0218 | Cross-check of JVET-AQ0182 (EE1-4.1: adaptive quantization and hardware optimization for NNIP) | Aasaipriya Chandran (Huawei) |

Recommendations

The AHG recommended to:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.

- Continue study of the implementation aspects for hybrid codec with NN-based tools and End-to-End AI codec.

- Analyze device interoperability and bit-exact reconstruction for all NNVC algorithms.

- Recommend promising technologies for the next EE1 round (if any).

Plan review of 5.1/5.2 (EE1/NNVC, and ECM etc.) on Wednesday

JVET-AQ0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs)]

- Activities

The exploration experiment on enhanced compression beyond VVC capability (EE2) was finalized at the previous meeting, and the contributions adopted at that meeting were subsequently integrated into ECM-20.0. The combined improvements of ECM-20.0 over the VTM-11.0ecm20.0 anchor for the AI, RA, and LB configurations are summarized below:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -14.90% | -16.51% | -27.38% | 1214.6% | 584.7% |
| Class A2 | -21.55% | -25.07% | -29.33% | 1148.4% | 632.4% |
| Class B | -15.12% | -23.14% | -21.14% | 1116.1% | 694.7% |
| Class C | -15.34% | -12.36% | -13.56% | 1070.7% | 660.2% |
| Class E | -19.71% | -23.60% | -21.83% | 1013.8% | 648.9% |
| Overall | -16.97% | -20.04% | -21.97% | 1109.0% | 649.6% |
| Class D | -13.10% | -9.15% | -10.04% | 1019.5% | 696.0% |
| Class F | -31.06% | -35.35% | -35.73% | 661.7% | 686.8% |
| Class TGM | -43.82% | -49.39% | -48.74% | 552.7% | 800.5% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -27.75% | -25.60% | -37.70% | 1240.3% | 1262.7% |
| Class A2 | -31.04% | -35.61% | -41.12% | 1192.9% | 1441.3% |
| Class B | -25.64% | -35.41% | -32.48% | 1004.1% | 1331.1% |
| Class C | -27.44% | -24.79% | -25.87% | 1130.3% | 1502.4% |
| Class E |  |  |  |  |  |
| Overall | -27.62% | -30.66% | -33.49% | 1119.0% | 1382.2% |
| Class D | -28.06% | -25.46% | -26.60% | 1078.4% | 1593.9% |
| Class F | -33.74% | -37.37% | -38.28% | 848.0% | 911.7% |
| Class TGM | -43.06% | -49.02% | -48.99% | 746.7% | 781.3% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -22.74% | -40.91% | -36.05% | 1102.3% | 1259.7% |
| Class C | -25.17% | -29.43% | -31.33% | 1046.9% | 1332.2% |
| Class E | -22.73% | -29.46% | -28.52% | 1040.4% | 771.9% |
| Overall | -23.55% | -34.22% | -32.60% | 1068.0% | 1135.5% |
| Class D | -26.73% | -30.24% | -31.92% | 1036.2% | 1442.9% |
| Class F | -31.42% | -42.14% | -42.29% | 879.9% | 921.6% |
| Class TGM | -41.38% | -51.81% | -51.63% | 791.4% | 758.6% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-19.1’s {-27.55%, -30.58%, -33.31%} to ECM-20.0’s {-27.62%, -30.66%, -33.49%}. For SCC sequences (class TGM) the rate reduction for RA configuration changed from ECM-19.1’s -43.03%, -48.96%, -48.99%} to ECM-20.0’s { -43.06%, -49.02%, -48.99%}.

- Contributions

There are 2 AHG12-related contributions to this meeting:

JVET-AQ0192, "Improvements of NS2 software at 0.2x under CfP conditions",	G. Laroche, P. Onno, B. Galmiche, N. Ouedraogo (Canon)

JVET-AQ0194, "AHG10/AHG12: Clean-Slate NextSoftware2 implementation without future Video Coding Tools",	W. Ahmad, K. Andersson, L. Litwic, M. Pettersson, R. Sjöberg, N. Stegmaier, J. Ström, P. Wennersten (Ericsson), C. Bartnik, J. Brandenburg, B. Bross, V. George, J. Güther, G. Hege, C. Helmrich, A. Henkel, T. Hinz, C. Lehmann, D. Marpe, T. Nguyen, J. Pfaff, T. Schierl, H. Schwarz, B. Stallenberger, K. Sühring, A. Wieckowski, T. Wiegand (Fraunhofer HHI), P. Astola, S. Blasi, F. Cricri, D. Bugdayci Sansli, C. Feldmann, D. Fortin, J. Funnell, A. Hallapuro, M. M. Hannuksela, S. Hong, I. Jumakulyyev, J. Lainema, N. Le, N. Neumann, J. Ridge, D. Rusanovskyy, S. Schwarz, H. Zhang, N. Zou (Nokia)

JVET-AQ0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, P. de Lagrange (co-chairs), S. Deshpande, A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs)]

- Discussion

This period was focused on preparing the draft text of the second edition of ITU Rec. H.Sup 21 | TR 23002-9.

- Related contributions

Three contributions related to AHG13 were identified as of 04/23/26.

- One contribution was the AHG report:

  - JVET-AP0013 JVET AHG report: Film grain technologies (AHG13)

- Two other contributions were uploaded at the time of the report’s drafting:

  - JVET-AQ0211 AHG13: Film grain analysis and synthesis support in HDRTools

  - JVET-AQ0222 AHG13: proposed updates to the technical report on film grain synthesis technology for video applications

  - Contributions

There were two contributions registered other than the AHG report.  All were uploaded as of 07/07/26.

    - JVET-AQ0211 AHG9/AHG13: Film grain analysis and synthesis support in HDRTools

This contribution reports on film grain analysis and synthesis functionality that has been integrated into the HDRTools software (version 0.27-dev). HDRTools now provides a dedicated FilmGrainTool, as well as film grain synthesis within the HDRConvert conversion tool, supporting two different film grain models, the Film Grain Characteristics (FGC) and the AOMedia Film Grain Synthesis 1 (AFGS1) models. The functionality supports both the estimation (analysis) of film grain parameters from source and denoised content and the synthesis of film grain onto decoded or denoised content, with parameters carried in a human-readable JSON metadata format. This document describes the added functionality, the supported models, the analysis and synthesis pipelines, the JSON metadata format, and the associated configuration parameters.

    - JVET-AQ0222 AHG13: proposed updates to the technical report on film grain synthesis technology for video applications

At the 42nd JVET meeting, a request for 2nd edition of the technical report on film grain synthesis technology for video applications was issued, and a new draft has been output as JVET-AP2020. This contribution elements that are listed in the request but still missing in the draft.

Updates during the current meeting are expected to propose text to address the missing points.

- Recommendations

The AHG recommended to:

- Continue editing the second edition for the TR;

- input contributions (non-AHG9) to be reviewed;

- any liaisons to be reviewed;

- to continue conformance discussion;

- to consider any newly proposed SEI message extensions;

- to discuss the possibility of creating, as appropriate, documents providing Engineering Guideline (EG) or recommended practices (RP) relating to film grain technologies and their applications; and

- to continue the study of film grain technologies in JVET.

It was planned to issue a WG 5 CDTR from the current meeting, DTR from next meeting, and submit H.Sup.21 in April 2027.

JVET-AQ0014 JVET AHG report: NNVC software development (AHG14) [F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice-chairs)]

- Software development

  - Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM in the NNVC branch.

NNVC software is based on latest VTM, VTM-24.0 at the time of software integration.

Legacy version (up to NNVC-13.0) can be found in branch VTM-11.0_nnvc and contains history of development of tools integrated up to version 13.0.

NNVC-17.1 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc is used for NNVC performance evaluation.

  - Software changes

    - Current NNVC

Several commits were merged in the NNVC repository. The following changes were integrated:

| status | target | MR | topic | contribution |
| --- | --- | --- | --- | --- |
| done | NNVC | 438 | compact dataset | JVET-AP0072 |
| done | NNVC | 439 | vtm 24 rebase |  |
| done | NNVC | 437 | EE1-4.2 frame level RDO | JVET-AP0183 |
| done | NNVC |  | sadl update |  |
| done | SADL | 207 | 3x3 speedup | JVET-AO0217 |
| done | SADL | 218 | memory policy managment | JVET-AP0211 |
| done | SADL | 217 | gridsample improvment | JVET-AP0053 |
| done | SADL | 216 | conv2dtranspose | JVET-AP0053 |
| done | SADL | 215 | simd 3x3 stride 2 | JVET-AP0053 |
| done | SADL | 219 | documentation update |  |
| done | SADL | 214 | fix issue in peeling |  |

NNVC 17 includes: VTM 24.0 alignment, SADL speedup, new SADL features, SADL new memory manager, hybrid framework improvement, new training dataset framework.

  - Software overview

The framework for Hybrid end-to-end coding of intra slice has been improved for flexibility of testing for EE1. Tests can now be done on clusters in parallel using external encoder/decoder.

The master branch is kept synchronized with the upstream repository (VTM).

  - NNVC-17

  - Software version

NNVC 17.1 was tagged 28th April 2026

NNVC 17.0 was tagged 25th April 2026

NNVC 16.2 was tagged 17th April 2026 (contains VTM-23.14, LOP7, LDRF/HDRF, NNSR2)

NNVC 16.1 was tagged 18th March 2026 (contains VTM-23.14, LOP7, LDRF/HDRF, NNSR2)

NNVC 16.0 was tagged 24th February 2026 (contains VTM-23.14, LOP7, LDRF/HDRF, NNSR2)

NNVC 15.1 was tagged 19th December 2025 (contains VTM 23.13)

NNVC-15.0 was tagged 14th November 2025.

NNVC-14.1 was tagged 28th August 2025

NNVC-14.0 was tagged July 28th, 2025 (contains VLOP4, DRF, hybrid e2e)

NNVC-13 was tagged April 28th, 2025 (contains LOP6)

NNVC-12 was tagged February 6th, 2025 (containing final LOP5 model)

NNVC-12rc1 was tagged February 5th, 2025 (containing new intermediate LOP5 model)

NNVC-11.0 was tagged December 10th, 2024 (bit accurate with 11rc)

NNVC-11.0rc was tagged December 3rd, 2024

NNVC-10.0 was tagged August 9th, 2024

NNVC-9.1 was tagged May 28th, 2024

NNVC-9.0 was tagged May 13th, 2024

NNVC-8.0 was tagged February 20th, 2024

NNVC-8.0rc3 was tagged February 20th, 2024.

NNVC-7.1 was tagged November 29th, 2023.

NNVC-7.0 was tagged November 3rd, 2023.

NNVC-6.1 was tagged September 25th, 2023 (fix).

NNVC-6.0 was tagged September 6th, 2023.

NNVC-5.1 was tagged July 19th, 2023.

NNVC-5.0 was tagged May 11th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st, 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th, 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th, 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th, 2021 (VTM-11.0 base with MCTF enabled).

- CTC performance

Performances are not changed compared to NNVC-16. Some SADL speed-up allow faster decoding time for some models.

Note: see configurations section for naming convention.

  - Comparison to VTM

    - NNVC-16.2 VTM vs NNVC-17.1 VTM

There is now change in VTM base in NNVC. The only change is coming from VTM-24.0 rebase but only for PQ sequences.

    - NNVC-17.1 VTM vs NNVC-17.1 anchor

Anchor: NNVC in VTM mode.

Test: NNLF LOP7 + NNIP.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.29% | -13.23% | -15.33% | -9.53% | -15.16% | -17.37% | 116% | 1997% |
| Class A2 | -8.54% | -14.99% | -10.30% | -8.74% | -15.13% | -10.22% | 115% | 1918% |
| Class B | -7.90% | -16.44% | -14.84% | -7.83% | -17.92% | -17.18% | 117% | 2067% |
| Class C | -7.61% | -14.94% | -14.29% | -7.91% | -15.06% | -15.67% | 115% | 2073% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.23% | -15.11% | -13.88% | -8.37% | -16.05% | -15.42% | 116% | 2024% |
| Class D | -8.12% | -14.13% | -13.57% | -6.99% | -15.22% | -15.31% | 109% | 1869% |
| Class F | -4.42% | -8.65% | -7.81% | -5.47% | -11.60% | -11.45% | 120% | 1195% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -6.17% | -11.56% | -10.85% | -6.37% | -10.70% | -15.09% | 108% | 2073% |
| Class C | -6.48% | -12.39% | -11.75% | -7.33% | -10.33% | -11.85% | 103% | 1993% |
| Class E | -6.52% | -6.57% | -6.50% | -7.88% | -4.91% | -8.13% | 110% | 2372% |
| Overall | -6.36% | -10.59% | -10.06% | -7.07% | -9.13% | -12.27% | 107% | 2116% |
| Class D | -7.04% | -9.84% | -8.90% | -6.75% | -5.72% | -9.32% | 100% | 1797% |
| Class F | -3.87% | -6.93% | -4.79% | -5.86% | -7.59% | -8.09% | 109% | 1296% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -6.84% | -14.59% | -13.29% | -6.78% | -15.01% | -18.77% | 112% | 2057% |
| Class C | -6.90% | -14.91% | -14.59% | -7.44% | -14.72% | -16.34% | 105% | 2039% |
| Class E | -7.69% | -8.48% | -7.30% | -8.97% | -8.07% | -10.81% | 112% | 2714% |
| Overall | -7.07% | -13.17% | -12.22% | -7.55% | -13.18% | -15.97% | 109% | 2198% |
| Class D | -7.61% | -11.51% | -12.60% | -6.72% | -7.25% | -16.11% | 101% | 1853% |
| Class F | -4.16% | -8.76% | -6.21% | -6.01% | -11.45% | -12.27% | 112% | 1337% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.38% | -15.09% | -17.28% | -9.34% | -18.59% | -19.25% | 169% | 1684% |
| Class A2 | -7.86% | -15.13% | -11.88% | -8.47% | -15.77% | -11.60% | 171% | 1453% |
| Class B | -7.77% | -15.05% | -15.93% | -7.91% | -17.51% | -18.00% | 169% | 1522% |
| Class C | -7.88% | -13.34% | -13.84% | -8.14% | -16.81% | -17.02% | 158% | 1245% |
| Class E | -11.56% | -17.12% | -18.46% | -11.86% | -17.56% | -19.45% | 165% | 1730% |
| Overall | -8.71% | -15.03% | -15.44% | -8.95% | -17.25% | -17.17% | 166% | 1501% |
| Class D | -7.76% | -11.42% | -12.74% | -7.27% | -15.69% | -16.24% | 150% | 1091% |
| Class F | -5.66% | -9.45% | -8.92% | -6.16% | -12.52% | -13.16% | 137% | 1240% |

Note: Results from Interdigital, crosschecked by xxx.

  - Comparison to NNVC-16.2 anchor

    - NNVC-16.2 anchor vs NNVC-17.1 anchor

Anchor: NNVC 16.2 NNLF LOP7 + NNIP

Test: NNVC 17.1 NNLF LOP7 + NNIP

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 16.2 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 89% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 88% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 86% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 87% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 87% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 84% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 88% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 16.2 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 88% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 86% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 87% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 87% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 83% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 89% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 16.2 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 87% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 86% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 88% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 87% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 83% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 88% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 16.2 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 89% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 89% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 88% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 88% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 89% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 89% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 88% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 89% |

Note: Results from Interdigital, crosschecked by xxx.

  - Comparison to NNVC-17.1 anchor

    - NNVC-17.1 anchor vs NNVC-17.1 HDRF

Anchor: NNLF LOP7 + NNIP

Test: NNLF LOP7 + NNIP + HDRF

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over anchor |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.61% | -0.86% | 0.38% | -1.76% | -1.41% | -1.17% | 147% | 3664% |
| Class A2 | -2.77% | -1.02% | -0.29% | -3.05% | -2.12% | -1.90% | 137% | 3427% |
| Class B | -2.13% | -0.74% | 0.90% | -2.42% | -1.67% | -1.04% | 142% | 4066% |
| Class C | -3.38% | -1.52% | -0.65% | -4.30% | -3.20% | -2.71% | 126% | 3611% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.49% | -1.03% | 0.14% | -2.91% | -2.12% | -1.68% | 137% | 3729% |
| Class D | -3.71% | 0.37% | 1.17% | -3.97% | -1.63% | -1.58% | 125% | 3435% |
| Class F | -1.05% | -0.36% | 0.05% | -1.58% | -0.95% | -0.80% | 172% | 10406% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-17.1 anchor vs NNVC-17.1 LDRF

Anchor: NNLF LOP7 + NNIP

Test: NNLF LOP7 + NNIP + LDRF

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 17.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.25% | 3.43% | -1.16% | -1.44% | 0.07% | -1.43% | 115% | 1122% |
| Class A2 | -2.01% | 0.57% | -1.65% | -2.49% | -1.86% | -2.12% | 109% | 1052% |
| Class B | -1.13% | 0.28% | -1.14% | -1.69% | -1.16% | -1.67% | 112% | 1230% |
| Class C | -1.56% | -1.36% | -1.94% | -2.73% | -2.10% | -2.77% | 106% | 1105% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.44% | 0.53% | -1.46% | -2.08% | -1.31% | -2.01% | 110% | 1137% |
| Class D | -1.23% | -1.07% | -1.63% | -2.18% | -2.14% | -2.56% | 106% | 1055% |
| Class F | -0.61% | -0.42% | -0.62% | -1.19% | -0.75% | -1.16% | 123% | 3081% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-17.1 anchor vs NNVC-17.1 VLOP4

Anchor: NNLF LOP7 + NNIP

Test: NNLF VLOP4 + NNIP

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 17.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 1.56% | 7.29% | 11.12% | 0.45% | 8.36% | 12.91% | 99% | 51% |
| Class A2 | 2.50% | 9.52% | 6.42% | 2.07% | 8.81% | 6.11% | 99% | 48% |
| Class B | 2.42% | 9.81% | 9.74% | 1.62% | 10.60% | 11.14% | 99% | 50% |
| Class C | 2.70% | 9.43% | 9.37% | 1.89% | 8.53% | 9.78% | 100% | 50% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 2.34% | 9.15% | 9.25% | 1.55% | 9.24% | 10.13% | 99% | 50% |
| Class D | 3.34% | 10.12% | 9.73% | 1.73% | 9.84% | 9.79% | 101% | 48% |
| Class F | 1.51% | 5.54% | 4.46% | 1.55% | 7.34% | 7.06% | 98% | 56% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-17.1 anchor vs NNVC-17.1 HOP5

Anchor: NNLF LOP7 + NNIP

Test: NNLF HOP5 + NNIP

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 17.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.21% | -12.30% | -15.36% | -6.01% | -14.07% | -14.60% | 331% | 8841% |
| Class A2 | -4.83% | -16.31% | -25.90% | -3.88% | -15.64% | -23.50% | 302% | 8212% |
| Class B | -4.72% | -22.25% | -16.22% | -3.22% | -19.41% | -13.33% | 308% | 8616% |
| Class C | -5.73% | -15.16% | -15.60% | -3.68% | -11.87% | -11.70% | 237% | 8136% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.91% | -17.18% | -17.82% | -4.03% | -15.58% | -15.18% | 290% | 8448% |
| Class D | -6.27% | -16.37% | -16.91% | -3.21% | -10.74% | -9.41% | 229% | 8307% |
| Class F | -3.55% | -10.65% | -10.52% | -2.81% | -9.10% | -7.76% | 425% | 8175% |

Note: Results from InterDigital, crosschecked by xxx.

- Contributions

We have 4 contributions for AhG14 and 1 telco report.

| JVET-AQ0042 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| --- | --- | --- |
| JVET-AQ0138 | AHG14: SADL Update | F. Galpin (InterDigital) |
| JVET-AQ0151 | AHG11] [AHG14] Huawei response to MPEG survey on AI hardware | E. Alshina, Y. Zhao (Huawei) |
| JVET-AQ0171 | AhG14: CUDA backend for SADL | F. Galpin, S. Cizel (InterDigital) |
| JVET-AQ0173 | [AHG11][AHG 14] Real-Time Implementation of an End-to-End AI Video Codec | S. Cizel, R. Mullakhmetov, A. Berk, C. Finlay, F. Galpin, E. François (InterDigital) |

- Configurations

The following configurations are used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration |
| --- | --- | --- |
| NNVC VTM mode | None | encoder_xxx_vtm.cfg |
| NNVC Anchor/EE1 | Intra Pred + LOP.6 | encoder_xxx_nnvc.cfg |
| NNVC. HOP | Intra Pred + HOP.5 | encoder_xxx_nnvc.cfg + nn-based/HOP5.cfg |
| NNVC. VLOP | Intra Pred + VLOP.4 | encoder_xxx_vtm.cfg + nn-based/vlop4.cfg |
| NNSR | Super-resolution | nn-based/nnsr.cfg + nn-based/nnsr_classA_multiratio.cfg |
| RPR | Anchor RPR | nn-based/rpr.cfg + nn-based/nnsr_classA_multiratio.cfg |
| NNPF | Adaptive post-filters | nn-based/nnpf/nnpf_xxx.cfg |
| CALOP | Intra pred+adaptive LOP | encoder_randomaccess_nnvc.cfg+nn-based/intra.cfg+nn-based/calop.cfg |
| NNVC DRF | NNVC anchor + DRF | encoder_xxx_nnvc.cfg + drf.cfg |

- Recommendations

The AHG recommended to:

- Review all input contributions.

- Keep synchronized with VTM upstream

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

- Encourage people to submit merge requests fixing identified bugs.

JVET-AQ0015 JVET AHG report: Gaming content compression (AHG15) [C. Lehmann, S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice-chairs)]

Test results

Test results were obtained according to the latest Gaming CTC document JVET-AO2027. The list of sequences/classes is copied for convenience below:

| Class | Sequence name | Frame count | Low delay frame count | Frame rate | Bit depth | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 (SDR) | Level1_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | Darktree_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | ARPG2_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | DesertTown3_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | SunTemple3_SDR | 600 | 300 | 60 | 10 | M | M | M |
| G2 | GTAV | 600 | 300 | 60 | 8 | M | M | M |
|  | Minecraft | 600 | 300 | 60 | 8 | M | M | M |
|  | JianlingTemple | 600 | 300 | 60 | 8 | M | M | M |
|  | BaoleiYard | 300 | - | 60 | 8 | M | M | - |
|  | Wukong2 | 300 | - | 60 | 10 | M | M | - |
|  | Wukong3 | 300 | - | 60 | 10 | M | M | - |
| G3 (HDR) | ARPG2_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | DesertTown3_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | SunTemple3_HDR | 600 | 300 | 60 | 10 | O | O | O |

VTM-11.0ecm19.1 vs ECM-20.0

Please note that the anchors were generated using ECM-20.0. Since there is no equivalent VTM version for this version, the last anchor VTM-11.0ecm19.1 was used for the comparison.

SDR

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11ecm19.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -12.0% | -24.9% | -19.6% | 1213% | 941% | 359.2% |
| Class G2 | -15.5% | -23.4% | -26.4% | 1259% | 994% | 347.3% |
| Overall | -13.9% | -24.1% | -23.3% | 1238% | 970% | 352.7% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over VTM-11ecm19.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -28.0% | -47.0% | -38.3% | 991% | 2674% | 284.7% |
| Class G2* | -25.5% | -37.5% | -38.6% | 1107% | 2214% | 278.3% |
| Overall* | -26.9% | -42.8% | -38.4% | 1041% | 2459% | 281.8% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over VTM-11ecm19.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1** | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class G2** | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over VTM-11ecm19.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1** | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class G2** | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

* QP22/27 for Wukong2 and QP22/27/32 for Wukong3 are still missing for ECM. Summary is without those.

** LDB/P simulations are still running.

HDR

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11ecm19.1 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  |  |  | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 |  |  | -15.6% | -57.6% | -55.7% | -15.5% | -57.9% | -55.3% | 1105% | 672% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm20.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  |  |  | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 |  |  | -35.2% | -71.6% | -70.8% | -35.4% | -72.1% | -70.7% | 911% | 1751% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm20.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  |  |  | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 |  |  | -26.1% | -75.2% | -72.9% | -26.3% | -75.8% | -72.8% | 882% | 1726% |
|  | Low Delay P |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm19.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  |  |  | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 |  |  | -25.4% | -75.2% | -72.0% | -25.7% | -75.8% | -72.0% | 831% | 1412% |

Input contributions

There are no contributions relating to the mandates of AHG15.

Recommendations

The AHG recommended to:

- Identify if/how auxiliary information can be used for coding of gaming content

JVET-AQ0016 JVET AHG report: Hardware implementation complexity (AHG16) [Y. Zhao, I. Moccagatta, K. Naser (co-chairs), H. Huang, T. Ikai, X. Li, J. Park, N. Song, G. Verba (vice-chairs)]

- HIC software development and maintenance

It was agreed at the last meeting to create an AHG16 Git repository for further experimentation. The AHG16 software branch, with an initial implementation of hardware complexity measurement and encoding simulation, has been created under the following Git repository (branch: AHG_HIC):

https://vcgit.hhi.fraunhofer.de/jvet-ahg-hic/VVCSoftware_VTM

The first tag is HIC-1.0, which is based on VTM-24.0 and was released on June 9, 2026.

The initial version includes the algorithms proposed in JVET-AP0191. It is a preliminary attempt toward building a simulation platform that can reflect major constraints of hardware codecs, and more contributions are welcome.

One issue was identified after the release of HIC-1.0 and has been fixed.

- Input contributions

The contributions listed below have been identified as related to the mandates of AHG16.

| Category | JVET number | Title | Source |
| --- | --- | --- | --- |
| RDO counting and restriction | JVET-AQ0045 | [AHG16] On per-CU RDO quota | M. Wu, J. Yu (HiSilicon), Y. Zhao, E. Alshina (Huawei) |
|  | JVET-AQ0150 | [AHG16] Improvements on Hardware Implementation Complexity (HIC) Software | K. Naser, F. Le Léannec, F. Galpin, F. de Lagrange (InterDigital), Y. Zhao, E. Alshina (Huawei) |
|  | JVET-AQ0185 | AHG16: Area-ratio extension to the RDO selection count restriction | H. Bai, T. Ikai, Y. Tokumo, R. Ishimoto (Sharp) |

- Recommendations

The AHG recommended to:

- Review related input contributions.

- Collect opinions and suggestions from experts to further improve the HIC software platform.

JVET-AQ0017 JVET AHG report: Preparation of Call for Proposals (AHG17) [J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice-chairs)]

Activities

Two online AHG calls were held on 2026-05-19 and 2026-05-28, respectively. The reports of these meetings are available in JVET-AQ0041.

The bitstreams for the VTM configurations defined in the CfP document were prepared and uploaded to the JVET content server.

During the AHG calls and in offline communication, several clarification aspects and potential improvements for the CfP document were collected. Proposed changes based on these inputs were collected in input contribution JVET-AQ0201 which includes an edited version of the final draft CfP document JVET-AP2026 with change marks. It is suggested to review and discuss the proposed changes at the meeting.

An initial version of a draft template for proposal reporting has been generated as JVET-AQ0202 as input for the development of a template contribution document to be submitted for the registered responses to the CfP.

Related contributions

| JVET number | Title | Source |
| --- | --- | --- |
| JVET-AQ0017 | JVET AHG report: Preparation of Call for Proposals (AHG17) | J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice-chairs) |
| JVET-AQ0041 | AHG17: AhG meeting notes | M. Wien, J. Ohm, F. Bossen |
| JVET-AQ0101 | AHG17: On characteristics of CfP test sequences | T. Chujoh, H. Kato, Y. Kidani, K. Kawamura (KDDI) |
| JVET-AQ0153 | [AHG17] Comments on CfP document | E. Alshina, Y. Zhao, X. Ma, S. Ikonin, T. Solovyev (Huawei) |
| JVET-AQ0201 | AHG17: Suggested editorial changes to the Final Draft Joint Call for Proposals on Video Compression with Capability beyond VVC | M. Wien, J.-R. Ohm, F. Bossen |
| JVET-AQ0202 | AHG17: Suggested initial text of "Template for Proposal Description Documents of Responses to the Joint CfP on Video Compression with Capability beyond VVC" | J.-R. Ohm, M. Wien |
| JVET-AQ0220 | [AHG17] Suggested Editorial Changes to the CfP Document | R. Chernyak (Tencent) |

Recommendations

The AHG recommended to:

- To review the changes proposed for the CfP document and generate the final CfP document as output of this meeting.

- To generate a draft reporting template for responses of the CfP based on input contribution JVET-AQ0202.

- To review the remaining input contributions related to AHG17 during the meeting.

It was verbally reported that 9 laboratories have expressed interest in participating in the test. In case that this is not sufficient (depending on the number of proposals) to include all test cases in all categories, it is to be decided in the October meeting (or better AHG telco(s) in the interim period after the registration deadline in early September) which cases should be omitted.

JVET-AQ0018 JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) [S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs)]

- Software development

Adoptions of last JVET meeting, namely JVET-AP0200 Random Access GOP4 based on standalone Network model of JVET-AO0123, and JVET-AP0222: NN-based lost picture generation, have been published in AHG18 repository (https://vcgit.hhi.fraunhofer.de/jvet-ahg-ull/VVCSoftware_VTM) as MR8, MR9 correspondingly, crosscheck is pending.

- Related contributions

A total of 5 contributions (not including this AHG report) are identified relating to the mandates of AHG18. They are listed below. Three contributions are also related to the work of AHG9.

| Proposals |  |  |
| --- | --- | --- |
| JVET-AQ0183 | AHG18: Forward Error Correction for ULL simulation software | I. Gribushin, K. Malyshev, G. Gotovskii, B. Shevchenko, S. Ikonin, E. Alshina (Huawei) |
| JVET-AQ0199 | AHG18: Feedback-Aware Reference Preservation for Ultra-Low Latency | Y. Xiao, Y. Hu, X. Ma (MLS labs) |
| JVET-AQ0081 | AHG9: On persistence etc. and error resilience of the GSI SEI message | Y.-K. Wang, J. Xu, Y. Xu, Q. Yang, K. Zhang (Bytedance) |
| JVET-AQ0124 | AHG9: On delayed transmission of QM SEI | T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia) |
| JVET-AQ0196 | AHG9: Loss concealment purpose for NNPFC SEI message | S. Mate, J. Boyce, M. M. Hannuksela (Nokia) |

- Recommendations

The AHG recommended to:

- Review all input contributions.

- Discuss test conditions and evaluation methodology.

- Collect test cases and specific requirements.

- Continue development of simulation software.

- Continue the study of ultra-low latency and packet loss resilience technologies in JVET.

JVET-AQ0019 Report of joint AHG on Gaussian splat coding [J. Jung, W. Husak (JAHG co-chairs on behalf of JVET)]

Was presented as a slide deck on Wednesday 15 July 1015-1050. JVET-AQ0019 in an excerpt from m77135, only items related to SEI messages are included.

All AHG recommendations on SEI messages were confirmed in the meeting on Tuesday, plus two more agreements made in the Tuesday joint meeting (see notes in section 7.3.1).

# Project development (21)

## AHG1: Deployment and advertisement of standards (0)

This section is kept as a template for future use.

## AHG2: Text development and errata reporting (0)

This section is kept as a template for future use.

Documents in 6.1 are related.

## AHG3: Software development (1)

Contributions in this area were discussed during 1040–1100 on Friday 10 July 2026 (chaired by JRO).

JVET-AQ0211 in section 4.11 is also related to software

JVET-AQ0187 Consolidating the HEVC Reference Software: Adding SHVC Support to the dev-multiview444 Branch [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri, S. Choi, W. Zia (Apple)]

The reference software for HEVC is today spread across several separate codebases — the main HM, the format range extensions, the multiview and 3D extensions (the dev-multiview444 branch), and the scalable extension (SHM) — each supporting only a subset of the standard’s profiles and coding tools. This fragmentation makes it hard for implementers to see how the various HEVC profiles relate to one another and to exploit the substantial machinery they share, which in turn works against the adoption of the full range of HEVC profiles. This contribution reports a step toward consolidating these codebases: the integration of Scalable HEVC (SHVC, Annex H) support into the MV-HEVC dev-multiview444 branch of HM, so that the multiview, range-extension and scalable profiles are exercised by a single, common reference software built on the full functionality of the base HM. Alongside the SHVC integration, the contribution provides the fixes needed to make per-layer MCTF and film-grain analysis and Apple-Silicon (arm64) builds work correctly in the multilayer setting, together with a number of bug fixes (including a hierarchical-B multiview GOP-scheduling correction and the addition of the Monochrome 10 profile). The complete set of changes is available as merge request !115 against the upstream jvet/HM dev-multiview444 branch, and as a unified diff attached to this contribution.

The implementation of scalable profiles in HM would make the SHM software obsolete, which is very welcome as it has hardly had any maintenance during the recent years (it is noted that some investigation had been done in the context of recent scalable multi-layer test, but it had not been possible to make a reasonable update on integrating newer features such as MCTF, for example).

After the merge has been conducted, further investigation/experimentation should be conducted.

The next version of the reference software planned to be issued in April 2027 should with highest priority contain support multiview profiles contained in the latest version of HEVC, but it might be desirable to remove SHM from H.265.2 and 23008-8 if it was sufficiently confirmed that scalable profiles are completely supported in HM.

## AHG3: Test conditions and metrics (0)

This section is kept as a template for future use.

## AHG4: Subjective quality testing and verification testing (0)

This section is kept as a template for future use.

## AHG4: Test and training material (0)

This section is kept as a template for future use.

## AHG5: Conformance test development (1)

Contributions in this area were discussed during 1100–1110 on Friday 10 July 2026 (chaired by JRO).

JVET-AQ0186 MV-HEVC Conformance Bitstreams for the Multiview Monochrome 10/12 and Multiview 4:4:4 10/12 Profiles [A. M. Tourapis, D. Podborski, J. Kim (Apple)]

This document provides a set of eight conformance bitstreams for the Multiview Monochrome 10, Multiview Monochrome 12, Multiview 4:4:4 10, and Multiview 4:4:4 12 profiles of the multiview format range extensions specified in Annex G of Rec. ITU-T H.265 | ISO/IEC 23008-2. The bitstreams exercise 10-bit and 12-bit monochrome auxiliary-picture coding (depth and unspecified auxiliary types) and 10-bit and 12-bit 4:4:4 two-view coding with inter-layer (inter-view) prediction, under both IDR-only and CRA refresh. They are intended for inclusion in the HEVC conformance bitstream set. The bitstream specifications follow the format of the existing MV-HEVC conformance documentation to facilitate direct integration by the editors. This revised version (v2) corrects the monochrome chroma bit-depth signalling in the SPS so that it matches the VPS representation format, simplifies the IDR bitstreams to a single IDR at the start (removing an unintended mid-sequence IDR and its associated leading pictures), packages each bitstream as a flat archive, and renames the bitstreams to the MVHEVCS_Z … MVHEVCS_AG series.

It is noted that bitstreams for Monochrome 16 are still missing (Bytedance responsible) – it was anticipated that those should be available by end of the meeting (A. Tourapis took over responsibility to generate them). A description text to be included in JVET-AQ1008 (and later in CD) should be provided by the end of the meeting (Y.-K. Wang).

Decision: Adopt JVET-AQ0186 into JVET-AQ1008

As software and conformance for the 444 multiview profiles are existing now, it would be possible to issue a request and CDAM for new HEVC amendment (likely also including some improvements on SEI interfaces) – was further discussed in Sunday plenary.

## AHG7: Tool assessment (0)

This section is kept as a template for future use.

## AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (0)

This section is kept as a template for future use, or can be removed after finalization of the TR/Sup.

## AHG10: Encoding algorithm optimization (0)

This section is kept as a template for future use.

## AHG13: Film grain technology (2)

Contributions in this area were discussed during 1135–1220 on Friday 10 July 2026 (chaired by JRO).

JVET-AQ0211 Film grain analysis and synthesis support in HDRTools [A. M. Tourapis, [J. Kim, D. Podborski (Apple)] [late]

This contribution reports on film grain analysis and synthesis functionality that has been integrated into the HDRTools software (version 0.27-dev). HDRTools now provides a dedicated FilmGrainTool, as well as film grain synthesis within the HDRConvert conversion tool, supporting two different film grain models, the Film Grain Characteristics (FGC) and the AOMedia Film Grain Synthesis 1 (AFGS1) models. The functionality supports both the estimation (analysis) of film grain parameters from source and denoised content and the synthesis of film grain onto decoded or denoised content, with parameters carried in a human-readable JSON metadata format. This document describes the added functionality, the supported models, the analysis and synthesis pipelines, the JSON metadata format, and the associated configuration parameters.

For FGC, only the frequency domain model is supported.

AFGS1 implementation in the package falls under same BSD licensing conditions.

It was announced that further developments are planned for implementing quality metric implementations for film grain.

Software of JVET-AQ0211 candidate for being referenced in the TR. Synthesis part might also be directly attached.

JVET-AQ0222 AHG13: proposed updates to the technical report on film grain synthesis technology for video applications [P. de Lagrange, M. Radosavljević (InterDigital)] [late]

At the 42nd JVET meeting, a request for 2nd edition of the technical report on film grain synthesis technology for video applications was issued, and a new draft has been output as JVET-AP2020. This contribution elements that are listed in the request but still missing in the draft.

Updates during the current meeting are expected to propose text to address the missing points.

The request for 2nd edition of the technical report on film grain synthesis technology for video applications (ISO/IEC TR 23002-9) listed the following expected changes compared to the first edition:

–	Technical changes to align with ITU-T H.Sup.21;

–	Editorial changes to generally improve the report regarding understanding and usage of the technology;

–	Provide updated information on new metadata elements supporting the use of the technology (e.g. some other SEI such as manifest); and

–	Provide updated information on related tools regarding model parameter estimation and other aspects

Text alignment has been mostly completed, together with some editorial improvements and updated information on analysis techniques (both conventional and learning-based), and synthesis (soft cut-off for frequency model).

However, information is still missing on the recent extension of the FGC SEI message regarding reference picture size and other recent SEI messages that could be relevant.

Note that attaching software was considered, but is not mentioned in the request.

About the attachment of software: Attaching a possibly imperfect version at CDTR is less critical, as there may be comments that require improvement that can be resolved in DTR stage. If such comments would come at DTR stage, a second DTR would be needed.

It was asked to provide a list of software elements that are deemed to be mature enough to be included in CDTR.

A list of candidate software elements was provided in v2 of JVET-AQ0222, including software developed by JVET as well as other parties. It was suggested that only software maintained by JVET should be considered as candidate attachments for the CDTR. This would be the version of VFGS developed in the AHG on Film Grain (jvet-ahg-fgt) in the JVET git repository.

## Implementation studies (2)

Contributions in this area were discussed during 0910–0930 on Friday 10 July 2026 (chaired by JRO).

JVET-AQ0190 Prototype VVC decoder supporting dynamic layer switching during multi-layer bitstream playback [S. Iwamura, S. Nemoto, K. Unno, Y. Kondo, A. Ichigaya (NHK)] [late]

This contribution reports a prototype decoder for VVC multilayer bitstreams targeting service-like operation where the multilayer configuration may change dynamically when switching between programs. The prototype analyzes high-level syntax elements (VPS, SPS and PPS) at each IDR picture boundary, derives the currently available layer configuration, and supports dynamic reconfiguration of the decoder when the number of layers and/or layer dependencies change. A user interface presents selectable layer options (including spatial scalability and content-layering options) and triggers output-layer selection via an external USB switch. The implementation is built on top of the VVdeC-multilayer decoding library previously reported in JVET-AM0239 and is integrated into a receiving pipeline that extracts VVC elementary streams from CMAF/MMT payloads. Real-time 4K/60p multilayer decoding is demonstrated on a compact PC platform. A live demonstration of the prototype is planned during the 43rd JVET meeting, showcasing dynamic program switching between multilayer bitstreams with different layer configurations and user-driven output-layer selection.

Information well received by other experts.

JVET-AQ0191 Browser-based VVC player supporting CMAF/DASH streaming on Android [S. Iwamura, S. Nemoto, K. Unno, Y. Kondo, A. Ichigaya (NHK)] [late]

This contribution reports the latest progress of a browser-based VVC playback software built on a customized Chromium browser integrated with the VVdeC-multilayer decoding library. In a previous contribution JVET-AM0242, playback of multilayer VVC content on a PC browser was reported. Since then, three functional extensions have been implemented and are reported in this contribution: (1) support for streaming playback of CMAF streams delivered over MPEG-DASH, in which an MPEG-DASH player retrieves pre-segmented CMAF content directly from an HTTP server; (2) a temporal-sublayer (TID) selective-decoding feature that allows the user to specify a target TemporalId so that only pictures with TemporalId up to the specified value are decoded, reducing the decoding complexity and enabling smooth playback on lower-performance devices at a reduced frame rate; and (3) operation of the player on an Android device, confirmed on a recent commercial Android smartphone. Together these results demonstrate that VVC multilayer coding combined with temporal scalability can operate in a practical delivery-and-device scenario (MPEG-DASH/CMAF over a browser, on mobile), supporting the practical viability of VVC. A live demonstration of the browser-based player is under consideration during the Geneva meeting.

Information in the contribution was appreciated by other experts.

## Profile/tier/level specification (1)

Contributions in this area were discussed during 1405–1430 on Monday 13 July 2026 (chaired by JRO).

JVET-AQ0189 HEVC Profile Signalling Reference [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri, W. Zia (Apple)]

This contribution is a  re-submission  of JVET-AP0246, which  was noted but not fully discussed. The contribution provides a comprehensive and consolidated reference for the profile, tier, and level signalling mechanisms in Rec. ITU-T H.265 | ISO/IEC 23008-2 High Efficiency Video Coding (HEVC). The definitions and characteristics of the numerous HEVC profiles are distributed across multiple annexes and clauses of the specification, requiring significant effort from implementers to interpret correctly. This contribution consolidates this scattered information into a single, accessible guide. It covers the profile identification mechanism, constraint flag signalling, per-profile characteristics, backward compatibility requirements, and the distinct constraint scoping rules for multi-layer profiles, with the goal of reducing implementation ambiguity and providing a clear reference for the HEVC community.

One possible option that was suggested is defining profile as a separate part (as JPEG is doing). It was commented that this might be problematic if defining tools and profiles is not sufficiently synchronized.

It is asserted that this is generally beneficially to have a better clarification of the purpose of mechanisms/parameters used in profile definition, and relationship between different profiles.

Proponents of JVET-AQ0189 are asked to further study and provide a version in the next meeting that could be used as a starting point to be later included as an informative annex in HEVC.

It was further asserted that it would be beneficial to consider such additional information about profiles in the development of a next generation video standard.

## AHG15: Gaming content compression (0)

This section is kept as a template for future use.

## AHG16: Hardware implementation complexity (3)

Contributions in this area were discussed during 0930–1035 on Friday 10 July 2026 (chaired by JRO).

JVET-AQ0151 and JVET-AQ0152 (in 5.1.3) are also related.

JVET-AQ0045 [AHG16] On per-CU RDO quota [M. Wu, J. Yu (HiSilicon), Y. Zhao, E. Alshina (Huawei)]

According to the discussion of per-CU RDO constraint during the 42nd meeting, this study investigates the impact of several Rate-Distortion Optimization (RDO) algorithms on hardware implementation and provides recommendations for improving RDO constraints to achieve less coding efficiency drop with reasonable hardware encoder complexity increase.

- RDO in HD should be scaled down by factor of two relative to 4K

- Half number of intra RDO compared to inter is desirable

- Maximum encoding time should be limited per CU, also considering CU size (smaller CUs require deeper pipelining)

It is noted that this is a preliminary analysis, no results available yet how much gain could finally be achieved by more flexible resource allocation (preliminary results in JVET-AQ0150, which implements the recommended restrictions).

It was asked if more flexibility could be achieved by using dual-tree for intra pictures, as this might allow conducting more RDOs. This was not studied so far.

It was commented that developing faster RDO might be better than disabling a certain amount of RDOs as they are implemented in reference software. Not studied so far.

It was asked if there is impact of the CU shape (e.g. narrow rectangular ones) on the complexity? Not studied so far.

JVET-AQ0150 [AHG16] Improvements on Hardware Implementation Complexity (HIC) Software [K. Naser, F. Le Léannec, F. Galpin, F. de Lagrange (InterDigital), Y. Zhao, E. Alshina (Huawei)]

This contribution proposes multiple improvements to the HIC software, following the recommendations by hardware experts comments from last meeting. Specifically, it proposes the following:

- The RDO budget of intra coding modes in I frame is increased by reusing the resources for inter mode RDOs, where intra modes can additionally use half of the inter RDOs, according to the suggestion provided in JVET-AQ0045.

- Defining a per-CU RDO budget related to CU sizes, such that smaller CU’s are more restricted in terms of the number of RDOs (vice-versa).

It is reported that the new performance of constraint encoder (reduced runtime environment, GOP8, 1 reference frame and 10 RDOs for large CUs & 5 RDOs for small ones) compared to VTM-24.0 anchor is:

20.86%	31.03%	31.30%	13% 104%

HIC-1.0 SW (10 RDOs for all CUs) vs. VTM-24.0 anchor is:

20.11%	29.79%	30.29%	14%	105%

And HIC-1.0 SW (5 RDOs for all CUs) vs. VTM-24.0 anchor is:

23.27%	36.43%	36.17%	12%	105%

Test1: Impact of using half max inter RDOs in intra frames

Anchor: HIC-1.0 with GOP8 + 1 reference frame + RR3 +10 IntraRDOs + 10 InterRDOs

Test1: Anchor + Intra modes in I Frames use additionally half inter RDOs (15 RDOs)

|  |  |  | Random access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-23.9 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.09% | -0.23% | 100% | 99% |
| Class A2 | -0.28% | 0.02% | -0.15% | 100% | 100% |
| Class B | -0.30% | 0.05% | -0.53% | 101% | 100% |
| Class C | -0.41% | -0.11% | -0.46% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.03% | -0.38% | 101% | 100% |
| Class D | -0.25% | 1.25% | 0.03% | 102% | 102% |
| Class F | -1.78% | -2.27% | -1.98% | 102% | 101% |

It can be concluded that 0.3% coding gain can be obtained without sacrificing the encoder time.

Test2: Impact of using half max inter RDOs in intra frames and further restricting RDOs for smaller CUs

Anchor: same as in Test1

Test2: Test1 + 5 to 10 Max RDOs as in the table below.

|  |  |  | Random access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-23.9 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.71% | 0.81% | 0.38% | 96% | 100% |
| Class A2 | 0.31% | 0.79% | 0.49% | 96% | 100% |
| Class B | 0.53% | 0.65% | 0.60% | 96% | 100% |
| Class C | 0.91% | 1.22% | 1.37% | 94% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.62% | 0.86% | 0.74% | 95% | 99% |
| Class D | 1.45% | 3.32% | 2.30% | 94% | 102% |
| Class F | 1.74% | 1.39% | 0.95% | 95% | 99% |

Clearly, this comes with a coding loss, but also with reduction of encoding time. It can also be observed that there is some larger loss for small resolution sequences, as fewer RDOs are used.

Test3: VTM as anchor

Anchor: VTM-24.0

Test3a: HIC-1.0 with GOP8 + 1 reference frame + RR3 +10 IntraRDOs + 10 InterRDOs

Test3b: HIC-1.0 with GOP8 + 1 reference frame + RR3 +5 IntraRDOs + 5 InterRDOs

Test3c: HIC-1.0 with GOP8 + 1 reference frame + RR3 +5 to 10 IntraRDOs + 5 to 10 InterRDOs + intra frame further use half max inter RDO

Test3a: HIC-1.0 with GOP8 + 1 reference frame + RR3 +10 IntraRDOs + 10 InterRDOs

|  |  |  | Random access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-24.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 15.26% | 24.48% | 26.66% | 13% | 106% |
| Class A2 | 22.32% | 34.66% | 32.08% | 14% | 104% |
| Class B | 22.13% | 32.52% | 34.50% | 14% | 105% |
| Class C | 19.55% | 26.71% | 26.41% | 13% | 103% |
| Class E |  |  |  |  |  |
| Overall | 20.11% | 29.79% | 30.29% | 14% | 105% |
| Class D | 15.13% | 19.01% | 23.39% | 13% | 93% |
| Class F | 24.18% | 26.03% | 28.21% | 20% | 104% |

Test3b: HIC-1.0 with GOP8 + 1 reference frame + RR3 +5 IntraRDOs + 5 InterRDOs

|  |  |  | Random access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-24.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 18.71% | 29.94% | 31.52% | 11% | 107% |
| Class A2 | 25.08% | 41.63% | 37.20% | 13% | 104% |
| Class B | 25.04% | 39.64% | 40.99% | 13% | 106% |
| Class C | 23.13% | 33.38% | 32.85% | 11% | 103% |
| Class E |  |  |  |  |  |
| Overall | 23.27% | 36.43% | 36.17% | 12% | 105% |
| Class D | 18.61% | 27.52% | 30.84% | 11% | 94% |
| Class F | 32.60% | 39.56% | 41.78% | 17% | 103% |

Test3c: HIC-1.0 with GOP8 + 1 reference frame + RR3 +5 to 10 IntraRDOs + 5 to 10 InterRDOs + intra frame further use half max inter RDO

|  |  |  | Random access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-24.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 16.08% | 25.55% | 27.18% | 12% | 105% |
| Class A2 | 22.67% | 35.78% | 32.73% | 14% | 104% |
| Class B | 22.80% | 33.60% | 35.27% | 14% | 105% |
| Class C | 20.65% | 28.37% | 28.35% | 12% | 102% |
| Class E |  |  |  |  |  |
| Overall | 20.86% | 31.03% | 31.30% | 13% | 104% |
| Class D | 16.85% | 23.00% | 26.54% | 12% | 94% |
| Class F | 26.29% | 28.09% | 29.65% | 19% | 102% |

It is noted that modified SW (Test3c) achieves almost the same coding performance as HC SW with 10 RDOs, while not reducing the maximum RDOs for the small CUs. It also save ~2.5% of BD rate compared to test2, where all CU’s are restricted to 5 RDOs. Therefore, it is recommended to use the Test3c configuration for the HIC software.

It was suggested to investigate whether the macros from the software could be used to measure the RDO count in VTM in the context of the complexity reporting template for the CfP (both for anchor and faster encoding methods). This could also be provided to proponents as an example how they could measure the RDO count.

Decision(SW): include JVET-AQ0150 software in the AHG16 software

JVET-AQ0185 AHG16: Area-ratio extension to the RDO selection count restriction [H. Bai, T. Ikai, Y. Tokumo, R. Ishimoto (Sharp)] [late]

This contribution reports on an extension to the AHG_HIC VTM based software for constraining per-CU max RDOs proposed in JVET-AP0191. In the current AHG_HIC VTM, a uniform maximum number of RDOs is applied regardless of CU size. However, it is asserted that more RDOs can be used for larger CUs in implementations. In response, the proposed extension (a.k.a. area-ratio extension) scales the per-CU max RDOs by the ratio of the CU area to an 8×8 base block, allowing larger CUs to perform a greater number of RDOs. In terms of EncT ratio vs performance balance, it is reported that the effect of this extension depends on the sequence class: For high-resolution sequences such as Class A with larger CUs, the Y BD-rate loss at the same encoding time is reduced. For sequences dominated by smaller CUs such as Class C, no improvement is observed. Based on these findings, it is recommended that the area-ratio extension be applied selectively to high-resolution content.

It was commented that also in JVET-AQ0045, the RDO budget for larger CUs is increased, but not proportional with the CU area, as in  JVET-AQ0185. Even though the results of JVET-AQ0185 indicate that the impact on software runtime is not significant and even gives better results when the restriction is made less stringent, it is asserted that the motivation of the stronger restriction suggested in JVET-AQ0045 is motivated by hardware constraints.

It is also noted that the results are highly content dependent, particularly on resolution/class.

More study needed in context of AHG.

## AHG17: CfP on video coding technology beyond VVC (6)

JVET-AQ0041 AHG17: AhG meeting notes [M. Wien, J. Ohm, F. Bossen]

No need for separate presentation – part of AHG report.

### CfP text (3)

Contributions in this area were discussed during 1530–1700 on Tuesday 7 July 2026 (chaired by JRO), and 1430–1545 on Wednesday 8 July 2026 (chaired by JRO).

JVET-AQ0153 [AHG17] Comments on CfP document [E. Alshina, Y. Zhao, X. Ma, S. Ikonin, T. Solovyev (Huawei)]

This contribution summarizes comments on CfP documents, including some suggestion and aspects to be clarified.

See discussion under JVET-AQ0201.

JVET-AQ0201 AHG17: Suggested editorial changes to the Final Draft Joint Call for Proposals on Video Compression with Capability beyond VVC [M. Wien, J.-R. Ohm, F. Bossen]

The attachment of this document includes edits and changes on top of JVET-AP2026_v1 (Final draft Joint Call for Proposals on video compression with capability beyond VVC) based on observations of the editors and comments collected during the AHG period.

Aspects discussed:

- It was commented that the derivation of transmitted post-processing parameters needs to be counted as part of the encoder run time, in addition to the post processing itself as part of the decoder run time.

- About the random access aspect, it was agreed that a sentence should be added that proponents should be prepared to demonstrate that the functionality works by removing a part of the bitstream.

- It was suggested to add a reference to the implementation in VTM that performs conversion yuv<>pyuv.

- Further discussion necessary on the aspect of measuring parallel encoding runtime in RA (counting I pictures once or twice)

- In addition, aspects from JVET-AQ0153 and JVET-AQ0220 that require further consideration during the meeting were included in JVET-AP2026_v4 (uploaded in JVET-AQ0201_v2.zip) of as a basis for further editing towards the CfP. Aspects of JVET-AQ0220 related to more precise definition of run time measurements are for further discussion.

Follow-up discussion Wednesday 8 July 1430-1545: JVET-AP2026v5 (in JVET-AQ0201v3) which was edited to further modify the text reflecting comments/discussion from Tuesday was presented by M. Wien. The modifications based on discussion in the session on Wednesday were made available as JVET-AP2026v6 (in JVET-AQ0201v4). It was further noted that, by moving the paragraph that allows proponents to select the number of test cases which should be included in subjective testing from the end of section 5 (constrained encoder runtime) to section 8 (testing fee), it should be clear in the registration sheet that subjective testing stays mandatory for the test case on improved compression (section 4). On the other hand, submission could be done to test cases of constrained encoding runtime, with selection of zero cases to be included in subjective testing.

Follow-up discussion on Friday 10 July 1530. JVET-AQ0201v4 contains another version beyond JVET-AP2026v6 with further improvements suggested by editors, entitled as JVET-AP2026v7. This was presented. v7 shows changemarks relative to v6. It was noted that comments were not removed yet, but all issues from those had been resolved in some of the versions up to v7.

Some more changes were made during the session. The result was made available as JVET-AP2026v8 in JVET-AQ0201v4, once with changemarks and once as a clean version. The plan is to present this during the joint meeting with parent bodies planned for Thursday 14 July, and convert this into JVET-AQ2026.

The document was reviewed in the JVET main track on July 13th at 16:00h. Two minor changes were presented and agreed to be included in the draft CfP document. A corresponding update of JVET-AQ0201 was uploaded after the session.

JVET-AQ0220 [AHG17] Suggested Editorial Changes to the CfP Document [R. Chernyak (Tencent)] [late]

The attachment to this document contains several editorial changes to Annex D of the latest version of the Final Draft Joint Call for Proposals on Video Compression with Capability Beyond VVC (JVET-AQ0201).

See discussion under JVET-AQ0201

### Preparation of submissions and testing (3)

Contributions in this area were discussed during 1545–2000 on Wednesday 8 July 2026 (chaired by JRO).

JVET-AQ0101 AHG17: On characteristics of CfP test sequences [T. Chujoh, H. Kato, Y. Kidani, K. Kawamura (KDDI)]

This contribution reports the characteristics of the final test sequences for the Call for Proposals (CfP). Our objective is to ensure that the CfP subjective evaluation is conducted appropriately and to provide participants with useful information. At the previous meeting, JVET-AP0045 investigated the chroma sample locations of the test sequences using experimental results obtained from adaptive cross-component prediction (CCP) techniques. Since several sequences have been updated since then, this contribution provides updated information on the chroma sample locations and pixel value ranges of the final test sequences.

No specific action foreseen. No need to change CfP or subjective test.

It is recommended that care should be taken by proponents of test sequences in the future to follow expected properties of test sequences in terms of range and chroma location. However, running ECM may not be the best way to find this out, due to high encoder complexity.

It is noted that chroma location might implicitly be wrong with vertical sequences captured from smartphone (such as Gregory or Camellia).

JVET-AQ0202 AHG17: Suggested initial text of "Template for Proposal Description Documents of Responses to the Joint CfP on Video Compression with Capability beyond VVC" [J.-R. Ohm, M. Wien]

This document was developed based on JVET-I1003 (Template for Proposal Description Documents for Responses to the Joint CfP on Video Compression with Capability beyond HEVC). For convenience, a version with changemarks of that previous document is included in the zip container. It is suggested to include JVET-AO2040 (complexity reporting template) or an update thereof in the proposal description package template, which allows removal of two previous annexes. A first version of the proposal description template should be issued as an output document from the 43rd meeting, with the option of further updating into a final version by the time of the 44th meeting. It is further suggested not to attach the .csv files as part of the proposal description packages, as those are mandatory to be provided by proponents when submitting results, and can be made available to JVET members (in case that proponents agree or if the proposal is selected to be considered for further investigation in JVET) by the time of the 45th meeting. This would also require some further updates of wording in the final CfP JVET-AQ2027.

Initial discussion July 7 1700-1840 (chaired by JRO and MW)

It was expressed that it should be left to proponents if they want to include results from the main and supplemental submission packages. On the other hand, due to the cross-checking of csv files, it would be impossible to keep proponents anonymous.

It was also commented that including results to some extent is useful to get the full image about a proposal, but not to the detail that the csv files give.

Another important aspect that needs to be clarified (in the CfP): To which extent results will be made publicly available (agreement of proponents, at which point will proposals be identified, only subjective results?), and what will be done with the data collected in the csv files.

The following was agreed:

It is asserted to be important that proposals can be judged by both subjective and objective results, e.g. if subjective is not available for all encoder run times, or compare performance for both main and supplemental submission. The best solution for this is that the input report contains both without associating proposal numbers with description document numbers.

The individual objective results of a given proposal in the form they would be reported could be provided to proponents early enough to allow them a decision if they want to attach it to their description. At this moment, the proposal will no longer be anonymous.

Follow-up discussion on Wednesday 8 July, JVET-AQ0202v2 was presented that reflects the previous discussion by modifying section 4 in alignment with the newest version of the CfP; some wording improvements were made. In the discussion, it was further agreed that it is not practical to include submission of additional functionality as part of the main submission package as test conditions cannot be defined beforehand. Instead reporting of results on that functionality was in section 4.2.

Regarding sections 5 and 6 (decoder/encoder complexity analysis) it was suggested to better integrate the content of JVET-AO2040 word document, sections/tables on codec-level complexity analysis and module-level complexity analysis, investigate whether aspects need to be added for encoder complexity assessment from section 6. Furthermore, an Excel reporting template should be developed similar as the Excel from JVET-AO2040 for VVC module-level, to be filled by proponents containing

- 1 sheet on codec-level analysis, without run time that is available from .csv

- 1 sheet on module-level for decoder, with spare columns for additional modules

- 1 sheet on module-level for encoder, listing some default modules such as motion estimation, quantization, filter coefficients derivation, RDO for mode decision, etc.; this could be filled separately for the different test cases.

X. Li was asked to integrate a correspondingly extended version of JVET-AO2040 into JVET-AQ0202 in coordination with M. Wien for follow-up discussion.

Further edits on sections 2 and 3 were done in the subsequent discussion to add more detail on what should be described in the different sections. As it was not practical to do this in the large group, a side activity (P. de Lagrange, X. Li, J. Gan, R. Chernyak, E. Alshina, F. Bossen) was tasked to further continue on top of JVET-AQ0202v3, and also coordinating with the complexity reporting template prepared by X. Li.

The result was presented on Thursday 9 July at 1420-1530. So far, mostly section 2 was edited.

It was agreed that a better separation of sections 2.4 and 2.5 would be such that only elements that make use of reconstructed samples or need to generate prediction samples to determine parameters are subsumed under the latter.

Re-structuring, adding of missing sections, or removal of unnecessary sections should be done as appropriate.

A first version of integrating the complexity reporting template was presented, including a table with parameters for encoder complexity. As it is expected that for lower encoder complexity also the decoder module complexity and codec-level complexity may become lower (e.g. by disabling tools), a separate complexity reporting template of all three sheets should be submitted for each test case. It was suggested to include some mechanism to easily identify the differences (e.g. differential sheet, colour marking?).

It was suggested that the example complexity reporting template might also contain some aspects of ECM or NNVC modules, if modules of such type are not available in VTM.

The side activity was mandated to continue after the closing of the main session on Thursday, and met until 1800. The result was made available in JVET-AP0202v4 and was presented at 1600 in the main session before lunch break on Friday.

Further edits were made based on the discussion, with result uploaded as JVET-AP0202v5. Filling the subsections in 4 (encoder description) and section 7 (software implementation) still need to be filled with text description. Furthermore, some improvements on the complexity analysis (sections 5 and 6 plus spreadsheet) were deemed necessary (e.g., including internal bitdepth/register length, integer/float, etc.). It was requested to continue the side activity over the weekend to make the draft more complete, and conduct a follow-up review in JVET main track on Monday morning (to be chaired by F. Bossen).

Follow-up review on July 13 0900-1025 (chaired by FJB)

Further edits were made during the review of JVET-AP0202v6 (main document and attached spreadsheet). Most edits were editorial in nature. Clarifications were added, for example in section 4.3 where reporting of encoder optimization performance could also be made by disabling optimizations in the proposal. A new version JVET-AP0202v7 is to be uploaded.

JVET-AQ0202v7 was finally reviewed in the July 13th 16:00h session (chaired by M. Wien). The edits made over the weekend and in the morning were presented. On the decoder run time measurement, the activation of writing YUV at the decoder in order to assert full operation of post processing tools. It was confirmed that the VTM would operated accordingly in case the output is directed to /dev/null.

JVET-AQ0204 How to calculate average PSNR for a video sequence [J. Samuelsson-Allendes, S. Deshpande (Sharp)] [late]

This contribution relates to the calculation of average PSNR values for a video sequence. Two methods are compared;

- Picture-based PSNR (P-PSNR for short) where a PSNR value is calculated for each picture and the average is calculated across those PSNR values

- Sequence-based PSNR (S-PSNR for short) where the MSE is calculated across all sample values in the sequence and the PSNR value for the sequence is calculated from the sequence level MSE

The two methods are compared with a literature analysis, an industry practice analysis, a logical analysis, an empirical analysis and an evaluation on CTC data (for both codec-level and tool-level).

The proposal is to retire the use of P-PSNR and instead start using S-PSNR whenever PSNR numbers are reported for video sequences and when calculating BD-Rate numbers. It is suggested to perform the shift to S-PSNR before the evaluation of CfP responses and then use it consistently in the standardization phase of the upcoming video coding standard.

An analysis shown during the presentation indicates that the BD rate result ECM vs. VTM is almost identical for both types of PSNR. On tool level, temporal filter and LMCS were the only ones showing major differences (LMCS mainly for one sequence, MarketPlace).

It was commented that an extreme case where P-PSNR is wrong would be a black frame with zero MSE. Also sequences with scene cuts and PSNR largely different in the two parts are problematic. In particular, might also be problematic for screen content.

Conversion from data of picture-wise PSNR with 2-digit precision to MSE and from that to S-PSNR is possible.

Various experts also expressed preference for S-PSNR. It was however argued that any PSNR is still far away from visual quality.

One argument pro P-PSNR is that it better reflects the advantage of QP modifications of B hierarchies in terms of visual quality. This should however not be over-stressed as it may improve pumping. In comparing tools when using same temporal structure it may not be relevant which version to use.

It was generally agreed that it is a good point in time to move from P-PSNR to S-PSNR when entering development phase of a new video compression standard. Further investigation should be conducted until the next meeting based on the analysis script that is planned to be used, to study whether the precision foreseen for the frame-wise PSNR with the .csv files is sufficient for conversion into S-PSNR.

It is noted that further study is necessary if the switch to S-PSNR is also beneficial for WPSNR used for PQ content.

## AHG18 Ultra-low latency and packet loss resilience (2)

Contributions in this area were discussed during 1220–1300 on Friday 10 July 2026 (chaired by JRO).

JVET-AQ0183 AHG18: Forward Error Correction for ULL simulation software [I. Gribushin, K. Malyshev, G. Gotovskii, B. Shevchenko, S. Ikonin, E. Alshina (Huawei)]

During discussions of AHG18 ULL-related contributions, it was noted that traditionally error resilience is handled at the network or application level, e.g., by applying Forward Error Correction (FEC). This contribution describes experiments applying packet-based Reed-Solomon codes as FEC in the AHG18 ultra-low latency simulation software. The FEC functionality is implemented as part of the Network simulation application. Encoding is performed after packetization but before network transmission, whereas decoding is performed in the jitter-buffer upon packet reception, just before the received data is passed to the decoding process. The evaluation results demonstrate moderate improvement in specific configurations based on ULL CTC. It is proposed to include FEC functionality in the AHG18 simulation software and continue the study within AHG18.

It was commented that in re-transmission a packet-by-packet strategy may be unrealistic, rather than re-transmitting chunks of packets.

Decision(SW): integrate JVET-AQ0183 in AHG18 software

JVET-AQ0199 AHG18: Feedback-Aware Reference Preservation for Ultra-Low Latency [Y. Xiao, Y. Hu, X. Ma (MLS labs)] [late]

This proposal presents a feedback-aware reference picture preservation method for ultra-low latency cases. By designating the latest confirmed picture, which is successfully decoded, as an additional reference candidate, the proposed method improves robustness under lossy unicast PriorON transmission with minor impact on the original low-delay reference structure.

The experiments were conducted following the general test conditions specified in JVET-AN2039. The proposed method was evaluated in the ultra-low latency unicast PriorON test scenario. Different quantization parameters, latency constraints, and network trace conditions were tested to cover various lossy transmission cases.

It was commented that the proposal might require keeping more pictures in the reference picture memory. According to proponents, the number of reference pictures is kept identical, but the strategy which pictures are kept in the DPB is modified by modifying RPL management at the encoder side.

Generally, the idea is asserted to be interesting, but it should be investigated by in the AHG if it can be implemented without changing normative behaviour of VVC. As it shows improved performance under lossy conditions, it would be an interesting candidate to replace the current VTM anchor in the AHG18 CTC.

## Video CICP, generic aspects of metadata (1)

Contributions in this area were discussed during the closing plenary on Wednesday 15 July 2026 (chaired by JRO).

JVET-AQ0043 On the Placement of Metadata and Coordination with File Format [D. Podborski, A. Tourapis (Apple)] [late]

This document is a resubmissionof JVET-AP0283. This revision (v2) adds a characterization of the metadata under consideration together with a file-format-aware evaluation of specific metadata types

Some metadata may be more effectively carried at the system level than at the codec level. As the VSEI specification continues to expand — from 33 SEI messages in the 3rd edition to 48 in the 4th edition, with additional candidate SEI messages under consideration — it may be timely to reflect on whether each metadata type is being standardized at the most suitable layer of the media stack. The authors do not claim to have a definitive answer to this question, nor do they propose removing any SEI message or systematically replicating VSEI metadata at the file-format level. Rather, for a number of metadata types that are primarily descriptive of content or intended presentation, file-format-level representations in ISOBMFF may offer practical advantages, including improved accessibility, greater robustness in remuxing and transcoding workflows, and reuse across multiple codecs. This contribution shares observations and a possible checklist that the group may find useful for future metadata work, and suggests closer coordination between video coding and systems groups when such questions arise.

Was presented Tuesday 14 July 1020.

It was commented that standards defined in MPEG systems (such as ISOBMFF for which such an analysis is made in the document, comparing with functionality of some SEI messages proposed for v5) may not be the only specifications relevant at systems layer.

It was commented that additionally, better understanding of typical workflows of players etc. would be relevant to determine the value of metadata at different places of the overall stack.

It was commented that the message conveyed by the document is well appreciated, however among experts active in definition of SEI messages a good amount of knowledge exists about alternative solutions, as they are also participating other activities defining specifications at systems layers.

It was also commented that the table that had been generated and been used to identify SEI messages for inclusion in the VSEIv5 WD was already containing a column regarding the aspect of duplicating systems-layer solutions and additional benefit of an SEI solution, but this aspect should probably be analysed in a more systematic way, and also requested by proponents of new SEI messages to bring such information.

It was commented that an advantage of messages in VSEI is that they are developed in a more codec agnostic way, or that adaptation to different codecs is explicitly done as part of the development. That might simplify implementation.

CICP is another example of metadata specification that is also codec agnostic, but used more in the systems layer, also containing some SEI messages such as frame packing.

It was commented that the role of CICP is more about conveying identifiers, whereas SEI messages are more closely related to the video stream, and may contain a larger amount of syntax.

It was commented that in some cases it had been tried to keep common syntax and semantics of metadata when defined in different standards, but often not successful.

It was suggested by the proponents of JVET-AQ0043 that one possibility could be to define metadata in one standard, and define interfaces in another standard.

It was agreed that the aspects raised in the contribution and in the discussion should be severely considered in future developments. For example, the next generation video coding standard might be defined in a way such that it does not need to be changed when a new message is added to VSEI.

# Low-level tool technology proposals (17)

## AHG11/AHG14: Neural network-based video coding (14)

### Summary and BoG reports

Contributions in this area were discussed during 0910–1055 on Wednesday 8 July 2026 (chaired by JRO).

JVET-AQ0023 EE1: Summary report of exploration experiment on neural network-based video coding [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators)]

Summary of conducted tests

- EE1-1: LOP in-loop filter

Comparison point NNVC-LOP in-loop filter:

- Parameter counts: 242 k

- MACs/pixel: 16.5 k

  - EE1-1.1 – Dynamic convolution for LOP7 neural in-loop filtering JVET-AQ0071 (KHU, KBS). Cross-checker: Huawei (training JVET-AQ0178), Samsung  inference JVET-AQ0193).

- EE1-1.1.1: Integer model of dynamic convolution.

- EE1-1.1.2: 1.1.1 + Y–UV training-path refinement for balanced coding gain –priority #3 (withdrawn)

- EE1-1.1.3: 1.1.1/1.1.2 + Reduced training-time strategy.

All tests will have the same complexity

  - Parameter counts: 835 k

  - MACs/pixel: 16.52 k

In this test fixed-kernel static convolution layers in the LOP7 in-loop filter are replaced with dynamic convolution. Unlike static convolution, which uses a single fixed kernel for all inputs, dynamic convolution generates a per-sample effective kernel as a weighted combination of multiple expert kernels, where the weights are derived from the input feature via a lightweight router (fully connected layer) as shown in the two figures below.

Changes compared to LOP7 are illustrated in the second figure below: Blue-shaded CONV blocks are replaced with dynamic convolution (DynConv); the yellow-shaded CONV block indicates the chroma input convolution whose output channel count is d₂ (the parameter varied across sub-tests). Subtests use different number of expert kernels and different chroma input channels (d2).

The only difference between Tests EE1-1.1.1 and EE1-1.1.3 is faster training time. Proponent reports that Stage 3 training for EE1-1.1.1 requires 9.4 days, but for EE1-1.1.3 the same training requires only 5.4 days. For anchor (LOP7) the same training requires 4.6 days.  The training configuration used in EE1-1.1.3 relies on optimizations designed for NVIDIA Ampere (sm_80) or newer architectures, including Automatic Mixed Precision (AMP), torch.compile, and cudnn.benchmark. Accordingly, the reported training time and memory usage are representative only on Ampere-class or newer GPUs (e.g., A100, RTX A6000 Ada, and H100). On earlier architectures, such as the V100, these optimizations may not be fully supported, resulting in different memory usage and training time.

Difference between static and dynamic convolutions.

Process of router-based expert kernel generation.

Modifications compared to LOP7

EE1-1.1 tests summary

| Test | Cross Check | Training Stage 3 | #ExpertKernels | d2 | kMAC/pixel | # Pars | RA cfg, BD-Rate (PSNR) |  |  | RunTime |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Y | U | V | Enc | Dec |
| LOP7 |  | 4.6 | 1 | 6 | 16.54 | 242 | 0.0% | 0.0% | 0.0% | 100% | 100% |
| EE1-1.1.1 | Inference Training | 9.4 | 4 | 4 | 16.52 | 835 | -0.6% | -3.7% | -3.1% | 97% | 125% |
| EE1-1.1.3 | Inference | 5.4 | 4 | 4 | 16.52 | 835 | -0.7% | -4.9% | -4.4% | 89% | 119% |

Feedback from the cross-checkers:

For both EE1-1.1.1 and EE1-1.1.3 perfect match was observed during inference cross-check.

After re-training of EE1-1.1.1 the BD-rate difference (Y component) of 0.1% (RA cfg) and 0.0% (all intra cfg test) was observed.

Re-training of EE1-1.1.3 is still ongoing (about a half of epochs is done).

Observations /conclusions:

- dynamic convolution introduced into NN-based LOP in-loop filter, improve compression performance 0.6..0.7% BD-rate (same kMAC/pxl, 4 larger model)

- The increase of training time compared to the LOP7 has been reduced to just 20% (5.4 days vs 4.5 days)

From discussion in JVET Wednesday 8 July:

- Gain of 1.1.1 and 1.1.3 is attractive, kMAC not increased relative to LOP7, but both come with substantial increase of memory usage, and increase in training time (less for 1.1.3, but training crosscheck not yet finished).

- Also increase in decoding time (on CPU), and could also be more difficult to implement on GPU.

- For the NNVC exploration, the dynamic convolution is interesting, even though it may be less practical for implementation than LOP7 – Candidate for adoption.

In the discussion in the JVET session on July 13th 1600h it was confirmed by the software coordinator that the SADL implementation has a sufficient quality and does not break existing tools.

It was further reported that EE-1.1.1 has been confirmed previously. The simulations confirming the performance of EE-1.1.3 were started after successful training. It was considered unlikely to receive full results of the simulations by the end of the meeting. It was suggested to conditionally adopt EE-1.1.3 based on confirmation in the AHG phase. Otherwise, EE-1.1.1 would be adopted. EE-1.1.1 was suggested to be integrated into the NNVC software and potentially be replaced later upon successful performance confirmation.

Decision: Adopt JVET-AQ0071 EE1-1.1.1. as LOP8 (also replacing LOP7 in NNVC default). If the training crosscheck of EE1-1.1.3 is successful by the time of the AHG telco planned on July 30, the model from 1.1.3 shall be used instead for LOP8.

- EE1-2 NN-Inter

Comparison points LDRF1 / HDRF1:

- Parameter counts: 3600 k / 6064 k

- MACs/pixel:     62 k / 477 k

  - EE1-2.1 – Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement  JVET-AQ0047 (Wuhan Uni) Cross-checker: InterDigital (inference JVET-AQ0137),  Huawei (training and inference - JVET-AQ0217)

EE1-2.1/2 VLDRF / VLDRF(16)

- Parameter counts: 243 / 257 k

- MACs/pixel: 8.8 / 9.7 k

  - EE1-2.2 – Improved H-DRF with Weighted Fusion and Optimized YUV Processing  JVET-AQ0048 (Wuhan Uni) Cross-checker: InterDigital (inference JVET-AQ0137),  Huawei (model quantizer and inference – JVET-AQ0206, JVET-AQ0216)

- Parameter counts: 6064 k

- MACs/pixel: 477 k

Changes are

  - Frame fusion is changed from simple averaging to HardSigmoid-based (analog of clipping) weighted fusion.

  - YUV-to-RGB conversion is removed from optical-flow computation by keeping H-DRF and L-DRF inference in the YUV domain.

  - The down/up sampling method for YUV420/YUV444 of H-DRF is changed from bilinear to Lanczos-3, aligning with the L-DRF inference procedure.

  - The quantization method for the GridSample layer is optimized.

Processing flow of the GridSample layer

The figure above shows the processing flow of the “GridSample” layer. “GridSample” uses the input tensor and optical flow to generate a flow-warped output tensor. During grid generation (green block), terms such as 2/(W-1) and 2/(H-1) are involved, which can introduce additional error during float32 to int16 conversion. Therefore, quantization precision of grid generation (green block) is increased when W/H become larger to improve coding performance.

The quantization operation is performed as:

In the previous implementation, the quantization parameter for the grid generation part was fixed to Q=11 for all spatial resolutions.

In the optimized version, the quantization parameter is empirically selected according to the spatial resolution. Q=14 is essential for 256×256 to reduce the quantization error, while lower Q values are sufficient for other resolutions with limited performance impact:

This adaptive quantization improves the precision of the grid generation process and reduces the float32-to-int16 conversion error.

The GridSample grid generation contains two parts. The base grid generation is handled during the offline quantization process. During inference, the optical flow also needs to be normalized to the [-1,1] coordinate system by applying the scale factors 2/(W−1) and 2/(H−1) before being added to the base grid.

    - EE1-2.2.1 changes are applied to the HDRF (the most more complex DRF model in NNVC)--> HDRF2

    - EE1-2.2.2 same changes applied to the LDRF (middle complexity DRF model in NNVC)-->LDRF2

  - EE1-2.3 – Deep Reference Frame Generation for Inter Prediction Enhancement with motion compensation JVET-AQ0049  (InterDigital) Cross-checker: Wuhan Uni (only inference, training is not modified JVET-AQ0134)

Illustration of the motion compensation of the input patches 0 and 1 (EE1-2.3)

The patches used as input of a DRF models are first displaced (pel motion) using the collocated motion vector.

For YUV420/YUV444 instead of bi-linear of Lanzoc-3 (EE1-2.2) VVC-RPR filters are used.

  - EE1-2.4 – Combination of test 2.3 (Deep Reference Frame Generation with motion compensation) and tests 2.1 and 2.2 JVET-AQ0050 (Wuhan Uni, InterDigital) Cross-checker: Huawei (training and inference – JVET-AQ0213)

EE1-1.2 tests summary

| Legend | Tests | kMAC/pixel | Pars,K | BD-Rate (PSNR) |  |  | BD-Rate (MS-SSIM) |  |  | RunTime |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Y | U | V | Y | U | V | Enc | Dec |
| vs NNVC 17.1 (LOP filter on, NN-Intra On) |  |  |  |  |  |  |  |  |  |  |  |
| VLDRF | EE1-2.1.1 | 8.8 | 243 | -0.4% | -0.3% | -0.6% | -0.8% | -0.7% | -0.8% | 101% | 381% |
| VLDRF(16) | EE1-2.1.2 | 9.7 | 257 | -0.5% | -0.6% | -0.5% | -0.9% | -0.9% | -0.9% | 101% | 275% |
| HDRF2 | EE1-2.2.1 | 477 | 6064 | -2.8% | -3.0% | -3.1% | -3.2% | -3.7% | -3.6% | 147% | 4258% |
| HDRF1 | HDRF1 | 477 | 6064 | -2.5% | -1.0% | 0.1% | -2.9% | -2.1% | -1.7% | 118% | 3343% |
| LDRF2 | EE1-2.2.2 | 62 | 3600 | -1.7% | -1.3% | -0.5% | -2.4% | -2.0% | -2.1% | 111% | 1057% |
| LDRF1 | LDRF1 | 62 | 3600 | -1.4% | 0.5% | -1.5% | -2.1% | -1.3% | -2.0% | 111% | 1051% |
| EE1-2.3-LDRF1 | EE1-2.3.1 | 62 | 3600 | -1.9% | 1.0% | -2.2% | -2.7% | -1.7% | -2.6% | 108% | 1134% |
| EE1-2.3-HDRF1 | EE1-2.3.2 | 477 | 6064 | -3.1% | -2.5% | -1.6% | -3.6% | -3.4% | -3.0% | 136% | 3726% |
| EE1-2.3-VLDRF | EE1-2.4.1 | 8.8 | 243 | -0.6% | -0.2% | -0.6% | -1.1% | -0.9% | -1.1% | 101% | 359% |
| EE1-2.3-VLDRF(16) | EE1-2.4.2 | 9.7 | 257 | -0.8% | -0.6% | -0.4% | -1.3% | -1.1% | -1.2% | 100% | 293% |
| EE1-2.3-LDRF2 | EE1-2.4.3 | 62 | 3600 | -2.0% | -1.9% | -0.9% | -3.0% | -2.6% | -2.8% | 109% | 1149% |
| EE1-2.3-HDRF2 | EE1-2.4.4 | 477 | 6064 | -3.2% | -3.9% | -4.0% | -3.7% | -4.3% | -4.1% | 136% | 3753% |
| vs NNVC 17.1 as VTM (LOP filter off, NN-Intra Off) |  |  |  |  |  |  |  |  |  |  |  |
| VLDRF(16) | EE1-2.1.3* | 9.7 | 257 | -0.9% | -1.0% | -1.2% | -1.2% | -1.2% | -1.2% | 102% | 3982% |

Feedback from the cross-checkers:

EE1-2.1.1 was successfully cross-checked (training and inference).

EE1-2.2.1 (HDRF) training is still not yet cross-checked successfully

EE1-2.2.2 (LDRF) after re-training the difference is 0.13% (BD-rate Y).

Inference cross-check is succesful.

DRF test results visualized: BD-rate vs complexity

Observations /conclusions:

- both encoding and decoding run time for LDRF relatively to NNVC has been reduced compared to the last EE1 round (likely due to the SADL optimization).

- Improvements from EE1-2.2 (Weighted Fusion and Optimized YUV Processing) improve performance of both LDRF and HDRF by 0.3%.

- Patched displacement from EE1-2.3 improve compression performance of all versions of DRF by 0.3..0.4%. This change is generic and complementary to improvements from EE1-2.2 as was shown in EE1-2.4.

From discussion in JVET July 8:

- 2.1.1 and 2.1.2 (VLDRF) successfully crosschecked, provide gain of 0.4%/0.5%, seem only partially overlapping with other NNVC tools (2.1.3*).

- With similar changes to optical flow estimation applied to HDRF (2.2.1) and LDRF (2.2.2) additional gain is achieved, but decoder runtime shows substantial increase in particular in case of HDRF

- 2.3.x is adding a global motion estimation to initialize the local patches in the existing models of LDRF and HDRF, and uses correct chroma location of sequences in interpolation/downsampling

- 2.4.x is applying 2.3.x to new models of VLDRF, LDRF, and HDRF.

- 2.4.2 shows 0.8% gain for VLDRF(16), 2.4.3 2% gain for LDRF, both fully cross-checked

- Opinion was expressed that, though the tradeoff of best VLDRF vs. decoding time increase of 3x is not attractive as such, it would not put a burden in running the NNVC default (encoding time is not increased)

Decision: Adopt JVET-AQ0050, containing 2.4.2 VLDRF(16) to be added to the NNVC default, 2.4.3. to replace current LDRF1 (to become LDRF2), and 2.4.4. to replace current HDRF1 (to become HDRF2)

Follow-up discussion on Tuesday 14 July, to reflect some intent in the original discussion on July 8, where the following was forgotten to be marked as revisit. EE1-2.2.1 (HDRF modifications) cross-check completed, was added to the adoptions under JVET-AQ0050.

- EE1-3: Single layer framework for externally coded pictures

Withdrawn

- EE1-4: NN-Intra bit-exact reproducibility

Comparison point NNVC-NN-Intra:

- Model weights - 16 bits, ‘off-line’ quantization

- Activations - 16 bits (bit-depth of temporal buffer between convolutions)

  - EE1-4.1 – Adaptive Quantization and Hardware Optimization for NNIP JVET-AQ0182  (Qualcomm) Cross-checker: Huawei (inference, training is confirmed in earlier tests – JVET-AQ0218), InterDigital (inference – JVET-AQ0205).

- EE1-4.1.1, 10-bit dynamic activation quantization and 8-bit static weight quantization: 0.00% luma loss in RA configuration.

- EE1-4.1.2, 8-bit dynamic activation quantization and 8-bit static weight quantization: 0.03% luma loss in RA configuration.

- EE1-4.1.3: withdrawn?

- EE1-4.1.4a, 10-bit static activation quantization and 8-bit static weight quantization: 0.18% luma loss in RA configuration.

- EE1-4.1.4b, 12-bit static activation quantization and 8-bit static weight quantization: 0.04% luma loss in RA configuration.

- EE1-4.1.4c, 14-bit static activation quantization and 8-bit static weight quantization: 0.00% luma loss in RA configuration.

Dynamic quantization approach, each 8-element activation block is quantized at runtime based on its local dynamic range. For every 8-element block, the maximum absolute value is determined, and a corresponding right shift is computed so that all values fit within the target bit-depth.

Static quantization in this test: for each 8-element block, required shift values are collected off-line over a representative dataset (BVI-AOM), and a single shift value is selected based on a predefined criterion. During inference, each block is quantized using its precomputed shift and clipped to the target bit-depth. During the matrix multiplication, the applied block-wise shifts are compensated in the accumulation process to preserve the correct numerical scaling.

Minor mismatch between results from proponent and cross-checkers have been observed (4th digit after decimal point). Proponent and cross-checkers exchanged streams, logs, found perfect match in per-frame results. So the issue relates to parsing scripts.

EE1-4.1 Trade-offs results (BD-rate numbers is from random access configuration test).

Beside of bit-depth reduction for weights and activations two changes which speed up NNIP have been introduced:

- Fusion of Activation Functions into Matrix Multiplication (leaky ReLU is fused with fully connected layer) – modification of NNIP which potentially changes results (less values will be clipped)

- SIMD-Friendly Sparse Matrix Multiplication (sparse weight matrices are packed in block sizes aligned to SIMD execution 8-elements units) – implementation aspect, doesn’t change results

Model parameters quantized statically in all tests. Activations are quantized dynamically or statically. Shift values for each 8-elements block becomes a part of the model. Total number of parameters increases by 12.5%. Since bit-depth of model parameters reduced to 8 bits (in all tests) the total size of memory for model parameters is reduced by half (dynamic quantization) and by 43% in case of static quantization. In the table below, the Enc and Dec run time are from proponent.

One cross-checker also performed limited test out-side of NNVC CTC, using synthetic image.

EE1-1.3 tests summary (anchor NNVC17.: LOP filter on, NN-Intra on)

| Legend | Tests EE1- | Num Par., M | Par. Mem, MB | Random Access cfg |  |  |  |  | All Intra cfg |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| stat. a16w16 | NNVC | 4.5 | 8.9 | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% |
| dyn. a10w8 | 4.1.1 | 4.5 | 4.5 | 0.0% | -0.1% | 0.0% | 101% | 100% | 0.0% | 0.0% | 0.1% | 101% | 100% |
| dyn. a8w8 | 4.1.2 | 4.5 | 4.5 | 0.0% | -0.1% | 0.0% | 101% | 100% | 0.1% | 0.1% | 0.2% | 101% | 101% |
| stat. a10w8 | 4.1.4a | 5.0 | 4.8 | 0.2% | 0.0% | 0.1% | 101% | 99% | 0.6% | 0.5% | 0.6% | 101% | 95% |
| stat. a12w8 | 4.1.4b | 5.0 | 4.8 | 0.0% | -0.0% | 0.0% | 101% | 100% | 0.2% | 0.1% | 0.1% | 100% | 99% |
| stat. a14w8 | 4.1.4c | 5.0 | 4.8 | 0.0% | 0.0% | 0.1% | 100% | 99% | 0.0% | 0.0% | 0.0% | 101% | 100% |

Observations /conclusions:

- static quantizing of NNIP model parameters to 8 bits doesn’t lead to the performance degradation

- additionally dynamic quantizing of activations in NNIP model to 8 bits lead to the negligible performance degradation (0.0% random access and 0.1% in all-intra tests)

- in performed tests dynamic quantizing of activations in NNIP increases decoding run time 5% compared to the static quantizing (stat. sa10w8 and dyn. a10w8 comparison)

- the performance of NNIP is much more sensitive to the bit-depth of activations: static quantizing of activations to 14 bits – no drop, to 10 bits 0.6% BD-rate drop in ‘all intra’ test.

- bit-depth of model parameters has been reduced to 8 bits, so the total size of memory for model parameters is reduced by half in case of dynamic quantization and by 43% in case of static quantization.

From discussion in JVET July 8:

- The main intent of EE1-4.x is to avoid overflow that could accidentally happen in NN intra, and could cause mismatch between different devices.

- Cross-check JVET-AQ0218 shows for the example of a non-CTC image (screen content with large flat area) that dynamic quantization (4.1.1/4.1.2) provides >1% better performance than static. It is asserted that this may not be too relevant as the image is simple.

- Concern was raised that the dynamic quantization solutions could be problematic in SADL.

- Among the static solutions, 4.1.4c has practically no loss under CTC, and would be a safe choice, solving the issue without problems in implementation.

- It was commented that in a standard specification, it would need to be clearly defined how to handle overflow such that no encoder/decoder mismatch occurs. In this context, implementation friendliness as well as performance would be relevant. In the context of the NNVC exploration, 4.1.4c is a suitable solution to solve the issue.

The adoption of 4.1.4c was discussed in the session of July 13th 1600h. It was reported that the implementation inside SADL would imply difficulties and that the logic pertaining to model quantization should be integrated into the NNVC software. It was reported that some small changes to SADL would still be expected. It was agreed tentatively adopt 4.1.4c subject to review of the implementation in a early meeting of the AHG and confirming the appropriateness of the integration.

Decision: Conditional adoption of JVET-AQ0182 EE1-4.1.4c. To be confirmed by AHG telco on July 30, provided that the corresponding is confirmed to be appropriate by the software coordinator.

JVET-AQ0042 [AHG11] [AHG14] Teleconference on NNVC [E. Alshina, F. Galpin]

### EE1 contributions: Neural network-based video coding (6)

There was no presentation or discussion about specific proposals in this category – contributions were discussed in the context of the EE summary report JVET-AQ0023.

JVET-AQ0047 EE1-2.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement [T. Shu, X. Chen, W. Zhang, N. Fu, Z. Chen (Wuhan Univ.)]

JVET-AQ0217 Crosscheck of JVET-AQ0047 (EE1-2.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) [N. Bhaskar, T. Solovyev (Huawei)] [late]

JVET-AQ0048 EE1-2.2: Improved H-DRF with Weighted Fusion and Optimized YUV Processing [X. Chen, J. Zhang, Z. Chen (Wuhan Univ.)]

JVET-AQ0137 Crosscheck of JVET-AQ0047 (EE1-2.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) and JVET-AQ0048 (EE1-2.2: Improved H-DRF with Weighted Fusion and Optimized YUV Processing) [P. Bordes, F. Galpin (InterDigital)]

JVET-AQ0206 Crosscheck of JVET-AQ0048 (EE1-2.2.2: Improved L-DRF with Weighted Fusion and Optimized YUV Processing) [F. Kozhamkulova (Huawei)] [late]

JVET-AQ0216 Crosscheck of JVET-AQ0048 (EE1-2.2.1: Improved H-DRF with Weighted Fusion and Optimized YUV Processing) [N. Bhaskar, F. Kozhamkulova, T. Solovyev (Huawei)] [late]

JVET-AQ0049 EE1-2.3: Deep Reference Frame Generation for Inter Prediction Enhancement with motion compensation [P. Bordes, F. Galpin, F. Lo-Bianco, M. Paquiry (InterDigital)]

JVET-AQ0134 Crosscheck of JVET-AQ0049 (EE1-2.3: Deep Reference Frame Generation for Inter Prediction Enhancement with motion compensation) [X. Chen, Z. Chen (Wuhan Univ.)]

JVET-AQ0050 EE1-2.4: Combination of test 2.3 (Deep Reference Frame Generation with motion compensation) and tests 2.1 and 2.2 [P. Bordes, F. Galpin, F. Lo-Bianco, M. Paquiry (InterDigital), X. Chen, T. Shu, J. Zhang, Z. Chen (Wuhan Univ.)]

JVET-AQ0213 Crosscheck of JVET-AQ0050 (EE1-2.4: Combination of test 2.3 (Deep Reference Frame Generation with motion compensation) and tests 2.1 and 2.2) [F. Kozhamkulova, N. Bhaskar, T. Solovyev, E. Alshina (Huawei)] [late]

JVET-AQ0071 EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering [J. Lee, M. Jeon, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)]

JVET-AQ0178 Crosscheck of JVET-AQ0071 (EE1-1.1.1 and EE1-1.1.2: Dynamic convolution for LOP7 neural in-loop filtering) [V. Khamidullin, S. Ikonin, E. Alshina (Huawei)]

JVET-AQ0193 Crosscheck of JVET-AQ0071 (EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering) [N. J. Gadgil, M. N. Hosmane, R. N. Gadde, W. I. Choi, K. P. Choi (Samsung)] [late]

JVET-AQ0182 EE1-4.1: Adaptive Quantization and Hardware Optimization for NNIP [A. Akhtar, S. Esenlik, Y. Matsuba, M. Karczewicz (Qualcomm)]

JVET-AQ0205 Cross-check of JVET-AQ0182 (EE1-4.1: adaptive quantization and hardware optimization for NNIP) [T. Dumas (Interdigital)] [late]

JVET-AQ0218 Cross-check of JVET-AQ0182 (EE1-4.1: adaptive quantization and hardware optimization for NNIP) [A. Chandran (Huawei)] [late]

### EE1 related and beyond-EE contributions: Neural network-based video coding (6)

Contributions in this area were discussed during 1120–1225 on Wednesday 8 July 2026 (chaired by July).

JVET-AQ0146 EE1-related: chroma location information handling in DRF (EE1-2.3 and EE1-2.4) [F. Galpin, P. Bordes (InterDigital)]

This contribution presents additional results on the correction/improvement on the chroma location handling during processing of input/output of the tensors sent to DRF used in EE1 2.3 and EE1 2.4.

Contribution for information – no specific action.

JVET-AQ0151 AHG11] [AHG14] Huawei response to MPEG survey on AI hardware [E. Alshina, Y. Zhao (Huawei)]

This contribution is a copy of ISO SC 29 WG2 contribution which provides a response from Huawei to MPEG survey on AI hardware.

For information – no need for presentation.

JVET-AQ0152 Runtime and Energy Consumption Estimation for Neural Network-based In-loop Filters in NNVC [S. Caglar, Z. Chen, M. Y. Saritas, H. B. Dogaroglu, C. Eteke, E. Steinbach (TU Munich)]

This informative contribution investigates runtime and energy consumption measurement and estimation for neural network-based in-loop filtering in NNVC. The results show that MAC count alone is not sufficient to fully describe the complexity of CNN runtime and energy. For the LOP7 model inference on the NVIDIA A40 GPU, PReLU operation accounts for 9.7% of runtime and 11.3% of energy, while other zero-MAC operations, such as Slice, Concat, Max, Min, Resize, and MaxPool, together contribute 13.6% of runtime and 11.8% of energy.

The empirical results show that, on CPU, the runtime and energy consumption of CNNs can be estimated with MAPEs of 3.79% and 38.71%, respectively, using kernel-level estimation models augmented with a linear correction term. For GPUs, kernel-level estimation models can also provide useful runtime and energy estimates. However, kernel-level GPU measurements deviate from full-model measurements more than on CPU, due to inter-layer interactions and hardware-specific optimizations. To address this, we introduce a cumulative layer-wise measurement method that enforces consistency between layer-level and full-model runtime and energy.

Further, runtime and energy impact of LOP7 inside the NNVC decoder for a single frame are investigated. On the AMD CPU platform, enabling LOP7 increases the decoder runtime by 271.23 ms and the net energy by 3.54 J. On the MacBook M4 Pro CPU, LOP7 adds 437 ms and 3.14 J in the decoder. We also compare these decoder-integrated measurements with isolated LOP7 model inference with a different execution backend to show the gap between standalone model execution and the full decoder integration.

The runtime and energy measurement scripts used in this work are publicly available at https://github.com/caglar-se/nn-runtime-energy-measurement

Presented Friday 10 July 1300.

It was commented that the comparison CPU vs. GPU was not in all cases under comparable conditions, e.g. int vs. float, etc., and that the measurements might largely change when further optimization would be conducted. It might also depend on the way ONNX compiled the graph per layer.

It was asked if it would be possible to use an external tool. At least in comparing two different tools operated in same environment it should be possible.

It was commented that the results reported appeared to be incomplete for the cases of Macbook and phone.

Contribution for information – no specific action at this moment.

JVET-AQ0172 AHG11: VLOP4 Modification for Complexity Reduction [H. Cho, J. Kim, S. Bahk, T. Lee, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI)] [late]

This contribution modifies VLOP4 by using coding parameter-guided feature modulation to reduce computational complexity. Specifically, this contribution proposes to process the coding parameters among the VLOP4 inputs, namely QPbase, QPblock, and IPB information, through a separate operation and use them to scale and shift intermediate features. The proposed method reduces the complexity while maintaining the Y BD-rate performance. The proposed method has been implemented based on VLOP4 of NNVC-16 using SADL integer operations.

- Complexity

  - Parameter counts: 69.2k (VLOP4 Anchor: 72.72k)

  - MACs/pixel: 4.84k (VLOP4 Anchor: 5.09k)

- Over VLOP4

  - RA: {0.01%, 2.05%, 1.71%}

  - AI: {-0.04%, 1.44%, 1.94%}

Additionally, the Random-Access performance results for the CfP contents are as follows.

- Over VLOP4

  - RA: {-0.05%, 3.2%, 1.54%}

It was asked whether the scaling and shift used in the CPFM block is necessary in terms of compression benefit.

It was asked if the large drop in chroma is due to replacement of 3x3 convolution by 1x1? Has not been investigated – proponent reports that they are currently running an additional test with other luma/chroma balance in training.

It was commented that the benefit of using local QP as input might be rather low in CTC where constant QP is used.

Only small interest was expressed for investigation in EE at this moment. Further study recommended along the questions above.

JVET-AQ0173 [AHG11][AHG 14] Real-Time Implementation of an End-to-End AI Video Codec [S. Cizel, R. Mullakhmetov, A. Berk, C. Finlay, F. Galpin, E. François (InterDigital)]

This contribution presents a demonstration of an end-to-end low-delay AI codec running on consumer-grade devices using built-in AI acceleration. The implementation relies on publicly available neural-accelerator APIs and achieves real-time Full HD playback on both laptop and mobile platforms. The demonstration made at the 43rd JVET meeting in Geneva shows the practical feasibility of fully AI-based video codec execution on personal devices.

MacBook Pro (2023):

- 10-core CPU with 6 performance cores and 4 efficiency cores

- 16-core GPU

- 16-core Neural Engine

- 200GB/s memory bandwidth

- 16 GB RAM

- Metal 3 Support

- MacOS 15.3.1

iPhone 15 Pro (2023)

- A17 Pro chip

- 6‑core CPU with 2 performance and 4 efficiency cores

- 6‑core GPU

- 16‑core Neural Engine

- iOS 18.2

- Battery Health: 98%

The results of the measurements for 1080p are summarised in the following table:

| Device | Encode FPS | Decode FPS |
| --- | --- | --- |
| Macbook Pro (M2Pro) | 18 | 62 |
| iPhone15 Pro (A17 Pro) | - | 50 |

It was asked why the encoder is slower? Larger network is used.

It was commented that it would be interesting to compare against a conventional encoder in terms of run time, battery consumption, and quality at same bit rate.

A demo is announced for the afternoon coffee break.

In the device architectures, the memory transfer between MPU and CPU is minimum.

Implementation in FP, but as long as IoS is used the interoperability between devices is not a problem.

It was commented that integer implementation might give further speedup.

In terms of compression, comparable to HEVC LD visually

Contribution for information – no specific action.

JVET-AQ0200 [AHG11] Training-Time Optimization for LOP7 [J. Lee, M. Jeon, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)] [late]

This contribution reports the validation of the training-time reduction strategy from JVET-AQ0071 on the conventional fixed-kernel LOP7 anchor within NNVC 17.1. In the implementation, three acceleration techniques—cuDNN benchmarking, torch.compile, and Automatic Mixed Precision (AMP) with GradScaler—are employed to improve computational and memory efficiency. Experimental results show that the proposed training strategy achieves a 31.3% reduction in overall training time (i.e., 1.45× speedup). Despite this substantial reduction in training time, the coding performance remains essentially unchanged relative to the NNVC 17.1 LOP7 anchor

The training was not changed.

Contribution for information – no specific action.

### SADL and NNVC implementation, CTC (2)

Contributions in this area were discussed during 1225–1245 on Wednesday 7 July 2026 (chaired by JRO).

JVET-AQ0138 AHG14: SADL Update [F. Galpin (InterDigital)]

New in version v17

Improvements

  - MR210/MR201: JVET-AO0073 speed-up

  - MR219: documentation update

  - MR217: grid sample improvement

  - MR216: conv2D transpose improvement

  - MR215: conv2D stride 3 improvement

New

  - MR218: flexible memory management

Fixes

  - MR211: fix issues for sparse Tensor. Was only affecting float sparse Tensor.

For information – already implemented in SADL.

JVET-AQ0171 AhG14: CUDA backend for SADL [F. Galpin, S. Cizel (InterDigital)]

This contribution presents a CUDA backend for the SADL inference library, enabling GPU execution of int16 neural network models with no change to the existing model files and bit-exact results with respect to the CPU implementation. Across the tested models, the speedup on pure inference ranges from about x8 to x35. On the NNVC anchor, this translates to a ~x5 decoder speedup.

Several experts commented this is very useful, as GPU implementation of NN technology is more realistic than GPU. Runtime report on GPU is already in CTC as an option.

Overall benefit on runtime saving highly depends on data exchange/memory transfer and allocation of tasks between CPU and GPU.

Decision(SW): Adopt JVET-AQ0171

## AHG6/AHG12: Enhanced compression beyond VVC capability (3)

Contributions in this area were discussed during 1245–1310 on Wednesday 8 July 2026 (chaired by JRO).

JVET-AQ0198 Visualization of ECM Progress: Capturing Faint Prints of Tool Evolution AHG 6 [T. Ikai, K.-W. Liang (Sharp)] [late]

The Enhanced Compression Model (ECM) has been developed through multiple exploration phases, during which a large number of coding tools and syntax elements have been introduced, modified, or removed. While coding-efficiency measurements remain the primary criterion for evaluating coding technologies, they do not always provide a direct view of how the model structure and syntax space have evolved over time. The analysis focuses on static characteristics such as source-code growth, macro-based tool adoption, and the number of unique normalized syntax elements observed across ECM versions.

The intention of this analysis is not to replace coding-efficiency or complexity measurements. Instead, it is intended to complement such evaluations by providing a reproducible overview of the ECM exploration process from a syntax and source-code perspective.

No specific action

JVET-AQ0192 Improvements of NS2 software at 0.2x under CfP conditions [G. Laroche, P. Onno, B. Galmiche, N. Ouedraogo (Canon)]

This informative contribution reports results of a set of modifications of the NS2 software evaluated under the CfP conditions. In particular, the proposed modifications target the 0.2x and 1.0x configurations of the CfP. These modifications include adopted ECM technologies, enhancements to existing NS2 technologies and new added technologies.

When combined, these modifications achieve a YUV BDR improvement of 13% at 0.2x encoder runtime, and almost 20% at 1.0x encoder run time, compared with VTM-24.0 operating at 1.0x encoder runtime.

4 changes: RPR resampling and decision modification  provides highest speedup, then temporal prediction of affine, clipping refinements and temporal partition prediction.

Contribution for information – no specific action.

JVET-AQ0214 Cross-check of JVET-AQ0192 “Improvements of NS2 software at 0.2x under CfP conditions” [F. Le Léannec (InterDigital)] [late]

JVET-AQ0194 AHG10/AHG12: Clean-Slate NextSoftware2 implementation without future Video Coding Tools [W. Ahmad, K. Andersson, L. Litwic, M. Pettersson, R. Sjöberg, N. Stegmaier, J. Ström, P. Wennersten (Ericsson), C. Bartnik, J. Brandenburg, B. Bross, V. George, J. Güther, G. Hege, C. Helmrich, A. Henkel, T. Hinz, C. Lehmann, D. Marpe, T. Nguyen, J. Pfaff, T. Schierl, H. Schwarz, B. Stallenberger, K. Sühring, A. Wieckowski, T. Wiegand (Fraunhofer HHI), P. Astola, S. Blasi, F. Cricri, D. Bugdayci Sansli, C. Feldmann, D. Fortin, J. Funnell, A. Hallapuro, M. M. Hannuksela, S. Hong, I. Jumakulyyev, J. Lainema, N. Le, N. Neumann, J. Ridge, D. Rusanovskyy, S. Schwarz, H. Zhang, N. Zou (Nokia)] [late]

In the JVET CfE, Ericsson, Fraunhofer HHI and Nokia made a response based on an alternative implementation of many of the well-studied ECM tools. Together with some improvements on the VTM base of the software and optimized configurations, fast working points with good compression performance were presented.

To enable the study of the proposed software basis relative to its two bases, VTM and ECM, we propose to share the pure VTM-based base (without block-size restrictions) with the applied changes. The removal of ECM tools removes the controversy of tools being on or off, and allows to provide a more clean and readable software-base, to study encoder-side and implementation specific aspects. Additionally, the clean slate software provides a reasonably generally optimized software base for further encoding algorithm experimentation, without being standard specific and thus penalizing novel developments.

Was presented Friday 10 July 1325.

The external intra picture is similar to the “single layer” method in NNVC investigation, and also an interface for NNLF is included.

Normative changes which are not fully documented: ISP is removed, ALF is different from VTM.

The speedup comes from both software improvements and some modifications of encoder (including partitioning description).

Decision(SW/BF): Adopt software from JVET-AQ0194 into the AHG12 repository of JVET-AN0267.

It was also requested to clearly state in an update of JVET-AQ0194 which normative modifications to VVC were made at tool level, and that decoding normative VVC streams generated from VTM is also not possible due to HLS changes in general.

# High-level syntax (HLS) and related proposals (129)

## AHG9: Study of SEI messages in VSEI, VVC, HEVC and AVC (3)

Contributions in this area were discussed during 0900–1005 on Wednesday 8July 2026 (chaired by J. Boyce).

JVET-AQ0061 AHG9: Errata report on VSEI and HEVC [Y. He, M. Karczewicz (Qualcomm), Y.-K. Wang (Bytedance)]

This contribution reports an erratum of the ACI SEI message in VSEI v4 and HEVC v11.

Agreed.

JVET-AQ0089 AHG2/AHG9: Errata on general SEI signalling mechanism for VVC, VSEI, HEVC, and AVC [Y.-K. Wang, J. Xu (Bytedance)]

This contribution proposes the following errata changes on general SEI signalling mechanism for VVC, VSEI, HEVC, and AVC:

- In VVC:

  - (for both VUI and SEI) Introduce the variable PayloadExtraBits.

    - Derive PayloadExtraBits for VUI as follows: If vui_payload_bit_equal_to_one is present, the variable PayloadExtraBits is set equal to the number of vui_payload_bit_equal_to_zero syntax elements at the end of the vui_payload( ) syntax structure plus 1. Otherwise (vui_payload_bit_equal_to_one is not present), PayloadExtraBits is set equal to 0.

    - Derive PayloadExtraBits for SEI as follows: If sei_payload_bit_equal_to_one is present, the variable PayloadExtraBits is set equal to the number of sei_payload_bit_equal_to_zero syntax elements at the end of the sei_payload( ) syntax structure plus 1. Otherwise (sei_payload_bit_equal_to_one is not present), PayloadExtraBits is set equal to 0.

    - Add PayloadExtraBits, besides PayloadBits, for which the values are passed the parser of the vui_parameters( ) syntax structure specified in Rec. ITU-T H.274 | ISO/IEC 23002-7.

    - Add PayloadExtraBits, besides PayloadBits, for which the values are passed the parser of the SEI message syntax structures specified in Rec. ITU-T H.274 | ISO/IEC 23002-7, ISO/IEC 23001-11, and ISO/IEC 23090-13.

  - (for both VUI and SEI) Derive PayloadBits as follows: The variable PayloadBits is set equal to 8 * payloadSize − PayloadExtraBits.

  - Specify the length, in bits, of vui_reserved_payload_extension_data and sei_reserved_payload_extension_data to be equal to 8 * payloadSize − nEarlierBits − PayloadExtraBits.

  - Change "payload_bit_equal_to_one shall be equal to 1" to "sei_payload_bit_equal_to_one shall be equal to 1".

  - Change "payload_bit_equal_to_zero shall be equal to 0" to "sei_payload_bit_equal_to_zero shall be equal to 0".

- In VSEI:

  - (for both VUI and SEI) Introduce the variable PayloadExtraBits, which is the length in bits of the extra bits in the container besides the PayloadBits, which is in the range of 0 to 8, inclusive.

  - (for both VUI and SEI) Derive payloadSize from being equal to Ceil( PayloadBits ÷ 8 ) to ( PayloadBits + PayloadExtraBits ) / 8.

  - Add the FGC, NNPFC, NNPFA, GFV, GEFV SEI messages into the list of SEI messages for which the values of PayloadBits and payloadSize are used for the parsing of the syntax.

  - Update the description of the pattern of bits for extension data: clarify that the extension data is followed by the extra bits instead of ending with the extra bits, and add the case when the number of extra bits is less than 8, which is currently missing in the specification.

  - Add the definition of the curr_bit_position_in_structure( ) function: curr_bit_position_in_structure( ) returns the current position in bits in the specific SEI message syntax structure (e.g., the film_grain_characteristics( ) syntax structure).

  - Update the definition of the function more_data_in_payload( ): replace "the current position in a container syntax structure (e.g., the sei_payload( ) or vui_payload( ) syntax structure in Rec. ITU-T H.266 | ISO/IEC 23090-3)" with "the current position in the specific SEI message syntax structure (e.g., the film_grain_characteristics( ) syntax structure)".

  - Remove the definition of the function payload_extension_present( ).

  - In both the film_grain_characteristics( ) and nn_post_filter_activation( ) syntax structures, the syntax condition "if( more_data_in_payload( )  &&  payload_extension_present( ) )" is replaced with "if( curr_bit_position_in_structure( ) < PayloadBits )".

- In HEVC (for SEI only):

  - Introduce the variable PayloadExtraBits.

    - Derive PayloadExtraBits as follows: If payload_bit_equal_to_one is present, the variable PayloadExtraBits is set equal to the number of payload_bit_equal_to_zero syntax elements at the end of the sei_payload( ) syntax structure plus 1. Otherwise (payload_bit_equal_to_one is not present), PayloadExtraBits is set equal to 0.

    - Add PayloadExtraBits, besides PayloadBits, for which the values are passed the parser of the SEI message syntax structures specified in Rec. ITU-T H.274 | ISO/IEC 23002-7 and ISO/IEC 23001-11.

  - Derive PayloadBits as follows: The variable PayloadBits is set equal to 8 * payloadSize − PayloadExtraBits.

  - Specify the length, in bits, of reserved_payload_extension_data to be equal to 8 * payloadSize − nEarlierBits − PayloadExtraBits.

  - Add ISO/IEC 23001-11 to the list of external specs specifying the SEI message syntax structures for which the values of PayloadBits and PayloadExtraBits are passed to the parser.

- In AVC:

  - Add the missing definition of the functions more_data_in_payload( ) and payload_extension_present( ), to be basically the same as in VVC.

  - Add the missing row of reserved_sei_message( payloadSize ) in the sei_payload( ) syntax structure.

  - Rename the two same-named syntax elements "bit_equal_to_one" in the sei_payload( ) syntax structure to be "sei_payload_bit_equal_to_one_a" and "sei_payload_bit_equal_to_one_b", respectively.

  - Add missing semantics for the syntax element sei_reserved_payload_extension_data.

  - Replace nPayloadAlignmentBits with PayloadExtraBits, and specify PayloadExtraBits in the same style as for VVC and HEVC.

  - Add ISO/IEC 23001-11 to the list of external specs specifying the SEI message syntax structures for which the values of PayloadBits and PayloadExtraBits are passed to the parser.

The detailed proposed changes are provided in the four attachments to this contribution. For VSEI, VVC, and HEVC, modifications are based on the latest ITU-T published H.274/VSEI, H.266/VVC, and H.265/HEVC specifications. For AVC, modifications are based on the latest draft AVC text in JVET-AP1016.

It is asserted that the proposed changes don’t change the operations of VSEI v4 context.

Regarding item 1, it was questioned if there is a SW implementation to verify that the proposed changes do not impact VSEI v4. No SW has been implemented yet, but could be done.

Agreed to adopt the contribution to AVC, HEVC, VVC, and VSEI. Need to clarify which output document the changes would be included in – errata or WD. Proponent promises to provide software before the next meeting.

JVET-AQ0093 AHG2/AHG9: On the FPA and CTI SEI messages [J. Xu, Y.-K. Wang (Bytedance)]

This document proposes the following changes:

- In HEVC, clarify the semantics of upsampled_aspect_ratio_flag according to the value of frame_packing_arrangement_cancel_flag in the frame packing arrangement (FPA) SEI message.

- In AVC, rename the syntax element frame_packing_arrangement_extension_flag to be frame_packing_arrangement_bit_equal_to_zero and change its semantics accordingly.

- In AVC, add a payloadType to reference the frame packing arrangement (FPA) specified in VSEI.

- In VSEI, change the semantics of colour_transform_persistence_flag according to previous output documents.

Agreed to item 1.

For item 2, it is understood that future extension using the existing syntax element would be problematic. It was suggested that imposing shall be equal to 0 without mention of future versions of the standard seems overly limiting.

It was suggested to add a note. Further discussion requested after note language is available.

It was noted that the VSEI’s version of frame packing arrangement has an aspect ratio feature that is missing in AVC’s version.

Agreed to item 3. To be determined which output document to add it to, probably either a WD for AVC or the TuC.

Agreed to item 4.

Further discussed 10 July. Agreed to add a note as follows:

NOTE 13 – The frame_packing_arrangement_extension_flag is outside of the syntax condition "if( !frame_packing_arrangement_cancel_flag )". This means that it is present even when frame_packing_arrangement_cancel_flag is equal to 1. In this case the SEI message cancels the persistence of any previous frame packing arrangement SEI message in output order and does not carry frame packing arrangement information.

JVET-AQ0114 AHG9: On DSCS SEI message in VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes a modification that is asserted to provide improvement to digitally signed content selection (DSCS) SEI message in VSEI v4. It is asserted that there is a conflict in the semantics that specifies the presence of DSCS SEI message.

It was suggested that the particular language could be improved. It was suggested that “, which” should be “that” and that “VCL verification inclusions NAL units” should be “VCL NAL units”.

Agreed with the modified language as suggested above, to include in a VSEI v4 errata or WD.

## AHG9: Study JVET-AP2032 TuC for VSEI (58)

### General (3)

Contributions in this area were discussed during 1730–1800 on Friday 10 July 2026 (chaired by J. Boyce).

JVET-AQ0072 AHG9: Editorial improvements for TuC [X. Xu, S. Liu (Tencent)]

In this contribution, a number of editorial changes are suggested to improve the quality of the TuC document (JVET-AQ2032-v2). They are summarized as follows:

- Replace “color” with “colour”

- Replace “-” with “−” in arithmetic operations

- Replace if ( * ) with if( * ), and for ( * ) with for( * )

- Replace “/” with “ / ”

- Replace “==” or “ = = ” with “  = =  ”

- Replace “&&” or “ && ” with “  &&  ”

- Replace “+=” or “ += ” with “  +=  ”

- Replace “!=” or “ != ” with “  !=  ”

- Replace “>=” or “ >= ” with “  >=  ”

- Replace “<=” or “ >= ” with “  <=  ”

- Replace “>>” or “ >> ” with “  >>  ”

- Replace “<<” or “ << ” with “  <<  ”

- Replace “in the range of xx .. yy” and “in the range xx to yy” with “in the range of xx to yy”

- Adjust proper spacing in between symbols, brackets and parentheses

- Fix various typos

Delegate all items to the editors.

JVET-AQ0075 AHG9: On video full range indication [X. Xu, S. Liu (Tencent)]

In this contribution, a number of issues are raised, and suggestion are made to improve the usage of the full range indication flag in the SEI messages in TuC document (JVET-AP2032). They are summarized as follows:

1)	Full range flag indication in CMI SEI

2)	Bug fix of full range flag indication in ECFI SEI

For item 1, it is preferred to define an interface variable in the CMI SEI message for FullRangeFlag. Further discussion requested after language is available for the HEVC and VVC interfaces, and the CMI SEI message.

For item 2, a problem was identified for the ECFI SEI message relating to VUI parameters. It was suggested to refer to the CICP specified parameters. Further study encouraged to resolve this. Additional SEI messages may also be impacted.

Further discussed 12 July 2026 at 1830.

A -v2 version was uploaded.

It was suggested to modify the language describing video full range flag in the CMI semantics to match those from VSEI rather than from HEVC.

In addition, it might make sense to update the language in HEVC as well.

Agreed to the -v2 version with the suggested wording change above.

Further study to consider the best approach for the VVC interface text which refers to a syntax element in the VUI defined in VSEI.

JVET-AQ0163 AHG9: on SeiExtenionEnableFlag Interface for SII, PRI, EOI [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes to add missing variable SeiExtensionEnabledFlag for SII, PRI and EOI SEI extensions in VSEI TuC.

Agreed.

It was suggested that it may not be necessary to have the interface variables if all codecs will have extensibility for that particular message, e.g. for AVC the new messages can be extensible. Further study on this.

### Alpha channel information SEI message (1)

Contributions in this area were discussed during 1010–1030 on Wednesday 8 July 2026 (chaired by J. Boyce).

JVET-AQ0129 AHG9: On the Alpha Channel Information SEI message [C.-H. Demarty, A. Ak, E. François, P. de Lagrange, F. Urban, N. Caramelli (InterDigital)]

At the previous meeting, a new value of alpha_channel_use_idc was introduced to distinguish auxiliary alpha layers intended for film grain blending from other usages.

This contribution proposes a complementary update to align this change with the ISO/IEC 23001-11 (green metadata) specification. Since green metadata attenuation maps are conveyed using auxiliary layers of type AUX_ALPHA and ISO/IEC 23001-11 already specifies alpha_channel_use_idc equal to 3 for this purpose, this contribution proposes reserving value 3 for display adaptation and assigning a different value to film grain blending.

The proposed changes remove potential ambiguities when the same auxiliary picture type is referenced by different SEI messages while maintaining consistency between VSEI and the published green metadata specification.

It was questioned if the green metadata specification is a normative reference to VSEI.

A wording change was suggested to use “shall be equal” rather than “is equal to”.

It is noted that the language of the constraint refers to “Green SEI message” which is not the name of the green metadata SEI message.

It was suggested that the constraint be imposed in the ACI SEI message semantics for alpha_channel_use_idc rather than in the SDI SEI message. This is also relevant for the existing language in the SDI SEI regarding FGRC and ACI SEI messages.

Agreed to the -v2 version.

### SEI processing order (SPO) SEI message (2)

Contributions in this area were discussed during 1150–1205 on Friday 10 July 2026 (chaired by S. Deshpande).

JVET-AQ0099 AHG9: On VSEI version indication in SPO SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

Chaired by S. Deshpande during 11:50-11:55 on 10 July 2026.

This contribution proposes to indicate the minimum VSEI version the decoding system shall support to apply the critical processes indicated by the SPO SEI message.

v2 proposes to indicate both specification and version number required to properly apply the critical processes.

This topic is related to JVET-AQ0057. See additional discussions regarding the problem statement in the notes there.

Further study was recommended.

JVET-AQ0158 AHG9: On detection of SEI extensions in SPO [T. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (HHI)]

Chaired by S. Deshpande during 11:55-12:05 on 10 July 2026.

This document is a revision of JVET-AP0215, addressing review comments received at the previous meeting. It proposes an extension to the SPO SEI message that allows decoding systems to recognize whether SEI message extension bits require decoder support for correct interpretation.

SPO includes provisions for decoding systems may not be able to understand certain SEI messages, e.g. by indicating payload types and prefix bits in the SPO SEI message. However, no dedicated provision in SPO allows decoding systems to efficiently recognize the presence of SEI message extension bits or whether they are required in the context of the processing chain.

The proposal adds a flag (po_sei_ext_support_required_flag[ i ]) for each indicated SEI message type in the SPO SEI message extension to indicate whether an SEI message type contains an SEI message payload extension that requires decoder support to correctly interpret and apply the SEI message.

This topic is related to JVET-AQ0057. See additional discussions regarding the problem statement in the notes there.

It was commented that the wording regarding “necessary” or “criticial” resulted in choosing the word “required” in this contribution.

It was commented that the key issue for these three proposals (this one, JVET-AQ0099, JVET-AQ0057) should show a clear example showing the usefulness of having different level of necessity for base and extension part of SEI.

An example of applying FGC followed by post processing (e.g. upsampling) was given. IT was asked if in practice it would be done in that order.

Further study was recommended.

### Neural-network post-filter / related URIs (NNPFC / NNPFA) SEI messages (7)

Contributions in this area were discussed during 1030–1520 on Wednesday 8 July 2026 (chaired by J, Boyce).

JVET-AQ0052 AHG9: On backward compatibility of NNPF alternative temporal extrapolation versions [M. M. Hannuksela, F. Cricri (Nokia)]

See notes for JVET-AQ0058.

JVET-AQ0053 AHG9: On multilayer NNPF [M. M. Hannuksela, J. Boyce (Nokia)]

Chaired by S. Deshpande during 10:40-10:55 on 8 July 2026.

The contribution relates to the cross-layer NNPF approach that is based on the use of the scalable nesting SEI message in the VSEI TuC. According to the approach, when an NNPFA SEI message is included in a scalable nesting SEI message, the input pictures to the NNPF are selected from the layers indicated by the scalable nesting SEI message. It is asserted that a scalable-nested NNPFA SEI message may become non-nested in the sub-bitstream extraction and this causes wrong selection of input pictures to the NNPF in temporal dimension.

It is proposed to specify a new bit of nnpfc_purpose to indicate cross-layer filtering. Since legacy decoders are to ignore an NNPF with this new bit set, legacy decoders will not select input pictures in the temporal dimension within a single layer for an NNPF intended for cross-layer filtering.

It was asked if the following constraint is too restrictive: When an NNPFA SEI message is included in a scalable nesting SEI message with sn_ols_flag equal to 0 and sn_subpic_flag equal to 0, CrossLayerFlag derived from the NNPFC SEI message with nnpfc_id equal to nnpfa_target_id of the NNPFA SEI message shall be equal to 1.

Proponents were ok to remove that constraint. This would need some condition to be added checking CrossLayerflag equal to 1. A revised v2 test will be uploaded with the above change.

V2 was agreed.

JVET-AQ0058 AHG9: On NNPF Multiple Inferences Information Signalling [S. Deshpande (Sharp), M. M. Hannuksela, F. Cricri (Nokia)]

Chaired by Y.-K. Wang.

Following is proposed related to NNPF multiple inferences in TuC:

It is asserted that VSEI V4 includes the temporal extrapolation purpose and thus the current syntax in VSEI TuC related to multiple instances which signals nnpfc_num_alt_instances_minus1 and if present nnpfc_mult_inferences_flag, when TemporalExtrapolationFlag is 1, is not backward compatible with VSEI V4. Two alternative options are proposed to resolve this:

• Option 1: It is proposed to use a new value of nnpfc_auxiliary_inp_idc to signal the multiple instances information when TemporalExtrapolationFlag is 1.

• Option 2: It is proposed to move the signalling of multiple instances information and multiple text prompts and to gate those syntax elements.

Option 1 is preferred.

It is proposed to add the proposed approach to the TuC.

In version 2 of this contribution, text for merged proposal (with JVET-AQ0052) is provided by revision of option 1

Meeting minutes:commented that changes are indeed needed to fix the backward compatibility issue and the proposed changes in the Option 1 that has been merged with JVET-AQ0052 in version 2 of this contribution look good.

The Option 1 that has been merged with JVET-AQ0052 in version 2 of this contribution was agreed.

JVET-AQ0059 AHG9: Bug Fixes for Neural-network Post-filter in TuC [S. Deshpande (Sharp)]

Following is asserted bug-fixes are proposed related to Neural-network Post-filter (NNPF) in VSEI TuC:

- Proposal 1: Asserted bug-fix is proposed in AccessOutputVersions process for chroma extraction when nnpfc_output_order_idc is equal to 2 or 3.

- Proposal 2: Asserted bug-fix is proposed related to the calculation of xSrc variable when nnpfc_output_order_idc is equal to 3 to fix the resolution correctly for interleaved 4:2:0.

It is proposed to add the proposed bug-fixes to the TuC.

This contribution is related to the multiple instance feature of NNPF.

Agreed to add proposal 1 and 2 to TuC.

It is noted that software would be useful.

JVET-AQ0155 AHG9: On NNPFC updates [T. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (HHI), M. M. Hannuksela, F. Cricri (Nokia)]

This contribution addresses ambiguities in applying NNPFC SEI messages containing updates to a base NNPF when neural network information is conveyed using framework formats such as ONNX and PyTorch in addition to MPEG-NNR. It presents two independent proposals:

- Constrain update NNPFC SEI messages to use the same neural network format as the associated base NNPF.

- Extend NNPFC SEI message to explicitly signal the type of update operation for non-NNR formats.

For proposal 2, the contribution proposes 4 values of nnpfc_update_type_idc.

The value of 0 allows to replace the contents of the model. The current spec doesn’t allow the contents of a base model to change within a CLVS. So it suggested that an update could be used to replace the model, while using the same value of nnpfc_id.

The proposal adds syntax to the metadata extension, but it was suggested it may be more appropriate to be in the regular extension.

Proposal 2 from v2 was discussed on 10 July from 18:25-18:35 chaired by S. Deshpande. Value 1 from v1 is now value 0 in the revised v2 option. Also the application defined value is included in the table. The values which were previously 0 and 2 in v1 are removed.

Previously there was a concern about value 2 from v1 regarding whether metadata section is the right place for the new syntax elements. In v2 since that value is removed that concern no longer exists.

Agreed to Proposal 2 from v2 version of the document.

It was commented that the NNR standard enables use of an NNR update of a PyTorch or ONNX model. It was suggested to check the proposed language to see if it precludes that use case.

Proposal 1 is withdrawn after offline discussion.

JVET-AQ0157 AHG9: On NNPFC framework formats and model parameter compression [R. Skupin, T. Borges, Y. Sanchez, C. Hellge, T. Schierl (HHI)]

This contribution proposes an extension of the NNPFC SEI message for the newly introduced mode in which framework formats are used as the primary container for neural network information. The proposed signalling adds a flag indicating the presence of compressed model parameters and an associated URI identifying an ISO/IEC 15938-17 bitstream. The extension enables model topology information to remain in the framework-format representation while compressed model parameters, such as weights and biases, are carried separately, with a conformance constraint defining the separation between the two portions.

The proponent suggests a use case where the model topology is represented as PyTorch or ONNX and the weights/biases in NNR to take advantage of its better compression, and the PyTorch/ONNX is signalled inband and the weights/biases are signalled via a URI.  The proponent suggests that the same bitstream can be used for diverse devices where some devices are capable of performing NN postprocessing, and avoids increasing the bitrate of the bitstream by including the weights/biases inband.

It was suggested that it is unnecessary to get the topology to determine if a device is able to perform NN postprocessing, as other info in SEI message already describes the type of framework and various complexity parameters.

It was questioned why there is a need to separate the topology and the weights/biases rather than providing both together.

It was suggested that a new value of nnpfc_mode_idc could be allocated rather than using nnpfc_ mode_idc equal to 2 for this hybrid mode.

Further study was recommended.

JVET-AQ0196 AHG9: Loss concealment purpose for NNPFC SEI message [S. Mate, J. Boyce, M. M. Hannuksela (Nokia)] [late]

Chaired by S. Deshpande during 14:50-15:20 on 8 July 2026.

This contribution proposes extension for the NNPFC SEI message to support loss concealment. The proposal includes the following:

- Addition of a new purpose for loss concealment to NNPFC SEI message.

- Addition of decoder activation criteria for decoder side determination of criteria to trigger loss concealment processing.

It was suggested by the proponents that aspect 1 could be considered on its own for inclusion. However aspect 2 builds on top of aspect 1 and needs to be considered together.

The error concealment is proposed to be done as a post processing. It was suggested that most decoders should be able to do the error concealment (rather than as a post processing). It was asked if any example exists where decoder can’t do error concealment and a post processing needs to do it.

It was asked if decoder sees this SEI should it not do error concealment, if supports doing it.

It was asked how error concealment will work as proposed in low delay cases.

It was asked how the proposed method aspect 2 works for slice loss. There are two types defined for this.

So far the NNPF design philosophy is NNPFA activates usage of NNPF and NNPFC does not. This proposed approach requires NNPFC to activate the loss concealment mechanism on the decoder side, which was commented to be a big change to our architecture.

It was commented that so far NNPF design has been for post processing and now this is changing it to do an operation which is more decoder related.

It was commented by one participant that the error concealment area is less important as network conditions get better. It is asserted that frame freeze may be better than error concealment in some cases.

It was asked why NNPFC needs to do the activation and not NNPFA. The proponents answered that since you don’t know which picture(s) are lost, you don’t know which pictures need to have NNPFA be sent for.

It was asked if POC gap is known by the post processing entity.

### Encoder optimization information (EOI) SEI message (1)

Contributions in this area were discussed during 1120–1135 on Wednesday 8 July 2026 (chaired by J. Boyce).

JVET-AQ0104 AHG9: On spatial quality optimization in EOI SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

This contribution proposes refinements to the signalling of spatial quality optimization in the encoder optimization information (EOI) SEI message. The proposed aspects are as follows:

- Item 1: To indicate the processing characteristics of spatial quality optimization

- Item 2: To indicate the picture component category or categories affected by spatial quality optimization

In version 2, an additional option has been added to Item 2.

It was suggested that the 4 bits for eoi_spatial_quali_optimization is small, and 8 or 16 may be better.

The proponent suggested that even if other EOI extensions were moved to a WD this meeting, this proposal could be included in the TuC.

The proponent suggested that a showcase could be provided to demonstrate its usefulness.

Agreed to add option 1 of item 1 to TuC.

For item 2, it was noted that the EOI is a layer-wise SEI message, so it was questioned why alpha or depth are described. It was noted that the CR SEI message would permit alpha and depth to be in the same picture as the texture.alternative was proposed to have a bit mask of length 2 bits to indicate luma or chroma. If both bits are equal to 0, would indicate unknown.

Agreed for TuC, based on the -v3 version.

### Modality information (MI) SEI message (2)

Contributions in this area were discussed during 1520–1620 on Wednesday 8 July 2026 (chaired by J. Boyce).

JVET-AQ0106 AHG9: On multi-layer spectral representations in modality information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE), L. Kerofsky (Qualcomm)]

The modality information (MI) SEI message supports signalling of modality and wavelength information associated with individual pictures or channels. However, signalling of multispectral or hyperspectral representations spanning multiple layers may benefit from additional mechanisms. This contribution proposes signalling of multi-layer spectral representations and presents the following options:

– Option 1: Extension of the MI SEI message.

– Option 2: Definition of a new SEI message.

It was suggested to use the scalable nesting SEI message to associate multiple MI SEI messages. The proponents consider it not appropriate because the scalable nesting SEI message would use the same SEI message content to apply to multiple layers while the use case is for different SEI message content to represent the different wavelengths.

It is noted that the existing SEI message syntax can be used to represent multiple wavelengths, but would not identify groups. It was questioned what use cases would include multiple groups in the same bitstream.

For a multi-view bitstream, view ID could be used to identify groupings.

The proponent suggested that a note could be added to describe that multiple MI SEI message could be used.

Further discussion requested.

Further discussed on 10 July 2026.

It suggested to reword the note to avoid mention of encoders and decoders and instead describe what the bitstream means when MI SEI messages are present in more than one layer.

Further discussion requested.

Further discussed 12 July 2026 at 1900.

In the -v3 version, two options of a note are suggested. The first option is agreed, as follows:

NOTE – All pictures associated with modality information SEI messages with mi_modality_type equal to 4 in an AU are representations of the same multispectral picture.

JVET-AQ0109 AHG9: On the modality information SEI message extension [J. Xu, Y.-K. Wang (Bytedance)]

This document proposes the following items to the modality information SEI message extension in the TuC:

- Move all the extension syntax to be under the syntax condition of "if( !mi_modality_info_cancel_flag )".

- Split the line "if( more_data_in_payload( )  &&  payload_extension_present( ) ) {" into two consecutive lines: "if( more_data_in_payload( )" and "if( payload_extension_present( ) ) {".

- Clarify the wavelength information signalled for each channel.

Agreed  to item 1.

The proponent said that item 2 does not need to be discussed because of decision on JVET-AQ0089.

The current MI extension seems to be under-specified regarding how to interpret the output picture’s 3 colour components.

The proposed language uses RGB, which may probably undesirable.

CICP does provide a mechanism to convert YUV to RGB.

Some clarification is needed to describe interpretation of the 3 colour components when the identity matrix is not used.

The Gaussian splatting SEI message likely has a similar issue.

Previous contribution JVET-AP0095 proposed signalling VUI-type information in the MI SEI extension.

Further study encouraged.

### Digitally signed content (DSC) SEI messages (8)

Contributions in this area were discussed during 1445–1550 on 10 July 2026 (chaired by S. Deshpande).

JVET-AQ0044 AHG9: Proposed clarifying improvements to subpicture DSC [I. Sodagar (Dolby)]

See notes under JVET-AQ0228.

JVET-AQ0055 AHG9: On subpicture mapping of the DSCS SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

See notes under JVET-AQ0228.

JVET-AQ0056 AHG9: On subpicture support of the DSC SEI messages [M. M. Hannuksela, J. Boyce (Nokia)]

See notes under JVET-AQ0228.

JVET-AQ0063 AHG9: On the DSC SEI message interface text [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

Chaired by S. Deshpande during 15:35-15:50 on 10 July 2026.

This contribution proposes to update DSC interface text for HEVC to support temporal motion-constrained tile sets as picture segments.

For HEVC the value of dsci_picture_segment_mode_flag shall be equal to 0. This is changed to also allow dsci_picture_segment_mode_flag equal to 1 by providing text to apply to MCTS.

It was asked whether information like sub picture id in VVC also exists for MCTS of HEVC. MCTS also has a id. It was asked if range of the picture segment ids should be clarified/ limited to the maximum id value allowed for MCTS id. Some thought is needed where such a constraint/ limit should be define in VSEI or in interface text.

It was asked if this will have interaction to the signalling and association with DSCS. For example does MCTS provide same type of information as VVC subpictures – e.g. such as number of NAL units per MCTS. However number of nal units can change for subpictures also, so this is not an issue.

The proposed text replaces VVC concept of subpicture with MCTS concept of HEVC.

It was asked if the MCTS extraction process is as clear as VVC subpicture extraction process. We have MCTS extraction process also defined.was asked that VCL nal unit lists was not changed.

It was commented that slice address of extracted MCTS needs to be rewritten when MCTS are extracted.  This is a problem which prevents the concept of signing.

Further study was recommended.

JVET-AQ0116 AHG9: On DSCI SEI message in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Chaired by S. Deshpande during 15:20-15:35 on 10 July 2026.

Item 3 is not covered by JVET-AQ0228. Item 3 relates to: signalling associated picture segment information for a repetition case.

It is proposed to modify the current design so that the associated segment information for one cycle is signalled, and the signalled information applies repeatedly to the remaining verification substreams.

This is a bit efficiency proposal for DSCI, which only occurs once in verification period. Also the method is based on removing redundancy when there are patterns and the savings is related e.g. to how many temporal sub-layers are present. Multiple participants commented that saving bits by adding more complexity/ syntax for DSCI may not be a good trade off. Also such a principle was previously used (e.g. in the previous meeting). It was commented by proponents  that this may also depend upon how long is a verification period.

No action.

JVET-AQ0117 AHG9: On DSCS SEI message in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

See notes under JVET-AQ0228.

JVET-AQ0118 AHG9: On non-VCL NAL units signing in DSC SEI messages in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]See notes under JVET-AQ0228.

JVET-AQ0228 AHG9: Proposed joint text on DSC SEI message extensions [M. M. Hannuksela, J. Boyce (Nokia), I. Sodagar (Dolby), H. Tan, J. Lee (LGE), M. Pettersson (Ericsson)] [late]

Chaired by S. Deshpande during 14:45-15:20 on 10 July 2026.

The contribution proposes results of off-line work that merges aspects of JVET-AQ0044, JVET-AQ0055, JVET-AQ0056, JVET-AQ0115, JVET-AQ0116, JVET-AQ0117, and JVET-AQ0118. The following items are proposed:

- Modifications related to VCL NAL unit mapping to verification substreams:

- Add dsci_au_scope_flag to specify whether the VCL NAL unit mapping provided in the DSCS SEI messages applies AU-wise or PU-wise. (JVET-AQ0055 item 1 with a modification to have a single flag for all verification substreams rather than substream-specific flags)

- Specify the inheritance of PU-wise VCL NAL unit mapping similarly to the AU-wise mapping but within a CLVS. (JVET-AQ0055 item 2)

- To enable temporal sublayer extraction, inherit the VCL NAL unit mapping in the last preceding AU (for AU-wise mapping) or PU within the current CLVS (for PU-wise mapping) in decoding order with temporal sublayer identifier less than or equal to tId is used for the AU. (JVET-AQ0056 item 1)

- Specify that when the k-th VCL NAL unit of the current AU is present for k greater than dscs_num_vcl_nalus_minus1, dscs_nalu_included_in_signature_flag[ k ] is inferred to be equal to 0. (JVET-AQ0117 item 7, JVET-AQ0044, JVET-AQ0055)

- Allow dscs_num_vcl_nalus_minus1 + 1 to be greater than the actual number of VCL NAL units in the current AU (for the AU-wise mapping) or in the current PU (for the PU-wise mapping). (JVET-AQ0117, JVET-AQ0044, JVET-AQ0055)

- Miscallaneous

- Add an informative note related to ignoring verification substreams that have no associated DSCS or DSCV SEI messages. (JVET-AQ0044 item 4)

- Bug fix to remove the constraint of only one DSCS SEI message with a particular value of dscs_id and dscs_signed_segment_mode_flag equal to 1 (by adding "a particular value of dscs_verification_substream_id" to the condition of the constraint). (JVET-AQ0115)

- Remove dsci_picture_segment_mode_flag from extension part and using it to replace dsci_segment_mode_enabled_flag. (JVET-AQ0116 item 1 option 2)

- Clarify that due to sub-bitstream extraction, there may be no VCL NAL units associated with a verification substream, in which case the verification may be omitted. (Proposed resolution to the issue raised in JVET-AQ0116, item 2)

- Allow a DSCS SEI message without dscs_signed_segment_mode_flag when dsci_picture_segment_mode_flag is equal to 1. (JVET-AQ0117 item 1)

- Allow a DSCS SEI message dscs_signed_segment_mode_flag equal to 0 to be used for SEI message signing. (JVET-AQ0117 item 2, JVET-AQ0055)

- For the picture segment mode flag, add a requirement to have a DSCS SEI message present in the first AU (when dsci_au_scope_flag is equal to 1) or in the first PU of a layer (when dsci_au_scope_flag is equal to 0) within a verification period. (Proposed resolution to the issue raised in JVET-AQ0117 item 3)

- Specify that when dsci_picture_segment_mode_flag is equal to 1, VPS is included in all verification substreams in the current verification period. (JVET-AQ0118 item 1)

- Specify that when dsci_picture_segment_mode_flag is equal to 1, APS is included in all verification substreams in the current verification period. (JVET-AQ0118 item 2)

- Specify that a PH NAL unit of a particular PU is associated with all substreams that are associated with one or more VCL NAL units of the PU. (JVET-AQ0118 item 3)

The proposed specification text modifications are included in the accompanying document. The extension syntax should consider using the syntax agreed from JVET-AQ0089.

There was support for the proposed approach.

Agreed.

### Film grain regions characteristics (FGRC) SEI message (1)

Contributions in this area were discussed during 1620–1650 on Wednesday 8 July 2026 (chaired by J. Boyce).

JVET-AQ0092 AHG9: On the FGRC SEI message [Y.-K. Wang, J. Xu (Bytedance)]

This contribution proposes the following changes to the FGRC SEI message:

- In the VVC interface text for FGRC, infer the film grain synthesis picture width and height to be sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples, respectively.

- Move the persistence flag to be earlier, similarly as in almost all other SEI messages with cancel and persistence flags.

- Update the condition on when conforming decoders shall ignore FGRC SEI messages with fgr_alpha_channel_adaptation_flag equal to 1, such that the constraint is updated to be as follows:

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 and sdi_associated_layer_idx[ i ][ j ] equal to the index of the current layer in the SDI SEI message for at least one pair of values of i and j, decoders conforming to this version of this Specification shall ignore (i.e., remove from the bitstream and discard) the FGRC SEI messages with fgr_alpha_channel_adaptation_flag equal to 1.

- Update the condition on when SDI SEI message sdiA shall be present in the CVS, such that the constraint is updated to be as follows:

It is a requirement of bitstream conformance that, when fgr_alpha_channel_adaptation_flag is equal to 1, an SDI SEI message sdiA that satisfies all the following shall be present in the CVS:

–	Let currLayerId and currLayerIdx be, respectively, the layer identifier of the current layer and the index of the current layer in the SDI SEI message such that sdi_layer_id[ currLayerIdx ] is equal to currLayerId.

–	There is at least one value of i in the range of 0 to sdi_max_layers_minus1, inclusive, such that sdi_aux_id[ i ] is equal to 1.

–	For at least one of such i values, there is at least one value of j in the range of 0 to sdi_num_associated_layers_minus1[ i ], inclusive, such that sdi_associated_layer_idx[ i ][ j ] is equal to currLayerIdx.

–	sdiA precedes the FGRC SEI message in decoding order.

- Remove the persistence specification of the alpha channel sample values in the semantics of the FGRC SEI message, as the same persistence specification always exists in the ACI SEI message semantics.

For proposal 1, the language needs to be updated to refer to the max cropped decoded picture size variable rather than the max decoded picture size.

Agreed to item 1 with updated language as described above, item 2, item 4, item 5.

It is noted that the SDI SEI does not have a payload type for HEVC, so constraints that require the presence of the SDI SEI are problematic. It is noted that the object mask SEI message in v4 has a problem in that it refers to syntax from the SDI SEI. Could define an interface variable. For object mask it is suggested to move the constraint to VVC interface text. Y. K. Wang to prepare text for this.

If HEVC support were needed, could define a variable in the interface text for both VVC and HEVC and impose the constraint on the variable in VSEI.

For the SEI messages in TuC, can consider if the constraints is not strictly needed and could be removed, or to move them to interface text.

It was suggested that the decoder language could be removed.

For item 3, agreed to remove the existing constraint.

### Constituent rectangles (CR) SEI message (1)

Contributions in this area were discussed during 1725–1800 on Wednesday 8 July 2026 (chaired by J. Boyce).

JVET-AQ0096 AHG9: On the CR SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes incorporating the BRI SEI message bitdepth range format into the CR SEI message.

It was suggested that the increase in complexity for the CR is modest to incorporate the bitdepth range feature.

It was suggested that grouping may need to be addressed. The proponent said that the CR rect type equal to 7 can be used to identify the group.

Agreed to add this functionality to the CR SEI and remove the CRI SEI from the TuC.

JVET-AQ0105 AHG9: On miscellaneous aspects of SEI messages in TuC [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

This contribution proposes miscellaneous modifications to the constituent rectangles (CR), enhanced colour format information (ECFI) SEI and danmu information(DI) messages. The proposed modifications are as follows:

(Related to CR SEI message)

1	On inclusion of gain map, segmentation plane, confidence map in cr_rect_type_idc

2	On signalling of cr_group_type_description

–	Fix out-of-bounds access for cr_group_type_description_present_flag[i]

–	Conditionally perform byte alignment based on cr_num_groups

It was questioned if anything is needed to fix the other SEI messages that describe the additional CR types.

The question also applies to the alpha, depth, and object mask.

A similar question applies for ECFI.

It was suggested to modify the order of the rect type idc values.

If the CR SEI were to be moved into the WD for VSEI v5, would still need a TuC “extension” that includes the aux type values that are defined in the TuC.

Further discussion requested on item 1.

Agreed to item 2.

Further discussed 12 July at 1920.

An issue was identified with associating individual CRs with an aux type. Further study encouraged on this issue.

Agreed to item 1.

### Quality metrics (QM) SEI message (6)

Contributions in this area were discussed during 1800–2000 on Wednesday 8 July 2026 (chaired by J. Boyce).

JVET-AQ0073 AHG9: On Quality Metric SEI message [X. Xu, S. Wenger, S. Liu (Tencent)]

In this contribution, a number of issues are raised, and suggestion are made to improve the QM SEI message in TuC document (JVET-AP2032). They are summarized as follows:

- Default value for qm_range_present_flag[ i ]

- Consistency constraints on QM syntax element values, when presented in multiple SEI message of the same CLVS

- Clarification on semantics of GainRefPicList[ * ]

- Editorial fixes

For item 1, the precise inference language is suggested to be changed to be consistent with others.

Agreed to item 1 with modified inference language, item 2, item 3, item 4.

JVET-AQ0108 AHG9: On quality metrics SEI message [J. Nam, H. Tan, C. Kim, J. Lee, J. Lim, S. Kim (LGE)]

This contribution proposes the following changes for quality metrics (QM) SEI message.

- Related to the quality metric of type PSNR-YUV

- Related to derivation of average quality metric values for CLVS

It is noted that the bitstream conformance language is not needed here, and the constraint can be expressed without it

Agreed to add constraints as proposed in item 1, with modified language.

It was questioned if the constraint would cause a problem for ECFI or CR that create a target picture with 3 colour components from a monochrome picture or pictures. Could the quality metric apply to the target picture rather than the decoded picture?

Further study encouraged for the applicability of the QM SEI for ECFI or CR target picture.

Agreed to item 2.

It was suggested to have the capability to signal quality values on a temporal sublayer basis.

JVET-AQ0123 AHG9: On reference and source in QM SEI [T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande during 18:50-19:20 on 8 July 2026.

The QM SEI does not provide a way to define what the reference of a metric is, whether it is the encoder input or the origin source content. The proposal introduces signalling to indicate what the reference is, and if the reference is different, what the reference format is and how the conversion was done to compute a metric.

Proposal 1: Identifier: It was commented that identifier by itself does not identify different stages. It was answered that identifier enables application to define how to do that.

Also as proposed the identifier is not used by other syntax elements.

Proposal JVET-AQ0139 is related. So it was suggested to decide on proposal 1 after reviewing that contribution.

Further discussion requested after JVET-AQ0139

Proposal 2: Add signalling of source content format. It was commented that as proposed you must signal the size of the source if source metric is enabled

There was some support to indicate the source format (but allowing not signalling additional syntax which is supported in v2).

Name of the flag in the syntax and semantics did not match for one flag.

Proposal 3: Add to QM SEI a reference description and conversion mechanism.

As proposed the syntax of proposal 2 and 3 is combined (and not directly separable).

It was commented that different metrics in the loop can have different sources. And whether the syntax supports this. This was discussed further and a simpler design was preferred by proponent and offline participant.

Discussed proposal 2 and 3 after offline discussion on 10 July from 12:05-12:15 chaired by S. Deshpande

The qm_reference_idc_enabled flag should be editorially renamed.

Agreed to proposal 2 and 3 as in v2.

Further discussed 13 July at 184 chaired by S. Deshpande.

Agreed to add the qm_id.

JVET-AQ0124 AHG9: On delayed transmission of QM SEI [T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande during 19:20-19:40 on 8 July 2026.

The current QM SEI design support carriage of quality metric pertaining to the coded picture it is associated with. However, ultra-low latency encoding may require sending the coded picture before the quality metric is computed, due to strict constraint on encoding delay budget. In this ultra-low latency case, a quality metric for a given frame may be sent attached with a future access unit. This contribution proposes enabling delayed transmission of QM SEI messages.

Aspect 1 : Add editor’s note : agreed

Aspect 2: Adding the NOTE: agreed with some rewording about “low-latency” delegated to the editors.

Aspect 3: It was asked what is the exact low latency use case which needs this. This proposal brings in the aspect of POC into QM. SVTA indicates delayed quality via time offsets. It was asked what is the case where there is delay of multiple pictures. It was commented that usually the time taken to compute quality will be much less that the encoding time and why multiple picture delay is needed.

JVET-AQ0125 AHG9: On QM-SEI and SVTA-MQA [T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande during 18:35-18:50 on 8 July 2026.

SVTA has recently published a standard for transmission of Media Quality Assessment (MQA) data, which may question the relevance of the quality metrics (QM) SEI message currently being developed in the VSEI TuC. This contribution presents the scope and the target use-case of SVTA MQA and investigates the potential overlap and differentiators with QM SEI.

This is an informative contribution.

Noted.

JVET-AQ0130 AHG9: On the QM SEI message [C.-H. Demarty, A. Ak, R. James (InterDigital)]

This contribution is a follow-up to JVET-AP0139 on source quality metrics in the Quality Metrics (QM) SEI message.

First, it proposes a correction of the signalling conditions of qm_source_quality_metric_flag and qm_gain_flag, so that source quality metrics and gain quality metrics remain mutually exclusive as intended by their semantics.

Second, it proposes an alternative wording for the derivation process of source quality metrics. The proposed text separates the normative semantics of the syntax element from informative guidance related to concatenated encoding pipelines.

For item 1, further discussion requested after offline discussion.

For item 2, the term “current encoding stage” was suggested to be inaccurate, as it represents the original source of the first encoding stage”. Could reword to “first encoding encoding”.

It was unclear if a note is necessary.

Further discussion requested after offline discussion.

Further discussed 12 July 2026 at 1840.

In the -v2 version, item 2 is agreed.

Further study for item 1.

### Display overlays information (DOI) SEI message (2)

Contributions in this area were discussed during 1445–1550 on Thursday 9 July 2026 (chaired by J. Boyce).

JVET-AQ0097 AHG9: On the DOI SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes the following changes to the DOI SEI message.

- Add target picture chroma bit depth and initial value to align with existing SEI message design.

- Add a note that the ACI SEI messages associated with the display overlay alpha planes and target alpha plane should share the same value space.

- Update the target picture alpha derivation process

- Clip the target composite sample value

No action on item 1.

Further discussion requested on items 2 and 3.

Agreed to item 4.

Further discussed on 10 July 2026.

No action on item 2.

Agreed to item 3.

JVET-AQ0103 AHG9: On DOI SEI message [C. Kim, J. Nam, H. Tan, J. Lee, J. Lim, S. Kim (LGE)]

This contribution proposes several miscellaneous aspects regarding the DOI SEI message. The proposed aspects are as follows:

- Item 1: On signalling of overlay restriction information

- Item 2: On no modification restriction and resampling

- Item 3: On viewer advisory overlay

- Item 4: On supplementary content overlay

- Item 5: On signalling overlay source type

It is noted that the doi_overlay_restriction[ j ] description is not well specified and the constraints imposed are unclear.

It is proposed to allow separation of the signalling of restriction from signalling of purpose.

Further discussion requested on  item 1, based on if signalling of restriction should be maintained.

Further discussion requested on   item 2, based on understanding of what the restriction means.

Agreed to item 3.

It was suggested if no purpose were signalled, it would indicate normal content.

If all bit fields in the purpose are equal to 0, the purpose in unknown or unspecified. It is noted that the background layer is also considered an overlay, so is is appropriate for it to be “unknown or unspecified”.

A new “normal video” purpose could be used for the background or for additional content. It might be appropriate for it to be the first bit in the bit field rather than added to the end.

Further discussion requested on  item 4.

For item 5, it is not clear how a receiver would use this information.

No action on item 5.

Further discussion 10 July 2026.

It is request that the proponents of the doi_overlay_restriction[ j ] provide updated specification text that clarifies the intent of that syntax element.

Further study of item 2.

Agreed on item 1.

For item 4, in -v2 version of the contribution, a “normal video” was added to the purpose table.

Agreed to item 4 in the -v2 version. The joint text changes for overlapping items 3 and 4 are  provided in the -v3 version.

### Display rectangles (DR) SEI message (2)

Contributions in this area were discussed during 1550–1645 on Thursday 9 July 2026 (chaired by J. Boyce).

JVET-AQ0107 AHG9: On display rectangles SEI message [J. Nam, H. Tan, C. Kim, J. Lee, J. Lim, S. Kim (LGE), S. Sasse (Fraunhofer HHI)]

Display rectangles (DR) SEI message provides information on the list of display regions / rectangles when the decoded picture is displayed on the device. It is asserted that it is difficult to signal, within DR SEI message, all possible display rectangles that exactly match the resolution of the target display device and currently it is not clear how decoders should handle the situation when there is no DR with a size that matches its display size.

This contribution proposes how to handle resizing on the device.

- Option 1: Signal scaling type that specifies how to resize the display rectangle.

- Option 2: Signal a flag for each DR to indicate whether resizing of the DR is recommended or not.

The contribution proposes that it can be recommended to stretch a display rectangle to fill a display whose aspect ratio does not match the DR’s display aspect ratio.

It is noted that the encoder does not know the receiver’s display aspect ratio.

It was suggested that “scaling” might not be appropriate term, as it is describing handling of the mismatch between the device display’s aspect ratio and the DR.

The current DOI design has a signalled fill method which is used to fill the DR itself, while this contribution proposes filling the device when it doesn’t match the DR.

It was suggested that the CSS object fit property serves a somewhat related purpose for webpage display of images, and it has states that include fill, cover, contain, which correspond to the proposed values of dr_scaling_type.

There is support for adding this feature, but some naming changes are suggested for the syntax elements and the methods in the table.

Further discussion requested.

Further discussed 12 July at 1915.

A -v2 version was uploaded.

It was suggested that editorial changes be made to the descriptions of the values of dr_fitting_type[ i ] to be more clear that this is a recommended operation.

Agreed to the -v2 version. Editorial changes delegated to the editors.

JVET-AQ0127 AHG9: On miscellaneous aspects of display rectangles SEI message [H. Tan, J. Nam, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

It is asserted that some aspects in the design of display rectangle (D) SEI message either have problem or can be improved. This contribution proposes some changes for fixing the asserted problem and is also asserted to provide some improvement to the current design.

The aspects discussed in this contributions are related to:

- On the unit for rectangle position

- On variable DrUnitSize and DrUnitSizeInit

- On signalling of dr_log2_unit_size

- On derivation process / equation for DrFillHeight

- On constraint for DR SEI message for update and non-update

- On position of DR relative to area in the cropped decoded picture

- On invocation of filling process when it is actually not needed

- On the case when display aspect ratio of DR and cropped decoded picture are the same

- On the signalling of fill region position.

It is noted that there is an internal use header which should be removed in a revision of the document.

Agreed on item 1, item 2, item 3, item 4, item 5, item 6, item 7, item 8.

Regarding item 9, it is noted that there is a dr_fill_method_present_flag[ i ] which can be set to 0 when filling is not required.

### Picture segmentation information (PSI) SEI message (5)

Contributions in this area were discussed during 1645–1715 on Thursday 9 July 2026 (chaired by J. Boyce).

JVET-AQ0076 AHG9: On PSI SEI message [X. Xu, S. Liu (Tencent)]

In this contribution, some issues in PSI SEI message are raised, and suggestion are made to improve the clarity of the semantics in the SEI messages in TuC document (JVET-AP2032). They are summarized as follows:

1)	Definition of AuxId

2)	Definitions of ScalabilityId and LayerIdxInVps

3)	Derivation of Variable PsiValue[][]

It is noted that HEVC does not include a payload type for the SDI SEI message.

The PSI SEI message refers to AuxId, which is determined in HEVC by the VPS, but is not defined.

It is suggested that an interface variable could be defined for HEVC and for VVC.

Further study to develop interface text.

For item 3, a bug has been identified that should be fixed. The same problem is addressed in JVET-AQ0161 with a different solution.

For item 3, see notes for JVET-AQ0229.

JVET-AQ0161 AHG9: on PSI SEI [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes editorial and technical clarifications to the PSI SEI message in the VSEI TuC:

- Correction of psi_type_id semantics

- Clarification of PsiValue derivation in sparse mode

- Clarification of sparse mode ID association determination

- Clarification of overlapping segment associations across multiple AUX_SEGMENT pictures

Agreed to item 1.

It is noted that psi_type_id is signalled inside the loop, but does not have an array index.

For item 3, a bug has been identified in the pseudocode.

Further discussion requested to consider solution.

For item 4, there is support to add clarifying language.

.

For items 3 and 4, see notes for JVET-AQ0229.

JVET-AQ0226 Comparison of the Object Mask Information (OMI) and Picture Segmentation Information (PSI) SEI messages [A. M. Tourapis, D. Podborski (Apple)] [late]

During the 43rd JVET Meeting it was asked what differences exist between two SEI messages that both convey pixel-level mask information by means of coded auxiliary picture layers: the Object Mask Information (OMI) SEI message, integrated into ITU-T H.274 | ISO/IEC 23002-7 (JVET-AN1019), and the Picture Segmentation Information (PSI) SEI message under consideration in the VSEI Technologies under Consideration document (JVET-AP2032, clause 8.51). Although a lot of this work was also presented in prior meetings, this contribution tries to provide a comprehensive comparison between these two SEI messages. The two messages are examined in terms of functionality, capabilities, complexity, handling of compression, and intended applications.

The central observation is that they target different classes of application: OMI provides a comprehensive, object-centric description of the objects in a scene and spans all associated mask layers, whereas PSI provides a simple, per-layer segmentation description. This difference has practical consequences for the extraction, reuse, and complexity of the associated metadata.

JVET-AQ0229 AHG9: On the picture segmentation information SEI message — comments and proposed corrections (response to JVET-AQ0161) [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm), A. M. Tourapis, D. Podborski, J. Kim (Apple)] [late]

This contribution responds to JVET-AQ0161 (Qualcomm), which proposes editorial and technical clarifications to the picture segmentation information (PSI) SEI message in the VSEI Technologies under Consideration (TuC) document (JVET-AP2032, clause 8.51). The document agrees with the two editorial corrections (items 1 and 2). It is also agreed that item 3 identifies a genuine bug in the sparse-mode ID association process, but a simpler and more general correction than the one in JVET-AQ0161 is proposed. On item 4, it is noted that association of a collocated primary-picture sample with identifiers from more than one AUX_SEGMENT auxiliary picture is already supported by the current semantics, nonetheless it is agreed to also add explicit clarifying text for this functionality. We additionally identify several further issues in clause 8.51 and propose corrections and improvements to the text.

It was suggested to change single component flag syntax element to three component flag.

The constraint on the bit depth with single component flag is missing in the provided text.

It was suggested to add an array index to psi_sparse_id for improved semantics.

It is commented that many variables are derived in the syntax table rather than in the semantics.

It was requested to upload the slides.

It is noted that the changes are fairly significant in the pixel-wise derivation process and will require new software to implement these changes.  It was noted that were a significant number of other changes as well.

Agreed.

JVET-AQ0230 AHG9: A coding specification-agnostic layer and auxiliary-picture interface for VSEI SEI messages (response to JVET-AQ0076) [A.M. Tourapis, D. Podborski, J. Kim (Apple)] [late]

JVET-AQ0076 observes that the picture segmentation information (PSI) SEI message references variables that are not defined within the VSEI specification (ITU-T H.274 | ISO/IEC 23002-7) — namely AuxId, and the HEVC video parameter set (VPS) constructs ScalabilityId and LayerIdxInVps — and offers, for the latter, either to remove the associated-picture constraint or to add interface variable declarations. This contribution responds to JVET-AQ0076. We agree the problem is real, and we observe that it is not specific to the PSI SEI message: many VSEI SEI messages that operate on auxiliary pictures or on multiple layers specify their semantics in terms of one coding specification’s layer model and therefore do not port across AVC, HEVC, VVC and future coding specifications. Rather than correct each message individually, we propose that VSEI define a single coding specification-agnostic layer and auxiliary-picture interface — a small set of abstract variables and one association derivation, referenced by all such SEI messages — together with a normative per-host-coding specification binding. This generalizes the interface-variable direction of JVET-AQ0076, and because the binding draws on each host coding specification’s existing native signalling, existing HEVC behaviour is preserved.

Was presented Wednesday 15 July 1125 (chaired by JRO).

This is asserted as a useful concept, and was left for further study.

### Danmu information (DI) SEI message (3)

Contributions in this area were discussed during 1730–1850 on Thursday 9 July 2026 (chaired by J. Boyce).

JVET-AQ0074 AHG9: On Danmu Information SEI message [X. Xu, S. Wenger, S. Liu (Tencent)]

In this contribution, a number of issues are raised and suggestion are made to improve the quality of the DI SEI message in TuC document (JVET-AP2032). They are summarized as follows:

1)	Default value for di_rendered_text_size_idc

2)	Signalling of default Danmu parameters

3)	Editorial fix

For item 1, it isn’t considered necessary to have a default value, and it can be left as undefined. No action.

Proposes requiring that default parameters be signalled.

No action on item 2.

Item 3 has identified an editorial bug. Delegate to editors. It is noted that this type of bug can be reported as an issue.

JVET-AQ0126 AHG9: On Danmu SEI [T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia), C. Kim, H. Tan (LGE)]

Chaired by S. Deshpande during 17:50-18:20 on 8 July 2026.

The danmu information (DI) SEI is targeting archival use-case. It is identified that arguably one of the popular aspects of the danmu experience cannot be archived with the current design. In many popular applications, danmus can be upvoted by the users, which can be used by the platforms to increase their visibility. The current signalling does not support archiving of danmu’s popularity. This contribution proposes to introduce signalling supporting this use-case.

The contribution addresses aspects discussed during presentation in Ibiza:

- “It was commented that this may be restrictive (e.g. not having thumbs down or other reactions).”

  - This version adds reaction type and aligns with existing platforms supported features.

- “Other platforms may support other reactions”

  - This version enables users to use-their own reaction types, supporting standard like/dislike, but also application-defined reactions.

It was commented that the number of reaction types may be insufficient. Also for the unspecified reaction type, it is unclear how interoperability among different platforms can be achieved.

Response was you are only identifying the number of reactions – a count.

It was suggested that di_reaction_num’s data type should be carefully selected.

The feedback will be considered and offline discussion will be conducted to revise the document.

Discussed again on 18:20, 12 July:

A version 2 based on offline work was discussed.

Agreed to v2.

JVET-AQ0144 AHG9: On Danmu Information SEI [S. Xie, Y. Gao, S. Wang, W. Niu, Y. Bai (ZTE), J. Xu, Y.-K. Wang (Bytedance)]

While some popular Danmu platforms provide the function of danmu reply to enhance the interactivity among the audience, this contribution proposes to add support for danmu reply to the danmu information SEI message. When there is danmu reply detected, the index of the replied danmu shall be signalled.

It is noted that the start times of Danmu are not required to be in index order.

It was suggested that a constraint is could be that the replied index is less than the total number of danmus.

It was suggested that a constraint be placed on the start time of the current danmu and the replied index.

A constraint was suggested in the form of: for all i and j in the range of 0 .. X, when replied_idx[ i ] is equal to j, start_time[ i ] shall be greater or equal than start_time[ j ].

Further discussion requested, when revised language on the constraints is available.

Further discussed 10 July 2026.

Agreed to -v2 version of the contribution.

JVET-AQ0105 AHG9: On miscellaneous aspects of SEI messages in TuC [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

Item 6	Handling danmu display behind objects in DI SEI message

It is suggested to modify the language to use “recommended” rather than “may” and “may not”.

It was suggested that the user may want to decide whether to have danmus not visible in front of foreground objects.

No action.

### Auxiliary sampling alignment information (ASAI) SEI message (1)

Contributions in this area were discussed during 1925–1930 on Thursday 9 July 2026 (chaired by J. Boyce).

JVET-AQ0087 AHG9: On the ASAI SEI message [Y. He, M. Karczewicz (Qualcomm), J. Chen, Y. Ye (Alibaba)]

This contribution proposes to allow ASAI SEI message to provide parameters used to associate location and format subsampling for an auxiliary picture and respective parameters of either another auxiliary picture or the primary picture.

Agreed.

### Localization and mapping (LAM) SEI message (1)

Contributions in this area were discussed during 1850–1925 on Thursday 9 July 2026 (chaired by J. Boyce).

JVET-AQ0119 AHG9: On LAM SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE), L. Kerofsky (Qualcomm), G. Tenou (Tencent)]

This contribution proposes modifications that are asserted to provide improvement to localization and mapping (LAM) SEI message with multiple cameras. The proposed modifications are as follows:

1.	On PicWidthInLumaSamples and PicHeightInLumaSamples

–	Option 1: Specify that PicWidthInLumaSamples and PicHeightInLumaSamples indicate the width and height of the output picture (i.e., cropped decoded picture).

1.–	Option 2: Specify that PicWidthInLumaSamples and PicHeightInLumaSamples indicate the maximum value of width and height of the coded picture.

2.	On intrinsic camera parameters that are relative to picture size,

–	Option 1: Introduce syntax elements that indicates resolution of the base picture for the intrinsic camera parameters, and update the semantics of lam_focal_length_x_times100, lam_focal_length_y_times100, lam_focal_length_times100, and lam_rd_angle_poly[ i ] such that their unit is luma samples in the base picture.

–	Option 2: Update the semantics of lam_focal_length_x_times100, lam_focal_length_y_times100, lam_focal_length_times100, and lam_rd_angle_poly[ i ] such that their unit is millimeter.

–	Option 3: Update the semantics of lam_focal_length_x_times100, lam_focal_length_y_times100, lam_focal_length_times100, and lam_rd_angle_poly[ i ] such that their unit is luma samples in the picture defined by PicWidthInLumaSamples and PicHeightInLumaSamples.

3.	On a single layer bitstream including pictures from multiple cameras,

–	Option 1: Introduce syntax elements to support the case where the pictures from multiple cameras are temporally interleaved.

–	Option 2: Introduce a flag that indicate a LAM SEI message applies to a temporal sublayer.

In the v2 of this contribution, unnecessary wording in the document header was removed.

In the v3 of this contribution, an editorial error in option 2 of item 2 was fixed.

In the v4 of this contribution, item 1 option 2 and item 2 option 3 were added.

In the v5 of this contribution, the figure in item 1 option 1 was modified.

For item 2, it is noted that in the text in JVET-AQ0086 avoids duplicating plain language and equation for the same purpose.

It was suggested that it may be unnecessary to reference the picture width and height.

There was some uncertainty of if the focal related parameters should be in units of millimeters rather than in picture samples.

Further discussion requested on item 1 and 2.

For item 3, it is proposed to enable having multiple camera views temporally interleaved within a single layer, and to allow more efficient signalling of the LAM SEI message to avoid having to signal it for each picture. The proponent notes that not all codec products support multi-layer.

Subpictures would be an alternative but the proponents note that subpictures are not available in HEVC and AVC.

It was suggested that Constituent Rectangles may be an alternative, but would require some solution to associate different LAM SEI message content with different CRs.

It was suggested that different camera may have different resolutions.

It was suggested that a systems layer could support synchronizing multiple separate video bitstreams, which would avoid the need for a multi-layer video codec.

No action on item 3.

Further discussed on 12 July 2026 at 1850.

Agreed to item 1 option 2 and item 2 option 4 (and for item 2 option 4, also make corresponding syntax changes as implied in the semantics changes, i.e., in the syntax table, replace lam_focal_length_x_times100 with lam_camera_intrinsic_fx_times100, and replace lam_focal_length_y_times100 with lam_camera_intrinsic_fy_times100).

### Sample interleaving SEI message (1)

Contributions in this area were discussed during 1930–1945 on Thursday 9 July 2026 (chaired by J. Boyce).

JVET-AQ0141 AHG9: Fixes to sample interleaving SEI message [J. Chen, Y. Ye (Alibaba), S. He (Nokia)]

This contribution identifies a few issues in the current design of the sample interleaving (SI) SEI message in the VSEI TuC and proposes corresponding fixes. Specifically, it is proposed to:

- Restore the persistence of the SI SEI message that were present in VSEI TuC draft 11, but are missing in draft 12.

- Either change the semantics of si_interleaving_params_present_flag, or remove the inference value of si_interleaving_needed_flag and conditionally signal syntax elements si_num_hor_regions_minus1 and si_num_ver_regions_minus1 to avoid semantic contradictions.

- Correct the derivation of the sample-coordinate mapping from the decoded picture to the target picture in mosaic arrangement.

Agreed to item 1.

Discussed item 2 and item 3 on 10 July chaired by S. Deshpande

Option 1 of item 2 was preferred during the offline discussion. There was support to this option.

Agreed to option 1 of item 2 in v2.

Item 3 there was a confirmation that a problem needs to be solved. The proposed approach had support. Agreed as in v2.

### Gain map (GM) SEI message (3)

Contributions in this area were discussed during 1230–1315 on Friday 10 July 2026 (chaired by J. Boyce).

JVET-AQ0078 AHG9: On Gain map SEI [S. Wang, S. Xie, Y. Gao, Y. Bai (ZTE)]

This document proposes three syntax optimizations for the Gain Map (GM) SEI message. The proposed changes include:

- modify the payload loop starting offset from 16 to 2 bytes;

- remove the version conditional constraint that restricts payload parsing exclusively to the case where gm_version equals 0;

- clearly define gm_version semantic as the minimum version required for gain map metadata parsing, which aligns with the minimum version definition specified in ISO 21496-1.

In the v2 of this document, the GainMapVersion struct and the GainMapMetadata struct in ISO 21496-1 are specified additionally in the introduction.

Agreed on item 1.

Regarding item 2, it was suggested that the version condition is needed but that semantics language regarding extension should be modified.

It is mentioned that using payloadSize in the syntax table would make version 0 not further extensible.

No action item 2, 3.

The semantics for gm_version should be modified to indicate that value 0 corresponds to ISO 21496-1 and other values are reserved. And gm_version should be modified to be a type, payload type, model or similar, rather than version.

Exact language delegated to the editors.

It was suggested to also have an unspecified value .

JVET-AQ0110 AHG9: On the gain map information SEI message [J. Xu, Y.-K. Wang (Bytedance)]

This document proposes the following items to the gain map information (GMI) SEI message in the TuC:

1)	Change the initial value of i from 16 to 2 for the loop of gm_metadata_iso21496_1_payload.

2)	Change the semantics of gm_persistence_flag so that the persistence does not depend on a picture being an output picture or not.

3)	Clarify that a gain map information SEI shall be associated in a CLVS of type AUX_GAIN_MAP.

4)	Allow a gain map information SEI to be associated with a primary picture.

5)	Allow the presence of a gain map information SEI when there is no SDI SEI message.

6)	Clarify the semantics of gm_reserved_6bits.

For item 1, see notes for JVET-AQ0078.

Agreed to item 2, with the bug fix to replace “sphere rotation”.

Agreed to item 3, item 4, item 5, item 6.

It was noted that the use case support in HEIF allows cascading of gain maps, where a second gain map is applied not to the decoded picture but to the derived picture formed from applying a first gain map. An extension to the SDI SEI message may be needed to address that. Further study suggested on that.

JVET-AQ0162 AHG9: on SDI and GM SEI [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposed to:

- Add picture resolution constraint to the gain map information SEI message in VSEI. The gain map auxiliary picture layer dimensions shall be less than or equal to those of each associated primary layer, consistent with ISO 21496-1 clause 4.2 and a fix of left over syntax correction “	for( i = 2 16; i < payloadSize; i++ )”

- Identifies a constraint inconsistency in the SDI SEI message introduced by GM adoption

No action on the proposed restriction on picture width and height.

No action on item 2.

### Generalized alpha plane information SEI message (2)

Contributions in this area were discussed during 0900–0950 on XXday 10 July 2026 (chaired by J. Boyce).

JVET-AQ0098 AHG9: On the generalized alpha plane information SEI messages [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm), E. Thomas (Xiaomi), A. Tourapis (Apple) ]

This contribution proposes the following changes to the non-auxiliary alpha plane information SEI message.

- Propose gen_alpha_use_idc syntax semantic

- Propose gen_alpha_bit_depth_minus8 syntax semantic and update the semantics of other syntax elements

For item 1, also proposed in JVET-AQ0112. See notes for JVET-AQ0112.

Further discussion requested on item 2.

Further discussed on 10 July 2026.

It was suggested to have a description of how to handle mismatch between the metadata bitdepth and the coded picture bitdepth.

Further discussion requested.

Further discussed on 12 July 2026 at 1905.

A -v4 version was presented. It was suggested to move the location of the gen_alpha_value_length_minus1  to earlier in the SEI message, following gen_alpha_use_idc.

Agreed to the -v4 version with the change in the syntax element location suggested above.

JVET-AQ0112 AHG9: On the generalized alpha plane information SEI message [J. Xu, Y.-K. Wang (Bytedance)]

This document proposes the following items to the generalized alpha plane information (GAPI) SEI message in the TuC:

- Clarify what happens when both a GAPI SEI message and an alpha channel information (ACI) SEI message are present.

- Change the semantics of the persistence so that it does not need to check gen_cancel_flag.

- Add the semantics of gen_alpha_use_idc.

- Change the constraint on gen_alpha_value_length_minus9.

For item 1, it was suggested to disallow both any picture to be within the persistence scope of both the GAPI and ACI SEI messages. Some text is needed for that.

Conditionally adopt subject to text being provided in a new version of the contribution.  This decision also applies to to JVET-AQ0111.

Agreed to item 2, item 3.

Item 4 is suggested to add an interface variable for the luma bit depth.

Further discussed 12 July at 1913.

Item 4 is no longer needed because of the actions on JVET-AQ0098.

Further discussed 13 July.

The -v2 version is agreed for item 1.

### Generalized depth information SEI message (2)

Contributions in this area were discussed during 1945–2000 on Thursday 9 July 2026 (chaired by J. Boyce).

JVET-AQ0111 AHG9: On the generalized depth information SEI message [J. Xu, Y.-K. Wang (Bytedance)]

This document proposes the following items to the generalized depth information (GDI) SEI message in the TuC:

1)	Remove nadpi_persistence_flag from the syntax.

2)	Clarify what happens when both a GDI SEI message and a depth representation information (DRI) SEI message are present.

3)	Change the semantics of the persistence so that it does not need to check nadpi_cancel_flag.

4)	Specify the value range for nadpi_depth_representation_type.

5)	Specify minVal and maxVal according to the signal range.

Agreed to item 1.

Agreed to item 2 with changing the shall to should.

Agreed to item 3, item 4

Item 5 proposes to change min and max values based on full range flag. It is noted that this would differ from the pre-existing DRI SEI. Further study was recommended.

Further discussed 10 July. See notes for JVET-AQ0112.

JVET-AQ0165 AHG9: on NADPI BitDepthY Variable [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes to add missing BitDepth Variable interface for NADPI SEI message in TuC.

Agreed.

### VSEI RBSP (3)

Contributions in this area were discussed during 0950–1040 on Friday 10 July 2026 (chaired by J. Boyce), and S. Deshpande thereafter.

JVET-AQ0057 AHG9: On indicating necessity of SEI message extensions [M. M. Hannuksela, J. Boyce (Nokia), H. Tan, C. Kim (LGE)]

Chaired by S. Deshpande during 10:40-11:25 on 10 July 2026.

It is asserted that the SEI message relevance signalling in the VSEI RBSP design of VSEI TuC has shortcomings, such as an inherent constraint that only SEI messages of the same relevance can be included in the same VSEI RBSP. The contribution proposes changes to overcome the asserted shortcomings and to include the signalling of the essentiality of SEI message extensions into the SEI message relevance signalling of the VSEI RBSP design.

Version 2 of the contribution includes three options. Version 3 of the contributions adds one more option with two variants.

Version 3 of the document was discussed.

| Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- |
| In vsei_rbsp( ), a loop of vsei_relevance[ i ] to indicate the relevance of the i-th SEI message payload and the count of necessary extensions | In vsei_rbsp( ), a loop of vsei_relevance[ i ] and vsei_necessary_ext_cnt[ i ] to indicate the relevance of the i-th SEI message and the count of necessary extensions, respectively. | If SEI messages share the same relevance, vsei_rbsp_relevance and vsei_rbsp_necessary_ext_cnt in vsei_rbsp( ) to indicate the relevance of the SEI messages and the count of necessary extensions, respectively. Otherwise, vsei_relevance and vsei_necessary_ext_cnt in versatile_sei_message( ). | SEI messages in the VSEI RBSP share the same relevance indicated in vsei_rbsp( ). Variant 1: No syntax change, vsei_relevance semantics 0 = unnecessary, 1 = undetermined, 2 = base part necessary, extensions unnecessary, 3 = both base and extensions necessary. Variant 2: vsei_relevance and vsei_necessary_ext_cnt to indicate the relevance of the i-th SEI message and the count of necessary extensions, respectively. |

Regarding option 4 variant 2: it was asked how the number of extensions would be known to a decoder. Ane example was show which uses a SeiExtCnt variable which gets incremented every time a new extension is added.

Opinion 1: We should not consider that the legacy decoder does anything more than ignore the extensions that they don’t understand. Also there is no way to change the behavior of legacy decoders regarding extension(s). An extension should never have an necessity that is higher than those in base part of the SEI. And in such a case a new SEI should be defined. Support for first part of opinion 1 from another participant was expressed.

It was answered that the option 4  variant 1 does not envisage an extension that is more essential than base part of the SEI.

A comment made that adding a new SEI where just some “small” new syntax is added compared to a previously defined SEI may not be the best design practice.

Opinion 2: Adding a generic design to do this is not the right approach. New SEI can be created for the case when an “essential” syntax is added to an extension.

It was answered (related to above opinions) that the JVET-AP meeting proposal was scoped differently than this one which is for VSEI.

Present document and past document proponents commented that the proposed mechanism can also apply to previously define SEIs if they are carried in such a newly designed VSEI RBSP.

It was commented that even considering this, a new design like this should not be done to fix cases where a previously define SEI needs to have something “essential” added.

This design relates to VSEI RBSP which could e.g. apply to design of new codec.

It was commented that so far the two examples shown do not seem compelling and does not indicate that this is a problem to be solved.

It was answered that multiple companies (proponents of this document and one from the previous meeting) think that there is an actual problem to be solved and doing something proactively may be better.

JVET-AQ0099 is somewhat related.

It was suggested that a more clear problem statement that is agreeable to non-proponents may be a good next step. This is for further study.

JVET-AQ0090 AHG9: On SEI position, payload type, and payload size signalling in the general SEI signalling mechanism in VSEI TuC [Y.-K. Wang, J. Xu (Bytedance)]

This contribution proposes the following changes to the general SEI signalling mechanism in VSEI TuC:

1)	Make the following changes to the vsei_rbsp( ) syntax structure, including both syntax and semantics:

a)	Replace u(2)-coded vsei_position with u(1)-coded vsei_prefix_flag, indicating prefix SEI (vsei_prefix_flag = 1) or suffix SEI (vsei_prefix_flag = 0).

b)	Add u(1)-coded vsei_pt_2bytes_flag, indicating the length of the payload type syntax element (SE) being 1 byte (vsei_pt_2bytes_flag = 0) or 2 bytes (vsei_pt_2bytes_flag = 1).

c)	(Editorial) Change the name of vsei_payload_size_length to vsei_ps_len, to avoid one SE name being a subset of another SE name.

2)	Make the following changes to the versatile_sei_message( ) syntax structure, including both syntax and semantics:

a)	Replace the current payload type coding based on a loop of u(8)-coded vsei_payload_type_byte with a single u(v)-coded vsei_payload_type, the length of which is either 1 byte (vsei_pt_2bytes_flag = 0) or 2 bytes (vsei_pt_2bytes_flag = 1).

b)	Specify the payload type value as VseiPyldType = vsei_payload_type + 256 * vsei_pt_2bytes_flag.

c)	Specify the payload size value VseiPyldSize as follows:

if( vsei_ps_len  = =  1 )

VseiPyldSize = vsei_payload_size

else if( vsei_ps_len  = =  2 )

VseiPyldSize = vsei_payload_size + 256

else if( vsei_ps_len  = =  3 )

VseiPyldSize = vsei_payload_size + 65 536 + 256

3)	In the example VVC part, add the allocation of the NAL unit type for VSEI_NUT, to make the example VVC part complete.

The detailed proposed changes, based on JVET-AP2032-v3, are provided in an attachment to this contribution.

It was suggested that for the newly proposed 2 byte payload type, the highest value could be reserved to allow for the possibility of using that value to indicate another extension.

Agreed for item 1a.

Agreed for item 1b with the highest value reserved. New spec language is needed for this, to be provided in a new version.

Item 1c delegated to editors.

Agreed for item 2a.

Items 2b and 2c are withdrawn.

Item 3 provides an example of how VVC would be modified to use the new SEI message signalling mechanism. No action.

JVET-AQ0091 AHG9: On SEI payload size semantics and its impacts in the general SEI signalling mechanism in VSEI TuC [Y.-K. Wang, J. Xu, K. Zhang, L. Zhang (Bytedance)]

This contribution proposes the following changes to the general SEI signalling mechanism in VSEI TuC:

1)	Clarify the meaning of the semantics of vsei_payload_size, which specifies the payload size in bytes of a versatile SEI message, as follows (which basically says: When LastPayloadFlag is equal to 0, the meaning of payloadSize is the same as before, i.e., same as in VVC v4 and VSEI v4. When LastPayloadFlag is equal to 1, RBSP trailing bits are considered as part of the payload bytes if the payload does not contain exactly an integer number of bytes and not otherwise):

a)	When the payload of the versatile SEI message contains exactly nBytes1 * 8 + nBits bits, where nBytes1 is a non-negative integer and nBits is an integer in the range of 1 to 7, inclusive (note that in this case LastPayloadFlag is equal to 1), the payload size in bytes of the versatile SEI message is nBytes1 + 1.

b)	When the payload of the versatile SEI message contains exactly nBytes2 * 8 bits, where nBytes2 is a non-negative integer (note that in this case LastPayloadFlag may be equal to either 1 or 0), the payload size in bytes of the versatile SEI message is nBytes2.

2)	Update the inference of vsei_payload_size, when not present, to be as follows: When vsei_payload_size is not present (note that in this case LastPayloadFlag is equal to 1), it is inferred to be equal to NumBytesInRbsp − numPayloadTypeBytes − 1 if the number of bits contained in the payload of the versatile SEI message is not divisible by 8 and it is inferred to be equal to NumBytesInRbsp − numPayloadTypeBytes − 2 otherwise.

3)	In the example additions and/or changes to a video coding standard (relative to VVC) for VVC, make the following changes to the SEI payload semantics:

a)	The variable PayloadBits is set equal to the number of bits in the sei_payload( ) syntax structure that precede the sei_payload_bit_equal_to_one syntax element.

b)	The variable PayloadExtraBits is derived as follows:

If LastPayloadFlag is equal to 0, the following applies:

–	If sei_payload_bit_equal_to_one is present, the variable PayloadExtraBits is set equal to the number of sei_payload_bit_equal_to_zero syntax elements at the end of the sei_payload( ) syntax structure plus 1.

–	Otherwise (sei_payload_bit_equal_to_one is not present), PayloadExtraBits is set equal to 0.

Otherwise (LastPayloadFlag is equal to 1), the following applies:

–	If PayloadBits % 8 is greater than 0, the variable PayloadExtraBits is set equal to is the number of rbsp_alignment_zero_bit syntax elements at the end of the vsei_rbsp( ) syntax structure containing the sei_payload( ) syntax structure plus 1.

–	Otherwise (PayloadBits % 8 is equal to 0), PayloadExtraBits is set equal to 0.

In this proposal, it is assumed that the VSEI changes proposed in JVET-AQ0089 are in place, wherein the definition of the function payload_extension_present( ) is no longer needed for the VSEI spec.

The detailed proposed changes, based on JVET-AP2032-v3, are provided in an attachment to this contribution.

Agreed.

### ECFI (1)

Contributions in this area were discussed during 1550–1610 on Friday 10 July 2026 (chaired by J. Boyce).

JVET-AQ0105 AHG9: On miscellaneous aspects of SEI messages in TuC [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

(Related to ECFI SEI message)

3	Clarification of the high-level description

4	Introduction of an application indicator

5	Introduction of a generalized target chroma format indicator

Regarding item 3, it was questioned why the suggested language is included in a note. The proposed language uses “may” in a note, which is not permitted.

It is suggested to provide revised language for the first paragraph of the SEI message semantics, without a note, with exact language delegates the editors.

For item 4, it was questioned why a bit mask is used rather than an indicator.

Further discussion requesteditem 4 once new language is available using an indicator.

No action on item 5.

Further discussed 12 July at 1930.

Agreed on item 4. Further study to define additional purposes.

## Requests for VSEI v5 working draft (13)

Contributions in this area were discussed during 0900–1315 on Thursday 9 July 2026 (chaired by JRO).

Criteria for inclusion were discussed:

- Maturity of syntax and semantics

- Usage scenario

- Availability of showcase (may not always be needed, depending on obviousness of usage scenario, and clarity of how it could be implemented)

- Availability of software, and which pieces of software would be necessary to make the SEI message usable – in some cases simple writing and parsing may be sufficient, in other cases generation of SEI parameters from video pictures, and processing the decoded pictures would be useful to be provided (not necessarily by the time of elevating a proposal to WD, but by the time the standard is maturing)

- Value of a new SEI message, if it fulfils a purpose that is already supported by an existing one, or value of the functionality in case of an SEI message extension

- Standalone criterion: Value of a new SEI message, if it fulfils a purpose that is already supported by another mechanism at systems level

- Competition with developments in other MPEG WGs or ITU SGs

JVET-AQ0051 AHG9: Encoder optimization information (EOI) SEI message extension for VSEI version 5 [M. M. Hannuksela, J. Boyce (Nokia), J. Chen, Y. Ye (Alibaba), C.-H. Demarty, A. Ak, F. Aumont (InterDigital), X. Xu, S. Wenger, G. Teniou (Tencent)]

This contribution summarizes the features included in the extension of the encoder optimization information (EOI) SEI message of the VSEI TuC. The contribution reports that source code implementing the EOI SEI message extension has been submitted and merged to the VTM TuC branch. Furthermore, the contribution notes the additional EOI SEI message showcase documents submitted to this meeting, namely JVET-AQ0046, JVET-AQ0128, and JVET-AQ0140.

In relation to the summary table of VSEI TuC presented in JVET-AP0290 and included as an addendum of the TuC, the contribution proposes to update the table row of the EOI SEI message extension.

It is proposed to adopt the EOI SEI message extension from the VSEI TuC to VSEI version 5 working draft.

Also in JVET-AQ0086

As per criteria:

- Maturity: good

- Usage scenario: clear

- Showcase: Three available in JVET-AQ0046, JVET-AQ0128, JVET-AQ0140

- Software: available, write/parse sufficient

- Added value in SEI universe: yes

- Added value compared to systems level solutions: n/a

- Competition with other developments:  n/a

JVET-AQ0054 AHG9: Neural-network post-filter (NNPF) extension for VSEI version 5 [M. M. Hannuksela, F. Cricri, J. Boyce (Nokia)]

In relation to the summary table of VSEI TuC presented in an addendum of the TuC, the contribution proposes an update of the table rows of the NNPF SEI message extensions. The updated table may serve as an input for discussing whether and which NNPF extensions are moved from the VSEI TuC to VSEI version 5 working draft.

It was proposed to adopt the following NNPF features from the TuC to VSEI version 5 working draft:

- Filtering purpose auxiliary input

- Additional tag URIs

- New nnpfc_mode_idc for in-band signalling of neural network information

It was commented that it would be difficultv to establish a normative to ONNX and PyTorch in a standard for feature 2.

As per criteria for feature 1:

- Maturity: good

- Usage scenario: benefit of adaptation not fully clear

- Showcase: not available but desirable

- Software: not available, would be desirable to have parameter extraction and output processing

- Added value in SEI universe: not fully clear

- Added value compared to systems level solutions: n/a

- Competition with other developments:  n/a

As per criteria for features 2 and 3:

- Maturity: good

- Usage scenario: clear

- Showcase: n/a

- Software: not available, but would be simple to extend existing NNPF software

- Added value in SEI universe: clear

- Added value compared to systems level solutions: n/a

- Competition with other developments:  n/a

JVET-AQ0062 AHG9: SPO SEI message extension for VSEI v5 [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes to add SPO extension to indicate the critical processes complexity information into VSEI v5 WD. The critical processes complexity provides the minimum complexity capability requirement for the decoder to apply the processing chain. The design has been stable since the 38th JVET meeting (JVET-AL0064, 2025/3) and SW implementation was available.

As per criteria:

- Maturity: good, uses same complexity parameters as NNPFC

- Usage scenario: not fully clear, a system could get the same parameters when opening the NNPFC message, but even then parameters to describe complexity of NN may not be sufficient to determine if the available processor is able to run in realtime.

- Showcase: n/a

- Software: available, write/parse sufficient

- Added value in SEI universe: not fully clear, see under b)

- Added value compared to systems level solutions: n/a

- Competition with other developments:  n/a

JVET-AQ0060 AHG9: Showcase for Photosensitive Content Information SEI [S. Deshpande (Sharp)]

This contribution describes a demonstration of the use of the Photosensitive Content Information (PCI) SEI message described in JVET-AP2032. The tool analyzes the video for the presence of photosensitive content information as per three different standards, and a creates a visualization per-standard on video timeline. It also displays an on-screen warning while photosensitive content is played and shows logs of the photosensitive content picture ranges.

The showcase is asserted to confirm that the PCI SEI can be generated and used in practice to indicate photosensitive content in a video. We propose to include Photosensitive Content Information SEI in a WD of VSEI V5.

As per criteria:

- Maturity: good

- Usage scenario: clear, parameters used defined in three different standards to associate with individual pictures where critical flashes etc. appear. It would be good to keep the value ranges flexible for future displays.

- Showcase: available in JVET-AQ0060

- Software: available, write/parse, but extraction would be simple as demonstrated in showcase

- Added value in SEI universe: clear

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

JVET-AQ0064 AHG9: proposing Shutter Interval Information (SII) extension for VSEIv5 [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes adding an extension of the Shutter Interval Information (SII) SEI message to support rolling shutter cameras into the working draft of version 5 of VSEI.  The extended SI SEI message is meant to be available for HEVC and VVC.

Several applications need rolling shutter information. The design was asserted to be stable after being in the TuC for several meeting cycles and SW implementation was previously made available. Therefore, the SII extension is proposed for including in VSEIv5 draft.

The v2 version of this document modifies the title to define SII.

Also in JVET-AQ0086

As per criteria:

- Maturity: good

- Usage scenario: clear, extends shutter interval to cases where not all pixels are sampled at same time during the interval.

- Showcase: available by demonstrating effects in picture examples in current contribution

- Software: available, write/parse

- Added value in SEI universe: clear

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

JVET-AQ0065 AHG9: proposing Localization And Mapping (LAM) for VSEIv5 [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), G. Teniou (Tencent)]

This contribution proposes adding the localization and mapping (LAM) SEI message into VSEI working draft 5.  The LAM SEI message is meant to be available for HEVC and VVC.

Both 3D asset creation and automotive applications benefit from the LAM message. The design was asserted to be stable after being in the TuC for several meeting cycles and SW implementation was previously made available.  Therefore, the LAM SEI message proposed for including in VSEIv5 draft.

Version 2 of this document modifies the title to define Localization And Mapping (LAM).

Also in JVET-AQ0086

As per criteria:

- Maturity: good

- Usage scenario: clear, many applications in robotics, automotive, personal navigation; more information may be useful to determine where the parameters come from; might also be additional sensors, not only camera pictures.

- Showcase: n/a

- Software: available, write/parse; extraction might be very application specific, seems not necessary

- Added value in SEI universe: clear

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

JVET-AQ0066 AHG9: proposing Lens Optical Correction (LOC) for VSEIv5 [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), G. Teniou (Tencent)]

This contribution proposes adding the lens optical correction (LOC) SEI message into VSEI working draft 5. The LOC SEI message is meant to be available for HEVC and VVC.

Imaging applications using non-ideal cameras needing distortion correction are addressed by this SEI.  The design was asserted to be stable after being in the TuC for several meeting cycles and SW implementations were previously made available.  Therefore, the LOC SEI message proposed for including in VSEIv5 draft.

The v2 version of this document modifies the title and includes some clean ups.

As per criteria:

- Maturity: good

- Usage scenario: clear, many camera systems have extreme optical distortion.

- Showcase: available, JVET-AI0160

- Software: available, write/parse/parameter extraction from input

- Added value in SEI universe: clear, existing camera parameter SEI messages are not able to express such extreme distortions

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

JVET-AQ0086 AHG2/AHG9: Initial proposed text for a potential WD of VSEI v5 [Y.-K. Wang, J. Xu, L. Zhang, K. Zhang (Bytedance), S. Deshpande, J. Samuelsson-Allendes (Sharp), Y. He, L. Kerofsky (Qualcomm), A. M. Tourapis, D. Podborski, J. Kim (Apple), E. François, C.-H. Demarty, A. Ak (InterDigital), S. Wenger, G. Teniou (Tencent), H. Tan (LGE)]

This contribution proposes, if the JVET decides to start a WD of VSEI v5 at this meeting, an initial proposed text for the potential WD of VSEI v5. The proposed initial text includes the following two categories of items from the VSEI TuC in JVET-AP2032:

–	Category 1 (clear yes for inclusion into the WD; the proponents think that for these items, the use cases look clear, the designs look good and the texts look mature, and software and meaningful showcases have been provided):

–	Extensions to the scalability dimension information (SDI) SEI message

–	Extensions to the shutter interval information (SII) SEI message

–	The film grain regions characteristics (FGRC) SEI message

–	The constituent rectangles (CR) SEI message

–	The localization and mapping (LAM) SEI message

–	The photosensitive content information (PCI) SEI message

–	The picture segmentation information (PSI) SEI message

–	The danmu information (DI) SEI message

–	The colour mapping information (CMI) SEI message

–	The enhanced colour format information (ECFI) SEI message

–	Category 2 (yes for inclusion into the WD if implementations and meaningful showcases are provided at this meeting; the proponents think that for these items, the use cases look clear, the designs look good and the texts look mature, but software and meaningful showcases are yet to be provided):

–	Extensions to the encoder optimization information (EOI) SEI message

–	Extensions to the digitally signed content (DSC) SEI messages

–	Extensions to the packed regions information (PRI) SEI message

The following three attachment files are included in this contribution:

–	The first file contains text changes for all category 1 items, with change marks relative to the latest ITU-T published text of H.274/VSEI v4.

–	The second file shows change marks for the new SEI messages relative to just copying and pasting the syntax and semantics from the TuC. All these changes are asserted to be purely editorial improvements. Since the style of each syntax row in the syntax tables has been changed to be consistently using the style named "table syntax", there are lots of formatting changes shown. In Word, under "Review", you can click "Show Markup" and check off "formatting", then all formatting change marks won’t be shown.

–	The third file contains text changes for the three category 2 items on top of changes for all category 1 items in the first file, using a different user ID such that these changes for the category 2 items can be easily identified.

It is suggested to start with a VSEI WD in the form of fully-integrated text instead of the amendment form. An advantage of this is that while during the development of the new version, hopefully a good full text is gradually developed, thus avoiding the last-minute hassle of integrating all amendment deltas, in which case some of the items can easily get lost. The provided texts are suitable for this purpose, as the used basis texts were the latest ITU-T published texts.

However, if the group decides to go with the amendment form, the proponents are happy to generate whatever agreed parts in the amendment form.

Initial discussion at plenary level Tuesday 7 July 1910-2000 (chaired by JRO).

The TuC JVET-AP2032 contains 23 new messages (including 3 on GSC), and extensions to 8 existing messages (where NNPF has 6 different extensions, and SPO has 2 different extensions).

As an initial step, all 12 contributions in this category were requested to provide a short description of the use case for each item (new message or extension) for further discussion.

Use cases might also be added to the table of the document 2032 (or a corresponding table to be created in the WD), unless the usage/showcase in the rightmost column is sufficient.

It is noted that the table may require some update on software availability.

It was commented that starting such a work item as new edition may not be possible in ISO, as edition 4 FDIS is still under ballot, so publication may not be before 2027.

Follow-up discussion Thursday 9 July 1115:

1) Extensions to the scalability dimension information (SDI) SEI message

As per criteria:

- Maturity: good

- Usage scenario: clear, includes also new aux pic types to signal confidence of another aux pic, which is useful e.g. for depth, and segmentation by signalling labels. Semantics of confidence may require some more precise definition in terms what confidence is expressing, which metric (probability, error in measurement), etc. is expressed by the confidence

- Showcase: n/a

- Software: not available for confidence, available for segmentation, write/parse of SDI would need to be extended by new types

- Added value in SEI universe: clear

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

The film grain regions characteristics (FGRC) SEI message

As per criteria:

- Maturity: good

- Usage scenario: clear

- Showcase: available JVET-AH0112

- Software: available for write/parse and synthesis of film grain, extraction of parameters would be desirable

- Added value in SEI universe: clear

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

The constituent rectangles (CR) SEI message

As per criteria:

- Maturity: good

- Usage scenario: clear

- Showcase: available by demonstrating picture examples in JVET-AH0162

- Software: available for write/parse and extraction of rectangles from decoded output pictures

- Added value in SEI universe: more flexible usage than using subpictures

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

The picture segmentation information (PSI) SEI message

As per criteria:

- Maturity: good

- Usage scenario: clear

- Showcase: available in JVET-AP0245 (part on picture segmentation tool)

- Software: available for write/parse and extraction or marking of segments from decoded output pictures; encoder input to aux picture needs to be prepared separately

- Added value in SEI universe: benefit over OMI should be clarified

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

The danmu information (DI) SEI message

As per criteria:

- Maturity: good

- Usage scenario: clear

- Showcase: available in JVET-AP0097

- Software: available for write/parse, sufficient for the purpose

- Added value in SEI universe: clear

- Added value compared to systems level solutions: more flexible than subtitles used somewhere which is not supporting larger number or dynamically moving text in video in an efficient way

- Competition with other developments: n/a

The colour mapping information (CMI) SEI message

As per criteria:

- Maturity: good

- Usage scenario: clear

- Showcase: available in JVET-AO0166

- Software: available for write/parse, and processing of decoder output

- Added value in SEI universe: mapping of monochrome to three-component pictures not available in any other SEI message

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

The enhanced colour format information (ECFI) SEI message

As per criteria:

- Maturity: good

- Usage scenario: clear

- Showcase: available by demonstrating picture examples in JVET-AI0257

- Software: available for write/parse, generation of input pictures, extraction of packing parameters, and processing of decoder output

- Added value in SEI universe: clear

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

Extensions to the digitally signed content (DSC) SEI messages

As per criteria:

- Maturity: not fully clear yet, various new input contributions on other aspects

- Usage scenario: clear

- Showcase: available in JVET-AQ0115

- Software: available for write/parse and marking parts of picture from showcase

- Added value in SEI universe: clear

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

Extensions to the packed regions information (PRI) SEI message

As per criteria:

- Maturity: clear

- Usage scenario: clear

- Showcase: not available; would be good to demonstrate that mixed content can be improved by coding the screen content part in 4:4:4 in a mixed content image at same bit rate

- Software: not available, would be needed for showcase

- Added value in SEI universe: may depend on showcase

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

JVET-AQ0159 AHG9: MI Multispectrum Extension for VSEIv5 [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes including the adopted Modality Information (MI) SEI extension for multispectral pictures into VSEI Working Draft V5.

As per criteria:

- Maturity: not fully clear (additional input at this meeting)

- Usage scenario: clear

- Showcase: not available

- Software: write/parse available; reconstruction of multispectral picture might be desirable

- Added value in SEI universe: clear

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

JVET-AQ0160 AHG9: GM SEI for VSEIv5 [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes including the Gain Map auxiliary picture type and Gain Map Information SEI message in VSEI Working Draft V5.

Gain maps are already standardized in ISO 21496-1 and are widely deployed across major imaging ecosystems including Android Ultra HDR, Apple HDR Gain Maps, and Adobe HDR imaging workflows. The Gain Map design adopted into the VSEI TuC enables carriage of gain map based HDR reconstruction information within a video bitstream using an auxiliary picture layer and associated metadata.

- Maturity: clear

- Usage scenario: clear

- Showcase: not available

- Software: write/parse available; conversion of HDR output to higher bit depth might be desirable

- Added value in SEI universe: clear

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

JVET-AQ0175 AHG9: Display overlays info (DOI) SEI message for VSEI v5 WD [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia), Y. He, S. Zhao, L. Kerofsky (Qualcomm), H. Tan, C. Kim, J. Lee (LGE), G. Tenniou, S. Wenger, A. Hinds (Tencent)]

The DOI SEI message was first proposed in July 2024 in contribution JVET-AI0181 and added to the VSEI TuC document output from that meeting.  Multiple improvements, including additional features, have since been added to the DOI SEI message design in the TuC, as included in JVET-AP2032. A showcase for the DOI SEI message was provided in JVET-AJ0159.

The information conveyed in the SEI message can be used at the receiver to form a target display picture, which is formed by overlaying multiple display overlays in a specified order, with optional alpha. Use cases for the proposed SEI message include score overlays, title overlays, logo insertion, product placement, and background replacement.

A software merge request aligned with the design in JVET-AP2032 has been provided at the following link: https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM/-/merge_requests/72

This software performs the pixel processing to output a target picture, in addition to syntax parsing.

It is proposed to move the DOI SEI message from the VSEI TuC to the VSEI version 5 working draft.

As per criteria:

- Maturity: clear

- Usage scenario: clear

- Showcase: available in JVET-AJ0159

- Software: available for write/parse and generation of output

- Added value in SEI universe: clear

- Added value compared to systems level solutions: Better support for dynamic content, in particular when transparency overlays are used (more study which solutions actually exist beyond OMAF where similar concepts are applied to 360 video)

- Competition with other developments: n/a

JVET-AQ0176 AHG9: Display rectangles (DR) SEI message for VSEI v5 WD [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia), Y. He, S. Zhao, L. Kerofsky (Qualcomm), H. Tan, C. Kim, J. Lee (LGE), R. Imichi, Y. Tokumo, T. Ikai (Sharp)]

The DR SEI message was first proposed in January 2025 in contribution JVET-AK0142 and added to the VSEI TuC document output from that meeting.  Multiple improvements, including additional features, have since been added to the DR SEI message design in the TuC, as included in JVET-AP2032. A showcase for the DOI SEI message was provided in JVET-AL0141.

Popular video streaming/social media platforms for user generated content (UGC) use different default video display aspect ratios, with some using vertical video and some using horizontal video. The display rectangles SEI message targets repurposing a coded video for a variety of video streaming platforms and devices that use different default display aspect ratios, using cropping and filling.

A software merge request aligned with the DR SEI message design in JVET-AP2032 has been provided at the following link: https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM/-/merge_requests/71

In addition to syntax parsing, this software performs the pixel processing to output a selected target picture when filling is not used and for a subset of the defined filling methods. The software will be updated to support all of the defined filling methods in the near future.

It is proposed to move the DR SEI message from the VSEI TuC to the VSEI version 5 working draft.

As per criteria:

- Maturity: clear

- Usage scenario: clear

- Showcase: available in JVET-AL0141

- Software: available for write/parse and generation of output (one method of filling – gradient – not supported)

- Added value in SEI universe: purpose cannot be achieved by conformance cropping

- Added value compared to systems level solutions: n/a

- Competition with other developments: n/a

JVET-AQ0177 AHG9: Bitdepth range info (BRI) SEI message for VSEI v5 WD [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia), Y. He, S. Zhao, L. Kerofsky (Qualcomm), H. Tan, C. Kim, J. Lee (LGE), R. Imichi, Y. Tokumo, T. Ikai (Sharp)]

This contribution proposes that the bitdepth range information (BRI) SEI message in the VSEI TuC be included in the initial VSEI version 5 working draft.

Request was withdrawn, obsolete at this moment due to decision on JVET-AQ0096.

General

- After further review of inputs related to the SEI messages and extensions requested to be included, decide which SEI messages and extensions should be selected for inclusion in WD. Based on new modifications proposed, the aspect a (maturity) might need to be re-assessed for certain cases. Only SEI messages and extensions that can be assessed to be sufficiently stable should be included in the first draft of the WD.

- Missing document numbers for showcases should be added

- A table should be created from the criteria a-g for the different proposed messages

Follow-up discussion was conducted in the HLS track on Monday 13 July at 1650, considering the spreadsheet summarizing status of SEI messages proposed for VSEI v5 WD (see under plenary discussion 7.1.2). The following issues were identified:

- The SW availability status of NNPF additional URI tags was wrong in the original table, as indeed software is available.

- The SDI SEI message extensions are considered to be necessary for inclusion in the WD because some other SEI messages that will be included rely on it, including CR and ECFI.

In a follow-up discussion in JVET conducted on Tuesday 14 July 1200. It was agreed to include NNPF additional URI tags in JVET-AQ2006.

As both CR and ECFI are incomplete without being signalled in SDI, it is asserted to be better first working towards full maturity of SDI, and putting the package of all three into the WD by the time that would have been achieved.

## Identify potential needs for additional SEI messages, including study of AVC and HEVC SEI messages for use in VVC (11)

Contributions in this area were discussed on Sunday 12 July 2026 (chaired by S. Deshpande), times of presentation and discussion as listed below the contributions.

JVET-AQ0122 AHG9: QR Code Information SEI [T. Biatek, S. He, D. Fortin, J. Boyce, M. M. Hannuksela (Nokia)]

Deshpande during 10:30-11:15 on 12 July 2026.

QR codes are widely used in video applications, especially in the broadcast and streaming areas. Although QR codes carry Reed-Solomon error correction codes enabling them a relatively strong robustness, they may still be lost or damaged too severely for being detected by the end user, losing service added value. It is proposed to create a dedicated SEI message carrying QR codes information enabling network nodes, encoders, or receivers to ensure the QR codes are preserved and properly detectable.

Version 2 adds a showcase to the contribution.

It was asked how the QR code is used for requesting customized video playback (Use case n3 in the slide deck). Answer: It is customizing QR codes per user via a custom SEI.

It was commented if doing this at the system layer may be better.

The system layer has a cost of announcing in MPD, having a separate stream/ track.

But for legacy system doing it at system level may be the better approach. For browser bases MSE, you don’t have access to NAL units.

Comment: this has all kind of security implication where SEI message may be replaced with other QR code.

About QR code burnt in: it was asked how common is it: it was answered that this is used in broadcast TV.

One case mentioned for use of this SEI is if the original brunet in video was transcoded, including resampling. It was asked if the content provider will make sure the QR code is handled correctly in that case. It was also asked if providing x, y position data in SEI would need change in case video is resampled.

It was commented that interactive TV standards exist (e.g. ATSC 3.0) which allows doing this via “Broadcaster Application”.

It was asked if this can be supported by using some existing SEI like DOI, DR with some additional syntax.

Proponents explained that they think those SEIs are not sufficient for this purpose.

It was commented that may be the focus should be on some use cases which are video centric and then to consider what should be signalled in that case.

Further study for answering above (including system vs elementary stream level, security aspect etc.), including considering if this can be done by modifying existing SEI such as DOI, DR.

JVET-AQ0135 AHG9: Support of regions of update and update priority SEI message [S. He, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande during 19:25-20:05 on 10 July 2026.

This document proposes a new SEI message for signalling update priority information associated with a region in the output picture. The proposed SEI message enables flexible update methods for different display devices.

Q1: How often do people use E-Ink displays for video.

Q2: Is the identified selected region the update will be at higher frame rate? Refresh rate is not defined in the SEI. There is also a full refresh flag.

Q3: About full refresh flag: that is the default behavior if SEI message is not present? Why the flag is needed

Q4: Is 30 frame per second a full frame refresh? Answer: No, it is for part of screen.

Q5: Will the refresh rate improve in future E-Ink displays? Answer: Hard to predict. Also refresh costs power.

Q6: Is there ghosting effect when displaying video in E-Ink displays?

Would the ghosting last longer?

Q7: Does hardware support region to have different refresh rate?

Comment: Colour experience on E-Ink is not very enjoyable.

Comment: Different viewers may care for different regions of video as having high priority.

Q8: What is the bottleneck in refresh: power consumption is one bottleneck and this proposal can provide an benefit. Potentially the power consumption may be useful.

Q9: if part of the video are shown at higher refresh rate and other parts at lower rate, would the video appear “un-natural”?

Q10: Can this functionality be done by extension of packed region information SEI. Answer: No

Comment: the encoder side functionality may have conceptual overlap with spare picture SEI of AVC.

It was suggested that a showcase may be useful to see how different parts having different refresh rates show as an experience.

Also the type of content with mixed refresh rate should be carefully considered.

Further study was recommended.

JVET-AQ0136 AHG9: Composited Pictures Information SEI [S. He, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AQ0136 and JVET-AQ0188 were discussed together.

Chaired by S. Deshpande during 15:20-15:50 on 12 July 2026.

This document proposes a new SEI message for signalling composited pictures information recommended for use in the formation of one or more target pictures by compositing constituent cropped decoded pictures from one or more layers. The proposed SEI message enables subpicture-like functionality in HEVC by using its layering functionality.

JVET-AQ0136-v2 added following updates:

- Allow rescaling: a cpi_rescaling_enabled_flag is added in the syntax and semantics, and cpi_resampled_pic_width_minus1[ i ] and cpi_resampled_pic_height_minus1[ i ] are added in the layer’s loop when rescaling enabled.

- A cpi_atlas_flag is added, to allow grouping a list of cropped decoded pictures with same partitioning in the target picture together. The syntax element cpi_copy_pos_idx[ i ] is added to indicate the index of layer that the sizes and positions of the constituent cropped decoded picture of the i-th layer is based on partitioning parameters of the cpi_copy_pos_idx[ i ]-th layer.

- Editorial fix in the syntax.

Instead of grid based approach, this proposal uses cropped decoded picture pixels based approach.

Asked if this is similar to PRI SEI (maybe like multiple target pictures instead of one target picture as in PRI). PRI is in VSEI V4. If we want to have multiple target pictures in one PRI, it could be done via its extension.

Why is auxiliary id needed and can it conflict with what VSP already have. It is to provide info in the SEI and is optional.

JVET-AQ0188 Subpicture support in HEVC [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri (Apple)]

Chaired by S. Deshpande during 14:30-15:20 on 12 July 2026.

This contribution is a resubmission of JVET-AP0263. It updates the subpicture support proposal for HEVC first presented in JVET-AI0256 and further developed in JVET-AJ0219. The updated design introduces a revised subpicture_info SEI message with a cleaner architecture based on the concept of composited pictures. The SEI defines an atlas grid for spatial layout, maps HEVC layers to subpicture regions, and associates each layer with a composited picture type that determines its rendering method and purpose. Per-layer properties such as view identification, auxiliary type, resolution, and colour description are obtained from the existing VPS, SPS, and VUI, avoiding duplication. Decoded layer content is scaled to fit the subpicture region, decoupling coded resolution from atlas geometry.  In this version, an optional modification of the Atlas Definition is additionally described, in which the subpictures (atlas segments) are defined directly by their location and size, in a single step, instead of through the region grid; this is offered should the group consider the region-grouping design too complex.

Slides to be uploaded in a revision.

The intent is not to replicate VVC subpicture functionality, but to have a much more generic version than VVC functionality.

Full atlas can be an output picture or a container for output pictures.

Atlas allows understanding the relation of layers together.

It was commented that this is beyond the scope of coding specification (like some system level information).

One atlas corresponds to one time instance.

JVET-AQ0136 concept of target picture is similar.

What is the use case to allow overlapping: an example is picture-in-picture.

Priority id metadata tells how to handle overlap and order of processing.

The proposal is not HEVC specific and can also apply to a future codec (VVC already has subpictures).

Atlas can also have holes (empty regions).

Display overlay SEI in TuC is most related but does not have all the functionality proposed in this SEI.

Different subpictures can have different number of associated layers.

Combined discussion for JVET-AQ0136 and JVET-AQ0188:

-Do we want this functionality in SEI:

-Subpicture functionality: has more non-proponent support

-Composition functionality: some non-proponents felt this should be done at system level.

-If so, should it be a new SEI or extension of some existing SEI: It was suggested to check if PRI SEI can be extended to support the subpicture functionality, including supporting multiple target pictures.

Regarding the aspect of supporting multiple target pictures in PRI: there was some objection to add that, and rather only support subpicture functionality.

It was also commented that an complicated SEI to support just the required functionality may not be best.

It was commented that PRI already has some concept of composition, although not a full fledged composition functionality.

It was noted that only one of the two proposals have multiple target pictures.

It was commented that a possible drawback of PRI is that there are N regions and each region is associated with only one layer. And so if PRI is used for this functionality this aspect should be considered/ modified.

Further study based on above direction.

JVET-AQ0139 AHG9: Historical Information SEI message [J. Pardo, T. Solovyev, E. Alshina (Huawei), X. Xu, S. Liu (Tencent)]

Chaired by S. Deshpande during 12:15-12:45 on 12 July 2026.

Following CfE and CfP sequences selection we can see that user generated content (UGC) plays a bigger role into the elaboration of the next video codec standard. It is proposed that UGC sequences characteristics should be also considered in the VSEI tracks. One of these characteristics that is proposed is to add transcoding information

The showcase had provided the information about Resolution and PSNR only to the model.

It was asked why the PSNR drops when bitrate is increased between data point 2 and 3 in the slides.

Persistence is not title (bitstream) level, but is RAP period/ CLVS level.

TVD dataset was used for the tests (it included 86 videos).

It was also suggested to consider sharing the model or code which maximizes PSNR based on this historical information may be beneficial to study the merit of this approach.

It was commented that the answers to the questions/ comments (as we all those under JVET-AQ0179) need to be provided to convince the benefit of the history information.

Further study

JVET-AQ0145 AHG9: IR camera adaptation parameters SEI [V. Zakharchenko, J. Boyce, M. Hannuksela (Nokia)]

Chaired by S. Deshpande during 11:15-12:00 on 12 July 2026.

An SEI message is proposed to provide infrared (IR) camera parameter information associated with decoded video pictures, including corrective parameters for compensating spatial sensitivity irregularities and nonlinear response characteristics of the sensor. The proposed SEI message supports a variety of IR sensor configurations by providing syntax elements to signal matrix-based sensitivity correction parameters and nonlinear mapping parameters enabling improved utilization of the sensor dynamic range.

Carrying IR camera correction and adaptation information in an SEI message is asserted to be useful for use cases such as thermal imaging, surveillance, machine vision, image/video analysis, and generation of derivative or enhanced IR content.

Slides presented will be uploaded in v2.

Is it calibration for visualization? No

How do current applications handle calibration: each camera will have firmware that supports this.

Expectation is encoder will embed this data in SEI message.

Q: Does it give monochrome video?

It was asked if confidence map (CM) is enough. CM does not tell a correction signal.

How much is the expect calibration map data: like size of sensor. Several operating condition data. One calibration map needed per target environment.

The expectation is only the correction data for the capture environment is sent by encoder in this SEI after learning about the capture environment.

How is the decoder going to use the data: decoder can correct the sample values using the correction information.

Why the encoder does not do the correction for the capture environment before encoding.

Why send a separate calibration data stream in another layer and have the decoder do it, instead of doing it on the encoder? Answer: the image may be already encoded and calibration may be done later.We can logically separate the calibration part of the proposal and the mapping part of the proposal.

About mapping: does it depend on the rendering side? No it is only related to the capture.

Discussed this further on 18:00-18:20, 12 July 2026 after offline discussions:

Modified proposal is to signal only the mapping information and not the calibration related syntax elements nor the auxiliary picture information.

Instead of mapping a higher bit-depth could be used but then higher level is needed.

Agreed to v2 (was edited during discussion/ will be uploaded).

JVET-AQ0147 AHG9: Layer priority SEI message [J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande during 15:50-16:30 on 12 July 2026.

A layer priority SEI message is proposed to describe the priority of pictures of one or more layers in a bitstream. The priority values can be used to recommend relative instantaneous display priority of pictures among the one or more described layers. This information can be used by receivers of bitstreams containing a single layer or multiple layers. Receivers in “multiview” applications, which receive multiple independently-coded simulcast videos, can use this information for purposes such as selection of one or more layers for display, selecting to render the audio of the highest priority layer, determination of the display size of individual layers, etc. Single layer bitstreams can use the priority information to select ranges of pictures with higher priority for display or other processing.

Multiview here refers to multiple cameras or views of same or different activity which may be observed simultaneously by end user.

It was commented that for the example of separate individual bitstreams, this type of information may be better signalled/ handled at system level. It was commented that HLS streaming system in existence today does this at system level for multiple bitstream case.

There was comment that doing this for multiple layers may make sense but not for multiple separate bitstreams.

It was commented that in DASH (mpeg dash and before that 3gpp dash), priority signalling exists since beginning. It was asked if DASH priority is same as the priority considered by the use case mentioned in this proposal. DASH specification has selection priority attribute. The time span is for entire period. It was also commented that there is Role attribute. It was also commented that usually all audio streams may not be delivered and you need to switch the audio. So switching on finer granularity would need a new DASH Period. DASH also supports events which may be used for this.

It was commented that JVET-AQ0188 includes a syntax element priority_id which is similar to this concept.

It was asked if instead of having a new SEI whether this should be included there.

It was commented that in the proposal the semantics of priority just says priority without any aspect about rendering. There was a suggestion to consider this priority signalling in the context of the contributions JVET-AQ0136, JVET-AQ0188.

Further study was recommended.

JVET-AQ0149 AHG9: Classification label info SEI [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)]

Chaired by S. Deshpande during 16:30-17:00 on 12 July 2026.

Two options are proposed to address the use case described in JVET-AP0085 “AHG9: Subject Recognition Information (SRI) SEI message.” In the first option, a classification label information SEI message is proposed to describe the subject or scene of a picture using a classification label. In this option, label types corresponding to each of several popular image classification datasets are specified, and applications may indicate custom classification label types by signalling a URL with a link to a CSV file containing a label list.  In the second option, the text descriptions SEI message is extended to add an object label purpose.

Since normative referencing of these dataset defined label would be problematics, actual values may be define in our standard, or an application defined list via URL to a csv file may be signalled. Integer or string label can be assigned. Only one label type per SEI is proposed.

For option 2, the text description is not free form string, but further extends the TDI SEI.

It was asked if the lists were created for images or for video. That seems to be the case.

How will decoder use this information: There may be specific image quality mechanisms which are done based on this.

If such information is useful, will it be associated with regions or entire picture: this is the main subject or scene.

Can annotated region SEI be used or extended instead? It is possible.

Is there any licensing issue in using the labels from these datasets. Also they could be added as Bibliography instead of normative references. Also it was commented that this classification may be tied to version of that dataset and it could be fast changing/ moving.

If you don’t have a standard use of the label how could the manufacturer use it.

There was a concern that why these 3 datasets are chosen. There are many others.

Extending TDI with additional syntax elements in an extension part for this functionality was not favored.

It was requested to address the above questions in a further study.

JVET-AQ0154 AHG9: Proposed GDR Info SEI message [S. Mate, M. M. Hannuksela, J. Boyce (Nokia)]

Chaired by S. Deshpande during 17:15-18:00 on 12 July 2026.

This contribution proposes a gradual decoding refresh (GDR) info SEI message that provides information about clean areas during the recovery process. This SEI message can be transmitted separately from the slices it describes to avoid losing the slices and the SEI message together.

It was asked and confirmed by proponents that the information signalled is really useful/ targeted only if there is loss.

It was commented that is this information only useful if recovery is not uniform (as otherwise you can derive it based on recovery point picture information). But the direction is not known.

With this SEI, decoder can conclude if it lost a clean or dirty slice and if a dirty slice was lost no need to take any action.

I was commented that AVC slice header signalled similar information for GDR.

It was commented that aspects of error concealment related comments on other proposal also apply here.

It was suggested that some demonstration of the benefit of this information in loss use case may be useful.

If we start from a GDR picture, we could use only the clean area for display and this would be a different use case than loss handling. It was asked if the virtual boundary information in VVC already provides this information. But that information is in PPS. Also slice based GDR does not include virtual boundary information.

Non lossy case of early display was discussed. This signalling would allow showing clean areas only and not showing dirty areas.

It was asked if the experience is quite improved since the duration may be short.

It was commented that HEVC does not have GDR picture so the language is agnostic codec.

The GDR concept is same across codecs but VVC adds normative behavior.

There was a comment that this topic should be discussed/ considered in the context of next video codec where the exact design choice of where and what information to signal (or not signal) related to GDR is TBD.

It was asked if this improves the use experience for the non-loss case compared to not having this information.

It was asked if AVC/ HEVC legacy devices will utilize such a new SEI which may be standardized after some time.

It was asked if VVC GDR support layered design.

Further study was recommended.

JVET-AQ0166 AHG9: C2PA SEI Message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

Deferred to next meeting.

JVET-AQ0179 AHG9: Processing History Information SEI [C. Feldmann, T. Biatek, J. Boyce (Nokia), J. Pardo, T. Solovyev, E. Alshina (Huawei), X. Xu, S. Liu (Tencent)]

Chaired by S. Deshpande during 12:00-12:15 on 12 July 2026.

In a real-world streaming application, the recorded video traverses many steps, such as filtering, transcoding, resampling, …, before the final output bitstreams reach the end user.  Keeping track of the individual steps or debugging problems in the entire processing chain poses a significant problem. With a full history of the processing, it would be possible to detect issues with the video quality early on and enable detection of errors in individual processing steps or the combination of processing steps.

In the -v3 version, it is proposed to combine with proposal JVET-AQ0139 in section 4.

Nesting SEIs example: is QM SEI or EOI SEI.

Comment: it is unclear what the decoder will do with this information and the design is complicated.

Showcase: JVET-AQ0139 has a showcase. Sees additional notes there.

It was asked if text description can provide history just textually. The SEI provides much more information that just text description.

Further study

## Usage of SEI messages for Gaussian splatting technology (35)

### Summary reports and general documents (1)

JVET-AQ0022 Reports of joint exploration experiments JEE6.7 and JEE6.9 on Gaussian splat coding [J. Jung, W. Husak (JAHG co-chairs on behalf of JVET)]

Excerpts from WG 5 m77945 (for JEE 6.7) and m78054 (for JEE 6.9), for information

### Syntax and semantics of Gaussian splat coding SEI messages in TuC (7)

Contributions in this area were discussed in HLS track during 1040–1230 on Monday 13 July 2026 (chaired by J. Boyce).

JVET-AQ0067 in section 6.5.3 is also related, answering questions asked in HLS track in 42nd meeting.

JVET-AQ0079 AHG9: Editorial changes for the GSI SEI message in VSEI TuC [Y.-K. Wang, Y. Xu, J. Xu, Q. Yang (Bytedance)]

This contrition proposes a large number of assertedly purely editorial changes to the syntax and semantics of the GSI SEI message in VSEI TuC. Marked changes are provided in this contribution document itself.

Delegate to the editors.

It was noted that JVET-AQ0131 and JVET-AQ0102 has some overlap.

JVET-AQ0080 AHG9: On coding of some syntax elements in the GSI SEI message [Y.-K. Wang, J. Xu, Y. Xu, Q. Yang, K. Zhang (Bytedance)]

JVET-AQ0080 item 1 is a superset of JVET-AQ0131 item 9.

JVET-AQ0080 items 4, 5, and 8 and JVET-AQ0132 item 1 should be discussed together.

This contribution proposes the following changes to the GSI SEI message in TuC clause 8.58.1:

1)	Skip the signalling of the last instance of gsi_patch_num_splats_minus1[ p ] and infer the value instead, and specify that the value of gsi_patch_num_splats_minus1[ p ] shall be in the range of 0 to gsi_num_gaussians_minus1, inclusive.

2)	Change the coding of gsi_quantization_idc from u(1) to u(3).

3)	Change the coding of gsi_transform_idc[ i ] from ue(v) to u(3).

4)	Change gsi_num_sh_levels to gsi_num_sh_levels_minus1, change the semantics accordingly, and specify that the value of gsi_num_sh_levels_minus1 shall be in the range of 0 to 3, inclusive.

5)	Change gsi_num_sh_channels[ i ] to gsi_num_sh_channels_minus1[ i ], change the semantics accordingly, and specify that the value of gsi_num_sh_channels_minus1[ i ] shall be in the range of 0 to 6 * i + 2, inclusive.

6)	Change gsi_num_regions_minus1[ i ][ c ] to gsi_num_regions_minus2[ i ][ c ], change the semantics accordingly, and specify that the value of gsi_num_regions_minus2[ i ][ c ] shall be in the range of 0 to 2, inclusive.

7)	Change the name of the semantics gsi_num_bits_for_transition_point[ i ][ c ] to gsi_num_bits_for_transition_point_minus1[ i ][ c ] and change the semantics accordingly.

8)	Change gsi_tr_row_num_groups to gsi_tr_row_num_groups_minus1 and change the semantics accordingly.

9)	Change the coding of gsi_row_tr_size_minus1[ i ] from u(4) to u(2).

Agreed to item 1.

Item 2, agreed to change to 2 bits from 1.

Items 4, 5, and 8 is related to JVET-AQ0132 item 1. Defer until that is presented. Items 4 and 8 agreed as noted under JVET-AQ0132.

Further study item 6.

Item 7 is not needed because of JVET-AQ0174 decision.

Item 9, agreed to change the semantics to have reserved values.

JVET-AQ0081 AHG9: On persistence etc. and error resilience of the GSI SEI message [Y.-K. Wang, J. Xu, Y. Xu, Q. Yang, K. Zhang (Bytedance)]

This contribution proposes the following changes related to the GSI SEI message in TuC clause 8.58.1:

1)	Add the following constraint: When there is a GSI SEI message with gsi_cancel_flag equal to 0 present for a picture in a CLVS, there shall be a GSI SEI message with gsi_cancel_flag equal to 0 present for the first picture in the CLVS in either decoding or output order.

2)	Add the following constraint: When one picture in a CLVS is in the persistence scope of a GSI SEI message, each picture in the CLVS shall be in the persistence scope of a GSI SEI message.

3)	Add the following constraint: It is a requirement of bitstream conformance that NumConstituentFrames shall be the same within a CLVS.

4)	Add the following constraint: It is a requirement of bitstream conformance that a CLVS containing a GSI SEI message shall have m * NumConstituentFrames coded pictures, where m is a positive integer.

5)	Add the following constraint: It is a requirement of bitstream conformance that a GSI SEI message shall only be present at the i-th picture in output order in a CLVS where i is equal to n * NumConstituentFrames, where n is a non-negative integer.

6)	Replace the semantics of gsi_persistence_flag equal to 1 with the following: gsi_persistence_flag equal to 1 specifies that the GSI SEI message applies to the current and subsequent NumConstituentFrames − 1 decoded picture(s) and persists for all subsequent pictures in output order in the current CLVS until the next PU, when present, that follows the current PU in output order and contains a GSI SEI message in the current CLVS.

7)	Add a new SEI message named GSI constituent picture index (GCPI) SEI message, which signals the value of the constituent picture index of a constituent picture.

Items 1, 2, and 6, chaired by S. Deshpande.

For items 1, 2, and 6, agreed to remove the persistence and cancel flags and add the following language:

When a GSI SEI message is present for any picture of a CLVS of a particular layer, a GSI SEI message shall be present for the CLVSS picture. The GSI SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS or until replaced by the next GSI SEI message in output order.

Other items chaired by J. Boyce.

For items 3, 4 and 5 there was some concern if the proposed constraint is too constraining, especially for the case of dynamic Gaussian splats.

For item 7, usefulness of the identification of the constituent pictures was unclear.

JVET-AQ0082 AHG9: On value ranges and decoder handling of reserved values for syntax elements in the GSI SEI message [Y.-K. Wang, J. Xu, Q. Yang, Y. Xu, K. Zhang (Bytedance)]

JVET-AQ0082 items 3.b and 3.c, JVET-AQ0102 item 5, and the corresponding items in JVET-AQ0168 should be decided together.

This contribution proposes the following changes for the GSI SEI message:

1)	Add value ranges for the following ue(v)-coded syntax elements:

a.	gsi_num_patches_minus1: 0..gsi_num_gaussians_minus1

b.	gsi_comp_location_idc: 0..7

2)	Change value ranges for the following ue(v)-coded syntax elements:

a.	gsi_component_id[ i ]: from 0..62 to 0..1023

3)	Add value ranges for the following fl(32)-coded syntax elements:

a.	gsi_quantization_params[ i ][ j ]: > 0

b.	gsi_transform_scaling_factor[ i ]: > 0

c.	gsi_transform_base[ i ]: > 0

d.	gsi_region_step_size[ i ][ c ][ m ]: > 0

e.	gsi_klt_coeff[ i ][ m ][ n ]: −1.0..1.0

f.	gsi_klt_std_dev[ i ][ m ]: >= 0

4)	Add decoder handling of reserved values for the following syntax elements:

a.	gsi_comp_location_idc: ignore the GSI SEI message

b.	gsi_quantization_idc: ignore the GSI SEI message

c.	gsi_rotation_idc: ignore the GSI SEI message

d.	gsi_component_id[ i ]: ignore the i-th component information

e.	gsi_transform_idc[ i ]: ignore the i-th component information

Agreed to item 1a, item 1b.

No action on item 3a.

For item 3 (value ranges of float syntax elements), ed. notes should be added in the semantics that those are missing. This would also require some alignment with software development, and further study might be needed whether usage of float values is desirable for these syntax elements, as rounding errors might also cause mismatch in cross-checks.

Agreed to item 4a, 4b, 4c, 4e.

Further study on item 4d and consider interaction with JVET-AQ0132 item 5.

JVET-AQ0085 AHG9: On semantics of some syntax elements in the GSI SEI message [Y. Xu, Q. Yang, J. Xu, Y.-K. Wang, K. Zhang, L. Zhang (Bytedance)]

AQ0085-item 2 and JVET-AQ0132 item 1 should be discussed together.

JVET-AQ0085 items 3 to 5, JVET-AQ0088 item 2, and JVET-AQ0131 item 8 should be discussed together.

JVET-AQ0085 item 6, JVET-AQ0102 item 4, and JVET-AQ0131 item 4 should be discussed together.

This contribution proposes the following changes related to the GSI SEI message in TuC clause 8.58.1:

1)	On the semantic of gsi_block_width and gsi_block_height: specify conditions and corresponding cases to derive the Gaussian index based on gsi_block_width and gsi_block_height.

2)	On the semantic of gsi_num_gaussians_minus1: specify the condition to associate the first gsi_num_gaussians_minus1 + 1 samples of the region with the Gaussian splats.

3)	On the semantic of gsi_comp_location_idc: extend the constraint on constituent frame to constituent picture and/or colour plane.

4)	On the semantic of gsi_comp_location_idc: specify how to process attributes with N components when N is not equal to 3.

5)	On the semantic of gsi_comp_location_idc: specify conditions and corresponding cases to associate the first several samples of the region with the Gaussian splats based on the value of gsi_comp_location_idc.

6)	On the semantic of gsi_colour_plane_idc: specify the inferred value for gsi_colour_plane_idc when gsi_implicit_packing_flag is equal to 0 and gsi_comp_location_idc is equal to 1.

7)	On the semantic of gsi_transition_point: add a constraint as, gsi_transition_point[ i ][ c ][ m ] shall be less than gsi_transition_point[ i ][ c ][ n ] when m is less than n.

Further study encouraged. The current semantics are under described and it is useful to provide more clarity. This contribution probably should have been categorized differently.

JVET-AQ0131 AHG9: On the GSI SEI message [S. Harribey, M. Pettersson, M. Damghanian, R. Sjöberg (Ericsson)]

JVET-AQ0131 items 1 and 2 are also in JVET-AQ0079.

This contribution proposes the following changes to the Gaussian splatting information (GSI) SEI message in the TuC (JVET-AP2032, section 8.58.1):

1.	Change the name of several syntax elements in the semantics of gsi_patch_position_msb_present_flag, gsi_patch_postion_msb_len_minus1 and gsi_comp_location_idc to match the names of the corresponding syntax elements in the syntax table. Add the word “specifies” in the semantics of gsi_comp_location_idc.

2.	Correct a typo in the semantics of gsi_component_id[ i ].

3.	Change the constraint on the value range for gsi_num_components_per_frame_minus1 to “0 to min(NumBlocksPerFrame − 1, gsi_number_of_components_minus1)”.

4.	Replace the constraint “When gsi_implicit_packing_flag is equal to 1” by “When gsi_implicit_packing_flag is equal to 0” in the semantics of gsi_colour_plane_idc[ i ].

5.	Add a reference to the VSEI table defining the variables SubWidthC and SubHeightC.

6.	Correct a typo in the scanning process for the variable NumComponentsPerFrame.

7.	Signal gsi_block_width_minus1 and gsi_block_height_minus1 instead of gsi_block_width and gsi_block_height. Change the range for gsi_num_gaussians_minus1 to “0 to 232 − 1”.

8.	In the semantics of gsi_comp_location_idc, replace “separate constituent frames” by “separate blocks” and “same constituent frame” by “same luma block”.

9.	In the semantics of gsi_patch_num_splats_minus1[ p ] change the range to “0 to gsi_num_gaussians_minus1”.

10.	Use descriptor ue(v) for gsi_sh_conversion and reserve values greater than 3 for future use.

Item 5, 6 delegated to editors.

No action on item 7 aspect 2 because of avoiding more than 64 bits in a ue(v) to express the value.

Agreed to item 8, with exact language delegated to editors.

It was suggested to include the equations for the transform.

For item 10, the SH conversion is use to convert spherical harmonics from YUV to RGB to invert the conversion done at the encoder side.

It is noted that the current software uses YUV to RGB conversion of BT.601.

There is support for allocating reserved values for the SH conversion.

The current semantics are under-specified in that they don’t make clear what the actual transformation process is. If the standards for colour conversion are used, they would need to be properly referenced.

It is noted that the V-PCC spec defines the use of additional colour spaces include YCoCg.

It was suggested that a fixed length code with more bits may be more appropriate than ue(v).

Agreed with u(4) coding.

JVET-AQ0132 AHG9: On signalling of various parameters in the GSI SEI message [S. Harribey, M. Pettersson, M. Damghanian, R. Sjöberg (Ericsson)]

This contribution proposes updates to the signalling and inference of various parameters in the Gaussian splatting information (GSI) SEI message in the TuC for future extension of VSEI. The following items are proposed:

- Changes to the signalling of SH quantization and 2D transforms parameters. Two alternatives are proposed:

  - Remove the two presence flags gsi_sh_quant_regions_present_flag and gsi_transform_2d_present_flag. The presence of SH quantization and 2D transform parameters may instead be inferred from the value of gsi_num_sh_levels, respectively gsi_tr_row_num_groups, being equal or greater than zero,

  - Keep the two presence flags but signal gsi_num_sh_levels_minus1 and gsi_tr_row_groups_minus1 as these two syntax elements cannot be zero when SH quantization parameters, respectively 2D transform parameters, are present.

Both alternatives also propose to signal the number of channels with gsi_num_sh_channels_minus1.

- Infer the number of Gaussians from a signalled number of non-gaussians samples and the block size in luma samples.

- In implicit mode, infer the total number of conveyed components from a signalled number of components per frame and a signalled number of constituent frames.

- In implicit mode, infer the number of components per frame from a signalled number of empty blocks per frame and the total number of blocks per frame.

- Two alternatives are proposed to signal the number of components in explicit mode

  - The total number of components and the component IDs are not signalled. Instead, a loop over all possible components is used and for each component a flag indicates if the component is signalled or not.

  - The current loop over conveyed components with signalling of the component ID is kept but the total number of components is inferred from a signalled number of unused components and the maximum number of components that may be conveyed.

- In explicit mode, infer the top-left position of a block from a signalled block index.

- Changes to the signalling of target bitdepth. Two alternatives are proposed:

  - Signal target bitdepth minus 8 for both position-related and attribute 3DGS components with ue(v).

  - Use an index to signal target bitdepth with the most common values.

Agreed to item 1 option 2 for gsi_num_sh_levels_minus1. Suggest to consider changing the name to be a non-uniform quantization enable flag rather than a presence flag. And to have gsi_tr_row_num_groups_minus1.

Further study on item 2. Semantics language and interaction with patches should be considered.

For item 3, it was noted that the semantics change places restrictions on the number of components or require the use of empty components. It was suggested that the derivation of the new variable would need modification.

For item 4, it was suggested that the proposed naming of num empty blocks is misleading because empty may be explicitly signalled. Alternatively it could be named num undescribed blocks, or unsignalled blocks or similar.

Agreed on item 4, with naming of the syntax element delegated to the editors.

On item 5, the proposal is for explicit mode to set the signalling order in index order with a presence flag rather than the current design which can signal in any order with an explicit id signalled.

An alternative was suggested to code the component id delta to save bits.

Option 1 was preferred.

Further study on item 5 option 1, considering extensibility in the number of component id values.

For item 6, it was noted that this proposal would limit flexibility in location of blocks to align with a grid, but would provide bitrate savings. Further study was recommended.

Agreed to item 7 option 1.

### Reports of experimentation and software (9)

Contributions in this area were discussed in Joint AHG meeting during on Saturday 11 July 2026 an Sunday 12 July 2026 (chaired by J. Jung and W. Husak).

JVET-AQ0068 AHG9: [GSC][JEE6.7] TV-GS: triplane video-based implicit Gaussian splatting representation compression [B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)]

Registered as WG 5 M77865

JVET-AQ0236 Crosscheck of JEE 6.7 Test 11 Triplane video-based implicit representation compression JVET-AQ0068 and JVET-AQ0069 [P. Rondao Alface, B. van Hauwermeiren (Nokia)] [late]

Registered as WG 5 M77566

JVET-AQ0067 AHG 9: Carrying Gaussian splat data in Main10 bitstreams using JVET-AP0100 [L. Kerofsky, J. Jung, Y. He, M. Karczewicz (Qualcomm)]

Registered as WG 5 M77421

JVET-AQ0170 AHG9: Multi-frame improvement of the GSC V-SEI framework [J. Ricard, G. Teniou, S. Wenger (Tencent), J. Jung (Qualcomm)]

Registered as WG 5 M77438

JVET-AQ0203 [GSC][JEE.6.7][Test 1] Cross-check of non-linear transforms [J. Ricard, G. Teniou, S. Wenger (Tencent)] [late]

Registered as WG 5 M77540

JVET-AQ0207 AHG9: [GSC][JEE6.7] Cross-check results and comments on the GSC VSEI framework [J. Jung, Y. Shao, M. Karczewicz (Qualcomm)] [late]

Registered as WG 5 M77350 which was uploaded on time

JVET-AQ0208 AHG9: Evaluation of the GSC VSEI framework [J. Jung, L. Kerofsky, Y. He, M. Karczewicz (Qualcomm)] [late]

Registered as WG 5 M77353 which was uploaded on time

JVET-AQ0209 AHG9: Improvements of the GSC VSEI framework [J. Jung, L. Kerofsky, Y. He, M. Karczewicz (Qualcomm)] [late]

Registered as WG 5 M77358 which was uploaded on time

JVET-AQ0224 [GSC][JEE6.7] Crosscheck on Improvements of the GSC VSEI framework [J. Do, H. Lee, G. Bang (ETRI)] [late]

Registered as WG 5 M77890

JVET-AQ0210 AHG9: Spatial Random Access implementation in GSC VSEI framework [Y. Shao, J. Jung, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)] [late]

Registered as WG 5 M77364 which was uploaded on time

JVET-AQ0225 AHG9: [GSC] Cross-check of Spatial Random Access [J. Ricard, G. Teniou, S. Wenger (Tencent)] [late]

Registered as WG 5 M77942

JVET-AQ0219 AHG9: [GSC][JEE6.7][JEE6.9] On the implementation of the implicit gaussian splatting information SEI message [S. Sasse, Y. Berendsohn, T. M. Borges, Y. Sanchez, S. Lee, R. Skupin, C. Hellge, T. Schierl (HHI)] [late]

Registered as WG 5 M77937

JVET-AQ0237 [GSC] [JEE 6.7] Crosscheck of Test 10 Implicit intermediate representation-based compression JVET-AP0213 [P. Rondao Alface, B. van Hauwermeiren (Nokia)] [late]

Registered as WG 5 M77511

JVET-AQ0223 AHG9: [GSC][JEE6.7] CE Test Description for Test 1 Quantization – Generic Exponential Transform ]J. Do, H. Lee, G. Bang (ETRI)] [late]

Registered as WG 5 M77627 which was uploaded on time

JVET-AQ0227 [GSC][JEE.6.5] Optimization of the GSC metric software [J. Ricard, G. Teniou, S. Wenger (Tencent)] [late]

Registered as WG 5 M77943

### Potential extensions of GSC SEI messages (18)

Contributions in this area were discussed in Joint AHG meeting during on Saturday 11 July 2026 an Sunday 12 July 2026 (chaired by J. Jung and W. Husak), unless otherwise noted. In some cases, follow-up discussion was conducted on syntax and semantics definition principles in the JVET HLS track, in cases requested by the JAHG.

JVET-AQ0069 AHG9: [GSC] Coordinate-remapping in triplane video-based implicit Gaussian splatting framework [B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)]

Registered as WG 5 M 77866

JVET-AQ0212 Crosscheck of JVET-AQ0069 (AHG9: [GSC] Coordinate-remapping in triplane video-based implicit Gaussian splatting framework) [X. Wu (City Univ. HK), B. Chen (Alibaba)] [late]

Registered as WG 5 M 77901

JVET-AQ0232 AHG9: Crosscheck of JVET-AQ0069 Coordinate-remapping in triplane videos [B. Kroon (Philips)] [late]

Registered as WG 5 M 77653

JVET-AQ0070 AHG9: [GSC] Improved SEI message for triplane video-based implicit Gaussian splatting representation [B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)]

Registered as WG 5 M 77867

Referred from the Joint AHG to the HLS track.

Chaired by J. Boyce on Monday 13 July at 0900.

This contribution proposes an improved SEI message for triplane video-based implicit Gaussian splatting representation in JVET-AP0079. The SEI message is incorporated into JVET-AP2032, “Technologies under consideration for future extensions of VSEI (version 12)”.

The contribution covers the following three aspects:

- Proposes a cleaner design for implicit Gaussian splatting representation: removing the syntax elements that are not needed for implicit representation

- Add the missing binary mask feature from JVET-AQ0068 and the new coordinate remapping feature from JVET-AQ0069

- Add sections for 3DGS implicit components reconstructions, 3DGS implicit components inverse quantization, and an example of 3DGS implicit rendering process: triplane video-based Gaussian splatting (TV-GS) representations, to complete the specification.

V2.0 contains some bug fix of the specification text.

A question was raised about why gsi_num_lut_x_minus1 is coded as ue(v). It was noted that a range is needed to be specified.

In Note 1, it was suggested to refer to “the number” of gaussians.

It is noted that gsii_quantization_type_flag does not need to have a table, or if a table was used, it should have a reserved value listed.

It is noted that the table for gsii_component_id[ i ] has different attribute components defined than are defined in the first GSI SEI message included in the TuC (in section 8.58.1).

Block widths differ per component with this approach. It does not have implicit packing mode.

The prior syntax included many syntax elements from the AP0100 design which were not directly used and are removed in this version.

Approach uses 2 bitstreams for the experiments, but would be expected to migrate to a single bitstream, with an SEI message in each PU.

First bitstream includes anchor position, MLP, LUT, and binary mask, all of which is losslessly coded.

Second bitstream contains the feature planes and is lossy coded.

In the EE tests, the picture in the first bitstream are padded to be the same size as the pictures in the second bitstream.

Both bitstreams contain an SEI message, with different components described, according what is the present in the picture containing the SEI message.

Agreed.

JVET-AQ0083 AHG9: Non-uniform quantization for scale and opacity support in the GSI SEI message [Y. Xu, Q. Yang, Y.-K. Wang, J. Xu, K. Zhang, L. Zhang (Bytedance)]

Registered as WG 5 M 77527

JVET-AQ0084 AHG9: Patch quantization offset and LoD information in the GSI SEI message [Y. Xu, Y.-K. Wang, J. Xu, K. Zhang, Q. Yang, L. Zhang (Bytedance)]

Registered as WG 5 M 77528

JVET-AQ0088 AHG9: On the GSI SEI message syntax and semantics [Y. He, J. Jung, R. Hooda, L. Kerofsky, M. Karczewicz (Qualcomm)]

Also registered as WG 5 M 77339 but pure syntax/semantics improvement of existing SEI, therefore also discussed in JVET HLS track independent of the JAHG.

JVET-AQ0088 item 4 is the same as JVET-AQ0131 item 7 part 1.

This contribution proposes the following modification to the GSI SEI message.

1.	propose syntax element to differentiate common packing and patch packing

2.	update gsi_comp_location_idc semantic

3.	propose the inference value of gsi_patch_3d_position_msb[ p ][ c ]

4.	change gsi_block_width and gsi_block_height to gsi_block_width_minus1 and gsi_block_height_minus1

5.	Condition the component quantization and the relevant parameters

6.	Condition the presence of bitdepth and transform syntax elements

Further study on item 1, 2.

For item 3, it isn’t clear that inference is necessary because the syntax element is not used.

It is noted that the usage of many syntax elements is not fully described in the current semantics. It was suggested that a process should be described that is sufficient to reconstruct the input format available to the encoder, but it isn’t considered to be necessary to describe the process to render a 2D picture from the splats.

Agreed on item 4.

Item 4 also proposed in JVET-AQ0131 item 7 part 1.

Agreed on item 5. Further study on if the enable and present parameters should also apply to non-uniform quantization.

For item 6, the expected gains are small and in some cases may have loss.

No action on item 6.

JVET-AQ0094 AHG9: On SH non-uniform quantization signalling in the GSI SEI message [H. Lee, J. Do, G. Bang (ETRI)]

Registered as WG 5 M 77616

JVET-AQ0095 AHG9: On unified signalling of SH transforms in the GSI SEI message [H. Lee, J. Do, G. Bang (ETRI)]

Registered as WG 5 M 77617

JVET-AQ0100 AHG9: On the GSI SEI message for spatial random access [Y. He, J. Jung, Y. Shao, L. Kerofsky, M. Karczewicz (Qualcomm)]

Registered as WG 5 M 77340

JVET-AQ0102 AHG9: Syntax and semantics clean-up of the GSI SEI message [J. Do, H. Lee, G. Bang (ETRI)]

Also registered as WG 5 M 77628 but pure syntax/semantics improvement of existing SEI, therefore also discussed in JVET HLS track independent of the JAHG.

JVET-AQ0102 item 1, JVET-AQ0131 item 3, and JVET-AQ0132 item 4 should be discussed together.

The editorial part (removal of index) of JVET-AQ0102 item 2 is also in JVET-AQ0079.

This contribution proposes clean-up modifications to the Gaussian Splatting Information (GSI) SEI message for video-based Gaussian Splat Coding (GSC). The proposed changes keep the existing GSI SEI syntax and semantics structure, while resolving several inconsistencies identified during review of the current text.

Agreed on item 2, item 3.

Further study for item 4.

For item 5, see notes for JVET-AQ0082 item 3.

JVET-AQ0113 AHG9: On Gaussian Splatting Information SEI message enhancements [Y. Hu, X. Ma (MLS Labs), Y. Xu, K. Yang (SJTU)]

Registered as WG 5 M 77365

JVET-AQ0148 AHG9: Support for multiple bitstreams and alternative splat types in Gaussian splatting info SEI [J. Boyce, M. M. Hannuksela (Nokia)]

Registered as WG 5 M77518

JVET-AQ0164 AHG9: Cross-platform determinism and bit-exact conformance of the GSC V-SEI framework [J. Ricard, G. Teniou, S. Wenger (Tencent)]

Registered as WG 5 M77431

JVET-AQ0167 AHG9: Integer-only GSC V-SEI decoder [J. Ricard, G. Teniou, S. Wenger (Tencent)]

Registered as WG 5 M77433

m77899 [GSC JEE6.7] Cross-check of m77433 on integer-only VSEI decoder [J. Jung, Y. Shao (Qualcomm)] [late]

Not registered as JVET document

JVET-AQ0168 AHG9: High-level syntax modifications and bitstream specification for an integer-based GSC V-SEI decoder pipeline [J. Ricard, G. Teniou, S. Wenger (Tencent)]

Registered as WG 5 M77435

JVET-AQ0169 AHG9: Single-video GSC V-SEI framework [J. Ricard, G. Teniou, S. Wenger (Tencent)]

Registered as WG 5 M77436

JVET-AQ0174 AHG9: On the Gaussian splatting information SEI message [R. Hooda, J. Jung, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

Registered as WG 5 M77458

Referred by Joint AHG to the HLS track.

Chaired by J. Boyce on 13 July at 0955.

The GSC SEI message, as described in JVET-AP0100, includes non-uniform transform and PCA transform tools inherited from V-PCC amd.1. During the MPEG#154 meeting, some syntax details for these tools were modified in V-PCC Amd.1. This contribution proposes corresponding updates of the GSC SEI message to align with this new syntax, for non-uniform quantization and PCA transform of spherical harmonics. It is proposed to adopt these changes in the JVET TuC.

It was suggested that the name region was somewhat confusing and another terminology might be used, such as quantization intervals.

It was suggested that a range limit is needed for gsi_transition_point_delta[ ][ ][ ].

The proposal reduces the bitrate of the SEI message contents but does not affect the coded picture sample values.

Agreed.

JVET-AQ0184 AHG9: [GSC][JEE6.7][JEE6.9] On the Implicit GSI SEI message [Y. Sanchez, S. Sasse, T. M. Borges, S. Lee, Y. Berendsohn, R. Skupin, C. Hellge, T. Schierl (HHI)]

Registered as WG 5 M77936

Referred by Joint AHG to the HLS track.

Chaired by J. Boyce on 13 July at 1015.

This contribution proposes a cleaned-up version of the Gaussian Splatting Information (GSI) SEI message for implicit representations, building on JVET-AP0213. Unused syntax that was inherited from basing the SEI message on the initial contribution for Gaussian Splatting in JVET, i.e., JVET-AM0212, has been removed. Each 3DGS attribute (scale, rotation, opacity, colour) may be carried as an explicit representation or as an implicit representation using feature planes and small neural networks. For the implicit representation case, two options are allowed for recovering the attribute values, namely using NNPFC and a new option in which the parameters of an MLP are signalled directly in the GSI SEI message, resulting in a self-contained SEI message.

The prior design uses the NNPF SEI messages to recover the attributes.

This version adds an option for signalling the MLP explicitly. It does not use NNC which is used in the NNPF approach. Explicit signalling of the MLP is asserted to be simpler than NNC but is less efficient.

It is noted that the second GSI SEI message has an alternative approach to signal the MLP as sample values.

The MLP explicit signalling uses fl(32) for the weighs and biases. It was questioned if 32 bits was need in all cases. It would be possible to explicitly signal the length of the floating point number.

It was suggested to change the layer size to be minus1.

It was suggested to have unique names for the three GS SEI messages in the TuC to ease in referring to them. Perhaps: GSI, GSII, and hybrid GSEI.

The contribution removes syntax from the GSI SEI that are not used in this approach, including the layer ID and colour representation.

The proposed addition of the explicit signalling of the MLP parameters does not change the coded picture samples, but increases the bitrate of the SEI message but avoids the need to use NNPF/NNC.

Agreed.

JVET-AQ0215 AHG9: New transform type for Gaussian splatting information SEI message [H. Yang, D. Kim, B. Choi, J.-G. Kim (KAU), K. Oh, G. Lee (ETRI)] [late]

Registered as WG 5 M77668 which was uploaded on time

JVET-AQ0234 AHG9: Crosscheck for m77668/JVET-AQ0215 [Y. Xu, K. Zhang, L. Zhang (Bytedance)] [late]

Registered as WG 5 M77950

JVET-AQ0235 AHG9: Rotation representation extensions of Gaussian Splatting Information SEI message [Z. Zhu, S. Liu, L. Yu, Y. Liao (Zhejiang University)] [late]

Registered as WG 5 M77844

This contribution proposes to extend the Gaussian splatting information (GSI) SEI message to support a Rodrigues-vector rotation representation. The underlying canonicalization and rotation representation technique has been proposed, studied, and cross-checked in previous contributions. This proposal specifies the syntax and semantics needed to signal the representation in the GSI SEI message. This contribution is also registered as MPEG proposal with ID m77844.

Was presented in joint meeting Monday 13 July 1615 (see section 7.3.1).

NF: Forward facing & Object-centric sequences

1F: Forward facing sequences

1F: Object-centric sequences

This method had been proven to provide benefit in the context of GSCS (platform for video-based solutions in GS coding) and G-PCC, as per results above. It was agreed to include it in the next round of JEE6.7 to investigate its performance in the context of GSI SEI.

JVET-AQ0239 AHG9: [GSC][JEE6.9] Scalable 3DGS Syntax in GSI SEI [R. Koo, Y.-G. Kim, E.-S. Ryu (SKKU)] [late]

Registered as WG 5 M77844

The following WG 5 documents also fall into this category and were reviewed in the Joint AHG:

m77658  [GSC JEE6.3 JEE.6.7] FreeTimeGS-based A-3DGS extending I-3DGS to NF in the GSC V-SEI framework: experimental results [Hong-Chang Shin, Gwangsoon Lee, Kwan-Jung Oh, Jun Young Jeong] [late]

Not registered as JVET document

m77843  [GSC] SH Dimension-reduction extensions of Gaussian splatting information SEI message [Zhiwei Zhu, Lu Yu, Yiyi Liao] [late]

Not registered as JVET document

## Usage of SEI messages for avatar coding (3)

Contributions in this area were discussed in a joint meeting with MPEG WGs 3 and 7 during 1535–1605 on Tuesday 14 July 2026 (chaired by JRO, M. Preda and T. Stockhammer).

It was reported by T. Stockhammer on behalf of WG 3 that JVET-AQ0142 had been reviewed in a BoG on animation compression (see report m77834) that would fit into a wider framework of avatar compression. An initial conclusion had been drawn that it would be premature to go into definition of binary representation formats such as SEI messages, as some more fundamental aspects need  to be clarified first. Document m78036 had been created on this basis, and was shortly presented in the session. It is planned to create a WG 7 output document on this basis.

JVET-AQ0142 AHG9: [GSC-related] An animation-based 3DGS Avatar Coding [Z. Zhang, S. Yin, X. Wu, S. Wang (CityUHK), B. Chen, J. Chen, R.-L. Liao, Y. Yan (Alibaba), A. Trioux, F. Yang (Xidian Univ), C. Wang (PCL), R. Wang (PKUSZ)]

This proposal demonstrates an efficient 3DGS avatar coding framework that leverages compact animation parameters and canonical-to-target transformation to enable high-quality 3D human avatar video compression at ultra-low bit rates. At the encoder side, a canonical avatar is trained to represent the appearance while the animation parameters are fitted via a human template to model the temporal pose changes. At the decoder side, the canonical avatar is deformed under the guidance of animation parameters to reconstruct the target avatars. The canonical avatar can be represented using the GSC method being explored in JVET, and additional signalling of the compact animation parameters can be studied as an extension to support wider applications. Our experiments show that the proposed method can significantly outperform the anchor method in terms of rate-distortion performance.

A short presentation was given in the session.

The concept is basically similar as in generative face SEI, but different definition of models/parameters/keypoints etc., and significantly higher bitrate. Significant improvement in compression relative to anchor used in animation compression AHG was reported.

Further study in wider context across WGs was suggested.

JVET-AQ0143 AHG9: [GSC-related] Avatar Generation Information SEI message J. Chen, B. Chen, Y. Yan (Alibaba), S. Yin, Z. Zhang, S. Wang (CityUHK), [A. Trioux, F. Yang (Xidian Univ.)]

This contribution proposes an avatar generation information (AGI) SEI message to support the animation-based 3DGS avatar coding framework introduced in JVET-AQ0142. The proposed SEI message conveys the avatar attributes required to generate target avatar frames from a base 3DGS avatar. By signalling these avatar attributes together with the base 3DGS avatar, the decoder can derive target 3DGS avatars via human-template-based deformation, thereby enabling efficient avatar coding.

For information – no need for presentation according to proponents, further study in wider context across WGs, no action at this moment

JVET-AQ0221 AHG9: [Avatar-related] Avatar Representation Information SEI Message for Coded Video Bitstreams [J. P. Cova Regateiro, G. Martin-Cocher (InterDigital)] [late]

This contribution proposes an Avatar Representation Information (ARI) Supplemental Enhancement Information (SEI) message. The proposed SEI message provides an optional video-bitstream-level mechanism to carry, reference, update, cancel, or synchronize avatar representation information associated with a video sequence, access unit, output time, or communication session.

The ARI SEI message proposed in this contribution provides a structured message for static avatar information, animation information, 3DGS model or update information, and references to MPEG-I Avatar Representation Format (ARF) resources allowing support for multiple avatar generation, animation, or representation processes and different avatar representation formats. The coded video bitstream is not proposed as the root container for a complete immersive scene. Instead, the SEI message provides a timed binding between avatar information and the coded video stream when such binding is useful.

The ARI SEI message is not intended to replace MPEG-I ARF or scene description. ARF and scene description remain appropriate mechanisms for describing avatar assets and rich immersive scenes. The ARI SEI message provides a supplemental video-bitstream-level association when avatar information is related to a coded video sequence, access unit, output time, or communication session.

For information – no need for presentation according to proponents, further study in wider context across WGs, no action at this moment

## SEI software and showcases (7)

Contributions in this area were discussed during 1610–1635 on Friday 10 July 2026 (chaired by J. Boyce).

JVET-AQ0046 AHG9: Implementation and showcase of the luma adaptation extension of the EOI SEI message [H. Zhang, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande.

This contribution reports that an independent implementation of the luma adaptation extension for the encoder optimization information (EOI) SEI message was done by a software developer for both encoding and decoding the extended EOI SEI message that supports luma adaptation for machine analysis tasks. A showcase using the extended EOI SEI message in a GStreamer-based end-to-end pipeline with object detection in the decoding side is demonstrated.

The luma adaptation benefit was apparent from the example.

JVET-AQ0115 AHG9: Showcase for DSC SEI messages for subpicture-based signing in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE), M. Petterson, R. Sjöberg (Ericsson)]

This contribution describes the showcase implementation for the extension of digitally signed content (DSC) SEI messages for subpicture-based signing, which were proposed / improved in several contributions and adopted into TuC of VSEI. Based on the demonstrated functionality it is proposed to add DSC SEI messages in TuC into VSEI v5 with the proposed bug fix.

In the v2 of this contribution, new authors were added, unnecessary wording in the document header was removed.

In the v3 of this contribution, patent rights declarations were updated.

JVET-AQ0128 AHG9: Software implementation of luma adaptation optimization in the EOI SEI message and showcase for display power adaptation [C.-H. Demarty, A. Ak, F. Aumont (InterDigital)]

Luma adaptation optimization has been adopted in the Encoder Optimization Information (EOI) SEI message in the VSEI TuC (JVET-AK0155 and JVET-AL0221).

This contribution reports a software implementation of the luma adaptation optimization extension, integrated in the VTM TuC software. It also presents a showcase demonstrating the use of luma adaptation for display power saving purposes.

Together with the companion contribution JVET-AQ0051 summarizing the EOI SEI message extension and its associated software and showcase activities, this contribution provides supporting evidence of the technical maturity and practical applicability of the luma adaptation optimization feature for inclusion in VSEI version 5.

JVET-AQ0140 AHG9: A showcase for picture quality indication in EOI SEI message extension [B. Chen, J. Chen, Y. Ye (Alibaba), S. Yin, X. Ju, S. Wang (CityUHK)]

This contribution provides a showcase for the picture quality indication extension included in the Encoder Optimization Information (EOI) SEI message in the VSEI TuC. The contribution demonstrates how the picture quality indication can be used to support a standard-conformant generative video compression scheme, in which decoded pictures marked by the EOI SEI message with different quality levels are treated differently at the decoder side. This use case illustrates a practical application of the picture quality indicator and supports the adoption of the EOI extension into VSEI version 5.

JVET-AQ0156 AHG9: Proposal for a showcase playground [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)] [late]

This contribution presents a web-based playground application for SEI message parsing and showcase. The tool uses the WebCodecs API for hardware-accelerated video decoding in the browser and other Web APIs. There first showcase presented is the decoder picture hash which computes per-frame hashes to verify against the Decoded Picture Hash SEI messages (type 132) embedded in the bitstream. The application is implemented in TypeScript with React and is proposed as a new JVET project.

The WebCodecs API does not expose decoder internals.

Contribution uses a low level decoder rather than the full player. Have to dig into the player to access parameter sets.

The decoded picture hash SEI message was injected into the bitstream created with x.265.

It is suggested to talk to the WebAPI people at W3C.

It is suggested that it might be helpful to have example bitstreams with SEI messages.

WebCodecs may output different formats for YUV. In the proponent’s experiment, the HEVC decoder outputted NV12 ( Y plane + interleaved UV plane)

The JVET TuC repository group Gitlab could potentially host such software. However, this work is not directly related to the TuC, but to HEVC and existing SEI messages.

Github or gitlab can be used to host as well.

A follow-up presentation of this contribution was given in the JVET plenary.

It was asked whether this could also be used for SEI messages that are transporting more than metadata, but additional post processing of the decoder output, or pre-analysis to generate the parameters of the SEI.

It was asked whether the platform that is used is flexible enough to support all standards and all profiles that might be needed in a context of an SEI. More information on that would be used.

It is asserted that the demonstrated tool is useful, but it would need to be considered whether using showcases to make public advertisement of SEI messages (or in case of the proposed tool, a subset of SEI messages) is an effort that should be made by JVET. The current purpose of showcases is for JVET internal use making the potential application of an SEI message more clear.

Developing products out of SEI messages is not the business of JVET, and having a common platform for showcases is not required fo JVET’s purposes.

Experts are encouraged to consider JVET-AQ0156 and investigate whether the benefit of their SEI message could be demonstrated by that environment.

It was agreed to put the software into TuC branch for further consideration

JVET-AQ0180 AHG9: Showcase for infrared camera adaptation parameters SEI [V. Zakharchenko (Nokia)] [late]

Deferred to next meeting.

JVET-AQ0181 AHG9: Showcase for auxiliary sampling alignment information SEI message [V. Zakharchenko (Nokia)] [late]

Chaired by S. Deshpande 13 July at 1900.

This contribution provides a concise showcase of the auxiliary sampling alignment information (ASAI) SEI message. The showcase demonstrates how ASAI parameters enable a decoder or post-decoding process to determine the spatial correspondence between samples of an auxiliary picture and samples of its associated picture, including simple subsampling-grid alignment, cropping-offset signalling, and affine alignment. The results illustrate that the ASAI SEI message can support practical carriage and interpretation of auxiliary-picture alignment information for VSEI version 5.

Summary: This contribution is for information and illustrates decoder-side interpretation of ASAI SEI message parameters for auxiliary-picture alignment relative to an associated picture.

In the showcase, a python program determines the alignment parameters and inserts them in an ASAI SEI message. A player uses the contents of the SEI message to fuse the images for playback.

Noted.

## Non-SEI HLS aspects (0)

Section kept as template for future use.

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## General

### Plenary on Sunday 12 July, 0900-1015

The following topics were discussed in this JVET plenary:

- Scheduling for next days (joint meetings with MPEG WGs not clear yet)

  - Sunday HLS from 1030

  - Sessions Monday morning: HLS from 0900, review CfP proposal description template from 0900 (chaired by F. Bossen)

  - JVET plenary Monday 1330

  - With WG 2, AG 5, Q6/21, WP3/21 on CfP (Tuesday morning)

  - With WG 3 and WG 7 on SEI for Avatars, no review in HLS before

  - With WG 4 and WG 7 on GSC, need to finish all conclusion related to SEI topics until Tuesday (JEE 6.7 and 6.9, CTC, AHG mandates)

- Request/CD for WG 5 deliverables:

  - CDTR on film grain / content of software attachment

  - Request and CD for HEVC conformance 23008-5 (Iole)

  - Request and CD for HEVC software 23008-8 (Frank)

  - Request and CDAM for HEVC extensions: Multiview 444, SEI interface improvements (Ye-Kui and Alexis)

- SG21 deliverables:

  - H.Sup.MACVC (Jakob)

- WG 5 Ballot results 23888-3.2 m77164  CIB m77878

- Liaisons received

  - 3GPP TSG SA4-S4- 261360 LS on HEVC multi-layer profiles, chroma subsampling and bit depth combinations

  - ISO/IEC JTC1/SC29/WG1-N101487(SC29-N23492) LS on JPEG Trust

  - ISO/IEC JTC1/SC29/WG1-N101352 (SC29-N23183) LS on JPEG AI

  - None in MPEG DMS so far, but at this meeting JVET liaison is handled via SG21

- Review of JVET-AQ0043 (deferred for Monday), JVET-AQ0189 (deferred for Monday), , JVET-AQ0156

- Status of CfP and proposal description template (new version to be made available later on Sunday)

- Status of VSEIv5 progression (deferred for Monday)

### Plenary on Monday 13 July, 1330-1530

The following topics were discussed in this JVET plenary:

- Scheduling for next two days

  - Joint meetings with WG 4 and WG 7 on GSC: Daily 1600-1800 Crowne Plaza, Rm. Zürich

  - Joint meeting with WP3/21, Q6/21, WG 2, AG 2 on final CfP: Tue 14 July 0830-0900 in Popov

  - Joint meeting with WG 3 and WG 7 on avatar: Tue. 14 July 1530-1600 Crowne Plaza, Rm. Zürich

  - Final review of CfP and proposal description template, and revisits today from 1600 (chaired by M. Wien)

  - Not clear yet whether main track or plenary will be needed on Tuesday after joint meeting

  - Closing plenary Wednesday 0900-1400

  - For HLS, see calendar

- Liaisons relevant for JVET (outgoing liaison review planned for Tuesday 0915)

  - 3GPP TSG SA4-S4- 261360 LS on HEVC multi-layer profiles, chroma subsampling and bit depth combinations, also addressed to WG 5 as m77997 (YKW, AMT, GT to work on drafting response)

  - ISO/IEC JTC1/SC29/WG1-N101352 (SC29-N23183) LS on JPEG AI, also addressed to WG 5 as m78006 (EA to draft response)

  - ISO/IEC JTC1/SC29/WG1-N101487(SC29-N23492) LS on JPEG Trust seems irrelevant for JVET – there is also one on media authenticity, m78009 addressed to WG 2

  - A liaison from SMPTE m77998 addressed to WG 5 but not sent to SG21 does not require a response

  - Another one from JPEG is m78008 on radiance fields addressed to WG 2/4/5/7 seems related to GS activities – to be discussed in joint meetings

- Planning of outputs and AHGs

- Review of JVET-AQ0189 (see notes in 4.13)

- Status of VSEIv5 progression was discussed:

  - The following table was presented by J. Boyce that had been reviewed in the HLS track based on the discussion and assessment of the SEI messages in section 6.3, and subsequent review of some of the messages in terms of maturity based on track review of input contributions:

An updated version of this table was later made available in JVET-AQ0240.

JVET-AQ0240 Status of SEI messages in the VSEI TuC [J. Boyce]

It was agreed to include all SEI messages and extension in JVET-AQ2006 which are assessed to be mature enough an well understood in usefulness by the criteria that had been defined, as expressed of all fields of the corresponding row marked as green. These elements should be removed in JVET-AQ2032. At this moment, a differential text is defined as WD, which can better be managed than an integrated version.

It is emphasized by software coordinators that the current practice of doing frequent software changes in case of relatively early study on a large number imposes a large amount of work. It would be desirable to have a discussion on practices for implementing SEI messages in software, and potentially having a dedicated software coordinator would be necessary, also to avoid delays in the implementation. To be discussed in a future meeting.

### Plenary on Wednesday 15 July 0900-1400

The following topics were discussed in this JVET plenary:

- Remaining revisits were conducted

- Report of JAHG JVET-AQ2019 and presentation of JEE6.7/6.9 summary report JVET-AQ0022

- Establishment of AHGs – see section 9

- Establishment of Joint AHG on GSC, and announcement of two F2F meetings Wed./Thu.

- Review of JVET-AQ0230

- List of HLS adoptions was approved

- Approval of output docs (including review as needed) – see section 10

- Review of WG 5 meeting recommendations – see annex C

- Software timelines were defined – see section 8.1

- It was agreed to establish a new AHG6 on non-reference software assets – see section 9

- Future planning, a.o.b.

- The meeting was closed at 1400 CEST.

## MPEG information sharing meetings (update)

Reports about JVET work status were given during information sharing sessions with other WGs and AGs of the MPEG community, held on Monday 13 July 0900–1230 (report about JVET activities was given), Wednesday 15 July 0900–1000 (no report about JVET activities), and Friday 17 July 1400–1600 (report about JVET activities was given).

The status and plans for the work in the MPEG WGs and AGs was reviewed at these information sharing sessions.

## Joint meetings

### Joint sessions on Gaussian splat coding: MPEG WG 4 / Video, MPEG WG 5 / JVET, MPEG WG 7 3D Graphics, and VCEG (ITU-T Q6/21)

##### Joint meeting session 1 (Mon. 13 July 1600-1800)

The first joint session was chaired by Lu Yu (MPEG WG 4 Convenor), Jens-Rainer Ohm (JVET Chair), , and M. Preda (MPEG WG 7 Convenor). The following notes were taken by JRO. The following topics were discussed:

- M 77844 (JVET-AQ0235) was presented (see notes in 6.5.4)

- M 78721 (JVET-AQ0238) was presented

JVET-AQ0238 AHG9: On software availability and CTC results [B. Kroon (Philips), G. Teniou (Tencent), J. Boyce (Nokia), L. Kerofsky (Qualcomm)] [late]

This contribution proposes rules for software available and CTC results of JEE and TuC-related proposals on GS related SEI messages. The establishment of such rules had been requested to be suggested during the joint AHG meeting on 12 July.

The following modified text based on further editing the proposal in JVET-AQ0238v2 was agreed:

For EE inclusion, a contribution should normally provide:

- a clear description of the proposed method;

- identification of the affected part of VSEI: syntax, semantics, generation of SEI parameters and input to video encoder, reconstruction of GS from video decoder output;

- software (source code);

- CTC results (when the proposal affects coding efficiency), by giving a complete reporting as per template, including objective quality, complexity per encoder/decoder runtime;

- enough information to reproduce the results, including software version, configurations, command lines, and test material;

- an indication of whether and how the result can be cross-checked.

For TuC inclusion of proposals that do not impact coding performance under CTC, A contribution should provide:

- complete proposed text of both syntax and semantics

- commitment to collaborate on software integration with the software coordinator, potentially with negotiated editing period on issues found in the merge request;

- software integration is handled by the software coordinator, with negotiated editing period, with support of proponents if multiple proposals are interacting;

For TuC inclusion of proposals that impact coding performance under CTC, the requirements should be stronger. A contribution should provide:

- complete proposed text, when syntax, semantics, decoding, conformance, or reconstruction are affected;

- software implemented in the VSEI reference software or an agreed branch by issuing a merge request;

- commitment to collaborate on software integration with the software coordinator, potentially with negotiated editing period on issues found in the merge request;

- CTC results on top of the applicable anchor (as per CTC or as defined in the JEE where the proposal was investigated);

- at least one successful cross-check confirming the reported results;

- a clear statement whether the proposal is to be enabled in CTC, integrated but disabled, included only for EE study, or included in the TuC.

As a general rule, positive discussion should not be sufficient for TuC inclusion if the required software, results, or cross-check are missing. In such cases, the outcome should normally be “further study,” with the missing evidence explicitly identified.

For training-dependent or implicit-representation proposals, the required software should include the complete pipeline needed to reproduce the coded representation and results, including training or model-generation steps when these affect the result.

For software-only or CTC/script changes, the contribution should show that the change is reproducible and should clearly state whether previous and new results remain comparable.

A possible compact rule could be:

A VSEI contribution that affects syntax, decoding, reconstruction, conformance, CTC results, or reference software operation should not be included in the TuC unless the corresponding text, software, CTC results, and appropriate cross-check evidence are available. For JEE inclusion, the same information is expected when applicable, but the group may allow earlier study inclusion if the missing items are explicitly identified.

Elements included in the TuC at prior meetings will be removed if the rules above are not followed, i.e. for elements included in JVET-AQ2032 this will apply in October.

- Review of CTC N 826 was conducted, and some wording improvement on definition of anchors, and results to be delivered by proponents (to make comparison against anchors useful) were made

- It was reported that proposals relating to V-PCC are mostly comparing against the V-PCC anchor, and the proposals relating to G-PCC are mostly comparing against the G-PCC anchor. Also here, the problem exists that V-PCC may perform better for some cases, and G-PCC for other.

- The question had been raised in the JAHG meeting whether it would be useful to also define specific anchor(s) for SEI-based proposals.

The following proposal was agreed for anchors in the exploration:

- JVET-AP0100

- V-PCC GS (as per definition from WG 7 Amd1 development)

- G-PCC GS (as per definition from WG 7 Amd1 development)

All anchors would be including adoptions of the current meeting (From joint activity, or WG 7 in case of V-PCC and G-PCC).

##### Joint meeting session 2 (Tue. 14 July 1600-1800)

Agenda for Tuesday:

- Approve JAHG recommendations

- Review JEE6.1 for potential new test sequences in CTC

- Review mandates of next JAHG

- Remaining 6.7

- Review JEEs 6.3, 6.4, 6.5, 6.6, 6.2

- CfP discussion planned on Wednesday with AG 5

An initial version of the JAHG report M77135 (elements collected and available in git) was presented

Regarding JAHG recommendations, aspects on general principles have already been conducted in the joint meeting on Monday (CTC, rules of implementation, reporting, etc.), or will be further conducted in the current or subsequent joint meetings.

Regarding recommendations made by the AHG that relate to SEI contributions, better coordination should be implemented how this is reaching the SEI experts in JVET.

All recommendations related to proposals on SEI-based technology were agreed in the joint meeting, except for aspects of JVET-AQ0167. This relates to the integer implementation of operations  in the reconstruction pipeline that are currently implemented in floating point representation, which however comes with a loss of 0.5% in the results presented in the proposal.

This does not have impact on the current syntax and semantics of GSI. As long as the reconstruction process is not defined to be normative, it might even be possible to do reconstruction either by integer and floating point processing. It was agreed to adopt this to the software (also in SEI anchor).

Another revisit was found necessary on JVET-AQ0168. This proposes a conversion of quantization and transform parameters to integer representation in the syntax. The loss reported for JVET-AQ0167 also includes already this modification (separate impact of the two changes is unknown).

It was agreed to include the changes of JVET-AQ0168 in JVET-AQ2032 (next TuC), and also implement in software.

JEE6.1:

Presentation of following documents:

m77290 A-3DGS Dynamic Representation Sequences Generated using FreeTimeGS Kelei Liu, Lu Yu, Yiyi Liao

Presented on Tuesday 14 July

Investigation on conversion between I-3DGS and A-3DGS for current test material, generated with FreeTimeGS. The latter produces more compact Gaussian splats (total number of Gaussians below is for 32 frames each). In Manwithfruit, some artifacts are visible at object boundaries.

m78033 360-degree multiview basketball dataset for Gaussian Splat Coding [Chao Wang, Jiayu Yang, Song Xu, Qi Wang, Ronggang Wang]

Presented on Tuesday

Sports scene from stadium, captured by 36 fixed cameras, 700 frames at 25 fps. Metadata such as camera parameters, as well as the original camera captures would also be available.

For consideration in CfP (not in CTC at this meeting)

It was asked to provide usage/licensing conditions in an update of the contribution

Joint AHG mandates were discussed by the end of the meeting of Tuesday. It was agreed to keep the existing mandates 1-12 unchanged. A mandate 13 was agreed to be added to “coordinate the software integration of the codecs under 	investigation.”

Furthermore, the list of output documents (all to be made available as WG 5 outputs in the dms.mpeg.expert) was agreed as follows, including some minor updates of titles of JEEs (no new EE was introduced), as well as editing periods.

| No. | Title | In Charge | TBP | Available |
| --- | --- | --- | --- | --- |
|  | Explorations |  |  |  |
| 420 | Common test conditions for Gaussian splat coding | Bart Kroon | N | 2026-08-14 |
| 421 | Draft CfP of Gaussian splat coding | Ralf Schaefer | N | 2026-07-31 |
| 422 | Call for content for Gaussian splat coding | Ralf Schaefer | Y | 2026-07-31 |
| 423 | FAQ on Gaussian splat coding | Euee Jang | Y | 2026-09-11 |
| 424 | JEE 6.1 on data preparation | Patrice Rondao Alface | N | 2026-08-14 |
| 425 | JEE 6.2 on anchor generation | Gustavo Sandri | N | 2026-08-14 |
| 426 | JEE 6.3 on new coding technologies | Gun Bang | N | 2026-08-14 |
| 427 | JEE 6.4 on collecting requirements and use cases for Gaussian splat coding | Patrice Rondao Alface | N | 2026-08-14 |
| 428 | JEE 6.5 on 3DGS software | Julien Ricard | N | 2026-08-14 |
| 429 | JEE 6.6 on coordinating the 1F-geo Track | Hyejung Hur | N | 2026-08-14 |
| 430 | JEE 6.7 on video-based coding technologies | Yiyi Liao | N | 2026-08-14 |
| 431 | JEE 6.8 on Gaussian Splat Coding evaluation and CTC | Anthony Trioux | N | 2026-08-14 |
| 432 | JEE 6.9 on evaluation of encapsulation methods for Gaussian Splats | Joel Jung | N | 2026-08-14 |

Two more joint sessions were held on Wednesday 15 July 1600 and Thursday 16 July 1600, which from the perspective of JVET, after the closing of the JVET meeting, were already activities in the context of the Joint AHG. On behalf of JVET, these sessions were co-chaired by W. Husak. One major activity during these session was review of remaining documents submitted in the context of JEEs 6.1-6.6, as well as a one remaining document of JEE 6.7 (none of the reviewed documents related to study of SEI messages for Gaussian splats). The results of discussions during these sessions can be found in the JAHG report m77135.

### Joint session 0830-0900 Tuesday 14 July on Joint Call for Proposals: MPEG WG 2 / Requirements, MPEG WG 5 / JVET, MPEG AG 5 Visual Quality Assessment, and ITU-T WP3/21)

This joint session was chaired by Jens-Rainer Ohm (JVET Chair and WG 5 Convenor), Justin Ridge WP3/21 Co-chair), Mathias Wien (AG 5 Convenor), and Igor Curcio (WG 2 Convenor).

This section is based on notes taken by JRO and Gary J. Sullivan (Q6/21 Rapporteur).

J. Ridge opened the meeting, mentioning that the meeting was requested by WP3/21, considering that the Joint CfP was planned to be issued as document by WP3/21.

M. Wien presented the latest draft of the CfP (JVET-AP2026v6 available as attachment in JVET-AQ0201).

It was discussed whether the selection of test model by October 2028 (as written in the timeline) would be not sufficiently ambitious, but as it is expressed as “at latest” and the timeline being tentative, that should not be a problem and not be changed.

It was discussed whether the statement about software availability would be sufficient, but as also the proposal description template contains a section where proponents need to unveil under which conditions their software would be available, and it is also expressed that it needs to be compatible with common patent policy and licensing rules imposed by ITU-T and ISO/IEC, this is asserted to be sufficient.

No issues were found in the presented document. The version presented was requested to be prepared as output document JVET-AQ2021 and to be delivered for final approval as WG 5 and WP3/21 documents.

The Joint CfP was to be approved in WP3/21 on Thursday 16 July.

### Joint session 1535-1605 Tuesday 14 July on SEI messages for avatar coding: MPEG WG 5 / JVET, MPEG WG 3 Systems and MPEG WG 7 3D Graphics

This session was chaired by Jens-Rainer Ohm (JVET Chair and WG 5 Convenor), Marius Preda (WG 7 Convenor), and Thomas Stockhammer (on behalf of WG 3).

See notes under section 6.6.

## BoGs (0)

Section kept as template for future use.

## Liaison communications (3 incoming, 2 replies generated)

ISO/IEC JTC1/SC29/WG1-N101352 (SC29-N23183) LS on JPEG AI, also addressed to WG 5 as m78006

A draft response was presented by E. Alshina on Tuesday 14 July 0920-1000.

3GPP TSG SA4-S4-261360 LS on HEVC multi-layer profiles, chroma subsampling and bit depth combinations, also addressed to WG 5 as m77997

A draft response was presented by Y.-K. Wang on Tuesday 14 July 1000-1020.

Various further edits were made during the presentations of the drafts. The new versions of the documents were requested to be sent to JRO and GJS for further processing and forwarding for approval by SG21.

- The drafts were forwarded for further processing in Q6/21, and issued as follows:

Liaison to JPEG on JPEG AI and explorations on video coding, included as item 1 in SG21 TD354/WP3

Liaison to 3GPP SA4 on HEVC multi-layer profiles, chroma subsampling and bit depth combinations, included as item 2 in SG21 TD354/WP3

Another liaison statement m77998 was addressed to WG 5 via SC 29 by SMPTE on SEI messages (no corresponding communication via SG21). It was informally inspected, and concluded that it does not need a response.

(kept for future use) The liaison response was also presented by XXXX in the MPEG AG 3 Communication meeting on Thursday XXXX during 1500 – 1700.

# Project planning

## Software timeline

The NNVC 18.0 codebase software was planned to be available 2 weeks after the meeting (31 July). Additional integration of various SADL aspects and harmonization with VTM may be done as appropriate in later versions.

Additional versions on top of VTM 24.0 may be released as appropriate.

An initial update on top of HM18.0 should be generated for submission of the ISO/IEC CD 23008-5 3rd within 2 weeks (July 31), integrating at least the elements for supporting new multiview profiles..

Further updates on top of HM18.0 and JM19.1 software were planned to be released later (integration and updates of SEI messages included in JVET-AN1018 and JVET-AN1017, and merging them with the new HEVC multiview software). Additional versions may be released as appropriate, also investigating the integration of full SHVC support in HM.

As a general rule in software development, a person who is executing a merge shall not be from the same company as the person who submitted that merge request.

## Core experiment and exploration experiment planning

The exploration experiment on improved compression compared to VVC (EE2) had been discontinued by the 42nd meeting. At the 43rd meeting, the exploration experiment on neural network-based video coding (EE1) was discontinued as well.

Joint EEs on Gaussian splat technology are planned to be conducted within the Joint AHG. The proposals that are planned to be investigated are listed in the Joint AHG report JVET-AQ2019, as confirmed during joint meetings (see section 7.3.1). Technology related to SEI messages will be investigated in JEE 6.7 and JEE 6.9, based on common test conditions JVET-AQ2028. The JAHG was mandated to finalize the CTC document and the nine JEE descriptions during a telco on August 11, to be made available with an editing period until August 15 (description of JEEs which have relation to the investigation of performance and improvements of SEI messages will also be made available in JVET-AQ2040).

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- Normative contributions (relating to changes in bitstream/decoder) shall include draft specification text

- Proposals shall contain all details relevant for understanding and be self-contained. In cases where the document is a follow-up of a previous contribution, the overall concept and the novelties should be highlighted at minimum

- Coding tool and encoder optimization proposals shall contain Excel sheets that allow assessment on a per-sequence basis

- Algorithm description text is strongly encouraged for non-normative contributions that are intended to be included in model description documents (VTM, ECM, etc.), and that is required for inclusion in TR drafts.

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions (not exercised currently)

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology. Withdrawing parts of experiments that were intended to show the individual benefits of a tool or parts of a tool is strongly discouraged. Combination tests may not be considered in such cases. Any changes made to individual tools in a combination shall be documented.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). In cases where combinations of different elements are planned to be tested, mutual cross-checking of the individual elements by other parties of the combination is discouraged. The combination must be cross-checked by an independent party. The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-AL2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g., not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow was previously described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

However, it was noted that the link doesn’t seem to exist anymore.

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation was requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

Chairs of AHGs had been asked to send draft mandates to JRO before 1800 on 14 July, preferably copy from the table below and sending with changemarks or yellow highlight of changes.

Review of AHG plans was conducted during the plenary on Wednesday 15 July 2026 at 0905–1015.

| Title and Email Reflector | Chairs | Interim mtg. |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents. - Produce an update of the overview on IT systems JVET-AQ1012. | J.-R. Ohm, M. Wien (co-chairs), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AQ1006). - Collect reports of errata for VVC, VSEI, HEVC, AVC, CICP, and the published related technical reports. - Coordinate with AHG3 to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Prepare the draft of the next edition of HEVC reference software JVET-AQ1017. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages (for both VTM and HM), and SEI messages in TuC (the latter in a separate branch of VTM). - Discuss and make recommendations on the software development process. - Perform comparative tests of test model behaviour using common test conditions, including HDR, high bit depth and high bit rate. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Prepare drafts of merged and updated CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material, in coordination with other AHGs, as appropriate. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct expert viewing for purposes of subjective quality evaluation. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing, as well as studying objective metrics in that context. - Coordinate with AHG18 on investigating visual impact of data losses. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AQ1008, and further develop related conformance bitstreams. - Prepare the draft of the next edition of HEVC conformance by integrating the relevant bitstreams from JVET-AQ1008. - Coordinate with AHG3 on implementation of the new HEVC multiview profiles. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice-chairs) | N |
| Non-reference software assets (AHG6) (jvet@lists.rwth-aachen.de) - Collect a list of software assets available in JVET software repositories, which had been used during standards development or exploration efforts, but have not been made available as parts of reference software. - Propose actions to make such assets better visible and accessible for JVET members or to the public. | Y. Ye (chair), F. Bossen, R. Chernyak, P. de Lagrange, K. Sühring (vice-chairs) | N |
| Tool assessment (AHG7) (jvet@lists.rwth-aachen.de) - Investigate methodology of tool assessment, such as the aspects of memory access and bandwidth, number of maximum processing cycles, block decoding dependencies, number of context coded bins, pipeline and parallelization. - Study JVET-AO2040 and suggest improvements. - Prepare reporting of tool assessment results as applicable. - Develop methodology of more reliable runtime measurement. - Collaborate with AHG16 on encoding complexity assessment. | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice-chairs) | Y (tel., 2 weeks notice) |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Study JVET-AQ2006 and JVET-AQ2032, including study of SEI messages with different options (when present) and propose improvements. - Maintain the table of the summary of VSEI TuC status that is in an attachment of JVET-AQ2032, and generate a similar table for JVET-AQ2006. - Discuss and refine criteria for the assessment of SEI messages based on JVET-AQ0240. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages. - Study SEI messages specified in HEVC and AVC for potential use in the VVC context and SEI messages in VSEI for potential use in the HEVC or AVC context. - Study the alignment of the same SEI messages in different standards. - Coordinate with AHG3 for software support of SEI messages for JM, HM, and VTM. - Coordinate with the joint AHG on Gaussian splat coding about the design and software implementation of the Gaussian splat coding related SEI messages in the TuC. | J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM. - Study optimized encoding and suitable test settings for noisy materials, such as sequences containing film grain. - Study optimized encoding and tool combinations for low latency and for low complexity. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, higher objective quality, or encoding with improved complexity/performance tradeoff, and coordinate such efforts with AHG3, AHG12, and AHG17. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice-chairs) | N |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools, architectures and content adaptation with NN parameters overfitting. - Study potential improvements of the NNVC CTC document JVET-AP2016. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies and identify suitable material for testing and training. Promote the call for training materials, distribute it, and actively communicate with content owners. - Discuss and propose improved metrics to perform complexity analysis of NN architectures in preparation of the upcoming CfP, and develop complexity reductions of candidate technology. - Investigate device interoperability of NN-based methods on various platforms in coordination with AhG14. - Coordinate with other groups, including SC 29/AG 5 on the evaluation and assessment of visual quality, and with AHG12 on the interaction with ECM coding tools. - Coordinate with AHG14 on items related to NNVC software development, in particular, evaluate the interface in the NNVC for end-to-end optimized AI coded reference pictures. | E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien (vice-chairs) | Y (tel., 2 weeks notice), first on July 30 |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Coordinate with AHG7 to study the performance and complexity tradeoff of video coding tools. - Support the ECM software coordinators in maintenance of the software, associated configuration files and documentation. - Identify any mismatches between the ECM20 software and the algorithm description JVET-AP2025, suggest updates and cleanups to the software as appropriate. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs) | N |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Propose refinements to the draft of the TR 2nd ed. JVET-AQ2020, and investigate for which additional elements described in the TR software might be attached. - Study alternative film grain models and their associated documentation. - In consultation with AHG4, study and define content characteristics and test conditions that are desirable for the study and testing of film grain technologies, and perform an assessment of newly available test materials in that regard. - Investigate metrics for measuring film grain fidelity in itself, or as present in a video. - Discuss the potential need for film grain conformance guidelines. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Study Film grain region characteristics information SEI in JVET-AQ2006 and propose improvements as necessary - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files in coordination with AHG3. | W. Husak, P. de Lagrange (co-chairs), S. Deshpande, A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-18.0 software version (and potential updates), based on updated VTM with adopted contributions and hybrid framework, and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AP2016. Study the impact of the addition of new dataset on the already integrated models. - Continue to bridge the gap between NNVC and most recent VTM as necessary. - Continue to develop missing functionalities for hybrid end-to-end framework exploration. - Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) and Very Low Operation Point (VLOP) configurations. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AQ2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. | F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice-chairs) | Y (tel., 2 weeks notice), first on July 30 |
| Gaming content compression (AHG15) (jvet@lists.rwth-aachen.de) - Identify gaming content application scenarios and their requirements for codec operation. - Identify and characterize required types of content; solicit contributions, collect, and make a variety of gaming content available, in coordination with AHG4 and AG 5. - Produce VTM and ECM anchor encodings according to CTC JVET-AO2027, and provide test results at the next meeting. - Develop and maintain interfaces for supporting use cases of camera parameters and depth maps in gaming applications, including mechanisms for efficient transporting these elements in the coded video bitstream. - Develop and maintain software for estimation of depth maps for gaming sequences, and evaluate the effect of the estimated maps against the original depth maps on coding efficiency where they are available using established accuracy metrics - Evaluate JVET test models (such as ECM, VTM, NNVC, etc.) under the proposed test conditions. - Investigate possibilities to enhance compression capability for gaming content. - Investigate the possibility to estimate depth maps and camera parameters for those gaming sequences where they are not available. - Study conversion of depth maps using integer representation, and identifying efficient bit-depth resolution of depth maps to support identified use-cases that will be an input to compression. - Solicit contributions from industry on typical bitrate/quality/resolution used for gaming content compression. | C. Lehmann, S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice-chairs) | N |
| Hardware implementation complexity (AHG16) (jvet@lists.rwth-aachen.de) - Investigate hardware encoding complexity constraints in typical applications (e.g., mobile devices and hardware transcoding), and identify challenges and evaluate coding tools from hardware encoding implementation perspectives. - Solicit and develop hardware encoding complexity measurements (e.g., maximum number of RDO decisions per CU / CTU), and use them to analyse the performance of test models (e.g., VTM) and coding tools. - Design, develop and maintain a hardware-encoding-mimicking simulation framework on top of test models, e.g., constraining RDO number per CU, constraining coding tree decision complexity (e.g., maximum tree depth, per-node split modes), low-complexity CU/TU-level bit estimation function, etc. - Collaborate with AHG7 on aspects of hardware encoding and decoding complexity perspectives. | Y. Zhao, I. Moccagatta, K. Naser (co-chairs), H. Huang, T. Ikai, L. Li, X. Li, N. Song, G. Verba (vice-chairs) | Y (tel., 2 weeks notice) |
| Preparation of Call for Proposals (AHG17) (jvet@lists.rwth-aachen.de) - Study the draft of the template for proposal description documents in JVET-AQ2022, and suggest updates as appropriate. - Study the complexity reporting template in JVET-AQ2022, suggest improvements, and prepare examples of the encoder complexity analysis for the case of VTM anchors in coordination with AHG7. - Coordinate with AG 5 on preparing logistics and organization of the visual assessment in the context of the CfP. | J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice-chairs) | Y (tel., 2 weeks notice) |
| Ultra-low latency and packet loss resilience (AHG18) (jvet@lists.rwth-aachen.de) - Study JVET-AN2039 common test conditions and propose improvements as appropriate. - Investigate creation of practical simulation software, including network transmission aspects, including consideration of network functionality that can aid timely video-coding based error correction, and conduct performance evaluation. - Identify potential requirements and feasibility of standard based technologies to support ultra-low delay requirements, including packet loss resilient decoding. - Investigate packet loss resilient technologies beyond VVC supporting ultra-low delay coding for interactive and live broadcasting scenarios. - Coordinate with AHG4 and AHG17 on investigating visual impact of data losses and appropriate evaluation procedure development. | S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |

As a follow-up of discussions conducted in previous meetings, it had been confirmed during the opening plenary on 7 July that it would be highly beneficial to make software that had been developed in various activities (AHGs, explorations conducted by JVET or previous joint teams, processing related to SEI messages, etc.) better visible, and potentially publicly available. It was suggested that a first step could be issuing a document which would summarize the existence of such JVET software assets, and indicate where these can be found. To take further action, it was agreed during the closing plenary on 15 July to establish a new AHG 6 on the topic of non-reference software assets, rather than making it an additional mandate of AHG3 (which is responsible for development of reference software).

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N 418) in order to make it easy to reference.

A Joint AHG group was installed with ISO/IEC JTC 1/‌SC 29/‌WG 4 and ISO/IEC JTC 1/‌SC 29/‌WG 7 as follows (discussed and approved 1050-1100 on Wednesday 15 July). The last mandate was added, other mandates are identical to those of the last meeting cycle. Dates of meetings require updates, and other WGs are free in nominating their co-chairs until the end of their respective meetings.

| Gaussian Splat Coding (mpeg-gsc@lists.aau.at) - Discuss refinements of use cases and requirements for Gaussian splat coding - Improve and maintain datasets for Gaussian splat coding - Discuss possible refinements of common test conditions for Gaussian splat coding - Prepare anchors for Gaussian splat coding - Investigate new Gaussian splat representation and associated coding technologies - Consolidate the software tools for Gaussian splat coding - Coordinate the activity on Lightweight GS (1F) - Collaborate with AG 5 on preparation of subjective evaluation - Coordinate development of GS coding technology and its standardization across JVET, WG 4, and WG 7 - Study encapsulation/high-level syntax technologies including those based on V-PCC, V3C, and VSEI. - Study GS technology that allows cross-encapsulation compatibility and definition of related conformance - Coordination of the preparation of the Call for proposal - Coordinate the software integration of the codecs under investigation. | Y. Liao, G. Bang (co-chairs on behalf of WG 4), J. Jung, W. Husak (co-chairs on behalf of JVET), A. Zaghetto, H. Hur (co-chairs on behalf of WG 7) | Y (tel., UTC) Aug. 11 1400–1600 Aug. 25 2100–2300 (later cancelled) Sep. 15 0500–0800 Oct. 6 1400–1700 Physical / remote access meeting on 17 and 18 October, full day each |
| --- | --- | --- |

The definition of the Joint AHG was issued as WG 5 N 419.

It is noted that only one instance of the joint AHG is routinely installed over all MPEG WGs, and at the current meeting, JVET WG 5 was responsible, as well as for managing all input and output documents of the joint activity.

JVET members are invited to subscribe to the reflector of this AHG via https://lists.aau.at/mailman/listinfo/mpeg-gsc.

GS content is stored at https://content.mpeg.expert/data/Explorations/GSC/CTC/

For accessing software, test materials and anchors, JVET members should contact the AHG chairs. In the next meeting, WG 7 will manage input and output documents of the joint activity. Documents relevant for usage of SEI messages in Gaussian splat representation should be submitted both as WG 7 documents and as JVET documents, such that JVET members who are participating via ITU also can get access. Documents not relevant for usage of SEI messages shall only be submitted as WG 7 documents.

For definition of joint exploration experiment on topics relevant for JVET, see section 8.2 of this report. CTC is available as JVET-AP2028 (duplicating N 420 of WG 5).

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to GJS and JRO for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N 418, as noted in section 9.

A list of JVET-only output documents from the current meeting (including links to jvet-experts.org) was issued as a WG 5 document WG 5 N 417.

JVET-AQ1000 Meeting Report of the 43rd JVET Meeting [J.-R. Ohm] [WG 5 N 408] (2026-08-14)

Initial versions of the meeting notes (d0 … d8) were made available on a daily basis during the meeting.

Remains valid – not updated: JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

Update seems useful after software integration of multiview  and SHVC (integrate content from JCTVC-V1007 and JCT3V-K1003?)

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne, K. Sharman, R. Sjöberg, G. J. Sullivan] [WG 5 N 103]

Primary editor: C. Rosewarne

Remains valid – not updated: JVET-AH1003 Coding-independent code points for video signal type identification (Draft 3) [G. J. Sullivan, A. Tourapis]

Primary editor: G. J. Sullivan.

JVET-AQ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Y.-K. Wang, J. Boyce, B. Bross, I. Moccagatta, C. Rosewarne, K. Sühring, G. J. Sullivan] (2026-08-14)

Primary editor: Y.-K. Wang.

Some issues found in latest editions of VSEI/VVC/HEVC/AVC – see notes under following documents (some aspects on HEVC may be included in JVET-AQ1006 instead)

- JVET-AQ0061 AHG9: Errata report on VSEI and HEVC

- Item 1 of JVET-AQ0075 AHG9: On video full range indication

- JVET-AQ0089 AHG2/AHG9: Errata on general SEI signalling mechanism for VVC, VSEI, HEVC, and AVC

- JVET-AQ0093 AHG2/AHG9: On the FPA and CTI SEI messages

- JVET-AQ0114 AHG9: On DSCS SEI message in VSEI v4

Remains valid – not updated: JVET-AP1005 Future CICP extensions (Draft 3) [J. Boyce, E. Thomas, A. Tourapis]

Primary editor: J. Boyce.

JVET-AQ1006 HEVC additional profiles (Draft 2) [A. Tourapis, Y.-K. Wang, B. Bross, G. J. Sullivan] (2026-07-31)

Primary editor: A. Tourapis.

Contains new 4:4:4 10/12 profiles from JVET-AP0242 (were already in draft 1) plus corrections/clarifications on VSEI interfaces.

The content of JVET-AQ1006 is planned to be submitted for CDAM ballot as WG 5 N 411. A Request WG 5 N 410 was reviewed and approved on Wednesday 15 July 1150.

Remains valid – not updated: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman (editors)] [WG 11 M 32658]

This document may require merging with JVET-T2013 in the future. JCTVC-P1006 only defines 10-bit sequences for the cases on 4:4:4 and 4:2:2 chroma sampling, and 16-bit sequences for higher bit depth, no 12 bit cases. All sequences specified in this document are available in the JVET content server. It is also noted that JVET-AA1100 could be removed once such a merge is done.

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

Could be removed when SHVC functionality would be integrated into HM. Needs to be checked whether encoder parameters and settings can be configured similarly.

JVET-AQ1008 Conformance testing for HEVC multiview extended and monochrome profiles (Draft 3) [I. Moccagatta, T. Fu, S. Paluri, A. Tourapis] (2026-07-31)

Primary editor: I. Moccagatta

Include streams from JVET-AQ0186

This document includes new streams for all multiview profiles (including 444 that are in current draft JVET-AQ1006).

The content of JVET-AQ1008 (as far as already defined multiview profiles are concerned) is planned to be integrated into ISO/IEC 23008-8 (together with previous Amd.1) and submitted for CD ballot as WG 5 N 415. A Request WG 5 N 414 was reviewed and approved on Wednesday 15 July 1205.

Progression into H.265.1 planned for April 2027

It is asserted that 444 multiview profile streams might be added in case that they would be included in a new edition/version of HEVC by April 2027.

Remains valid – not updated: JVET-AC1009 Common test conditions for SHVC [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

Remains valid – not updated: JVET-AJ1011 White paper on HEVC [B. Bross, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang] [AG 3 N 174]

JVET-AQ1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien] (2026-10-10)

New version to potentially include access changes in git (after implementation), update of bug tracking system description (if available), enforcing rules for not recording zoom meetings, completing list of CTC, etc.

Remains valid – not updated: JCT3V-G1003 3D-AVC Test Model 9 [D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] [WG 11 N 14239]

Remains valid – not updated: JCT3V-K1003 Test Model 11 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] [WG 11 N 15141]

Remains valid – not updated: JVET-AE1013 Common test conditions of 3DV experiments [K. Sühring, M. Wien]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

Remains valid – not updated: JVET-AC1015 Common test conditions for SCM-based screen content coding [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AP1016 AVC with extensions and corrections (Draft 6) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang]

This had been submitted to ITU consent on H.264 16th ed. It is noted that this may not be identical with the final version published by ITU-T.

JVET-AQ1017 Updates of HEVC reference software (draft 1) [K. Sühring, F. Bossen, A. Tourapis] (2026-07-31)

Primary editor: K. Sühring

Integration of new software developed in context of new multiview profiles, and support for SEI messages in VSEIv4.

It is planned to integrate new software elements developed in the context of multiview profiles into HM, and convert it into a new edition of ISO/IEC 23008-5 (together with previous Amd.1, where SCC will still coexist as separate software package),  to be submitted for CD ballot as WG 5 N 413. A Request WG 5 N 412 was reviewed and approved on Wednesday 15 July 1205.

Progression into H.265.2 planned for April 2027

It is asserted that enabling signalling 444 multiview profiles might be added in case that they would be included in a new edition/version of HEVC by April 2027. Also, an integration of SHVC support into HM (as per JVET-AQ0187) would be desirable for DIS or FDIS stage, to remove the separate SHM software package from the reference software specification

Remains valid – not updated: JVET-AN1018 HEVC with extensions and corrections (Draft 3) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis]

Primary editor: Y.-K. Wang.

This had been submitted to ITU consent on H.265 11th ed. It is noted that this may not be identical with the final version published by ITU-T.

Remains valid – not updated: JVET-AN1019 VSEI with extensions and corrections (Draft 1) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang]

Primary editor: J. Boyce.

This had been submitted to ITU consent on H.274 4th ed. It is noted that this may not be identical with the final version published by ITU-T.

No output: JVET-Axx1020 through JVET-Axx1099

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman]

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet. See note under JCTVC-P1006 above.

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AN2001 VVC with extensions and corrections (Draft 1) [Y.-K. Wang, B. Bross, M. M. Hannuksela, G. J. Sullivan]

Primary editor: Y.-K. Wang.

This had been submitted to ITU consent on H.266 4th ed. It is noted that this may not be identical with the final version published by ITU-T

Remains valid – not updated: JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23) [Y. Ye, A. Browne, S. Kim]

Primary editor: Y. Ye.

New elements from notes elsewhere in this report (kept for future use):

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

Remains valid – not updated: JVET-AM2005 Additions and corrections for VVC version 4 (Draft 13) [G. J. Sullivan, B. Bross, M. M. Hannuksela, Y.-K. Wang] (2025-08-01)

Primary editor: G. J. Sullivan.

JVET-AQ2006 Additional SEI messages for VSEI version 5 (Draft 1) [Y.-K. Wang, J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, G. J. Sullivan, H. Tan, A. Tourapis] (2026-08-21)

Primary editor: Y.-K. Wang.

It is noted that the availability date above is for the final version, and it is expected to have an initial version available by 2026-07-31.

Included elements (list had been reviewed and approved in JVET plenary on Wednesday 15 July at 1130):

The following SEI messages and extensions of SEI messages already existing in VSEI v4 are moved from the TuC JVET-AP2032 to VSEI v5 WD:

- EOI extension

- Shuter internal info (SII) extension

- DSC extension

- NNPF extension on tag URIs

- Photosensitive content info (PCI) SEI

- Localization and mapping (LAM) SEI

- Lens Optical Correction (LOC) SEI

- Film grain regions characteristics (FGRC) SEI

- Danmu info (DI) SEI

- Colour mapping info (CMI) SEI

- Display overlays info (DOI) SEI

- Display rectangles (DR) SEI

In this context, the following modifications are made:

VSEI errata items

- JVET-AQ0089 AHG2/AHG9: Errata on general SEI signalling mechanism for VVC, VSEI, HEVC, and AVC

- Item 1 of JVET-AQ0075 AHG9: On video full range indication

- Item 4 of JVET-AQ0093 AHG2/AHG9: On the FPA and CTI SEI messages

- JVET-AQ0114 AHG9: On DSCS SEI message in VSEI v4

On multiple SEI messages

- JVET-AQ0163 AHG9: On SeiExtensionEnableFlag Interface for SII, PRI, EOI

Alpha channel info

- JVET-AQ0129 AHG9: On the Alpha Channel Information SEI message

Film grain regions characteristics (FGRC)

- JVET-AQ0092 AHG9: On the FGRC SEI message

Display overlays info (DOI)

- JVET-AQ0097 AHG9: On the DOI SEI message

- JVET-AQ0103 AHG9: On DOI SEI message

Display rectangles (DR)

- JVET-AQ0107 AHG9: On display rectangles SEI message

- JVET-AQ0127 AHG9: On miscellaneous aspects of display rectangles SEI message

Danmu information (DI)

- JVET-AQ0126 AHG9: On Danmu SEI

- JVET-AQ0144 AHG9: On Danmu Information SEI

- JVET-AQ0074 AHG9: On Danmu Information SEI message

Localization and mapping (LAM)

- JVET-AQ0119 AHG9: On LAM SEI message

Digitally Signed Content

- JVET-AQ0228 AHG9: Proposed joint text on DSC SEI message extensions

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AJ2007 Guidelines for NNVC software development [F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li]

This might benefit from an update in the future, regarding some details of SADL implementation.

Remains valid – not updated: JVET-AN2008 Conformance testing for versatile video coding (Draft 8) [I. Moccagatta, F. Bossen, S. Iwamura, P. de Lagrange]

Primary editor: I. Moccagatta

Remains valid – not updated: JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2) [F. Bossen, K. Sühring, X. Li] [WG 5 DIS N 322)]

Software relating to H.266.2 and ISO/IEC 23090-16 can be found at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/2nd-edition.

Primary editor: F. Bossen.

[kept for future meetings] A request for next edition(s) of reference software(s) could be issued in the next meeting, and also CD(s) could be produced by the same meeting, as SC 29 is meeting beginning of February and could approve such projects without issuing a ballot. Issuing CD(s) by the January meeting (if mature enough) could avoid that an ISO version would lag behind an ITU version which might be submitted for consent in July 2026. Another consideration could be to start the process for ISO early enough in 2026, such that FDIS and ITU consent could be synchronized in April 2027.

Remains valid – not updated: JVET-AP2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring]

Remains valid – not updated: JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AP2016 Common test conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, F. Galpin, R.-L. Liao, S. Liu, A. Segall]

Remains valid – not updated: JVET-AI2017 Common test conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz, Y. Ye]

This document contains test conditions relating to HDR as well as SDR, although ECM testing does not typically include HDR results. Note that a separate output document was issued for HDR test conditions in JVET-AO2011.

It is to be noted that further updates on this document are not planned, and it not intended to be used as basis for future video coding standards development.

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu]

Links to test sequences need to be updated due to the change of the content server.

JVET-AQ2019 Description of algorithms version 16 and software version 18 in neural network-based video coding (NNVC) [F. Galpin, Yue Li, Yun Li, D. Rusanovskyy, T. Shao, J. Ström, L. Wang] (2026-09-11)

New elements in text and software from notes elsewhere in this report:

JVET-AQ0071 EE1-1.1.1. as LOP8 (also replacing LOP7 in NNVC default). If the training crosscheck of EE1-1.1.3 is successful by the time of the AHG telco planned on July 30, the model from 1.1.3 shall be used instead for LOP8.

JVET-AQ0050, containing 2.4.2 VLDRF(16) to be added to the NNVC default, 2.4.3. to replace current LDRF1 (to become LDRF2), and 2.4.4. to replace current HDRF1 (to become HDRF2)

Conditional adoption of JVET-AQ0182 EE1-4.1.4c. To be confirmed by AHG telco on July 30, provided that the corresponding is confirmed to be appropriate by the software coordinator.

JVET-AQ2020 Film grain synthesis technology for video applications ed. 2 (Draft 6) [W. Husak, P. de Lagrange, M. Radosavljević, A. Tourapis] (2026-07-31)

See notes in section 4.11 under JVET-AQ0222.

A CDTR was issued as WG 5 N 409 (content agreed during JVET plenary Wednesday 15 July at 1240)

JVET-AQ2021 Joint Call for Proposals on video compression with capability beyond VVC [J.-R. Ohm, M. Wien, F. Bossen] [WG 5 N 416] (2026-07-15)

Developed from latest version of JVET-AP2026 in JVET-AQ0201, and approved during joint meeting on Tuesday 14 July.

JVET-AQ2022 Draft template for proposal description documents of responses to the Joint CfP on video compression with capability beyond VVC [J.-R. Ohm, M. Wien, E. Alshina, F. Bossen, R. Chernyak, J. Gan, P. de Lagrange, X. Li] (2026-07-15)

Developed from latest version of JVET-AQ0202

No output: JVET-Ax2023, JVET-Ax2024

Remains valid – not updated: JVET-AP2025 Algorithm description of Enhanced Compression Model 20 (ECM 20) [M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang]

Remains valid – not updated: JVET-AO2027 Common test conditions for gaming applications [J. Sauer, R. Chernyak, S. Puri, S. Thiebaud] (2026-01-31)

JVET-AQ2028 Common test conditions on Gaussian splat coding [B. Kroon, P. Rondao Alface, J. Jung, G. Sandri] [WG 5 N 420] (2026-08-14)

JVET copy of an identical WG 5 document, which may have further updates during JAHG editing period.

Remains valid – not updated: JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC [F. Bossen, J.-R. Ohm, M. Wien] [AG 5 N 180] (2025-12-19)

Remains valid – not updated: JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10) [S. Liu, J. Chen, C. Hollmann, J. Ström]

Primary editor: S. Liu.

The content of the document was submitted for approval by SG21, and will be made publicly available as ITU-T H.Sup.22.

Remains valid – not updated: JVET-AI2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu, C. Hollmann]

JVET-AQ2032 Technologies under consideration for future extensions of VSEI (version 13) [S. Deshpande, J. Boyce, J. Chen, M. M. Hannuksela, Y. He, K. Sühring, G. J. Sullivan, H. Tan, A. Tourapis, J. Xu, X. Xu] (2026-08-28)

Primary editor: S. Deshpande

It is noted that the availability date above is for the final version, and it is expected to have an initial version available by 2026-08-07. Alignment with software development of the GSI SEI is of high priority, as the the software is needed to conduct the related JEEs

New elements or improvements (list had been reviewed and approved in JVET plenary on Wednesday 15 July at 1130):

VSEI errata items

- (The aspect that affects syntax of SEI extensions included in the TuC) AVC JVET-AQ0089 AHG2/AHG9: Errata on general SEI signalling mechanism for VVC, VSEI, HEVC, and AVC

Editorial:

- JVET-AQ0072 AHG9: Editorial improvements for TuC

Multiple SEI messages

- JVET-AQ0163 AHG9: on SeiExtenionEnableFlag Interface for SII, PRI, EOI

- JVET-AQ0105 AHG9: On miscellaneous aspects of SEI messages in TuC

NNPF

- JVET-AQ0053 AHG9: On multilayer NNPF

- JVET-AQ0058 AHG9: On NNPF Multiple Inferences Information Signalling

- JVET-AQ0059 AHG9: Bug Fixes for Neural-network Post-filter in TuC

- JVET-AQ0155 AHG9: On NNPFC updates

Modality Information

- JVET-AQ0106 AHG9: On multi-layer spectral representations in modality information SEI message

- JVET-AQ0109 AHG9: On the modality information SEI message extension

Encoder optimization info (EOI)

- JVET-AQ0104 AHG9: On spatial quality optimization in EOI SEI message

Constituent rectangles (CR)

- JVET-AQ0096 AHG9: On the CR SEI message

- Items 1 and 2 of JVET-AQ0105 AHG9: On miscellaneous aspects of SEI messages in TuC

Enhanced colour format information (ECFI)

- Item 2 of JVET-AQ0075 AHG9: On video full range indication

- JVET-AQ0105 AHG9: On miscellaneous aspects of SEI messages in TuC

Versatile SEI RBSP

- JVET-AQ0090 AHG9: On SEI position, payload type, and payload size signalling in the general SEI signalling mechanism in VSEI TuC

- JVET-AQ0091 AHG9: On SEI payload size semantics and its impacts in the general SEI signalling mechanism in VSEI TuC

Quality metric (QM)

- JVET-AQ0073 AHG9: On Quality Metric SEI message

- JVET-AQ0108 AHG9: On quality metrics SEI message

- JVET-AQ0123 AHG9: On reference and source in QM SEI

- JVET-AQ0124 AHG9: On delayed transmission of QM

- JVET-AQ0130 AHG9: On the QM SEI message

Picture segmentation information (PSI)

- JVET-AQ0161 AHG9: on PSI SEI

- JVET-AQ0229 AHG9: On the picture segmentation information SEI message — comments and proposed corrections

Auxiliary sample alignment info (ASAI)

- JVET-AQ0087 AHG9: On the ASAI SEI message

Sample interleaving (SI)

- JVET-AQ0141 AHG9: Fixes to sample interleaving SEI message

Gain Map (GM)

- JVET-AQ0078 AHG9: On Gain map SEI

- JVET-AQ0110 AHG9: On the gain map information SEI message

Generalized alpha plane information (GAPI)

- JVET-AQ0098 AHG9: On the generalized alpha plane information SEI messages

- JVET-AQ0112 AHG9: On the generalized alpha plane information SEI message

Generalized depth information (GDI)

- JVET-AQ0111 AHG9: On the generalized depth information SEI message

- JVET-AQ0165 AHG9: on NADPI BitDepthY Variable

Gaussian splat info (GSI)

- Editorial: JVET-AQ0079 AHG9: Editorial changes for the GSI SEI message in VSEI TuC

- JVET-AQ0080 AHG9: On coding of some syntax elements in the GSI SEI message

- JVET-AQ0081 AHG9: On persistence etc. and error resilience of the GSI SEI message

- JVET-AQ0082 AHG9: On value ranges and decoder handling of reserved values for syntax elements in the GSI SEI message

- JVET-AQ0088 AHG9: On the GSI SEI message syntax and semantics

- JVET-AQ0102 AHG9: Syntax and semantics clean-up of the GSI SEI message

- JVET-AQ0131 AHG9: On the GSI SEI message

- JVET-AQ0132 AHG9: On signalling of various parameters in the GSI SEI message

- JVET-AQ0174 AHG9: On the Gaussian splatting information SEI message

- JVET-AQ0168 AHG9: High-level syntax modifications and bitstream specification for an integer-based GSC V-SEI decoder pipeline

Gaussian splat implicit info (GSII), also referred to as Triplane video-based implicit GS SEI

- JVET-AQ0070 AHG9: [GSC] Improved SEI message for triplane video-based implicit Gaussian splatting representation

Hybrid Gaussian splat info (HGSI), also referred to as Implicit GSI SEI

- JVET-AQ0184 AHG9: [GSC][JEE6.7][JEE6.9] On the Implicit GSI SEI message

New: IR camera adaptation parameters

- JVET-AQ0145 AHG9: IR camera adaptation parameters SE

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AP2033 Report of verification tests on VVC multi-layer coding [O. Chubach, S. Iwamura, P. de Lagrange, M. Wien] [AG 5 N 201)]

Remains valid – not updated: JVET-AI2034 Call for new HDR materials for future video coding development [E. François, W. Husak, S. Iwamura, D. Rusanovskyy, A. Segall, M. Wien] [WG 5 N 312)]

Remains valid – not updated: JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding [S. McCarthy, B. Chen]

Remains valid – not updated: JVET-AG2036 Call for training materials for neural network-based video coding tool development [E. Alshina, F. Galpin, S. Liu, M. Wien] [WG 5 N 266)]

Remains valid – not updated: JVET-AJ2037 Report on subjective quality testing of the FGC SEI message (AG 5 N 140) [P. de Lagrange, W. Husak, M. Wien] [AG 5 N 140)]

Remains valid – not updated: JVET-AM2038 White paper on VSEI [J. Boyce, S. McCarthy, S. Deshpande, G. J. Sullivan, Y. Sanchez, Y.-K. Wang] [AG 3 N209]

An update of the white paper on VVC (from 2021) should be done in a future meeting, removing the section on VSEI and referencing to JVET-AM2038.

Remains valid – not updated: JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience [S. Ikonin, S. Deshpande, V. Zakharchenko] (2025-10-31)

JVET-AQ2040 Description of SEI-related exploration experiments on Gaussian splat coding [J. Jung, W. Husak] (2026-08-14)

SEI related parts from JEE6.7 (WG 5 N 430) and JEE 6.9 (WG 5 N 432) to be copied into this JVET document

No output: JVET-AO2041 and higher

# Future meeting plans, expressions of thanks, a.o.b., and closing of the meeting

The draft of the WG 5 recommendations (see Annex C) was reviewed and approved in JVET at 1300–1315 on Wednesday 15 July. It was agreed to also include the following documents related to Gaussian splat coding which are not available as direct copy into a corresponding JVET document (it was noted that the list may be extended by documents not relevant in scope of JVET based on post-meeting recommendation of the JAHG when meeting with WG 4 and WG 7 experts (to be documented in the final version of this report); by the time of closing the JVET meeting, the following documents were foreseen (list based on documents issued by default in recent meetings of the joint activity):

N 421 – Draft CfP GSC

N 422 – Call for new test materials for GSC (new as per AHG recommendation)

N 423 – FAQ GSC

N 424 – N 432 JEE6.1…6.9 (removal, renaming, or addition of JEEs other than JEE6.7 and JEE6.9 may be done based on post-meeting recommendations of the JAHG).

Future meeting plans were established with the following general guidelines (assuming face-to-face meetings), foreseen for the phase of developing the next-generation video standard from January 2027 onwards:

- Meeting under ITU-T SG21 auspices when it meets (ordinarily starting meetings on the weekend before the first week and closing it around noon on the Wednesday of the second week of the SG21 meeting – a total of 10-11 meeting days under the consideration that JVET cannot meet on the first day of the SG21 meeting while parent-body plenaries are held), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Tuesday or Wednesday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 10-11 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined, or AHG meetings might be scheduled prior to the meeting. In cases of online meetings, no sessions should be held on weekend days, such that meetings would typically start two days earlier.

Some specific future meeting plans were established as follows:

- During 17 – 23 October 2026, 44th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN, to be conducted as physical meeting with remote participation,

- During 13 – 22 January 2027, 45th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU, to be conducted as physical meeting with remote participation,

- During 17 – 28 April 2027, 46th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 7 – 16 July 2027, 47th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI, to be conducted as physical meeting with remote participation,

- During 20 – 29 October 2027, 48th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN, to be conducted as physical meeting with remote participation,

- During 12 – 21 January 2028, 49th meeting under ISO/IEC JTC 1/‌SC 29 in Thessaloniki, GR, to be conducted as physical meeting with remote participation,

- During 25 March – 5 April 2028, 50th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 19 – 28 July 2028, 51st meeting under ISO/IEC JTC 1/‌SC 29 auspices in Fukuoka, JP, to be conducted as physical meeting with remote participation ng,

- During October 2028, 52nd meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

- During January 2029, 53rd meeting under ITU-T SG21 auspices, date and location t.b.d.

The agreed document deadline for the 44th JVET meeting was planned to be Friday 9 October 2026.

Marius Preda was thanked for his great support in maintaining and improving the document site jvet-experts.org. Institut Mines-Télécom was thanked for hosting the sites.

ITU was thanked for the excellent hosting of the 43rd JVET meeting in Geneva. Stefano Polidori and Hiba Tahawi were thanked for the help in preparation, registration and organisation of the meeting. Gent Bajrami and Marc Antoine Zanou were thanked for their responsiveness, dedication and daily help in the setup of the meeting and technical facilities.

The 43rd JVET meeting was closed at approximately 1400 hours CEST on Wednesday 15 July 2026.

# Annex A to JVET report:
List of documents

Dates and times in the table below are in Paris/Geneva time (2 hr. ahead of UTC at the time of the current meeting). It is noted that, if title or authorship of a document deviates from the title or author list in the body of the report, the list of documents in this annex contains the correct title and authors.

| JVET number | MPEG number | Created | First upload | Last upload | Title | Authors |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AQ0001 | m77231 | 2026-06-14 18:00:37 | 2026-07-06 21:03:14 | 2026-07-08 21:58:14 | JVET AHG report: Project Management (AHG1) | J.-R. Ohm M. Wien (co-chairs) G. J. Sullivan (vice-chair) |
| JVET-AQ0002 | m77400 | 2026-06-30 11:22:12 | 2026-07-07 09:41:45 | 2026-07-07 09:41:45 | JVET AHG report: Draft text and test model algorithm description editing (AHG2) | B. Bross C. Rosewarne (co-chairs) F. Bossen A. Browne S. Kim S. Liu J. R. Ohm G. J. Sullivan A. Tourapis Y.-K. Wang Y. Ye (vice-chairs) |
| JVET-AQ0003 | m77401 | 2026-06-30 11:22:37 | 2026-07-07 09:37:27 | 2026-07-15 10:19:12 | JVET AHG report: Test model software development (AHG3) | F. Bossen X. Li K. Sühring (co-chairs) E. François Y. He K. Sharman V. Seregin A. Tourapis (vice-chairs) |
| JVET-AQ0004 | m77402 | 2026-06-30 11:22:55 | 2026-07-07 09:12:05 | 2026-07-07 09:12:05 | JVET AHG report: Test material and visual assessment (AHG4) | V. Baroncini T. Suzuki M. Wien (co-chairs) W. Husak S. Iwamura P. de Lagrange S. Liu X. Meng S. Puri A. Segall S. Wenger (vice-chairs) |
| JVET-AQ0005 | m77403 | 2026-06-30 11:23:15 | 2026-07-06 08:42:22 | 2026-07-07 06:33:07 | JVET AHG report: Conformance testing (AHG5) | I. Moccagatta (chair) F. Bossen T. Ikai S. Iwamura H.-J. Jhu K. Kawamura P. de Lagrange S. Paluri K. Sühring Y. Yu (vice-chairs) |
| JVET-AQ0006 | m77404 | 2026-06-30 11:23:47 | 2026-07-06 19:43:13 | 2026-07-06 19:43:13 | JVET AHG report: ECM software development (AHG6) | V. Seregin (chair) J. Chen R. Chernyak F. Le Léannec K. Zhang (vice-chairs) |
| JVET-AQ0007 | m77405 | 2026-06-30 11:24:04 | 2026-07-07 10:28:04 | 2026-07-07 10:28:04 | JVET AHG report: Tool assessment (AHG7) | X. Li (chair) L.-F. Chen Z. Deng J. Gan E. François R. Ishimoto H.-J. Jhu J. Lainema X. Li J. Pardo A. Stein H. Wang (vice-chairs) |
| JVET-AQ0008 | m77406 | 2026-06-30 11:24:26 | 2026-07-06 19:29:48 | 2026-07-06 19:29:48 | JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) | S. Liu J. Ström S. Wang M. Zhou (AHG chairs) |
| JVET-AQ0009 | m77407 | 2026-06-30 11:24:43 | 2026-07-06 00:10:48 | 2026-07-06 00:10:48 | JVET AHG report: SEI message studies (AHG9) | J. Boyce Y.-K. Wang (co-chairs) T. Chujoh S. Deshpande M. M. Hannuksela Y. He P. de Lagrange G. J. Sullivan H. Tan A. Tourapis S. Wenger P. Wu (vice-chairs) |
| JVET-AQ0010 | m77408 | 2026-06-30 11:25:26 | 2026-07-06 21:43:49 | 2026-07-06 21:43:49 | JVET AHG report: Encoding algorithm optimization (AHG10) | K. Andersson P. de Lagrange A. Duenas (co-chairs) T. Ikai T. Solovyev A. Tourapis (vice-chairs) |
| JVET-AQ0011 | m77409 | 2026-06-30 11:25:45 | 2026-07-06 19:41:14 | 2026-07-07 15:52:12 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina F. Galpin S. Liu (co-chairs) J. Li Y. Li R.-L. Liao M. Santamaria T. Shao M. Wien (vice-chairs) |
| JVET-AQ0012 | m77410 | 2026-06-30 11:26:13 | 2026-07-05 04:00:05 | 2026-07-05 04:00:05 | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) | M. Karczewicz Y. Ye L. Zhang (co-chairs) B. Bross R. Chernyak X. Li K. Naser Y. Yu (vice-chairs) |
| JVET-AQ0013 | m77411 | 2026-06-30 11:26:28 | 2026-07-07 13:00:24 | 2026-07-07 13:00:24 | JVET AHG report: Film grain technologies (AHG13) | W. Husak P. de Lagrange (co-chairs) S. Deshpande A. Duenas X. Meng M. Radosavljević A. Segall G. Teniou A. Tourapis (vice-chairs) |
| JVET-AQ0014 | m77212 | 2026-06-05 14:30:14 | 2026-06-05 14:31:03 | 2026-07-06 18:32:50 | JVET AHG report: NNVC software development (AHG14) | F. Galpin (chair) R. Chang A. Karabutov Yue Li Yun Li M. Santamaria J. N. Shingala Z. Xie (vice-chairs) |
| JVET-AQ0015 | m77412 | 2026-06-30 11:26:56 | 2026-07-07 05:29:45 | 2026-07-07 05:29:45 | JVET AHG report: Gaming content compression (AHG15) | C. Lehmann S. Puri J. Sauer (co-chairs) R. Chernyak A. Duenas L. Wang V. Zakharchenko (vice-chairs) |
| JVET-AQ0016 | m77413 | 2026-06-30 11:27:22 | 2026-07-06 07:31:24 | 2026-07-06 07:31:24 | JVET AHG report: Hardware implementation complexity (AHG16) | Y. Zhao I. Moccagatta K. Naser (co-chairs) H. Huang T. Ikai X. Li J. Park N. Song G. Verba (vice-chairs) |
| JVET-AQ0017 | m77414 | 2026-06-30 11:28:06 | 2026-07-07 09:10:11 | 2026-07-07 09:10:11 | JVET AHG report: Preparation of Call for Proposals (AHG17) | J.-R. Ohm M. Wien F. Bossen (co-chairs) E. Alshina V. Baroncini J. Chen R. Chernyak Z. Deng P. de Lagrange C. Lehmann L. Li P. Nikitin D. Rusanovskyy G. Verba (vice-chairs) |
| JVET-AQ0018 | m77415 | 2026-06-30 11:28:23 | 2026-07-07 10:37:57 | 2026-07-07 10:37:57 | JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) | S. Deshpande S. Ikonin V. Zakharchenko (co-chairs) S. Fößel C. Kim X. Ma S. Puri J. Ström S. Wenger (vice-chairs) |
| JVET-AQ0019 | m77416 | 2026-06-30 11:29:26 |  |  | Report of joint AHG on Gaussian splat coding | J. Jung W. Husak (JAHG co-chairs on behalf of JVET) |
| JVET-AQ0022 | m77418 | 2026-06-30 11:33:24 |  |  | Reports of joint exploration experiments JEE6.7 and JEE6.9 on Gaussian splat coding | J. Jung W. Husak (JAHG co-chairs on behalf of JVET) |
| JVET-AQ0023 | m77417 | 2026-06-30 11:31:07 | 2026-07-05 20:35:20 | 2026-07-08 00:14:39 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina R. Chang F. Galpin Yue Li Yun Li M. Santamaria T. Shao J. Ström Z. Xie (EE coordinators) |
| JVET-AQ0041 | m77184 | 2026-05-18 09:47:35 | 2026-05-19 14:19:28 | 2026-05-29 07:16:09 | AHG17: AhG meeting notes | M. Wien J. Ohm F. Bossen |
| JVET-AQ0042 | m77192 | 2026-05-27 16:48:22 | 2026-05-28 13:56:07 | 2026-06-30 09:23:48 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina F. Galpin |
| JVET-AQ0043 | m77224 | 2026-06-11 20:30:24 | 2026-07-08 12:45:24 | 2026-07-10 10:31:53 | On the Placement of Metadata and Coordination with File Format | D. Podborski A. Tourapis (Apple) |
| JVET-AQ0044 | m77251 | 2026-06-22 22:52:27 | 2026-06-22 23:09:54 | 2026-06-22 23:09:54 | AHG9: Proposed clarifying improvements to subpicture DSC | I. Sodagar (Dolby) |
| JVET-AQ0045 | m77283 | 2026-06-29 10:46:54 | 2026-06-30 16:18:08 | 2026-07-10 08:11:48 | [AHG16] On per-CU RDO quota | M. Wu J. Yu (HiSilicon) Y. Zhao E. Alshina (Huawei) |
| JVET-AQ0046 | m77284 | 2026-06-29 11:34:00 | 2026-06-30 20:34:10 | 2026-06-30 20:34:10 | AHG9: Implementation and showcase of the luma adaptation extension of the EOI SEI message | H. Zhang M. M. Hannuksela (Nokia) |
| JVET-AQ0047 | m77286 | 2026-06-29 13:49:59 | 2026-06-29 16:51:50 | 2026-07-03 13:47:35 | EE1-2.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement | T. Shu X. Chen W. Zhang N. Fu Z. Chen (Wuhan Univ.) |
| JVET-AQ0048 | m77287 | 2026-06-29 13:51:36 | 2026-06-29 17:09:47 | 2026-07-12 10:47:57 | EE1-2.2: Improved H-DRF with Weighted Fusion and Optimized YUV Processing | X. Chen J. Zhang Z. Chen (Wuhan Univ.) |
| JVET-AQ0049 | m77288 | 2026-06-29 14:35:18 | 2026-06-29 15:52:07 | 2026-06-29 15:52:07 | EE1-2.3: Deep Reference Frame Generation for Inter Prediction Enhancement with motion compensation | P. Bordes F. Galpin F. Lo-Bianco M. Paquiry (InterDigital) |
| JVET-AQ0050 | m77289 | 2026-06-29 14:51:32 | 2026-06-29 15:05:21 | 2026-06-29 15:05:21 | EE1-2.4: Combination of test 2.3 (Deep Reference Frame Generation with motion compensation) and tests 2.1 and 2.2 | P. Bordes F. Galpin F. Lo-Bianco M. Paquiry (InterDigital) X. Chen T. Shu J. Zhang Z. Chen (Wuhan Univ.) |
| JVET-AQ0051 | m77293 | 2026-06-29 17:41:18 | 2026-06-30 20:34:57 | 2026-07-08 18:51:00 | AHG9: Encoder optimization information (EOI) SEI message extension for VSEI version 5 | M. M. Hannuksela J. Boyce (Nokia) J. Chen Y. Ye (Alibaba) C.-H. Demarty A. Ak F. Aumont (InterDigital) X. Xu S. Wenger G. Teniou (Tencent) |
| JVET-AQ0052 | m77294 | 2026-06-29 17:44:02 | 2026-06-30 20:35:42 | 2026-06-30 20:35:42 | AHG9: On backward compatibility of NNPF alternative temporal extrapolation versions | M. M. Hannuksela F. Cricri (Nokia) |
| JVET-AQ0053 | m77295 | 2026-06-29 17:52:22 | 2026-06-30 20:36:18 | 2026-07-10 12:46:12 | AHG9: On multilayer NNPF | M. M. Hannuksela J. Boyce (Nokia) |
| JVET-AQ0054 | m77296 | 2026-06-29 17:54:51 | 2026-06-30 20:36:49 | 2026-07-08 18:51:43 | AHG9: Neural-network post-filter (NNPF) extension for VSEI version 5 | M. M. Hannuksela F. Cricri J. Boyce (Nokia) |
| JVET-AQ0055 | m77297 | 2026-06-29 17:56:48 | 2026-06-30 20:37:20 | 2026-06-30 20:37:20 | AHG9: On subpicture mapping of the DSCS SEI message | M. M. Hannuksela J. Boyce (Nokia) |
| JVET-AQ0056 | m77298 | 2026-06-29 17:58:02 | 2026-06-30 20:37:49 | 2026-06-30 20:37:49 | AHG9: On subpicture support of the DSC SEI messages | M. M. Hannuksela J. Boyce (Nokia) |
| JVET-AQ0057 | m77299 | 2026-06-29 18:06:37 | 2026-06-30 20:43:36 | 2026-07-10 09:51:16 | AHG9: On indicating necessity of SEI message extensions | M. M. Hannuksela J. Boyce (Nokia) H. Tan C. Kim (LGE) |
| JVET-AQ0058 | m77300 | 2026-06-29 18:10:02 | 2026-06-30 22:44:59 | 2026-07-07 15:21:57 | AHG9: On NNPF Multiple Inferences Information Signalling | S. Deshpande (Sharp) M. M. Hannuksela F. Cricri (Nokia) |
| JVET-AQ0059 | m77301 | 2026-06-29 18:11:01 | 2026-06-30 22:45:48 | 2026-06-30 22:45:48 | AHG9: Bug Fixes for Neural-network Post-filter in TuC | S. Deshpande (Sharp) |
| JVET-AQ0060 | m77302 | 2026-06-29 18:11:47 | 2026-06-30 22:46:44 | 2026-06-30 22:46:44 | AHG9: Showcase for Photosensitive Content Information SEI | S. Deshpande (Sharp) |
| JVET-AQ0061 | m77303 | 2026-06-29 18:19:39 | 2026-06-30 17:30:09 | 2026-06-30 17:30:09 | AHG9: Errata report on VSEI and HEVC | Y. He M. Karczewicz (Qualcomm) Y.-K. Wang (Bytedance) |
| JVET-AQ0062 | m77304 | 2026-06-29 18:29:29 | 2026-06-30 17:30:52 | 2026-06-30 17:30:52 | AHG9: SPO SEI message extension for VSEI v5 | Y. He S. Zhao L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0063 | m77305 | 2026-06-29 18:33:09 | 2026-06-30 17:31:09 | 2026-06-30 17:31:09 | AHG9: On the DSC SEI message interface text | Y. He S. Zhao L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0064 | m77306 | 2026-06-29 18:43:31 | 2026-06-29 19:14:56 | 2026-07-08 09:01:40 | AHG9: proposing Shutter Interval Information (SII) extension for VSEIv5 | L. Kerofsky Y. He S. Zhao M. Karczewicz (Qualcomm) J. Xu Y.-K. Wang (Bytedance) |
| JVET-AQ0065 | m77307 | 2026-06-29 18:47:38 | 2026-06-29 19:37:47 | 2026-07-08 09:07:09 | AHG9: proposing Localizaation And Mapping (LAM) for VSEIv5 | L. Kerofsky Y. He S. Zhao M. Karczewicz (Qualcomm) G. Teniou (Tencent) |
| JVET-AQ0066 | m77308 | 2026-06-29 18:49:28 | 2026-06-29 19:38:57 | 2026-07-08 08:56:33 | AHG9: proposing Lens Optical Correction (LOC) for VSEIv5 | L. Kerofsky Y. He S. Zhao M. Karczewicz (Qualcomm) G. Teniou (Tencent) |
| JVET-AQ0067 | m77309 | 2026-06-29 18:50:25 | 2026-06-29 19:39:18 | 2026-06-30 21:47:05 | AHG 9: Carrying Gaussian splat data in Main10 bitstreams using JVET-AP0100 | L. Kerofsky J. Jung Y. He M. Karczewicz (Qualcomm) |
| JVET-AQ0068 | m77310 | 2026-06-29 22:09:01 | 2026-06-30 20:51:57 | 2026-07-11 10:21:41 | AHG9: [GSC][JEE6.7] TV-GS: triplane video-based implicit Gaussian splatting representation compression | B. Kathariya D. Y. Lee T. Huang F. Pu G. Su P. Yin G. J. Sullivan S. Oh W. Husak (Dolby) |
| JVET-AQ0069 | m77311 | 2026-06-29 22:13:19 | 2026-06-30 21:09:37 | 2026-07-11 10:22:11 | AHG9: [GSC] Coordinate-remapping in triplane video-based implicit Gaussian splatting framework | B. Kathariya D. Y. Lee T. Huang F. Pu G. Su P. Yin G. J. Sullivan S. Oh W. Husak (Dolby) |
| JVET-AQ0070 | m77312 | 2026-06-29 22:17:20 | 2026-06-30 21:10:46 | 2026-07-11 10:22:37 | AHG9: [GSC] Improved SEI message for triplane video-based implicit Gaussian splatting representation | B. Kathariya D. Y. Lee T. Huang F. Pu G. Su P. Yin G. J. Sullivan S. Oh W. Husak (Dolby) |
| JVET-AQ0071 | m77315 | 2026-06-30 01:49:13 | 2026-06-30 06:31:40 | 2026-07-02 02:19:45 | EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering | J. Lee M. Jeon K. Choi (KHU) B.-S. Kim I. Cho S. Hahm (KBS) |
| JVET-AQ0072 | m77317 | 2026-06-30 04:04:45 | 2026-06-30 08:34:16 | 2026-07-10 17:15:24 | AHG9: Editorial improvements for TuC | X. Xu S. Liu (Tencent) |
| JVET-AQ0073 | m77318 | 2026-06-30 04:05:00 | 2026-06-30 13:01:28 | 2026-07-08 18:16:41 | AHG9: On Quality Metric SEI message | X. Xu S. Wenger S. Liu (Tencent) |
| JVET-AQ0074 | m77319 | 2026-06-30 04:05:13 | 2026-06-30 13:03:36 | 2026-07-09 17:29:04 | AHG9: On Danmu Information SEI message | X. Xu S. Wenger S. Liu (Tencent) |
| JVET-AQ0075 | m77320 | 2026-06-30 04:05:20 | 2026-06-30 13:04:08 | 2026-07-12 18:46:30 | AHG9: On video full range indication | X. Xu S. Liu (Tencent) |
| JVET-AQ0076 | m77321 | 2026-06-30 04:05:36 | 2026-06-30 13:04:35 | 2026-07-09 16:39:24 | AHG9: On PSI SEI message | X. Xu S. Liu (Tencent) |
| JVET-AQ0077 | m77322 | 2026-06-30 04:26:08 |  |  | Withdrawn |  |
| JVET-AQ0078 | m77323 | 2026-06-30 04:40:52 | 2026-06-30 04:43:01 | 2026-07-08 03:41:32 | AHG9: On Gain map SEI | S. Wang S. Xie Y. Gao Y. Bai (ZTE) |
| JVET-AQ0079 | m77324 | 2026-06-30 05:08:20 | 2026-06-30 17:04:34 | 2026-06-30 17:04:34 | AHG9: Editorial changes for the GSI SEI message in VSEI TuC | Y.-K. Wang Y. Xu J. Xu Q. Yang (Bytedance) |
| JVET-AQ0080 | m77325 | 2026-06-30 05:10:27 | 2026-06-30 17:16:26 | 2026-06-30 17:16:26 | AHG9: On coding of some syntax elements in the GSI SEI message | Y.-K. Wang J. Xu Y. Xu Q. Yang K. Zhang (Bytedance) |
| JVET-AQ0081 | m77326 | 2026-06-30 05:10:28 | 2026-06-30 17:25:01 | 2026-06-30 17:25:01 | AHG9: On persistence etc. and error resilience of the GSI SEI message | Y.-K. Wang J. Xu Y. Xu Q. Yang K. Zhang (Bytedance) |
| JVET-AQ0082 | m77327 | 2026-06-30 05:10:28 | 2026-06-30 17:32:23 | 2026-06-30 17:32:23 | AHG9: On value ranges and decoder handling of reserved values for syntax elements in the GSI SEI message | Y.-K. Wang J. Xu Q. Yang Y. Xu K. Zhang (Bytedance) |
| JVET-AQ0083 | m77328 | 2026-06-30 05:10:28 | 2026-06-30 13:58:49 | 2026-06-30 13:58:49 | AHG9: Non-uniform quantization for scale and opacity support in the GSI SEI message | Y. Xu Q. Yang Y.-K. Wang J. Xu K. Zhang L. Zhang (Bytedance) |
| JVET-AQ0084 | m77329 | 2026-06-30 05:10:29 | 2026-06-30 14:00:59 | 2026-06-30 14:00:59 | AHG9: Patch quantization offset and LoD information in the GSI SEI message | Y. Xu Y.-K. Wang J. Xu K. Zhang Q. Yang L. Zhang (Bytedance) |
| JVET-AQ0085 | m77330 | 2026-06-30 05:10:29 | 2026-06-30 15:17:38 | 2026-06-30 15:17:38 | AHG9: On semantics of some syntax elements in the GSI SEI message | Y. Xu Q. Yang J. Xu Y.-K. Wang K. Zhang L. Zhang (Bytedance) |
| JVET-AQ0086 | m77331 | 2026-06-30 05:12:00 | 2026-06-30 13:36:45 | 2026-07-09 08:37:14 | AHG2/AHG9: Initial proposed text for a potential WD of VSEI v5 | Y.-K. Wang J. Xu L. Zhang K. Zhang (Bytedance) S. Deshpande J. Samuelsson-Allendes (Sharp) Y. He L. Kerofsky (Qualcomm) A. M. Tourapis D. Podborski J. Kim (Apple) E. François C.-H. Demarty A. Ak (InterDigital) S. Wenger G. Teniou (Tencent) H. Tan (LGE) |
| JVET-AQ0087 | m77332 | 2026-06-30 05:14:28 | 2026-06-30 17:31:36 | 2026-06-30 17:31:36 | AHG9: On the ASAI SEI message | Y. He M. Karczewicz (Qualcomm) J. Chen Y. Ye (Alibaba) |
| JVET-AQ0088 | m77333 | 2026-06-30 05:14:43 | 2026-06-30 17:32:33 | 2026-06-30 17:32:33 | AHG9: On the GSI SEI message syntax and semantics | Y. He J. Jung R. Hooda L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0089 | m77334 | 2026-06-30 05:25:17 | 2026-06-30 15:38:06 | 2026-06-30 15:38:06 | AHG2/AHG9: Errata on general SEI signalling mechanism for VVC, VSEI, HEVC, and AVC | Y.-K. Wang J. Xu (Bytedance) |
| JVET-AQ0090 | m77335 | 2026-06-30 05:25:44 | 2026-06-30 15:51:35 | 2026-06-30 15:51:35 | AHG9: On SEI position, payload type, and payload size signalling in the general SEI signalling mechanism in VSEI TuC | Y.-K. Wang J. Xu (Bytedance) |
| JVET-AQ0091 | m77336 | 2026-06-30 05:25:55 | 2026-06-30 16:07:57 | 2026-06-30 16:07:57 | AHG9: On SEI payload size semantics and its impacts in the general SEI signalling mechanism in VSEI TuC | Y.-K. Wang J. Xu K. Zhang L. Zhang (Bytedance) |
| JVET-AQ0092 | m77337 | 2026-06-30 05:28:43 | 2026-06-30 20:48:23 | 2026-06-30 20:48:23 | AHG9: On the FGRC SEI message | Y.-K. Wang J. Xu (Bytedance) |
| JVET-AQ0093 | m77338 | 2026-06-30 05:31:28 | 2026-06-30 20:46:24 | 2026-07-08 12:26:09 | AHG2/AHG9: On the FPA and CTI SEI messages | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AQ0094 | m77341 | 2026-06-30 05:52:40 | 2026-06-30 12:55:49 | 2026-06-30 12:55:49 | AHG9: On SH non-uniform quantization signalling in the GSI SEI message | H. Lee J. Do G. Bang (ETRI) |
| JVET-AQ0095 | m77342 | 2026-06-30 05:56:02 | 2026-06-30 12:56:23 | 2026-06-30 12:56:23 | AHG9: On unified signalling of SH transforms in the GSI SEI message | H. Lee J. Do G. Bang (ETRI) |
| JVET-AQ0096 | m77343 | 2026-06-30 06:00:09 | 2026-06-30 17:35:13 | 2026-06-30 17:35:13 | AHG9: On the CR SEI message | Y. He S. Zhao L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0097 | m77344 | 2026-06-30 06:00:28 | 2026-06-30 17:37:27 | 2026-06-30 17:37:27 | AHG9: On the DOI SEI message | Y. He S. Zhao L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0098 | m77345 | 2026-06-30 06:00:54 | 2026-06-30 17:39:25 | 2026-07-12 19:18:50 | AHG9: On the generalized alpha plane information SEI messages | Y. He S. Zhao L. Kerofsky M. Karczewicz (Qualcomm) E. Thomas (Xiaomi) A. Tourapis (Apple) |
| JVET-AQ0099 | m77346 | 2026-06-30 06:01:09 | 2026-06-30 17:44:48 | 2026-07-04 06:28:23 | AHG9: On VSEI version indication in SPO SEI message | Y. He S. Zhao L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0100 | m77347 | 2026-06-30 06:01:19 | 2026-06-30 17:55:58 | 2026-06-30 17:55:58 | AHG9: On the GSI SEI message for spatial random access | Y. He J. Jung Y. Shao L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0101 | m77348 | 2026-06-30 06:13:46 | 2026-06-30 10:24:25 | 2026-07-09 09:26:02 | AHG17: On characteristics of CfP test sequences | T. Chujoh H. Kato Y. Kidani K. Kawamura (KDDI) |
| JVET-AQ0102 | m77349 | 2026-06-30 06:30:47 | 2026-06-30 11:41:11 | 2026-07-13 18:13:45 | AHG9: Syntax and semantics clean-up of the GSI SEI message | J. Do H. Lee G. Bang (ETRI) |
| JVET-AQ0103 | m77351 | 2026-06-30 07:11:22 | 2026-06-30 18:17:38 | 2026-07-10 12:40:35 | AHG9: On DOI SEI message | C. Kim J. Nam H. Tan J. Lee J. Lim S. Kim (LGE) |
| JVET-AQ0104 | m77352 | 2026-06-30 07:12:06 | 2026-06-30 18:33:41 | 2026-07-10 10:20:25 | AHG9: On spatial quality optimization in EOI SEI message | C. Kim H. Tan J. Nam J. Lee J. Lim S. Kim (LGE) |
| JVET-AQ0105 | m77354 | 2026-06-30 07:13:08 | 2026-06-30 18:22:33 | 2026-07-13 07:14:49 | AHG9: On miscellaneous aspects of SEI messages in TuC | C. Kim H. Tan J. Nam J. Lee J. Lim S. Kim (LGE) |
| JVET-AQ0106 | m77355 | 2026-06-30 07:13:39 | 2026-06-30 18:14:06 | 2026-07-12 10:41:11 | AHG9: On multi-layer spectral representations in modality information SEI message | C. Kim H. Tan J. Lee J. Nam J. Lim S. Kim (LGE) L. Kerofsky (Qualcomm) |
| JVET-AQ0107 | m77356 | 2026-06-30 07:14:45 | 2026-06-30 12:52:06 | 2026-07-12 17:36:36 | AHG9: On display rectangles SEI message | J. Nam H. Tan C. Kim J. Lee J. Lim S. Kim (LGE) S. Sasse (HHI) |
| JVET-AQ0108 | m77357 | 2026-06-30 07:15:04 | 2026-06-30 12:52:39 | 2026-07-08 18:44:20 | AHG9: On quality metrics SEI message | J. Nam H. Tan C. Kim J. Lee J. Lim S. Kim (LGE) |
| JVET-AQ0109 | m77360 | 2026-06-30 07:29:50 | 2026-06-30 18:47:05 | 2026-06-30 18:47:05 | AHG9: On the modality information SEI message extension | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AQ0110 | m77361 | 2026-06-30 07:30:02 | 2026-06-30 18:47:41 | 2026-06-30 18:47:41 | AHG9: On the gain map information SEI message | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AQ0111 | m77362 | 2026-06-30 07:31:09 | 2026-06-30 18:48:02 | 2026-07-12 17:41:50 | AHG9: On the generalized depth information SEI message | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AQ0112 | m77363 | 2026-06-30 07:31:19 | 2026-06-30 18:48:21 | 2026-07-12 17:42:34 | AHG9: On the generalized alpha plane information SEI message | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AQ0113 | m77365 | 2026-06-30 08:19:01 | 2026-06-30 15:04:50 | 2026-06-30 15:04:50 | AHG9: On Gaussian Splatting Information SEI message enhancements | Y. Hu X. Ma (MLS Labs) Y. Xu K. Yang (SJTU) |
| JVET-AQ0114 | m77366 | 2026-06-30 08:21:18 | 2026-07-01 00:10:06 | 2026-07-01 11:19:24 | AHG9: On DSCS SEI message in VSEI v4 | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AQ0115 | m77367 | 2026-06-30 08:21:28 | 2026-07-01 00:10:43 | 2026-07-01 11:20:10 | AHG9: Showcase for DSC SEI messages for subpicture-based signing in TuC | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) M. Petterson R. Sjöberg (Ericsson) |
| JVET-AQ0116 | m77368 | 2026-06-30 08:21:34 | 2026-07-01 00:11:21 | 2026-07-01 11:20:52 | AHG9: On DSCI SEI message in TuC | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AQ0117 | m77369 | 2026-06-30 08:21:42 | 2026-07-01 00:12:18 | 2026-07-01 11:21:18 | AHG9: On DSCS SEI message in TuC | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AQ0118 | m77370 | 2026-06-30 08:21:43 | 2026-07-01 00:12:52 | 2026-07-01 11:21:43 | AHG9: On non-VCL NAL units signing in DSC SEI messages in TuC | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AQ0119 | m77371 | 2026-06-30 08:21:44 | 2026-07-01 00:13:21 | 2026-07-12 15:39:48 | AHG9: On LAM SEI message | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) L. Kerofsky (Qualcomm) G. Teniou (Tencent) |
| JVET-AQ0120 | m77372 | 2026-06-30 08:21:49 |  |  | Withdrawn |  |
| JVET-AQ0121 | m77373 | 2026-06-30 08:21:55 |  |  | Withdrawn |  |
| JVET-AQ0122 | m77374 | 2026-06-30 08:28:48 | 2026-06-30 09:15:35 | 2026-07-10 10:18:47 | AHG9: QR Code Information SEI | T. Biatek S. He D. Fortin J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AQ0123 | m77375 | 2026-06-30 08:29:03 | 2026-06-30 09:36:17 | 2026-07-10 08:32:31 | AHG9: On reference and source in QM SEI | T. Biatek S. He J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AQ0124 | m77376 | 2026-06-30 08:29:15 | 2026-06-30 13:54:23 | 2026-07-01 07:35:49 | AHG9: On delayed transmission of QM SEI | T. Biatek S. He J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AQ0125 | m77377 | 2026-06-30 08:29:33 | 2026-06-30 10:03:43 | 2026-06-30 10:03:49 | AHG9: On QM-SEI and SVTA-MQA | T. Biatek S. He J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AQ0126 | m77378 | 2026-06-30 08:29:47 | 2026-06-30 08:58:40 | 2026-07-12 08:22:36 | AHG9: On Danmu SEI | T. Biatek S. He J. Boyce M. M. Hannuksela (Nokia) C. Kim H. Tan (LGE) |
| JVET-AQ0127 | m77379 | 2026-06-30 08:36:43 | 2026-06-30 08:57:51 | 2026-07-09 16:57:25 | AHG9: On miscellaneous aspects of display rectangles SEI message | H. Tan J. Nam J. Lee C. Kim J. Lim S. Kim (LGE) |
| JVET-AQ0128 | m77380 | 2026-06-30 08:44:24 | 2026-06-30 18:07:58 | 2026-06-30 18:07:58 | AHG9:Software implementation of luma adaptation optimization in the EOI SEI message and showcase for display power adaptation | C.-H. Demarty A. Ak F. Aumont (InterDigital) |
| JVET-AQ0129 | m77381 | 2026-06-30 08:47:25 | 2026-06-30 18:11:00 | 2026-07-08 11:33:40 | AHG9: On the Alpha Channel Information SEI message | C.-H. Demarty A. Ak E. François P. de Lagrange F. Urban N. Caramelli (InterDigital) |
| JVET-AQ0130 | m77383 | 2026-06-30 08:49:55 | 2026-06-30 18:13:25 | 2026-07-12 15:13:09 | AHG9: On the QM SEI message | C.-H. Demarty A. Ak R. James (InterDigital) |
| JVET-AQ0131 | m77384 | 2026-06-30 09:15:03 | 2026-06-30 16:03:06 | 2026-06-30 16:03:06 | AHG9: On the GSI SEI message | S. Harribey M. Pettersson M. Damghanian R. Sjöberg (Ericsson) |
| JVET-AQ0132 | m77385 | 2026-06-30 09:15:05 | 2026-06-30 16:04:18 | 2026-06-30 16:04:18 | AHG9: On signalling of various parameters in the GSI SEI message | S. Harribey M. Pettersson M. Damghanian R. Sjöberg (Ericsson) |
| JVET-AQ0133 | m77387 | 2026-06-30 09:26:20 | 2026-07-02 05:34:52 | 2026-07-02 09:03:40 | Withdrawn |  |
| JVET-AQ0134 | m77388 | 2026-06-30 09:36:06 | 2026-06-30 09:42:35 | 2026-06-30 09:42:35 | Crosscheck of JVET-AQ0049 (EE1-2.3: Deep Reference Frame Generation for Inter Prediction Enhancement with motion compensation) | X. Chen Z. Chen (Wuhan Univ.) |
| JVET-AQ0135 | m77389 | 2026-06-30 10:03:48 | 2026-06-30 16:15:33 | 2026-06-30 16:15:33 | AHG9: Support of regions of update and update priority SEI message | S. He T. Biatek J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AQ0136 | m77390 | 2026-06-30 10:05:00 | 2026-06-30 21:36:00 | 2026-07-12 10:29:16 | AHG9: Composited Pictures Information SEI | S. He T. Biatek J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AQ0137 | m77391 | 2026-06-30 10:29:04 | 2026-06-30 10:55:03 | 2026-06-30 10:55:03 | Crosscheck of JVET-AQ0047 (EE1-2.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) and JVET-AQ0048 (EE1-2.2: Improved H-DRF with Weighted Fusion and Optimized YUV Processing) | P. Bordes F. Galpin (InterDigital) |
| JVET-AQ0138 | m77392 | 2026-06-30 10:38:06 | 2026-06-30 18:40:11 | 2026-07-07 09:41:41 | AHG14: SADL Update | F. Galpin (InterDigital) |
| JVET-AQ0139 | m77395 | 2026-06-30 10:45:50 | 2026-06-30 20:34:37 | 2026-07-09 14:16:03 | AHG9: Historical Information SEI message | J. Pardo T. Solovyev E. Alshina (Huawei) X. Xu S. Liu (Tencent) |
| JVET-AQ0140 | m77396 | 2026-06-30 11:11:35 | 2026-06-30 16:56:55 | 2026-06-30 16:56:55 | AHG9: A showcase for picture quality indication in EOI SEI message extension | B. Chen J. Chen Y. Ye (Alibaba) S. Yin X. Ju S. Wang (CityUHK) |
| JVET-AQ0141 | m77397 | 2026-06-30 11:11:42 | 2026-06-30 16:57:36 | 2026-07-10 16:58:25 | AHG9: Fixes to sample interleaving SEI message | J. Chen Y. Ye (Alibaba) S. He (Nokia) |
| JVET-AQ0142 | m77398 | 2026-06-30 11:11:51 | 2026-06-30 17:11:00 | 2026-06-30 17:11:00 | AHG9: [GSC-related] An animation-based 3DGS Avatar Coding | Z. Zhang S. Yin X. Wu S. Wang (CityUHK) B. Chen J. Chen R.-L. Liao Y. Yan (Alibaba) A. Trioux F. Yang (Xidian Univ) C. Wang (PCL) R. Wang (PKUSZ) |
| JVET-AQ0143 | m77399 | 2026-06-30 11:13:09 | 2026-06-30 18:23:47 | 2026-06-30 18:23:47 | AHG9: [GSC-related] Avatar Generation Information SEI message | J. Chen B. Chen Y. Yan (Alibaba) S. Yin Z. Zhang S. Wang (CityUHK) A. Trioux F. Yang (Xidian Univ.) |
| JVET-AQ0144 | m77419 | 2026-06-30 11:58:24 | 2026-06-30 12:00:02 | 2026-07-10 12:00:50 | AHG9: On Danmu Information SEI | S. Xie Y. Gao S. Wang W. Niu Y. Bai (ZTE) J. Xu Y.-K. Wang (Bytedance) |
| JVET-AQ0145 | m77423 | 2026-06-30 15:49:11 | 2026-06-30 22:22:17 | 2026-07-12 18:29:47 | AHG9: IR camera adaptation parameters SEI | V. Zakharchenko J. Boyce M. Hannuksela (Nokia) |
| JVET-AQ0146 | m77424 | 2026-06-30 15:56:29 | 2026-06-30 18:44:27 | 2026-07-07 09:33:27 | EE1-related: chroma location information handling in DRF (EE1-2.3 and EE1-2.4) | F. Galpin P. Bordes (InterDigital) |
| JVET-AQ0147 | m77425 | 2026-06-30 16:25:20 | 2026-06-30 16:37:41 | 2026-06-30 16:37:41 | AHG9: Layer priority SEI message | J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AQ0148 | m77426 | 2026-06-30 16:25:22 | 2026-06-30 16:54:47 | 2026-06-30 16:54:47 | AHG9: Support for multiple bitstreams and alternative splat types in Gaussian splatting info SEI | J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AQ0149 | m77427 | 2026-06-30 16:52:43 | 2026-06-30 18:51:26 | 2026-07-12 00:07:27 | AHG9: Classification label info SEI | J. Boyce M. M. Hannuksela T. Biatek (Nokia) |
| JVET-AQ0150 | m77428 | 2026-06-30 16:54:58 | 2026-06-30 17:00:17 | 2026-06-30 17:00:17 | [AHG16] Improvements on Hardware Implementation Complexity (HIC) Software | K. Naser F. Le Léannec F. Galpin F. de Lagrange (InterDigital) Y. Zhao E. Alshina (Huawei) |
| JVET-AQ0151 | m77430 | 2026-06-30 17:21:51 | 2026-06-30 17:23:17 | 2026-06-30 17:23:17 | AHG11] [AHG14] Huawei response to MPEG survey on AI hardware | E. Alshina Y. Zhao (Huawei) |
| JVET-AQ0152 | m77432 | 2026-06-30 17:28:21 | 2026-06-30 17:51:26 | 2026-07-08 10:04:16 | Runtime and Energy Consumption Estimation for Neural Network-based In-loop Filters in NNVC | S. Caglar Z. Chen M. Y. Saritas H. B. Dogaroglu C. Eteke E. Steinbach (TU Munich) |
| JVET-AQ0153 | m77434 | 2026-06-30 17:29:06 | 2026-06-30 18:36:10 | 2026-07-07 15:51:15 | [AHG17] Comments on CfP document | E. Alshina Y. Zhao X. Ma S. Ikonin T. Solovyev (Huawei) |
| JVET-AQ0154 | m77437 | 2026-06-30 17:31:41 | 2026-06-30 17:34:41 | 2026-06-30 17:34:41 | AHG9: Proposed GDR Info SEI message | S. Mate M. M. Hannuksela J. Boyce (Nokia) |
| JVET-AQ0155 | m77439 | 2026-06-30 17:33:02 | 2026-06-30 23:02:37 | 2026-07-10 18:10:02 | AHG9: On NNPFC updates | T. Borges Y. Sanchez R. Skupin C. Hellge T. Schierl (HHI) M. M. Hannuksela F. Cricri (Nokia) |
| JVET-AQ0156 | m77440 | 2026-06-30 17:35:31 | 2026-07-10 02:05:48 | 2026-07-12 12:32:23 | AHG9: Proposal for a showcase playground | E. Thomas E. Potetsianakis E. Alexiou M.-L. Champel (Xiaomi) |
| JVET-AQ0157 | m77441 | 2026-06-30 17:35:32 | 2026-06-30 23:03:27 | 2026-06-30 23:03:27 | AHG9: On NNPFC framework formats and model parameter compression | R. Skupin T. Borges Y. Sanchez C. Hellge T. Schierl (HHI) |
| JVET-AQ0158 | m77442 | 2026-06-30 17:38:18 | 2026-06-30 23:04:43 | 2026-06-30 23:04:43 | AHG9: On detection of SEI extensions in SPO | T. Borges Y. Sanchez R. Skupin C. Hellge T. Schierl (HHI) |
| JVET-AQ0159 | m77444 | 2026-06-30 18:07:35 | 2026-06-30 20:09:30 | 2026-06-30 20:09:37 | AHG9: MI Multispectrum Extension for VSEIv5 | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0160 | m77445 | 2026-06-30 18:08:29 | 2026-06-30 20:12:24 | 2026-06-30 20:12:24 | AHG9: GM SEI for VSEIv5 | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0161 | m77446 | 2026-06-30 18:09:59 | 2026-06-30 20:07:04 | 2026-06-30 20:07:04 | AHG9: on PSI SEI | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0162 | m77447 | 2026-06-30 18:11:31 | 2026-06-30 19:58:52 | 2026-06-30 19:58:52 | AHG9: on SDI and GM SEI | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0163 | m77448 | 2026-06-30 18:12:35 | 2026-06-30 20:02:18 | 2026-06-30 22:07:17 | AHG9: on SeiExtenionEnableFlag Interface for SII, PRI, EOI | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0164 | m77449 | 2026-06-30 18:12:54 | 2026-06-30 23:10:43 | 2026-06-30 23:10:43 | AHG9: Cross-platform determinism and bit-exact conformance of the GSC V-SEI framework | J. Ricard G. Teniou S. Wenger (Tencent) |
| JVET-AQ0165 | m77450 | 2026-06-30 18:13:21 | 2026-06-30 19:57:26 | 2026-06-30 19:57:26 | AHG9: on NADPI BitDepthY Variable | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0166 | m77451 | 2026-06-30 18:14:12 | 2026-06-30 20:13:17 | 2026-06-30 20:13:17 | AHG9: C2PA SEI Message | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0167 | m77452 | 2026-06-30 18:14:54 | 2026-06-30 23:11:09 | 2026-07-13 23:56:55 | AHG9: Integer-only GSC V-SEI decoder | J. Ricard G. Teniou S. Wenger (Tencent) |
| JVET-AQ0168 | m77453 | 2026-06-30 18:15:58 | 2026-06-30 23:11:31 | 2026-07-15 09:54:38 | AHG9: High-level syntax modifications and bitstream specification for an integer-based GSC V-SEI decoder pipeline | J. Ricard G. Teniou S. Wenger (Tencent) |
| JVET-AQ0169 | m77454 | 2026-06-30 18:16:49 | 2026-06-30 23:11:56 | 2026-06-30 23:11:56 | AHG9: Single-video GSC V-SEI framework | J. Ricard G. Teniou S. Wenger (Tencent) |
| JVET-AQ0170 | m77455 | 2026-06-30 18:18:14 | 2026-06-30 23:12:30 | 2026-06-30 23:12:30 | AHG9: Multi-frame improvement of the GSC V-SEI framework | J. Ricard G. Teniou S. Wenger (Tencent) J. Jung (Qualcomm) |
| JVET-AQ0171 | m77459 | 2026-06-30 18:46:30 | 2026-06-30 18:49:27 | 2026-06-30 18:49:27 | AhG14: CUDA backend for SADL | F. Galpin S. Cizel (InterDigital) |
| JVET-AQ0172 | m77460 | 2026-06-30 18:49:32 | 2026-07-04 01:24:20 | 2026-07-05 16:32:29 | AHG11: VLOP4 Modification for Complexity Reduction with Coding Parameter-guided Feature Modulation | H. Cho J. Kim S. Bahk T. Lee H. Y. Kim (KHU) D. Kim S.-C. Lim (ETRI) |
| JVET-AQ0173 | m77461 | 2026-06-30 19:04:25 | 2026-06-30 19:16:27 | 2026-07-08 09:16:08 | [AHG11][AHG 14] Real-Time Implementation of an End-to-End AI Video Codec | S. Cizel R. Mullakhmetov A. Berk C. Finlay F. Galpin E. François (InterDigital) |
| JVET-AQ0174 | m77462 | 2026-06-30 19:27:04 | 2026-06-30 22:05:48 | 2026-07-09 18:39:59 | AHG9: On the Gaussian splatting information SEI message | R. Hooda J. Jung Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0175 | m77463 | 2026-06-30 19:46:50 | 2026-06-30 23:19:42 | 2026-06-30 23:19:42 | AHG9: Display overlays info (DOI) SEI message for VSEI v5 WD | J. Boyce M. M. Hannuksela T. Biatek (Nokia) Y. He S. Zhao L. Kerofsky (Qualcomm) H. Tan C. Kim J. Lee (LGE) G. Tenniou S. Wenger A. Hinds (Tencent) |
| JVET-AQ0176 | m77464 | 2026-06-30 19:46:53 | 2026-06-30 23:20:58 | 2026-06-30 23:20:58 | AHG9: Display rectangles (DR) SEI message for VSEI v5 WD | J. Boyce M. M. Hannuksela T. Biatek (Nokia) Y. He S. Zhao L. Kerofsky (Qualcomm) H. Tan C. Kim J. Lee (LGE) R. Imichi Y. Tokumo T. Ikai (Sharp) |
| JVET-AQ0177 | m77465 | 2026-06-30 19:46:56 | 2026-06-30 23:22:18 | 2026-07-01 22:04:48 | AHG9: Bitdepth range info (BRI) SEI message for VSEI v5 WD | J. Boyce M. M. Hannuksela T. Biatek (Nokia) Y. He S. Zhao L. Kerofsky (Qualcomm) H. Tan C. Kim J. Lee (LGE) R. Imichi Y. Tokumo T. Ikai (Sharp) |
| JVET-AQ0178 | m77467 | 2026-06-30 20:15:19 | 2026-06-30 20:19:06 | 2026-07-06 16:22:29 | Crosscheck of JVET-AQ0071 (EE1-1.1.1 and EE1-1.1.2: Dynamic convolution for LOP7 neural in-loop filtering) | V. Khamidullin S. Ikonin E. Alshina (Huawei) |
| JVET-AQ0179 | m77468 | 2026-06-30 20:47:03 | 2026-06-30 20:49:59 | 2026-07-11 19:41:13 | AHG9: Processing History Information SEI | C. Feldmann T. Biatek J. Boyce (Nokia) J. Pardo T. Solovyev E. Alshina (Huawei) X. Xu S. Liu (Tencent) |
| JVET-AQ0180 | m77469 | 2026-06-30 20:56:08 | 2026-07-11 11:53:31 | 2026-07-11 11:53:31 | AHG9: Showcase for infrared camera adaptation parameters SEI | V. Zakharchenko (Nokia) |
| JVET-AQ0181 | m77470 | 2026-06-30 20:58:21 | 2026-07-12 09:23:18 | 2026-07-12 15:35:01 | AHG9: Showcase for auxiliary sampling alignment information SEI message | V. Zakharchenko (Nokia) |
| JVET-AQ0182 | m77471 | 2026-06-30 21:20:04 | 2026-06-30 23:30:50 | 2026-07-07 15:07:19 | EE1-4.1: Adaptive Quantization and Hardware Optimization for NNIP | A. Akhtar S. Esenlik Y. Matsuba M. Karczewicz (Qualcomm) |
| JVET-AQ0183 | m77472 | 2026-06-30 23:41:46 | 2026-07-01 00:41:12 | 2026-07-09 18:21:02 | AHG18: Forward Error Correction for ULL simulation software | I. Gribushin K. Malyshev G. Gotovskii B. Shevchenko S. Ikonin E. Alshina (Huawei) |
| JVET-AQ0184 | m77473 | 2026-06-30 23:59:39 | 2026-07-01 00:03:58 | 2026-07-01 00:03:58 | AHG9: [GSC][JEE6.7][JEE6.9] On the Implicit GSI SEI message | Y. Sanchez S. Sasse T. M. Borges S. Lee Y. Berendsohn R. Skupin C. Hellge T. Schierl (HHI) |
| JVET-AQ0185 | m77475 | 2026-07-01 02:47:22 | 2026-07-05 23:44:48 | 2026-07-05 23:45:34 | AHG16: Area-ratio extension to the RDO selection count restriction | H. Bai T. Ikai Y. Tokumo R. Ishimoto (Sharp) |
| JVET-AQ0186 | m77478 | 2026-07-01 06:35:09 | 2026-07-01 08:19:32 | 2026-07-09 15:47:43 | MV-HEVC Conformance Bitstreams for the Multiview Monochrome 10/12 and Multiview 4:4:4 10/12 Profiles | A. M. Tourapis D. Podborski J. Kim (Apple) |
| JVET-AQ0187 | m77479 | 2026-07-01 06:36:25 | 2026-07-01 08:29:59 | 2026-07-01 08:29:59 | Consolidating the HEVC Reference Software: Adding SHVC Support to the dev-multiview444 Branch | A. M. Tourapis D. Podborski J. Kim S. Paluri S. Choi W. Zia (Apple) |
| JVET-AQ0188 | m77480 | 2026-07-01 06:38:58 | 2026-07-01 08:35:51 | 2026-07-11 22:54:03 | Subpicture support in HEVC | A. M. Tourapis D. Podborski J. Kim S. Paluri (Apple) |
| JVET-AQ0189 | m77481 | 2026-07-01 06:39:52 | 2026-07-01 08:19:05 | 2026-07-01 08:19:05 | HEVC Profile Signalling Reference | A. M. Tourapis D. Podborski J. Kim S. Paluri W. Zia (Apple) |
| JVET-AQ0190 | m77483 | 2026-07-01 07:49:32 | 2026-07-09 15:23:12 | 2026-07-09 15:23:12 | Prototype VVC decoder supporting dynamic layer switching during multi-layer bitstream playback | S. Iwamura S. Nemoto K. Unno Y. Kondo A. Ichigaya (NHK) |
| JVET-AQ0191 | m77484 | 2026-07-01 07:50:36 | 2026-07-09 15:23:27 | 2026-07-09 15:23:27 | Browser-based VVC player supporting CMAF/DASH streaming on Android | S. Iwamura S. Nemoto K. Unno Y. Kondo A. Ichigaya (NHK) |
| JVET-AQ0192 | m77487 | 2026-07-01 11:24:50 | 2026-07-01 11:51:57 | 2026-07-01 11:51:57 | Improvements of NS2 software at 0.2x under CfP conditions | G. Laroche P. Onno B. Galmiche N. Ouedraogo (Canon) |
| JVET-AQ0193 | m77488 | 2026-07-01 12:00:53 | 2026-07-01 12:14:12 | 2026-07-03 05:26:30 | Crosscheck of JVET-AQ0071 (EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering) | N. J. Gadgil M. N. Hosmane R. N. Gadde W. I. Choi K. P. Choi (Samsung) |
| JVET-AQ0194 | m77493 | 2026-07-01 16:07:33 | 2026-07-01 16:12:05 | 2026-07-13 10:28:55 | AHG10/AHG12: Clean-Slate NextSoftware2 implementation without future Video Coding Tools | W. Ahmad K. Andersson L. Litwic M. Pettersson R. Sjöberg N. Stegmaier J. Ström P. Wennersten (Ericsson) C. Bartnik J. Brandenburg B. Bross V. George J. Güther G. Hege C. Helmrich A. Henkel T. Hinz C. Lehmann D. Marpe T. Nguyen J. Pfaff T. Schierl H. Schwarz B. Stallenberger K. Sühring A. Wieckowski T. Wiegand (Fraunhofer HHI) P. Astola S. Blasi F. Cricri D. Bugdayci Sansli C. Feldmann D. Fortin J. Funnell A. Hallapuro M. M. Hannuksela S. Hong I. Jumakulyyev J. Lainema N. Le N. Neumann J. Ridge D. Rusanovskyy S. Schwarz H. Zhang N. Zou (Nokia) |
| JVET-AQ0195 | m77494 | 2026-07-01 16:10:59 |  |  | Withdrawn |  |
| JVET-AQ0196 | m77502 | 2026-07-01 22:10:49 | 2026-07-01 22:16:52 | 2026-07-01 22:16:52 | AHG9: Loss concealment purpose for NNPFC SEI message | S. Mate J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AQ0197 | m77508 | 2026-07-02 12:41:55 | 2026-07-06 11:53:47 | 2026-07-06 11:53:47 | Withdrawn |  |
| JVET-AQ0198 | m77529 | 2026-07-03 05:16:12 | 2026-07-05 10:14:31 | 2026-07-09 11:57:42 | Visualization of ECM Progress: Capturing Faint Prints of Tool Evolution AHG 6 | T. Ikai K.-W. Liang (Sharp) |
| JVET-AQ0199 | m77530 | 2026-07-03 05:21:30 | 2026-07-06 08:15:49 | 2026-07-06 08:15:49 | AHG18: Feedback-Aware Reference Preservation for Ultra-Low Latency | Y. Xiao Y. Hu X. Ma (MLS labs) |
| JVET-AQ0200 | m77531 | 2026-07-03 07:30:07 | 2026-07-03 07:34:32 | 2026-07-06 21:55:42 | [AHG11] Training-Time Optimization for LOP7 | J. Lee M. Jeon K. Choi (KHU) B.-S. Kim I. Cho S. Hahm (KBS) |
| JVET-AQ0201 | m77536 | 2026-07-03 12:50:27 | 2026-07-03 16:29:39 | 2026-07-13 17:54:43 | AHG17: Suggested editorial changes to the Final Draft Joint Call for Proposals on Video Compression with Capability beyond VVC | M. Wien J.-R. Ohm F. Bossen |
| JVET-AQ0202 | m77537 | 2026-07-03 12:53:43 | 2026-07-03 17:09:46 | 2026-07-13 17:36:33 | AHG17: Suggested initial text of "Template for Proposal Description Documents of Responses to the Joint CfP on Video Compression with Capability beyond VVC" | J.-R. Ohm M. Wien |
| JVET-AQ0203 | m77541 | 2026-07-03 16:17:10 | 2026-07-03 16:18:54 | 2026-07-03 16:18:54 | [GSC][JEE.6.7][Test 1] Cross-check of non-linear transforms | J. Ricard G. Teniou S. Wenger (Tencent) |
| JVET-AQ0204 | m77546 | 2026-07-04 11:17:20 | 2026-07-04 12:19:53 | 2026-07-04 12:19:53 | How to calculate average PSNR for a video sequence | J. Samuelsson-Allendes S. Deshpande (Sharp) |
| JVET-AQ0205 | m77547 | 2026-07-04 14:37:59 | 2026-07-06 09:11:09 | 2026-07-07 08:35:37 | Cross-check of JVET-AQ0182 (EE1-4.1: adaptive quantization and hardware optimization for NNIP) | T. Dumas F. Galpin (InterDigital) |
| JVET-AQ0206 | m77548 | 2026-07-04 16:08:34 | 2026-07-06 05:09:39 | 2026-07-08 08:54:41 | Crosscheck of JVET-AQ0048 (EE1-2.2.2: Improved L-DRF with Weighted Fusion and Optimized YUV Processing) | F. Kozhamkulova (Huawei) N. Bhaskar (Huawei) T. Solovyev (Huawei) E. Alshina (Huawei) |
| JVET-AQ0207 | m77559 | 2026-07-05 16:41:44 | 2026-07-05 16:44:20 | 2026-07-06 19:48:09 | AHG9: [GSC][JEE6.7] Cross-check results and comments on the GSC VSEI framework | J. Jung Y. Shao M. Karczewicz (Qualcomm) |
| JVET-AQ0208 | m77560 | 2026-07-05 16:45:22 | 2026-07-05 16:47:49 | 2026-07-06 19:49:06 | AHG9: Evaluation of the GSC VSEI framework | J. Jung L. Kerofsky Y. He M. Karczewicz (Qualcomm) |
| JVET-AQ0209 | m77561 | 2026-07-05 16:50:04 | 2026-07-05 16:52:16 | 2026-07-06 19:49:46 | AHG9: Improvements of the GSC VSEI framework | J. Jung L. Kerofsky Y. He M. Karczewicz (Qualcomm) |
| JVET-AQ0210 | m77562 | 2026-07-05 16:52:58 | 2026-07-05 16:54:53 | 2026-07-06 19:49:33 | AHG9: Spatial Random Access implementation in GSC VSEI framework | Y. Shao J. Jung Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AQ0211 | m77567 | 2026-07-06 00:20:24 | 2026-07-06 00:43:35 | 2026-07-06 00:43:35 | Film grain analysis and synthesis support in HDRTools | A. M. Tourapis J. Kim D. Podborski (Apple) |
| JVET-AQ0212 | m77579 | 2026-07-06 04:48:39 | 2026-07-06 04:52:17 | 2026-07-06 04:52:17 | Crosscheck of JVET-AQ0069 (AHG9: [GSC] Coordinate-remapping in triplane video-based implicit Gaussian splatting framework) | X. Wu (City Univ. HK) B. Chen (Alibaba) |
| JVET-AQ0213 | m77584 | 2026-07-06 05:31:53 | 2026-07-06 05:47:21 | 2026-07-08 09:21:37 | Crosscheck of JVET-AQ0050 (EE1-2.4: Combination of test 2.3 (Deep Reference Frame Generation with motion compensation) and tests 2.1 and 2.2) | F. Kozhamkulova N. Bhaskar T. Solovyev E. Alshina (Huawei) |
| JVET-AQ0214 | m77681 | 2026-07-06 11:42:30 | 2026-07-06 13:43:31 | 2026-07-06 13:43:31 | Cross-check of JVET-AQ0192 â€œImprovements of NS2 software at 0.2x under CfP conditionsâ€ | F. Le Léannec (InterDigital) |
| JVET-AQ0215 | m77687 | 2026-07-06 12:04:11 | 2026-07-06 13:18:03 | 2026-07-12 08:42:35 | AHG9: New transform type for Gaussian splatting information SEI message | H. Yang D. Kim B. Choi G. Moon J.-G. Kim (KAU) K. Oh G. Lee (ETRI) |
| JVET-AQ0216 | m77720 | 2026-07-06 13:59:06 | 2026-07-06 23:05:07 | 2026-07-13 11:48:31 | Crosscheck of JVET-AQ0048 (EE1-2.2.1: Improved H-DRF with Weighted Fusion and Optimized YUV Processing) | N. Bhaskar F. Kozhamkulova T. Solovyev (Huawei) |
| JVET-AQ0217 | m77721 | 2026-07-06 14:02:13 | 2026-07-06 21:48:50 | 2026-07-07 23:05:25 | Crosscheck of JVET-AQ0047 (EE1-2.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) | N. Bhaskar T. Solovyev (Huawei) |
| JVET-AQ0218 | m77747 | 2026-07-06 16:40:22 | 2026-07-06 20:15:21 | 2026-07-07 18:26:33 | Cross-check of JVET-AQ0182 (EE1-4.1: adaptive quantization and hardware optimization for NNIP) | A. Chandran (Huawei) |
| JVET-AQ0219 | m77787 | 2026-07-06 20:54:49 | 2026-07-06 20:59:08 | 2026-07-06 20:59:08 | AHG9: [GSC][JEE6.7][JEE6.9] On the implementation of the implicit gaussian splatting information SEI message | S. Sasse Y. Berendsohn T. M. Borges Y. Sanchez S. Lee R. Skupin C. Hellge T. Schierl (HHI) |
| JVET-AQ0220 | m77793 | 2026-07-06 22:02:21 | 2026-07-06 22:21:34 | 2026-07-06 22:21:34 | [AHG17] Suggested Editorial Changes to the CfP Document | R. Chernyak (Tencent) |
| JVET-AQ0221 | m77808 | 2026-07-06 22:56:07 | 2026-07-08 20:59:44 | 2026-07-11 11:52:33 | AHG9: [Avatar-related] Avatar Representation Information SEI Message | J. P. Cova Regateiro G. Martin-Cocher (InterDigital) |
| JVET-AQ0222 | m77826 | 2026-07-07 00:01:29 | 2026-07-07 00:22:13 | 2026-07-13 13:11:07 | AHG13: proposed updates to the technical report on film grain synthesis technology for video applications | P. de Lagrange M. Radosavljević (InterDigital) |
| JVET-AQ0223 | m77868 | 2026-07-07 10:46:14 | 2026-07-07 10:50:56 | 2026-07-07 10:50:56 | AHG9: [GSC][JEE6.7] CE Test Description for Test 1 Quantization â€“ Generic Exponential Transform | J. Do H. Lee G. Bang (ETRI) |
| JVET-AQ0224 | m77891 | 2026-07-08 07:15:13 | 2026-07-08 07:17:39 | 2026-07-08 07:17:39 | [GSC][JEE6.7] Crosscheck on Improvements of the GSC VSEI framework | J. Do H. Lee G. Bang (ETRI) |
| JVET-AQ0225 | m77909 | 2026-07-08 16:54:33 | 2026-07-08 16:57:24 | 2026-07-08 16:57:24 | [GSC][JEE6.9] Cross-check of Spatial Random Access | J. Ricard G. Teniou S. Wenger (Tencent) |
| JVET-AQ0226 | m77928 | 2026-07-09 14:31:12 | 2026-07-09 14:32:37 | 2026-07-09 14:32:37 | Comparison of the Object Mask Information (OMI) and Picture Segmentation Information (PSI) SEI messages | A. M. Tourapis D. Podborski (Apple) |
| JVET-AQ0227 | m77935 | 2026-07-09 20:07:59 | 2026-07-09 20:18:31 | 2026-07-09 20:18:31 | [GSC][JEE.6.5] Optimization of the GSC metric software | J. Ricard G. Teniou S. Wenger (Tencent) |
| JVET-AQ0228 | m77953 | 2026-07-10 14:23:44 | 2026-07-10 14:32:10 | 2026-07-10 14:32:10 | AHG9: Proposed joint text on DSC SEI message extensions | M. M. Hannuksela J. Boyce (Nokia) I. Sodagar (Dolby) H. Tan J. Lee (LGE) M. Pettersson (Ericsson) |
| JVET-AQ0229 | m77961 | 2026-07-11 01:35:52 | 2026-07-11 02:15:53 | 2026-07-13 05:54:04 | AHG9: On the picture segmentation information SEI message â€” comments and proposed corrections (response to JVET-AQ0161) | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) A. M. Tourapis D. Podborski J. Kim (Apple) |
| JVET-AQ0230 | m77962 | 2026-07-11 03:15:10 | 2026-07-11 03:17:29 | 2026-07-11 03:17:29 | AHG9: A coding specification-agnostic layer and auxiliary-picture interface for VSEI SEI messages (response to JVET-AQ0076) | A.M. Tourapis D. Podborski J. Kim (Apple) |
| JVET-AQ0231 | m77969 | 2026-07-11 10:43:05 | 2026-07-11 10:44:14 | 2026-07-11 10:44:14 | Withdrawn |  |
| JVET-AQ0232 | m77970 | 2026-07-11 10:46:28 | 2026-07-11 10:47:00 | 2026-07-12 09:04:41 | AHG9: Crosscheck of JVET-AQ0069 Coordinate-remapping in triplane videos | B. Kroon (Philips) |
| JVET-AQ0233 | m77971 | 2026-07-11 10:49:27 | 2026-07-11 10:49:53 | 2026-07-11 10:49:53 | Withdrawn |  |
| JVET-AQ0234 | m77976 | 2026-07-11 16:31:34 | 2026-07-11 16:41:55 | 2026-07-11 16:41:55 | AHG9: Crosscheck for m77668/JVET-AQ0215 | Y. Xu K. Zhang L. Zhang (Bytedance) |
| JVET-AQ0235 | m77989 | 2026-07-12 13:52:22 | 2026-07-12 15:17:45 | 2026-07-12 15:17:45 | AHG9: Rotation representation extensions of Gaussian Splatting Information SEI message | Z. Zhu S. Liu L. Yu Y. Liao (Zhejiang University) |
| JVET-AQ0236 | m78011 | 2026-07-12 23:33:54 | 2026-07-12 23:44:46 | 2026-07-12 23:44:46 | Crosscheck of JEE 6.7 Test 11 Triplane video-based implicit representation compression JVET-AQ0068 and JVET-AQ0069 | P. Rondao Alface B. van Hauwermeiren (Nokia) |
| JVET-AQ0237 | m78012 | 2026-07-12 23:49:26 | 2026-07-12 23:56:09 | 2026-07-12 23:56:09 | [GSC] [JEE 6.7] Crosscheck of Test 10 Implicit intermediate representation-based compression JVET-AP0213 | P. Rondao Alface B. van Hauwermeiren (Nokia) |
| JVET-AQ0238 | m78020 | 2026-07-13 14:57:58 | 2026-07-13 15:15:53 | 2026-07-13 15:42:36 | AHG9: On software availability and CTC results | B. Kroon (Philips) G. Teniou (Tencent) J. Boyce (Nokia) L. Kerofsky (Qualcomm) |
| JVET-AQ0239 | m78024 | 2026-07-13 16:48:14 | 2026-07-13 16:53:53 | 2026-07-13 17:10:31 | AHG9: [GSC][JEE6.9] Scalable 3DGS Syntax in GSI SEI | R. Koo Y.-G. Kim E.-S. Ryu (SKKU) |
| JVET-AQ0240 | m78063 | 2026-07-15 09:55:59 | 2026-07-15 10:48:45 | 2026-07-15 10:48:45 | Status of SEI messages in the VSEI TuC | J. Boyce |
| JVET-AQ0267 | m77489 | 2026-07-01 14:33:02 | 2026-07-01 15:00:41 | 2026-07-01 15:00:41 | Withdrawn |  |
| JVET-AQ1000 | m78037 | 2026-07-14 12:37:14 |  |  | Meeting Report of the 43rd JVET Meeting | J.-R. Ohm |
| JVET-AQ1004 | m78038 | 2026-07-14 12:38:35 | 2026-08-06 01:13:48 | 2026-09-28 20:53:15 | Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP | Y.-K. Wang J. Boyce B. Bross I. Moccagatta C. Rosewarne K. Sühring G. J. Sullivan |
| JVET-AQ1006 | m78039 | 2026-07-14 12:39:18 | 2026-09-21 05:40:38 | 2026-09-21 05:41:45 | HEVC additional profiles (Draft 2) | A. Tourapis Y.-K. Wang B. Bross G. J. Sullivan |
| JVET-AQ1008 | m78040 | 2026-07-14 12:41:18 | 2026-07-31 11:13:53 | 2026-07-31 11:13:53 | Conformance testing for HEVC multiview extended and monochrome profiles (Draft 3) | I. Moccagatta T. Fu S. Paluri A. Tourapis |
| JVET-AQ1012 | m78041 | 2026-07-14 12:41:47 |  |  | Overview of IT systems used in JVET | J.-R. Ohm I. Moccagatta K. Sühring M. Wien |
| JVET-AQ1017 | m78081 | 2026-07-15 16:29:40 | 2026-08-12 04:40:44 | 2026-08-12 04:40:44 | Updates of HEVC reference software (draft 1) | K. Sühring F. Bossen A. Tourapis |
| JVET-AQ2006 | m78042 | 2026-07-14 12:44:05 | 2026-07-31 19:23:15 | 2026-09-28 23:39:45 | Additional SEI messages for VSEI version 5 (Draft 1) | Y.-K. Wang J. Boyce J. Chen S. Deshpande M. M. Hannuksela G. J. Sullivan H. Tan A. Tourapis |
| JVET-AQ2019 | m78043 | 2026-07-14 12:44:40 |  |  | Description of algorithms version 16 and software version 18 in neural network-based video coding (NNVC) | F. Galpin Yue Li Yun Li D. Rusanovskyy T. Shao J. Ström L. Wang |
| JVET-AQ2020 | m78044 | 2026-07-14 12:45:26 |  |  | Film grain synthesis technology for video applications ed. 2 (Draft 6) | W. Husak P. de Lagrange M. Radosavljević A. Tourapis |
| JVET-AQ2021 | m78045 | 2026-07-14 12:46:08 | 2026-07-15 12:49:06 | 2026-07-15 12:49:06 | Joint Call for Proposals on video compression with capability beyond VVC | J.-R. Ohm M. Wien F. Bossen |
| JVET-AQ2022 | m78046 | 2026-07-14 12:48:12 | 2026-07-17 20:57:07 | 2026-07-17 20:57:07 | Draft template for proposal description documents of responses to the Joint CfP on video compression with capability beyond VVC | J.-R. Ohm M. Wien E. Alshina F. Bossen R. Chernyak J. Gan P. de Lagrange X. Li |
| JVET-AQ2028 | m78047 | 2026-07-14 12:50:28 | 2026-09-17 18:30:04 | 2026-09-17 18:30:04 | Common test conditions on Gaussian splat coding | B. Kroon P. Rondao Alface J. Jung G. Sandri |
| JVET-AQ2032 | m78048 | 2026-07-14 12:52:02 | 2026-07-31 18:24:01 | 2026-08-29 00:29:51 | Technologies under consideration for future extensions of VSEI (version 13) | S. Deshpande J. Boyce J. Chen M. M. Hannuksela Y. He K. Sühring G. J. Sullivan H. Tan A. Tourapis J. Xu X. Xu |
| JVET-AQ2040 | m78082 | 2026-07-15 16:31:09 | 2026-09-17 18:54:54 | 2026-09-17 18:54:54 | Description of SEI-related exploration experiments on Gaussian splat coding | J. Jung W. Husak |

# Annex B1 to JVET report:
List of meeting participants attending in person

The participants who were personally present at the meeting site of the forty-third meeting of the JVET, according to badge pick-up and a sign-in sheet circulated in the JVET meeting rooms (approximately 200 people in total), were as follows:

- Mohsen Abdoli (Xiaomi – FR)

- Kiyofumi Abe (Panasonic – JP)

- Ali Ak (InterDigital – FR)

- Anique Akhtar (Qualcomm – US)

- Elena Alshina (Huawei – DE)

- Kenneth Andersson (Ericsson – SE)

- Suyong Bahk (Kyung Hee Univ. – KR)

- Hidenori Bai (Sharp – JP)

- Yaxian Bai (ZTE – CN)

- Gun Bang (ETRI – KR)

- Vittorio Baroncini (GBTech – IT)

- Daniel Becking (Fraunhofer HHI & IIS – DE)

- Aaron Berk (InterDigital – UK)

- Tomás Borges (Fraunhofer HHI – DE)

- Frank Bossen (Bossen Technologies – CA)

- Jill Boyce (Nokia – US)

- Jens Brandenburg (Fraunhofer HHI – DE)

- Benjamin Bross (Fraunhofer HHI – DE)

- Madhukar Budagavi (Samsung – US)

- Janko Calic (BBC – UK)

- Alessia Casella (Sisvel Tech – IT)

- Jie Chen (Alibaba – CN)

- Lulin Chen MediaTek – US)

- Yi-Hsin Chen (MediaTek – US)

- Zhenzhong Chen (Wuhan Univ. – CN)

- Roman Chernyak (Tencent – US)

- Hyundong Cho (Kyung Hee Univ. – KR)

- Kiho Choi (Kyung Hee Univ. – KR)

- Woong Il Choi (Samsung – KR)

- Hyon-Gon Choo (ETRI – KR)

- Olena Chubach (MediaTek – US)

- Takeshi Chujoh (KDDI – JP)

- Sebastjan Cizel (InterDigital – FR)

- Tim Claßen (RWTH Aachen – DE)

- Philippe de Lagrange (InterDigital – FR)

- Claire-Hélène Demarty (InterDigital – FR)

- Sachin Deshpande (Sharp – US)

- Jihoon Do (ETRI – KR)

- Virginie Drugeon (Panasonic – DE)

- Alberto Duenas (Warner Bros. Discovery – US)

- Stanislav Efremov (LG Electronics – RU)

- Semih Esenlik (Qualcomm – US)

- Simone Ferrara (V-Nova – UK)

- Alexey Filippov (TCL – CN)

- Siegfried Fößel (Fraunhofer IIS – DE)

- Denis Fortin (Nokia – FR)

- Edouard François (InterDigital – FR)

- Per Fröjdh (Ericsson – SE)

- Benjamin Galmiche (Canon Inc. – JP)

- Franck Galpin (InterDigital – FR)

- Jonathan Gan (OPPO – AU)

- Patrick Garus (Panasonic – DE)

- Xiao Jun Gu (Huawei – CN)

- Miska Matias Hannuksela (Nokia – FI)

- Sabine Harribey (Ericsson – SE)

- Xuan He (Nokia – FR)

- Yong He (Qualcomm – US)

- Viktor Herrmann (Fraunhofer HHI & IIS – DE)

- Arianne Hinds (Tencent – US)

- Myungoh Hong (LG Electronics – KR)

- Sean Huang (Acer – US)

- Yu-Wen Huang (MediaTek – US)

- Yongkai Huo (Transsion – CN)

- Walt Husak (Dolby Labs – US)

- Kenta Iida (NEC – JP)

- Tomohiro Ikai Sharp – JP)

- Sergey Ikonin (Huawei – CN)

- Takaaki Ishikawa (Canon – JP)

- Shunsuke Iwamura (NHK – JP)

- Byeungwoo Jeon (SKKU – KR)

- Hyunki Jeong (SKKU – KR)

- Joël Jung (Qualcomm – US)

- Alexander Karabutov (Huawei – DE)

- Marta Karczewicz (Qualcomm – US)

- Kei Kawamura (KDDI – JP)

- Joachim Keinert (Fraunhofer IIS – DE)

- Louie Kerofsky (Qualcomm – US)

- Aro Kim (TTA – KR)

- Chul Keun Kim (LG Electronics – KR)

- Eunji Kim (Kyungpook Nat. Univ. – KR)

- Jae-Gon Kim (Korea Aerosp. Univ. – KR)

- Kyu Ri Kim (Korea Aerosp. Univ. – KR)

- Minsub Kim (Digitalinsights – KR)

- Seung-Hwan Kim (LG Electronics – US)

- Yongseong Kim (Kyung Hee Univ. – KR)

- Kruthika Koratti Sivakumar (Google – US)

- Philipp Kreowsky (HHI – DE)

- Jani Lainema Nokia – FI)

- Olivier Le Meur InterDigital – FR)

- Brian Lee (Dolby Labs – US)

- Hahyun Lee (ETRI – KR)

- Minhun Lee (Kwangwoon Univ. – KR)

- Tae Hwa Lee (Kyung Hee Univ. – KR)

- Yen-Wei Lee (Acer America – US)

- Christian Lehmann (Fraunhofer HHI & IIS – DE)

- Shawmin Lei (MediaTek – US)

- Ming Li (OPPO – CN)

- Xiang Li (Google – US)

- Yiming Li (Tencent – CN)

- Kai-Wen Liang (Sharp – JP)

- Jaehyun Lim (LG Electronics – KR)

- Seong Yong Lim (ETRI – KR)

- Sung-Won Lim (KT – KR)

- Youngkwon Lim (Samsung – KR)

- Ching-Chieh Lin (ITRI – US)

- Lukasz Litwic (Ericsson – SE)

- Du Liu (Ericsson – SE)

- Jingyun Liu (Wuhan Univ. – CN)

- Po-Kai Liu (ITRI – US)

- Xiang Ma (Huawei – CN)

- Detlev Marpe (Fraunhofer HHI – DE)

- Sujeet Mate (Nokia – FI)

- Yasutomo Matsuba (Qualcomm – US)

- Dominik Mehlem (RWTH Aachen – DE)

- Ulf Mertens (Panasonic – DE)

- Mattia Mondino (Sisvel – IT)

- Gihwa Moon (KAU – KR)

- Ruslan Mullakhmetov (InterDigital – UK)

- Ohji Nakagami (Sony – JP)

- Junghak Nam (LG Electronics – KR)

- Nicolas Neumann (Nokia – DE)

- Tung Nguyen (Fraunhofer HHI – DE)

- Jens-Rainer Ohm (RWTH Aachen – DE)

- Jörn Ostermann (Univ. Hannover – DE)

- Nicolai Otto (Main Concept – DE)

- Naël Ouedraogo (Canon – FR)

- Johan Pardo (Huawei – DE)

- Jae Seung Park (Sejong Univ. – KR)

- Junhwan Park (TTA – KR)

- Min Woo Park (Samsung Electronics – KR)

- Sang-Hyo Park (Kyungpook Nat. Univ. – KR)

- Youngo Park (Samsung Electronics – KR)

- Martin Pettersson (Ericsson – SE)

- Jonathan Pfaff (Fraunhofer HHI – DE)

- Pierrick Philippe (Orange – FR)

- Yinji Piao (Samsung – KR)

- Sophie Pientka (HHI – DE)

- Marius Preda (Inst. Mines-Télécom – FR)

- Sophie Puttkammer (Fraunhofer HHI – DE)

- Thomas Richter (Fraunhofer IIS – DE)

- Justin Ridge (Nokia – FI)

- Christopher Rosewarne (Canon – AU)

- Vasily Rufitskiy (TCL – CN)

- Dmytro Rusanovskyy (Nokia – US)

- Jonatan Samuelsson-Allendes (Sharp – US)

- Yago Sanchez de la Fuente (Fraunhofer HHI – DE)

- Maria Santamaria (Nokia – FI)

- Mohammed Sarwer (Google – US)

- Simon Sasse (Fraunhofer HHI – DE)

- Thomas Schierl (Fraunhofer HHI – DE)

- Heiko Schwarz (Fraunhofer HHI – DE)

- Sebastian Schwarz (Nokia – DE)

- Yeon-Joo Shim (Korea Telecom – KR)

- Masato Shima (Canon – JP)

- Donggyu Sim (Kwangwoon Univ. – KR)

- Robert Skupin (Fraunhofer HHI – DE)

- Iraj Sodagar (Dolby – US)

- Christoffer Söderberg (Ericsson – SE)

- Timofey Solovyev (Huawei – DE)

- Jianwen Song (Canon – AU)

- Jacob Ström (Ericsson – SE)

- Toshiyasu Sugio (Panasonic – JP)

- Gary J. Sullivan (Dolby – US)

- Teruhiko Suzuki (Sony – JP)

- Niclas Svensson (Ericsson – SE)

- Hendry Tan (LG Electronics – US)

- Gilles Teniou (Tencent – US)

- Han Boon Teo (Panasonic – SG)

- Emmanuel Thomas (Xiaomi – NL)

- Alexandros Tourapis (Apple – US)

- Kyohei Unno (NHK – JP)

- Aditya Vishwanath (Panasonic – SG)

- Dong Wang (OPPO – CN)

- Fan Wang (OPPO – CN)

- Sheng-Po Wang (ITRI – US)

- Xianglin Wang (Kwai – US)

- Xinhui Wang (ZTE – CN)

- Ye-Kui Wang (Bytedance – US)

- Stephan Wenger (Tencent – US)

- Adam Wieckowski (Fraunhofer HHI – DE)

- Thomas Wiegand (Fraunhofer HHI – DE)

- Mathias Wien (RWTH Aachen – DE)

- Shaowei Xie (ZTE – CN)

- Zhihuang Xie (OPPO – CN)

- Xiaoyu Xiu (Kwai – US)

- Jizheng Xu (Bytedance – US)

- Luhang Xu (OPPO – CN)

- Xiaozhong Xu (Tencent – US)

- Toru Yamada (NEC – JP)

- Yan Ye (Alibaba – US)

- Peng Yin (Dolby Labs – US)

- Haoping Yu (OPPO – US)

- Lu Yu (Zhejiang Univ. – CN)

- Yue Yu (OPPO – US)

- Hui Yuan (OPPO – CN)

- Vladyslav Zakharchenko (Nokia – US)

- Wenhao Zhang (Disney – CN)

- Zhuhai Zhang (Transsion – CN)

- Shuai Zhao (Qualcomm – US)

- Dan Zou (OPPO – CN)

# Annex B2 to JVET report:
List of meeting participants attending remotely

The remote participants of the forty-third meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately 286 people in total, not including those who had attended the meeting in person at least part-time (see Annex B1), and not including those who attended only the joint sessions with other groups), were as follows:

- Mouad Aderdor (Huawei – DE)

- Waqas Ahmad (Ericsson – SE)

- Byungjun Ahn (KWU – KR)

- Yongjo Ahn (Qualcomm – US)

- Jukka Ahonen (Nokia – FI)

- Alireza Aminlou (Nokia – FI)

- David Appelgren (Ericsson – SE)

- Jeeva Raj Arumugam (Ittiam – IN)

- Pekka Astola (Nokia – FI)

- Shushik Avagyan (Qualcomm – US)

- Tae Meon Bae (Sharp – US)

- Junho Bang (HNU – KR)

- Stefano Battista (BSOFT – IT)

- Florian Beye (NEC – JP)

- Nisha Bhaskar (Huawei – DE)

- Thibaud Biatek (Nokia – FR)

- Saverio Blasi (Nokia – UK)

- Médéric Blestel (Ofinno – US)

- Atanas Boev (Huawei – DE)

- Philippe Bordes (InterDigital – FR)

- Angelo Bruccoleri (RAI – IT)

- Serdar Caglar (TUM – DE)

- Jiyuan Cai (Transsion – CN)

- Eric Chai (Ubilinx – US)

- Aasaipriya Chandran (Huawei – DE)

- Renjie Chang (Tencent – CN)

- Yao-Jen Chang (Qualcomm – US)

- Ching-Yeh Chen (MediaTek – US)

- Chingyeh Chen (MediaTek – US)

- Chun-Chi Chen (Qualcomm – US)

- Hong-Hui Chen (MediaTek – US)

- Lien-Fei Chen (Tencent – US)

- Minzhe Chen (OPPO – CN)

- Wei Chen (Kwai – US)

- Xinxin Chen (WHU – CN)

- Yi-Wen Chen (MediaTek – US)

- You-Qi Chen (ITRI – US)

- Zongxie Chen (TUM – DE)

- Man-Shu Chiang (MediaTek – US)

- Injoon Cho (KBS – KR)

- Hyejeong Choi (KWU – KR)

- Jangwon Choi (Hyundai – KR)

- Jung-Ah Choi (Hyundai – KR)

- Keiichi Chono (NEC – JP)

- Peter Chuang (MediaTek – US)

- Tzu-Der Chuang (MediaTek – US)

- Muhammed Coban (Qualcomm – US)

- Damian Ruiz Coll (Ofinno – US)

- Francesco Cricri (Nokia – FI)

- Kai Cui (Qualcomm – US)

- Han Chung Dean (Panasonic – SG)

- Zhipin Deng (Bytedance – CN)

- Keqin Ding (TCL – CN)

- Tianyu Dong (TCL – CN)

- Huan Dou (BBC – UK)

- Didier Doyen (InterDigital – FR)

- Thierry Dumas (InterDigital – FR)

- Zheming Fan (Sharp – JP)

- Yulu Fei (OPPO – CN)

- Christian Feldmann (Nokia – DE)

- Chris Finlay (InterDigital – CA)

- Alice Foster (BBC – UK)

- Nianxiang Fu (WHU – CN)

- John Funnell (Nokia – UK)

- Neeraj Gadgil (Samsung – IN)

- Jingying Gao (Panasonic – SG)

- Shuo Gao (Xiaomi – CN)

- Ying Gao (ZTE – CN)

- Ivan Gribushin (Huawei – RU)

- Vladimir Gritsenko (Ericsson – SE)

- Dan Grois (AnyAI – IL)

- Tiansheng Guo (Huawei – DE)

- Woowoen Gwun (Qualcomm – US)

- Jaemin Ha (SJU – KR)

- Sangjin Hahm (KBS – KR)

- Chung Dean Han (Panasonic – SG)

- Shane He (Nokia – FR)

- Jin Heo (Hyundai – KR)

- Christopher Hollmann (TCL – CN)

- Donghyun Hong (KWU – KR)

- Jae-Woo Hong (SJU – KR)

- Seungwook Hong (Nokia – US)

- Sujun Hong (Sharp – JP)

- Reetu Hooda (Qualcomm – US)

- Shih-Ta Hsiang (MediaTek – US)

- Chih-Wei Hsu (Mediatek – US)

- Nan Hu (Qualcomm – US)

- Han Huang (Tencent – US)

- Hang Huang (OPPO – CN)

- Roberto Iacoviello (RAI – IT)

- Atsuro Ichigaya (NHK – JP)

- Noritaka Iguchi (Panasonic – JP)

- Masaru Ikeda (Sony – JP)

- Ruri Imichi (Sharp – JP)

- Ryo Ishimoto (Sharp – JP)

- Russell James (InterDigital – UK)

- Marine Jeon (KHU – KR)

- Sanghun Jeon (LGE – KR)

- Hong-Jheng Jhu (Kwai – US)

- Lina Ji (ZTE – CN)

- Menghu Jia (ZTE – CN)

- Xiaochun Jiang (PKU – CN)

- Daeyeon Ju (KWU – KR)

- Mao Jue (Huawei – CN)

- Ikram Jumakulyyev (Nokia – DE)

- Yunsik Jung (LGE – KR)

- Kashyap Kammachi Sreedhar (Nokia – FI)

- Jewon Kang (Ewha W. Univ. – KR)

- Jungwon Kang (ETRI – KR)

- Birendra Kathariya (Dolby – US)

- Vyacheslav Khamidullin (Huawei – RU)

- Yoshitaka Kidani (KDDI – JP)

- Dongha Kim (KAU – KR)

- Donghyun Kim (ETRI – KR)

- Jaeil Kim (SKT – KR)

- Jiyoung Kim (WILUS – KR)

- Jong Wook Kim (KHU – KR)

- Jongho Kim (ETRI – KR)

- Jungsun Kim (Apple – US)

- Kyungyong Kim (Wilus – KR)

- Mintae Kim (KWU – KR)

- Yeeun Kim (KWU – KR)

- Yongheon Kim (HNU – KR)

- Youchan Kim (KWU – KR)

- Youngwook Kim (KWU – KR)

- Konstantinos Konstantinides (Dolby Labs – US)

- Moonmo Koo (LGE – KR)

- Fatima Kozhamkulova (Huawei – DE)

- Bowen Ku (MediaTek – US)

- Po-Wen Ku (MediaTek – US)

- Gosala Kulupana (Nokia – UK)

- Che-Wei Kuo (Kwai – US)

- Toshihiko Kusakabe (Panasonic – JP)

- Hyukmin Kwon (HYU – KR)

- Naseong Kwon (KWU – KR)

- Guillaume Laroche (Canon – FR)

- Nam Le (Nokia – FI)

- Jangwon Lee (LGE – KR)

- Jeonghyun Lee (KHU – KR)

- Jinho Lee (ETRI – KR)

- Jongmin Lee (KAU – KR)

- Jung-Kyung Lee (Ofinno – US)

- Louis Lee (Acer – US)

- Soeun Lee (HNU – KR)

- Sunyoung Lee (Atins – KR)

- Julien Lémotheux (Orange – FR)

- Bin Li (Hisense – CN)

- Dan Li (TCL – CN)

- Xinwei Li (Alibaba – CN)

- Yue Li (SJTU – CN)

- Yun Li (Qualcomm – US)

- Zhengang Li (ZTE – CN)

- Zishan Li (Huawei – DE)

- Shen Yi Lim (Panasonic – SG)

- Eugene Lim (SJU – KR)

- Sung-Chang Lim (ETRI – KR)

- Wang-Q Lim (HHI – DE)

- Kai Lin (Bytedance – CN)

- Po-Han Lin (Qualcomm – US)

- Ao Liu (Huawei – CN)

- Cheng-Yun Liu (ITRI – US)

- Peter Liu (AMD – CA)

- Shan Liu (Tencent – US)

- Yutian Liu (Transsion – CN)

- Zhikai Liu (OPPO – CN)

- Muhammad Asad Lodhi (Qualcomm – US)

- Fedor Loginov (Huawei – RU)

- Riyu Lu (Tencent – CN)

- Ajay Luthra (Picsel Labs – US)

- Zhuoyi Lyu (vivo – CN)

- Fabrice Le Léannec (InterDigital – FR)

- Changyue Ma (Kwai – US)

- Wenzhuo Ma (WHU – CN)

- Yanzhuo Ma (Xidian University – CN)

- Kirill Malyshev (Huawei – RU)

- Xuewei Meng (Qualcomm – US)

- Philipp Merkle (HHI – DE)

- Akira Minezawa (Mitsubishi – JP)

- Iole Moccagatta (Intel – US)

- Luka Murn (Nokia – UK)

- Hiroya Nakamura (JVCKENWOOD – JP)

- Shimpei Nemoto (NHK – JP)

- Canh Nguyen (Dolby – US)

- Eric Nguyen (Canon – JP)

- Weihong Niu (ZTE – CN)

- Seungmin Noh (HNU – KR)

- Patrice Onno (Canon – FR)

- Seethal Paluri (Apple – US)

- Krit Panusopone (Nokia – US)

- Naeri Park (LGE – KR)

- Seori Park (LGE – KR)

- Seungwook Park (HMC – KR)

- Dimitri Podborski (Apple – US)

- Srivatsa Prativadibhayankaram (Fraunhofer IIS – DE)

- Saurabh Puri (InterDigital – CA)

- Hongdong Qin (TCL – CN)

- Mohammed Raad (LIU – LB)

- Miloš Radosavljević (InterDigital – FR)

- Adarsh Ramasubramonian (Qualcomm – US)

- Julien Ricard (Tencent – US)

- Marc Rivière (Ateme – FR)

- Valentina Rizzello (Ericsson – SE)

- Antoine Robert (InterDigital – FR)

- Mehdi Salehifar (Bytedance – US)

- Döne Bugdayci Sansli (Nokia – FI)

- Jan Schneider (RWTH – DE)

- Vadim Seregin (Qualcomm – US)

- Tong Shao (Dolby – US)

- Vladislav Shchukin (Qualcomm – US)

- Jay Shingala (Ittiam – IN)

- Tian Shu (WHU – CN)

- Ajay Shyam (Ittiam – IN)

- Ahmed Sidiya (Sharp – US)

- Praneet Singh (Dolby – US)

- Nan Song (OPPO – CN)

- Noah Stegmaier (Ericsson – SE)

- Jan Studeny (Ericsson – SE)

- Karsten Sühring (HHI – DE)

- Zexing Sun (OPPO – CN)

- Maxim Sychev (Huawei – RU)

- Keiichirō Takada (Sharp – JP)

- Hendry Tan (LGE – US)

- Hamed Rezazadegan Tavakoli (Nokia – FI)

- Yew Wei Teh (Panasonic – SG)

- Sergio Tejeda (HHI – DE)

- Chih-Yu Teng (Sharp – JP)

- Herbert Thoma (Fraunhofer IIS – DE)

- Alexandre Tissier (Xiaomi – FR)

- Chia-Ming Tsai (MediaTek – US)

- Hsin-Yi Tseng (MediaTek – US)

- Renan Utida (InterDigital – CA)

- Gleb Verba (Qualcomm – US)

- Biao Wang (Tencent – US)

- Chen Wang (OPPO – CN)

- Guiqi Wang (vivo – CN)

- Hongtao Wang (Qualcomm – US)

- Huifen Wang (China Telecom – CN)

- Limin Wang (Nokia – US)

- Liqiang Wang (Tencent – CN)

- Qimeng Wang (HUST – CN)

- Shengjuan Wang (ZTE – CN)

- Yifan Wang (Tencent – US)

- Yifei Wang (WHU – CN)

- Yingbin Wang (Tencent – CN)

- Kai Wen (Sharp – JP)

- Martin Winken (HHI – DE)

- Eugene Won (Ewha – KR)

- Muxuan Wu (HiSilicon – CN)

- Ping Wu (ZTE – UK)

- Ziyue Xiang (Tencent – US)

- Yingran Xiao (MLS – CN)

- Alisa Xing (Hisense – CN)

- Jianfeng Xu (KDDI – JP)

- Yingzhan Xu (Bytedance – CN)

- Zhuowei Xu (TCL – CN)

- Yoichi Yagasaki (Sony – JP)

- Ning Yan (Bytedance – CN)

- Qi Yang (Bytedance – US)

- Ruiying Yang (Nokia – FI)

- Zheyuan Yang (Ericsson – SE)

- Emma Ye (Qualcomm – US)

- Jeeye Yoon (LGE – KR)

- Yong-Uk Yoon (Tencent – US)

- Hongjie You (Huawei – DE)

- Ramin Youvalari (Xiaomi – FI)

- Hualong Yu (ZJU – CN)

- Ruoyang Yu (Qualcomm – US)

- Nishikawa Yuma (Panasonic – JP)

- Weimin Zeng (Ubilinx – US)

- Honglei Zhang (Nokia – FI)

- Hongli Zhang (OPPO – CN)

- Junxi Zhang (WHU – CN)

- Lai Zhang (OPPO – CN)

- Li Zhang (Bytedance – US)

- Qian Zhang (BOE – CN)

- Yan Zhang (Qualcomm – US)

- Zhi Zhang (Qualcomm – US)

- Jane Zhao (LGE – US)

- Yin Zhao (Huawei – CN)

- Chuan Zhou (vivo – CN)

- Minhua Zhou (Broadcom – US)

- Han Zhu (Tencent – CN)

- Weijia Zhu (TCL – CN)

- Nannan Zou (Nokia – FI)

- Naima Zouidi (Ofinno – US)

- Ivan Zupancic (TCL – CN)

# Annex C to JVET report:
Recommendations of the 24th meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Experts Team with ITU-T SG21

ISO/IEC JTC 1/SC 29/WG 5 N 407

## Recommendations of the 24th WG 5 meeting

## 1. Reports

### 1.1 Meeting Reports

### 1.1.1 WG 5 approves

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
| 396 | Report of the 23rd JTC 1/SC 29/WG 5 meeting | Jens-Rainer Ohm | N | 2026-05-29 | 26278 |

## 2. MPEG-C (ISO/IEC 23002 - MPEG video technologies)

### 2.1 Part 9 - Film grain synthesis technology for video applications

### 2.1.1 WG 5 approves the following document

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23002-9 - Film grain synthesis technology for video applications |  |  |  |  |
| 409 | Text of ISO/IEC CDTR 23002-9 Film grain synthesis technology for video applications (2nd ed.) | Walt Husak | N | 2026-07-31 | 26488 |

| ### 2.1.2 |  | ### WG 5 requests not to use the OSD tool for preparation of the text of ISO/IEC 23002-9:202x (2nd ed.) Film grain synthesis technology for video applications, as this is a collaborative project with ITU-T SG21 that requires coordination between the organizations to ensure alignment of the content. |
| --- | --- | --- |

## 3. MPEG-H (ISO/IEC 23008 - High efficiency coding and media delivery in heterogeneous environments)

### 3.1 Part 2 - High efficiency video coding

### 3.1.1 WG 5 approves the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23008-2 - High efficiency video coding |  |  |  |  |
| 410 | Request for ISO/IEC 23008-2:202x/Amd.1 | Ye-Kui Wang | N | 2026-07-17 | 26489 |
| 411 | Text of ISO/IEC 23008-2:202x CDAM1 Additional profiles and improved VSEI interfaces | Alexandros Tourapis | N | 2026-07-31 | 26491 |

| ### 3.1.2 |  | ### WG 5 requests not to use the OSD tool for preparation of the text of ISO/IEC 23008-2:202x/Amd.1, as this is a collaborative project with ITU-T SG21 that requires coordination between the organizations to ensure alignment of the content. |
| --- | --- | --- |

| ### 3.1.3 |  | ### WG 5 notes that ISO/IEC 23008-2:200x/Amd.1 is joint work with ITU-T, planned to be issued as twin text in a new version of ITU-T H.265. |
| --- | --- | --- |

### 3.2 Part 5 - Reference software for high efficiency video coding

### 3.2.1 WG 5 approves the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23008-5 - Reference software for high efficiency video coding |  |  |  |  |
| 412 | Request for ISO/IEC 23008-5:202x Reference software for HEVC (3rd edition) | Frank Bossen | N | 2026-07-17 | 26492 |
| 413 | Text of ISO/IEC CD 23008-5:202x Reference software for HEVC (3rd edition) | Karsten Sühring | N | 2026-07-31 | 26493 |

| ### 3.2.2 |  | ### WG 5 requests not to use the OSD tool for preparation of the text of ISO/IEC 23008-5:202x (3rd ed.) Reference software for HEVC, as this is a collaborative project with ITU-T SG21 that requires coordination between the organizations to ensure alignment of the content. |
| --- | --- | --- |

| ### 3.2.3 |  | ### WG 5 notes that ISO/IEC 23008-5:200x (3rd ed.) is joint work with ITU-T, planned to be issued as twin text in a new version of ITU-T H.265.2. |
| --- | --- | --- |

### 3.3 Part 8 - Conformance specification for HEVC

### 3.3.1 WG 5 approves the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23008-8 - Conformance specification for HEVC |  |  |  |  |
| 414 | Request for ISO/IEC 23008-8:202x Conformance specification for HEVC (3rd edition) | Iole Moccagatta | N | 2026-07-17 | 26494 |
| 415 | Text of ISO/IEC CD 23008-8:202x Conformance specification for HEVC (3rd edition) | Iole Moccagatta | N | 2026-07-31 | 26495 |

| ### 3.3.2 |  | ### WG 5 requests not to use the OSD tool for preparation of the text of ISO/IEC 23008-8:202x (3rd ed.) Conformance specification for HEVC, as this is a collaborative project with ITU-T SG21 that requires coordination between the organizations to ensure alignment of the content. |
| --- | --- | --- |

| ### 3.3.3 |  | ### WG 5 notes that ISO/IEC 23008-8:200x (3rd ed.) is joint work with ITU-T, planned to be issued as twin text in a new version of ITU-T H.265.1. |
| --- | --- | --- |

## 4. Explorations

### 4.1 Enhanced compression beyond VVC capability

### 4.1.1 WG 5 approves the following document

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | Explorations |  |  |  |  |
| 416 | Joint Call for Proposals on video compression with capability beyond VVC | Mathias Wien | Y | 2026-07-15 | 26496 |

### 4.2 Gaussian splat coding

### 4.2.1 WG 5 recommends approval of the following documents, which have been developed jointly with WG 4 and WG 7

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | Explorations |  |  |  |  |
| 420 | Common test conditions for Gaussian splat coding | Bart Kroon | N | 2026-08-14 | 26500 |
| 421 | Draft CfP of Gaussian splat coding | Ralf Schaefer | N | 2026-07-31 | 26654 |
| 422 | Call for content for Gaussian splat coding | Ralf Schaefer | Y | 2026-07-31 | 26655 |
| 423 | FAQ on Gaussian splat coding | Euee Jang | Y | 2026-09-11 | 26656 |
| 424 | JEE 6.1 on data preparation | Patrice Rondao Alface | N | 2026-08-14 | 26657 |
| 425 | JEE 6.2 on anchor generation | Gustavo Sandri | N | 2026-08-14 | 26658 |
| 426 | JEE 6.3 on new coding technologies | Gun Bang | N | 2026-08-14 | 26659 |
| 427 | JEE 6.4 on collecting requirements and use cases for Gaussian splat coding | Patrice Rondao Alface | N | 2026-08-14 | 26660 |
| 428 | JEE 6.5 on 3DGS software | Julien Ricard | N | 2026-08-14 | 26661 |
| 429 | JEE 6.6 on coordinating the 1F-geo Track | Hyejung Hur | N | 2026-08-14 | 26662 |
| 430 | JEE 6.7 on video-based coding technologies | Yiyi Liao | N | 2026-08-14 | 26663 |
| 431 | JEE 6.8 on Gaussian Splat Coding evaluation and CTC | Anthony Trioux | N | 2026-08-14 | 26664 |
| 432 | JEE 6.9 on evaluation of encapsulation methods for Gaussian Splats | Joel Jung | N | 2026-08-14 | 26665 |

## 5. Management

### 5.1 Collaboration with ITU-T

| ### 5.1.1 |  | ### WG 5 approves the following documents related to technology studies conducted by JVET |
| --- | --- | --- |

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | General |  |  |  |  |
| 417 | List of output documents from the 43rd JVET meeting | Jens-Rainer Ohm | N | 2026-07-17 | 26497 |

| ### 5.1.2 |  | ### The JVET chair proposes to hold the 44th JVET meeting during 17 – 23 October 2026 (with document deadline 9 October 2026) under ISO/IEC JTC 1/SC 29 auspices in Hangzhou, CN. Subsequent meetings are planned to be held during 13 – 22 January 2027 under ISO/IEC JTC 1/SC 29 auspices in Brisbane, AU; during 17 - 28 April 2027 under ITU-T SG21 auspices in Geneva, CH; during 7 – 16 July 2027 under ISO/IEC JTC 1/SC 29 auspices in Tampere, FI; during 20 – 29 October 2027 under ISO/IEC JTC 1/SC 29 auspices in Shenzhen, CN; during 12 – 21 January 2028 under ISO/IEC JTC 1/SC 29 auspices in Thessaloniki, GR; during 25 March – 5 April 2028 under ITU-T SG21 auspices in Geneva, CH; during 19 – 28 July 2028 under ISO/IEC JTC 1/SC 29 auspices in Fukuoka, JP; during October 2028 under ISO/IEC JTC 1/SC 29 auspices, date and location t.b.d.; and during January 2029 under ITU-T SG21 auspices, date and location t.b.d. |
| --- | --- | --- |

### 5.2 Ad hoc groups

### 5.2.1 WG 5 approves the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | Ad hoc groups |  |  |  |  |
| 418 | List of AHGs established at the 24th WG 5 meeting | Jens-Rainer Ohm | N | 2026-07-17 | 26498 |
| 419 | Joint AHG on Gaussian splat coding | Jens-Rainer Ohm | N | 2026-07-17 | 26499 |

### 5.2.2 WG 5 notes that the mandates of the joint AHG were agreed jointly with WG 4 and WG 7

### 5.3 Expression of Thanks

| ### 5.3.1 |  | ### JVET thanks Marius Preda for his excellent maintenance and continuous improvements of the document site jvet-experts.org. Institut Mines-Télécom is thanked for providing resources to host the sites. |
| --- | --- | --- |
| ### 5.3.2 |  | ### JVET thanks ITU for the excellent hosting of the 43rd JVET meeting in Geneva. Stefano Polidori and Hiba Tahawi are thanked for the help in preparation, registration and organisation of the meeting. Gent Bajrami and Marc Antoine Zanou are thanked for their responsiveness, dedication and daily help in the setup of the meeting and technical facilities. |

## The meeting was closed at 1400 CEST on 2026-07-15.