| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 44th Meeting, Hangzhou, CN, 17–23 October 2026 | Document: JVET-AR_notes_d0 |
| --- | --- |

| Title: | Meeting Report of the 44th Meeting of the Joint Video Experts Team (JVET), Hangzhou, CN, 17–23 October 2026 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contacts): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its forty-fourth meeting during 17 – 23 October 2026 at the Hangzhou Blossom Water Museum Hotel, Hangzhou, China. The meeting was held as a physical meeting with remote participation, provided on best-effort basis for experts who were unable to travel.

For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the twenty-fifth meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.16 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and preparations towards future standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 0900 CST (0100 UTC) on Saturday 17 October 2026. Meeting sessions were held on all days including the weekend days of Saturday and Sunday 17 and 18 October 2026, until the meeting was closed at approximately XXXX hours CST on Friday 23 October 2026. Approximately XXX people attended the JVET meeting (XXX in person and XXX remotely), and approximately XXX input documents (not counting crosschecks, reports, and summary documents), 17 JVET AHG reports, 1 report of the Joint AHG on Gaussian splat coding, one summary report of the Joint EE on Gaussian splat coding, X BoG report(s), and X incoming liaison document(s) were discussed. The meeting took place in coordination with a meeting of various SC29 Working Groups and Advisory Groups – where WG 5 is representing the Joint Video Coding Team(s) and their activities from the perspective of the SC 29 parent body, under whose auspices this JVET meeting was held. The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the joint EE on Gaussian splat coding, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development. In the latter context, progressing the Joint Call for Proposals on video compression with capability beyond VVC and of related documents for preparation of submissions was a major activity at his meeting.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the forty-third JVET meeting in producing the following documents:

a) JVET documents

JVET-AQ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AQ1006 HEVC additional profiles (Draft 1), also submitted for ballot as WG 5 CDAM N 411

JVET-AQ1008 Conformance testing for HEVC multiview extended and monochrome profiles (Draft 3)

JVET-AQ1012 Overview of IT systems used in JVET

JVET-AQ1017 Updates of HEVC reference software (draft 1)

JVET-AQ2006 Additional SEI messages for VSEI version 5 (Draft 1)

JVET-AQ2019 Description of algorithms version 16 and software version 18 in neural network-based video coding (NNVC)

JVET-AQ2020 Film grain synthesis technology for video applications ed. 2 (Draft 6), also submitted for ballot as WG 5 CDTR N 409

JVET-AQ2021 Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 415 and SG21 TD348/Plen

JVET-AQ2022 Draft template for proposal description documents of responses to the Joint CfP on video compression with capability beyond VVC

JVET-AQ2028 Common test conditions on Gaussian splat coding, also issued as WG 5 N 420

JVET-AQ2032 Technologies under consideration for future extensions of VSEI (version 13)

JVET-AQ2040 Description of SEI-related exploration experiments on Gaussian splat coding (Excerpt from WG 5 N 430 and N 432)

b) documents produced as WG 5 documents only:

WG 5 N 410 Request for ISO/IEC 23008-2:202x/Amd.1 Additional HEVC profiles

WG 5 N 412 Request for ISO/IEC 23008-5:202x Reference software for HEVC (3rd edition)

WG 5 N 413 Text of ISO/IEC CD 23008-5:202x Reference software for HEVC (3rd edition)

WG 5 N 414 Request for ISO/IEC 23008-8:202x Conformance specification for HEVC (3rd edition)

WG 5 N 415 Text of ISO/IEC 23008-8:202x Conformance specification for HEVC (3rd edition)

WG 5 N 421 Draft CfP of Gaussian splat coding

WG 5 N 422 Call for content for Gaussian splat coding

WG 5 N 423 FAQ on Gaussian splat coding

WG 5 N 424 JEE 6.1 on data preparation

WG 5 N 425 JEE 6.2 on anchor generation

WG 5 N 426 JEE 6.3 on new coding technologies

WG 5 N 427 JEE 6.4 on collecting requirements and use cases for Gaussian splat coding

WG 5 N 428 JEE 6.5 on 3DGS software

WG 5 N 429 JEE 6.6 on coordinating the 1F-geo Track

WG 5 N 430 JEE 6.7 on video-based coding technologies

WG 5 N 431 JEE 6.8 on Gaussian Splat Coding evaluation and CTC

WG 5 N 432 JEE 6.9 on evaluation of encapsulation methods for Gaussian Splats

- c) documents produced for handling by ITU-T:

SG21 TD326/Plen H-Series Supplement 22 (ex. H.Sup-MACVC) Optimization of encoders and receiving systems for machine analysis of coded video content

Liaison to JPEG on JPEG AI and explorations on video coding, included as item 1 in SG21 TD354/WP3

Liaison to 3GPP SA4 on HEVC multi-layer profiles, chroma subsampling and bit depth combinations, included as item 2 in SG21 TD354/WP3

As main results, the JVET produced XX output documents from the current meeting (update):

JVET-AQ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AQ1006 HEVC additional profiles (Draft 1), also submitted for ballot as WG 5 CDAM N 411

JVET-AQ1008 Conformance testing for HEVC multiview extended and monochrome profiles (Draft 3)

JVET-AQ1012 Overview of IT systems used in JVET

JVET-AQ1017 Updates of HEVC reference software (draft 1)

JVET-AQ2006 Additional SEI messages for VSEI version 5 (Draft 1)

JVET-AQ2019 Description of algorithms version 16 and software version 18 in neural network-based video coding (NNVC)

JVET-AQ2020 Film grain synthesis technology for video applications ed. 2 (Draft 6), also submitted for ballot as WG 5 CDTR N 409

JVET-AQ2021 Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 415 and SG21 TD348/Plen

JVET-AQ2022 Draft template for proposal description documents of responses to the Joint CfP on video compression with capability beyond VVC

JVET-AQ2028 Common test conditions on Gaussian splat coding, also issued as WG 5 N 420

JVET-AQ2032 Technologies under consideration for future extensions of VSEI (version 13)

JVET-AQ2040 Description of SEI-related exploration experiments on Gaussian splat coding (Excerpt from WG 5 N 430 and N 432)

- The following X documents were produced as WG 5 documents only, without a corresponding JVET output document or direct repetition of their content in this meeting report:

WG 5 N 410 Request for ISO/IEC 23008-2:202x/Amd.1 Additional HEVC profiles

WG 5 N 412 Request for ISO/IEC 23008-5:202x Reference software for HEVC (3rd edition)

WG 5 N 413 Text of ISO/IEC CD 23008-5:202x Reference software for HEVC (3rd edition)

WG 5 N 414 Request for ISO/IEC 23008-8:202x Conformance specification for HEVC (3rd edition)

WG 5 N 415 Text of ISO/IEC 23008-8:202x Conformance specification for HEVC (3rd edition)

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. Another Joint AHG on Gaussian splat coding was established together with WG 4 and WG 7. At this meeting, several Joint EEs on Gaussian splat coding were defined together with WG 4 and WG 7, where JEE 6.7 and JEE 6.9 are related to topis of using SEI messages for that purpose. The next ten JVET meetings were planned for 13 – 22 January 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU; during 17 – 28 April 2027 under ITU-T SG21 auspices in Geneva, CH; during 7 – 16 July 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI; during 20 – 29 October 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN; during 13 – 22 January 2028 under ISO/IEC JTC 1/‌SC 29 auspices in Thessaloniki, GR; during 25 March – 5 April 2028 under ITU-T SG21 auspices in Geneva, CH; during 19 – 28 July 2028 under ISO/IEC JTC 1/‌SC 29 auspices in Fukuoka, JP; during October 2028 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; and during January 2029 under ITU-T SG21 auspices, date and location t.b.d.; and during April 2029 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. The most recent versions of JCT-VC and JCT-3V documents can now be accessed directly via the JVET site, whereas all uploaded versions are also available from http://wftp3.itu.int/av-arch/jctvc-site/ and http://wftp3.itu.int/av-arch/jct3v-site/, respectively. JVET documents are also provided via http://wftp3.itu.int/av-arch/jvet-site/, but as this is currently not maintained as a mirror with the jvet-experts.org site, documents are typically only made available with a delay of two or three meeting cycles, to avoid that late uploads of output documents would be missing.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 21 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its forty-fourth meeting during 17 – 23 October 2026 at the Hangzhou Blossom Water Museum Hotel, Hangzhou, China. The meeting was held as a physical meeting with remote participation, provided on best-effort basis for experts who were unable to travel.

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding (coordinated EE1 discontinued by 43rd meeting)

- Exploration on Enhanced Compression beyond VVC capability (coordinated EE2 discontinued by 42nd meeting)

- Joint exploration (with WG 4 and WG 7 of SC 29) on Gaussian splat coding

JVET had been tasked by the parent bodies to develop a call for proposals on video compression with capability beyond existing standards, where the call had been finalized by the 43rd meeting, and further discussion to prepare processing and evaluation of expected responses to be conducted during the 44th meeting.

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants, consisting of two parts, (B1) in-person attendees as recorded by a sign-in sheet circulated in meeting rooms, (B2) remote attendees as recorded by the teleconferencing tool used for the meeting

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2026_10_AR_Hangzhou/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the forty-third JVET meeting in producing the following output documents:

a) JVET documents

JVET-AQ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AQ1006 HEVC additional profiles (Draft 1), also submitted for ballot as WG 5 CDAM N 411

JVET-AQ1008 Conformance testing for HEVC multiview extended and monochrome profiles (Draft 3)

JVET-AQ1012 Overview of IT systems used in JVET

JVET-AQ1017 Updates of HEVC reference software (draft 1)

JVET-AQ2006 Additional SEI messages for VSEI version 5 (Draft 1)

JVET-AQ2019 Description of algorithms version 16 and software version 18 in neural network-based video coding (NNVC)

JVET-AQ2020 Film grain synthesis technology for video applications ed. 2 (Draft 6), also submitted for ballot as WG 5 CDTR N 409

JVET-AQ2021 Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 415 and SG21 TD348/Plen

JVET-AQ2022 Draft template for proposal description documents of responses to the Joint CfP on video compression with capability beyond VVC

JVET-AQ2028 Common test conditions on Gaussian splat coding, also issued as WG 5 N 420

JVET-AQ2032 Technologies under consideration for future extensions of VSEI (version 13)

JVET-AQ2040 Description of SEI-related exploration experiments on Gaussian splat coding (Excerpt from WG 5 N 430 and N 432)

b) documents produced as WG 5 documents only:

WG 5 N 410 Request for ISO/IEC 23008-2:202x/Amd.1 Additional HEVC profiles

WG 5 N 412 Request for ISO/IEC 23008-5:202x Reference software for HEVC (3rd edition)

WG 5 N 413 Text of ISO/IEC CD 23008-5:202x Reference software for HEVC (3rd edition)

WG 5 N 414 Request for ISO/IEC 23008-8:202x Conformance specification for HEVC (3rd edition)

WG 5 N 415 Text of ISO/IEC 23008-8:202x Conformance specification for HEVC (3rd edition)

WG 5 N 421 Draft CfP of Gaussian splat coding

WG 5 N 422 Call for content for Gaussian splat coding

WG 5 N 423 FAQ on Gaussian splat coding

WG 5 N 424 JEE 6.1 on data preparation

WG 5 N 425 JEE 6.2 on anchor generation

WG 5 N 426 JEE 6.3 on new coding technologies

WG 5 N 427 JEE 6.4 on collecting requirements and use cases for Gaussian splat coding

WG 5 N 428 JEE 6.5 on 3DGS software

WG 5 N 429 JEE 6.6 on coordinating the 1F-geo Track

WG 5 N 430 JEE 6.7 on video-based coding technologies

WG 5 N 431 JEE 6.8 on Gaussian Splat Coding evaluation and CTC

WG 5 N 432 JEE 6.9 on evaluation of encapsulation methods for Gaussian Splats

- c) documents produced for handling by ITU-T:

SG21 TD326/Plen H-Series Supplement 22 (ex. H.Sup-MACVC) Optimization of encoders and receiving systems for machine analysis of coded video content

Liaison to JPEG on JPEG AI and explorations on video coding, included as item 1 in SG21 TD354/WP3

Liaison to 3GPP SA4 on HEVC multi-layer profiles, chroma subsampling and bit depth combinations, included as item 2 in SG21 TD354/WP3

Further important goals were reviewing the results of the Joint EE on Gaussian Splat Coding, of other technical input on novel aspects of video coding technology, and preparing next steps for investigation of candidate technology towards further standard development by maintaining the ongoing Joint Call for Proposals on video compression with capability beyond VVC.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the UTC time, except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Other parenthetical comments may be used for describing the impact or motivation of a decision. Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for work items under study, non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair, or other experts that were appointed to chair parallel sessions. It is indicated who took the notes in review of a given category (or individual documents în a category), and by which day and time the review was conducted. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Friday, 9 October 2026. Any documents uploaded after 1159 hours Paris/Geneva time on Saturday 10 October 2026 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AR0191 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meeting, or documents which were requested to be produced for the purpose of improving specification text, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AR0XXX (a proposal on XXX), uploaded 10-XX,

- …,

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AR0XXX (a contribution on XXX), uploaded 10-XX,

- …,

Almost all cross-verification reports at this meeting (except for JVET-AR0XXX, …) were registered late, and/or uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AR0XXX, ….

The following cross-verification report(s) were still missing by the end of the meeting, but were uploaded later: JVET-AR0XXX, …. The following report(s) had not become available yet three weeks after the end of the meeting: JVET-AR0XXX, … . These were marked as withdrawn by the JVET chair, assuming the registration had become obsolete (paragraph kept for future use).

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation did not apply to any contributions. (did apply to document(s) JVET-AR0XXX, being initially empty of results and flagged as late in the list above, based on the time of the first reasonable document upload | sentence kept for future use).

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AQ1000, the Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP JVET-AQ1004, the HEVC additional profiles (Draft 2) JVET-AQ1006, the Conformance testing for HEVC multiview extended and monochrome profiles (Draft 3) JVET-AQ1008, the Overview of IT systems used in JVET JVET-AQ1012, the Updates of HEVC reference software (draft 1) JVET-AQ1017, the Additional SEI messages for VSEI version 5 (Draft 1) JVET-AQ2006, the Description of algorithms version 16 and software version 18 in neural network-based video coding (NNVC) JVET-AQ2019, the Film grain synthesis technology for video applications ed. 2 (Draft 6) JVET-AQ2020, the Joint Call for Proposals on video compression with capability beyond VVC JVET-AQ2021, the Draft template for proposal description documents of responses to the Joint CfP on video compression with capability beyond VVC JVET-AQ2022, the Common test conditions on Gaussian splat coding JVET-AQ2028, the Technologies under consideration for future extensions of VSEI (version 13) JVET-AQ2032, and the Description of SEI-related exploration experiments on Gaussian splat coding JVET-AQ2040, had been completed. The software implementations of (VTM version 24.x, - retained for future use) HM version XX, and NNVC version 18.0 and 18.1 were also approved.

It was further noted that some editorial modifications were necessary on the WG 5 versions of standard deliverables, to be compliant with the ISO/IEC rules.

Only minor editorial issues were found in the meeting report JVET-AQ1000; no need to produce an update was identified (see section 2.14 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annexes B1 and B2 of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/21 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts, and through the Q6/21 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- According to rules of JVET’s parent bodies, any recording of meeting session (regardless if conducted in the meeting room or from the zoom screen) is not permitted. Participants who are identified to perform recording will be removed from the session unless they stop recording upon request.

- Usage of transcription tools in zoom sessions is strongly discouraged, as they often give erroneous results and lead to misunderstanding of verbal expressions by other experts.

- Generally, do not use video for the teleconferencing calls in order to avoid overloading internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which shared screen they want to see.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Approval of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities, and of the joint AHG on Gaussian splat coding

Report of joint EEs on Gaussian splat coding

Planning related to the Joint Call for Proposals, definition of criteria for technology assessment, and finalization of the proposal description template

Consideration of contributions on high-level syntax, and further development of the working draft towards VSEI v5

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development for VVC, HEVC and AVC

Consideration of contributions on non-reference software maintenance

Consideration of contributions on software development and maintenance related to exploration activities

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology, and work towards the second edition of the TR

Consideration of contributions on gaming content compression

Consideration of contributions on assessment of implementation complexity of video coding technology

Consideration of contributions on aspects of ultra-low latency and packet loss resilience in the context of video compression

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The times of meeting sessions followed the needs of the face-to-face meeting, with highest priority given to the aim of achieving the goals of the meeting (see under section 2.15). Typical meeting hours were expected to be 0900-2000 CST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were anticipated to be necessary. Sessions were announced in the online JVET calendar in advance as far as possible, but it was agreed that some activities (such as breakout sessions) could be held at short notice.

## ISO, IEC and ITU-T Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100011.html.

https://www.iec.ch/basecamp/iec-code-conduct-technical-work

These include points relating to:

- Behave ethically

- Promote and enable all voices to be heard

- Engage constructively in ISO and IEC activities

- Respect others (in meetings, when writing, on social media)

- Declare actual and potential conflicts of interest and manage them appropriately

- Protect confidential information

- Protect ISO and IEC assets

- Avoid and prevent any form of bribery or corruption

- Uphold the consensus process

- When disputes arise, escalate, resolve and uphold agreed resolution

Participants were also reminded of the UN Codes of Conduct, applying to work under ITU-T, found at https://www.un.org/management/sites/www.un.org.management/files/un-system-model-code-conduct.pdf

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11 of June/July 2009. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. The site contains links for creating a user account for document uploading and for contacting an administrator – e.g., to update the email address associated with an account, and experts were reminded to keep this information up to date. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

It was noted that also the most recent versions of JCT-VC and JCT-3V documents can be accessed directly via the JVET site. A mirror for JCT-VC, JCT-3V and JVET document access (including all versions with original upload dates) has also been provided via the ITU ftp site, providing a “documents” subfolder in the directory of each respective meeting.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1399 (as of 15 Oct. 2026). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

It is further emphasized that the document JVET-AJ1012 (note that an update JVET-AQ1012 is an output from the current meeting) gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials. It was suggested to produce an updated version of this document, including refined guidance for the usage of Zoom functionality in alignment with ISO directives and policies as a mandate of an AHG.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – video coding standard developed by JVT, formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BIF: Bilateral filter

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardization

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted for study of coding technology

- CfE: Call for Evidence

- CfP: Call for Proposals

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CICP: Coding-independent code points (Rec. ITU-T H.273 | ISO/IEC 23091-2)

- CIIP: Combined inter/intra prediction

- CL-RAS: Cross-layer random-access skip

- CNN: Convolutional neural network

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DRF: Deep reference frame (for NNVC inter prediction)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EE: Exploration experiment – a coordinated experiment conducted for study of coding technology prior during exploration work prior to establishment of a formal project (EEn for some number n is exploration experiment number n)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase and test model algorithm for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EIP: Extrapolation based intra prediction

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FGRC: Film grain regions characteristics

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GFV: Generative face video

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- GS/GSC: Gaussian splatting / Gaussian splat coding

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a software codebase and video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HOP: High-complexity operating point for NNLF.

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC 3D video extensions)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (originally joint video exploration team, initially for VVC, later expanded in scope)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- kMAC: thousands of multiply-accumulate operations – used as a complexity metric

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LB-CCP: Local-boosting CCP

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LOP: Low-complexity operating point for NNLF.

- LP or LDP: Low delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MM-CCCM: Multi-model CCCM

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MOS: Mean opinion score (a metric for subjective quality evaluation)

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MSE: Mean-squared error, a distortion metric

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNLF: Neural network loop filter

- NNPF: Neural network post filter

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSPT: Non-separable primary transform

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display)

- ONNX: Open Neural Network Exchange – a format used to convert code from common neural network software packages into SADL code

- operation point: A temporal subset of an OLS

- PDPC: Position-dependent (intra) prediction combination

- PERP: Padded equirectangular projection (a 360° projection format)

- PH: Picture header

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format)

- PMMVD: Pattern-matched motion vector derivation

- POC: Picture order count

- PoR: Plan of record

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set

- PSNR: Peak signal-to-noise ratio, a fidelity metric derived from MSE

- PTL: Profile/tier/level combination

- QM: Quantization matrix

- QP: Quantization parameter (sometimes conflated with quantization step size)

- QT: Quadtree

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD)

- RADL: Random-access decodable leading (type of picture)

- RASL: Random-access skipped leading (type of picture)

- R-D: Rate-distortion

- RDO: Rate-distortion optimization

- RDOQ: Rate-distortion optimized quantization

- RDPCM: Residual DPCM

- ROI: Region of interest

- ROT: Rotation operation for low-frequency transform coefficients

- RPL: Reference picture list

- RPLM: Reference picture list modification

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC

- RPS: Reference picture set

- RQT: Residual quadtree

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q)

- RVM: Rate variation measure

- SAD: Sum of abolute differences, a measurement of distortion

- SADL: Small adhoc deep learning librar

- SAO: Sample-adaptive offset

- SBT: Subblock transform

- SbTMVP: Subblock based temporal motion vector prediction

- SCIPU: Smallest chroma intra prediction unit

- SCM: Screen content model – a software codebase and test model algorithm for the HEVC screen content coding extensions

- SD: Slice data; alternatively, standard-definition

- SDH: Sign data hiding

- SDT: Signal-dependent transform

- SE: Syntax element

- SEI: Supplemental enhancement information

- SGPM: Spatial geometric partition mode

- SH: Slice header

- SHM: Scalable HM – a software codebase and test model algorithm for SHVC

- SHVC: Scalable high efficiency video coding

- SIF: Switchable (motion) interpolation filter

- SIMD: Single instruction, multiple data

- SMVD: Symmetric MVD

- SPO: SEI processing order

- SPS: Sequence parameter set

- SR: Super-resolution

- STMVP: Spatial-temporal motion vector prediction

- STRP: Short-term reference picture

- STSA: Step-wise temporal sublayer access

- TALF: Temporal adaptive loop filter

- TBA/TBD/TBP: To be announced/determined/presented/public

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content

- TIMD: Template-based intra mode derivation

- TM: Template matching

- TMRL: Template-based multiple reference line prediction

- TMVP: Temporal motion vector prediction

- TS: Transform skip

- TSRC: Transform skip residual coding

- TT: Ternary tree

- TuC: Technology under consideration

- UGC: User-generated content

- UWP: Unequal weight prediction

- VCEG: Visual coding experts group (ITU-T Q.6/21, the relevant rapporteur group in ITU-T WP3/21, which is one of the two parent bodies of the JVET)

- VCM: Video coding for machines

- VLOP: Very low-complexity operating point for NNLF

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy

- VQA: Visual quality assessment

- VT: Verification testing

- VTM: VVC Test Model – a software codebase and test model algorithm for the VVC standard

- VUI: Video usability information (part of an SPS, similar to an SEI message that applies to a video sequence)

- VVC: Versatile Video Coding, video coding standard developed by JVET, formalized by ITU-T as Rec. ITU-T H.266 and by ISO/IEC as ISO/IEC 29090-3

- WCG: Wide colour gamut

- WG: Working group, a group of technical experts (usually used to refer to the WGs of ISO/IEC JTC 1/SC 29)

- WPP: Wavefront parallel processing (usually synonymous with ECS)

- Block and unit names in MPEG-2 and AVC:

  - MB: Macroblock – a 16x16 array for the luma component and corresponding 8x8 arrays for the chroma components (basically the same as a CTU in later standards, but with a fixed size)

  - MB pair: A pair of vertically contiguous macroblocks in a frame that is coupled for use in macroblock-adaptive frame/field decoding

  - Sub-MB: One quarter of the samples of a macroblock, i.e., an 8x8 luma block and two corresponding chroma blocks of which one corner is located at a corner of the macroblock for a picture that has three sample arrays or an 8x8 luma block of which one corner is located at a corner of the macroblock for a monochrome picture or a picture that is coded using three separate colour planes.

  - Sub-MB partition: A block of luma samples and two corresponding blocks of chroma samples resulting from a partitioning of a sub-macroblock for inter prediction for a picture that has three sample arrays or a block of luma samples resulting from a partitioning of a sub-macroblock for inter prediction for a monochrome picture or a picture that is coded using three separate colour planes

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU)

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions)

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape

  - PB: Prediction block, a luma or chroma block of a PU

  - PU: Prediction unit, has the same size as a CU in the VVC context

  - TB: Transform block, a luma or chroma block of a TU

  - TU: Transform unit, has the same size as a CU in the VVC context

## Standards, TRs, supplements and technical papers approval and publication status

- MPEG-2 | H.262 (the video coding specification is common text and not publicly available without cost per ISO special request)

  - ITU-T H.262 V3 was approved in 2012-02; its Amd 1 was approved in 2013-03 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-09

  - ISO/IEC 13818-2:2013 (Ed. 3) FDIS ballot closed 2012-05-08; FDAM 1 ballot closed 2013-04-12 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-10

  - Conformance testing (not joint with ITU-T)

    - ISO/IEC 13818-4:2004 (Ed. 2) FDIS closed 2004-08-22, published 2004-12-12; it specifies conformance testing for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), and Part 7 (AAC) (confirmed freely available 2026-05)

    - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing (confirmed freely available 2026-05)

    - Cor 1:2007, Cor 2:2009, Cor 3:2012 (publication availability unclear, as these do not appear to be directly available from the ISO website) may also have video relevance

  - Reference software (not joint with ITU-T)

    - ISO/IEC TR 13818-5:2005 (Ed. 2) final ballot closed 2005-07-24, published 2005-10; it specifies reference software for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), Part 7 (AAC) and Part 11 (IPMP) (confirmed freely available 2026-05)

- AVC (twin text)

  - ITU-T H.264 V14 was Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13 (ITU work programme link)

  - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

  - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 IPT-PQ-C2 issued at 28th meeting 2022-10, third draft issued at 29th meeting 2023-01, fourth draft issued at 30th meeting 2023-04, formal project requested and CD of 11th edition issued at 31st meeting 2023-07, DIS issued at 32nd meeting 2023-10, DAM ballot closed 2024-04-15, ISO/IEC 14496-10:2025 (Ed. 11) FDIS issued at 34th meeting 2024-04, DIS approved for registration as FDIS 2024-09-17, FDIS ballot issued 2025-01-14, FDIS ballot closed 2025-03-23; published 2025-07-10, not yet available without cost

  - ITU-T H.264 V15 Consented 2024-04-26, last call began 2024-07-16, approved 2024-08-13, published 2024-11-13

  - ISO/IEC 14496-10:202x (Ed. 12), initiated as ISO/IEC 14496-10:2025/Amd.1 to support some SEI messages of VSEI v4, requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, closed 2025-12-31, summary of voting in m75350, preliminary DoCR issued at January meeting in WG 5 N 384, FDIS issued 2025-05-01, DIS approved for registration as FDIS 2026-07-09, pending text submission, public availability requested 2026-04 (ISO Projects link)

  - ITU-T H.264 V16 Consented by WP3/21 2026-05-01, last call began 2026-05-16, approved 2026-06-13 (ITU work programme link)

- Post-meeting update: Published 2026-07-28

  - Conformance testing (twin text)

    - ITU-T H.264.1 V6 Approved 2016-02-13, published 2016-06-17

    - Various amendments of ISO/IEC 14496-4:2004 (Ed. 2), including:

      - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 43:2015 3D-AVC conformance testing (confirmed publicly available 2026-05)

      - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC (confirmed publicly available 2026-05)

  - Reference software (twin text)

    - ITU-T H.264.2 V6 Approved 2016-02-13, published 2016-05-30

    - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) have been published, including:

      - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC (confirmed publicly available 2026-05)

      - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC (confirmed publicly available 2026-05)

- HEVC (twin text)

  - ITU-T H.265 V7 approved 2019-11-29, published 2020-01-10

  - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

  - ITU-T H.265 V8 Consented at the 22nd meeting (with shutter interval information SEI message and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

  - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

  - ISO/IEC 23008-2:2023 (Ed. 5), initiated as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of 2022-04, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10, FDIS issued 29th meeting of 2023-01, FDIS ballot opened 2023-08-06, closed 2023-10-02, published 2023-10-30

  - ITU-T H.265 V9 Consented at 31st meeting 2023-07, approved 2023-09-13, and pre-published 2023-09, published 2023-11-24

  - Preliminary draft HEVC text for YCgCo-Re and YCgCo-Ro issued at 26th meeting 2022-04, second draft including SMPTE ST 2128 issued at 28th meeting 2022-10, third draft at 29th meeting 2023-01, fourth draft at 30th meeting 2023-04, formal work item requested and CDAM 1 issued 31st meeting 2023-07, DAM issued with new (multiview) profiles and SEI messages at 32nd meeting 2023-10, DAM ballot closed 2024-04-08

    - ITU-T H.265 V10 Consented 2024-04-26, last call opened 2024-07-01, closed 2024-07-28, approved 2024-07-29, pre-published 2024-08-06, published 2024-10-07

    - ISO/IEC 23008-2:2025 (Ed. 6) FDIS issued from 35th meeting 2024-04, DIS approved for registration as FDIS 2024-10-03, FDIS registered for formal approval 2024-10-13, FDIS ballot issued 2024-12-30, FDIS ballot closed 2025-02-24, published 2025-03-20

  - ISO/IEC 23008-2:2026 (Ed. 7), initiated as planned ISO/IEC 23008-2:2025/Amd.1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consultation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, FDIS issued from 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, FDIS registered for formal approval 2026-03-09, FDIS ballot issued 2026-06-08, public availability requested 2025-10 (ISO Projects link)

- Post-meeting update: FDIS ballot closed 2026-08-04, published 2026-09-27, not yet available without cost.

  - ITU-T H.265 V11 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19, published 2026-03-18 (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.265.1 V3 approved 2018-10-14, published 2019-01-15

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06, not yet available without cost

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15, not yet available without cost

  - Reference software (twin text)

    - ITU-T H.265.2 V3 approved 2016-12-22, published 2017-04-10

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, FDIS issued from 2016-02 meeting, published 2017-03-01, not yet available without cost

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, FDAM issued from 2017-04 meeting, FDAM ballot opened 2017-08-10, closed 2017-10-07, published 2017-11-09, not yet available without cost

- VVC (twin text)

  - ITU-T H.266 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

  - ITU-T H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

  - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, FDIS ballot opened 2022-06-29, closed 2022-08-24, published 2022-09-25

  - ISO/IEC 23090-3:2024 (Ed. 3), initiated as (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03, FDIS issued at WG level 2023-07, FDIS ballot opened 2024-05-11, closed 2024-06-26, published 2024-07-17

  - ITU-T H.266 V3 Consented 2023-07, approved 2023-09-29 and pre-published 2023-09, published 2023-11-29

    - H.266 (2023) Erratum 1 published 2025-05 to correct errors in a few figures

  - ISO/IEC 23090-3:2026 (Ed. 4), initiated as ISO/IEC 23090-3:2024/Amd.1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, FDIS registered for formal approval 2026-03-09, FDIS ballot initiated 2026-06-08, public availability requested 2025-10 (ISO Projects link)

- Post-meeting update: FDIS ballot closed 2026-08-04, published 2026-09-27, not yet available without cost.

  - ITU-T H.266 V4 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-22, published 2026-03-19 (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-15:2022 (Ed. 1) approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

    - ISO/IEC 23090-15:2024 (Ed. 2) began as Amd.1 Operation range extensions – CDAM 1 issued from 24th meeting 2021-10, DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15, consolidated into FDIS 2nd edition issued as an output of the 29th meeting in January 2023, ballot opened 2024-04-08, closed 2024-06-03, published 2024-07-04

    - ITU-T H.266.1 V2 Consented 2023-07, approved 2023-09-13 and pre-published 2023-09, published 2023-10-19

    - ISO/IEC 23090-15:2026 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, closed 2025-06-30, DIS issued at 39th meeting 2025-07, closed 2025-12-22, summary of voting in m75347, proceeded directly to publication at 41st meeting 2026-01, final text received 2026-04-27, pending proof review (ISO Projects link)

- Post-meeting update: Proof returned by secretariat 2026-08-03, published 2026-08-23, not yet available without cost.

    - ITU-T H.266.1 (V3) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19, published 2026-03-23 (ITU work prgramme link)

  - Reference software (twin text)

    - ITU-T H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to DIS approved for registration status in ISO Projects system 2022-04-21, FDIS registered for formal approval 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

    - ITU-T H.266.2 V2 Consented 2024-04-26, last call opened 2024-05-16, closed 2024-06-12, approved 2024-06-13, pre-published 2024-08-06, published 2024-10-09

    - ISO/IEC 23090-16:2025 (Ed. 2) Request & CD issued 2024-04, consultation deferred due to meeting timing, consultation initiated 2024-09-06, closed 2024-11-01, DIS issued at 36th meeting 2024-11, approved for registration as DIS 2024-12-18, DIS registered 2025-01-7, DIS ballot opened 2025-03-09, closed 2025-06-01, approved to proceed to publication without an FDIS at 29th meeting 2025-07, public availability requested 2025-07, published 2025-11-19, not yet available without cost. (Roughly corresponding to H.266.2 V2 already approved and published in ITU-T)

- VSEI (twin text)

  - ITU-T H.274 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

  - ITU-T H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

  - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

  - ISO/IEC 23002-7:2024 (Ed. 3) began as (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, DAM ballot closed 2022-04-06, FDIS 3rd edition issued 2023-07, FDIS ballot began 2024-07-25, closed 2024-09-20, published 2024-10-30

  - ITU-T H.274 V3 Consent 2023-07, approved 2023-09-29, pre-published 2023-10-11, published 2024-03-12

  - ISO/IEC 23002-7:2026 (Ed. 4) Request & CDAM 1 issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, FDIS registered for formal approval 2026-04-01, FDIS ballot initiated 2026-06-08, public availability requested 2025-10 (ISO Projects link)

- Post-meeting update: FDIS ballot closed 2026-08-04, published 2026-09-27, not yet available without cost.

  - ITU-T H.274 V4 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-02-04, published 2026-04-09 (ITU work programme link)

- CICP (twin text)

  - ITU-T H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

  - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

  - ITU-T H.273 V3 Consent 2023-07, approved 2023-09, not published due to waiting for publication of SMPTE ST 2128

  - ITU-T H.273 V4 Consented 2024-04-26, last call opened 2024-06-16, closed 2024-07-13, approved 2024-07-14, pre-published 2024-08-06, published 2024-10-07

  - ISO/IEC 23091-2:2025 (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, DIS ballot closed 2023-04-06, preliminary draft text for including SMPTE ST 2128 issued at 28th meeting, incorporated into preliminary FDIS at 30th meeting 2023-04, FDIS issued 2024-04 (after waiting for publication of SMPTE ST 2128, then proceeding), FDIS registered for formal approval 2024-10-24, FDIS ballot issued 2024-12-03, FDIS ballot closed 2025-01-28, published 2025-02-25, not yet available without cost

- Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics (twin text)

  - ITU-T H-Series Supplement H.Sup15 V1, approved 2017-01-27, published 2017-04-12

  - ISO/IEC TR 23008-14:2018 (Ed. 1) published 2018-08-06, not yet available without cost

- Signalling, backward compatibility and display adaptation for HDR/WCG video coding (twin text)

  - ITU-T H-Series Supplement H.Sup18 V1, approved 2017-10-27, published 2018-01-18

  - ISO/IEC TR 23008-15:2018 (Ed. 1) published 2018-08, not yet available without cost

- Usage of video signal type code points (twin text)

  - ITU-T H-Series Supplement H.Sup19 V3 approved 2021-04-30, published 2021-06-04

  - ISO/IEC TR 23091-4:2021 (Ed. 3) published 2021-05-23, not yet available without cost

- Working practices using objective metrics for evaluation of video coding efficiency experiments (twin text)

  - ITU-T H-Series Supplement HSTP-VID-WPOM V1 approved 2020-07-03, published 2020-11

  - ISO/IEC TR 23002-8:2021 (Ed. 1) published 2021-05-20, not yet available without cost

- Film grain synthesis technologies for video applications (twin text)

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting, CDTR issued at 29th meeting 2023-01, consultation period ended 2023-07-09, DTR text was issued from the 31st meeting in July 2023, put on hold by ISO staff editors, DTR ballot opened 2024-03-05, closed 2024-04-30, NB comments handled by ISO staff editor in consultation with project editors, published 2024-07-24

  - ITU-T H-Series Supplement H.Sup21 V1 (ex H.Sup-FGST), Agreement 2025-01-24 by ITU-T SG21, pre-publication 2025-03-26, published 2025-05-21

  - ISO/IEC TR 23002-9:202x (Ed. 2) request for new edition issued from 42nd meeting 2026-05-01, pending CDTR at current meeting

- Optimization of encoders and receiving systems for machine analysis of coded video content (twin text)

  - ISO/IEC TR 23888-3:2026 (Ed. 1) Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, closed 2025-11-03, summary of voting in m75361, second DTR ballot issued from 41st meeting 2026-01, ballot opened 2026-03-30, closed 2026-04-28, summary of voting in m76996, agreed to proceed to publication at 42nd meeting 2026-05-01, published 2026-06-14 (ISO Projects link), not yet available without cost

  - ITU-T provisional name H.Sup.MACVC, approval target date 2026-07, i.e., the current meeting (ITU work programme link)

- Freely available ISO/IEC standards (without cost) in the ISO Webstore and IEC Webstore include

  - ISO/IEC 13818-4:2004 Conformance testing for MPEG-2 (confirmed available 2025-10-31)

  - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing (confirmed available 2025-10-31)

  - ISO/IEC TR 13818-5:2005 Software simulation for MPEG-2 (confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-4:2004 specify conformance for AVC (base text confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) specify reference software for AVC (base text confirmed available 2025-10-31)

- Intended to be freely available (without cost) but unavailable (without cost) standards and technical reports include the following. (These should be checked for previously issued requests for free availability.)

  - ISO/IEC 14496-10:2025 (Ed. 11) AVC, published 2025-07-10 (public availability request to be confirmed, publication of Ed. 12 pending)

  - ISO/IEC 23002-7:2024 (Ed. 3) VSEI, published 2024-10-30 (public availability requested 2023-07, approved by SC 29 Res. 59/2023, publication of Ed. 4 pending)

  - ISO/IEC 23008-2:2025 (Ed. 6) HEVC, published 2025-03 (public availability requested 2024-04, publication of Ed. 7 pending)

  - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for HEVC, published 2017-03-01 (public availability requested 2016-02, reiterated 2025-10, approved by SC 29 Res. 24/2026)

  - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for HEVC screen content coding extensions, published 2017-11-09 (public availability not requested 2016-06, 2016-10, 2017-01, 2017-04, 2017-07, 2017-10, 2018-01, 2018-04, public availability requested 2025-10, approved by SC 29 Res. 24/2026)

  - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06 (public availability requested 2018-10, reiterated 2025-10, approved by SC 29 Res. 24/2026)

  - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15 (public availability requested 2025-10, approved by SC 29 Res. 24/2026)

  - ISO/IEC 23090-3:2024 (Ed. 3) VVC, published 2024-07-17 (public availabilty request unconfirmed, publication of Ed. 4 pending)

  - ISO/IEC 23090-15:2024 (Ed. 2) Conformance specification for VVC, published 2024-07-04 (public availability request unconfirmed, publication of Ed. 3 pending)

  - ISO/IEC 23090-16:2025 (Ed. 2) Reference software for VVC, published 2025-11-19 (public availability requested 2025-07, approved by SC 29 Res. 34/2025)

  - ISO/IEC 23091-2:2025 (Ed. 3) CICP, published 2025-02-25 (public availability requested 2024-04, reiterated 2025-10, approved by SC 29 Res. 24/2026)

  - ISO/IEC TR 23008-14:2018 (Ed. 1) Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics, published 2018-08-06 (public availability requested 2025-07, approved by SC 29 Res. 34/2025)

  - ISO/IEC TR 23008-15:2018 (Ed. 1) Signalling, backward compatibility and display adaptation for HDR/WCG video coding, published 2018-08 (public availability requested 2025-07, approved by SC 29 Res. 34/2025)

  - ISO/IEC 23090-16:2025 (Ed. 2) Reference software for VVC (public availability requested 2025-07, approved by  SC 29 Res. 34/2025)

  - ISO/IEC TR 23091-4:2021 (Ed. 3) Usage of video signal type code points, published 2021-05-23 (public availability requested 2025-07, approved by SC 29 Res. 34/2025)

  - ISO/IEC TR 23002-8:2021 (Ed. 1) Working practices using objective metrics for evaluation of video coding efficiency experiments, published 2021-05-20 (public availability requested 2025-07, approved by Res. 34/2025)

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Film grain synthesis technologies for video applications, published 2024-07-24 (not requested 2024-04, public availability requested 2025-07, approved by Res. 34/2025)

  - ISO/IEC TR 23888-3:2026 (Ed. 1) Optimization of encoders and receiving systems for machine analysis of coded video content (public availability requested 2026-05, not yet endorsed by SC 29)

## Draft standards progression status for active work items

- AVC ISO/IEC 14496-10: 202x (Ed. 12), initiated as ISO/IEC 14496-10:2025/Amd.1 to support some SEI messages of VSEI v4, requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, closed 2025-12-31, summary of voting in m75350, preliminary DoCR issued at January meeting in WG 5 N 384, FDIS issued 2025-05-01, DIS approved for registration as FDIS 2026-07-09, pending text submission, public availability requested 2026-04 (ISO Projects link)

  - ITU-T H.264 V16 Consented by WP3/21 2026-05-01, last call began 2026-05-16, approved 2026-06-13 (ITU work programme link)

- ISO/IEC 23008-2:2026 (Ed. 7), initated as planned ISO/IEC 23008-2:2025/Amd.1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consulation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, FDIS issued from 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, final text received or FDIS registered for formal approval 2026-03-09, FDIS ballot issued 2026-06-08, public availability requested 2025-10 (ISO Projects link)

- Post-meeting update: FDIS ballot closed 2026-08-04, published 2026-09-27, not yet available without cost.

  - ITU-T H.265 V11 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19, published 2026-03-18 (ITU work programme link)

- VVC ISO/IEC 23090-3:2026 (Ed. 4), initiated as ISO/IEC 23090-3:2024/Amd.1, Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, FDIS registered for formal approval 2026-03-09, initiated 2026-06-08, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.266 V4 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-22, published 2026-03-19 (ITU work programme link)

- VVC conformance ISO/IEC 23090-15:2026 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, closed 2025-06-30, DIS issued at 39th meeting 2025-07, closed 2025-12-22, summary of voting in m75347, proceeded directly to publication at 41st meeting 2026-01, final text received 2026-04-27, pending proof review (ISO Projects link)

- Post-meeting update: Proof returned by secretariat 2026-08-03, published 2026-08-23, not yet available without cost.

  - ITU-T H.266.1 (V3) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19, published 2026-03-23 (ITU work programme link)

- ISO/IEC 23002-7:2026 (Ed. 4) Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, FDIS registered for formal approval 2026-04-01, FDIS ballot initiated 2026-06-08, public availability requested 2025-10 (ISO Projects link)

- Post-meeting update: FDIS ballot closed 2026-08-04, published 2026-09-27, not yet available without cost.

  - ITU-T H.274 V4 consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-02-04, published 2026-04-09 (ITU work programme link)

- ISO/IEC TR 23002-9:202x (Ed. 2) Film grain synthesis technology for video applications – request for new edition issued from 42nd meeting 2026-05-01, pending CDTR at current meeting

- H.Sup.MACVC | ISO/IEC TR 23888-3:2026 (Ed. 1) Optimization of encoders and receiving systems for machine analysis of coded video content – ISO/IEC TR 23888-3 Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, closed 2025-11-03, summary of voting in m75361, second DTR ballot issued from 41st meeting 2026-01, ballot opened 2026-03-30, closed 2026-04-28, summary of voting in m76996, agreed to proceed to publication at 42nd meeting 2026-05-01, published 2026-06-14 (ISO Projects link), not yet available without cost

  - ITU-T provisional name H.Sup.MACVC, approval target date 2026-07, i.e., the current meeting (ITU work programme link)

- A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations or in an SC 29 Resolution. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the ongoing work items when they become finalized.

## Opening remarks

Remarks during the opening session of the meeting Saturday 17 October at XXXX–XXXX CST were as follows.

- Timing and organization of the meeting and online access and calendar posting of session plans were reviewed

  - The initial number of documents was approximately the same as in the last meeting, where an increase is again observed in SEI related contributions, whereas low-level coding tools categories further decreased in number (in particular, after both EE1 and EE2 were discontinued). Also, fewer contributions were received on CfP aspects, with only some left-over aspects in preparing the evaluation in January.

  - Parallel sessions were considered to be necessary (SEI and core coding aspects), starting Saturday afternoon. Two rooms (Baosheng Hall 1+2 and Baosheng Hall 3) will be available for JVET all time, except MPEG information exchange sessions which will use Baosheng Hall 1+2, and a workshop Thursday afternoon using Baosheng Hall 3 (JVET will get a replacement for this). VCEG is also planning room sharing with JVET.

  - Coffee breaks 1030 and 1530

  - Lunch is served 1130-1330, Dinner is served 1800-2000

  - Sessions after 1900 can be held, request the evening before

  - Some aspects of common interest may need discussion at plenary level

- Future non-virtual meetings will be held as physical meetings with remote participation (designation has been changed in ISO/IEC with new directives which are in force since 5 October).

- Other changes in new directives as far as relevant for JVET were summarized.

- Plans for subsequent physical meetings were reviewed: January 2027 (Brisbane), April 2027 (Geneva), July 2027 (Tampere), October 2027 (Shenzhen), January 2028 (Thessaloniki), April 2028 (Geneva), July 2028 (Fukuoka).

- Depending on the status of preparing future standardization activities, it may be necessary to extend the duration of meetings, or plan for AHG meeting days prior to the regular meeting (e.g., for analysis of CfP submissions)

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Access to the meeting was provided using Zoom. The meeting notes by the session chair were to be continually shared via zoom screen sharing

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

- The results of the previous meeting and the meeting report JVET-AQ1000 were reviewed (which was finalized X days before the meeting, but sufficiently mature drafts had been available before in the ITU ftp site). Only minor issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision.

- Documents JVET-AQ0019 and JVET-AQ0022 are still missing. A powerpoint deck regarding AQ0019 had been presented on the last day of the Geneva meeting.

- At the current meeting, on-site attendance will be recorded both via the traditional sign-in sheet. Participants were asked to correct their affiliation and email in the sign-in sheet in case these changed.

- It is noted that another sign-in sheet will be circulated by the organizers, which is for their own purpose of proving the number of meeting attendees across JVET, JPEG and all MPEG WGs.

- Remote attendance (of those not present in person) will be recorded via the zoom records. It is therefore important to follow the conventions of naming as mentioned before. Participants who cannot be correctly identified will not appear in the attendance sheet.

- There were no objections voiced in the opening plenary to the consideration of late contributions.

- There were again a significant number of documents registered where authors’ given names were not abbreviated according to the JVET custom (which helps produce shorter headings in the JVET meeting report), and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions. As for now, the JVET chair took action of correcting those cases in the document registry.

- Some general principles:

  - Experts were asked not to pick a specific JVET number for regular documents – this function is reserved for AHG reports, summary reports, and output docs. Reserving numbers without filling a precise title shall also be avoided – the chair may flag such documents as withdrawn, as they cannot be allocated to a certain category in the meeting notes.

  - Experts were asked to always register JVET documents via the “jvet-experts.org” site, not via the MPEG DMS site as WG 5 docs. At the current meeting, all documents related to technical modifications of GSC SEI messages in TuC shall be registered both as WG 7 (for handling in the joint AHG) and JVET documents (the latter are public). Documents that are only related to small correction/optimization of syntax and semantics of those existing messages may only be submitted to JVET.

  - Experts were asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes. Provisional titles shall be avoided.

- A joint AHG meeting on GSC was planned to be held during the weekend October 17/18. Discussions related to SEI messages will be conducted on Sunday (Will the HLS track meet in parallel, considering the large workload?). Further joint meetings on GSC are planned for Mon. 1600-1800, Tue. 1400-1800, Wed. 1400-1800, Thu. 1600-1800, Fri. 0900-1000. All those meetings will be held in Jixiang Hall (1st floor), see zoom link in the JVET calendar (zoom password of 43rd meeting in Geneva for the JAHG, as it was the case for all interim JAHG meetings)

- The following ballot results will become available through the SC 29 secretariat:

  - m78198  Summary of Voting on ISO/IEC CD 23008-5

  - m78199  Summary of Voting on ISO/IEC CD TR 23002-9

  - m78XXX  Summary of Voting on ISO/IEC CD 23008-8

  - m78XXX  Summary of Voting on ISO/IEC 23008-2 CDAM 1

- Would any of those be mature for ITU-T approval in January?

- The FDIS ballots for next editions of HEVC, VVC and VSEI have closed, and the new editions have been published. One obvious error in 23008-2 was found and has been corrected in last minute (deviating from H.265 since several years).

- The FDIS text of the new edition for 14496-10 was submitted for ballot shortly before the meeting.

- Plans for generating the next versions of reference software for AVC and VVC (which would need to support new SEI messages from VSEIv4) still need to be developed. Earliest target date for ITU-T consent could be at the next meeting (as WP 3/21 will meet in Brisbane) – would at least VVC be ready to go to consent by then, and should requests be made in ISO/IEC? For AVC, it might further be useful to generate a new part of ISO/IEC 14496 containing the entire newest JM software at least, and cancel the corresponding outdated amendments of 14496-5. See further discussion under AHG3.

- The primary goals of the meeting were:

  - Issue final version of the proposal description package, including complexity analysis to be provided

  - DIS on HEVC software/conformance

  - DAM on HEVC / convert into new edition now or at FDIS?

  - DTR 2nd edition on film grain

  - New versions TuC on VSEI, and WD VSEI v5

  - Consideration of including additional VSEI in WDv5

  - Consideration of new SEI proposals

  - SEI show cases and software

  - Gaussian splats – work of Joint AhG (meets Saturday/Sunday), and planning of Joint EE. All documents on this activity will be handled by WG 7 at the current meeting

  - Documents related to former explorations on neural network-based video coding, and enhanced compression beyond VVC

  - VVC white paper update (remove VSEI, some update of the 2021 version)

- Needs for joint meetings were initially discussed (see section 7.3 on joint meetings that were later actually held).

- New ISO/IEC directives are in force since 5 Oct. Impact on JVET mode of working – short presentation at some time during the week, or add information about webinars available.

- It has been recently announced that Benjamin Bross, Jens-Rainer Ohm and Gary J. Sullivan will receive the IEEE Arun N. Netravali Award for Video Analytics, Technology, and Systems in 2027, “for leadership in developing the HEVC/H.265 video coding standard”.

- Principles of standards development were discussed.

- Scheduling of sessions was discussed – see under sections 2.6 and 2.15.

## Scheduling of discussions (update)

- The times of the meeting sessions followed the needs of the onsite meeting arrangements, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were in the range of 0900-1900 CST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were noted to potentially be necessary. Sessions were announced in the JVET calendar as far as possible in advance, although it was acknowledged that some activities (such as breakout sessions) might be held at short notice.

Particular scheduling notes are shown below, although not necessarily 100% accurate or complete. Times are recorded in the local timezone of the meeting venue, except as otherwise noted:

- Sat. 17 Oct., 1st day

  - Morning session:

    - 0900–XXXX Opening remarks, review of practices, agenda, IPR policy reminder, scheduling

    - XXXX–1230 Reports of AHGs X

  - Afternoon session:

    - 1330–XXXX Reports of AHGs Y

    - XXXX–XXXX Main track: tbd

    - XXXX–XXXX HLS track: tbd (chaired by J. Boyce / S. Deshpande)

- Sun. 18 Oct., 2nd day

  - Morning sessions:

    - Main track: XXXX–XXXX tbd

    - HLS track: XXXX–XXXX tbd (chaired by J. Boyce / S. Deshpande)

  - Afternoon sessions:

    - Main track: XXXX–XXXX tbd

    - HLS track: XXXX–XXXX tbd (chaired by J. Boyce / S. Deshpande)

- Mon. 19 Oct., 3rd day

  - 0900–1230 MPEG information sharing session

  - Afternoon sessions:

    - Main track: XXXX–XXXX tbd

    - HLS track: XXXX–XXXX tbd (chaired by J. Boyce / S. Deshpande)

- Tue. 20 Oct., 4th day

  - Morning sessions:

    - Main track: XXXX–XXXX tbd

    - HLS track: XXXX–XXXX tbd (chaired by J. Boyce / S. Deshpande)

  - Afternoon sessions:

    - Main track: XXXX–XXXX tbd

    - HLS track: XXXX–XXXX tbd (chaired by J. Boyce / S. Deshpande)

- Wed. 21 Oct., 5th day

  - 0900–1030 MPEG information sharing session

  - Morning sessions:

    - Main track: XXXX–XXXX tbd

    - HLS track: XXXX–XXXX tbd (chaired by J. Boyce / S. Deshpande)

  - Afternoon sessions:

    - Main track: XXXX–XXXX tbd

    - HLS track: XXXX–XXXX tbd (chaired by J. Boyce / S. Deshpande)

- Thu. 22 Oct., 6th day

  - Morning sessions:

    - Main track: XXXX–XXXX tbd

    - HLS track: XXXX–XXXX tbd (chaired by J. Boyce / S. Deshpande)

  - Afternoon sessions:

    - Main track: XXXX–XXXX tbd

    - HLS track: XXXX–XXXX tbd (chaired by J. Boyce / S. Deshpande)

- Fri.. 23 Oct., 7th day

  - 0900–1400 JVET closing plenary:

    - Report of JAHG JVET-AQ2019 and presentation of JEE6.7/6.9 summary report JVET-AQ0022

    - Establishment of AHGs

    - Establishment of Joint AHG on GSC, and announcement of two F2F meetings Wed./Thu.

    - Review of JVET-AQ0230

    - List of HLS adoptions

    - Approval of output docs (including review as needed)

    - Review of WG 5 meeting recommendations

    - Software database for SEI etc.

    - Software timeline

    - Future planning, a.o.b.

    - Closing of meeting

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (18) (section 3)

- Project development (section 4)

  - AHG1: Deployment and advertisement of standards (1)

  - AHG2: Text development and errata reporting (3)

  - AHG3: Software development (1)

  - AHG3: Test conditions and metrics (2)

  - AHG4: Subjective quality testing and verification testing (0)

  - AHG4: Test and training material (1)

  - AHG5: Conformance test development (0)

  - AHG5: Conformance test development (0)

  - AHG6: Non-reference software assets (0)

  - AHG10: Encoding algorithm optimization (0)

  - AHG13: Film grain technology (3)

  - Implementation studies (0)

  - Profile/tier/level specification (0)

  - AHG15: Gaming content compression (0)

  - AHG16: Hardware implementation complexity (3)

  - AHG17: CfP on video coding technology beyond VVC (4)

  - AHG18: Ultra-low latency and error resilience (4)

  - Video CICP, generic aspects of metadata (0)

- Low-level tool technology proposals (section 5) with subtopics (number counts excluding BoG and summary reports)

  - AHG11/AHG14 and EE1: Neural network-based video coding (11) (section 5.1)

  - AHG6/AHG12: Enhanced compression beyond VVC capability (2) (section 5.2)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics

  - Study of SEI messages in VSEI, VVC, HEVC and AVC (9) (section 6.1)

  - Study SEI messages in VSEI v5 working draft (27) (section 6.2)

  - Study SEI messages in TuC for VSEI (40) (section 6.3)

  - Requests for new elements inVSEI v5 working draft (6) (section 6.4)

  - Potential needs for SEI messages on other topics (7) (section 6.5)

  - Usage of SEI messages for Gaussian splatting (31) (section 6.6)

  - SEI software and showcases (7) (section 6.7)

  - Non-SEI HLS aspects (0) (section 6.8)

- Joint meetings, plenary discussions, BoG reports (0) liaison (2), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (18)

These reports were discussed during XXXX–XXXX on Saturday 17 October 2026 (chaired by JRO).

JVET-AR0001 JVET AHG report: Project Management (AHG1) [J.-R. Ohm, M. Wien (co-chairs), G. J. Sullivan (vice chair)]

JVET-AR0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J. R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice chairs)]

JVET-AR0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice chairs)]

JVET-AR0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs)]

JVET-AR0005 JVET AHG report: Conformance testing (AHG5) [I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice chairs)]

JVET-AR0006 JVET AHG report: Non-reference software assets (AHG6) [Y. Ye (chair), F. Bossen, R. Chernyak, P. de Lagrange, K. Sühring (vice chairs)]

JVET-AR0007 JVET AHG report: Tool assessment (AHG7) [X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice chairs)]

JVET-AR0009 JVET AHG report: SEI message studies (AHG9) [J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs)]

JVET-AR0010 JVET AHG report: Encoding algorithm optimization (AHG10) [K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice-chairs)]

JVET-AR0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien (vice-chairs)]

JVET-AR0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs)]

JVET-AR0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, P. de Lagrange (co-chairs), S. Deshpande, A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs)]

JVET-AR0014 JVET AHG report: NNVC software development (AHG14) [F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice-chairs)]

JVET-AR0015 JVET AHG report: Gaming content compression (AHG15) [C. Lehmann, S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice-chairs)]

JVET-AR0016 JVET AHG report: Hardware implementation complexity (AHG16) [Y. Zhao, I. Moccagatta, K. Naser (co-chairs), H. Huang, T. Ikai, L. Li, X. Li, N. Song, G. Verba (vice-chairs)]

JVET-AR0017 JVET AHG report: Preparation of Call for Proposals (AHG17) [J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice-chairs)]

JVET-AR0018 JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) [S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs)]

JVET-AR0019 Report of Joint AHG on Gaussian splat coding [J. Jung, W. Husak]

# Project development (24)

## AHG1: Deployment and advertisement of standards (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0021 Deployment status of the VVC standard [G. J. Sullivan (SC 29 Chair & VCEG Rapp.)]

## AHG2: Text development and errata reporting (3)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0044 AHG2: Errata items for AVC, HEVC, and VVC [C. Rosewarne (Canon)]

JVET-AR0184 AHG2: Errata report on array and list value initialization specifications in VVC, HEVC, AVC and VSEI [G. J. Sullivan (Dolby Labs), Y.-K. Wang (Bytedance)]

JVET-AR0205 SpecBuild update: text-based authoring, conversion, and maintenance of JVET specifications [D. Podborski, A. M. Tourapis (Apple)] [late]

## AHG3: Software development (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0068 AHG3: Software cleanup of PU structure in VTM [X. Li (Google)]

## AHG3/AHG4: Test conditions and metrics (2)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0095 AHG4/AHG12: Towards more content variety in future all intra test conditions: analysis of ECM intra gains and content characteristics for CTC, CfP, and natural image sequences [Z. Fan, T. Ikai (Sharp)]

JVET-AR0140 AHG10: Suggestion to include BIM and GOP-based RPR in CTC [K. Andersson, P. Wennersten, W. Ahmad, N. Stegmaier (Ericsson)]

## AHG4: Subjective quality testing and verification testing (0)

This section is kept as a template for future use.

## AHG4: Test and training material (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0206 New raw UGC-style outdoor sequences [N. Neumann (Nokia)] [late]

## AHG5: Conformance test development (0)

This section is kept as a template for future use.

## AHG6: Non-reference software assets (0)

This section is kept as a template for future use.

## AHG7: Tool assessment (0+1)

This section is kept as a template for future use.

JVET-AR0162 AHG3/AHG7/AHG17: VTM Encoder Complexity Analysis [X. Li, F. Bossen, J. Gan, E. Alshina, R. Chernyak, P. de Lagrange]

See discussion in 4.16

## AHG10: Encoding algorithm optimization (2+1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0140 AHG10: Suggestion to include BIM and GOP-based RPR in CTC [K. Andersson, P. Wennersten, W. Ahmad, N. Stegmaier (Ericsson)]

See discussion in 4.4

JVET-AR0146 AHG10: Low delay configuration with a larger size QP hierarchy [K. Andersson (Ericsson)]

JVET-AR0185 AHG10: Random-access configuration with a hierarchy of 4 pictures and a new configuration parameter for GOP based RPR [K. Andersson, N. Stegmaier (Ericsson)]

## AHG13: Film grain technology (3)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX). (for discussion jointly with SEI track)

JVET-AR0152 AHG9/AHG13: On film grain synthesis soft conformance [L. Kerofsky, X. Meng, Y. He, M. Karczewicz (Qualcomm)]

JVET-AR0160 AHG9/AHG13: Supporting an exact reference for film grain synthesis [L. Kerofsky, X. Meng, Y. He, M. Karczewicz (Qualcomm)]

JVET-AR0161 AHG9/AHG13: On film grain blending modes [L. Kerofsky, X. Meng, Y. He, M. Karczewicz (Qualcomm)]

## Implementation studies (0)

This section is kept as a template for future use.

## Profile/tier/level specification (0)

This section is kept as a template for future use.

## AHG15: Gaming content compression (0)

This section is kept as a template for future use.

## AHG16: Hardware implementation complexity (3)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0131 [AHG16] Hardware encoder mimicking bit estimation in RDO [Y. Sun, Y. Zhao, E. Alshina (Huawei), M. Wu, J. Yu (HiSilicon)]

JVET-AR0138 [AHG16] RDO count in VTM-24.0 for CfP complexity reporting [K. Naser, F. Le Léannec, F. Galpin, P. de Lagrange (InterDigital)), Y. Zhao, Y. Sun, E. Alshina (Huawei)]

JVET-AR0201 [AHG16] HIC extension for constraining per-CTU RDOs in addition to per-CU RDOs [L Li, W. Choi, Y. Park, K. Choi (Samsung)] [late]

## AHG17: CfP on video coding technology beyond VVC (4)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0137 AHG17: Suggested assessment of CfP proposals [M. Wien, J.-R. Ohm]

JVET-AR0162 AHG3/AHG7/AHG17: VTM Encoder Complexity Analysis [X. Li, F. Bossen, J. Gan, E. Alshina, R. Chernyak, P. de Lagrange]

JVET-AR0183 AHG17: On objective metrics for CfP evaluation [F. Bossen, E. François, F. Galpin, F. Le Léannec, K. Naser (InterDigital)]

JVET-AR0187 On the importance of reduced complexity for wider adoption of the next formal standardization project at JVET beyond VVC [A. Duenas]

## AHG18 Ultra-low latency and packet loss resilience (4)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0112 AHG18: Optimization of Feedback-Based Reference Picture Marking [Y. Xiao, Y. Hu, X. Ma (MLS labs)]

JVET-AR0113 AHG18: Feedback-Aware Reference Preservation for Ultra-Low Latency [Y. Xiao, Y. Hu, X. Ma (MLS labs)]

JVET-AR0175 AHG18: GDR for ULL CTC in unicast scenario [M. Sychev, R. Tasmukhanov, A. Dzugaev, K. Malyshev, S. Ikonin, E. Alshina (Huawei)]

JVET-AR0186 AHG18: Random Access GOP4 Configuration for ULL CTC [I. Gribushin, S. Ikonin, B. Shevchenko, K. Malyshev, X. Ma, E. Alshina (Huawei)]

## Video CICP, generic aspects of metadata (0)

This section is kept as a template for future use.

# Low-level tool technology proposals (11)

## AHG11/AHG14: Neural network-based video coding (9)

JVET-AR0047 [AHG11] [AHG14] Teleconference on NNVC [E. Alshina, F. Galpin]

### Neural network technology contributions (7)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0092 [AHG11] Dynamic convolution for VLOP4 in-loop filtering [J. Lee, M. Jeon, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)]

JVET-AR0093 [AHG11] Multi-GPU training with distributed data parallel for LOP8 [M. Jeon, J. Lee, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS), X. Li (Google)]

JVET-AR0094 [AHG11] Information on objective quality metrics for VVC/H.266 and NNVC comparison [Q. He, J. Lee, M. Jeon, K. Choi (KHU), J. Lee (SJU), S. Park, J. Heo, J. Choi, J. Choi (Hyundai Motor Company)]

JVET-AR0097 [AHG11] Generative Backpropagation for End-to-End Intra-Picture Coding [V. Rizzello, J. Ström, M. Damghanian, D. Liu, P. Wennersten (Ericsson)]

JVET-AR0148 AHG11: Recent Developments in End-to-End Video Coding [Y. Li, S. Liu (Tencent), Z. Chen (Wuhan Univ.), X. Li (Google), E. Alshina (Huawei)]

JVET-AR0202 Guidelines for MPEG AI Codec specifications [F. Galpin, G. Martin-Cocher (InterDigital)] [late]

JVET-AR0203 Considerations on Training Data and Reproducibility for Learned Coding Tools [E. François, G. Martin-Cocher, F. Galpin, S. Cizel, C. Finlay, P. Guillotel (InterDigital)] [late]

### SADL and NNVC software implementation, evaluation and testing (2)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0091 [AHG14] SIMD Optimization of Global Average Pooling in the LOP8 [J. Lee, M. Jeon, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)]

JVET-AR0114 Runtime and Energy Consumption Measurement for Neural Network-based In-loop Filters with SADL Implementation in NNVC [S. Caglar, Z. Chen, M. Y. Saritas, H. B. Dogaroglu, C. Eteke, E. Steinbach (TU Munich)]

## AHG12: Enhanced compression beyond VVC capability (2)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0055 AHG12: Luma-dependent chroma constraint [M. Kim, D. Sim (DigitalInsights)]

JVET-AR0147 AhG12: Further performance improvements regarding the NS2 software [G. Laroche, P. Onno, B. Galmiche, N. Ouedraogo (Canon)]

JVET-AR0209 Cross-check of JVET-AR0147 “Additional improvements of NS2 software at 0.2x under CfP conditions” [F. Le Léannec (InterDigital)] [late] [miss[

# High-level syntax (HLS) and related proposals (X)

## AHG9: Study of SEI messages in VSEI, VVC, HEVC and AVC (9)

### Film grain characteristics/synthesis SEI messages (0+3)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX). (allocated in 4.11, potentially for plenary review)

JVET-AR0152 AHG9/AHG13: On film grain synthesis soft conformance [L. Kerofsky, X. Meng, Y. He, M. Karczewicz (Qualcomm)]

JVET-AR0160 AHG9/AHG13: Supporting an exact reference for film grain synthesis [L. Kerofsky, X. Meng, Y. He, M. Karczewicz (Qualcomm)]

JVET-AR0161 AHG9/AHG13: On film grain blending modes [L. Kerofsky, X. Meng, Y. He, M. Karczewicz (Qualcomm)]

### Encoder optimization information (EOI) (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0099 AHG9: On EOI SEI and ECFI message [B. Li, F. Xing (Hisense)] [late]

### General SEI signalling and extension mechanisms (2)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0046 AHG2/AHG9: On SEI extension mechanism in HEVC [Y.-K. Wang, J. Xu (Bytedance)]

JVET-AR0052 AHG2/AHG9: On SeiExtensionEnabledFlag and SeiExtensionBitsPresentFlag in VSEI, VVC, HEVC, and AVC [Y.-K. Wang, J. Xu (Bytedance)]

### Digitally signed content (DSC) SEI messages (5)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0106 AHG9: On DSC SEI messages [H. Tan, J. Lee, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AR0124 AHG9: Bug fixes for DSC SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AR0188 AHG9: On the latency issue related to DSCV SEI messages [N. Ouedraogo, P. Onno (Canon), B. Völcker, A. Keskikangas (Axis)]

JVET-AR0189 AHG9: On public key management in DSC SEI messages [N. Ouedraogo, P. Onno (Canon), B. Völcker, A. Keskikangas (Axis)]

JVET-AR0190 AHG9: On the provenance information for DSC SEI messages [N. Ouedraogo, P. Onno (Canon), B. Völcker, A. Keskikangas (Axis)]

### Packed regions information (PRI) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0119 AHG9: On the packed regions information SEI message extension [S. He, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

## AHG9: Study JVET-AQ2006 VSEI v5 draft (27)

### General editorial updates (6)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0041 AHG2/AHG9: Full VSEI text with change marks for changes in VSEI v5 WD1 [Y.-K. Wang, J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, G. J. Sullivan, H. Tan, A. M. Tourapis]

JVET-AR0050 AHG9: Miscellaneous changes to the VSEI WD [Y.-K. Wang, J. Xu (Bytedance)]

JVET-AR0090 AHG9: Miscellaneous bug fixes for VSEI v5 WD1 [X. Xu, Y. Li, S. Liu (Tencent)]

JVET-AR0127 AHG9: Miscellaneous aspects on VSEI v5 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AR0129 AHG9: Editorial updates for VSEI v5 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AR0191 AHG9: On a unified interface for SEI messages referencing scalability dimensions [J. Chen, Y. Ye (Alibaba)]

### Film grain region characteristics (FGRC) SEI messages (2)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0049 AHG9: On the FGRC SEI message in the VSEI WD [Y.-K. Wang, J. Xu (Bytedance)]

JVET-AR0103 AHG9: Miscellaneous changes to the FGRC SEI message [M. Damghanian, M. Pettersson, S. Harribey, R. Sjöberg (Ericsson)]

### Danmu information (DI) SEI message (8)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0074 AHG9: On coding of some of the syntax elements in the Danmu SEI message [Y. Yu, J. Gan, H. Yu, M. Li, D. Wang (OPPO)]

JVET-AR0086 AHG9: On DI SEI message [X. Xu, Y. Li, S. Liu (Tencent)]

JVET-AR0104 AHG9: On DI text size signaling [Y. Gao, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AR0105 AHG9: On DI SEI message [Y. Gao, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AR0125 AHG9: On signaling of reaction information in DI SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AR0126 AHG9: On signaling of language information in DI SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AR0133 AHG9: On danmu reply and replication signalling in DI SEI message [C. Kim, J. Nam, H. Tan, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AR0150 AHG9: On authored position for the Danmu information SEI message in VSEI V5 [S. Zhao, Y. He, Y. Gao, L. Kerofsky, M. Karczewicz (Qualcomm)]

### Display overlays information (DOI) SEI message (5)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0098 AHG9: On DOI SEI message [P.-H. Han (Hisense)] [late]

JVET-AR0110 AHG9: On DOI SEI message [Y. Gao, Y. He, M. Karczewicz (Qualcomm)]

JVET-AR0142 AHG9: DOI SEI message with recommendation for prior SEI message processing [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AR0143 AHG9: DOI SEI with constant alpha [J. Boyce, T. Biatken, M. M. Hannuksela (Nokia)]

JVET-AR0199 AHG9: On DOI SEI set and overlay restrictions [J. Boyce (Nokia)] [late]

### Display rectangles (DR) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0111 AHG9: On DR SEI message [Y. Gao, Y. He, M. Karczewicz (Qualcomm)]

### Encoder optimization information (EOI) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0132 AHG9: On EOI SEI message in VSEI V5 [C. Kim, J. Nam, H. Tan, J. Lee, J. Lim, S. Kim (LGE)]

### SEI processing order (SPO) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0128 AHG9: On inclusion of new SEI messages in VSEI v5 into processing chain [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

### Lens optical correction (LOC) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0163 AHG9: On LOC in VSEIv5 draft [L. Kerofsky, M. Karczewicz (Qualcomm)]

### Localization and mapping (LAM) SEI message (2)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0164 AHG9: On LAM in VSEIv5 draft [L. Kerofsky, Y. Gao, M. Karczewicz (Qualcomm)]

JVET-AR0180 AHG9: On the localization and mapping SEI massage [J. Xu, Y.-K. Wang (Bytedance)]

## AHG9: Study JVET-AQ2032 TuC for VSEI (40)

### General (2)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0089 AHG9: Miscellaneous bug fixes for VSEI TuC [X. Xu, Y. Li, S. Liu (Tencent)]

JVET-AR0134 AHG9: Miscellaneous aspects on VSEI TuC [C. Kim, J. Nam, H. Tan, J. Lee, J. Lim, S. Kim (LGE)]

### General SEI signalling and extension mechanisms (5)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0048 AHG9: On the general SEI extension mechanism in the TuC [Y.-K. Wang, J. Xu (Bytedance)]

JVET-AR0061 AHG9: On vsei_relevance of VSEI RBSP [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AR0070 AHG9: On the SEI relevance in the VSEI RBSP [Y. He, M. Karczewicz (Qualcomm)]

JVET-AR0167 AHG9: On versatile SEI message and SEI payload in VSEI TuC [J. Chen, Y. Ye (Alibaba)]

JVET-AR0198 AHG9: On derivation of PayloadExtraBits and PayloadBits in the TuC (a response to JVET-AR0167 aspect 1) [Y.-K. Wang, J. Xu, K. Zhang, L. Zhang (Bytedance), J. Chen, Y. Ye (Alibaba)] [late]

### Constituent rectangles (CR) SEI message (3)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0057 AHG9: On the constituent rectangles SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AR0085 AHG9: On CR SEI message [X. Xu, Y. Li, S. Liu (Tencent)]

JVET-AR0109 AHG9: On CR SEI message [Y. Gao, Y. He, M. Karczewicz (Qualcomm)]

### Enhanced colour format information (ECFI) SEI messages (1+1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0082 AHG9: Interface text for ECFI SEI message [X. Xu, Y. Li, S. Liu (Tencent)]

JVET-AR0099 AHG9: On EOI SEI and ECFI message [B. Li, F. Xing (Hisense)] [late]

Also in 6.1.2

### Encoder optimization information (EOI) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0084 AHG9: On EOI SEI message [X. Xu, Y. Li, S. Liu (Tencent)]

### Gain map (GM) SEI message (3)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0081 AHG9: On Gain map SEI [S. Wang, S. Xie (ZTE)]

JVET-AR0088 AHG9: On GM SEI message [X. Xu, Y. Li, S. Liu (Tencent)]

JVET-AR0151 AHG9: On the byte alignment of the gain map SEI message payload [S. Zhao, Y. He, M. Karczewicz (Qualcomm)]

### Generalized alpha plane information (GAPI) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0059 AHG9: On the generalized alpha plane information SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

### Generalized depth information (GDI) and depth representation SEI messages (4)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0060 AHG9: On the generalized depth information SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AR0087 AHG9: On GDI SEI message [X. Xu, Y. Li, S. Liu (Tencent)]

JVET-AR0173 AHG9: On the generalized depth information SEI massage [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AR0174 AHG9: Depth representation information SEI message extension [J. Xu, Y.-K. Wang (Bytedance)]

### Infrared camera information/adaptation SEI message (6)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0136 AHG9: On IRCI SEI message [C. Kim, J. Nam, H. Tan, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AR0149 AHG9: On infrared code points and the definition of the alpha plane in CICP [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AR0153 AHG9: update of the Infrared camera information SEI message for non-monochrome video [E. François, C.-H. Demarty, A. Ak (InterDigital)]

JVET-AR0157 AHG9: On the infrared camera information SEI message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AR0171 AHG9: On the infrared camera information SEI message [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AR0176 AHG9: Improvement of the IR camera adaptation parameters SEI [V. Zakharchenko, J. Boyce, M. Hannuksela (Nokia)]

### Modality information (MI) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0172 AHG9: On the modality information SEI massage extension [J. Xu, Y.-K. Wang (Bytedance)]

### Neural-network post-filter characteristics (NNPFC) SEI message (2)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0067 AHG9: Comments on NNPF multiple inferences and multiple prompts information signaling [S. Deshpande (Sharp)]

JVET-AR0178 AHG9: On showcasing the use of auxiliary layers for NNPF [S. Tejeda Pastor, Y. Sánchez, R. Skupin, T. M. Borges, C. Hellge, T. Schierl (Fraunhofer HHI)]

### Picture segmentation information (PSI) SEI message (4)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0058 AHG9: On the picture segmentation information (PSI) SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AR0069 AHG9: On the PSI SEI message [Y. He, M. Karczewicz (Qualcomm)]

JVET-AR0083 AHG9: Interface text for PSI SEI message [X. Xu, Y. Li, S. Liu (Tencent)]

JVET-AR0182 AHG9: On the picture segmentation information SEI massage [J. Xu, Y.-K. Wang (Bytedance)]

### Quality metrics (QM) SEI message (5)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0107 AHG9: On QM SEI message [Y. Gao, Y. He, M. Karczewicz (Qualcomm)]

JVET-AR0108 AHG9: On sub-bitstream quality metrics signaling [Y. Gao, Y. He, M. Karczewicz (Qualcomm)]

JVET-AR0115 AHG9: On the QM SEI message [C.-H. Demarty, A. Ak, R. James (InterDigital)]

JVET-AR0130 AHG9: On the QM SEI message for the target picture [Y. He, Y. Gao, M. Karczewicz (Qualcomm)]

JVET-AR0195 AHG9: On QM SEI [T. Biatek, S. Mate, J. Boyce (Nokia)] [late]

### Sample interleaving (SII) SEI message (2)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0166 AHG9: On Sample Interleaving SEI cleanups [L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AR0170 AHG9: On the sample interleaving SEI massage [J. Xu, Y.-K. Wang (Bytedance)]

### Syntax and semantics of Gaussian splat coding SEI messages in TuC (X)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

Update tbd: Documents from 6.6 after discussion in JAHG / joint meetings on GSC for follow-up review of syntax/semantics style, or agreed technical changes.

See details for initial characterization of SEI related contributions under

https://git.mpeg.expert/MPEG/Explorations/GSC/gsc-ahg-mandates/-/work_items/52#note_154270

## Requests for adding to VSEI v5 working draft (6)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0066 AHG9: Implementation and showcase of NNPFC SEI message with nnpfc_mode_idc equal to 2 [S. Deshpande (Sharp)]

JVET-AR0135 AHG9: On TDI SEI message for VSEI version 5 [C. Kim, J. Nam, H. Tan, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AR0141 AHG9: Add CR SEI and SDI SEI extensions to VSEI v5 working draft [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AR0155 AHG9: On the inclusion of the Gain Map auxiliary picture type and gain map information SEI message in VSEI Working Draft v5 [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AR0158 AHG9: On the inclusion of the modality information SEI extension for multispectral pictures in VSEI Working Draft V5 [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AR0168 AHG9: Software implementation and showcase for the PRI SEI massage extension [J. Xu, Y.-K. Wang (Bytedance)]

## Identify potential needs for additional SEI messages, including study of AVC and HEVC SEI messages for use in VVC (7)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0117 AHG9: Support of regions of update and update priority SEI message [S. He, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AR0122 AHG9: On indicating region with auxiliary data [J. Nam, H. Tan, C. Kim, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AR0139 AHG9: On classification information in the annotated regions SEI message [C. Kim, J. Nam, H. Tan, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AR0144 AHG9: Text descriptions SEI purpose for classification label [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AR0145 AHG9: Saliency map auxiliary pictures [J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AR0169 AHG9: Output resampling SEI message [J. Xu, C. Lin, Y.-K. Wang, K. Zhang, L. Zhang, J. Li (Bytedance)]

JVET-AR0194 AHG9: QR Code Information SEI [T. Biatek, S. He, D. Fortin, J. Boyce, M. M. Hannuksela (Nokia)] [late]

## Usage of SEI messages for Gaussian splatting technology (31)

### Summary reports and general documents (1)

JVET-AR0022 Reports of joint exploration experiments on SEI messages for Gaussian splat coding [J. Jung, W. Husak]

Excerpts from WG 7 mxxxxx (for JEE 6.7) and mxxxxx (for JEE 6.9), for information – to be compiled after JAHG review.

See details for initial characterization of SEI related contributions under

https://git.mpeg.expert/MPEG/Explorations/GSC/gsc-ahg-mandates/-/work_items/52#note_154270

### Proposed modifications/experimentation on GSI SEI message (23)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0043 AHG9: [GSC] Explicit packing mode implementation in GSC VSEI framework [Y. Shao, J. Jung, M. Karczewicz (Qualcomm)]

JVET-AR0045 [AHG9] [GSC] [JEE6.9] Proposed update to JVET TuC clause 8.58.1: reconstruction process for GSI SEI message [J. Jung, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AR0051 AHG9: Miscellaneous changes for the GSI SEI message in the TuC [Y.-K. Wang, J. Xu, Q. Yang, Y. Xu, K. Zhang, L. Zhang (Bytedance)]

JVET-AR0053 AHG9: [GSC][JEE6.9] On the coding of signed parameters in the GSI SEI message [H. Lee, J. Do, S. Lee, G. Bang (ETRI)]

JVET-AR0054 AHG9: [GSC][JEE6.7] Reflected mapping of the position LSB components in the GSI SEI message [H. Lee, J. Do, S. Lee, G. Bang (ETRI)]

JVET-AR0062 AHG9: [GSC][JEE6.7][JEE6.9] On the coding of transform syntax in the GSI SEI message [J. Do, H. Lee, S. Lee, G. Bang (ETRI)]

JVET-AR0207 AHG9: [GSC][JEE6.7][JEE6.9] Crosscheck Report for JVET-AR0062 [Y. Shao, J. Jung (Qualcomm)] [late] [miss]

JVET-AR0071 AHG9: [GSC][JEE6.9] On the GSI SEI message syntax improvement [Y. He, J. Jung, L. Kerofsky, R. Hooda, Y. Shao, M. Karczewicz (Qualcomm)]

JVET-AR0073 AHG9: [GSC][JEE6.9] On the descriptor for the GSI, TVGS and HGSI SEI message [Y. He, L. Kerofsky, J. Jung, M. Karczewicz (Qualcomm)]

JVET-AR0075 AHG9:[GSC] On coding of some of the syntax elements in the GSI SEI message [Y. Yu, H. Yu, J. Gan, M. Li, D. Wang (OPPO)]

JVET-AR0079 AHG9: [GSC][JEE6.7] Subpicture-based spatial random access implementation in the GSC VSEI framework [Y. Shao, J. Jung, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AR0200 [GSC][JEE6.7] Crosscheck on subpicture-based spatial random access for the GSC VSEI framework [J. Do, H. Lee, S. Lee, G. Bang (ETRI)] [late]

JVET-AR0080 AHG9: [GSC][JEE6.9] On the GSI SEI message for spatial random access [Y. Shao, J. Jung, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AR0096 AHG9: [GSC][JEE6.7][JEE6.9] Carriage of position LSBs outside the coded video using the GSI SEI message (m78413) [J. Xu, T. Chujoh, Y. Kidani (KDDI)]

JVET-AR0100 AHG9: On the syntax and semantics of the GSI SEI message [S. Harribey, M. Pettersson, M. Damghanian, R. Sjöberg (Ericsson)]

JVET-AR0101 AHG9: On the signalling of component IDs in the GSI SEI message [S. Harribey, M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

JVET-AR0102 AHG9: On the signalling of component bit depths in the GSI SEI message [S. Harribey, M. Pettersson, M. Damghanian, R. Sjöberg (Ericsson)]

JVET-AR0116 AHG9: [GSC][JEE6.9] On guard band indication in GSI SEI message [Y. Hu, X. Ma (MLS Labs)]

JVET-AR0120 AHG9: [GSC][JEE6.9] On supporting sub-block access in GSI SEI message [Y. Hu, X. Ma (MLS Labs), Y. Xu, K. Yang (SJTU)]

JVET-AR0121 AHG9: [GSC][JEE6.9] On LoD information indication in GSI SEI message [Y. Hu, X. Ma (MLS Labs)]

JVET-AR0123 AHG9: Improving GSI SEI with label-based coding [A. Dwivedi, M. Milovanovic, B. Kroon (Philips)]

JVET-AR0208 AHG9: [GSC][JEE6.7] Crosscheck report for JVET-AR0123 [Y. Shao, J. Jung (Qualcomm)] [late]

JVET-AR0156 AHG9: [GSC][JEE6.7] New transform type on GSI SEI message [H.-G. Yang, D.-H. Kim, B.-Y. Choi, J.-G. Kim (KAU), K.-J. Oh, G. Lee (ETRI)]

JVET-AR0181 AHG9: [GSC] Level-of-detail (LoD) signalling and selective reconstruction of Gaussian-splat data [M. Milovanovic, B. Kroon (Philips)]

JVET-AR0192 [GSC] Reference-Aware Component Ordering and Closed-Loop Graph Refinement for GSC VSEI [Y. Xu, D. Zhong, K. Yang, W. Huang, S. Xia, H. Huang (SJTU), Y. Hu, X. Ma (MLS Labs)] [late]

JVET-AR0193 AHG9: [GSC] [JEE6.7] Adaptive SH Degree Coding for VSEI [R. Koo, Y.-G. Kim, E. Ryu (SKKU), J. Lee, J. Lee (KETI)] [late]

### Proposed modifications/experimentation on TVGS SEI message (6)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0063 AHG9: [GSC][JEE6.7] TVGS: triplane video-based implicit Gaussian splatting representation compression [B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)]

JVET-AR0056 AHG9: Crosscheck of JVET-AR0063 (coding, and training + coding) [B. Kroon (Philips)]

JVET-AR0076 AHG9: [GSC][JEE6.7] Crosscheck report on TV-GS (m78312) R. Hooda, J. Jung (Qualcomm)

JVET-AR0064 AHG9: [GSC] Channel-to-spatial mapping in triplane video-based implicit Gaussian splatting representation compression [B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)]

JVET-AR0204 AHG9: Crosscheck of JVET-AR0064 [B. Kroon (Philips)] [late]

JVET-AR0065 AHG9: [GSC] Proposed improvements of the triplane video-based implicit Gaussian splatting (TVGS) SEI message [B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)]

JVET-AR0072 AHG9: [GSC][JEE6.9] On the TVGS and HGSI SEI message syntax improvement [Y. He, L. Kerofsky, J. Jung, M. Karczewicz (Qualcomm)]

JVET-AR0077 AHG9: [GSC][JEE6.9] HLS improvements on TVGS SEI framework (m78313) [R. Hooda, J. Jung, M. Karczewicz (Qualcomm)]

JVET-AR0078 AHG9: [GSC][JEE6.7] QP tuning of TVGS SEI framework (m78314) [R. Hooda, J. Jung, M. Karczewicz (Qualcomm)]

JVET-AR0073 is also related.

### Proposed modifications/experimentation on HGSI SEI message (2)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0042 AHG9: [GSC] On HGSI SEI Software Availability and CTC Results [S. Sasse, S. Lee, Y. Berendsohn, T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

JVET-AR0179 AHG9: [GSC][JEE6.7][JEE6.9] Updates on the HGSI SEI message [S. Sasse, Y. Sánchez, S. Lee, Y. Berendsohn, R. Skupin, T. M. Borges, C. Hellge, T. Schierl (Fraunhofer HHI)]

JVET-AR0072 and JVET-AR0073 is also related.

## SEI software and showcases (7)

Contributions in this area were discussed during XXXX–XXXX on XXday XX October 2026 (chaired by XXX).

JVET-AR0066 and JVET-AR0168 in 6.4 are also describing showcases.

JVET-AR0118 AHG9: Showcase for update priority SEI message [S. He, V. Zakharchenko, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AR0154 AHG9: Showcase of the Gain Map SEI message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AR0159 AHG9: Showcase of the modality information SEI message extension for multispectral picture [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AR0165 AHG9: Sample Interleaving SEI software showcase [L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AR0177 AHG9: Showcase of the infrared camera adaptation parameters SEI [V. Zakharchenko (Nokia)]

JVET-AR0196 AHG9: Showcase on DOI SEI [T. Biatek, S. He, D. Fortin, J. Boyce, M. M. Hannuksela (Nokia)] [late]

JVET-AR0197 AHG9: Showcase on NNPF SEI [T. Biatek (Nokia)] [late] [miss]

## Non-SEI HLS aspects (0)

Section kept as template for future use.

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## General (update plenary etc.)

### Plenary on XXday XX Oct., XXXX-XXXX

The following topics were discussed in this JVET plenary:

- Scheduling for next days (joint meetings with MPEG WGs not clear yet)

  - …

- Liaisons received

  - …

- Reports from tracks …

- Review of …

- Planning of outputs

### Plenary on Friday 23 Oct. 0900-1400

The following topics were discussed in this JVET plenary:

- Remaining revisits were conducted

- Report of JAHG JVET-AQ2019 and presentation of JEE6.7/6.9 summary report JVET-AQ0022

- Establishment of AHGs – see section 9

- Establishment of Joint AHG on GSC, and announcement of two F2F meetings Wed./Thu.

- Review of JVET-AQ0230

- List of HLS adoptions was approved

- Approval of output docs (including review as needed) – see section 10

- Review of WG 5 meeting recommendations – see annex C

- Software timelines were defined – see section 8.1

- It was agreed to establish a new AHG6 on non-reference software assets – see section 9

- Future planning, a.o.b.

- The meeting was closed at 1400 CEST.

## MPEG information sharing meetings

Reports about JVET work status were given during information sharing sessions with other WGs and AGs of the MPEG community, held on Monday 19 Oct. 0900–1230, Wednesday 15 July 0900–1000, and Friday 17 July 1400–1600.

The status and plans for the work in the MPEG WGs and AGs was reviewed at these information sharing sessions, and reports about JVET activities was given by the chair.

## Joint meetings

### Joint sessions on Gaussian splat coding: MPEG WG 4 / Video, MPEG WG 5 / JVET, MPEG WG 7 3D Graphics, and VCEG (ITU-T Q6/21)

##### Joint meeting session 1 (Mon. 19 Oct. 1600-1800)

The first joint session was chaired by Lu Yu (MPEG WG 4 Convenor), Jens-Rainer Ohm (JVET Chair), , and M. Preda (MPEG WG 7 Convenor). The following notes were taken by XXX. The following topics were discussed:

- …

### Joint session XXXX-XXXX XXday XX Oct. on Joint Call for Proposals: MPEG WG 2 / Requirements, MPEG WG 5 / JVET, MPEG AG 5 Visual Quality Assessment, and VCEG Q6/21)

This joint session was chaired by Jens-Rainer Ohm (JVET Chair and WG 5 Convenor), Gary J. Sullivan (Q6/21 Rapporteur), Mathias Wien (AG 5 Convenor), and Igor Curcio (WG 2 Convenor).

This section is based on notes taken by XXX.

## BoGs (0)

Section kept as template for future use.

## Liaison communications (X incoming, X replies generated)

ISO/IEC JTC1/SC29/WG1-N101352 (SC29-N23183) LS on JPEG AI, also addressed to WG 5 as m78006

A draft response was presented by XXX on XXday XX Oct. 0920-1000.

(kept for future use) The liaison responses were also presented by XXXX in the MPEG AG 3 Communication meeting on Thursday XXXX during 1500 – 1700.

# Project planning

## Software timeline (update)

The NNVC 18.0 codebase software was planned to be available 2 weeks after the meeting (31 July). Additional integration of various SADL aspects and harmonization with VTM may be done as appropriate in later versions.

Additional versions on top of VTM 24.0 may be released as appropriate.

An initial update on top of HM18.0 should be generated for submission of the ISO/IEC CD 23008-5 3rd within 2 weeks (July 31), integrating at least the elements for supporting new multiview profiles..

Further updates on top of HM18.0 and JM19.1 software were planned to be released later (integration and updates of SEI messages included in JVET-AN1018 and JVET-AN1017, and merging them with the new HEVC multiview software). Additional versions may be released as appropriate, also investigating the integration of full SHVC support in HM.

As a general rule in software development, a person who is executing a merge shall not be from the same company as the person who submitted that merge request.

## Core experiment and exploration experiment planning (update)

The exploration experiment on improved compression compared to VVC (EE2) had been discontinued by the 42nd meeting. At the 43rd meeting, the exploration experiment on neural network-based video coding (EE1) was discontinued as well.

Joint EEs on Gaussian splat technology are planned to be conducted within the Joint AHG. The proposals that are planned to be investigated are listed in the Joint AHG report JVET-AQ2019, as confirmed during joint meetings (see section 7.3.1). Technology related to SEI messages will be investigated in JEE 6.7 and JEE 6.9, based on common test conditions JVET-AQ2028. The JAHG was mandated to finalize the CTC document and the nine JEE descriptions during a telco on August 11, to be made available with an editing period until August 15 (description of JEEs which have relation to the investigation of performance and improvements of SEI messages will also be made available in JVET-AQ2040).

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- Normative contributions (relating to changes in bitstream/decoder) shall include draft specification text

- Proposals shall contain all details relevant for understanding and be self-contained. In cases where the document is a follow-up of a previous contribution, the overall concept and the novelties should be highlighted at minimum

- Coding tool and encoder optimization proposals shall contain Excel sheets that allow assessment on a per-sequence basis

- Algorithm description text is strongly encouraged for non-normative contributions that are intended to be included in model description documents (VTM, ECM, etc.), and that is required for inclusion in TR drafts.

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions (not exercised currently)

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology. Withdrawing parts of experiments that were intended to show the individual benefits of a tool or parts of a tool is strongly discouraged. Combination tests may not be considered in such cases. Any changes made to individual tools in a combination shall be documented.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). In cases where combinations of different elements are planned to be tested, mutual cross-checking of the individual elements by other parties of the combination is discouraged. The combination must be cross-checked by an independent party. The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-AL2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g., not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow was previously described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

However, it was noted that the link doesn’t seem to exist anymore.

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation was requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups (update)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

Chairs of AHGs had been asked to send draft mandates to JRO before 1800 on 22 Oct., preferably copy from the table below and sending with changemarks or yellow highlight of changes.

Review of AHG plans was conducted during the plenary on XXday XX Oct. 2026 at XXXX–XXXX.

| Title and Email Reflector | Chairs | Interim mtg. |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents. - Produce an update of the overview on IT systems JVET-AQ1012. | J.-R. Ohm, M. Wien (co-chairs), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AQ1006). - Collect reports of errata for VVC, VSEI, HEVC, AVC, CICP, and the published related technical reports. - Coordinate with AHG3 to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Prepare the draft of the next edition of HEVC reference software JVET-AQ1017. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages (for both VTM and HM), and SEI messages in TuC (the latter in a separate branch of VTM). - Discuss and make recommendations on the software development process. - Perform comparative tests of test model behaviour using common test conditions, including HDR, high bit depth and high bit rate. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Prepare drafts of merged and updated CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material, in coordination with other AHGs, as appropriate. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct expert viewing for purposes of subjective quality evaluation. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing, as well as studying objective metrics in that context. - Coordinate with AHG18 on investigating visual impact of data losses. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AQ1008, and further develop related conformance bitstreams. - Prepare the draft of the next edition of HEVC conformance by integrating the relevant bitstreams from JVET-AQ1008. - Coordinate with AHG3 on implementation of the new HEVC multiview profiles. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice-chairs) | N |
| Non-reference software assets (AHG6) (jvet@lists.rwth-aachen.de) - Collect a list of software assets available in JVET software repositories, which had been used during standards development or exploration efforts, but have not been made available as parts of reference software. - Propose actions to make such assets better visible and accessible for JVET members or to the public. | Y. Ye (chair), F. Bossen, R. Chernyak, P. de Lagrange, K. Sühring (vice-chairs) | N |
| Tool assessment (AHG7) (jvet@lists.rwth-aachen.de) - Investigate methodology of tool assessment, such as the aspects of memory access and bandwidth, number of maximum processing cycles, block decoding dependencies, number of context coded bins, pipeline and parallelization. - Study JVET-AO2040 and suggest improvements. - Prepare reporting of tool assessment results as applicable. - Develop methodology of more reliable runtime measurement. - Collaborate with AHG16 on encoding complexity assessment. | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice-chairs) | Y (tel., 2 weeks notice) |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Study JVET-AQ2006 and JVET-AQ2032, including study of SEI messages with different options (when present) and propose improvements. - Maintain the table of the summary of VSEI TuC status that is in an attachment of JVET-AQ2032, and generate a similar table for JVET-AQ2006. - Discuss and refine criteria for the assessment of SEI messages based on JVET-AQ0240. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages. - Study SEI messages specified in HEVC and AVC for potential use in the VVC context and SEI messages in VSEI for potential use in the HEVC or AVC context. - Study the alignment of the same SEI messages in different standards. - Coordinate with AHG3 for software support of SEI messages for JM, HM, and VTM. - Coordinate with the joint AHG on Gaussian splat coding about the design and software implementation of the Gaussian splat coding related SEI messages in the TuC. | J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM. - Study optimized encoding and suitable test settings for noisy materials, such as sequences containing film grain. - Study optimized encoding and tool combinations for low latency and for low complexity. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, higher objective quality, or encoding with improved complexity/performance tradeoff, and coordinate such efforts with AHG3, AHG12, and AHG17. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice-chairs) | N |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools, architectures and content adaptation with NN parameters overfitting. - Study potential improvements of the NNVC CTC document JVET-AP2016. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies and identify suitable material for testing and training. Promote the call for training materials, distribute it, and actively communicate with content owners. - Discuss and propose improved metrics to perform complexity analysis of NN architectures in preparation of the upcoming CfP, and develop complexity reductions of candidate technology. - Investigate device interoperability of NN-based methods on various platforms in coordination with AhG14. - Coordinate with other groups, including SC 29/AG 5 on the evaluation and assessment of visual quality, and with AHG12 on the interaction with ECM coding tools. - Coordinate with AHG14 on items related to NNVC software development, in particular, evaluate the interface in the NNVC for end-to-end optimized AI coded reference pictures. | E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien (vice-chairs) | Y (tel., 2 weeks notice), first on July 30 |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Coordinate with AHG7 to study the performance and complexity tradeoff of video coding tools. - Support the ECM software coordinators in maintenance of the software, associated configuration files and documentation. - Identify any mismatches between the ECM20 software and the algorithm description JVET-AP2025, suggest updates and cleanups to the software as appropriate. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs) | N |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Propose refinements to the draft of the TR 2nd ed. JVET-AQ2020, and investigate for which additional elements described in the TR software might be attached. - Study alternative film grain models and their associated documentation. - In consultation with AHG4, study and define content characteristics and test conditions that are desirable for the study and testing of film grain technologies, and perform an assessment of newly available test materials in that regard. - Investigate metrics for measuring film grain fidelity in itself, or as present in a video. - Discuss the potential need for film grain conformance guidelines. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Study Film grain region characteristics information SEI in JVET-AQ2006 and propose improvements as necessary - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files in coordination with AHG3. | W. Husak, P. de Lagrange (co-chairs), S. Deshpande, A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-18.0 software version (and potential updates), based on updated VTM with adopted contributions and hybrid framework, and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AP2016. Study the impact of the addition of new dataset on the already integrated models. - Continue to bridge the gap between NNVC and most recent VTM as necessary. - Continue to develop missing functionalities for hybrid end-to-end framework exploration. - Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) and Very Low Operation Point (VLOP) configurations. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AQ2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. | F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice-chairs) | Y (tel., 2 weeks notice), first on July 30 |
| Gaming content compression (AHG15) (jvet@lists.rwth-aachen.de) - Identify gaming content application scenarios and their requirements for codec operation. - Identify and characterize required types of content; solicit contributions, collect, and make a variety of gaming content available, in coordination with AHG4 and AG 5. - Produce VTM and ECM anchor encodings according to CTC JVET-AO2027, and provide test results at the next meeting. - Develop and maintain interfaces for supporting use cases of camera parameters and depth maps in gaming applications, including mechanisms for efficient transporting these elements in the coded video bitstream. - Develop and maintain software for estimation of depth maps for gaming sequences, and evaluate the effect of the estimated maps against the original depth maps on coding efficiency where they are available using established accuracy metrics - Evaluate JVET test models (such as ECM, VTM, NNVC, etc.) under the proposed test conditions. - Investigate possibilities to enhance compression capability for gaming content. - Investigate the possibility to estimate depth maps and camera parameters for those gaming sequences where they are not available. - Study conversion of depth maps using integer representation, and identifying efficient bit-depth resolution of depth maps to support identified use-cases that will be an input to compression. - Solicit contributions from industry on typical bitrate/quality/resolution used for gaming content compression. | C. Lehmann, S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice-chairs) | N |
| Hardware implementation complexity (AHG16) (jvet@lists.rwth-aachen.de) - Investigate hardware encoding complexity constraints in typical applications (e.g., mobile devices and hardware transcoding), and identify challenges and evaluate coding tools from hardware encoding implementation perspectives. - Solicit and develop hardware encoding complexity measurements (e.g., maximum number of RDO decisions per CU / CTU), and use them to analyse the performance of test models (e.g., VTM) and coding tools. - Design, develop and maintain a hardware-encoding-mimicking simulation framework on top of test models, e.g., constraining RDO number per CU, constraining coding tree decision complexity (e.g., maximum tree depth, per-node split modes), low-complexity CU/TU-level bit estimation function, etc. - Collaborate with AHG7 on aspects of hardware encoding and decoding complexity perspectives. | Y. Zhao, I. Moccagatta, K. Naser (co-chairs), H. Huang, T. Ikai, L. Li, X. Li, N. Song, G. Verba (vice-chairs) | Y (tel., 2 weeks notice) |
| Preparation of Call for Proposals (AHG17) (jvet@lists.rwth-aachen.de) - Study the draft of the template for proposal description documents in JVET-AQ2022, and suggest updates as appropriate. - Study the complexity reporting template in JVET-AQ2022, suggest improvements, and prepare examples of the encoder complexity analysis for the case of VTM anchors in coordination with AHG7. - Coordinate with AG 5 on preparing logistics and organization of the visual assessment in the context of the CfP. | J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice-chairs) | Y (tel., 2 weeks notice) |
| Ultra-low latency and packet loss resilience (AHG18) (jvet@lists.rwth-aachen.de) - Study JVET-AN2039 common test conditions and propose improvements as appropriate. - Investigate creation of practical simulation software, including network transmission aspects, including consideration of network functionality that can aid timely video-coding based error correction, and conduct performance evaluation. - Identify potential requirements and feasibility of standard based technologies to support ultra-low delay requirements, including packet loss resilient decoding. - Investigate packet loss resilient technologies beyond VVC supporting ultra-low delay coding for interactive and live broadcasting scenarios. - Coordinate with AHG4 and AHG17 on investigating visual impact of data losses and appropriate evaluation procedure development. | S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N XXX) in order to make it easy to reference.

A Joint AHG group was installed with ISO/IEC JTC 1/‌SC 29/‌WG 4 and ISO/IEC JTC 1/‌SC 29/‌WG 7 as follows (discussed and approved XXXX-XXXX on XXday XX Oct.).

| Gaussian Splat Coding (mpeg-gsc@lists.aau.at) - Discuss refinements of use cases and requirements for Gaussian splat coding - Improve and maintain datasets for Gaussian splat coding - Discuss possible refinements of common test conditions for Gaussian splat coding - Prepare anchors for Gaussian splat coding - Investigate new Gaussian splat representation and associated coding technologies - Consolidate the software tools for Gaussian splat coding - Coordinate the activity on Lightweight GS (1F) - Collaborate with AG 5 on preparation of subjective evaluation - Coordinate development of GS coding technology and its standardization across JVET, WG 4, and WG 7 - Study encapsulation/high-level syntax technologies including those based on V-PCC, V3C, and VSEI. - Study GS technology that allows cross-encapsulation compatibility and definition of related conformance - Coordination of the preparation of the Call for proposal - Coordinate the software integration of the codecs under investigation. | Y. Liao, G. Bang (co-chairs on behalf of WG 4), J. Jung, W. Husak (co-chairs on behalf of JVET), A. Zaghetto, H. Hur (co-chairs on behalf of WG 7) | Y (tel., UTC) Aug. 11 1400–1600 Aug. 25 2100–2300 (later cancelled) Sep. 15 0500–0800 Oct. 6 1400–1700 Physical / remote access meeting on 17 and 18 October, full day each |
| --- | --- | --- |

The definition of the Joint AHG was issued as WG 7 N XXX.

It is noted that only one instance of the joint AHG is routinely installed over all MPEG WGs, and at the current meeting, JVET WG 7 was responsible, as well as for managing all input and output documents of the joint activity.

JVET members are invited to subscribe to the reflector of this AHG via https://lists.aau.at/mailman/listinfo/mpeg-gsc.

GS content is stored at https://content.mpeg.expert/data/Explorations/GSC/CTC/

Software related to GSC SEI messages is available under

…

For accessing software, test materials and anchors, JVET members should contact the AHG chairs. In the next meeting, WG 4 will manage input and output documents of the joint activity. Documents relevant for usage of SEI messages in Gaussian splat representation should be submitted both as WG 4 documents and as JVET documents, such that JVET members who are participating via ITU also can get access. Documents not relevant for usage of SEI messages shall only be submitted as WG 4 documents.

For definition of joint exploration experiment on topics relevant for JVET, see section 8.2 of this report. CTC is available as JVET-ARXXXX (duplicating N XXX of WG 7).

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to JRO, MW, and GJS for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N XXX, as noted in section 9.

A list of JVET-only output documents from the current meeting (including links to jvet-experts.org) was issued as a WG 5 document WG 5 N XXX.

JVET-AQ1000 Meeting Report of the 43rd JVET Meeting [J.-R. Ohm] [WG 5 N 408] (2026-08-14)

Initial versions of the meeting notes (d0 … d8) were made available on a daily basis during the meeting.

Remains valid – not updated: JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

Update seems useful after software integration of multiview  and SHVC (integrate content from JCTVC-V1007 and JCT3V-K1003?)

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne, K. Sharman, R. Sjöberg, G. J. Sullivan] [WG 5 N 103]

Primary editor: C. Rosewarne

Remains valid – not updated: JVET-AH1003 Coding-independent code points for video signal type identification (Draft 3) [G. J. Sullivan, A. Tourapis]

Primary editor: G. J. Sullivan.

JVET-AQ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Y.-K. Wang, J. Boyce, B. Bross, I. Moccagatta, C. Rosewarne, K. Sühring, G. J. Sullivan] (2026-08-14)

Primary editor: Y.-K. Wang.

Some issues found in latest editions of VSEI/VVC/HEVC/AVC – see notes under following documents (some aspects on HEVC may be included in JVET-AR1006 instead)

- …

Remains valid – not updated: JVET-AP1005 Future CICP extensions (Draft 3) [J. Boyce, E. Thomas, A. Tourapis]

Primary editor: J. Boyce.

JVET-AQ1006 HEVC additional profiles (Draft 2) [A. Tourapis, Y.-K. Wang, B. Bross, G. J. Sullivan] (2026-07-31)

Primary editor: A. Tourapis.

Contains new 4:4:4 10/12 profiles from JVET-AP0242 (were already in draft 1) plus corrections/clarifications on VSEI interfaces.

The content of JVET-AR1006 is planned to be submitted for DAM ballot as WG 5 N XXX (or integrate into DIS of new edition?). This was reviewed and approved on XXday XX Oct. at XXXX.

Remains valid – not updated: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman (editors)] [WG 11 M 32658]

This document may require merging with JVET-T2013 in the future. JCTVC-P1006 only defines 10-bit sequences for the cases on 4:4:4 and 4:2:2 chroma sampling, and 16-bit sequences for higher bit depth, no 12 bit cases. All sequences specified in this document are available in the JVET content server. It is also noted that JVET-AA1100 could be removed once such a merge is done.

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

Could be removed when SHVC functionality would be integrated into HM. Needs to be checked whether encoder parameters and settings can be configured similarly.

JVET-AQ1008 Conformance testing for HEVC multiview extended and monochrome profiles (Draft 3) [I. Moccagatta, T. Fu, S. Paluri, A. Tourapis] (2026-07-31)

Primary editor: I. Moccagatta

The content of JVET-AQ1008 (as far as already defined multiview profiles are concerned) is planned to be integrated into ISO/IEC 23008-8 (together with previous Amd.1) and submitted for CD ballot as WG 5 N 415. A Request WG 5 N 414 was reviewed and approved on Wednesday 15 July 1205.

Remains valid? – DIS to be approved

Progression into H.265.1 planned for April 2027 (or January via WP3/21?)

It is asserted that 444 multiview profile streams might be added in case that they would be included in a new edition/version of HEVC by April 2027.

Remains valid – not updated: JVET-AC1009 Common test conditions for SHVC [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

Remains valid – not updated: JVET-AJ1011 White paper on HEVC [B. Bross, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang] [AG 3 N 174]

Remains valid – not updated: JVET-AQ1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien] (2026-10-10)

Remains valid – not updated: JCT3V-G1003 3D-AVC Test Model 9 [D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] [WG 11 N 14239]

Remains valid – not updated: JCT3V-K1003 Test Model 11 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] [WG 11 N 15141]

Remains valid – not updated: JVET-AE1013 Common test conditions of 3DV experiments [K. Sühring, M. Wien]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

Remains valid – not updated: JVET-AC1015 Common test conditions for SCM-based screen content coding [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AP1016 AVC with extensions and corrections (Draft 6) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang]

This had been submitted to ITU consent on H.264 16th ed. It is noted that this may not be identical with the final version published by ITU-T.

JVET-AQ1017 Updates of HEVC reference software (draft 1) [K. Sühring, F. Bossen, A. Tourapis] (2026-07-31)

Primary editor: K. Sühring

Integration of new software developed in context of new multiview profiles, and support for SEI messages in VSEIv4.

Submitted for DIS ballot as WG 5 N XXX, was reviewed and approved on XXday XX Oct. at XXXX.

Progression into H.265.2 planned for April 2027 (or January via WP3/21?)

It is asserted that enabling signalling 444 multiview profiles might be added in case that they would be included in a new edition/version of HEVC by April 2027. Also, an integration of SHVC support into HM (as per JVET-AQ0187) would be desirable for DIS or FDIS stage, to remove the separate SHM software package from the reference software specification

Remains valid – not updated: JVET-AN1018 HEVC with extensions and corrections (Draft 3) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis]

Primary editor: Y.-K. Wang.

This had been submitted to ITU consent on H.265 11th ed. It is noted that this may not be identical with the final version published by ITU-T.

Remains valid – not updated: JVET-AN1019 VSEI with extensions and corrections (Draft 1) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang]

Primary editor: J. Boyce.

This had been submitted to ITU consent on H.274 4th ed. It is noted that this may not be identical with the final version published by ITU-T.

No output: JVET-Axx1020 through JVET-Axx1099

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman]

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet. See note under JCTVC-P1006 above.

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AN2001 VVC with extensions and corrections (Draft 1) [Y.-K. Wang, B. Bross, M. M. Hannuksela, G. J. Sullivan]

Primary editor: Y.-K. Wang.

This had been submitted to ITU consent on H.266 4th ed. It is noted that this may not be identical with the final version published by ITU-T

Remains valid – not updated: JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23) [Y. Ye, A. Browne, S. Kim]

Primary editor: Y. Ye.

New elements from notes elsewhere in this report (kept for future use):

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

Remains valid – not updated: JVET-AM2005 Additions and corrections for VVC version 4 (Draft 13) [G. J. Sullivan, B. Bross, M. M. Hannuksela, Y.-K. Wang] (2025-08-01)

Primary editor: G. J. Sullivan.

JVET-AQ2006 Additional SEI messages for VSEI version 5 (Draft 1) [Y.-K. Wang, J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, G. J. Sullivan, H. Tan, A. Tourapis] (2026-08-21)

Primary editor: Y.-K. Wang.

New elements or improvements (list had been reviewed and approved in JVET plenary on XXday XX Oct. at XXXX):

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AJ2007 Guidelines for NNVC software development [F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li]

This might benefit from an update in the future, regarding some details of SADL implementation.

Remains valid – not updated: JVET-AN2008 Conformance testing for versatile video coding (Draft 8) [I. Moccagatta, F. Bossen, S. Iwamura, P. de Lagrange]

Primary editor: I. Moccagatta

Remains valid – not updated: JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2) [F. Bossen, K. Sühring, X. Li] [WG 5 DIS N 322)]

Software relating to H.266.2 and ISO/IEC 23090-16 can be found at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/2nd-edition.

Primary editor: F. Bossen.

[kept for future meetings] A request for next edition(s) of reference software(s) could be issued in the next meeting, and also CD(s) could be produced by the same meeting, as SC 29 is meeting beginning of February and could approve such projects without issuing a ballot. Issuing CD(s) by the January meeting (if mature enough) could avoid that an ISO version would lag behind an ITU version which might be submitted for consent in July 2026. Another consideration could be to start the process for ISO early enough in 2026, such that FDIS and ITU consent could be synchronized in April 2027.

Remains valid – not updated: JVET-AP2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring]

Remains valid – not updated: JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AP2016 Common test conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, F. Galpin, R.-L. Liao, S. Liu, A. Segall]

Remains valid – not updated: JVET-AI2017 Common test conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz, Y. Ye]

This document contains test conditions relating to HDR as well as SDR, although ECM testing does not typically include HDR results. Note that a separate output document was issued for HDR test conditions in JVET-AO2011.

It is to be noted that further updates on this document are not planned, and it not intended to be used as basis for future video coding standards development.

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu]

Links to test sequences need to be updated due to the change of the content server.

JVET-AQ2019 Description of algorithms version 16 and software version 18 in neural network-based video coding (NNVC) [F. Galpin, Yue Li, Yun Li, D. Rusanovskyy, T. Shao, J. Ström, L. Wang] (2026-09-11)

New elements in text and software from notes elsewhere in this report:

…

JVET-AQ2020 Film grain synthesis technology for video applications ed. 2 (Draft 6) [W. Husak, P. de Lagrange, M. Radosavljević, A. Tourapis] (2026-07-31)

See notes in section 4.11 under JVET-AQ0222.

A DTR was issued as WG 5 N XXX (approved on XXday XX Oct. at XXXX)

JVET-AQ2021 Joint Call for Proposals on video compression with capability beyond VVC [J.-R. Ohm, M. Wien, F. Bossen] [WG 5 N 416] (2026-07-15)

Developed from latest version of JVET-AP2026 in JVET-AQ0201, and approved during joint meeting on Tuesday 14 July.

JVET-AQ2022 Draft template for proposal description documents of responses to the Joint CfP on video compression with capability beyond VVC [J.-R. Ohm, M. Wien, E. Alshina, F. Bossen, R. Chernyak, J. Gan, P. de Lagrange, X. Li] (2026-07-15)

Developed from latest version of JVET-AQ0202

No output: JVET-Ax2023, JVET-Ax2024

Remains valid – not updated: JVET-AP2025 Algorithm description of Enhanced Compression Model 20 (ECM 20) [M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang]

Remains valid – not updated: JVET-AO2027 Common test conditions for gaming applications [J. Sauer, R. Chernyak, S. Puri, S. Thiebaud] (2026-01-31)

JVET-AQ2028 Common test conditions on Gaussian splat coding [B. Kroon, P. Rondao Alface, J. Jung, G. Sandri] [WG 5 N 420] (2026-08-14)

JVET copy of an identical WG 5 document, which may have further updates during JAHG editing period.

Remains valid – not updated: JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC [F. Bossen, J.-R. Ohm, M. Wien] [AG 5 N 180] (2025-12-19)

Remains valid – not updated: JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10) [S. Liu, J. Chen, C. Hollmann, J. Ström]

Primary editor: S. Liu.

The content of the document was submitted for approval by SG21, and will be made publicly available as ITU-T H.Sup.22.

Remains valid – not updated: JVET-AI2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu, C. Hollmann]

JVET-AQ2032 Technologies under consideration for future extensions of VSEI (version 13) [S. Deshpande, J. Boyce, J. Chen, M. M. Hannuksela, Y. He, K. Sühring, G. J. Sullivan, H. Tan, A. Tourapis, J. Xu, X. Xu] (2026-08-28)

Primary editor: S. Deshpande

New elements or improvements (list had been reviewed and approved in JVET plenary on XXday XX Oct. at XXXX):

…

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AP2033 Report of verification tests on VVC multi-layer coding [O. Chubach, S. Iwamura, P. de Lagrange, M. Wien] [AG 5 N 201)]

Remains valid – not updated: JVET-AI2034 Call for new HDR materials for future video coding development [E. François, W. Husak, S. Iwamura, D. Rusanovskyy, A. Segall, M. Wien] [WG 5 N 312)]

Remains valid – not updated: JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding [S. McCarthy, B. Chen]

Remains valid – not updated: JVET-AG2036 Call for training materials for neural network-based video coding tool development [E. Alshina, F. Galpin, S. Liu, M. Wien] [WG 5 N 266)]

Remains valid – not updated: JVET-AJ2037 Report on subjective quality testing of the FGC SEI message (AG 5 N 140) [P. de Lagrange, W. Husak, M. Wien] [AG 5 N 140)]

Remains valid – not updated: JVET-AM2038 White paper on VSEI [J. Boyce, S. McCarthy, S. Deshpande, G. J. Sullivan, Y. Sanchez, Y.-K. Wang] [AG 3 N209]

An update of the white paper on VVC (from 2021) should be done in a future meeting, removing the section on VSEI and referencing to JVET-AM2038.

Remains valid – not updated: JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience [S. Ikonin, S. Deshpande, V. Zakharchenko] (2025-10-31)

JVET-AQ2040 Description of SEI-related exploration experiments on Gaussian splat coding [J. Jung, W. Husak] (2026-08-14)

SEI related parts from JEE6.7 (WG 5 N 430) and JEE 6.9 (WG 5 N 432) to be copied into this JVET document

No output: JVET-AO2041 and higher

# Future meeting plans, expressions of thanks, a.o.b., and closing of the meeting

The draft of the WG 5 recommendations (see Annex C) was reviewed and approved in JVET at XXXX–XXXX on Friday 23 October.

Future meeting plans were established with the following general guidelines (assuming face-to-face meetings), foreseen for the phase of developing the next-generation video standard from January 2027 onwards:

- Meeting under ITU-T SG21 auspices when it meets (ordinarily starting meetings on the weekend before the first week and closing it around noon on the Wednesday of the second week of the SG21 meeting – a total of 10-11 meeting days under the consideration that JVET cannot meet on the first day of the SG21 meeting while parent-body plenaries are held), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Tuesday or Wednesday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 10-11 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined, or AHG meetings might be scheduled prior to the meeting. In cases of online meetings, no sessions should be held on weekend days, such that meetings would typically start two days earlier.

Some specific future meeting plans were established as follows:

- During 13 – 22 January 2027, 45th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU, to be conducted as physical meeting with remote participation,

- During 17 – 28 April 2027, 46th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 7 – 16 July 2027, 47th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI, to be conducted as physical meeting with remote participation,

- During 20 – 29 October 2027, 48th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN, to be conducted as physical meeting with remote participation,

- During 12 – 21 January 2028, 49th meeting under ISO/IEC JTC 1/‌SC 29 in Thessaloniki, GR, to be conducted as physical meeting with remote participation,

- During 25 March – 5 April 2028, 50th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 19 – 28 July 2028, 51st meeting under ISO/IEC JTC 1/‌SC 29 auspices in Fukuoka, JP, to be conducted as physical meeting with remote participation ng,

- During October 2028, 52nd meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

- During January 2029, 53rd meeting under ITU-T SG21 auspices, date and location t.b.d.

- During April 2029, 54th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

The agreed document deadline for the 45th JVET meeting was planned to be Wednesday 6 January 2027.

Marius Preda was thanked for his great support in maintaining and improving the document site jvet-experts.org. Institut Mines-Télécom was thanked for hosting the sites.

XXX was thanked for the excellent hosting of the 44th JVET meeting in Hangzhou. XXX were thanked for the help in preparation, registration and organisation of the meeting. XXX were thanked for their responsiveness, dedication and daily help in the setup of the meeting and technical facilities.

The 44th JVET meeting was closed at approximately XXXX hours CST on Friday 23 October 2026.

# Annex A to JVET report:
List of documents

Dates and times in the table below are in Paris/Geneva time (2 hr. ahead of UTC at the time of the current meeting). It is noted that, if title or authorship of a document deviates from the title or author list in the body of the report, the list of documents in this annex contains the correct title and authors.

# Annex B1 to JVET report:
List of meeting participants attending in person

The participants who were personally present at the meeting site of the forty-fourth meeting of the JVET, according to a sign-in sheet circulated in the JVET meeting rooms (approximately XXX people in total), were as follows:

# Annex B2 to JVET report:
List of meeting participants attending remotely

The remote participants of the forty-fourth meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately XXX people in total, not including those who had attended the meeting in person at least part-time (see Annex B1), and not including those who attended only the joint sessions with other groups), were as follows:

# Annex C to JVET report:
Recommendations of the 25th meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Experts Team with ITU-T SG21

ISO/IEC JTC 1/SC 29/WG 5 N XXX