| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O11 Filename: VCEG-O11.doc Generated: 27 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Thomas Wedi Institute of Communication Theory and Signal Processing University of Hannover, Germany | Tel: Fax: Email: | +49 511 7625304 +49 511 7625333 wedi@tnt.uni-hannover.de |
| Title: | H.26L Interpolation Filtering Ad Hoc Report |  |  |
| Purpose: | Report |  |  |

_____________________________

# Introduction

In Santa Barbara an Ad Hoc group on interpolation filtering was formed. The following topics were defined:

- Resolve open aspects with the tentative adoption of VCEG-N31 from Nokia. Incorporate the VCEG-N31 techniques into the test model if the group's tentative adoption is verified to be appropriate.

- Interpolation with padding (VCEG-N19)

- Adaptive interpolation filtering (VECEG-N28)

# Activities

### Reduced Complexity: VCEG-N31

In VCEG-N31 of the Santa Barbara meeting a reduced complexity, block based interpolation method was proposed:

“The coding efficiency and its decoder complexity are comparable to the direct interpolation method currently employed in the TML software. The main difference is the encoder complexity.

In case of 1/4 pel precision when encoder is implemented by pre-calculating 1/2 pixel values and interpolating 1/4 pixels on the fly direct interpolation requires memory 9x the size of the original frame to store 1/2 pixels comparing to 4x required by the proposed method. Moreover, interpolation of 1/4 pixels in case of direct interpolation is more complex since scaling and clipping is required in addition to bilinear interpolation.

For 1/8 precision interpolation when direct interpolation is used in the encoder all the sub-pixel values should be calculated before motion estimation is started which requires 64 times the memory of the original frame. Proposed method allows to pre-calculate 1/2 and 1/4 pixels which storage require 16 times the memory of the original frame and 1/8 pixels can be calculated as required by bilinear interpolation.”

A pixel based version of this technique was integrated into the TML software.

### Interpolation with padding

In Santa Barbara the improved interpolation with padding of VCEG-N19 from Sharp was adopted:

“Sub-pel-positions in interpolation are not uniform for an out-of-frame motion block. To eliminate this variety, we suggest a change in the interpolation process. It is to pad the image boundaries by several integer pixels in both encoder and decoder. The out-of-frame sub-pel positions will be interpolated from the padded integer pixels, on the same sub-pel positions as the inside-frame part.”

This technique was integrated into the software.

### Adaptive interpolation filtering

In Santa Barbara an adaptive interpolation filter scheme was presented in VCEG-N28. In VCEG-O16 more results on this adaptive interpolation filter are presented.