| # ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ Fifteenth Meeting: Pattaya, Thailand, 4-7 December, 2001 | Document VCEG-O21 Filename: VCEG-O21.doc Generated: 27 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 Shawmin Lei Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Fax: Email: Tel: Fax: Email: | 360-817-7531 360-817-8436 ssun@sharplabs.com 360-817-8446 360-817-8436 shawmin@sharplabs.com |
| Title: | GMVC: Core Experiment Report |  |  |
| Purpose: | Information |  |  |

_____________________________

1. Introduction

In VCEG-N16 [1], we proposed a motion vector coding technique, where global motion parameters are used for motion vector interpolation. The technique is proposed as a general-purpose video coding technique for low-bit-rate video coding. During the Santa Barbara meeting, the expert group decided to set up a Core Experiment to verify the proposed coding technique. This document is to provide the results of the core experiment as supporting evidence for the revised GMVC proposal presented in VCEG-O20 [2].

2. P frame coding

The coding efficiency experiments are conducted mainly following the “Simulation Common Conditions” specified in VCEG-N81 [3]. The proposed motion vector coding method is integrated into the latest TML8.5 [4] software to compare against the TML8.5 performance. The 300-frame CIF-format “coast-guard” sequence, 300-frame CIF-format “foreman” sequence, and 300-frame “Stefan” sequences in both QCIF and CIF formats are added to the testing set to demonstrate the advantage of the proposed GMVC technique. The motion search range parameter is set to 32; and the RD optimization option is turned on for the experiments.

RD curves for all the testing sequences are shown in the Appendix 1. Detailed experimental results are available in VCEG-O21.xls. There is almost no performance difference for sequences without global motion, such as “containet_qcif,” “news_qcif,” “silent_qcif,” and “paris_cif.” Table 1 summarizes the results of other sequences, where global motions are present. Bit savings can be observed at very low bit rate (or when QP is high). For example, when QP is 28, GMVC saves 15% for foreman_cif, 14% for coast_cif, 8.7% for mobile_cif, and 6.3% for foreman_qcif.

Side-by-side comparison is arranged for the meeting. QP’s in bold in Table 1 are chosen for the subjective evaluation. In summary, the conventional “copy” mode introduces some annoying artifacts when global motion is present in the scene, for example, in “coast” and “foreman” sequences. With the proposed GMVC technique, these artifacts can be significantly reduced.

Table 1. Comparison of TML8.5 and the proposed GMVC (IPPP)

Table 2. Comparison of TML8.5 and the proposed GMVC (IBBP)

3. B frame coding

Even though GMVC is proposed for H.26L baseline and B frame syntax has not been changed, questions have been raised in the Santa Barbara meeting regarding the B frame coding. Therefore, experiments are conducted in this Core Experiment to analyze the coding efficiency when B frame coding is enabled.

Subset of the sequences used in Section 2 is used for B frame coding, with two B frames inserted into each pair of P frames. The results are summarized in Table 2. RD curves are available in the Appendix 2 and detailed results are available in VCEG-O21.xls.

For “foreman” and “coast_guard” sequences, the average bit savings become less because the B frame coding. However, it is interesting to see that GMVC gives some bit savings for “Stefan” sequences even at high bit rate.

Side-by-side comparison is arranged for the meeting. QP’s in bold in Table 2 are chosen for the subjective evaluation.

4. Complexity analysis

A simple global motion estimation method is developed for the proposed codec. The global motion estimation takes very limited computation, less than 2.0% of the total encoding computation.

An example encoding profile is available in the Appendix 3. The first 50 frames of “Stefan_cif” is encoded and profiled. The global motion estimation functions take only 1.0% of the total encoding time.

Currently, the global motion estimation and the TML local motion search are both executed. It appears that the computation of local motion search potentially can be significantly reduced when the global motion information is considered. Therefore, our GMVC method will not introduce any significant extra requirement for the total computation in that case.

5. Low-complexity encoding

Comparing to the results shown in VCEG-N16 [1]. GMVC improvement over TML becomes less when Rdopt is turned on and search range is set to 32.

Table 3. Comparison of TML8.5 and GMVC with Rdopt off and search range of 16 (IPPP)

We have tested some sequences according to the testing condition used in VCEG-N16 [1], with 5 reference frames, search range of 16, and RDopt off. Results are summarized in Table 3. The RD curves are shown in Appendix 4. The results show that GMVC can give quite some benefit when encoding requirement is low.

6. Summary

We have demonstrated the advantages of the proposed Global Motion Vector Coding (GMVC) technique. This coding method gives certain benefit for low-bit-rate video coding by reducing bit rate and enhancing visual quality; it is especially useful for video applications where the global or camera motion is highly expected. The benefit of GMVC is more visible at low-complexity encoding cases. It takes only one bit per frame to signal off the GMVC option if the GMVC is not to be used; the overhead for coding efficiency is negligible. As we discussed in Section 4, there is no significant complexity requirement for GMVC. Therefore, we recommend VCEG to adopt the proposal [2] into the new TML.

# References

[1] Shijun Sun and Shawmin Lei, "Motion Vector Coding with Global Motion Parameters," document VCEG-N16, ITU-T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[2] Shijun Sun and Shawmin Lei, " Global Motion Vector Coding (GMVC)," document VCEG-O20, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, Thailand, 4-7 December 2001.

[3] Gary Sullivan and Gisle Bjontegaard, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan source Material,” document VCEG-N81, ITU- T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[4] Gisle Bjontegaard, "H.26L Test Model Long Term Number 8 (TML-8) draft0," document VCEG-N10, ITU-T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[5] Gisle Bjontegaard, "Calculation of average PSNR differences between RD curves," document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

[6] Mathias Wien, "GMVC Core Experiment Results" document VCEG-O33, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, Thailand, 4-7 December 2001.

# Appendix

# 1. R-D curves for P-frame coding (Section 2)

# 2. R-D curves for B-frame coding (Section 3)

# 3. Encdoing Profile (Section 4)

Stefan_cif; Qp=24; RDopt=ON; Search Range=32; No. of Refs=5; 50 frames; 30 Hz IPPP; PC=P-III 800MHz. Functions in bold are used for global motion estimation.

Func Time     %         Function

---------------------------------------------------------

757442.581  30.1   _SetupFastFullIntegerSearch (mv-search.obj)

748353.151  29.8   _FastFullIntegerSearch (mv-search.obj)

388565.939  15.5   _UMVPelY_11 (refbuf.obj)

205095.510   8.2   _SetupLargerBlocks (mv-search.obj)

91959.489   3.7    _FastLine16Y_11 (refbuf.obj)

79835.054   3.2    _find_sad (mv-search.obj)

76747.276   3.1    _FastPelY_14 (refbuf.obj)

22272.686   0.9    _writeMB_bits_for_4x4_luma (macroblock.obj)

20488.836   0.8    _GME0 (globalmotion.obj)

18344.577   0.7    _dct_luma (block.obj)

16831.367   0.7    _UnifiedOneForthPix (image.obj)

8377.699   0.3     _ChromaCoding_P (macroblock.obj)

7469.183   0.3     _dct_chroma (block.obj)

6814.542   0.3     _HalfPelSearch (mv-search.obj)

6313.062   0.3     _UMVPelY_14 (refbuf.obj)

5813.450   0.2     _SingleUnifiedMotionSearch (mv-search.obj)

5630.865   0.2      _warp0_sad (globalmotion.obj)

4681.080   0.2     _writeMotionInfo2NAL_Pframe (macroblock.obj)

4122.718   0.2     _RDCost_Macroblock (rdopt.obj)

3892.260   0.2     _PutPel_14 (refbuf.obj)

3324.148   0.1     _QuarterPelSearch (mv-search.obj)

2887.756   0.1     _LumaResidualCoding_P (macroblock.obj)

2842.176   0.1     _levrun_linfo_inter (uvlc.obj)

2453.206   0.1     _RD_Mode_Decision (rdopt.obj)

2311.761   0.1     _sign (mv-search.obj)

2291.655   0.1     _writeMB_bits_for_16x16_luma (macroblock.obj)

2240.007   0.1     _SetMotionVectorPredictor (mv-search.obj)

1906.658   0.1     _writeSyntaxElement_UVLC (uvlc.obj)

1594.054   0.1     _RDCost_Intra4x4_Block (rdopt.obj)

1394.448   0.1     _dct_luma2 (block.obj)

1209.707   0.0     _store_coding_state (rdopt_coding_state.obj)

1127.612   0.0     _intrapred_linfo (uvlc.obj)

1014.342   0.0     _RD_Intra4x4_Mode_Decision (rdopt.obj)

1002.688   0.0     _find_sad2 (macroblock.obj)

1001.633   0.0     _restore_coding_state (rdopt_coding_state.obj)

776.583   0.0      _dquant_linfo (uvlc.obj)

776.583   0.0      _mvd_linfo2 (uvlc.obj)

456.328   0.0      _DeblockMb (loopfilter.obj)

410.617   0.0      _intrapred_luma (block.obj)

348.384   0.0      _intrapred_luma_2 (block.obj)

331.859   0.0      _writeMB_bits_for_CBP (macroblock.obj)

304.241   0.0      _EdgeLoop (loopfilter.obj)

296.243   0.0      _n_linfo2 (uvlc.obj)

279.774   0.0      _writeMB_bits_for_DC_chroma (macroblock.obj)

257.141   0.0      _read_one_new_frame (image.obj)

252.276   0.0      _truncate (globalmotion.obj)

218.910   0.0      _PutPel_11 (refbuf.obj)

217.097   0.0      _intrapred_chroma (block.obj)

193.380   0.0      _find_snr (image.obj)

177.706   0.0      _Intra4x4_Mode_Decision (rdopt.obj)

164.645   0.0      _report (lencod.obj)

150.605   0.0      _write_one_macroblock (macroblock.obj)

90.962   0.0       _writeMB_bits_for_AC_chroma (macroblock.obj)

60.017   0.0       _GetVecDif (loopfilter.obj)

57.596   0.0       _writeMB_bits_for_luma (macroblock.obj)

55.599   0.0       _cbp_linfo_inter (uvlc.obj)

44.272   0.0       _start_macroblock (macroblock.obj)

38.385   0.0       _CheckAvailabilityOfNeighbors (macroblock.obj)

36.971   0.0       _proceed2nextMacroblock (macroblock.obj)

28.755   0.0       _encode_one_frame (image.obj)

24.904   0.0       _terminate_macroblock (macroblock.obj)

21.146   0.0       _writeMB_bits_for_Dquant (macroblock.obj)

17.703   0.0       _levrun_linfo_c2x2 (uvlc.obj)

15.786   0.0       _main (lencod.obj)

15.489   0.0        _warp0 (globalmotion.obj)

10.154   0.0       _SetRefFrameInfo_P (macroblock.obj)

10.108   0.0       _Configure (configfile.obj)

9.403   0.0        _get_mem2D (lencod.obj)

8.938   0.0        _ResetFastFullIntegerSearch (mv-search.obj)

8.379   0.0        _trace2out (uvlc.obj)

7.929   0.0        _encode_one_macroblock (macroblock.obj)

5.414   0.0        _InitializeFastFullIntegerSearch (mv-search.obj)

4.792   0.0        _ParseContent (configfile.obj)

2.868   0.0        _ClearFastFullIntegerSearch (mv-search.obj)

2.230   0.0        _free_mem4global_buffers (lencod.obj)

2.147   0.0        _terminate_slice (filehandle.obj)

1.983   0.0        _cbp_linfo_intra (uvlc.obj)

1.623   0.0        _get_mem_mv (lencod.obj)

1.521   0.0        _information_init (lencod.obj)

1.413   0.0        _get_mem4global_buffers (lencod.obj)

1.250   0.0        _free_mem_mv (lencod.obj)

0.610   0.0        _start_sequence (filehandle.obj)

0.541   0.0        _writeCBPandCoeffs2NAL (macroblock.obj)

0.494   0.0        _GetConfigFileContent (configfile.obj)

0.427   0.0        _terminate_sequence (filehandle.obj)

0.413   0.0        _init_img (lencod.obj)

0.300   0.0        _malloc_slice (lencod.obj)

0.277   0.0        _select_picture_type (header.obj)

0.250   0.0        _get_mem3Dint (lencod.obj)

0.240   0.0        _clear_rdopt (rdopt.obj)

0.218   0.0        _PictureHeader (header.obj)

0.216   0.0        _get_mem3D (lencod.obj)

0.214   0.0        _InitMotionVectorSearchModule (mv-search.obj)

0.155   0.0        _init_frame (image.obj)

0.144   0.0        _get_mem2Dint (lencod.obj)

0.142   0.0        _PatchInp (configfile.obj)

0.121   0.0        _InitRefbuf (refbuf.obj)

0.100   0.0        _SliceHeader (header.obj)

0.079   0.0        _interpolate_frame_0 (image.obj)

0.072   0.0        _init_slice (image.obj)

0.067   0.0        _writeUVLC2buffer (uvlc.obj)

0.048   0.0        _interpolate_frame (image.obj)

0.035   0.0        _start_slice (filehandle.obj)

0.028   0.0        _init (lencod.obj)

0.027   0.0        _init_rdopt (rdopt.obj)

0.026   0.0        _symbol2uvlc (uvlc.obj)

0.024   0.0        _write_reconstructed_image (image.obj)

0.017   0.0        _SequenceHeader (header.obj)

0.013   0.0        _delete_contexts_MotionInfo (cabac.obj)

0.007   0.0        _clear_coding_state (rdopt_coding_state.obj)

0.006   0.0        _free_slice (lencod.obj)

0.002   0.0        _init_stat (lencod.obj)

0.002   0.0        _init_coding_state (rdopt_coding_state.obj)

0.002   0.0         _gm_init (globalmotion.obj)

0.001   0.0        _free_img (lencod.obj)

0.000   0.0        _delete_contexts_TextureInfo (cabac.obj)

0.000   0.0      _UMVLine16Y_11 (refbuf.obj)

4. RD curves for low-complexity encoding (Section 5)

Search range=16; Rdopt=OFF; No. of Refs=5.

Frame rate: Foreman_qcif=10fps; Stefan_qcif=15 fps; foreman_cif, Stefan_cif, coast_cif=30fps.