| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O24r1 Filename: VCEG-O24r1.doc Generated: 26 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Ragip Kurceren, Marta Karczewicz, Jani Lainema Nokia Research Center 6000 Connection Drive, 2-700 Irving, TX, 75039 | Tel: Fax: Email: | +1 972 894 6907 +1 972 894 4589 ragip.kurceren@nokia.com |
| Title: | Improvements on CABAC |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

- Context-based Adaptive Binary Arithmetic Coding (CABAC) technique [1] [2], adopted into high-complexity profile of H26L-TML provides improved coding efficiency and adaptivity to source statistics compared with the alternative entropy coding using universal table of variable length codes (UVLC). However, as also pointed out in D.151 [3], the current design of the context models in CABAC is not optimal with regard to coding efficiency. In this document we investigate the performance of new context models for encoding of (RUN, LEVEL) pair in intra-coding. We also provide results of adaptive scan experiments for CABAC.

- The methods proposed in this documents are simplified versions of those introduced in VCEG-N41.

## New Context Models for (RUN, LEVEL) Encoding

- In the current CABAC in TML8.5, a (RUN, LEVEL) pair is encoded as follows: the runs and levels are first classified based on block/coefficient type: scanning mode, coefficient type (DC/AC), and coding mode (inter/intra or 16x16intra). Then the levels and runs are mapped onto a sequence of binary decisions, so-called bins. And each bin is assigned to a context based on its bin number which is defined as the location of the bin in the sequence representing the symbol. For example, for level encoding, there are 4 contexts. The first one is for the first bin; the second is for the second while the third context is for the rest of bins representing the magnitude of the level. The remaining context is used for the sign of the level. In summary, the context modeling of levels and runs are based on bin number and the coding/coefficient type.

In this document, we investigate new context models assigned to a bin representing levels of coefficients depending on the previously encoded (arithmetic encoder) or decoded (arithmetic decoder) coefficient level within the block. More specifically, similar to the current design, the symbols are first classified based on coefficient/coding mode, and then the following context models are applied for bins of levels and runs, respectively,

Proposed Context Model for Levels

if (bin_nr>3)

bin_nr=3;

end

if (Prev_level>MAX_LEVEL)

prev_level=MAX_LEVEL;

end

context_nr = (bin_nr-1)*MAX_LEVEL+Prev_level

Proposed Context Model for Run

if (bin_nr>2)

bin_nr=2;

end

if (level>MAX_RUNL)

level=MAX_RUNL;

end

context_nr = (bin_nr-1)*MAX_RUNL+level

where the Prev_level is initialized to zero at the beginning of each block. Note that for double-scan, this variable is initialized at the beginning of each scan, i.e., twice per block.

| QP | Container | Foreman | News | Silent | Tempete | Mobile | Paris | Average |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -2.86 | -3.65 | -2.54 | -4.28 | -2.67 | -3.23 | -3.7 | -3.28 |  |
| High | -1.86 | -2.16 | -1.53 | -2.56 | -1.88 | -2.48 | -2.58 | -2.15 |  |

Table 1: Performance improvement (Bjontegaard Delta bitrate savings in %) of new context models in CABAC Intra coding. QP = 2, 5, 10, 16 is used for low QP and QP = 16, 20, 24, 28 for high QP.

Table 1 illustrates the results of the proposed scheme in terms of the bitrate reduction in % from TML-8.5 with MAX_LEVEL=3 and MAX_RUNL=1. The test conditions from VCEG-M75 [4] were followed with an exception of coding all the frames as I-frames and additional QP results are included to illustrate further results.

Since double-scan is specifically designed for UVLC coding and, for the approach described above, results in initialization of context models twice per block, we have investigated the effect of removing double-scan for the proposed scheme in CABAC. Furthermore, in the following, we include results of alternative, adaptive scan methods.

In Adaptive Intra Coding, Annex I of H.263+, two scans in addition to zigzag scan are employed for encoding of the transform coefficients of intra-coded blocks. The scan order is selected based on the prediction direction used for the transform coefficients. In the following we provide results of a similar approach in H.26L. The additional scan orders and the corresponding relationship between the intra-prediction types and the scan order, that are considered in this document, are illustrated in Table 2 and Table 3, respectively.

| 1 | 2 | 3 | 6 |  | 1 | 2 | 4 | 9 |  | 1 | 3 | 6 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 5 | 7 | 10 |  | 3 | 5 | 6 | 11 |  | 2 | 4 | 7 | 11 |
| 8 | 9 | 11 | 15 |  | 7 | 8 | 10 | 13 |  | 5 | 8 | 9 | 14 |
| 12 | 13 | 14 | 16 |  | 12 | 14 | 15 | 16 |  | 12 | 13 | 15 | 16 |
| Vertical scan |  |  |  |  | Secondary diagonal |  |  |  |  | Diagonal scan |  |  |  |

Table 2 Additional Scan Orders

| Prediction type | Scan |
| --- | --- |
| 0 | Normal |
| 1 | Secondary diagonal |
| 2 | Vertical |
| 3 | Diagonal |
| 4 | Vertical (transposed) |
| 5 | Secondary diagonal (transposed) |

Table 3 Intra-Prediction Modes

| QP | Container | Foreman | News | Silent | Paris | Mobile | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -3.11 | -3.97 | -3.84 | -4.29 | -4.11 | -4.25 | -4.20 | -3.97 |
| High | -3.53 | -2.70 | -2.96 | -2.90 | -3.87 | -3.31 | -3.09 | -3.19 |

Table 4 Performance improvement (Bjontegaard Delta bitrate savings in %) of new context models with adaptive scan in CABAC Intra coding. QP = 2, 5, 10, 16 is used for low QP and QP = 16, 20, 24, 28 for high QP.

Similar to results illustrated in Table 1, Table 4 lists the results of the proposed context models but, additionally, adaptive scan scheme described earlier is employed. As can be observed from Table 4 the improvements by new context models increase with the adaptive-scan schemes. We now observe bitrate savings from 2.70 to 4.29% compared to TML 8.5. This translates to additional 0.7 % on low QPs and 1.0% on high QPs on top of the improvements by modified run/level coding.

## Conclusion

New context models are proposed for encoding of run/level pair in CABAC Intra coding. The results demonstrate average bitrate savings from 1.5 to 4.3%. We have also investigated the effects of removing double-scan from CABAC and introducing adaptive scan. The resulting performance of the proposed contexts including adaptive scan scheme indicates that additional average improvements of around 1% can be realized.

# References

- [1]	Detlev Marpe, Gabi Blättermann, Thomas Wiegand, “Adaptive Codes for H.26L“, document VCEG-L13, , ITU-T Video Coding Experts Group, Eibsee, Germany, 09-12 January 2001.

- [2]	Detlev Marpe, Gabi Blättermann, Guido Heising, Thomas Wiegand, “Further Results for CABAC Entropy Coding Scheme", document VCEG-M59, ITU-T Video Coding Experts Group, Austin, Texas, USA, 2-4 April, 2001.

- [3]	Detlev Marpe et al., "Improving coding efficiency of H.26L using CABAC", document D.151, ITU-T Q6/16, WP 3/16 ,Porto Seguro, 28 May – 8 June 2001.

[4]	Gisle Bjontegaard, " Recommended Simulation Conditions for H.26L", document VCEG-M75, ITU-T Video Coding Experts Group, Austin, Texas, USA, 2-4 April, 2001.

[5]	Chul-Woo Kim and Sang Woo Rhie, " Adaptive Double Scan", document VCEG-M69, ITU-T Video Coding Experts Group, Austin, Texas, USA, 2-4 April, 2001.

[6]	Gisle Bjontegaard, " Calculation of average PSNR differences between RD-curves ", document VCEG-M33, ITU-T Video Coding Experts Group, Austin, Texas, USA, 2-4 April, 2001.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Inc. |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX-75039 |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 972 8944188 |  |  |
| Fax | +1 972 8944589 |  |  |
| Email | Marta.Karczewicz@nokia.com |  |  |
| Place and date of submission | 1/2/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ### JVT |  |
| Title |  | Improvements on CABAC |  |
| Contribution number |  | VCEG-O24 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: