| # ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 3-7 Dec., 2001 | Document VCEG-O28 Filename: VCEG-O28.doc Generated: 26 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Hideaki Kimata NTT Cyber Space Laboratories 1-1 Hikari-no-oka, Yokosuka Japan | Tel: Fax: Email: | +81 468 59 3124 +81 468 59 2829 kimata@nttvdt.hil.ntt.co.jp |
| Title: | Proposal of GMVC syntax |  |  |
| Purpose: | Proposal |  |  |

_____________________________

1. Introduction

Global motion vector coding (GMVC) technique was proposed and the Core Experiment [1] was established in the 14th VCEG meeting. GMVC has a potential of increasing coding efficiency in sequences, which have global motion feature, such as pan or zoom. In this document, we confirm the efficiency of GMVC in terms of representing motion vectors, and propose the new syntax for GMVC entropy coding that achieves higher coding efficiency in those sequences.

2. Proposal of new syntax

We redesign the new syntax in MB level. The other parts, such as the syntax in Picture level, and the way of both motion vector coding and GMVC_COPY coding are the same as original one. Figure 1 illustrates the proposed syntax (indicated by P1). GMVC_SEL is 1 bit flag indicating whether motion vectors are calculated from GMV or not. GMVC_16 is represented by the syntax where GMVC_SEL is switched to on.

Figure 1. Proposed syntax, P1

Table 1. Codeword number of GMVC_SEL

And for experimenting the efficiency of entropy coding in case of using GMVC_SEL, another syntax is examined. Figure 2 illustrates this syntax. In this document, this syntax is indicated by R2 and the original syntax described in specified in VCEG-N77r1 [1] is indicated by R1 in this document. In R2, GMVC_16 is also represented by GMVC_SEL = on, accordingly it consumes only 1bit similarly to R1.

Figure 2. Reference syntax; R2

3. Experiments

Experiments are conducted following the conditions specified in VCEG-N77r1 [1]. CABAC is not used in this experiment. The syntax of GMVC is integrated into the TML-8.4 software.

3. 1. Global Motion Estimation

In order to evaluate the efficiency of the syntax itself, we experimented with another GME [2][3] in addition to one specified in VCEG-N77r1. In this document, we call the GME described in VCEG-N77r1 GME#1, and one described in this section GME#2.

In GME#2, global motion parameters are statistically calculated from the motion vectors searched by Block Matching in a whole picture. Since the results of normal motion estimation could be used, or the motion vectors could be simultaneously obtained in normal motion estimation, the additional complexity for GME could be negligible.

3. 2. Results and evaluation

Table 2 shows bitrate and PSNR in 1/4 pel MC with GME#2, and Table 3 shows these values in 1/8 pel MC with GME#2. And Table 4 shows these values in 1/8 pel MC with GME#1.

3. 2. 1. Coding efficiency of GMVC

As shown in both Table 2 and 3, GMVC achieves the most bit saving in foreman (QCIF), stefan (QCIF), mobile (CIF), tempete (CIF), coast (CIF), and stefan (CIF), with every type of syntax. These sequences have global motion feathers. In other sequences, GMVC consumes more bits than TML-8.4.

3. 2. 2. Coding efficiency of entropy coding GMVC syntax

As shown in Table 2, Table 3, and Table 4, P1 syntax achieves the most bit saving in all sequences where GMVC is useful. And in comparison R1 and R2, there is no remarkable tendency about bit saving each other. From the results, we can see GMVC_SEL based syntax  (P1) achieves more bit saving than the syntax specified in VCEG-N77r1 [1], regardless of GME method. And proposed syntax, P1, achieves the best coding efficiency of three, which representing GMVC.

Table 2. Comparison of GMVC and TML-8 in 1/4 pel MC with GME#2

Table 3. Comparison of GMVC and TML-8 in 1/8 pel MC with GME#2

Table 4. Comparison of GMVC and TML-8 in 1/8 pel MC with GME#1

4. Summary

We have confirmed the efficiency of GMVC itself in sequences that have global motion feature. And we have demonstrated the advantages of the proposed syntax for GMVC regardless of GME method. Therefore, we recommend VCEG to adopt the proposal, i.e. GMVC_SEL based GMVC technique into the new TML.

# Legal statement

NTT has intellectual property related to this contribution and has chosen to subscribe to subclause 2.2 for ITU-T patent policy under the condition of reciprocity.

# References

[1] Shijun Sun, Shawmin Lei, and Mathias Wien, " Core Experiment Description: Motion Vector Coding with Global Motion Parameters," document VCEG-N77r1, ITU-T Video Coding Experts Group (VCEG) Meeting, 2001.

[2] Kazuto Kamikura and Hiroshi Watanabe, "Video coding for digital storage media using hierarchical interframe scheme," Visual Communications and Image Processing '90, SPIE, Vol. 1360, pp. 1540-1550 (1990).

[3] Kazuto Kamikura, Hiroshi Watanabe, "Global Motion Compensation in Video Coding," Electronics and Communications in Japan, Part 1, Vol. 78, No. 4, pp. 91-102 (1995).

Appendix R-D curves

Figure (a). R-D curves of foreman_qcif in 1/4 pel MC

Figure (b). R-D curves of stefan_qcif in 1/4 pel MC

Figure (c). R-D curves of mobile_cif in 1/8 pel MC

Figure (d). R-D curves of tempete_cif in 1/8 pel MC

Figure (e). R-D curves of coast_cif in 1/8 pel MC

Figure (f). R-D curves of stefan_cif in 1/8 pel MC