| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 3-7 Dec., 2001 | Document VCEG-O29 Filename: VCEG-O29.doc Generated: 26 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Hideaki Kimata NTT Cyber Space Laboratories 1-1 Hikari-no-oka, Yokosuka Japan | Tel: Fax: Email: | +81 468 59 3124 +81 468 59 2829 kimata@nttvdt.hil.ntt.co.jp |
| Title: | Proposal of Global Motion Compensation enhancing Global Motion Vector Coding scheme |  |  |
| Purpose: | Proposal |  |  |

_____________________________

1. Introduction

Global motion compensation (GMC) techniques, such as adopted in MPEG-4 Advanced Coding Efficiency Profile and the “reference picture resampling” technique in Annex P of H.263+, have two effective functionalities;

- Reduction of motion vector information for blocks into global motion vector, and

- Reconstruction of motion vector information for each pixel from global motion vector, i.e. warping.

Global Motion Vector Coding (GMVC) specified in VCEG-N77r1 [1] or VCEG-O28 [2] could have the functionality (1), but not the functionality (2). We believe the functionality (2) is also effective to increase coding efficiency.

Regarding the functionality (2), GMC certainly needs more complexity to reconstruct motion vector information pixel by pixel. However it does not need the additional complexity to reconstruct pixel image data. The each pixel data can be reconstructed by the same way of TML-8. Therefore the additional complexity of GMC to GMVC is necessary only for calculating motion vectors for each pixel.

From the tradeoff between coding efficiency and complexity point of view, GMC is simply positioned to the enhancement of GMVC. In this proposal, we introduce a GMC technique, whose syntax is simply enhanced from that of GMVC, and demonstrate the efficiency of GMC in terms of coding efficiency compared with GMVC. Moreover we propose another GMC syntax for high bit rate cases where quantization scales are smaller than specified in VCEG-N77r1 [1].

Exploiting two types of GMC and GMVC, we propose the complexity scalable GMC scheme covering from low bitrates to high bitrates.

2. Proposal of GMC

2. 1. GMC syntaxes

We design the syntax for GMC based on GMVC. Since the syntax based on GMVC_SEL achieves highest coding efficiency in GMVC technique as shown in VCEG-O28 [2], the syntax of GMC is similar to with GMVC_SEL method. We introduce two types of syntax for GMC in MB level. Figure 1 illustrates these syntaxes, indicating by GMCW (GMC with Warping) and GMCWM (GMC with Warping plus normal Motion vector). GMVC can be treated as one mode of GMC, then the syntax is indicated by GMCNW (GMC with No Warping) in the following. To switching GMCNW, GMCP, and GMCWM. Picture level syntax becomes illustrated in Figure 2. Table 1 lists codeword number of newly defined syntaxes. The GMCW syntax is almost the same as that of GMCNW (GMVC), and GMCWM is simply extended form the GMCW syntax.

Figure 1. Proposed syntax in MB level, GMCW and GMCWM

Figure 2. Proposed syntax in Picture level

Table 1. Codeword number of GMC_FLAG, GMC_warping, GMC_type, and GMC_SEL

2. 2. GMCW

The difference between GMCW and GMCNW (GMVC) is only the calculation unit as for motion vectors. In GMCW the unit is a pixel, however in GMCNW (GMVC) it is a 4x4 block. The way of motion vector coding is almost the same as in GMCNW (GMVC). The motion vector of a pixel at (x, y) can be derived as the equation (1).

v(x,y)=roundr0HV+xVrx+yHry+xyrxyHV                      (1)

2. 3. GMCWM

In GMCWM, not only Inter16x16 MB but also any other Inter MB can be selected to GMC MB (GMC_SEL=on). A GMC MB can have motion vector data per each Block, where segmentation information is indicated by MB mode. Then the motion vector of a pixel at (x, y) can be derived as the equation (2). The mv(x,y) is motion vector data indicated within GMCWM syntax at a Block, where a pixel (x,y) exists.

v(x,y)=roundr0HV+xVrx+yHry+xyrxyHV+mv(x,y)               (2)

2. 4. Complexity scalability

2. 4. 1. From GMCNW (GMVC) to GMCW

The additional complexity from GMCNW (GMVC) to GMCW is only regarding calculation of motion vectors, both in an encoder and a decoder. The complexity for GME in an encoder is the same as for GMCNW (GMVC).

2. 4. 2. From GMCW to GMCWM

In a decoder, the additional complexity from GMCW is only regarding calculation of motion vectors described above, so it is negligible. In an encoder, besides a decoder’s, the additional complexity for ME to obtain mv(x,y) information is necessary. This additional ME scheme is simply constructed by the same way of that as if an additional reference picture could exist.

3. Experiments

Experiments are conducted following the same conditions specified in VCEG-N77r1 [1], and additionally, MIT test sequence (CIF, 150 frames) as for test sequence is used. And to experiment in high bitrate cases, quantization scales (4, 8, 12) are used. CABAC is not used in this experiment. The syntax of GMC is integrated into the TML-8.4 software.

3. 1. Results and evaluation

Table 2, Table 3, Table 4, and Table 5 show bitrate and PSNR in 1/8 pel MC, and Table 6, Table 7, Table 8, and Table 9 show these values in 1/4 pel MC. In Table 3, Table 5, Table 7, and Table 9, each average bitsaving or average db gain is against TML-8’s. And in Table 5 and Table 9, average value is calculated in quantization scale 4, 8, 12, and 16. PSNR plots which have remarkable difference in 1/8 pel MC mode are illustrated in Appendixes. All plots are illustrated in VCEG-O29.xls.

3. 2. 1. Coding efficiency of GMCW

As shown in all tables, GMC (GMCNW (GMVC), GMCW, and GMCWM) achieve more bit saving in foreman (QCIF), stefan (QCIF), mobile (CIF), tempete (CIF), coast (CIF), and stefan (CIF), than TML-8. These sequences have global motion feathers. In other sequences, GMC consume more bits than TML-8.

And shown in Table 3 and Table 7, there is tendency in low bitrate cases that GMCW achieves the most bit savings in 1/8 pel MC mode, and GMCNW (GMVC) achieves the most bit savings in 1/4 pel MC mode.

3. 2. 2. Coding efficiency of GMCWM

And from Table 5 and Table 9, there is strong tendency in high bitrate cases that GMCWM achieves the most bit savings in both 1/8 pel and 1/4 pel MC modes.

Table 2. Comparison of GMCs in 1/8 pel MC for low bitrate

Table 3. Average comparison of GMCs in 1/8 pel MC for low bitrate

Table 4. Comparison of GMCs in 1/8 pel MC for high bitrate

Table 5. Average comparison of GMCs in 1/8 pel MC for high bitrate

Table 6. Comparison of GMCs in 1/4 pel MC for low bitrate

Table 7. Average comparison of GMCs in 1/4 pel MC for low bitrate

Table 8. Comparison of GMCs in 1/4 pel MC for high bitrate

Table 9. Average comparison of GMCs in 1/4 pel MC for high bitrate

4. Summary

We have proposed two types of GMC enhanced from GMVC and demonstrated their efficiency in low bitrate and high bitrate cases. In low bitrates, GMCW achieves the best coding efficiency in 1/8 pel MC, and GMCNW (GMVC) achieves the best of it in 1/4 pel MC. In high bitrates, GMCWM achieves the best coding efficiency both in 1/8 pel and 1/4 pel MC modes. A series of GMC (GMCNW, GMCW, and GMCWM) are constructed within a simple syntax, and in the complexity scalable scheme both in an encoder and a decoder.

Therefore, we recommend VCEG to adopt the proposal, i.e. GMC and its scalable coding scheme into the new TML.

# Legal statement

NTT has intellectual property related to this contribution and has chosen to subscribe to subclause 2.2 for ITU-T patent policy under the condition of reciprocity.

# References

[1] Shijun Sun, Shawmin Lei, and Mathias Wien, " Core Experiment Description: Motion Vector Coding with Global Motion Parameters," document VCEG-N77r1, ITU-T Video Coding Experts Group (VCEG) Meeting, 2001.

[2] Hideaki Kimata, " Proposal of GMVC syntax," document VCEG-O28, ITU-T Video Coding Experts Group (VCEG) Meeting, 2001.

Appendix R-D curves

Figure (a). R-D curves of mit_cif in 1/8 pel MC, low bitrates

Figure (b). R-D curves of mit_cif in 1/8 pel MC, high bitrates