| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O31 Filename: VCEG-O31.doc Generated: 26 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Mathias Wien and Achim Dahlhoff Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52074 Aachen, Germany | Tel: Fax: Email: | +49-241-80-276-81 +49-241-80-22196 wien@ient.rwth-aachen.de dahlhoff@ient.rwth-aachen.de |
| Title: | # Intra Coding using Variable Block Sizes |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

Intra coding in the TML allows for 4x4 and 16x16 prediction modes. For blocks of size 4x4 pixels there are 6 including a DC prediction and 5 direcitional prediction modes. 16x16 pixel blocks can be predicted using DC, horizontal, vertical and plane prediction.

In this paper, the application of variable block sizes for intra prediction and transform coding using  variable block size transforms (VBST) is proposed. The prediction techniques employed in the current TML are extended and modified to fit the larger block sizes. For transform coding, the integer transforms that were proposed for ABT are employed.

Intra prediction is employed to reduce the energy of the prediction error that has to be transmitted. However, the correlation in the intra predicition error signal is higher than for inter coded blocks. Therefore, the application of larger transform blocks is beneficial and leads to an improved intra coding gain.

# New Intra Coding Method

For the new intra coding method, the TML intra macroblock modes are replaced by the macroblock modes used for inter coding (16x16, 16x8, 8x16, 8x8, 8x4, 4x8, 4x4). Therewith, the number of subblocks inside a macroblock varies from 1 to 16.

For each macroblock mode, the 6 prediction modes that are used for TML 4x4 intra prediction can be employed. The modifications to these prediction modes are described in the next subsection. For the new intra coding method, the macroblock mode and the prediction modes for the subblocks have to be encoded and transmitted to the decoder. Note that the overall number of encoded prediction modes strongly decreases with the proposed method, since the larger subblock sizes require fewer prediction modes.

We use the following notation: The pixels in a block with N pixels and M rows are referenced as

P(i,j),    i = 0,..,N-1,   j = 0,..,M-1

The vector of adjacent pixels that are used for prediction is denoted by V(n), e.g. for the pixels above the current block: V(n) = P(n,-1), n = 0,…,N-1.

## Prefiltered Prediction

The prediction modes given in the TML can be extended to larger block sizes. The prediction method is simply extended in the same way, as is was done in the 4x4 block.

However, a straight forward application of the TML intra prediction modes for larger block sizes leads to severe artifacts in the prediction signal. These artifacts have to be avoided to enable an efficient prediction.

Here, a solution is presented that uses prefiltered prediction for block boundaries that are larger than 4 pixels. Before applying the prediction vector V(n), it is prefiltered by a size dependent lowpass filter. We use a 3-tap filter for boundaries of length 8 and a 5-tap filter for boundaries of length 16.  The coefficients of the boundary filters are given below.

| Length | Filter Coefficients |  |
| --- | --- | --- |
| 8 | 7 18 7 | /32 |
| 16 | 3 7 12 7 3 | /32 |

Figure 1 Prediction of a 8x8 block using the TML prediction mode 1 (skew prediction from top left). The last row of the top adjacent macroblock is used for prediction. Left: Mode 1 is applied without prefiltering. Right: Prefiltering is applied first.

At the boundaries of the prediction vector, the last value is repeated. Thus, the influence of that pixel is strengthened at the boundary. An example for the impact of the prefiltering on the predicted block is given in Figure 1.

## Transform Coding

For transform coding of the remaining prediction error Varibale Block Size Transforms (VBST) are employed. These were proposed for inter coding with ABT. The employed integer transforms as well as the quantization are identical to the ones used for inter coding. For details, see [VCEG-N49].

## Encoding of the Prediction Modes and the Coefficients

For encoding of all symbols of the new intra mode, CABAC is employed. The macroblock modes that are used for intra macroblock modes in the TML are replaced by the TML inter macroblock modes. The inter MB mode coding technique is employed here.

The prediction modes for the subblocks inside a macroblock are encoded in pairs using the TML intra prediction mode coding technique. The formation of two prediction modes with (Prob0,Prob1) into one code word remains unchanged. For mode 1 (16x16), a second subblock does not exist. Here, the probability of the prediction mode is encoded as (Prob0,0).

The quantized coefficients are encoded using the technique described for ABT coefficients coding [VCEG-N49]. Here, a separate set of contexts is used for the intra coefficients. Currently, the contexts are initialized with the same values as for inter coefficients coding.

## Mode Decision

The overall performance of the proposed intra coding scheme is highly dependent on a careful selection of the proper transform size and the applied prediction mode. The mode selection for variable block size intra coding is performed in two steps. First, all possible prediction modes for each macroblock mode are applied and the best prediction mode is selected. Second, the macroblock mode with the minimum overall cost is identified.

Currently, a separate RD selection is performed. The rate is estimated using 1st order entropy measurements which were generated on a training set and stored in a look-up table. The integration of the intra mode decision into the TML RDopt routines will be given later.

We use the rate/distortion criterion for decision of the macroblock mode and prediction mode. The best mode is used for encoding. The complexity of the mode decision can be reduced, e.g. by using the SATD for prediction mode selection.

# Simulation Results

The technique was implemented into TML8.0 using the ABT transform, quantization, and coefficients encoding. These techniques were presented at the last meeting. CABAC was used for encoding of the coefficients. The TML RD optimization was used for the reference curves. For the new intra coding method, the RD optimization described above was used.

Results are given for the test sequences defined in the common conditions document [VCEG-N81]. Three settings were tested:

TML	: the original TML intra coding technique

VBST	: the proposed intra coding technique

SVBST	:	simplified VBST technique. Here the maximum transform block size is 
 			reduced to 8x8 coefficients. The prefiltered prediction is not changed. This 
 			technique corresponds to the SABT proposal [VCEG-O30].

In order to judge the performance of the scheme on a broader basis, the encoder were modified to allow for I-frame only coding. The sequences were encoded intra-only with a skip of 5. I.e. 50 frames of a 300 frames sequences were coded to average the intra coding performance. QP = 16, 20, 24, 28 was used for the experiments.

The resulting RD plots of all three coders are given in the Appendix of this document. Table 1 gives the difference in terms of rate and PSNR as calculated using the Bjontegaard measurement method [VCEG-M33]. Both, VBST and SVBST intra coding are compared to the performance of the original TML intra coding technique.

Visual examples for the improved subjective performance of VBST and SVBST are given in Figures 2 and 3.

| Test Sequences | VBST average % bit savings (avsnr) | VBST average dB PSNR gain (avsnr) | SVBST average % bit savings (avsnr) | SVBST average dB PSNR gain (avsnr) |
| --- | --- | --- | --- | --- |
| Container QCIF | 1.05 | 0.077 | -0.72 | -0.045 |
| Foreman QCIF | 2.83 | 0.168 | 2.11 | 0.125 |
| News QCIF | 3.25 | 0.270 | 2.86 | 0.238 |
| Silent QCIF | 5.23 | 0.288 | 4.70 | 0.259 |
| Paris CIF | 2.47 | 0.199 | 1.96 | 0.159 |
| Mobile CIF | 3.01 | 0.270 | 2.81 | 0.252 |
| Tempete CIF | 4.09 | 0.293 | 3.49 | 0.251 |
| average | 3.13 | 0.22 | 2.46 | 0.18 |

Table 1: Objective comparison between the two VBST proposals and TML intra coding.

## Discussion

The RD results reveal an improved overall performance of the proposed scheme with respect to the original TML intra coding technique. The gains increase at higher rates. Some minor additional gain can be expected if the VBST intra mode selection is integrated into the TML RD optimization routines.

The subjective quality is generally improved by the the VBST schemes. More detail is preserved.

# Conclusion

A variable block size intra coding technique is presented that includes variable block size transforms for encoding of the prediction error. The scheme improves the RD performance of the intra coding scheme by up to 0.5 dB. The visual quality of the intra frames is improved.

We propose to set up a core experiment on VBST intra coding. The technique is proposed to be integrated into a high complexity high efficiency profile. The impact of the new intra coding technique on inter frame coding has to be studied. In particular, the impact on cyclic I-frames has to be investigated.

# References

[VCEG-N10]	Gisle Bjontegaard, "H.26L Test Model Long Term Number 8 (TML-8) draft0," 
document VCEG-N10, ITU-T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[VCEG-N49]	M. Wien and A. Dahlhoff, 'New ABT Results using CABAC', ITU-T Q6/16 VCEG, 
Doc. VCEG-N49r1, Santa Barbara, CA, Sept. 2001.

[VCEG-M62]	M. Wien and A. Dahlhoff, 'Adaptive Block Transforms', ITU-T Q6/16 VCEG, 
Doc. VCEG-M62r1, Austin, TX, April 2001.

[VCEG-N81]	G. Bjontegaard and G. Sullivan, ' Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material', ITU-T Q6/16 VCEG, Doc. VCEG-N81, Santa Barbara, CA, Sept. 2001.

[VCEG-M33]	Gisle Bjontegaard, 'Calculation of average PSNR differences between RD 
curves,' document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

# IPR Statement

The patent holder has chosen to subscribe to subclause 2.2 for ITU-T patent policy under the condition of reciprocity.

Figure 2 :  Example for the improved subjective quality provided by coding with VBST. Depicted is the 1st frame of Paris CIF at QP=20. Left: TML intra coding. Right: VBST intra coding.

Figure 3 :  Example for the improved subjective quality provided by coding with SVBST. Depicted is the 1st frame of Paris CIF at QP=20. Left: TML intra coding. Right: SVBST intra coding.

# Appendix – Rate/Distortion Plots

container qcif TML intra

QP     Rate	PSNR

28    37669	28.78

24    59846	31.58

20    89797	34.47

16   131486	37.22

container qcif VBST intra

QP     Rate	PSNR

28    40650	29.19

24    62923	32.00

20    93778	34.86

16   134401	37.55

container qcif SVBST intra

QP     Rate	PSNR

28    41290	29.13

24    63930	31.96

20    94856	34.85

16   135256	37.54

foreman qcif TML intra

QP     Rate	PSNR

28    32214	28.70

24    54192	31.40

20    88053	34.16

16   133292	36.91

foreman qcif VBST intra

QP     Rate	PSNR

28    35434	29.35

24    57522	31.86

20    91072	34.55

16   135352	37.27

foreman qcif SVBST intra

QP     Rate	PSNR

28    35962	29.33

24    58002	31.86

20    91406	34.55

16   135694	37.26

news qcif TML intra

QP     Rate	PSNR

28    45488	28.57

24    70670	31.65

20   103294	34.75

16   142980	37.78

news qcif VBST intra

QP     Rate	PSNR

28    48246	29.23

24    72168	32.11

20   105206	35.15

16   144664	38.13

news qcif SVBST intra

QP     Rate	PSNR

28    48222	29.18

24    72162	32.06

20   105539	35.16

16   144398	38.12

silent qcif TML intra

QP     Rate	PSNR

28    32885	28.56

24    56130	31.19

20    92261	33.77

16   143262	36.43

silent qcif VBST intra

QP     Rate	PSNR

28    34766	29.24

24    57818	31.60

20    93626	34.12

16   145796	36.86

silent qcif SVBST intra

QP     Rate	PSNR

28    35222	29.21

24    58214	31.60

20    93778	34.12

16   146017	36.87

paris cif TML intra

QP     Rate	PSNR

28   228299	27.56

24   354083	30.62

20   527565	33.72

16   744822	36.72

paris cif VBST intra

QP     Rate	PSNR

28   240625	28.04

24   364576	31.03

20   535027	34.04

16   747353	37.03

paris cif SVBST intra

QP     Rate	PSNR

28   242175	28.01

24   366034	31.02

20   536701	34.03

16   748422	37.03

tempete cif TML intra

QP     Rate	PSNR

28   215918	27.03

24   359937	29.92

20   559124	33.01

16   809770	36.12

tempete cif VBST intra

QP     Rate	PSNR

28   226057	27.52

24   366218	30.31

20   563553	33.37

16   808528	36.48

tempete cif SVBST intra

QP     Rate	PSNR

28   227838	27.49

24   368080	30.29

20   565338	33.37

16   810274	36.48

mobile cif TML intra

QP     Rate	PSNR

28   400022	25.34

24   626187	28.51

20   922760	31.90

16  1271130	35.29

mobile cif VBST intra

QP     Rate	PSNR

28   418788	25.77

24   638654	28.90

20   927070	32.27

16  1259692	35.62

mobile cif SVBST intra

QP     Rate	PSNR

28   419723	25.76

24   639500	28.89

20   928521	32.27

16  1261059	35.63