| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O34r2 Filename: VCEG-O34r2.doc Generated: 26. November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Gero Bäse, Norbert Oertel, Jürgen Pandel CT IC 2 Otto Hahn Ring 6 81371 München Germany | Tel: +49 89 636 53193 Fax:+49 89 636 52393 Email: Gero.Baese@mchp.siemens.de Norbert.Oertel@mchp.siemens.de Juergen.Pandel@mchp.siemens.de |  |
| Title: | Enhanced coding of coefficients |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Summary

A further improvement of INTRA coding of coefficients based on context models representing LEVEL coding of coefficients depending on the LEVEL of the preceding coefficient is presented. The proposed algorithm is an extension to the existing proposal VCEG-N41r1.

The focus is on high quality applications while no additional complexity is needed. Further bitrate savings are being achieved by selecting the appropriate context model out of a new set of context models by the number of coefficients to be transmitted.

# Introduction

VCEG-N41r1 proposes a new method to code INTRA coefficients with CABAC. The LEVEL of the current coefficient will be coded based on the LEVEL of the preceding coefficient. More precise, the context model set to code the LEVEL of the current coefficient is chosen by the LEVEL of the previous coefficient.  Additional bitrate savings were achieved with a new scan method.

However the gain is flattening for smaller quantization parameters. Here a significant improvement can be achieved using additional context model sets selected by the number of coefficients to be transmitted.

# Description

VCEG-N41r1 defines context sets related to the LEVEL of the coefficients. According to these definition two new groups of context sets are being build. Depending on the number of coefficients to be coded the appropriate set is being selected. Thereby probability differences  are exploited for LEVEL transitions within larger and smaller groups of coefficients.

To switch between the three groups of context sets it appeared to be the best choice :

- number of coefficients  4

- 4 < Number of coefficients  9

- all other number of coefficients

The initialization is performed according to the initialization process in the reference software. Here, a significant potential for further improvement exists.

For the decoding process the number of coefficients following is needed first. Therefore the unary representation of the number of coefficients is coded with CABAC and transmitted before the coefficients of each block itself. Because of the very effective CABAC algorithm fewer bits are needed than for transmitting the number of coefficients as a special code word. Additionally, the knowledge about the number of coefficients within a block can be exploited for error resilience purposes.

The algorithm increases the demand for memory very minor. Algorithmic complexity is not affected.

# Results

Best results are achieved for the sequence “Mobile”. The bitrate savings are above 6%, whereas the additional gain is about 2%. The Sequence “Foreman” shows clearly the considerably potential for higher bitrate applications. The difference to VCEG-N41r1 for quantization parameter 28 is about 0,5%. Comprehensively results can be found in VCEG-O34.xls.

# Outlook

Potential for further improvements is seen in applying the proposed method to INTER coding. Additional the initialization process for all context sets used for the CABAC algorithm should be examined carefully.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Siemens AG; CT IC 2 |  |  |
| Mailing address | Otto Hahn Ring 6; 81730 München |  |  |
| Country | Germany |  |  |
| Contact person | Gero Bäse |  |  |
| Telephone | +49 89 636 53193 |  |  |
| Fax | +49 89 636 52393 |  |  |
| Email | Gero.Baese@mchp.Siemens.de |  |  |
| Place and date of submission | Pattaya, Thailand 03. Dec 2001 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  | Enhanced coding of coefficients |  |
| Contribution number |  | VCEG-O34 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: