| # ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O35 Filename: VCEG-O35.doc Generated: 26. November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Gero Bäse, Jürgen Pandel CT IC 2 Otto Hahn Ring 6 81371 München Germany | Tel: +49 89 636 53193 Fax:+49 89 636 52393 Email: Gero.Baese@mchp.siemens.de Juergen.Pandel@mchp.siemens.de |  |
| Title: | Improved quantizer selection for bitrate adjustment |  |  |
| Purpose: | Information |  |  |

_____________________________

## Summary

This contribution describes an extension of bitrate adjustment possibilities besides Dquant. It works on picture level while affecting coefficients on 4x4 block level. Subjective picture quality is maintained by taking advantage of masking effects.

## Introduction

Quantization has the strongest impact on the generated bitrate and picture quality. Changing quantization on picture level for bit rate adjustment leads to a rather big change in data rate. Fine adjustments could be done with Dquant, however there is a costly signaling overhead required. Moreover, adjustment can only be done on MB by MB basis. All coefficients of the 16x16 MB are similarly affected. This directly leads to a demand to perform a finer adjustment with significantly reduced signaling overhead.

The introduced algorithm allows for a much smoother adjustment on a 4x4 block basis. As a side effect the algorithm gets rid of small, isolated coefficients. Modification of the parameters for the introduced algorithm on picture level enables adaptation to the content as well as minimizes signaling overhead.

### Description

Depending on the LEVEL of the DC coefficient of a 4x4 block (INTRA prediction is taken into account) the decision is made to use a slightly modified quantizer. If these LEVEL indicates a very bright or very dark portion of the picture masking effects can be exploited. Proper adjustment of the “brightness” and “darkness” parameters preserves picture quality. Both parameters should be adjustable on picture level.

The formula for quantization is supplemented by a term to generate a dead-zone-quantizer. The range of the dead zone should be made adjustable on picture level according to the requirements of the application/channel.

The new quantizer is utilized for all AC coefficients of the appropriate 4x4 block indicated by the LEVEL of the related DC coefficient.

Only minor complexity is needed. Smooth adjustment of bitrate is possible without noticeable signaling overhead. Depending on the parameter settings degradation of subjective picture quality is avoided by exploiting masking effects.

### Parameters

All three parameters can be transmitted within the picture header. The parameter quant_zone modifies the range of the dead zone. Quant_zone equal zero means quantization as it is now.

The parameters for brightness and darkness are only present, if quant_zone indicates utilizing of the algorithm.

### Results

First experiments for INTRA coding showed promising results. The sequence “Paris” was used with QP 20. No visible degradation is noticeable whereas the PSNR dropped about 1,5dB (the pictures will be provided for examination). The bitrate was reduced about 6%. This corresponds to half of a quantizer step size.

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 <fax number here>

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Siemens AG |  |  |
| Mailing address | 81730 München |  |  |
| Country | Germany |  |  |
| Contact person | Gero Bäse |  |  |
| Telephone | +49 89 636 53193 |  |  |
| Fax |  |  |  |
| Email | Gero.Baese@mchp.Siemens.de |  |  |
| Place and date of submission | 11/25/01 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | Improved quantizer selection for bitrate adjustment |  |
| Contribution number |  | VCEG-O35 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks: