| # ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O36 Filename: VCEG-O36.doc Generated: 24 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Chul-Woo Kim McubeWorks 14 F, SK Telelcom Bldg, Sunae-Dong, Bundang-Gu, Sungnam Seoul, 463-784, Korea Sang Woo Rhie SK Telecom 99 Seorin-dong, Jongro-gu, Seoul, 110-110, Korea | Tel: Fax: Email: Tel: Fax: Email: | +82-31-601-5875 +82-31-601-5878 charlie@mcubeworks.com +82-2-2121-3463 +82-2-2121-3980 rainism@sktelecom.com |
| Title: | Verification of Adaptive Double Scan on Common Conditions |  |  |
| Purpose: | Proposal |  |  |

_____________________________

- Introduction

This contribution originated from VCEG-M69 that proposed the adaptive double scan to improve the compression efficiency of intra macroblocks. In Austin meeting (VCEG-M, April, 2001), there have been indications that the contribution did not provide enough experimental results under the common conditions. In this contribution are provided experimental results for the adaptive double scan algorithm which proposes alternative splitting of 4x4 block into two regions. Experimental results are given in VCEG-O36.xls and the results show that the overall bit saving of 0.1 ~ 1.3 % can be achieved with negligible complexity and code-size increase.

- Adaptive Double Scan

Current double scan defined in TML 8.0 is shown as in Fig. 1 (a). This double scan algorithm has been existing since the first release of TML to preserve the efficiency of 1 bit length of EOB which is a given factor in the current UVLC table. The adaptive double scan proposes one more double scan map shown in Fig. 1 (b)

0   1   2   5

0   2   3   6

1   3   4   7

4   5   6   7

0   0   1   4

1   2   3   5

2   3   4   6

5   6   7   7

(a)                        (b)

## Fig. 1 Double scan map of TML 6 and proposed algorithm

- Double scan map defined in TML 8.0 (D1)

- Proposed additional double scan map for adaptive double scan (D2)

The proposed double scan map is the transposed version of the one defined in TML 8.0. And the proposed algorithm is to use the D1 and D2 alternatively depending on the intra prediction mode. The alternating logic is defined in Table 1. By alternating the double scan map, the distribution of quantized transform coefficients are concentrated in the way that the collected run-lengths between successive coefficients can be reduced.

## Table 1. Algorithm for alternating the double scan map

| Prediction Mode | Scan Map |
| --- | --- |
| Mode 0 (DC Prediction) | D1 |
| Mode 1 (RL Prediction) | ## D2 |
| Mode 2 (Vertical Prediction) | ## D2 |
| Mode 3 (Diagonal Prediction) | D1 |
| Mode 4 (Horizontal Prediction) | D1 |
| Mode 5 (LR Prediction) | D1 |

This can be achieved without any additional calculation of parameters but with only one comparison or  table indexing operation using intra prediction mode.

- Simulation Results

TML 8.7a simulation results are shown in VCEG-O36.xls and Fig. 2. The percentage of bit saving relative to TML 8.7a is shown in the result. All the frames in the sequences are intra coded and QP set of (8, 12, 16, 20, 23) was used for the test. This QP set includes two QP values of 16 and 20, which are specified in common conditions. Akiyo, Mother and Daughter and Hall Monitor results are shown in addition to the results of sequences specified in VCEG-M75. In all the sequences, adaptive double scan shows better performance compared with the current double scan. Overall bit saving of from 0.1 to 1.3 % could be achieved through adaptive double scan. Though at sequences such as Tempete, Foreman, Container, Silent and Mobile, the bit saving is rather low (below 0.5%), results of Paris, News, Hall, Mother and Daughter and Akiyo show 0.5 ~ 1.3 % of bit saving.

- Complexity and Storage Analysis

- Additional Complexity due to Adaptive Double Scan

In the point of view of complexity both for decoder and encoder, ‘negligible’ additional complexity is required to adopt the adaptive double scan algorithm. It can be implemented using one ‘if’ and ‘branch’ operation pair or one table indexing operation for each double scanned block.

- Additional Memory and Code size

Assuming that two bytes are required to store the location in the scanning map, totally 32 bytes of memory is required to describe the proposed double scan map (D2) with several more instructions to perform branch operation.

- Summary

The simulation results reveal that the statistics of UVLC table becomes more efficient by adaptive double scan for 2-D run-length coding of quantized coefficients on double scanned blocks when the prediction mode contains strong vertical factors. And it improves the performance of TML when adaptive double scanning is adopted.

- IPR Strategy

McubeWorks and SK Telecom agreed and decided to have intellectual property related to this contribution and have chosen to subscribe to sub clause 2.2.1 of the ITU-T patent policy under the condition of reciprocity.

7.  Reference

[1] Gisle Bjontegaard, “H.26L Test Model Long Term Number 8 (TML-8) draft 0,” June. 2001.

Fig. 2. Performance of Adaptive Double Scan relative to TML 8.0