| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O39 Filename: VCEG-O39.doc Generated: 27 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Guy Côté, Lowell Winger, Michael Gallant VideoLocus Inc 109 Randall Dr, Waterloo, On. Canada, N2V 1C5 | Tel: Fax: Email: | +1.519.725.9797 +1.519.725.5345 gcote@videolocus.com, lwinger@videolocus.com, mgallant@videolocus.com |
| Title: | Lower complexity deblocking filter with in-place filtering |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Introduction

The current description of the deblocking filter requires that the macroblock pixels be copied to a temporary buffer before the vertical and horizontal filtering operations. Performing the filtering operations in-place can significantly reduce the complexity of the deblocking filter. This document compares these two methods.

# Modification to the deblocking filter

The only change to the deblocking filter is that the filtering operations are done in-place. Therefore, for filtering block boundaries within a macroblock, the input samples for the current block are the output (filtered) samples from the previous block.

# Results

Results are presented according to document VCEG-N81 for the baseline profile.

### PSNR Plots

### Tabulated Results

| ## Container, QCIF, frameskip=2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/ Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 37.301 | 40.6879 | 40.5973 | 27880 | 37.3018 | 40.7893 | 40.6021 | 27840 |
|  | Avg | 36.0825 | 40.486 | 40.0859 | 25.46 | 36.0863 | 40.4685 | 40.0773 | 25.41 |
| QP 20 | I-frame | 34.6138 | 38.9466 | 38.4937 | 19016 | 34.5921 | 38.6668 | 38.4296 | 19008 |
|  | Avg | 33.5382 | 38.9472 | 38.4862 | 14.2 | 33.5399 | 38.7009 | 38.7009 | 14.25 |
| QP 24 | I-frame | 31.5782 | 37.1607 | 36.4535 | 12912 | 31.6273 | 36.6039 | 36.1021 | 12976 |
|  | Avg | 30.7587 | 37.3783 | 36.839 | 8.53 | 30.7839 | 37.0162 | 36.5633 | 8.61 |
| QP 28 | I-frame | 28.7691 | 36.4662 | 35.2373 | 8112 | 28.815 | 36.1135 | 35.2082 | 8168 |
|  | Avg | 27.9597 | 36.636 | 35.7129 | 4.98 | 28.0359 | 36.5078 | 35.687 | 5.01 |

| ## News, QCIF, frameskip=2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 37.6877 | 39.3541 | 40.1694 | 31264 | 37.6734 | 39.4223 | 40.0482 | 31208 |
|  | Avg | 36.8549 | 39.1271 | 39.5972 | 48.63 | 36.8337 | 39.1302 | 39.5327 | 48.69 |
| QP 20 | I-frame | 34.5173 | 37.6112 | 38.3794 | 22120 | 34.5506 | 37.4546 | 38.1027 | 22144 |
|  | Avg | 33.9361 | 37.4989 | 38.2783 | 30.35 | 33.9655 | 37.3377 | 38.1379 | 30.32 |
| QP 24 | I-frame | 31.536 | 36.0745 | 36.3593 | 15728 | 31.5268 | 35.7455 | 36.0926 | 15576 |
|  | Avg | 31.0608 | 35.9697 | 36.7315 | 18.56 | 31.0577 | 35.8763 | 36.7132 | 18.44 |
| QP 28 | I-frame | 28.4593 | 34.3049 | 35.3465 | 9944 | 28.4189 | 34.2065 | 34.9749 | 9864 |
|  | Avg | 28.151 | 34.7196 | 35.6483 | 10.74 | 28.1351 | 34.452 | 35.4693 | 10.76 |

| ## Foreman, QCIF, frameskip=2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 36.9257 | 39.4116 | 41.5349 | 27024 | 36.9514 | 39.3644 | 41.4199 | 27064 |
|  | Avg | 36.0656 | 39.0969 | 40.5595 | 79.48 | 36.0627 | 39.0787 | 40.5767 | 79.62 |
| QP 20 | I-frame | 34.258 | 38.1711 | 40.0595 | 18272 | 34.2199 | 38.0573 | 40.1447 | 18152 |
|  | Avg | 33.6546 | 37.8852 | 38.9084 | 47.88 | 33.6536 | 37.7999 | 38.9304 | 48.06 |
| QP 24 | I-frame | 31.4613 | 37.1124 | 38.742 | 11736 | 31.3563 | 37.1977 | 38.4101 | 11592 |
|  | Avg | 31.4258 | 36.6732 | 37.4283 | 29.59 | 31.3615 | 36.5944 | 37.4303 | 29.78 |
| QP 28 | I-frame | 28.6142 | 36.5068 | 37.8457 | 7304 | 28.7407 | 36.4712 | 37.6295 | 7344 |
|  | Avg | 28.9072 | 35.9455 | 36.4877 | 18.87 | 28.7785 | 35.9514 | 36.4759 | 18.69 |

| ## Paris, CIF, frameskip=1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 36.7686 | 39.2431 | 39.1374 | 155944 | 36.7707 | 39.1893 | 39.2013 | 155784 |
|  | Avg | 35.6019 | 38.4511 | 38.4297 | 339.7 | 35.5904 | 38.3835 | 38.3893 | 339.83 |
| QP 20 | I-frame | 33.7542 | 36.995 | 37.1687 | 108776 | 33.7548 | 36.7535 | 37.0253 | 108664 |
|  | Avg | 32.6669 | 36.4213 | 36.6288 | 200.68 | 32.6721 | 36.318 | 36.6747 | 200.8 |
| QP 24 | I-frame | 30.6751 | 35.0343 | 35.4423 | 75016 | 30.7054 | 34.8327 | 35.1501 | 75248 |
|  | Avg | 29.7867 | 34.6027 | 34.9753 | 112.83 | 29.7988 | 34.5104 | 34.8704 | 113.39 |
| QP 28 | I-frame | 27.6111 | 33.7908 | 34.0853 | 47432 | 27.6231 | 33.6842 | 33.8369 | 47440 |
|  | Avg | 26.8439 | 33.1125 | 33.6057 | 60.48 | 26.8384 | 33.0156 | 33.4665 | 60.4 |

| ## Silent, QCIF, frameskip=1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 36.4086 | 38.6776 | 39.5074 | 29040 | 36.4238 | 38.6095 | 39.4017 | 29016 |
|  | Avg | 35.9594 | 38.3366 | 39.2011 | 61.18 | 35.967 | 38.3048 | 39.1234 | 61.24 |
| QP 20 | I-frame | 33.754 | 36.6913 | 38.0421 | 18544 | 33.6843 | 36.438 | 37.6762 | 18480 |
|  | Avg | 33.2975 | 36.5403 | 37.9809 | 37.41 | 33.2601 | 36.3953 | 37.7958 | 37.29 |
| QP 24 | I-frame | 31.2181 | 34.9919 | 36.9644 | 11224 | 31.1111 | 34.8064 | 36.8384 | 11184 |
|  | Avg | 30.7166 | 35.0528 | 36.8011 | 21.98 | 30.6871 | 34.8315 | 36.7689 | 22.09 |
| QP 28 | I-frame | 28.7226 | 34.435 | 36.3048 | 6704 | 28.5532 | 34.2784 | 36.3138 | 6672 |
|  | Avg | 28.2395 | 34.3549 | 36.0949 | 12.3 | 28.1011 | 34.2388 | 36.0845 | 12.27 |

| ## Mobile, CIF, frameskip=0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 35.5323 | 37.3703 | 37.1698 | 255872 | 35.5083 | 37.3618 | 37.1538 | 255616 |
|  | Avg | 34.0906 | 35.2332 | 35.1409 | 1635.18 | 34.0975 | 35.2323 | 35.1247 | 1636.97 |
| QP 20 | I-frame | 32.0934 | 35.3375 | 35.0727 | 181288 | 32.0954 | 35.2565 | 35.0031 | 181464 |
|  | Avg | 30.9221 | 33.1654 | 32.9852 | 836.24 | 30.922 | 33.1524 | 32.9784 | 836.15 |
| QP 24 | I-frame | 28.7015 | 33.4622 | 33.1113 | 127424 | 28.6901 | 33.2529 | 32.9546 | 127336 |
|  | Avg | 28.0346 | 31.2946 | 31.0034 | 422.11 | 28.0308 | 31.2399 | 30.9884 | 422.53 |
| QP 28 | I-frame | 25.5725 | 32.2978 | 32.0856 | 78280 | 25.5579 | 32.0937 | 31.942 | 78392 |
|  | Avg | 25.1876 | 30.0598 | 29.6597 | 236.73 | 25.187 | 29.9562 | 29.6165 | 238.04 |

| ## Tempete, CIF, frameskip=0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor |  |  |  |  |  | ## Proposed |  |  |  |
|  |  | Y | Cb | Cr | Bits/Kbps | Y | Cb | Cr | Bits/Kbps |
| QP 16 | I-frame | 36.014 | 37.5817 | 39.4985 | 176640 | 36.0069 | 37.5633 | 39.5413 | 176792 |
|  | Avg | 35.0735 | 36.7043 | 38.5653 | 1246.72 | 35.0713 | 36.6588 | 38.53 | 1246.61 |
| QP 20 | I-frame | 32.8855 | 35.7753 | 37.983 | 120936 | 32.8421 | 35.5956 | 37.9288 | 120712 |
|  | Avg | 32.1636 | 35.1613 | 37.2134 | 623 | 32.1619 | 35.121 | 37.1951 | 623.4 |
| QP 24 | I-frame | 29.7499 | 34.1852 | 36.6521 | 78608 | 29.6837 | 34.0916 | 36.4471 | 78288 |
|  | Avg | 29.5185 | 33.637 | 35.89 | 313.67 | 29.5024 | 33.5779 | 35.8194 | 313.64 |
| QP 28 | I-frame | 26.8379 | 33.3169 | 35.7112 | 45800 | 26.8073 | 33.1642 | 35.6404 | 45952 |
|  | Avg | 26.8395 | 32.5541 | 34.9178 | 168.56 | 26.8011 | 32.4716 | 34.8162 | 168.56 |

### Method of VCEG-M33

Performance difference between the anchor and the proposed method was computed with the avsbr4 software.

| Sequence | BDBitrate Increase (%) | BDSNR Gain (Db) |
| --- | --- | --- |
| Container | 0.13 | -0.008 |
| New | -0.32 | 0.019 |
| Foreman | 1.05 | -0.051 |
| Paris | 0.10 | -0.006 |
| Silent | 0.98 | -0.046 |
| Mobile | 0.13 | -0.006 |
| Tempete | 0.28 | -0.012 |

# Conclusions

Performing the filter operations in-place significantly reduces the complexity of the deblocking filter. Results indicate that the compression performance is almost the same.  We propose that the deblocking filter description be changed to adopt in-place filtering.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | VideoLocus, Inc. |  |  |
| Mailing address | 109 Randall Drive, Waterloo, Ontario, Canada N2V 1C5 |  |  |
| Country | Canada |  |  |
| Contact person | Lowell Winger |  |  |
| Telephone | +01-519-725-9797 |  |  |
| Fax | +01-519-725-5345 |  |  |
| Email | lwinger@videolocus.com |  |  |
| Place and date of submission | Pattaya, 4-6 Dec 2001 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Lower complexity deblocking filter with in-place filtering |  |
| Contribution number |  | VCEG-O39 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: