| # ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 3-7 Dec., 2001 | Document VCEG-O48 Filename: VCEG-O48.doc Generated: 25 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Zhong Xue, Sheng Mei Shen, Teck Wee Foo, CJ Lee (Panasonic Singapore Laboratories Pte Ltd) Shinya Kadono (Matsushita Electric Industrial Co., Ltd) | Tel: Fax: Email: | +65-5505466 +65-5505459 shen@psl.com.sg |
| Title: | An Automatic Mode Decision Method for Intra Frame Coding and Decoding |  |  |
| Purpose: | Proposal |  |  |

## Determining the Intra Pred Mode of the Current Block from its Previously Decoded Blocks

The idea of this proposal is to automatically determine the intra prediction mode of the current block automatically, using its previously reconstructed neighboring blocks, i.e. the left (A), upper-left (B) and upper (C) blocks as shown in Figure 1. The pixel values of the current block are represented using 2D array as shown in Figure 2.

B

C

A

X

Figure 1. The neighboring blocks of the current block X.

| … | … | … | … | … |
| --- | --- | --- | --- | --- |
| … | I(y,x) | I(y,x+1) | I(y,x+2) | I(y,x+3) |
| … | I(y+1,x) | I(y+1,x+1) | I(y+1,x+2) | I(y+1,x+3) |
| … | I(y+2,x) | I(y+2,x+1) | I(y+2,x+2) | I(y+2,x+3) |
| … | I(y+3,x) | I(y+3,x+1) | I(y+3,x+2) | I(y+3,x+3) |

Figure 2. The pixel values of block X.

It is noted that the intra prediction mode of a block (or the predicting direction) reflects the possible edge direction of that block. Hence the direction numbers are used to represents the corresponding prediction modes (see Table 1).

In this document, an automatic intra mode decision method is presented. In the method, smoothness of the pixel values along different directions in block A, B and C is investigated to determine the intra prediction mode of block X. Therefore, the mode information is not needed in the bit stream (i.e. prob0 and prob1 in TML8.0).

Table 1. The relationship among the 6 intra pred modes, possible edges

and the predicting directions.

| Intra Pred Mode | Possible Edge Direction | No. of the Direction |
| --- | --- | --- |
| 0 1 2 3 4 5 | No edge Upper right Vertical Diagonal Horizontal Lower left | 0 (No obvious direction) 1 2 3 4 5 |

The intra prediction mode is determined according to the availability of block A, B and C:

- Case 1 :

All of the blocks A, B, and C are not available, mode 0 is chosen, and the prediction value is 128.

- Case 2:

- All of the blocks A, B, and C are available, and the intra pred mode is determined by analyzing the smoothness of pixel values in five directions (Direction 1 to 5 as shown in Table 1).

For each direction, pixels are selected to form five lines as in Figure 3,

Direction 1                      Direction 2                    Direction 3                      Direction 4                    Direction 5

Figure 3: the pixels selected to form the lines for investigating

the smoothness in each direction.

For each line, a smooth parameter, Li,j(Direction i, Line j), i.e. the maximum absolute value of the pixel differences along the line, is calculated.

e.g. for Direction 1, Line 3 (from left to right), the smooth parameter L1,3 is calculated as,

L1,3=maxabs(I(y−1,x+1.5)−I(y−2,x+2)), abs(I(y−2,x+2)−I(y−3,x+2.5))

Note ½ resolution of the pixel position is used, e.g., “x+1.5”. The pixel value at the ½ resolution position is the average of its neighboring pixels at integer position. For example

I(y−1,x+1.5)=(I(y−1,x+1)+I(y−1,x+2)+1)2

- Case 3:

- The upper block of X is not available, hence the possible prediction directions of X are Direction 0, 4 or 5. For this case, four lines are used for each direction (Direction 4 and Direction 5).

Direction 4			Direction 5

- Case 4:

- The left block of X is not available, the possible predicting directions of X are Direction 0, 1 or 2. For this case, four lines of the luma pixel are used for each direction (Direction 1 and Direction 2).

Direction 1		Direction 2

After the smoothness parameters are obtained, a threshold T=15 is set to judge the smoothness of the pixel values in all the 5 possible directions.

For Direction i, D parameters, i.e.Di(i=1,…,5) and S parameters Si(i=1,…5) are calculated:

Di records the number of the smoothness parameters (Li,1to Li,5) that are lower than the threshold T, and Si is the summation of these smooth parameters.

Si=Li,j<T and j=1,...,5Li,j

D0 is set to 4 for case 1 and 2, and set to 3 for case 3 and 4. If Direction i is not applicable (when the corresponding neighboring blocks are not available), Di is set to 0.

Finally, the algorithm for determining the intra prediction mode of block X can be described using the following psudo-code,

mode = 0, min = 1001;   //   min is a large number selected a priory

for (i=1;i<=5;i++)

if ((D[i]>D[0])&&(S[i]<min)) { min=S[i]; mode = i;};

## New MB_type for Coding H26L I-Frames

A new MB_type is introduced to indicate that all the intra pred modes of the blocks in an MB is automatically determined using the proposed algorithm. Except that there is no Intra_pred_mode information, all the other information such as CBP, and the Tcoeffs for the residual of luma and chroma remains the same with the Intra4x4 MBs. The new MB_type is called “Intra4x4_Pred”. (See Table 2)

Table 2: Connection between codeword number and parameter values.

| Code_ number | RUN | MB_Type |  | Intra_pred_mode1 |  | MVD | CBP |  | Tcoeff_chroma_DC2 |  | Tcoeff_chroma_AC2 Tcoeff_luma2 Simple scan |  | Tcoeff_luma2 Double scan |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Intra | Inter | Prob0 | Prob1 |  | Intra | Inter | Level | Run | Level | Run | Level | Run |
| 0 | 0 | Intra4x4 | Copy | 0 | 0 | 0 | 47 | 0 | EOB | - | EOB | - | EOB | - |
| 1 | 1 | Intra4x4_Pred | 16x16 | 1 | 0 | 1 | 31 | 16 | 1 | 0 | 1 | 0 | 1 | 0 |
| 2 | 2 | 0,0,03 | 16x8 | 0 | 1 | -1 | 15 | 1 | -1 | 0 | -1 | 0 | -1 | 0 |
| 3 | 3 | 1,0,0 | 8x16 | 0 | 2 | 2 | 0 | 2 | 2 | 0 | 1 | 1 | 1 | 1 |
| 4 | 4 | 2,0,0 | 8x8 | 1 | 1 | -2 | 23 | 4 | -2 | 0 | -1 | 1 | -1 | 1 |
| 5 | 5 | 3,0,0 | 8x4 | 2 | 0 | 3 | 27 | 8 | 1 | 1 | 1 | 2 | 2 | 0 |
| 6 | 6 | 0,1,0 | 4x8 | 3 | 0 | -3 | 29 | 32 | -1 | 1 | -1 | 2 | -2 | 0 |
| 7 | 7 | 1,1,0 | 4x4 | 2 | 1 | 4 | 30 | 3 | 3 | 0 | 2 | 0 | 1 | 2 |
| 8 | 8 | 2,1,0 | Intra4x4 | 1 | 2 | -4 | 7 | 5 | -3 | 0 | -2 | 0 | -1 | 2 |
| 9 | 9 | 3,1,0 | 0,0,03 | 0 | 3 | 5 | 11 | 10 | 2 | 1 | 1 | 3 | 3 | 0 |
| 10 | 10 | 0,2,0 | 1,0,0 | 0 | 4 | -5 | 13 | 12 | -2 | 1 | -1 | 3 | -3 | 0 |
| 11 | 11 | 1,2,0 | 2,0,0 | 1 | 3 | 6 | 14 | 15 | 1 | 2 | 1 | 4 | 4 | 0 |
| 12 | 12 | 2,2,0 | 3,0,0 | 2 | 2 | -6 | 39 | 47 | -1 | 2 | -1 | 4 | -4 | 0 |
| 13 | 13 | 3,2,0 | 0,1,0 | 3 | 1 | 7 | 43 | 7 | 1 | 3 | 1 | 5 | 5 | 0 |
| 14 | 14 | 0,0,1 | 1,1,0 | 4 | 0 | -7 | 45 | 11 | -1 | 3 | -1 | 5 | -5 | 0 |
| 15 | 15 | 1,0,1 | 2,1,0 | 5 | 0 | 8 | 46 | 13 | 4 | 0 | 3 | 0 | 1 | 3 |
| 16 | 16 | 2,0,1 | 3,1,0 | 4 | 1 | -8 | 16 | 14 | -4 | 0 | -3 | 0 | -1 | 3 |
| 17 | 17 | 3,0,1 | 0,2,0 | 3 | 2 | 9 | 3 | 6 | 3 | 1 | 2 | 1 | 1 | 4 |
| 18 | 18 | 0,1,1 | 1,2,0 | 2 | 3 | -9 | 51 | 9 | -3 | 1 | -2 | 1 | -1 | 4 |
| 19 | 19 | 1,1,1 | 2,2,0 | 1 | 4 | 10 | 10 | 31 | 2 | 2 | 2 | 2 | 2 | 1 |
| 20 | 20 | 2,1,1 | 3,2,0 | 0 | 5 | -10 | 12 | 35 | -2 | 2 | -2 | 2 | -2 | 1 |
| 21 | 21 | 3,1,1 | 0,0,1 | 1 | 5 | 11 | 19 | 37 | 2 | 3 | 1 | 6 | 3 | 1 |
| 22 | 22 | 0,2,1 | 1,0,1 | 2 | 4 | -11 | 21 | 42 | -2 | 3 | -1 | 6 | -3 | 1 |
| 23 | 23 | 1,2,1 | 2,0,1 | 3 | 3 | 12 | 26 | 44 | 5 | 0 | 1 | 7 | 6 | 0 |
| 24 | 24 | 2,2,1 | 3,0,1 | 4 | 2 | -12 | 28 | 33 | -5 | 0 | -1 | 7 | -6 | 0 |
| 25 | 25 | 3,2,1 | 0,1,1 | 5 | 1 | 13 | 35 | 34 | 4 | 1 | 1 | 8 | 7 | 0 |
| 26 | 26 |  | 1,1,1 | 5 | 2 | -13 | 37 | 36 | -4 | 1 | -1 | 8 | -7 | 0 |
| 27 | 27 |  | 2,1,1 | 4 | 3 | 14 | 42 | 40 | 3 | 2 | 1 | 9 | 8 | 0 |
| 28 | 28 |  | 3,1,1 | 3 | 4 | -14 | 44 | 39 | -3 | 2 | -1 | 9 | -8 | 0 |
| 29 | 29 |  | 0,2,1 | 2 | 5 | 15 | 1 | 43 | 3 | 3 | 4 | 0 | 9 | 0 |
| 30 | 30 |  | 1,2,1 | 3 | 5 | -15 | 2 | 45 | -3 | 3 | -4 | 0 | -9 | 0 |
| 31 | 31 |  | 2,2,1 | 4 | 4 | 16 | 4 | 46 | 6 | 0 | 5 | 0 | 10 | 0 |
| 32 | 32 |  | 3,2,1 | 5 | 3 | -16 | 8 | 17 | -6 | 0 | -5 | 0 | -10 | 0 |
| 33 | 33 |  |  | 5 | 4 | 17 | 17 | 18 | 5 | 1 | 3 | 1 | 4 | 1 |
| 34 | 34 |  |  | 4 | 5 | -17 | 18 | 20 | -5 | 1 | -3 | 1 | -4 | 1 |
| 35 | 35 |  |  | 5 | 5 | 18 | 20 | 24 | 4 | 2 | 3 | 2 | 2 | 2 |
| 36 | 36 |  |  |  |  | -18 | 24 | 19 | -4 | 2 | -3 | 2 | -2 | 2 |
| 37 | 37 |  |  |  |  | 19 | 6 | 21 | 4 | 3 | 2 | 3 | 2 | 3 |
| 38 | 38 |  |  |  |  | -19 | 9 | 26 | -4 | 3 | -2 | 3 | -2 | 3 |
| 39 | 39 |  |  |  |  | 20 | 22 | 28 | 7 | 0 | 2 | 4 | 2 | 4 |
| 40 | 40 |  |  |  |  | -20 | 25 | 23 | -7 | 0 | -2 | 4 | -2 | 4 |
| 41 | 41 |  |  |  |  | 21 | 32 | 27 | 6 | 1 | 2 | 5 | 2 | 5 |
| 42 | 42 |  |  |  |  | -21 | 33 | 29 | -6 | 1 | -2 | 5 | -2 | 5 |
| 43 | 43 |  |  |  |  | 22 | 34 | 30 | 5 | 2 | 2 | 6 | 2 | 6 |
| 44 | 44 |  |  |  |  | -22 | 36 | 22 | -5 | 2 | -2 | 6 | -2 | 6 |
| 45 | 45 |  |  |  |  | 23 | 40 | 25 | 5 | 3 | 2 | 7 | 2 | 7 |
| 46 | 46 |  |  |  |  | -23 | 38 | 38 | -5 | 3 | -2 | 7 | -2 | 7 |
| 47 | 47 |  |  |  |  | 24 | 41 | 41 | 8 | 0 | 2 | 8 | 11 | 0 |
|  | .. |  |  |  |  | .. |  |  | .. | .. | .. | .. | .. | .. |

## Test Model Issues in H26L

The macroblock mode decision process for I-frames is the same as that in TML8.0, with an exception that three macroblock types are used, {Intra4x4, Intra4x4_Pred, Intra16x16}.

## Experimental Results

The original TML8.0 encoding/decoding software package is modified to implement the proposed algorithm. Four typical sequences, namely “mobileandclendar”, “tempete”, “foreman” and “news”, are used to test the algorithm. For each sequence, 20 pictures are encoded as I-frames separately, with QP varying from 1 to 31 for each picture. The bit length and the corresponding SNRY values are recorded.

(a) Results of “foreman”

(b) Results of “tempete”

(c) Results of “mobile and calendar”

(d) Results of “news”

Figure 3: Bits reduction in percentage w.r.t. TML8.0

Figure 4: Comparison of SNRY (foreman, cif).

Figure 3 shows the results for “foreman” and “tempete” sequences. The signature in the figures shows the mean, and the error bar shows the variance of the bit reduction in percentage. It can be seen that for all the video sequences, especially for larger QPs, the reduction in bit length is obvious (up to 3%). In addition to the gain in bit length, it can be seen from the results that the SNRY of the new method remains almost the same with that of the original method (see Figure 4, the comparison results of “foreman”), and at the same time, the image quality are close for the two algorithms.

- Computational Complexity

5.1 Computational Overload Analysis

The computational load of the decoder for the additional Intra4x4_Pred MB_type is analyzed in this section. In [3], the benchmark analysis of H.26L decoder function blocks (TML8.4) is given based on the number of cycles of microprocessor TMS320C54X. Hence in this section, for comparison purpose, the cycles used on the same processor are calculated to analyze the computational load of the proposed method.

Table 3: The operations and their corresponding cycles on TMS320C54X.

| Operations (integer) | Cycles |
| --- | --- |
| add/sub abs() if (s<t) max(a, b) shift | 1 2 5 5 1 |

Considering the most complex case, i.e. both of the upper block and the left block are available, the overall cycles to perform the mode decision of one MB is 7090 cycles, plus 800 additional cycles (the operations performed if some conditions hold). Therefore the worst case is 7890 cycles per Intra4x4_Pred MB. From [3], it is known that the total cycles are 65226 per MB, hence one Intra4x4_Pred MB needs up to 73116 cycles, increased 12.1% from the original. (0~12%).

Table 4: The Number of Intra4x4_Pred Mbs for coding I-Frames

| QP | 1 | 6 | 10 | 12 | 14 | 16 | 18 | 20 | 24 | 26 | 28 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Intra4x4_Pred MBs (“akiyo”, first frame) | 18 | 85 | 98 | 88 | 89 | 74 | 77 | 66 | 62 | 57 | 52 | 46 |
| Number of Intra4x4_Pred MBs (“foreman”, first frame) | 6 | 21 | 44 | 43 | 51 | 52 | 54 | 50 | 54 | 51 | 40 | 45 |
| Number of Intra4x4_Pred MBs (“tempete”, first frame) | 5 | 15 | 26 | 25 | 44 | 48 | 56 | 59 | 66 | 75 | 104 | 122 |

Fortunately, it is noted that not all the MBs are of Intra4x4_Pred type and they can be Intra4x4 or Intra16x16 MBs. The typical numbers of the Intra4x4_Pred MBs for different sequences and QPs are shown in Table 4. It can be seen from the Table that the number of Intra4x4_Pred MBs is generally not more than 1/3 of the total number of MB. Take 132 as the maximum number, the overall computational overload for I-frame is less than 4%. Also, it can be seen from the table that in most cases, the number of Intra4x4_Pred Mbs is much smaller than 132, hence it is reasonable to conclude that the average overload of the proposed algorithm for I-frame coding is around 2%.

Table 5: Comparison of the decoding time (for the first frame) between the proposed algorithm and the original TML8.0 algorithm. (“news, cif”)

| QP | Number of Intra4x4_Pred MPs | Original TML80 (ms) | New Algorithm (ms) | Computational overload (%) |
| --- | --- | --- | --- | --- |
| 12 16 20 24 28 | 78 91 75 92 74 | 102.1 100.7 99.6 98.5 98.1 | 105 102 102.5 99.5 99.9 | 2.8% 1.3% 2.9% 1.0% 1.8% |

Table 6: Comparison of the decoding time (for the first frame) between the proposed algorithm and the original TML8.0 algorithm. (“foreman, cif”)

| QP | Number of Intra4x4_Pred MPs | Original TML80 (ms) | New Algorithm (ms) | Computational overload (%) |
| --- | --- | --- | --- | --- |
| 12 16 20 24 28 | 43 52 54 54 40 | 101.32 102.09 99.21 97.75 96.44 | 102.10 102.28 99.88 98.33 97.64 | 0.8% 0.2% 0.7% 0.6% 1.2% |

Table 7: Comparison of the decoding time (for the first frame) between the proposed algorithm and the original TML8.0 algorithm. (“tempete, cif”)

| QP | Number of Intra4x4_Pred MPs | Original TML80 (ms) | New Algorithm (ms) | Computational overload (%) |
| --- | --- | --- | --- | --- |
| 12 16 20 24 28 | 25 48 59 66 104 | 106.95 105.04 103.27 100.77 99.26 | 107.97 106.15 104.05 101.58 100.54 | 0.95% 1.1% 0.76% 0.8% 1.3% |

5.2. Real Decoding Time Comparison (Using TML8.0)

In this experiments, the real time for decoding I-frame using the new algorithm and the original TML8.0 is compared. Table 5 to Table 7 are the comparison results of sequences: “news”, “foreman” and “tempete” respectively. It can be seen from the tables that the computational overload of the proposed algorithm is small as compared with the original TML8.0 algorithm.

- Conclusion

A spatial method, which automatically determines the intra pred mode of the current block from its previously decoded blocks, is proposed in this document.  The algorithm is based on analyzing the smoothness of pixel values in different directions. To implement the proposed method in H26L, an additional MB_type, Intra4x4_Pred, is utilized, and the corresponding UVLC is modified. Bit reduction is achieved up to 3% for I-frames with no SNR-Y reduction, and the average overload of the proposed algorithm is around 2%. Therefore, it is suggested to use the Intra4x4_Pred as an additional macroblock type for I-frames in H.26L.

# References

- ISO/IEC JTC 1/SC 29/WG 11 N 2502: Information Technology -- Generic Coding of Audio-Vidual Objects -- Part 2: Vidual

- ISO/IEC JTC1/SC29/WG11, MPEG2001/M7512: TML80 Document

- VCEG-N023: Minhua Zhou, Benchmark Analysis of H.26L Decoder Functional Blocks, MPEG meeting, Santa Barbara, CA, USA Sep 2001.

- VCEG-N054: Greg Conklin, New Intra Prediction Modes, Video Coding Experts Group (VCEG), Fourteenth meeting:  Santa Barbara, 24-27 Sep 2001.