| # ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 3-7 Dec., 2001 | Document VCEG-O49 Filename: VCEG-O49.doc Generated: 25 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | SM Shen, TW Foo, Xue Zhong, CJ Lee (Panasonic Singapore Laboratories Pte Ltd) Shinya Kadono (Matsushita Electric Industrial Co., Ltd) | Tel: Fax: Email: | +65-5505466 +65-5505459 shen@psl.com.sg |
| Title: | Restricted Multiple Reference Frames with Long Term Frame Buffer |  |  |
| Purpose: | Proposal |  |  |

# Introduction

Multiple Reference Frame prediction in current H.26L can increase coding efficiency and we should keep it. On the other hand it would be much costly in term of frame memory if the maximum number of Multiple Reference Frame has no restriction.

There is a conflict here between keeping shorter for the frame buffer and achieving better prediction, especially in the case AdditionalReferenceFrame is large as shown in ref 1. Sometimes it could be a few hundreds frames or even a few thousands frames.

There are many scenarios where video scenes appear in segment by segment or group by group. For example for news broadcasting case there are many news insertions in between the same newscaster, sometimes one newscaster, or sometimes two newscasters, as shown in Figure 1.

Another example is the sports scene, where similar kind of scenes in a group of pictures appears repeatly following by another group of scenes for a few seconds or even for a few minutes.

In order to balance between less frame buffer size and higher compression using multiple reference frames, a Restricted Multiple Reference Frame with Long Term Frame Buffer is proposed here in section 2.

# 2.	Restricted Multiple Reference Frame with Long Term Frame Buffer

The basic idea here is to allocate several Long Term Frame Buffers, let’s say 3 buffers to store those representative scenes for different groups of the pictures, respectively, as shown in Figure 1.

Long Term Buffer1

Long Term Buffer2

Long Term Buffer3

Figure 1 Illustration of Long Term Buffer for Storing the 1st frame as the Representative for Each Group of Pictures

In Figure 1, there are three Long Term Frame Buffers, Long Term buffer1, Long Term Buffer2, and Long Term Buffer3 to use for storing the 1st frame of each group of pictures (boundary frames). There are three different groups of pictures shown as an example in this case.

If Long Term Buffer is introduced, Ref_Frame can be restricted to a small number, let’s say 5 for multiple reference frames prediction from the nearest five frames. In addition to the five multiple reference frame buffers, the three long term buffers are introduced to assist in far distance prediction no matter how far.

The decoder will be informed which long term buffer is used for the prediction of the 1st frame of the group of pictures. Besides that every frame has to put flag to tell what I should do, predicting from multiple reference frames like current H.26L, or predicting from the long term buffer.

### Solution in Picture Level

If it is done in picture level, the possible syntax for picture header should be:

Table 1: Syntax for Picture_header with Long Term Buffer Flag

| Picture_header() { | No. Of bits | Notes |
| --- | --- | --- |
| Picture_start_code | 32 |  |
| Ref_frame | 5 | Can be restricted to small value for low delay mode |
| if (Ref_frame != 0) { |  | Multiple Reference Frames Prediction |
| Picture_type | 3 |  |
| Store_long_term_buffer | 1 | 0 is for no storing, 1 is for storing |
| if (Store_long_term_buffer == 1) |  | Store this frame in long term buffer |
| Long_term_buffer | 3 | 000 for 1st buffer, 001 for 2nd buffer, … |
| } |  |  |
| else |  |  |
| Long_term_buffer | 3 | 000 for 1st buffer, 001 for 2nd buffer, … |
| …….. |  |  |

As shown in the Table 1, if Ref_frame is not zero, then Picture_type will be used in the same way as in TML, and in this case the flag of Store_long_term_buffer is used to set to either 0 or 1 to indicate whether the frame should be stored in long term buffer for far distance prediction.

Long_term_buffer will tell which buffer is used for storing or which buffer is used for prediction.

The possibility for the frame set to be stored in long term buffer could be the case when Picture_type is Intra. By using scene change detection technology, the coded frame could be selected as new type of the picture from the previous, then Store_long_term_buffer will be set to 1, otherwise it is always 0.

If Ref_frame is zero, then Long_term_buffer will tell the current frame should be predict from the long term buffer whose number is indicated by the Long_term_buffer, as shown in Table 4.

Ref_frame is the same as the current TML:

Table 2: Code_number for ref_frame

| Code_number | Reference frame |
| --- | --- |
| 1 | The most recent previous frame (1 frame back) |
| 2 | 2 frames back |
| 3 | 3 frames back |
| … | … |

Picture_type is the same as the current TML, as shown in Table 3.

Table 3: Code_number for Picture_type

| Code_number | Picture Type |
| --- | --- |
| 0 | Inter picture with prediction from the most recent decoded picture only |
| 1 | Inter picture with possibility of prediction from more than one previous decoded picture. For this mode information reference picture for prediction must be signalled for each macroblock |
| 2 | Intra picture |
| 3 | B picture with prediction from the most recent previous decoded and subsequent decoded pictures only. |
| 4 | B picture with possibility of prediction from more than one previous decoded picture and subsequent decoded picture. When using this mode, information reference frame for prediction must be signaled for each macroblock |

Table 4: Code_number for Long Term Buffer

| Code_number |  |
| --- | --- |
| 0 | The 1st long term buffer |
| 1 | The 2nd long term buffer |
| 2 | The 3rd long term buffer |
| 3 | The 4th long term buffer |
| 4 | The 5th long term buffer |
| 5 | The 6th long term buffer |
| 6 | The 7th long term buffer |
| 7 | The 8th long term buffer |

### 2.2	Solution in Sequence or Group of Pictures Level

If we can use group of pictures and put the flag for the boundary of the group of pictures in the header of the GOP, it would save bits compared to the solution in picture level.

As shown in Table 5 for the possible syntax for GOP header, such flags: Store_long_term_buffer and Use_long_term_buffer are used to indicate that the 1st frame of the group of the pictures needs to be stored in the long term buffer after encoding, or it needs to use stored picture in the long term buffer for prediction.

Store_long_term_buffer: 0 is for not storing and 1 is for storing.

Use_long_term_buffer: 0 is for not using and 1 is for using.

After the 1st frame is encoded, the rest of the frames in the group will be coded in the same way as in the current TML.

The possible syntax for GOP header will be:

Table 5: Syntax for GOP Header with Long Term Buffer Flag

| group_of_pictures_header() { | No. Of bits | Notes |
| --- | --- | --- |
| GOP_start_code | 32 |  |
| Store_long_term_buffer | 1 | 0 is for not storing, 1 is for storing |
| if (Store_long_term_buffer == 1) |  |  |
| Long_term_buffer | 3 | 000 is the 1st buffer, 001 is 2nd , …. |
| Use_long_term_buffer | 1 | 0 is for not using, 1 is for using |
| if (Use_long_term_buffer == 1) |  |  |
| Long_term_buffer | 3 | 000 is the 1st buffer, 001 is 2nd , ….. |

The Code_number for Long Term Buffer is shown in Table 6.

Table 6: Code_number for Long Term Buffer

| Code_number |  |
| --- | --- |
| 0 | The 1st long term buffer |
| 1 | The 2nd long term buffer |
| 2 | The 3rd long term buffer |
| 3 | The 4th long term buffer |
| 4 | The 5th long term buffer |
| 5 | The 6th long term buffer |
| 6 | The 7th long term buffer |
| 7 | The 8th long term buffer |

The syntax for Picture_type without flag for Long Term buffer is shown in Table 7.

Table 7: Syntax for Picture_header without Long Term Buffer Flag

| Picture_header() { | No. Of bits | Notes |
| --- | --- | --- |
| Picture_start_code | 32 |  |
| Ref_frame | 5 | Can be restricted to small value for low delay mode |
| Picture_type | 3 |  |

As shown in Table 7, the Store_long_term_buffer and Long_term_buffer flag are removed from Table 1. There are bits saving for either 1, 4, or 3 bits every frame if compared Table 1 with Table  7.

It is not reasonable to use the flag of Store_long_term_buffer in every frame since there are many frames in each group which are not supposed to be stored in long term buffer. But if the solution is done in picture level, it has to include Store_long_term_buffer in every frame.

Ref_frame is the same as the current TML:

Table 8: Code_number for ref_frame

| Code_number | Reference frame |
| --- | --- |
| 1 | The most recent previous frame (1 frame back) |
| 2 | 2 frames back |
| 3 | 3 frames back |
| … | … |

Code_number for Picture_type is the same as in TML as shown in Table 9.

Table 9: Code_number for Picture_type

| Code_number | Picture Type |
| --- | --- |
| 0 | Inter picture with prediction from the most recent decoded picture only |
| 1 | Inter picture with possibility of prediction from more than one previous decoded picture. For this mode information reference picture for prediction must be signalled for each macroblock |
| 2 | Intra picture |
| 3 | B picture with prediction from the most recent previous decoded and subsequent decoded pictures only. |
| 4 | B picture with possibility of prediction from more than one previous decoded picture and subsequent decoded picture. When using this mode, information reference frame for prediction must be signaled for each macroblock |

# Discussion and Conclusion

As shown in the above syntax for the two solutions, it is reasonable to use the solution in the level of group of pictures in term of the concept for using long term buffer and the bit saving.

The concept of long term buffer is very useful for many application scenarios. Introducing the concept in encoding can increase coding efficiency.

It is suggested that the syntax either from the solution in GOP (group of pictures) level or the solution from the picture level should be considered to introduce for better compression for many applications.

# Refecences

[1] VCEG-N48 : Yann Le Maguet  and Cecile Dufour, “Long-Term Prediction Syntax”, Santa Barbara, USA, Sept. 2001

[2] ISO/IEC JTC1/SC29/WG11, MPEG2001/M7512: TML80 Document

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006, Kadoma, Kadoma-Shi, Osaka, 571-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | Pattaya, Thailand, 3 December 2001 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Restricted Multiple Reference Frames with Long Term Frame Buffer |  |
| Contribution number |  | VCEG-049/VCEG-049r1 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: