| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001 | Document VCEG-O60 Filename: VCEG-O60.doc Generated: 6 December ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Yuwen He, Rong Yan, Feng Wu, Shipeng Li Microsoft Research Asia 3F Sigma, No49 ZhiChun Rd, Haidian Beijing, 100080, China | Tel: Fax: Email: | +86-10-62617711-3119 +86-10-88097306 fengwu@microsoft.com |
| Title: | H.26L-based fine granularity scalable video coding |  |  |
| Purpose: | Proposal |  |  |

A new part of MPEG-4 standard has been launched jointly by MPEG and ITU-T [1]. The new standardization effort will use H.26L as the baseline, which may have non-backward compatible (NBC) with existing MPEG standards. H.26L aims at higher coding efficiency developed by ITU-T as a long-term standard. Some high complexity techniques, such as spatial prediction in intra coding, adaptive block size motion compensation and content adaptive binary arithmetic coding (CABAC), are used in this standard [2]. Although H.26L is not completed yet, the testing results show that H.26L can greatly outperform MPEG-4 ASP both at PSNR and visual quality [3].

Since video streaming applications are becoming more and more popular on the current Internet and wireless network, any developing or further video standard has to concern about how to cope with channel bandwidth variations [4]. Many efforts on switching among single bitstreams and scalable coding have been made on H.26L [5][6]. The baseline H.26L for the joint part of MPEG-4 standard does not include any scalable coding techniques so far. But bit rate scalability has been listed on the work plan as an important functionality that should be supported by the joint standard [1].

MPEG-4 FGS provides a fine-grain scalable coding technique, which compresses video sequence into two bitstreams. Base layer bitstream is generated by motion compensation and DCT transform techniques. The errors between original image and reconstructed image of base layer are compressed with bit plane coding technique to form enhancement bitstream. Since the bit plane technique provides an embedded bitstream and fine granularity scalability, the enhancement bitstream can be arbitrarily truncated to fit in the available channel bandwidth. The bit rate of base layer is the low bound of channel bandwidth variation covered by FGS coding.

The FGS coding has only one motion compensation loop at the base layer. The reference is always reconstructed from the low quality base layer to avoid possible drifting errors caused by truncations of enhancement bitstream. But the low quality reference makes motion compensation inefficient in the FGS coding. Compared with non-scalable coding, the FGS coding may lose 2.0dB or more at the same bit rate. Since motion compensation with adaptive block size is adopted by H.26L, non-scalable coding can greatly profit from accurate prediction. If the FGS coding technique is directly immigrated to H.26L, the coding efficiency gap between non-scalable coding and FGS coding would further increase.

The new joint part of MPEG-4 is expected to provide a significant improvement in coding efficiency (about on average 50% or more bit rate reduction for the same visual quality) over the current MPEG-4 ASP. Similarly, the fine granularity scalable coding techniques should be also further improved to match the gain in non-scalable coding. The initial efforts to improve the enhancement layer in coding efficiency were first proposed in [7][8] as progressive FGS (PFGS), where the second motion compensation loop is used at the enhancement layer. Subsequently, three proposals [9]-[11] have developed the initial idea into the framework for efficient and universal scalable video coding at the macroblock level. The proposed framework can significantly outperform MPEG-4 FGS, MPEG-4 FGST and MPEG-4 spatial scalability. There are two key points in the proposed framework: one for adding high quality reference and motion compensation at the enhancement layer, another for selecting the reference for motion compensation and reconstruction by the definitions of different enhancement macroblock coding modes. Some other techniques [12]-[16] proposed to improve coding efficiency can easily integrated into the proposed framework by adding some new coding modes at the enhancement layer.

This proposal integrates H.26L and the basic PFGS framework together and develops an effective scalable coding scheme as a candidate for the joint part of MPEG-4. Since H.26L improves the coding efficiency up to 2.0dB compared with MPEG-4 ASP (Advanced Simple Profile) at the base layer, the enhancement layer can benefit both from H.26L and PFGS coding. The coding efficiency of the proposed scheme is up to 4.0dB higher compared with MPEG-4 FGS at enhancement layer.

H.26L-based fine granularity scalable video coding with high coding efficiency

Figure 1 illustrates the proposed scalable encoder. The base layer part is exactly the same as the H.26L encoder. The reconstructed reference and the current reconstructed error image are introduced into the enhancement layer part. There are two predictions used at the enhancement layer. One is the current reconstructed base layer known as the low quality prediction. Another one is the sum of the prediction from the enhancement layer reference and the current reconstructed error image known as the high quality prediction. Since the low quality prediction is the result after deblocking process, the same filtering is also applied to the high quality prediction.

For each intra macroblock, the difference between the original image and low quality prediction is encoded at enhancement layer. For each inter macroblock, the enhancement layer can select the more effective one from the low quality prediction and the high quality prediction. In general, the same prediction is also used to reconstruct the enhancement layer reference. However, when the high quality prediction is used at the enhancement layer, if part of the enhancement bitstream could not be transmitted to the decoder due to channel bandwidth fluctuations or packet losses, the decoder would not be able to obtain the same high quality prediction as that at the encoder. This would inevitably cause the drifting errors at the enhancement layer at the decoder.

As mentioned above, a method is proposed [9] to effectively reduce the drifting errors at macroblock level. In this method, an enhancement macroblock may be encoded from high quality prediction but reconstructed from low quality prediction. In other words, different references are used for compensation and reconstruction. Since we assume that the low quality prediction at decoder is always consistent with that at encoder, the drifting errors propagated from previous frames can be reset with this method. Two switches S1 and S2 in Figure 1 are used to control the reference for compensation and reconstruction. The reference information is encoded as macroblock mode in the enhancement bitstream and transmitted to decoder.

Figure 1: The diagram of the proposed scalable encoder.

The residue after prediction at the enhancement layer is still transformed with 8x8 DCT, so that we do not have to modify the bit plane coding technique used in MPEG-4 FGS. Each 8x8 DCT block is reordered with zigzag scan and then divided into multiple bit planes. Each bit plane is organized into (Run, EOP) pairs for VLC coding. Assume that m(t) is the total generated bits at the current enhancement layer. Generally, only first n(t) (n(t)<m(t)) bits are used to reconstruct the enhancement reference for the next frame, where n(t) is determined by the bit rate for the enhancement layer reference.

In Figure 1, there is only one motion estimation module located at the base layer part. The same motion information estimated is applied to both motion compensation modules located at these two layers. Since adaptive block size motion compensation scheme is used in H.26L, if the reference for motion estimation always comes from the reconstructed base layer, the base layer can have the best motion compensation. However, it is hard for the enhancement layer to benefit from such an accurate motion prediction, since the motion data estimated by referencing the enhancement layer reference may significantly differ from that by referencing the base layer reference. Considering balancing the efficiency for both the base layer and the enhancement layer, an improved motion estimation algorithm is proposed.

As shown in Figure 1, both of the different quality references are input to the motion estimation module. Each reference is used to separately estimate one set of motion data. In order to save bits for coding motion data, one of them for motion compensation is selected for each macroblock. Thus, the criterion for motion data selection is defined as follows,

SAD=SADb+λ1SADh+λ2Ph−Pb	              	      (1)

where λ1 and λ2 are weighting factors. Function  denotes the mean absolute distance. SADb is the sum of absolute difference between the original image and the low quality prediction, and SADh is the sum of absolute difference between the original image and the high quality prediction. In general, λ1 is a bit more than 1.0 so as to bias the motion data for the high quality reference. Ph and Pb are predictions derived from the two different quality references, respectively. When the base layer encodes the predictive errors from the low quality prediction, and the enhancement layer encodes the predictive errors from the high quality prediction, the third item in formula (1) constrains large difference between predictions caused by different motion data. The motion data with smaller SAD is used in both of the motion compensation modules.

Figure 2: The diagram of the proposed scalable decoder.

The proposed decoder is illustrated in Figure 2. The base layer part is the exactly the same as that in H.26L. Two images are reconstructed at the enhancement layer. The first n(t) bits in the enhancement bitstream are used to reconstruct the enhancement layer reference, and all received bits are used to reconstruct the high quality image for display purpose. Despite that some macroblocks are predicted from the enhancement layer reference at encoder, since they are reconstructed from the base layer in the encoder, they have to be reconstructed from the same reference in the decoder for constraining possible drifting errors. Obviously, this is not necessary in reconstruction of the display image. In this case, the display image still uses the high quality prediction for better quality. Therefore, the proposed scheme allows different predictions in reconstruction of the enhancement layer reference and the display image, which is implemented by two switches S0 and S1 in the proposed decoder.

Experimental Results

This section compares four different scalable coding schemes from combinations from MPEG-4 ASP or H.26L as the base layer, and FGS or PFGS as the enhancement layer, namely, H.26L-noscalable, H.26L-FGS, H.26L-PFGS and MPEG4-FGS. The H.26L codec is used in the experiment is TML 8.0. The sequences Foreman, Coastguard, Table and Carphone are used in experiments with CIF and QCIF formats. The encoding frame rate is 10Hz. For QCIF sequence, the bit rate of the base layer is about 32kbps. For CIF sequence, the base layer bit rate is about 128kbps. For MPEG-4 FGS uses TM5 to control the bit rate of the base layer. Since H.26L does not support constant bit rate coding so far, fixed QP coding is used at the base layer. The range of motion vector is 16 pixels. Only first frame is encoded as I frame, and all the other frames are encoded as P frames. The parameters λ1 and λ2 are set to 1.2 and 0.05, respectively. The encoding parameters used in the H.26L codec are listed in . The bit rate for the high quality reference does not include the bit rate of the base layer.

Table 1: the encoding parameters are used in the H.26L codec.

| Res. | Bitrate(BL) | Reference (HL) | Hadamard | MultiRef-frame | Inter Mode | Entropy Coding | RD Optimize |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QCIF | 32kbps | 72kbps | On | 1 | 7 | UVLC | Off |
| CIF | 128kbps | 512kbps | On | 1 | 7 | UVLC | Off |

From experimental results, we can see that the PFGS technique provides about 2.0dB gain in coding efficiency at moderate or high bit rates compared with FGS technique with the same base layer. The coding efficiency of H.26L is about 2.0dB higher than that of MPEG-4 at base layer. Overall, the proposed H.26L-PFGS scheme with improved motion estimation achieves 4.0 dB coding efficiency gains over MPEG4-FGS at the enhancement layer, which is a quantum leap in making scalable video coding practical.

Conclusion

The scalable video coding scheme that combines H.26L and PFGS is investigated in this proposal. In order to benefit from the adaptive block size motion compensation at the enhancement layer, an improved motion estimation technique that uses both information from the base layer and the enhancement layer is proposed to achieve a good coding efficiency trade-off between the base layer and the enhancement layer. It is shown that such an improved motion estimation scheme is crucial to maintain the coding efficiency gain obtained at the base layer across a wide range of bit rates. The proposed scalable video coding scheme fully exploits the advantages of H.26L and PFGS. Compared with MPEG-4 FGS, the coding efficiency gain can be up to 4.0dB at the enhancement layer. Therefore, we propose to establish the core experiment under the joint video project and verify the advanced FGS techniques for the joint part of MPEG-4 standard.

Reference:

- MPEG requirement group, “Proposed work plan for the joint video project between ITU-T SG16 and ISO/IEC JTC 1/SC 29/WG 11”, ISO/IEC JTC1/SC29/WG11, N4341, Sydney, July, 2001.

- MPEG video group, “Anticipatory joint mode (JM) of enhanced-compression video coding”, ISO/IEC JTC1/ SC29/WG11, N4355, July 2001.

- MPEG test group, “Preliminary results of subjective assessment of responses to video call for new tools to further improve coding efficiency”, ISO/IEC JTC1/ SC29/WG11, N4355, July 2001.

- G. S. Greenbaum, “Remarks on the H.26L project: streaming video requirements for next generation video compression standards”, ITU-T SG16/Q.15, Q15-G-11, February 1999.

- G. Blättermann, G. Heising, D. Marpe, “A Quality Scalable Mode for H.26L”, ITU-T SG16/Q.15, Q15-J24, Osaka, Japan, Oct 2000.

- R. Kurceren, M. Karczewicz, “Improved SP-frame encoding”, ITU-T SG16/Q.15, VCEG-M73, Austin, USA, April 2001.

- S. Li, F. Wu, Y-Q. Zhang, “Study of a new approach to improve FGS video coding efficiency”, ISO/IEC MPEG 50th meeting, M5583, Maui, USA, Dec 1999.

- S. Li, F. Wu and Y-Q. Zhang, “Experimental Results with Progressive Fine Granularity Scalable (PFGS) Coding”, ISO/IEC MPEG 51th meeting, M5742, Noordwijkerhout, Netherland, Mar 2000.

- F. Wu, S. Li, X. Sun, R. Yan and Y-Q. Zhang, “Macroblock-based progressive fine granularity scalable coding”, ISO/IEC MPEG 55th meeting, M6779, Pisa, Italy, January, 2001.

- X. Sun, F. Wu, S. Li, W. Gao and Y-Q. Zhang, “Macroblock-based Progressive Fine Granularity Scalable-Temporal (PFGST) video coding”, ISO/IEC MPEG 55th meeting, M6780, Pisa, Italy, January, 2001.

- R. Yan, F. Wu, S. Li and Y-Q. Zhang, “Macroblock-based Progressive Fine Granularity Spatial Scalablility (mb-PFGSS)”, ISO/IEC MPEG 56th meeting, M7112, Singapore, March, 2001.

- W. Peng, H. Jiang, Y. Chen, “Adding prediction modes to progressive fine granularity scalable (PFGS) coding”, ISO/IEC MPEG 55th meeting, M6909, Pisa, Italy, January, 2001.

- W. Peng and Y. Chen, “Mode-adaptive Fine Granularity Scalability”, ICIP 2001, 993-996, Greece, Oct. 2001.

- R. Kalluri and M. Schaar, “Single-loop motion-compensated based fine-granular scalability (MC-FGS), with cross-checked results”, ISO/IEC MPEG 55th meeting, M6831, Pisa, Italy, January, 2001.

- A. Reibman and L.Bottou, “Managing drift in DCT-based scalable video coding”, DCC2001, 351-359, Salt Lake City, April, 2001.

- A. Reibman, L. Bottou and A. Basso, “DCT-based scalable video coding with drift”, ICIP 2001, 989-992, Greece, Oct. 2001.