| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B020 Filename: JVT-B020.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | ### Interlaced Field Coding Core Experiment A/B |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Michael Gallant, Lowell Winger, Guy Côté VideoLocus Inc. 97 Randall Drive Waterloo, Ontario, Canada N2V 1C5 | Tel: Email: | +1 (519) 725-9797 mgallant@videolocus.com |
| Source: | VideoLocus Inc. |  |  |

_____________________________

# 1.0 Introduction

In this document we summarize results on the portion of the interlaced coding core experiment, defined in VCEG-O59, being conducted by VideoLocus. Complete results are available in the associated document JVT-B020.xls. The functionality for this core experiment has been incorporated in the Test Model software, intermediate release TML8.7a. We would like to again point out the issue with direct mode prediction in B field pictures, as discussed in VCEG-N84 and VCEG-O40 and documented for this meeting in the interlace AHG report, JVT-B010. We continue to propose our solution, wherein direct mode prediction is made from the closest subsequent reference field picture.

# 2.0 Results

The table summarizes our results for picture level experiments for structure combinations A and B outlined in the common conditions of VCEG-O59.  In the table,  (PSNR and bits) represents the difference (dB and %) for coding a sequence as all fields versus coding a sequence as all frames. The significant gains for all but one sequence makes it clear that there is a benefit from the picture level interlaced coding tools. The significant degradation for one of the sequences suggests that the ability to switch between frame and field coding for some material is an important tool. One solution is to permit the adaptation to occur at the picture layer, defined as structure combination E in the common conditions of VCEG-O59 and tested for this meeting in JVT-B071.

| Sequence | Resolution | Configuration |  PSNR (dB) |  bits (%) |
| --- | --- | --- | --- | --- |
| Canoe | 601 | IP | 1.3 | -22.8 |
|  |  | IBBP | 1.303 | -24.2 |
|  | hhr | IP | 1.3128 | -25.77 |
|  |  | IBBP | 1.128 | -24.09 |
| F1 | 601 | IP | 1.307 | -30.79 |
|  |  | IBBP | 1.4 | -33.28 |
|  | hhr | IP | 1.864 | -40.77 |
|  |  | IBBP | 1.656 | -40.73 |
| Rugby | 601 | IP | 2.444 | -40.76 |
|  |  | IBBP | 2.235 | -39.02 |
|  | hhr | IP | 2.444 | -42.45 |
|  |  | IBBP | 2.211 | -40.98 |
| Bus | 601 | IP | 1.385 | -26.65 |
|  |  | IBBP | 1.818 | -34.38 |
|  | hhr | IP | 0.982 | -20.25 |
|  |  | IBBP | 1.427 | -29.79 |
| Football | 601 | IP | 2.357 | -42.97 |
|  |  | IBBP | 2.122 | -39.52 |
|  | hhr | IP | 2.478 | -44.93 |
|  |  | IBBP | 2.247 | -42.25 |
| Tempete | 601 | IP | -2.249 | 75.93 |
|  |  | IBBP | -1.626 | 57.29 |
|  | hhr | IP | -2.22 | 72.54 |
|  |  | IBBP | -1.546 | 54.25 |