| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B021 Filename: JVT-B021.doc Generated: 2002-01-22 |
| --- | --- |

| Title: | A modified Loop Filter of VCEG-O19 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min-Cheol Hong Soongsil University, Sangdo-Dong, Dongjak-Gu, Seoul, Korea Hyung Hoon Oh LG Electronics, Woomyeon-Dong Seoul, Korea | Tel: Email: | +82-2-820-0716 mhong@e.ssu.ac.kr +82-2-526-4404 hhoh@lge.co.kr |
| Source: | Soongsil University / LG Electronics |  |  |

_____________________________

We proposed the QP dependent loop filter to improve visual quality at Pattaya meeting, and it was shown that visual quality of the proposed loop filter at high bit rates outperformed the TML9 loop filter. According to the group request, we slightly modified the algorithm in order to reduce the complexity. Also, signal dependent loop filter is introduced to further improve visual quality.

- Complexity Reduction

The loop filter introduced in Reference [2] takes the frame-based structure, which requires temporal frame memory and expensive memory bandwidth with respect to its implementation. In order to reduce the complexity, we used MB-size buffer. Even though the MB-size buffering is very useful to reduce the memory access bandwidth, the filtered image may result in more or less blurring since the MB boundary pixels may be twice filtered.

2. QP and Signal dependent filtering

TML9 uses different QP table for luminance and chrominance, and therefore smoothness reducing the blocking artifact and clip function projected into pre-defined convex set should be different to different signal to obtain satisfactory results. In this proposal, we used different filter coefficients according to QP and signal. For example, the weak filter introduced in [2] is

L1=(fcoef2∗(l2+r1)+fcoef1∗l1+32)64L2=(fcoef2∗(l3+l1)+fcoef1∗l2+32)64R1=(fcoef2∗(l1+r2)+fcoef1∗r1+32)64R2=(fcoef2∗(r1+r3)+fcoef1∗r2+32)64fcoef2=(64−2∗fcoef1)2                                        (1)

where filter coefficients takes QP and signal dependent values such as

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fcoef1 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 32 | 32 | 32 | 30 | 30 | 28 | 28 | 26 | 26 | 26 | 24 | 24 | 24 | 24 | 22 | 22 | 22 | 22 | 22 | 22 |

for luminance,

| QP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 17 | 18 | 19 | 20 | 20 | 21 | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fcoef1 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 36 | 32 | 32 | 32 | 30 | 30 | 28 | 28 | 26 | 26 | 26 | 26 | 26 | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 24 |

for chrominance. The strong filter coefficients are defined in similar way.

- Experimental results

The modified loop filter has been tested with several QP values, various video sequences, frames rates, and resolutions. The experimental results show that the proposed algorithm effectively reduces blocking artifact both at high and low bit rates, and average15% complexity saving is obtained. The visual comparison at high bit rates was demonstrated, and examples of visual difference at low bit rates are shown in Figures 1-3. In order to see the difference both at high bit rates and at low bit rate, QP values that we tested with are 8, 10, 12, 14, 16, 20, 24, 28. We define H4 and L4 as the results of QP={8, 10,12,14} and QP={16,20,24,28}, respectively. PSNR comparison is in JVT-B021.xls, and demonstration will be shown in the meeting.

Also, the test condition is

MV Resolution: 1/4 pel

Number Reference Frames: 1

Slice Mode: not used

Symbol Mode: UVLC

Input Search Range: 32

RD Optimization: used

Plat-form used:

Pentium III-500 MHz

256 MB RAM

30 GB Hard Drive

- (b)

Figure 1. Example of visual comparison (QP=24)

(a) TML9 filtered frame (b) new filtered frame

(a)                            (b)

Figure 2. Example of visual comparison (QP=24)

(a) TML9 filtered frame (b) new filtered frame

(a)                            (b)

Figure 3. Example of visual comparison (QP=28)

(a) TML9 filtered frame (b) new filtered frame

4. References

[1] T. Wiegand (Ed.), ``H.26L Test Model Long Term Number 9 (TML-9)’’, Dec. 2001.

[2] M.-C. Hong and Y. M. Park, `` Further Experimental Results of VCEG-N30,’’, VCEG-O19, Nov. 2001.

| Submitting Organization or Person: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Organization name |  | Soongsil University/LG Electronics |  |  |  |  |
| Mailing address |  | Soongsil University, School of Electronic Engineering Sangdo-Dong, Dongjak-Gu Seoul Or LG Electronics Woomyeon-Dong Seoul |  |  |  |  |
| Country |  | Korea |  |  |  |  |
| Contact person |  | Min-Cheol Hong |  |  |  |  |
| Telephone |  | +82-2-820-0716 |  |  |  |  |
| Fax |  | +82-2-821-7653 |  |  |  |  |
| Email |  | mhong@e.ssu.ac.kr |  |  |  |  |
| Place and date of submission |  | Seoul / 2002.1.22 |  |  |  |  |
| Relevant Recommendation and, if applicable, Contribution: |  |  |  |  |  |  |
| Number (ex: “H.26L”) |  |  |  | H.26L |  |  |
| Title |  |  |  | A modified loop filter of VCEG-O19 |  |  |
| Contribution number |  |  |  | JVT-B021 |  |  |
| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |  |  |  |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation/Contribution. or, |  |  |  |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Contribution. In which case, |  |  |  |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation. |  |  |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation. Such negotiations are left to the parties concerned and are performed outside the ITU-T. |  |  |  |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |  |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation are affected. - a description of the patent claims covering the Recommendation; |  |  |  |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |  |  |  |
| Patent number(s)/status |  |  | On pending |  |  |  |
| Inventor(s)/Assignee(s) |  |  | Min-Cheol Hong & Hyung Hoon Oh/LG Electronics |  |  |  |
| Relevance to VCEG |  |  |  |  |  |  |
| Any other remarks: |  |  |  |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |  |  |  |

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |