| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B022 Filename: JVT-B022.doc Generated: 2002-01-22 |
| --- | --- |

| Title: | Range Decision for Motion Estimation of VCEG-N33 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min-Cheol Hong Soongsil University, Sangdo-Dong, Dongjak-Gu, Seoul, Korea Hyung Hoon Oh LG Electronics, Woomyeon-Dong, Seocho-Gu, Seoul, Korea | Tel: Email: | +82-2-820-0716 mhong@e.ssu.ac.kr +82-2-526-4404 hhoh@lge.co.kr |
| Source: | Soongsil University/LG Electronics |  |  |

_____________________________

At Santa Barbara meeting, we proposed a dynamic search range decision algorithm to reduce the computational cost of motion estimation, and there was consensus to merge the algorithm into TML9 fast motion estimation. We slightly modify the algorithm and implement the algorithm as pre-processing before doing motion estimation at macro-block level to reduce encoder complexity. It is well-known that motion vector of a block is highly correlated to it neighboring blocks. The dynamic search range decision algorithm introduced in Reference [2] always takes square shape since motion vector direction was not accounted. This contribution proposes an approach to determine dynamic search range separately in vertical and horizontal directions.

1. New Dynamic Search Range Decision

Let assume that E is a macro-block, and A, B, C are its neighboring 44 blocks, as shown in Fig. 1. Also, assume that motion vectors of A, B, and C are (MV_A_x, MV_A_y), (MV_B_x, MV_B_y), and (MV_C_x, MV_C_y), and input search range is input_search_range. The first step of the new algorithm is to estimate the local maximum vector of E. It is

max_MV_E_x=max(abs(MV_A_x),max(abs(MV_B_x),abs(MV_C_x))max_MV_E_y=max(abs(MV_A_y),max(abs(MV_B_y),abs(MV_C_y))   (1)

In Eq. (1), if a neighboring block is outside of image, the motion vector is replaced by input_search_range. The local maximum range that is used for motion search range works well to determine motion vector of E in the stationary region. However, when E is non-stationary moving area, we may have serious motion vector error. The motion vector error can be more serious when all vector components of A, B, and C are ‘0’. For avoiding the problem, local search range is modified as

local_search_range_x=max(k_x,  2×max_MV_E_x)local_search_range_y=max(k_y,  2×max_MV_E_y)   (2)

where k_x and k_y are determined as

k_i=

The result obtained from Eq. (2) may be outside of input search range, and therefore we define new search range to reduce motion vector search points. It is

new_search_range_x=min(input_search_range, local_search_range_x)new_search_range_y=min(input_search_range,local_search_range_y)   (4)

2. Experimental Results

The proposed dynamic search range decision algorithm has been tested with several sequences, resolutions, frame rates, and bit rates. In order to evaluate MV encoding saving, the following criterion is defined;

MV encoding saving=100-MV encoding time of TML9 fast motion estimation with dynamix search rangeMV encoding time of TML9 fast motion estimation×100   (5)

Also, the test condition is

MV Resolution: 1/4 pel

Number Reference Frames: 1

Slice Mode: not used

Symbol Mode: UVLC

Input Search Range: 32

RD Optimization: used

Loop Filter: used

Plat-form used:

Pentium III-500 MHz

256 MB RAM

Detailed performance comparison is in JVT-B022.xls. The results show that MV encoding time is dramatically reduced (about 50%) without significant loss in visual quality.

3 References

[1] T. Wiegand (Ed.), H.26L Test Model Long-Term Number 9 (TML-9), Q.6/SG16(VCEG), Dec. 2001.

[2] M.-C. Hong and Y. M. Park, ``Dynamic Search Range Decision for Motion Estimation,’’ VCEG-N33, Sept. 2001.

[3] A. Hallapuro and M. Karczewicz, `` Complexity Analysis of H.26L,’’ VCEG-M50, April 2001.

[4] S. Wenger, ``H.26LComplexity Analysis according to VCEG-L36 section 2.1.4,’’ VCEG-M23, April 2001.

| Submitting Organization or Person: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Organization name |  | Soongsil University/LG Electronics |  |  |  |  |
| Mailing address |  | Soongsil University, School of Electronic Engineering Sangdo-Dong, Dongjak-Gu Seoul LG Electronics Woomyeon-Dong Seoul |  |  |  |  |
| Country |  | Korea |  |  |  |  |
| Contact person |  | Min-Cheol Hong |  |  |  |  |
| Telephone |  | +82-2-820-0716 |  |  |  |  |
| Fax |  | +82-2-821-7653 |  |  |  |  |
| Email |  | mhong@e.ssu.ac.kr |  |  |  |  |
| Place and date of submission |  | Seoul / 2002.1.22 |  |  |  |  |
| Relevant Recommendation and, if applicable, Contribution: |  |  |  |  |  |  |
| Number (ex: “H.26L”) |  |  |  | H.26L, H.263 |  |  |
| Title |  |  |  | Range Decision for Motion Estimation of VCEG-N33 |  |  |
| Contribution number |  |  |  | JVT-B022 |  |  |
| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |  |  |  |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation/Contribution. or, |  |  |  |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Contribution. In which case, |  |  |  |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation. |  |  |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation. Such negotiations are left to the parties concerned and are performed outside the ITU-T. |  |  |  |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |  |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation are affected. - a description of the patent claims covering the Recommendation; |  |  |  |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |  |  |  |
| Patent number(s)/status |  |  | On pending |  |  |  |
| Inventor(s)/Assignee(s) |  |  | Min-Cheol Hong/LG Electronics |  |  |  |
| Relevance to VCEG |  |  |  |  |  |  |
| Any other remarks: |  |  |  |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |  |  |  |

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |