| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B024 Filename: JVT-B24.doc Generated: 2002-01-15 |
| --- | --- |

| Title: | Coding Performance when not using In-Picture Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Stephan Wenger TU Berlin Sekr. FR 6-3 Franklinstr. 28-29 D-10587 Berlin Germany | Tel: Email: | +49-172-3000813 stewe@cs.tu-berlin.de |
| Source: | TELES AG, ITU Sector Member |  |  |

_____________________________

## Summary

This document comments on the efficiency of the in-slice prediction mechanisms of H.26L.  Slices were used in order to break the in-picture prediction mechanisms intra-prediction and motion vector prediction.  At a slice size of one single macroblock, and when deducting the header overhead for the slices, a bit rate overhead of 0.6 to 22 per cent could be observed.  The results were verified by modifying the encoder to always use a predicted motion vector of zero, and disabling the intra prediction in the function CheckAvailabilityOfNeighbor() in macroblock.c.  The purpose of this document is to share the obtained knowledge with interested parties, and to support the proposal of JVT-B027.

## Simulations performed

The TML reference software version 9.2 was used.  This version is very similar to version 9.4 as made available recently.  The coding parameters were set as outlined in the common conditions document VCEG-M75, with some exceptions, as outlined below.

- The number of coded frames was limited to 30 in order to keep the simulation time reasonable.  Also, there still seems to be a bug in the encoder that makes it crash sometimes, when slices of the length 1 are used.  Due to the lack of time this bug could not be removed, however, when using 30 frames only, all sequence/QP permutations of the common condition can be run reliably.

- Motion Resolution was ¼ pel, with 16 pels search range

- Two reference frames were used (to minimize simulation time)

- No B or SP- frames were used

- The bit stream file format was used, and data partitioning was disabled

- ConstrainedIntraPrediction (intra prediction only from other intra blocks) was used.  Higher overhead can be anticipated if intra prediction would be allowed from inter MBs as well – however, doing so is not desirable in an error prone environment and, hence unrealistic in the environment in question.

- R/D optimization was disabled.  The author does not believe to fully understand the interaction between the R/D optimization and the changes made to the software.

- UVLC entropy coding was used, to avoid the inefficiency of CABAC when using slices of the size of a single macroblock.

In summary, the author believes to use a relatively close approximation to a real-world system’s complexity constraints, by disabling any non-essential complex features of the encoder.

## Results

The overhead introduced by not using any in-picture prediction mechanisms depends on the quantizer, on the sequence, and on the resolution.  Generally, the following statements seem to be correct – at least for the content tested:

- In picture prediction is more helpful at low QP, when more bits are spent for header data in general and motion vectors in particular, and savings there have a bigger impact.

- Bigger picture sizes make in picture prediction less valuable.

- Intra prediction contributes only insignificantly to the savings, in particular when using P pictures and preventing intra prediction from inter information.  It is, however, valuable for Intra pictures.

The two attached graphs show on the X axis the quantizer (16, 20, 24, 28), and on the Y axis the bit rate savings when not using in picture prediction, measured in per cent of the bit rate with in picture prediction.  The first graph was generated using the statistics of the first frame of each sequence, coded as an Intra picture, whereas the second graph is concerned with the results of the first 30 frames of the sequences, including the first Intra frame.