| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B032 Filename: JVT-B032.doc Generated: 2002-01-10 |
| --- | --- |

| Title: | P-Frame Coding with Interpolative Prediction Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Incorporated 12500 TI Boulevard, MS 8649 Dallas, Texas 75243, USA | Tel: Emal: | +1 214 480-3816 zhou@ti.com |
| Source: | Texas Instruments |  |  |

# Introduction

In this document we proposed P-frame coding with interpolative prediction mode, or a so-called quasi B-picture (QB) coding, for the H.26L baseline. Similar to Multihypothesis Motion Pictures (MMP) proposed in [2], a QB picture uses two reference pictures from past, and supports not only the forward prediction but also the interpolative (or bi-directional) prediction. However, unlike MMPs which require a significant syntax change and complexity increase on both encoder and decoder side, the proposed QB pictures are aimed to reduce the encoder complexity by using less number of reference frames, but still maintain the coding efficiency with a minor change in syntax.

# Description of Proposal

Figure 1. H.26L baseline coding: P-pictures with multiple reference picture prediction vs. proposed QB-pictures with prediction from two most recently encoded reference pictures

H.26L [1] is an advanced video compression standard being jointly developed by ITU-T and MPEG. Figure 1 (a) depicts the coding structure of the H.26L baseline, in which both I- and P-pictures are supported. A significant difference of the H.26L P-pictures from traditional P-pictures (in MPEG, H.261, H.263, H.263+) is the multiple reference picture prediction. A H.26L P-picture can have more than one reference picture. Though a macroblock is still limited to predict from one reference picture, which could be any of the multiple reference pictures in the past, but the reference picture number indicating which of reference pictures is selected can be changed from macroblock to macroblock. A macroblock-level syntax element “Ref_Frame” is used for such an indication, Ref_Frame=0 means prediction from the last decoded picture (1 picture back), Ref_Frame=1 means two pictures back, and so on.

Another significant difference is that the H.26L P-pictures support in total seven vector block sizes (i.e. 16x16, 16x8, 8x16, 8x8, 8x4, 4x8, 4x4, see Figure 2). The vector block size determines the number of motion vectors needed for a macroblock to do the forward prediction. For example, if vector block size is 16x16, then there is only one 16x16 vector for this macroblock; if vector size is 8x8, then there are four 8x8 vectors for the macroblock, and so on. The index of each mode in figure 2 also determines coding order of motion vectors in the bitstream.

Figure 2. Numbering of the vectors for the different vector block sizes depending on the inter-mode. For each block the horizontal component comes first followed by the vertical component

The typical number of reference pictures used for H.26L baseline is 5, which leads to a significant increase in both memory requirement and computational complexity. In order to achieve the highest coding efficiency, an H.26L baseline encoder not only has to buffer five reference pictures for P-pictures, but also it needs to do motion estimation for each of five reference pictures, which is extremely time-consuming. On the decoder side, the memory size used for buffering five reference frames will make a decoder expensive.

An alternative is to apply QB pictures to the H.26L baseline. As shown Figure 1 (b), all the P-pictures from the 3rd picture on are replaced by the QB pictures, each QB picture is predicated from the two most recently encoded reference pictures. In order to minimize the syntax changes, in the interpolative mode of the QB picture the macroblock is limited to have the same vector block size (i.e. one of 16x16, 16x8, 8x16, 8x8, 8x4, 4x8, 4x4) for both the reference pictures. With the help of this limitation, “Ref_frame=2” can be used to indicate the interpolative prediction mode, while “Ref_frame=0” & “Ref_frame=1” are still used to represent the forward prediction mode from the reference picture 0 and 1 as the H.26L P-pictures do. However, in the interpolative prediction mode the motion vectors of the second reference picture need to be encoded as well, in addition to the motion vectors of the first reference picture.

Figure 3. Syntax Diagram for H.26L P-frames and proposed H.26L QB-frames

The macroblock-level syntax diagrams of the H.26L P-pictures and the proposed QB pictures are depicted in Figure 3. Compared to the H.26L P-pictures, the QB pictures need a very small syntax modification. Except for the definition of “Ref_frame” and an additional vector coding block (i.e. MVD1 in Figure 3 b) added, everything else remains the same. For a QB picture, the picture coding type is still P-picture (“Ptype = P”), the skipped macroblock rule is still the same (RUN), the intra prediction mode (intra_pred_mode), coded block pattern (CBP) and DCT-coefficient coding (T_coeff) remain unchanged, and the motion vector coding (MVD) is exactly same as in H.26L P-pictures if a macroblock of  a QB-picture uses the forward prediction mode from reference picture 0 or 1. The two major differences are:

- Ref_frame: in the H.26L P-pictures Ref_frame can have any value (typically 0 to 4 for baseline, i.e. 5 reference pictures), “Ref_frame=k” means the forward prediction from the kth reference picture (k+1 pictures back).  In the QB-pictures, Ref_frame has values of 0, 1 or 2, “Ref_frame=0” and “Ref_frame=1” means the forward prediction from reference picture 0 and 1, respectively. “Ref_frame=2” is used to indicate the interpolative prediction which averages predictions from reference picture 0 and 1, motion vectors of both reference pictures are transmitted. The information that only two reference pictures are used can be indicated in profile@level definition.

- MVD1:  if a macroblcok of a QB-picture uses the interpolative mode, the motion vectors of reference picture 0 is encoded in block MVD of figure 3, while the motion vectors of reference picture 1 are encoded by using the following pseudo code.

for ( i = 0; i< cnt_vec; i++) {

DMVX = MVX1[i] - MVX0[i];

DMVY = MVY1[i] – MVY0[i];

H26L_MVD_UVLC(DMVX);

H26L_MVD_UVLC(DMVY);

}

where “cnt_vec” is the number of motion vectors for the current macroblock determined by the vector block size indicated in MB_Type (see also Figure 2), { (MVX1[i], MVY1[i]) | i = 0, 1, … cnt_vec-1} and { (MVX0[i], MVY0[i]) | i = 0, 1, … cnt_vec-1} are block motion vectors of reference picture 1 and 0, respectively. H26L_MVD_UVLC is the differential vector coding using Universal Variable Length Coding defined in H.26L[1].

Therefore, in the cases of less than three reference pictures used (1 or 2 reference pictures), the QB-picture is a superset of the current H.26L P-picture.

# Experimental Results

Experiments were carried out to verify the efficiency of the proposed QB-pictures compared to the H.26L P-pictures. In total six CIF (352 x288) sequences, Akiyo, Coastguard, Foreman, Hall_Monitor, Mother_Daughter, News, are used in the simulation. The sequences are encoded at 10 frame/s, 100 pictures each, with six quantization scales QP = 26, 22, 17, 12, 8, 4 to simulate the wide range of bit-rates. The coding results were measured for three cases: H.26L baseline with two reference pictures (H.26L_2ref), H.26L baseline with five reference pictures (H.26L_5ref) and H.26L baseline with QB pictures (H.26L_QB). The coding structure followed the one illustrated in Figure 1, i.e. only one I-picture at the beginning followed by all P- or QB-pictures. The bit-rates and average PSNR values were computed over the entire sequences.

The experimental results are listed in Table 1. Where H.26l_QB vs. H.26l_2ref and H.26l_QB vs. H.26l_5ef show the improvements of the proposed QB-pictures over the H.26L P-pictures in terms of bit-rate reductions at the same qualities and PSNR improvements at the same bit-rates. As shown in the last row of Table 1, in average the proposed method provides about 4.0% bit-rate reduction (or 0.24 dB PSNR increase), and about 2.2% bit-rate reduction (or 0.13 dB PSNR increase) compared to the H.26L baseline with two and five reference pictures, respectively. The most promising results are in sequence Hall_Monitor, in which peak improvements of about 13% (or 0.75 dB PSNR increase) and about 10% (or 0.59 dB PSNR increase) were measured against the two H.26L baseline cases, respectively.

| QP | H.26L_2ref (with 2 reference pictures) |  | H.26l_5ref (with 5 reference pictures) |  | H.26l_QB (with Quasi B pictures) |  |  |  | H.26l_QB vs. H.26l_2ref |  | H.26l_QB vs. H.26l_5ef |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | bit_rate[kbit/s] | PSNR_Y [dB] | bit_rate[kbit/s] | PSNR_Y [dB] | bit_rate[kbit/s] | PSNR_Y [dB] |  |  | bit_rate[%] | PSNR [dB] | bit_rate[%] | PSNR [dB] |
| Akiyo, CIF (352 x 288), 10 frame/s, 100 frames |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | 22.44 | 33.30 | 22.13 | 33.25 | 22.55 | 33.37 |  | 1.07 |  | 0.05 | 0.53 | 0.03 |
| 22 | 34.83 | 35.90 | 33.65 | 35.94 | 34.65 | 36.02 |  | 2.07 |  | 0.16 | -1.93 | -0.14 |
| 17 | 63.03 | 39.06 | 60.76 | 39.11 | 61.85 | 39.20 |  | 3.81 |  | 0.28 | -0.50 | -0.04 |
| 12 | 121.69 | 42.14 | 117.73 | 42.17 | 118.19 | 42.26 |  | 4.80 |  | 0.32 | 1.05 | 0.07 |
| 8 | 213.22 | 44.42 | 207.01 | 44.45 | 204.65 | 44.53 |  | 5.94 |  | 0.33 | 2.65 | 0.14 |
| 4 | 373.04 | 46.70 | 363.29 | 46.72 | 357.78 | 46.79 |  | 5.62 |  | 0.31 | 2.73 | 0.15 |
| AV | 138.04 | 40.25 | 134.10 | 40.27 | 133.28 | 40.36 |  | 3.88 |  | 0.24 | 0.75 | 0.04 |
| Coastguard, CIF (352 x 288), 10 frame/s, 100 frames |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | 165.17 | 28.10 | 168.95 | 28.04 | 164.20 | 28.14 |  | 2.04 |  | 0.06 | 5.99 | 0.19 |
| 22 | 297.66 | 30.36 | 298.98 | 30.35 | 294.22 | 30.43 |  | 2.43 |  | 0.13 | 3.01 | 0.16 |
| 17 | 615.85 | 33.66 | 614.01 | 33.66 | 605.63 | 33.72 |  | 2.60 |  | 0.17 | 2.37 | 0.15 |
| 12 | 1192.90 | 37.37 | 1190.56 | 37.38 | 1167.64 | 37.44 |  | 2.96 |  | 0.23 | 2.72 | 0.21 |
| 8 | 1937.13 | 40.65 | 1934.05 | 40.65 | 1892.84 | 40.71 |  | 2.94 |  | 0.26 | 2.82 | 0.25 |
| 4 | 2937.14 | 43.84 | 2930.51 | 43.84 | 2865.90 | 43.89 |  | 2.90 |  | 0.28 | 2.71 | 0.26 |
| AV | 1190.97 | 35.66 | 1189.51 | 35.65 | 1165.07 | 35.72 |  | 2.65 |  | 0.19 | 3.27 | 0.20 |
| Foreman, CIF (352 x 288), 10 frame/s, 100 frames |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | 121.39 | 30.54 | 120.18 | 30.49 | 118.21 | 30.59 |  |  | 3.66 | 0.16 | 3.95 | 0.17 |
| 22 | 186.05 | 32.71 | 184.18 | 32.67 | 181.04 | 32.78 |  |  | 3.76 | 0.24 | 3.50 | 0.23 |
| 17 | 331.86 | 35.71 | 325.38 | 35.70 | 322.22 | 35.80 |  |  | 4.26 | 0.30 | 2.39 | 0.17 |
| 12 | 623.95 | 38.99 | 609.52 | 38.99 | 607.08 | 39.09 |  |  | 4.14 | 0.30 | 1.82 | 0.13 |
| 8 | 1058.47 | 41.87 | 1034.33 | 41.87 | 1031.85 | 41.96 |  |  | 3.78 | 0.27 | 1.53 | 0.11 |
| 4 | 1718.53 | 44.66 | 1682.92 | 44.66 | 1677.15 | 44.73 |  |  | 3.35 | 0.25 | 1.37 | 0.10 |
| AV | 673.37 | 37.41 | 659.42 | 37.40 | 656.26 | 37.49 |  |  | 3.83 | 0.25 | 2.43 | 0.15 |
| Hall_Monitor, CIF (352 x 288), 10 frame/s, 100 frames |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | 36.45 | 31.22 | 35.97 | 31.28 | 36.13 | 31.27 |  |  | 2.45 | 0.07 | -1.05 | -0.03 |
| 22 | 70.01 | 33.84 | 68.79 | 33.88 | 68.84 | 33.92 |  |  | 3.03 | 0.17 | 0.64 | 0.04 |
| 17 | 152.69 | 36.97 | 148.93 | 37.01 | 148.46 | 37.11 |  |  | 5.00 | 0.31 | 2.03 | 0.12 |
| 12 | 396.88 | 39.72 | 384.16 | 39.75 | 378.36 | 39.93 |  |  | 8.90 | 0.43 | 5.14 | 0.24 |
| 8 | 838.40 | 42.02 | 810.65 | 42.09 | 778.19 | 42.30 |  |  | 12.90 | 0.64 | 8.33 | 0.40 |
| 4 | 1597.16 | 44.56 | 1561.01 | 44.59 | 1458.36 | 44.80 |  |  | 12.58 | 0.75 | 10.20 | 0.59 |
| AV | 515.26 | 38.05 | 501.59 | 38.10 | 478.06 | 38.22 |  |  | 7.48 | 0.40 | 4.22 | 0.23 |
| Mother_Daughter, CIF (352 x 288), 10 frame/s, 100 frames |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | 36.73 | 32.90 | 35.94 | 32.86 | 36.85 | 32.98 |  |  | 1.37 | 0.06 | 0.00 | -0.00 |
| 22 | 56.10 | 35.25 | 55.26 | 35.25 | 56.08 | 35.38 |  |  | 1.97 | 0.14 | 0.50 | 0.03 |
| 17 | 97.57 | 38.34 | 94.96 | 38.36 | 96.55 | 38.46 |  |  | 2.67 | 0.20 | -0.24 | -0.01 |
| 12 | 188.27 | 41.36 | 182.66 | 41.37 | 185.29 | 41.49 |  |  | 3.68 | 0.24 | 0.45 | 0.03 |
| 8 | 361.20 | 43.75 | 352.58 | 43.76 | 352.58 | 43.86 |  |  | 4.55 | 0.24 | 2.06 | 0.10 |
| 4 | 668.18 | 45.95 | 651.08 | 45.99 | 647.34 | 46.05 |  |  | 5.08 | 0.25 | 1.84 | 0.09 |
| AV | 234.67 | 39.59 | 228.75 | 39.60 | 229.11 | 39.70 |  |  | 3.22 | 0.19 | 0.77 | 0.04 |
| News, CIF (352 x 288), 10 frame/s, 100 frames |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | 57.08 | 31.19 | 56.60 | 31.18 | 56.46 |  | 31.24 |  | 1.95 | 0.09 | 1.46 | 0.07 |
| 22 | 89.57 | 33.89 | 88.67 | 33.89 | 88.43 |  | 33.94 |  | 2.02 | 0.15 | 0.93 | 0.07 |
| 17 | 154.24 | 37.38 | 152.88 | 37.37 | 152.02 |  | 37.46 |  | 2.37 | 0.20 | 1.57 | 0.13 |
| 12 | 273.55 | 40.77 | 271.28 | 40.78 | 268.55 |  | 40.86 |  | 2.91 | 0.23 | 1.94 | 0.15 |
| 8 | 449.12 | 43.45 | 444.94 | 43.46 | 439.59 |  | 43.53 |  | 3.25 | 0.23 | 2.21 | 0.15 |
| 4 | 725.14 | 45.93 | 717.85 | 45.94 | 706.85 |  | 46.00 |  | 3.61 | 0.24 | 2.39 | 0.16 |
| AV | 291.45 | 38.77 | 288.70 | 38.77 | 285.32 |  | 38.84 |  | 2.69 | 0.19 | 1.75 | 0.12 |
| Average Result Over Six Test Sequences |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 507.30 | 38.29 | 500.34 | 38.30 | 491.18 |  | 38.39 |  | 3.96 | 0.24 | 2.20 | 0.13 |

Table 1. Experimental results with six test sequences. AV = Average over six quantization scales

# Conclusions

The proposed QB-pictures require a tiny syntax modification to be adopted into the H.26L standard. Compared to the H.26L baseline with two reference pictures, the proposed method provides a significant improvement in coding efficiency (up to 13% bit-rate reduction) on the cost of a slight coding complexity increase on both encoder and decoder sides due to the interpolative mode. When compared to the H.26L baseline with five reference pictures, the proposed QB-pictures are still able to provide up to 10% bit-rate reduction. However, in this case both the encoder complexity and memory size are significantly reduced by about 50% due to the less time spent on motion estimation and decreased reference picture number from 5 to 2.

# References

[1]  H.26L Test Model Long Term Number 8 (TML-8) draft0, Q.6/SG16(VCEG), 6/28/01

[2] Further Investigation of Multihypothesis Motion Pictures, Q.6/SG16(VCEG-M40), 03/27/01

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 <fax number here>

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Teaxs Instruments Inc. |  |  |
| Mailing address | 12500 TI Blvd., TX-75093 |  |  |
| Country | USA |  |  |
| Contact person | Minhua ZHOU |  |  |
| Telephone | +1 214 480 3816 |  |  |
| Fax |  |  |  |
| Email | zhou@ti.com |  |  |
| Place and date of submission | 01/18/2002 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | P-frame coding with interpolative prediction mode |  |
| Contribution number |  | JVT-B030 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks:

[end]