| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 |  |  | Document: JVT-B036 Filename: JVT-B036.doc Generated: 2002-01-16 |
| --- | --- | --- | --- |
| Title: | Low-Complexity Arithmetic Codec Engine |  |  |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaolin Wu MinimalMass Inc. 17-683 Windermere Rd, London, On. Canada, N5X 3T9 Polytechnic University Brooklyn, NY 11201 Lowell Winger, Michael Gallant VideoLocus Inc. 97 Randall Drive Waterloo, Ontario, Canada N2V 1C5 | Tel: Fax: Email: Tel: Email: | +1.732.6422482 +1.519.6721270 eyelement@rogers.com, xwu@poly.edu +1 (519) 725-9797 lwinger@videolocus.com |
| Source: | [Sourcing Organization] |  |  |

# I.  Introduction

The accompanying document JVT-B036.xls contains rate-distortion results comparing three methods:

[1] CACM/BAC.26L  (current TML9.4 AC engine from Ian H. Witten, Radford M. Neal, John G. Cleary:

Arithmetic Coding for Data Compression. CACM 30(6): 520-540 (1987)).

[2] CACM+/ACM98 (improved AC engine from "Arithmetic Coding Revisited", Moffat, Neal, Witten.

ACM Transactions on Information Systems, 16(3):256-294, July 1998.)

[3] WAC (Xiaolin Wu’s proposed low-complexity AC engine)

This document provides the information required to compare complexity of various AC engines:

1) The algorithms used in QM/MQ and in CACM+/WAC are shown in flowchart form in Figure 1.

2) Table 1 compares operation counts and flushing bits required by the decoders of

QM/MQ, CACM+, CACM/BAC.26L and WAC.

As the AC engine is a larger portion of the complexity of the decoder, we have focused on the decoder for our complexity analysis here.  But the comparison results for the encoder are similar.

N.B. while WAC is claiming 2.2 IPR, CACM+ is to our knowledge a 2.0 IPR claim. (note the 2 attached claims)

A = A - Q

C >= A

Decode(S)

C = C-A

A = Q

D = LPS

Pr-Estimate after LPS

Renorm_d

A>=0x8000

D = MPS

Renorm_d

*Pr-Estimate after MPS

Pr-Estimate after MPS

Get Q

Return D

A: current register

C: code register

*: belong to WAC only

Figure 1.  Simplified decoding flowchart for comparing QM/MQ and CACM+/WAC.

| II. Complexity Analysis Functions | QM/MQ | CACM+ ACM’98 | CACM/ BAC.26L | WAC |
| --- | --- | --- | --- | --- |
| Get Q | 1 LUT | 1 MUL,1 DIV OR 2 MUL,1SHIFT,1 LUT | 2 MUL 2 SHIFT 1 LUT | 1 MUL 1 SHIFT |
| Probability Estimation | 1 LUT 0/1 CMP | 1 CMP 1 ADD 0/1 ADD | 1 CMP 1 ADD 0/1 ADD | 1 CMP 1 SHIFT 0/1 ADD |
| # of Flushing Bits | 4 | 2 | 2 | 2 |

Table 1.  Counts of Decoder operations for QM/MQ, CACM+, CACM/BAC.26L and WAC.

Memory Costs of Context Modeling:

- QM/MQ keeps one state ID of FSM for every context (QM: 112 MQ: 45).

- CACM+ keeps count_0 (total) and count_1 (count of ‘1’) for each context.

- BAC.26L keeps count_0, count_1 and a SCALE_DOWN interval for each context.

- WAC keeps WIN (a 32-bit integer (or two 16-bit integers)) and PONE (a 16-bit integer for count of ‘1’) for each context.

The Techniques to Avoid Division:

- QM/MQ uses an approximation table.   However, the coding efficiency loss can be as much as 5% compared to WAC.

- BAC.26L eliminates DIV by two MUL, two SHIFT, and one LUT:

Q = (A*( (count_1*CUM_FREQ_TABLE[count_0])>>16))>>10;

- CACM+	does direct division:

Q = A*count_1/count_0;

- WAC only requires one MUL and one SHIFT:

Q = (A>>9)*PONE.

Computational Costs of Probability Estimate/Updating Statistics

See Table 1.

Optimized Implemenation on Intel:

WAC has been incorporated into VM-8 of JPEG-2000 (generally considered to be a high-quality implementation of AC that uses the MQ coder). WAC was at least as fast as MQ despite the use of counts for context modeling (more accurate) instead of the states that are used by MQ. An even more favorable comparison was seen when WAC replaced the QM coder in JBIG. Executables can be provided by Xiaolin Wu upon request to compare execution time and compression ratio.

## III. Notes

- A significant (decoder) improvement (not reflected in the table or the figure) between CACM’87/BAC.26L and CACM+/ACM’98/WAC is in the renormalization method [2]. This improves decoding speed by about 10% [2]. The code provided by Videolocus in JVT-B036.zip clearly shows the greatly simplified renormalization loop.

- Get Q (finding the unit interval size) of CACM+ uses one less shift in comparison to CACM/BAC.26L when using the look-up table method. This slightly reduces precision in favor of reduced complexity.

- In the probability estimation (update of cumulative frequencies), CACM/BAC.26L uses 1 add, 1 compare, and 1 additional add that is performed only when a one is decoded. The code in JVT-B036.zip for CACM+ uses 2 adds and 1. However, by nesting two consecutive compares (and increasing the code size slightly) it is possible to have the same operation count for CACM+ as for CACM ie. 1.5 adds. The proper choice will be architecture dependent.

- 16-bit precision for CACM’87/BAC.26L, CACM+/ACM’98 and WAC.

e)    Arithmetic encoder complexity is also reduced by the improved (over CACM’87/BAC.26L) low-  complexity AC engines (see [2] for details).

f)   Novelty of WAC:

- Pre-normalization of denominator to power of two to eliminate division.

- The use of a k-bit shift register to keep the window of most recent k samples (k=32 for modern architecture).  This improves the probability estimate, adaptation to non-stationarity, and eliminates count scaling.

- The main difference between QM/MQ (or the main advantage of QM/MQ) and CACM+/WAC is in that QM/MQ updates count statistics only after renorm_d (i.e., after takes in one input bit), while CACM+/WAC updates counts after each decoded symbol.  Therefore, if the compression ratio of AC is R:1, then CACM+/WAC updates count statistics R times more frequently.  At high bit rate (R gets lower) then the speed difference between QM/MQ and CACM+/WAC becomes smaller, but the coding efficiency of CACM+/WAC gets significantly higher than QM/MQ, because QM/MQ relies on poor probability estimates on relatively high-entropy sources.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | VideoLocus, Inc. |  |  |
| Mailing address | 97 Randall Drive, Waterloo, Ontario, Canada N2V 1C5 |  |  |
| Country | Canada |  |  |
| Contact person | Lowell Winger |  |  |
| Telephone | +01-519-725-9797 |  |  |
| Fax | +01-519-725-5345 |  |  |
| Email | lwinger@videolocus.com |  |  |
| Place and date of submission | Geneva, 29-1 Jan-Feb 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Low-Complexity Arithmetic Codec Engine - CACM+/ACM’98 |  |
| Contribution number |  | JVT-B036 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

# (additional form continues on next page!)

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | MinimalMass Inc. |  |  |
| Mailing address | 17-683 Windermere Rd, London, Ontario, Canada N5X 3T9 |  |  |
| Country | Canada |  |  |
| Contact person | Xiaolin Wu |  |  |
| Telephone | +01-732-642-2482 |  |  |
| Fax | +01-519-672-1270 |  |  |
| Email | xwu@poly.com |  |  |
| Place and date of submission | Geneva, January 28, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Low-Complexity Arithmetic Codec Engine - WAC |  |
| Contribution number |  | JVT-B036 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Provisional patent filed. |  |
| Inventor(s)/Assignee(s) |  | Xiaolin Wu MinimalMass Inc. |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: