| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B037 Filename: JVT-B037.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | PDS, a Low Complexity deblocking for JVT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT. |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Jung, Estelle Lesellier, Yann Le Maguet, Carolina Miró, Jean Gobert Philips Research France 51, rue Carnot B.P. 301 92156 Suresnes Cedex France | Tel: Email: | (33 1) 47 28 35 65/76/77/89/57 yann.lemaguet@philips.com |
| Source: | Philips Research France |  |  |

_____________________________

# Introduction

We propose here a new deblocking algorithm (referred to PDS (Philips Deblocking Solution) in the rest of the document). Note that this deblocking has been tested as a post process function and is to be implemented in the coding loop soon. Our algorithm has been compared to the TML9 deblocking in terms of both quality and complexity. The results so far show that PDS is equivalent to the TML9 deblocking in terms of perceived visual quality (for CIF sequences), and is 3.5 times less complex.

# Presentation of the algorithm

This section describes the proposed deblocking method. First a general overview is given and then the core algorithm is described.

## Overview

The general scheme of the proposed method is given in Figure 1. First, each block is classified according to its spatial activity. Then, the filter to be applied on the edge of each block is selected taking into account the classification, the quantization step, and the pixel values. Finally, a spatial filtering is applied between each 4x4 block to restore the image.

Figure 1: General scheme of the proposed deblocking method.

## Core Algorithm

### Classification

The strength of the filtering depends on the local activity. So, a segmentation in four distinct classes is performed for each 4x4 luminance block.  The selection of the classes is described below, according to the notation given in Figure 2.

Figure 2: Pixels numbering

- Class C1 is the “Homogeneous class”: the 16 pixels of the 4x4 block have all quite the same value. The block ‘a’ is classified C1, if condition (1) is true.

maxa−mina<TClassif=					       (1)

with maxa=maxaiji=1..4,j=1..4 and mina=minaiji=1.4,j=1..4

- Class C2 is the “Column class”: the 4x4 block contains uniform columns. The block ‘a’ is classified C2 if condition (2) is true.

maxaj−minajj=1..4<TClassif					      (2)

with maxaj=maxaiji=1..4 and minaj=minaiji=1..4

- Class C3 is the “Row class”: the 4x4 block contains uniform rows. The block ‘a’ is classified C3 if condition (3) is true.

maxai−minaii=1..4<TClassif					      (3)

with maxai=maxaijj=1.4 and minai=minaijj=1..4

- Class C4 is the “Discontinuity class”: it includes blocks that do not belong to C1, C2 and C3 classes.

Note that a block is in class C1 if it is in both classes C2 and C3.

In our simulations, TClassif has been set to 5. An example of such a classification is given in Figure 3.

|  | Class C1 - Uniform Areas |  | Class C2 –Columns |  | Class C3 – Rows |  | Class C4 –Discontinuities |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 3: Results of the 4-class segmentation on the first Intra frame of the  “Paris” sequence (q=24).

### Filtering process

Four spatial filters of increasing strength are proposed in Figure 4. Black points represent values before filtering, and blue point values after filtering.

| Filter F0: no filtering is applied |  |
| --- | --- |

Figure 4: The three filters used on block edges.

Filter F1 is a soft filter: it changes the values of the two pixels in the neighborhood of the discontinuity. Filters F2 and F3 change respectively 4 and 6 values. They perform more and more strong smoothing. Filter F0 keeps the pixel values unchanged.

Filtering is applied on the horizontal and vertical edges of each 4x4 luminance block and 2x2 chrominance block.

### Filter Selection

The filter selection is the heart of the proposed deblocking. It is based on the classes of the two blocks on each side of the discontinuity and the quantization step q. The key idea is to filter only when it is required and to select the lowest complexity filter as often as possible. No filtering is also applied if:

- The block border is detected as a real-discontinuity (section 2.2.3.1).

- The block border is detected as invisible (section 2.2.3.2).

Between these cases, the lowest complexity filter is selected, depending on the assessed visibility of the discontinuity.

#### Real Discontinuity Detection (Filter Skipping)

A critical point is to decide whether the discontinuity must be filtered or not. This is checked by estimating the luminance gap between the two blocks. In the case of a vertical discontinuity, the discontinuity will be filtered only if both conditions (1) and (2) are met:

mina−minb<gap∗q 					(1)

maxa−maxb<gap∗q 					(2)

with mina and maxa as defined in 2.2.1 and gapis the selection threshold (set to 1.8 in our simulations).

Otherwise, the edge is considered as a real discontinuity and is not filtered.

#### Texture Masking

The basic idea is that distortions are less visible in the textured areas than in homogeneous areas. For a given edge, one of the four filters is selected according to the quantization factor and the classes of the two blocks around this edge. The classification in four classes generates 10 possible configurations. Consequently, an accurate tuning of the filtering, depending on the activity of the blocks is possible. Let us consider we want to filter an horizontal edge (vertical filtering).

- Strong filtering is required in homogeneous areas (C1|C1) and on "column" blocks (C2|C2).

- Weak filtering is sufficient in textured and discontinuities areas (C4|C4).

- No filtering is applied on “row” blocks (C3|C3).

The selection of the filter is consequently based on the expected visibility of the block border, depending on the block activity. According to this statement, filtering rules have been set. Figure 5 represents the selection of the filters for the vertical filtering (filters for the horizontal filtering are obtained by swapping C2 and C3).

Figure 5: Selection of the filter according to the areas and the quantization factor.

#### Chrominance Filter Selection

Chrominance U and V are filtered on the edges of each 2x2 block. The filter used for chrominance is derived from the filter selected for the luminance as shown in Table 1.

| Filter for Luminance | F3 | F2 | F1/F0 |
| --- | --- | --- | --- |
| Filter for Chrominance | F1 | F1 | F0 |

Table 1: Filter selection for chrominance.

#### Results of the Selection

Table 2 shows the frequency of each filter. Figure 6 is a visual representation of the selected filters for the first frame of the “Paris” (q=24) sequence. Filter F3 is represented by a red line, F2 by an orange one, and F1 by a yellow one. Other parts are not filtered.

|  | F0 | F1 | F2 | F3 |
| --- | --- | --- | --- | --- |
| Container (20) | 0.40 | 0.15 | 0 | 0.45 |
| Foreman (24) | 0.29 | 0.51 | 0.01 | 0.19 |
| News (24) | 0.27 | 0.35 | 0.04 | 0.34 |
| Silent (20) | 0.66 | 0.23 | 0 | 0.11 |
| Paris (16) | 0.68 | 0.16 | 0.16 | 0 |
| Mobile (20) | 0.88 | 0.06 | 0.06 | 0 |
| Tempete (28) | 0.18 | 0.54 | 0.09 | 0.19 |

Table 2: Luminance filter selection frequency.

Figure 6: Example of filters selected on the first Intra frame of the  “Paris” sequence (q=24).

### Implementation details

- The various filters are implemented using bit shifting instead of divisions:

Figure 7: Pixel numbering around an edge

F1 filtering:

Q1 = (11*q1 +  5*p1) >> 4;

P1 = ( 5*q1 + 11*p1) >> 4;

F2 filtering:

Q2 = (13*q2 +  3*p2) >> 4;

Q1 = (10*q2 +  6*p2) >> 4;

P1 = ( 6*q2 + 10*p2) >> 4;

P2 = ( 3*q2 + 13*p2) >> 4;

F3 filtering:

Q3 = (14*q3 +  2*p3) >> 4;

Q2 = (11*q3 +  5*p3) >> 4;

Q1 = ( 9*q3 +  7*p3) >> 4;

P1 = ( 7*q3 +  9*p3) >> 4;

P2 = ( 5*q3 + 11*p3) >> 4;

P3 = ( 2*q3 + 14*p3) >> 4;

- Deblocking must be done so that intra-frame prediction is done with not deblocked pixels.

- The order of filtering is the following: the image is filtered macroblock by macroblock. For each macroblock, horizontal edges, except the most bottom one, are filtered from top to bottom, then vertical edges, except the rightmost one,  are filtered from left to right.

- Note that picture borders are not filtered.

# Experimental Results

We have performed simulations using the TML 9 with the following parameters:

- CABAC for entropy coding.

- 1 reference frame for motion prediction.

- 1/4 pixel accuracy motion prediction.

- 16 pixel search for motion estimation.

- No B frames (IPPPPPPP....).

- 7 motion prediction block shapes.

- RD optimization on.

## Objective results

Table 3 shows the bit rate and SNR gains for a set of sequences. Complete bit rate versus SNR curves can be found in JVT-B037.xls. As it can be observed, the average loss in terms of SNR is about 0.4 dB.

| Sequences | Delta Bit Rate | Delta SNR |
| --- | --- | --- |
| Container (QCIF, 10 Hz) | -7.64 | -0.358 dB |
| Foreman (QCIF, 10 Hz) | -9.28 | -0.430 dB |
| News (QCIF, 10 Hz) | -11.48 | -0.656 dB |
| Silent Voice (QCIF, 15Hz) | -6.66 | -0.314 dB |
| Paris (CIF 15 Hz) | -11.56 | -0.602 dB |
| Mobile (CIF 30 Hz) | -8.4 | -0.391 dB |
| Tempete (CIF 30 Hz) | -7.26 | -0.299 dB |
| Incredible (CIF 30 Hz) | -6.34 | -0.350 dB |

Table 3: Bit rate and SNR gains

PDS parameters have been tuned to increase the perceived visual quality.

Firstly, this means that it does not optimize the SNR. Our experience in post-processing and image enhancement of MPEG-2 and MPEG-4, and our participation to the VQEG (Video Quality Expert Group) have revealed that the SNR is unable to assess the perceived quality of a video, and consequently to rank efficiently different algorithms, particularly when the bit rate decreases. The main reasons are that:

- Image quality does not mean image fidelity.

- SNR does not consider the temporal behavior of the sequence.

- HVS properties are such that the human eye is more sensitive to some artifacts than others (luminance, texture or temporal masking for instance).

Nevertheless we agree that no reliable objective metric exists today.

We remain confident with this SNR loss, given that the main idea of PDS is to leave the artifacts if they are not visible. This principle is of course not reflected by the SNR.

Secondly, it means that the sequence quality is optimized considering real-life viewing conditions. For instance, sequences are displayed continuously (no still images), and zooms are possible on QCIF sequences.

## Subjective results

### Test Method

Subjective testing is an experimental method intended to find significant changes in perceived quality. A derivative of the Double Stimulus Impairment Scale (DSIS) method has been chosen. Each test consists of a pair of stimuli, including the reference (JVT v9 deblocking filter) and a sequence processed by PDS, displayed in a random order.  Figure 8 shows the impairment scale used for quality scoring.

Figure 8: Scale for comparative quality scoring.

Each video segment and its reference were presented continuously (no still images) during about 60 seconds. QCIF sequences were magnified by a factor 2. A combination of evaluation by 6 experts and 4 naïve viewers was used. Finally, the mean opinion score (MOS) was computed.

Viewers were asked to rate the PDS deblocked video with respect to the TML9 reference. They were also encouraged to use the entire scale, not just the marked divisions, to convey that the assessment is a continuous value.

Concerning viewing conditions, the viewing distance was in the range of 4H to 6H, four to six times the height of the picture, as recommended by the VQEG, and compliant with the Recommendation ITU-R BT.500-10. The monitor was a Philips 21”, with standard brightness and contrast settings. The maximum observation angle was 30o, and the room illumination was low.

### Test Results

The test set for the CIF sequences (Tempete, Mobile, Paris) has been extended with new sequences with more motion (Schumacher, Incredible), and scene cuts (Incredible, Flattv).

Results are given in Table 4. A positive score means that TML9 deblocking is better than PDS whereas a negative score means the contrary.

| Sequence | Container | Foreman | News | Silent | Paris |
| --- | --- | --- | --- | --- | --- |
| Score | 1.0 | 1.7 | 1.3 | 1.6 | 0.15 |
| Sequence | Mobile | Tempete | Incredible | Flattv | Schumacher |
| Score | 0.75 | -0.75 | -0.94 | -0.78 | -0.11 |

Table 4: Subjective scores for CIF and QCIF sequences.

The conclusions of the subjective tests are the following:

- QCIF results are said “better” or “slightly better” for the TML9 deblocking.

- CIF results are said “equivalent”, or even “slightly better” for PDS.

Concerning QCIF, we believe that the tuning of the parameters is not adequate yet. Moreover, the including of PDS in the encoding loop will help. For CIF sequences, the reduction of the blocking effect in homogeneous areas seems more efficient, while the details preservation seems equivalent. The Intra frame and the frames following scene cuts are clearly better.

# Complexity

The deblocking is the most resource consuming process of a JVT video decoder, hence a particular attention must be devoted to its implementation complexity.

## Complexity factors

The complexity point of view explained hereafter differs somewhat from conventional approaches that often restrict to a count of arithmetic operations. However we claim that we have a better indicator of the implementation complexity when considering the target system on which a video Codec will be implemented. Several criteria are listed by decreasing order of importance on Table 5 with their impact on software or hardware implementation.

| Criteria | Impact on software implementation | Impact on hardware implementation |
| --- | --- | --- |
| Number and locality of memory accesses | Bus bandwidth, cache efficiency, power consumption, number of load/store instructions | Bus bandwidth, cost of dedicated memory, power consumption |
| Data dependency | Length of the critical path, amount of possible parallelism, overall performance | Length of the critical path, amount of possible parallelism, overall performance |
| Data-dependant control flow | Execution flow break, pipeline stall, number of jump instructions, amount of possible parallelism | Execution flow break, pipeline stall, amount of possible parallelism |
| Internal data size requirements | suitability of SIMD instructions | Datapath width of logic operators |
| Computational complexity | Number of arithmetic/logic instructions | Number of logic operators |

Table 5: implementation complexity criteria

For a software implementation, it is very likely that the preferred platform is a DSP or Mediaprocessor that gives better performance than a conventional general purpose processor. These types of platforms support several of the following features:

- SIMD instructions allowing to process collectively several data with the same instruction (e.g. handling four 8-bit pixels packed in 32-bit words).

- Dedicated instruction set for pixel processing (e.g. filter primitives, absolute value, min, max, etc).

- Instruction level parallelism, enabling to launch simultaneously several instructions (e.g. superscalar or VLIW architectures).

- Pipeline stall avoidance instructions, for minimizing the effects of conditional control flow (e.g. mux, guarded instructions).

The Pentium with its MMX instruction set extension, the ARM V6, the Texas Instrument C6X  DSP and the Philips TriMedia are examples of multimedia  platforms.

It has to be noted that impacts on hardware and on software are quite similar, which can be explained by the convergence between Mediaprocessors and dedicated logic.

## Implementation choices of PDS

The PDS algorithm was designed with a high concern of low implementation complexity without compromising performance. The criteria described in the previous section were used as guideline for choosing the algorithm steps.

### Scheduling of horizontal and vertical operations

The scheduling  of filtering horizontal and vertical edges has to be specified in the standard as it affects the results. This choice is very important for the locality of pixel accesses. The proposed  scanning on macroblock basis combines a good locality of accesses to pixels and a medium-grain data dependency between horizontal and vertical filters.

### Filter selection granularity

A basic characteristic of PDS is to apply the same filter to 4 consecutive pixels of an edge, instead of making this choice on a pixel basis. As a consequence, the decision process itself is needed less frequently, and, more importantly, four consecutive pixels can be processed in parallel as they have the same control flow.

### Regular filters

The filters are only linear combinations so that there is no final or intermediary step of clipping. A consequence of this property combined with the larger filter selection granularity makes possible to implement the filter using common multimedia SIMD instructions.

### Filter selection

#### Memory accesses

The filter selection consists essentially in block classification and gap computation. Both are based on the pixel values of reconstructed blocks (and on the quantizer step for classification).  Filter selection can be done advantageously on the fly, at block reconstruction, for optimizing the locality of pixel accesses. The result, i.e. the filter type of edge segments, can be stored in memory for further use by the filter processing itself that must be done with one macroblock row delay. The first criterion of the Table 5 is then favored with this approach as input pixels are already available (in cache for a SW implementation, in the reconstruction local memory for a HW implementation) while output results represent a very modest memory footprint and bandwidth.

#### Processing

Block classification and gap computation need regular processing consisting essentially of min and max operations that can be easily arranged in short sequences of operations. The Min and max operators were chosen because they do not increase the data size of the internal results, so that they can be implemented by successions of SIMD multimedia instructions working on 8-bit element vectors.

The Min, Max, row and column computations are mostly independent, giving a lot of opportunies to the designer for making full use of the available parallelism.

## Comparison of complexity of the current TML9 deblocking and PDS

The PDS algorithm was compared to the current TML9 deblocking scheme for validating the simplification gain.

### Methodology

Critical steps of both algorithms were implemented in C and optimized on a Philips TriMedia DSP that has a VLIW architecture and a multimedia instruction set. Using the compilation tools, it is possible to monitor :

- -	the critical path of important processing steps,

- the potential degree of parallelism obtainable on a virtual processor without resource constraints or achieved on the current processor,

- and finally, the number of CPU cycles necessary to perform a particular function.

We estimate that the obtained results are fairly representative of what can be achieved on a typical Mediaprocessor.

### Parallelism

The intrinsic parallelism of filters across horizontal edges is illustrated on graphs of Figure 9 and Figure 10, for TML9 and PDS respectively.

Figure 9: Execution flow graph of TML9 filters 1 and 2 for a single pixel. Due to data dependencies, parallelism is low.

Figure 10: Execution flow graph (part) of PDS filters 1, 2 and 3 for four pixels. Note the high degree of parallelism (5th to 8th row).

These graphs are drawn for an “infinite” machine, i.e. without any limit on the resources that can work in parallel. On these pictures, the nodes represent operations and the arrows represent dependencies. There is one clock cycle per row of the diagram so that the number of lines gives the minimum number of cycles necessary to perform the whole function. On an actual implementation, the achievable parallelism is not as high as illustrated on the previous graphs because the number of resources is limited and execution of some operation must be deferred. The figures used for the following section were obtained by scheduling on a TriMedia processor able of issuing a maximum of five instructions by cycle.  Compared to the “infinite resource” graph of previous figures, the cycle count increases typically by 30 to 60%.

### Cycle count

The results of compilation and scheduling were used to evaluate  the total number of cycles for CIF sequences at a frame rate of 30 Hz. Statistics of filter occurrences were obtained on several sequences and used to weight the usage of different portions of code (see JVT-B037.xls for details).

#### PDS deblocking

Table 6 shows the results obtained for the PDS algorithm. Note that the number of cycles is larger for the vertical than for the horizontal edges filtering. This difference is not due to the core of the filter itself, performed by SIMD instructions, but to the memory accesses. The read and write that can be performed by words of 32 bits for horizontal edges but must be decomposed in individual pixel accesses for vertical edges.

The total number of necessary CPU cycles is  about 20 Mcycles/s. Taking into account a Cycle Per Instruction (CPI) of 1.5, this leads to a total number of about 30 Mcycles/s.

Table 6: CPU cycle count of the PDS algorithm

#### TML9 deblocking

The TML9 deblocking algorithm is more difficult for implementation on a multimedia processor. Indeed, the artifact detection and the filter selection granularity is of one pixel. This property and the data dependencies of the filters make difficult to exploit the parallelism at instruction level. Moreover, dedicated instructions for pixel processing are hardly usable with the internal steps of clipping.

Table 7: CPU cycle count of the TML9 deblocking algorithm

Table 7 presents the cycle count obtained for the TML9 deblocking algorithm. The total number of necessary CPU cycles is about 71 Mcycles/s, i.e. a total of 106 Mcycles/s when taking into account the CPI.

#### Comparison of results

Comparing results of Table 6 and Table 7, the PDS algorithm needs about 3.5 times less cycles than the current TML9 proposal. These results were obtained on the TriMedia processor, nevertheless we think that they are good indicators of the implementation on any multimedia processor and also of the complexity of a hardware implementation.

# Conclusions

We propose a new deblocking algorithm that provides similar perceived quality as the TML9 deblocking for CIF sequences while reducing the complexity by a factor 3.5. The complexity reduction is obtained by avoiding the filtering when the blocking artifact is not visible, according to perceptual properties, and by the strong parallelism of the algorithm.

Next step consists in including this new algorithm in the encoding loop (this task has already been started). This will mainly require some tuning of the filter selection, especially for QCIF sequences. Having the PDS in the encoding loop should improve the quality due to the re-use of the deblocked image for the motion compensation. This should also reduce the complexity since less blocking artifacts will be present and thus the use of filters F0 and F1 will increase.

Furthermore some new ideas are under investigation:

- Considering luminance masking principle to increase the use of filter F0. The visibility of the blocking artifact is assessed according to the average luminance of the neighborhood.

- Performing chrominance filtering only on 4x4 blocks (instead of on 2x2 blocks) to allow better parallelism.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Philips |  |  |
| Mailing address | See above |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission | JVT meeting Geneva, January 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | PDS, a Low Complexity deblocking for JVT. |  |
| Contribution number |  | JVT-B037 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: