| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B039 Filename: JVT-B039.doc Generated: 2002-01-14 |
| --- | --- |

| Title: | Low Complexity Transform and Quantization – Part II: Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Antti Hallapuro, Marta Karczewicz Nokia Research Center, Tampere, Finland Henrique Malvar Microsoft Research One Microsoft way Redmond, WA 98052-6399 | Tel: Email: Tel: Email: | +358718035390 antti.hallapuro@nokia.com +1 (425) 936-2978 malvar@microsoft.com |
| Source: | Nokia Corporation and Microsoft Corporation |  |  |

_____________________________

# Introduction

This document complements the low-complexity transform and quantization algorithms that we presented in a companion document [1]. We describe here four extensions that may be of interest to JVT: i) support for 11-bit and 13-bit input residuals; ii) extended quantization step size range; iii) a modification of the transform that allows for exact integer invertibility without divisions, which can be used as a building block for lossless encoding; and iv) support for quantization weighting functions.

Throughout this document we assume that the reader is familiar with the contents of [1].

# Support for higher bit-depth of input residuals

Video processing hardware usually works with 8 bits per color component. Some high-end equipment support 10 and 12 bits per color, though. Since pixel prediction residuals have one more bit of dynamic range, there may be a desire to extend the transform and quantization procedures of the current TML [2] to support not only 9-bit, but also 11-bit and 13-bit residuals. We can support those three bit-depths by extending the encoder operations in [1] as described in this section. Decoder remains unchanged.

Note that if for 11- and 13-bit residuals, we should use encoder the implementation as specified in [1] for 9-bit residuals, to maintain the maximum precision in the encoder. In that case, the encoder implementation will require 32-bit multiplications. However, the decoder can still be implemented using only 16-bit multiplications.

As in [1], the symbol // denotes division with rounding to the nearest integer:

## 4x4 luma and chroma transform and quantization

Forward transform

We recall that our proposed 2-D transform is defined by:

For 11-bit and 13-bit residuals, we need to perform post-scaling. For 11-bit residuals it is done as follows:

,

where S11=0000010100000101

For 13-bit residuals, post-scaling is performed by:

,

where S13=1212232312122323 and the values of k are listed in Table 1.

Table 1 – Values of k

|  | k(QP,S13(i,j)) |  |
| --- | --- | --- |
| S13(i,j) | even QP | Odd QP |
| 1 | 1 | 0 |
| 2 | 3 | 1 |
| 3 | 4 | 4 |

Quantization

Quantization is performed according to the following equation

,

where YC are transformed coefficients, Q are the quantization coefficients and f is equal to 217+QP/6/3 for intra and 217+QP/6/6 for inter frames and has the same sign as the coefficient that is being quantized. The quantization coefficients Q are given in the Appendix. Which matrix Sp is used depends on the residual precision p. In what follows a subscript p = 9, 11, 13 will be denoting the input residual precision. Note that for 9-bit residual Sp(i,j) is always 0.

As in [1], note that while the intermediate value inside square brackets in the equation above has a 32-bit range, the final value YQ is guaranteed to fit in 16 bits.

## 4x4 luma DC transform and quantization

The luma DC coefficients of a 1616 block are grouped into a 44 block and further transformed, for intra frames, to improve compression.

Forward transform

The input matrix XD is formed by picking out DC coefficients from the 16 transformed 44 blocks. DC coefficients are then transformed using a symmetric Hadamard transform (since it leads to essentially the same performance as the DCT-like transform in TML), in the form:

with , , and .

Quantization

Quantization is performed using the following equation

,

with , , and.

## 2x2 chroma DC transform and quantization

Forward transform

Chroma DC transform is added on top of chroma transform. Input matrix is formed by picking out DC coefficients from the 4 transformed 4x4 blocks. If these coefficients are denoted as XD then transform is computed as

with , , and .

Quantization

Quantization is performed using the following equation

,

with, , and.

# Complexity

Table 2 summarizes the dynamic range requirements for each stage. There are three numbers in each cell corresponding to 9, 11, and 13-bit input residuals.

Table 2 – Precision

| Luma and chroma transform and quantization |  |  |
| --- | --- | --- |
|  | Input bits | Output bits |
| Forward transform | 9, 11, 13 | 15, 16, 16 |
| Quantization | 15, 16, 16 | 10, 12, 14 |
| De-quantization | 10, 12, 14 | 15, 17, 19 |
| Inverse transform | 15, 17, 19 | 9, 11, 13 |
| Luma DC transform and quantization |  |  |
|  | Input bits | Output bits |
| Transform | 13, 15, 16 | 16, 16, 16 |
| Quantization | 16, 16, 16 | 12, 14, 16 |
| Inverse transform | 12, 14, 16 | 12, 14, 16 |
| De-quantization | 12, 14, 16 | 15, 17, 19 |
| Chroma DC transform and quantization |  |  |
|  | Input bits | Output bits |
| Transform | 13, 15, 16 | 15, 16, 16 |
| Quantization | 15, 16, 16 | 11, 13, 15 |
| Inverse transform | 11, 13, 15 | 11, 13, 15 |
| De-quantization | 11, 13, 15 | 15, 17, 19 |

Since the propose transform can be implemented using only additions and shifts, multiplications are only used during quantization and de-quantization. It can be seen from the Table 2 that all the quantization and de-quantization can be computed with 16-bit multiplication since input values for these operations never exceed 16 bits for 9, 11 and 13 bit residual data. The proposed method can be implemented fully with 16-bit arithmetic when 9-bit residuals are used.

Table 3 summarizes the number of operations required by the encoder. Decoder operations are as specified for 9-bit residuals [1], with the exception that 32-bit memory accesses are needed for 11- and 13-bit residual data. The numbers of operations are computed for the fastest implementation of the transform and inverse transform that are possible using formulas and coefficients presented in [1] and here.

Table 3 – Encoder operations

| 4x4 transform for 24 4x4 luminace and chrominance blocks |  |  |
| --- | --- | --- |
| Process | Operation | Number per block |
| 4x4 forward transform | + | 64 |
|  | << | 16 |
|  | // | 0,4 or 16 |
| 4x4 transform for one 4x4 luminace DC block |  |  |
| Process | Operation | Number per block |
| 4x4 forward transform | + | 64 |
|  | // | 16 |
| 2x2 transform for two 2x2 Chrominace DC blocks |  |  |
| Process | Operation | Number per block |
| 4x4 forward transform | + | 8 |
|  | // | 0 or 4 |
| quantization for coefficient |  |  |
| Process | Operation | Number per coefficient |
| Quantization | * | 1 |
|  | // | 1 |

# Experimental Results

As discussed in [1], we have modified the TML 8.5 software to compute the transform and quantization as specified in Section 3, with no changes in the other parts of encoder and decoder. The encoder was implemented to ensure only 16 bit multiplications for 9-, 11- and 13-bit residual data.

In our tests only 9-bit residual data was used. The reasons were lack of testing material and the current TML software, which dos not support 11- and 13-bit residuals. Still, such tests are a good indication of how the transform would perform for 11- and 13-bit residual data.

Several sequences were processed, for several values of QP, and the resulting peak SNR (PSNR) and bit rates are listed in [3], where we see that the rate-distortion (R/D) curves are nearly identical to those of the unmodified TML. Those R/D curves were then compared, and their distances computed according to the method in [4]. Table 5 shows the results for 11- and 13-bit residuals (for 9-bit residuals, see Table 3 in [1]).

Table 5 (a) – PSNR gains for the proposed method (dB) – 11-bit residuals

| Sequence | PSNR, Y | PSNR, U | PSNR, V |
| --- | --- | --- | --- |
| News | 0.032 | 0.019 | 0.076 |
| Container | -0.002 | -0.046 | -0.069 |
| Silent | 0.030 | 0.069 | 0.052 |
| Foreman | -0.009 | 0.001 | -0.059 |
| Average | 0.013 | 0.011 | 0.000 |

Table 5 (b) – PSNR gains for the proposed method (dB) – 13-bit residuals

| Sequence | PSNR, Y | PSNR, U | PSNR, V |
| --- | --- | --- | --- |
| News | -0.025 | 0.027 | 0.021 |
| Container | 0.032 | -0.008 | -0.088 |
| Silent | 0.038 | 0.090 | 0.006 |
| Foreman | 0.004 | 0.050 | 0.003 |
| Average | 0.025 | 0.040 | -0.015 |

As in [1], we note that the average PSNR gain between the proposed transform and quantization algorithm and the original 32-bit TML code is slightly positive. So, for those test sequences, our proposed transform actually produced a slightly better average PSNR (over all frames of those sequences, for all QP specified in [4]). As we mentioned in [1], for the relatively small data set that we processed, PSNR differences of around 0.02–0.04 dB are not statistically significantly different from zero. Therefore, we can state that our proposed method achieves the same R/D performance of the current TML method, even for the implementations that support 11-bit and 13-bit residuals.

# Extended Range of quantization steps

In some applications it may be desired to encode video signals at very high fidelity, with PSNRs of 50 dB or higher. Depending on the content, it may not be possible to achieve the desired high PSNR even for the smallest value of QP in TML, namely QP = 0. On the other hand, for content that is uncommonly easy to encode, such as simple computer-generated video clips, the highest allowed value in TML for QP, namely QP = 31, may not lead to compression ratios as high as possible with such content.

Thus, an extension of the range for QP may be desired. We now describe a simple modification to our proposal that extends the quantization step sizes by a factor of 4 at both ends. That is equivalent to extending the QP range by 12 values at each end. All equations in Section 3 remain the same, except for two modifications:

New quantization tables

For the de-quantization modification below to work with as low truncation error as possible, we modify slightly the de-quantization coefficients R(·), so all its entries are multiples of 4. Thus, we need to recompute the quantization coefficients Q(·), so the product Q(·)R(·) remains as close as possible to its ideal value. The new values for the quantization tables are shown in the Appendix.

New de-quantization formulas

To support QP < 0, the luma de-quantization equation is replaced by

and the de-quantization formulas for luma and chroma DC are replaced respectively by

and

where QP/6 is truncated towards more negative values, e.g. 1/6 = 0 and (–1)/6 = –1.

New normalization formula

The inverse transformed luma and chroma coefficients are normalized with

Experimental results have shown that for all QP the R/D curves obtained with the new formulas above match those of our original design in [1] within distances of less than 0.01 dB, which is statistically negligible. Since the R(·)’s are multiple of 4, the equations above produce no additional truncation error for |QP/6| >= –2. Also, it is easy to see that even the original formulas support QP beyond the TML maximum value of 31. Thus, for symmetry we extend the QP upper limit by allowing |QP/6| <= 7. Therefore, we expect good performance for values of QP in the range [–12…47].

Notice that the described solution also enables usage of quantization weighting matrices, such as the “quantization tables” used in JPEG [6], if so desired. To support weighting all we need is to replace QP by QP(i,j).

# Transform modification for lossless reconstruction

It is possible to obtain a transform that is perfectly reversible in integer arithmetic, without the need for divisions. All that is needed is a small modification in the transform equations in Subsection 2.1.

Forward transform

M0 = X0 + X3;

M3 = X0 - X3;

M1 = X1 + X2;

M2 = X1 – X2;

2

-

Y0 = M0 + M1;

Y2 = M0 – M1;

-

-1/2

Y3 = M3 - (M2 << 1);

X0

-

X1

X2

X3

-

-

2

Y0

Y2

Y3

Y1

M0

M1

M2

M3

Y1 = (M3 << 1) – (Y3 >> 1);(*)

Inverse transform

M0 = Y0 + Y2;

M1 = Y0 - Y2;

M3 = Y1 + (Y3 >> 1);

M2 = (M3 >> 1) - Y3; (*)

X0 = M0 + M3;

1/2

-

X3 = M0 - M3;

1/2

X1 = M1 + M2;

X0

-

X1

X2

X3

-

-

Y0

Y2

Y3

Y1

M0

M1

M2

M3

X2 = M1 - M2;

The asterisks indicate the only two equations that were modified from the original proposal in [1]. It is easy to see that if we cascade the direct and inverse 1-D transforms, we recover exactly the same data multiplied by 4. Therefore, a final 2-bit right shift of all values recovers the original data exactly. Thus, the transforms above make an integer-reversible pair without divisions.

Note that the modifications above make the transform a biorthogonal pair, with basis functions different from those in our original transform in Subsection 2.1 (in fact, only the first odd basis function is modified, the others stay the same). The basis functions corresponding to the modified transform in this section are scaled versions of those presented in [5].

# Conclusion

We presented extensions to our low-complexity transform and quantization scheme in [1]. With the extended transform and quantization equations as presented, we show that the PSNR loss is negligible not only for 9-bit input residuals, but also for 11- and 13-bit residuals. For 9-bit input residuals, only 16-bit arithmetic is required. A small change in the luma de-quantization formula allows for an extended QP range, from –12 to 47.

We also presented small modifications to the transform equations that make our 44 transform exactly invertible without divisions. Finally, we noted that quantization weighting matrices can be easily supported simply by signaling QPs for each coefficient.

# References

- A. Hallapuro, M. Karczewicz, and H. Malvar, “Low Complexity Transform and Quantization – Part I: Basic Implementation,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6 Document JVT-B038, January 2002.

- T. Wiegand, “H.26L Test Model Long-Term Number 9 (TML-9) draft0”, ITU-T Q.6/SG16 Document TML9.doc (VCEG-N83d1), December 2001.

- A. Hallapuro, M. Karczewicz, “Low Complexity (I)DCT”, ITU-T Q.6/SG16 Document VCEG-O52, November, 2001. See accompanying file “VCEG-O25.xls”.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-curves,” ITU-T Q.6/SG16 Document VCEG-M33, March 2001.

- J. Liang, T. Tran, W. Dai, and P. Topiwala, “Fast Integer Transform and 16-bit architecture for H.26L,” ITU-T Q.6/SG16 Document VCEG-N24, September, 2001.

- W. Pennebaker and J. Mitchell. JPEG Still Image Compression Standard. New York: Van Nostrand, 1993, Chapter 4.

# Appendix - Quantization coefficients

Q[QP%6][i][j] = quantMat[QP%6][0] for (i,j) = {(0,0),(0,2),(2,0),(2,2)},

Q[QP%6][i][j] = quantMat[QP%6][1] for (i,j) = {(1,1),(1,3),(3,1),(3,3)},

Q[QP%6][i][j] = quantMat[QP%6][2] otherwise.

R[QP%6][i][j] = dequantMat[QP%6][0] for (i,j) = {(0,0),(0,2),(2,0),(2,2)},

R[QP%6][i][j] = dequantMat[QP%6][1] for (i,j) = {(1,1),(1,3),(3,1),(3,3)},

R[QP%6][i][j] = dequantMat[QP%6][2] otherwise.

quantMat[6][3] = {{13107, 5243, 8066}, {11916, 4660, 7490}, {10082, 4194, 6554}, {9362, 3647, 5825}, {8192, 3355, 5243}, {7282, 2893 , 4559}};

dequantMat[6][3] = {{40, 64, 52}, {44, 72, 56}, {52, 80, 64}, {56, 92, 72}, {64, 100, 80}, {72, 116 , 92}};

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 936 7329

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia |  |  |
| Mailing address | 6000 Connection Drive Irving, TX 75039 |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 214 763 1314 |  |  |
| Fax |  |  |  |
| Email | marta.karczewicz@nokia.com |  |  |
| Place and date of submission | 01/23/02 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | Low Complexity Transform and Quantization – Part II |  |
| Contribution number |  | JVT-B039 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks:

[end]

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 936 7329

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft |  |  |
| Mailing address | One Microsoft Way, Redmond, WA 98052 |  |  |
| Country | USA |  |  |
| Contact person | Henrique Malvar |  |  |
| Telephone | +1 425 706 2978 |  |  |
| Fax |  |  |  |
| Email | malvar@microsoft.com |  |  |
| Place and date of submission | 01/23/02 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | Low Complexity Transform and Quantization – Part II |  |
| Contribution number |  | JVT-B039 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks:

[end]