| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B043 Filename: JVT-B043.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Coding of Scene Transitions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author: | Miska M. Hannuksela Nokia Mobile Phones P.O. Box 68 33721 Tampere Finland | Tel: Email: | +358 40 5212845 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Introduction

Many types of video material, such as news, movies trailers, and music videos, contain frequent scene cuts. Sometimes scene cuts are abrupt, but in many cases, transitions, such as fades or wipes, are applied. Coding of a scene transition is often a challenging problem from compression efficiency point of view, because motion compensation may not be a powerful enough method to represent the changes between pictures in the transition. A scene transition can also be composed so that scenes are coded separately and the instructions how to reconstruct the transition are coded and transmitted with the coded picture data. However, known methods falling into this category have some shortcomings briefly presented in section 4. The paper proposes a method for coding scene transitions in a scalable and efficient manner.

# Proposal

## Overview

The proposed coding method is described using an example illustrated in Figure 1. There is a cross-faded scene transition between the two scenes. The first presented frame is the last normal picture in the first scene (Coastguard). The second frame consists of Coastguard with 67 % intensity and Foreman with 33 % intensity. The third frame contains Coastguard with 33 % intensity and Foreman with 67 % intensity. The fourth frame is first normal picture of the upcoming scene. Notice that the sequences contain movement.

Figure 1. Cross-faded scene transition.

Pictures in scene 1 (Coastguard) are coded independently from pictures in scene 2 (Foreman). The reconstructed pictures from the two scenes, called component images, are stored in the multi-picture buffer to enable efficient motion compensation during the transition. The cross-faded scene transition is composed from component pictures for display purposes only. Overlapping component images are overlaid so that the top picture is partially transparent. The bottom picture is also called as the source picture. The cross-fade is defined as a filter operation between a source picture and the top picture.

The scene transition can be coded scalably or non-scalably in terms of bit-rate. In scalable coding of the scene transition, scalability layers and sub-sequences proposed in JVT-B042 are applied. The top component pictures are placed in their own sub-sequence in an enhancement layer. Thus, streaming servers can discard them for traffic shaping purposes or apply a weaker transport error protection for them. Figure 2 shows the layer and sub-sequence structure of the example. The number after the picture type is the sub-sequence identifier, and pictures having the same sub-sequence identifier lie in the same sub-sequence. An enhancement layer sub-sequence can be discarded. The first presented picture lies in the base layer at the end of the sub-sequence with identifier 0. The remaining pictures in the Coastguard sequence have been placed in the sub-sequence residing in the first enhancement layer. The pictures in the Foreman scene lie in the base layer in their own sub-sequence. If the enhancement layer is not transmitted or decoded, the cross-fade becomes an abrupt scene change as illustrated in Figure 3.

Figure 2. Layer and sub-sequence structure for cross-faded scene transition.

Figure 3. Result of enhancement layer disposal.

## Modifications to Slice Header

A new slice type, Overlay (O) slice, is defined, and assigned to first free UVLC code index relative to the current slice type definition. If O slice is indicated, the following additional UVLC fields are present in the slice header:

Slice coding type (SCT). This field indicates the slice coding type (Intra, Inter, B, etc.). An overlay slice shall not be indicated again.

Source direction (SDIR). If this field is zero, the source picture for filtering is coded earlier than the current picture, and its picture number shall be calculated by subtracting SPND from the current picture number (in modulo MAX_PN arithmetic). If this field is one, the source picture is coded later than the current picture, and its picture number shall be calculated by increasing the current picture number by SPND (in modulo MAX_PN arithmetic).

Source picture number difference (SPND). The codeword index equals to SPND-1. SDIR and SPND indicate the source picture for transition.

Filter identifier (FID). The field indicates the filter to use. Value zero indicates a cross-fade. Other values are reserved for the time being.

Number of filtering parameters (NFP). This field indicates the number of codewords following the current one and defining the specific parameters for filtering. The semantics of the codewords depend on the filter identifier.

The cross-fade filter includes one filtering parameter, namely the opacity of the current slice. It is a value between one and 255, and the UVLC codeword index equals to opacity - 1. Opacity equal to zero would indicate a non-transparent slice and 256 would indicate a fully transparent slice, whereas values between 0 and 256 indicate a linear transition from non-transparent to transparent defined as follows. If A is a pixel in the source picture and B is the spatially corresponding pixel in the current slice, the pixel resulting from the cross-fade operation shall be equal to (A x opacity + (256 – B) x opacity) / 256, where / is division by truncation.

## Random Access to Scene Transitions

Scalable scene transitions are coded according to JVT-B042, whereas non-scalable ones do not require other changes than described in the previous section. The same sequence can contain both scalable and non-scalable scene transitions. Content creators should use non-scalable scene transitions when the exact visual outlook of the transition is essential. Otherwise, scalable transitions should be used.

Scalable scene transitions typically provide a random access point, as the part of the transition lying in the enhancement layer can be safely disposed. Decoding can start from the base layer, as if the scene transition was abrupt.

Coding and transmission order of the component pictures in a scene transition is not fixed. All pictures in the first scene can be coded and transmitted before any pictures in the second scene. Alternatively, component pictures can be coded and transmitted in ascending order of display time. In any case, a random access to a scene transition may result into a situation where pictures from the previous non-decoded scene are received. These pictures may compensate motion from such pictures that do not exist in the multi-picture buffer of the decoder. Thus, in order to avoid unnecessary confusions of invalid reference pictures, the following text should be added in an appropriate position of the multi-picture buffer section of the standard text:

“If temporal scalability was indicated and if an “Unused” picture was referred to in any multi-picture buffer operation of an O slice, the decoder shall discard the slice.“

## Profile and Level Considerations

The proposal is targeted for “streaming” and “storage” applications. It plays no role in a conversational application, and therefore it is not proposed for baseline.

The syntax or semantics does not limit the number of overlaid component images. For practical implementations, the number has to be fixed. This falls into the scope of level definitions.

# IPR Statement

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission | Geneva, Switzerland, January 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Coding of Scene Transitions |  |
| Contribution number |  | JVT-B043 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

# Appendix A. Comparison to earlier solutions

## Introduction

This section lists other solutions to code scene transitions and gives a short analysis on their strengths and weaknesses compared to the proposal.

## Conventional Coding

Pictures within a scene transition can be coded as any other pictures, of course. Coding of a scene transition is often a challenging problem from compression efficiency point of view, because motion compensation may not be a powerful enough method to represent the changes between pictures in the transition. Moreover, if the transition contains sharp edges between content belonging originally to different scenes, coding artifacts may occur in the neighborhood of the edges.

## Weighted Averaging of B Pictures

Q15-K44, which proposed to scale or weigh the calculation of the prediction block based on the distance between a B-picture and its anchor pictures, was adopted in H.26L. This coding method suits faded scene transitions, although we suspect that its compression efficiency might be slightly worse than in the proposed method. B pictures also provide scalability in terms of bit-rate. However, disposal of B pictures causes a fluctuation in picture rate, which is typically considered somewhat annoying.

## MPEG-4

MPEG-4 allows objects being laid on top of each other. Each object may be associated with an alpha plane that signals its opacity. These features enable coding of scene transitions among other things. However, a separate elementary stream is required for each scene, which makes the system unnecessarily complex. Moreover, video communication systems may have limitations how many simultaneous MPEG-4 visual objects they can support. Finally, there is no mechanism to apply bit-rate scalability.

## SMIL 2.0

The SMIL 2.0 standard (http://www.w3.org/TR/smil20/) allows use of transition filters to any renderable elements, such as images and video. A filtering process is applied to one source or between two sources, and the filtering output is rendered in a specified viewing area. The filter defines a transition from the origin to the destination. The state of the transition is modeled with a progress parameter, and all other parameters of the transition are assumed to be part of the filter effect itself. A transition filter is applied during a specified period in the presentation time-line. SMIL 2.0 does not allow accurate synchronization of the transition effect to the rendered media, but the accuracy of timing is up to implementations. The SMIL 2.0 standard defines a number of wipes and fades. Implementations can ignore non-supported transitions safely but no bit-rate scalability of the media is defined.