| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B053 Filename: JVT-B053.doc Generated: 2002-01-20 |
| --- | --- |

| Title: | ABT Coding for Higher Resolution Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mathias Wien Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52056 Aachen, Germany | Tel: Fax: Email: | ++49-241-8027681 ++49-241-8022196 wien@ient.rwth-aachen.de |
| Source: | RWTH Aachen |  |  |

_____________________________

# Introduction

In this paper, application of Adaptive Block Transforms (ABT) for encoding of ITU-R 601 source material is presented. At the Pattaya meeting, a simplified ABT concept was presented using only the 4x4 and the 8x8 transform matrix [VCEG-O30]. With these transform matrices, 4x4,4x8,8x4, and 8x8 blocks can be transformed. The 8x8 transform is also applied for the blocks of size 8x16, 16x8 and 16x16. Here, we present the coding tools for simplified ABT coding of inter and intra blocks.

The paper is organized as follows: First we define the coding elements needed for ABT inter and intra coding relative to the TML. In the second part, we demonstrate the performance of ABT in the TML-9.0 software. A set of simulation results for ITU-R 601 sequences according to the simulation conditions defined by the Interlaced and the Transform AdHoc groups [VCEG-O59]. The results reveal ABT performance gains of more than 0.4 dB or 8.5% bitrate savings on average and peak gains of more than 1.0 dB or 15% maximum bitrate savings.

# Adaptive Block Transforms

## Inter ABT Coding

With inter ABT, the block size of the transform is connected to the block size used for motion compensation up to a block size of 8x8 pixels. The larger blocks (like 16x8, 8x16, and 16x16) are encoded using the 8x8 block transform. The block tilings and the transform blocks are given in Figure 1.

Figure 1: Macroblock modes and transform block sizes as used with ABT. The transform block size is marked blue if smaller than the subblock size. For block sizes smaller than 8x8, the transform block size is identical to the MC block size.

## Intra ABT Coding

This mode was basically proposed as SVBST coding in [VCEG-O31]. For sake of simplicity, we denote both intra and inter modes by ABT in the following.

### Coding modes

For ABT intra coding 7 macroblock modes with the block sizes from Figure 1 are used. I.e., for intra coding the same macroblock modes are used as for inter coding. The 6 TML intra prediction modes for 4x4 intra coding (directional and DC prediction) are employed for 4x4, 4x8,8x4, and 8x8 blocks. The 8x16,16x8 and 16x16 blocks only use DC prediction.

### Prefiltered Prediction

Directional prediction of blocks larger than 4x4 is improved by prefiltering the boundary pixels prior to prediction. This technique is employed on block boundaries of length 8. In the following, we describe the filtering technique for a prediction from the top boundary of a block with 8xM pixels. Filtering at the left boundary is performed accordingly.

Let P(i,j), i = 0,..,N-1, j = 0,..,M-1, denote the current block of size NxM pixels, with N=8, M{4,8}. I.e., P(n,-1), n = 0,..,N-1, denotes the boundary pixels of the already decoded top neighboring block. P(n,-1) is filtered using a 3-tap filter with the coefficients

[7, 18, 7]/32.

Figure 2 Prediction of a 8x8 block using the TML prediction mode 1 (skew prediction from top left). The last row of the top adjacent block is used for prediction. Left: Mode 1 is applied without prefiltering. Right: Prefiltering is applied first.

At n=0 and n=N-1, the values are repeated for filtering. The filtered signal P'(n,-1) is then used for directional prediction. An example for prefiltered prediction using intra prediction mode 1 (skew prediction from top left) is given in Figure 2.

## Transform Matrices

Two transform matrices of size 4x4 and 8x8 are used for ABT. The 4x4 transform matrix is given in the TML as

T4 = [

13  13  13  13

17   7  –7 –17

13 –13 –13  13

7 –17  17  -7];

The 8x8 transform matrix is defined as

T8 = [

17  17  17  17  17  17  17  17

24  20  12   6  -6 -12 –20 -24

23   7  -7 -23 -23  -7   7  23

20  -6 -24 –12  12  24   6 -20

17 -17 –17  17  17 -17 –17  17

12 -24   6  20 -20  -6  24 -12

7 –23  23  -7  -7  23 -23   7

6 -12  20 –24  24 -20  12  -6 ];

The subblocks of a macroblock are transformed by

Cnxm = Tm x Bnxm  x  TnT,

where Bnxm denotes a block with n pixels and m rows, n,m{4,8}. Tn and Tm are the horizontal and vertical transform matrices  of size nxn and mxm, respectively. Cnxm denotes the transformed nxm block. The transform matrices are not normalized. Normalization is regarded during the quantization and dequantization procedures.

## Quantization

The quantization is performed as described in the TML. Here, two additional quantization tables are needed for the 4x8/8x4 and 8x8 modes.

A4x4(QP=0,..,31) = 
620,553,492,439,391,348,310,276,246,219,195,174,155,138,123,110,98,87,78,69,62,55,49,44,39,35,31, 27,24,22,19,17

B4x4(QP=0,..,31) = 
3881,4351,4890,5481,6154,6914,7761,8718,9781,10987,12339,13828,15523,17435,19561,21873,24552,27656,30847,34870,38807,43747,49103,54683,61694,68745,77615,89113,100253,109366,126635, 141533

A8x4(QP=0,..,31) =
335,299,266,237,211,188,168,149,133,118,105,94,84,75,67,59,53,47,42,37,34,30,26,24,21,19,17,15,13,12,10,9

B8x4(QP=0,..,31) =
2100,2353,2645,2968,3334,3742,4188,4721,5289,5962,6700,7484,8375,9380,10500,11924,13274,14968,16750,19014,20691,23450,27058,29313,33500,37026,41382,46900,54116,58625,70350,78167

A8x8(QP=0,..,31) = 
181,162,144,128,114,102,91,81,72,64,57,51,45,40,36,32,29,25,23,20,18,16,14,13,11,10,9,8,7,6,6,5

B8x8(QP=0,..,31) =
1136,1270,1428,1607,1804,2017,2260,2539,2857,3214,3609,4033,4571,5142,5714,6428,7093,8228,8943,10285,11428,12856,14693,15823,18700,20570,22855,25712,29385,34283,34283,41139

The relation between A() and B() is: 	Anxm(QP) * Bnxm(QP) * N²nxm = 240, where N²nxm denotes the squared norm of the nxm block transforms given below.

| Block Size | N²nxm |
| --- | --- |
| 8x8 | 8 * 172 |
| 8x4 | 4 * sqrt(2) * 13 * 17 |
| 4x8 | 4 * sqrt(2) * 13 * 17 |
| 4x4 | 4 * 132 |

It is assumed that a coefficient K is quantized in the following way:

LEVEL = (KxA(QP) + fx220)/220      f is in the range (0-0.5) and f has the same sign as K.

Dequantization:

K' = LEVELxB(QP)

After inverse transform this results in pixel values that are 220 too high.  A shift of 20 bits (with rounding) is therefore needed on the reconstruction side. The definition of transform and quantization is designed so that no overflow will occur with use of 32 bit arithmetic. Assuming the quantizer step size to increase by 12% per QP, the quantization tables could be extended by 3 finer quantizers for 8x8 blocks and 9 finer quantizers for 8x4/4x8 blocks for high quality transform coding within 32 bit arithmetic.

## Scanning Method and Coded Block Pattern CBPY

4x4

4x8

8x8

8x4

For encoding of the quantized ABT coefficients the blocks are zigzag scanned using full scans of each subblock of the coded macroblock. Figure 2 depicts the scans for 4x4, 4x8, 8x4, and 8x8 blocks. The coefficients are encoded using 2D (Level,Run) symbols for all block sizes. Since the maximum transform block size is 8x8, the TML CBPY can be used for signaling the existence of any coefficient in a 8x8 luma block. The number of subblocks affected by the CBPY bits varies from one 8x8 block to four 4x4 blocks.

Figure 2. Scans for 4x4, 4x8, 8x4 and 8x8 blocks.

## Deblocking Filter

Deblocking is employed on all subblock and macroblock boundaries. No modifications are introduced to the deblocking filter besides the fact that no filtering is performed at 4x4 block boundaries inside the transformed blocks. I.e., the number of block boundaries that have to be filtered is reduced by 1/2 for blocks of size 8x8 and larger and by a factor of 1/4 for 4x8 and 8x4 blocks.

The determination of the filtering strength and the number of filtered pixels in a block is performed according to the TML.

## Entropy Coding

For encoding of the ABT coefficients CABAC is employed. New contexts and binarizations are introduced for encoding of the coefficients of ABT blocks.

### Intra ABT Modes

The block modes used for intra ABT coding have to be signaled. The set of possible intra block modes is identical to the set of macroblock modes used for inter coding. For encoding the ordering of the macroblock modes is flipped.

| Code Number | Intra Block Mode |
| --- | --- |
| 0 | 4x4 |
| 1 | 4x8 |
| 2 | 8x4 |
| 3 | 8x8 |
| 4 | 8x16 |
| 5 | 16x8 |
| 6 | 16x16 |

The reordered intra block modes are encoded using the Macroblock Type context models of the inter macroblock modes.

For INTRA frames, the TML intra macroblock modes are replaced by the ABT intra block modes. For INTER frames, the inter MB_Type indicates intra coding of a macroblock. The intra block mode is encoded next using the Macroblock Type context models.

Intra prediction modes have to be encoded for the 4x4, 4x8, 8x4 and 8x8 intra block modes. The context models for the TML 4x4 intra prediction modes are used for encoding of the ABT intra prediction modes. Two prediction modes are gathered in one symbol for encoding, as it is done for the TML 4x4 intra prediction modes. The larger ABT intra block modes use DC prediction only. Therefore, no intra prediction mode has to be encoded here.

### Coefficients Encoding: CBPY, Level, and Run

#### CBPY

While the bits of CBPY address a variable number of transform blocks, encoding of CBPY remains unchanged from the TML.

#### Level

The Level values of mode 5-7 are coded using the TML Level coding routine. For modes 1-4, CBP = 1 directly signals the existence of at least one non-zero Level in the encoded 8x8 transform blocks. Therefore,  the first Level coefficient in a 8x8 transform block is coded without the first (EOB-) Bin of the TML Level binarization.

#### Run

The maximum Run value for ABT coefficients is dependent on the transform block size. E.g., the maximum Run value is 64 for an 8x8 block, which cannot be encoded efficiently with the unary binarization used in the TML. The binarization used here for the run is given in the following Table 1. 4x4 runs are coded using the TML approach.

|  | Mode |  |  | 4x8,8x4 |  |  |  |  | 8x8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 |  |  |  |  |  |  |  |  |
| 1 | 0 | 1 |  |  |  |  |  |  |  |
| 2 | 0 | 0 | 1 |  |  |  |  |  |  |
| 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 5 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 6 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 7 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| ... | 0 | 0 | 0 | ... |  |  |  |  | . |
| Bin_no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |

Table 1 : Binarization of the Run symbols for the ABT transform block sizes. Runs > 2 are 
binarized by a fixed length binarization.

The first three Runs are binarized by means of the unary code tree. The remaining runs are coded using a fixed length binarization (bit plane binarization).  The number of bins for the binarization of the larger Run symbols is determined by the maximum possible Run in the ABT transform block. 8 bins are used for 4x8 and 8x4 blocks. An additional 9th bin is used for 8x8 blocks. For encoding of the bins 11 contexts are used. The assignment of the contexts is given in Table 2.

| Block Size | Bin_no. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 8x8 | 1 | 2 | 2 | 5 | 6 | 7 | 9 | 10 | 11 |
| 8x4,4x8 | 3 | 4 | 4 | 6 | 8 | 9 | 10 | 11 | - |
| 4x4 | 3 | 4 | 4 | 6 | 8 | 9 | 10 | 11 | - |

Table 2 : Contexts for the Bins depending on the MB mode.

## Motion Estimation

The variable transform block sizes of ABT are taken into account during the motion estimation process. In the TML, the Hadamard transform is performed on blocks of size 4x4. The sum of the absolute values of the transformed block differences (SATD) is used as the error criterion for motion estimation on sub-pel positions.

For ABT, the SATD is calculated using the Hadamard transform on blocks corresponding to the transform block size. Non-normalized Hadamard transform matrices of size 4x4 and 8x8 are employed. Precise normalization of the SATD values would require a division by a factor containing sqrt(2) for the non-square block sizes. Here, the divisions are replaced by integer multiplications and shifts, using 181/512 as an approximation for sqrt(1/8).

| Block size | Normalization |
| --- | --- |
| 8x8 | 1/4 |
| 8x4,4x8 | 181/512 |
| 4x4 | 1/2 |

## Intra Mode Selection

The decision about which intra modes to use for encoding of a macroblock is performed in two steps. The cost criterion applied depends on the complexity mode of the encoder. In the low complexity mode, the SATD chosen as the criterion throughout the decision process. The high complexity mode employs a rate/distortion criterion J using the sum of squared differences (SSD) for distortion D and the real bit cost R.

J = D + MODER

The TML MODE is used as the weighting factor in the rate/distortion criterion.

First, the optimum prediction modes are determined for each intra block mode. This decision has to be performed for the intra 4x4, 4x8, 8x4, and 8x8 modes. The larger modes only use DC prediction. For each subblock, the prediction mode with the minimum cost is selected and the total cost for the intra block is summarized. Then, the total cost for each intra block mode are compared and the optimum intra block mode is chosen.

# Simulations

Simulation results are given according to the common interlaced coding conditions that were agreed on by the Interlaced and the Transform Size AdHoc groups [VCEG-O59]. The ABT coding tools were integrated into the TML9.0 software that was released at the Pattaya meeting [JVT-A003].

## Encoding of interlaced Video

The common condition for interlaced video include field based encoding of interlaced sequences with B-frames. It was agreed that the following setting should be used:

IP BB BB PP BB BB PP …

i.e., the top field of frame #0 is coded I, the bottom field P, the fields of frames #1,#2 are coded B, the two fields of frame #3 are coded P, and so forth.

The reference software as well as the ABT implementation were modified to enable IP BB BB PP coding of a field sequence. Encoding of the B-frames is performed after encoding of the top field of the next P-frame.

## Simulation Conditions

The following common conditions were used for the simulations

| Picture Structure | - II II II - IP PP PP PP - IP BB BB PP BB BB PP |
| --- | --- |
| Motion Resolution | ¼ pel |
| Quantization Parameters | 16,20,24,28 [QP(B) = QP(I/P)+2] |
| Search Range | 16 pixel |
| Hadamard (SATD) | ON |
| Reference Frames | 2 |
| Entropy Coding | CABAC |

The following test sequences were used

| Sequence Name | Frame Size | No. of Frames |
| --- | --- | --- |
| F1_car | 720x576 | 220 |
| Canoa | 720x576 | 220 |
| Rugby | 720x576 | 220 |
| Tempete | 720x480 | 260 |
| Football | 720x480 | 260 |
| Bus | 720x480 | 150 |

The sequences (except for Bus) are available from the VQEG ftp site (www.vqeg.org). They were converted from 422 to 420 using the converter tool provided by Peter Borgwardt [VCEG-N85]. In a pre-experiment all sequences showed improved coding performance for field based coding compared to frame based coding. Therefore, only field based coding was used for the simulations.

### ABT Modes

For performance evaluation of ABT two settings were used for the simulations:

- ABT Inter only. For intra coding the TML intra techniques are used

- ABT Inter&Intra. ABT is used for Inter and Intra Coding

## Results

The following table gives the difference in terms of rate and PSNR as calculated using the Bjontegaard measurement method [VCEG-M33]. The performance of ABT inter and ABT inter&intra is compared to the performance of the TML coder. The corresponding RD plots are given in the Appendix. Note that also for the 720x576 sequences, a frame rate of 30 Hz was used for the calculation of the bitrate according to [VCEG-O59].

| Seq | IBBP Rate% PSNR | IP Rate% PSNR | III Rate% PSNR |
| --- | --- | --- | --- |
| bus | 5.43 % 0.261 dB | 7.11 % 0.333 dB | 6.58 % 0.433 dB |
| canoa | 10.00 % 0.380 dB | 13.16 % 0.489 dB | 9.05 % 0.447 dB |
| f1_car | 5.40 % 0.242 dB | 10.43 % 0.465 dB | 7.07 % 0.461 dB |
| football | 8.78 % 0.389 dB | 11.24 % 0.500 dB | 8.57 % 0.436 dB |
| rugby | 8.88 % 0.438 dB | 10.05 % 0.494 dB | 8.89 % 0.536 dB |
| tempete | 7.12 % 0.290 dB | 8.54 % 0.376 dB | 6.47 % 0.499 dB |
| average | 7.60% 0.333dB | 10.09 % 0.443 dB | 7.77 % 0.469 dB |

Table 3: Average objective performance gains (avsnr).

# Discussion and Conclusion

The concept of Adaptive Block Transforms is proposed for Inter and Intra coding. The scheme makes use of the existing TML 4x4 transform and an additional 8x8 integer transform matrix. The simulations reveal an improved performance of the proposed scheme. The gains are more than 0.4 dB or 8.5% bitrate savings on average. At high rates, gains of about 1.0dB or 15% bitrate savings can be observed. Some IBBP sequences show slight losses for ABT with Inter only. In these cases, Intra coding is widely used due to the small search range and the strong motion in the sequence.

Impact on high resolution sequences for Digital Cinema applications have to be investigated. Further gains are expected for the ABT concept relative to TML on this material.

We propose to adopt ABT into the JM for Interlaced and High Resolution Video.

# References

[VCEG-O30]	Mathias Wien and Jens-Rainer Ohm, 'Simplified Adaptive Block Transforms',
Document VCEG-O30, ITU-T Video Coding Experts Group (VCEG) Meeting 15, Pattaya, Thailand, 4-6 Dec 2001.

[VCEG-O31]	Mathias Wien and Achim Dahlhoff, 'Intra Coding using Variable Block Sizes',
Document VCEG-O31, ITU-T Video Coding Experts Group (VCEG) Meeting 15, Pattaya, Thailand, 4-6 Dec 2001.

[VCEG-O59] 	Peter Borgwardt, 'Core Experiment on Interlaced Video Coding', Document VCEG-O59, ITU-T Video Coding Experts Group (VCEG) Meeting 15, Pattaya, Thailand, 4-6 Dec 2001.

[JVT-A003]	Thomas Wiegand (ed), 'Joint Model Number 1', Document JVT-A003, Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, Pattaya, Thailand, 4-6 Dec. 2001.

[VCEG-N85] 	Peter Borgwardt, 'Core Experiment on Interlaced Video Coding', ITU-T Q6/16 VCEG, Document VCEG-N85r1, Santa Barbara, CA, USA, Sep. 2001.

[VCEG-M33]	Gisle Bjontegaard, 'Calculation of average PSNR differences between RD 
curves,' Document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

# Appendix

## IP BB BB PP

f1 car ccir field TML90 IBBP

QP     Rate	PSNR

28   295270	26.52

24   736506	29.00

20  1504487	31.76

16  2657472	34.57

f1 car ccir field ABT90 IBBP inter

QP     Rate	PSNR

28   378064	27.05

24   834008	29.60

20  1544367	32.18

16  2594857	34.90

f1 car ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

28   405418	27.23

24   899390	29.90

20  1652320	32.51

16  2729950	35.31

canoa ccir field TML90 IBBP

QP     Rate	PSNR

28   383467	26.29

24   781272	28.68

20  1426159	30.96

16  2563608	33.07

canoa ccir field ABT90 IBBP inter

QP     Rate	PSNR

28   469719	26.92

24   853322	29.19

20  1489396	31.28

16  2644086	33.34

canoa ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

28   482240	27.36

24   886996	29.60

20  1579161	31.68

16  2870245	33.70

rugby ccir field TML90 IBBP

QP     Rate	PSNR

28   423065	26.86

24   788036	29.35

20  1367492	31.85

16  2279146	34.49

rugby ccir field ABT90 IBBP inter

QP     Rate	PSNR

28   489579	27.46

24   855461	29.84

20  1448816	32.32

16  2375723	34.96

rugby ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

28   505140	27.81

24   879325	30.25

20  1489825	32.75

16  2430196	35.42

tempete ccir field TML90 IBBP

QP     Rate	PSNR

28   218942	25.40

24   502697	28.05

20  1045075	30.79

16  1950152	33.67

tempete ccir field ABT90 IBBP inter

QP     Rate	PSNR

28   295589	26.40

24   589712	28.88

20  1120160	31.43

16  1991673	34.10

tempete ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

28   297432	26.44

24   594279	28.92

20  1128385	31.47

16  2004394	34.14

football ccir field TML90 IBBP

QP     Rate	PSNR

28   247673	28.60

24   443460	30.72

20   766674	32.96

16  1294804	35.31

football ccir field ABT90 IBBP inter

QP     Rate	PSNR

28   282716	29.14

24   484704	31.21

20   827407	33.44

16  1384240	35.78

football ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

28   289698	29.35

24   496261	31.53

20   847510	33.84

16  1408579	36.24

bus ccir field TML90 IBBP

QP     Rate	PSNR

28   289235	26.05

24   547655	28.60

20   970537	31.18

16  1684094	33.91

bus ccir field ABT90 IBBP inter

QP     Rate	PSNR

28   358350	26.69

24   626193	29.17

20  1067207	31.75

16  1773724	34.47

bus ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

28   358546	26.94

24   633952	29.49

20  1097288	32.08

16  1836014	34.74

## IP PP PP

f1 car ccir field TML90 IP

QP     Rate	PSNR

28   430686	27.37

24  1071492	30.13

20  2140445	32.95

16  3900176	35.79

f1 car ccir field ABT90 IP inter

QP     Rate	PSNR

28   549763	28.22

24  1142747	30.80

20  2120490	33.51

16  3541678	36.14

f1 car ccir field ABT90 IP inter&intra

QP     Rate	PSNR

28   561441	28.17

24  1163236	30.83

20  2170079	33.59

16  3575923	36.26

canoa ccir field TML90 IP

QP     Rate	PSNR

28   528874	27.14

24  1108812	29.54

20  2054568	31.78

16  3755467	33.95

canoa ccir field ABT90 IP inter

QP     Rate	PSNR

28   607639	28.03

24  1155678	30.19

20  2127324	32.31

16  3954428	34.31

canoa ccir field ABT90 IP inter&intra

QP     Rate	PSNR

28   618178	28.13

24  1200654	30.38

20  2244834	32.55

16  4201972	34.60

rugby ccir field TML90 IP

QP     Rate	PSNR

28   600731	27.82

24  1115588	30.29

20  1940620	32.80

16  3249381	35.48

rugby ccir field ABT90 IP inter

QP     Rate	PSNR

28   659144	28.52

24  1173748	30.92

20  2038461	33.51

16  3324820	36.12

rugby ccir field ABT90 IP inter&intra

QP     Rate	PSNR

28   671966	28.60

24  1199267	31.07

20  2078516	33.68

16  3377024	36.31

tempete ccir field TML90 IP

QP     Rate	PSNR

28   348476	25.75

24   826454	28.77

20  1695687	31.75

16  3064302	34.67

tempete ccir field ABT90 IP inter

QP     Rate	PSNR

28   467177	27.03

24   942609	29.65

20  1788926	32.40

16  3074429	35.05

tempete ccir field ABT90 IP inter&intra

QP     Rate	PSNR

28   468693	27.03

24   945242	29.67

20  1795107	32.42

16  3085978	35.07

football ccir field TML90 IP

QP     Rate	PSNR

28   326445	29.34

24   600809	31.56

20  1040957	33.78

16  1764349	36.12

football ccir field ABT90 IP inter

QP     Rate	PSNR

28   356778	29.93

24   635940	32.22

20  1108285	34.59

16  1846245	36.89

football ccir field ABT90 IP inter&intra

QP     Rate	PSNR

28   363017	29.92

24   646062	32.32

20  1127833	34.72

16  1872345	37.04

bus ccir field TML90 IP

QP     Rate	PSNR

28   332912	26.79

24   648731	29.41

20  1194171	32.09

16  2125498	34.96

bus ccir field ABT90 IP inter

QP     Rate	PSNR

28   413759	27.84

24   751569	30.33

20  1342702	33.01

16  2268410	35.67

bus ccir field ABT90 IP inter&intra

QP     Rate	PSNR

28   416288	27.87

24   754353	30.37

20  1350306	33.04

16  2279277	35.71

## II II II

f1 car ccir field TML90 III intra

QP     Rate	PSNR

28  1837590	28.27

24  3294571	31.11

20  5155546	34.03

16  7547563	36.86

f1 car ccir field ABT90 III intra

QP     Rate	PSNR

28  1782876	28.43

24  3124545	31.29

20  4977916	34.24

16  7228862	37.08

canoa ccir field TML90 III intra

QP     Rate	PSNR

28  1673495	28.16

24  2854419	30.61

20  4484711	32.93

16  7217064	35.22

canoa ccir field ABT90 III intra

QP     Rate	PSNR

28  1577750	28.54

24  2665164	30.87

20  4371740	33.12

16  7132787	35.37

rugby ccir field TML90 III intra

QP     Rate	PSNR

28  1616240	28.80

24  2785548	31.40

20  4442442	34.15

16  6737172	36.94

rugby ccir field ABT90 III intra

QP     Rate	PSNR

28  1536515	29.09

24  2617677	31.65

20  4296152	34.42

16  6480278	37.21

tempete ccir field TML90 III intra

QP     Rate	PSNR

28  2533736	26.95

24  4029918	29.98

20  5898119	33.01

16  8444834	35.93

tempete ccir field ABT90 III intra

QP     Rate	PSNR

28  2392939	27.17

24  3794610	30.15

20  5752280	33.19

16  8216266	36.06

football ccir field TML90 III intra

QP     Rate	PSNR

28   975273	30.23

24  1700846	32.46

20  2808196	34.88

16  4361800	37.35

football ccir field ABT90 III intra

QP     Rate	PSNR

28   938738	30.33

24  1622178	32.71

20  2728435	35.18

16  4213422	37.64

bus ccir field TML90 III intra

QP     Rate	PSNR

28  1724381	27.56

24  2890259	30.31

20  4522024	33.23

16  6716814	36.16

bus ccir field ABT90 III intra

QP     Rate	PSNR

28  1643895	27.79

24  2760994	30.51

20  4417033	33.44

16  6528546	36.36

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Institut und Lehrstuhl für Nachrichtentechnik, RWTH Aachen |  |  |
| Mailing address | Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 53056 Aachen, Germany |  |  |
| Country | Germany |  |  |
| Contact person | Mathias Wien |  |  |
| Telephone | +49-241-8027681 |  |  |
| Fax | +49-241-8022196 |  |  |
| Email | wien@ient.rwth-aachen.de |  |  |
| Place and date of submission | Geneva, Switzerland January 29th 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | ABT Coding for Higher Resolution Video |  |
| Contribution number |  | JVT-B053 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | Robert Bosch GmbH |  |  |
| Mailing address |  | PO Box 106050 70049 Stuttgart |  |  |
| Country |  | Germany |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: