| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B055 Filename: JVT-B055.doc Generated: 2002-01-11 |
| --- | --- |

| Title: | ###### New Macroblock Modes for SP-frames |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ragip Kurceren, Marta Karczewicz and Jani Lainema | Tel: Email: | +1 972 894 6907 +1 972 894 4589 ragip.kurceren@nokia.com |
| Source: | Nokia Research Center 6000 Connection Drive, 2-700 Irving, TX, 75039 |  |  |

_____________________________

# Introduction

In VCEG-O-47[1], we have proposed new frame types called, SI-frames, to be used in conjunction with SP-frames[2], [3]. The prediction block in SI-frames is formed identically to Intra-frames whereas the rest of the decoding of the SI-frames is similar to SP-frames. The resulting property of the proposed frame is that SI-frame/slice makes use of only spatial prediction and identically reconstructs a corresponding SP-frame/slice which makes use of motion-compensated prediction. This property provides functionalities in random access, splicing and error resiliency/recovery. Please refer to [4] for detailed discussion of each of these functionalities.

- We have since then provided the software[7] that implements SI-frames in TML 9.0 and the corresponding software tools to demonstrate the usage of SI-frames in random access and bitstream switching.

- In the following we provide a description of SI-pictures and an example of how an SP-frame can be converted into an SI-frame, i.e., SI-frame encoding. These descriptions are copied from VCEG-O-47, for the completeness of this document.

## SI-pictures

The basic principle with SP-frames[2][3] is the tranform and quantization of the predicted block. In the current SP-frames, these steps are applied to the motion-compensated predicted block. We propose a new S- frame type called SI-frame that are similar to I-frames, i.e., employing spatial prediction for a block from the neighboring pixels, whereas the blocks are reconstructed as in SP-frames, i.e., applying the transform and quantization steps for the predicted block from the intra prediction. An additional macroblock type, as illustrated in Table 1, is defined to denote if the current macroblock is an SI or the normal intra. Otherwise, all the syntax elements and their interpretation, e.g., prediction modes and forming the prediction block etc., for this mode is the same as in I-pictures while the decoding of the SI blocks is identical to the SP-frames with the exception of the generation of the predicted block.

Table 1 Macroblock Types for SI-pictures

| Code Number | MB_type |  |
| --- | --- | --- |
|  | Intra | SIntra |
| 0 | Intra-4x4 | SIntra-4x4 |
| 1 | 0,0,0 | Intra-4x4 |
| 2 | 1,0,0 | 0,0,0 |
| 3 | 2,0,0 | 1,0,0 |
| 4 | 3,0,0 | 2,0,0 |
| 5 | 0,1,0 | 3,0,0 |
| 6 | 1,1,0 | 0,1,0 |
| 7 | . | 1,1,0 |
| 8 | . | . |
| 9 | . | . |

## SP-to-SI frame Conversion

In the following we provide an example how an SP-frame can converted into an SI-frame. Note that SP-frames consist of intra(4x4 or 16x16) and inter-type macroblocks. Since intra type macroblock encoding/decoding in SP-frames is identical to that of intra-macroblocks in I and P-type pictures, the bitstream for intra-type macroblocks from SP-frames are copied into SI-pictures with minor alteration. Specifically, the MB_type value is altered to reflect Intra 4x4 in SI-pictures. Therefore, in the following we focus on conversion of inter-type macroblocks from SP-frames into SI-macroblocks in SI-frames.

An inter-type macroblock in an SP-frame is decoded by the reconstructed levels which are calculated by:

L_rec	=	(level*Fq1q2+C_pred*A(QPSP)+ fx2^20)/2^20;

where

F(QP1,QP2)= (A(QPSP)x220 + A(QP)/2)/ A(QP);

- and level denotes the received prediction error coefficient levels and C_pred denotes the transform coefficients of the motion compensated predicted block and A(.) is defined in [5]. Here QP and QPSP denote the quantization parameters for the prediction error block and the predicted block, respectively.

- In order to have identical reconstruction of an SP-macroblock to be calculated as above, SI macroblocks are encoded as follows: First set QP= QPSP and calculate the difference levels:

Level_SI	=	L_rec -  L_intra

- Where L_intra is the quantized transform coefficients of the predicted block from an intra-prediction. A search over the possible intra prediction modes is performed such that the encoding of the difference levels together with the prediction modes leads to the least number of bits similar to intra-type macroblock coding.

## Results

In the following we report simulation results based on the recommended conditions in [6] with TML 8.5. In the simulations, all the frames are encoded as SP-frame except the first frame which is an I-frame and later all the SP-frames are converted into SI-frames or I-frames. Here, we compare the two cases: SI-frame approach and the intra coding with a simple DC prediction ( in this case, dc prediction is done in the coefficient domain rather than spatial domain to have identical reconstruction of the SP-frame). Table 2 illustrates the results of the proposed scheme in terms of the bitrate reduction in % from the I-picture approach. Only the bitrate reductions are reported since the SI and I-frames have identical reconstruction of the corresponding SP-frame, i.e., same PSNR values. The results indicate bitrate savings from 5 to 25%.

| QP | Container | Foreman | News | Silent | Tempete | Mobile |
| --- | --- | --- | --- | --- | --- | --- |
| 16 | 14.79 | 13.42 | 13.19 | 11.54 | 10.92 | 5.68 |
| 20 | 17.07 | 13.18 | 13.78 | 12.26 | 11.47 | 5.43 |
| 24 | 22.74 | 18.68 | 17.97 | 18.07 | 7.48 | 8.85 |
| 28 | 25.52 | 16.36 | 15.78 | 16 | 7.27 | 8.38 |
| Ave.Bitrate Diff.* (%) | 20.12 | 15.58 | 15.39 | 14.74 | 9.23 | 7 |

Table 2 Performance improvement (Bjontegaard Delta bitrate savings in %) of SI-pictures wrt to I-pictures (with only dc prediction) when both identically reconstructs an SP-frame.

## Summary

We have presented a new SP-picture type called SI-pictures. SI-pictures make use of the existing coding blocks for SP-frames and the syntax elements for I-frames with the exception of the additional macroblock type. This new picture-type provides functionalities in random access, splicing and error resiliency/recovery.

# References

[1]	Ragip Kurceren and Marta Karczewicz, " New Macroblock Modes for SP-frames", document VCEG-O47, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG, Pattaya, Thailand, 4-6 Dec., 2001.

[2]	Marta Karczewicz and Ragip Kurceren, " A Proposal for SP-frames", document VCEG-L27, ITU-T Video Coding Experts Group Meeting, Eibsee, Germany, 09-12 January 2001.

[3]	Ragip Kurceren and Marta Karczewicz, " Improved SP-frame Encoding", document VCEG-M73, ITU-T Video Coding Experts Group Meeting, Austin, TX, 02-04 April 2001.

[4]	Ragip Kurceren and Marta Karczewicz, " SP-frame demonstrations", document VCEG-N42r1, ITU-T Video Coding Experts Group Meeting, Santa Barbara, CA, 24-27 September 2001.

[5]	Gisle Bjontegaad, “H.26L Test Model Long Term Number 8 (TML-8) draft 0”, document VCEG-N10, ITU-T Video Coding Experts Group Meeting, Santa Barbara, CA, 24-27 September 2001.

[6]	Gisle Bjontegaard, " Recommended Simulation Conditions for H.26L", document VCEG-M75, ITU-T Video Coding Experts Group, Austin, Texas, USA, 2-4 April, 2001.

[7]       VCEG-O47_software.zip, available at ftp:/standard.pictel.com/video-site/0112_Pat.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX-75039 |  |  |
| Country | USA |  |  |
| Contact person | Ragip Kurceren |  |  |
| Telephone | +1 972 8946907 |  |  |
| Fax | +1 972 8944589 |  |  |
| Email | Ragip.kurceren@nokia.com |  |  |
| Place and date of submission | 11/27/01 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | New Macroblock Modes for SP-frames |  |
| Contribution number |  | JVT-B055 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: