| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B056 Filename: JVT-B056.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Proposal of Improved 2D-VLC coding for High-Bitrate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Shinya Kadono, Kiyofumi Abe and Martin Schlockermann 1006 Kadoma, Kadoma, 571-8501, Japan | Tel: Email: | +81-6-6900-9689 kadono@drl.mei.co.jp |
| Source: | Matsushita Electric Industrial Co., Ltd. |  |  |

_____________________________

## Introduction

We propose a new 2D-VLC encoding scheme for high-bitrate (small QP). Basic idea of this technique has already proposed at 1st JVT meeting as VCEG-O27. In Technical change from VCEG-O27 is only introducing is_level_not_scaled flag to indicate level scaling.

We show that the proposed scheme reduce bits about 4-7% at QP = 2 for I-pictures and about 1-2% bit saving is achieved at QP = 2 for P-pictures.

## Proposed Method

### Basic Idea

If QP is small, absolute value of quantized coefficients become large. In that situation, VLC table that includes codes for combinations of Run and Level for 2D-VLC, may not suitable. That is, average run length become shorter and average level size becomes larger than that of large or medium QP size.

Our purpose is to improve 2D-VLC coding efficiency by modifying VLC table, that is optimised for small QP and depends on the numbers of uncoded quantized coefficients in the block. We have developed the following three methods.

- Level scaling 2D-VLC table design

- position adaptive 2D-VLC table design

- inverse order zigzag scanning

These methods do not need extra memory to store new 2D-VLC table because each code can be arithmetically calculated by referring original VLC table.

### Level scaling 2D-VLC table design

For small QP size, larger absolute level value occurred more frequently than for medium or large QP size. Therefore, it is better to assign shorter codes for combination of large absolute level value and short run by sacrificing codes for combination of small absolute level value and long run.

To achieve this by simple arithmetic operation,  we assign ( Level, Run ) event to code number

2 * { code2DVLC(Level/2, Run) - Level modulo 2 } + 1	for Level > 0

2 * { code2DVLC(Level/2, Run) - Level modulo 2 }	for Level < 0

where code2DVLC(Level, Run) function gives code number of (level, run) event. Figure 1 shows the scaled table, that is converted from the original Tcoeff_chrom_DC table of TML9.0.

Figure 1: Scaled table for Tcoeff_chrom_DC.

### Position adaptive 2D-VLC table design

VLC codes of (Level, Run) events have never used if Run is greater than the number of uncoded quantized coefficients in the target block. Therefore, we can optimize 2D-VLC table by eliminating these (Level, Run) events depending on the number of uncoded quantized coefficients (Nuncoded) in the target block.

However, changing the table depending on Nuncoded may raise implementation complexity. To reduce it, we introduce the following two constraint.

- 2D-VLC table consists of two part, the irregularly ordered part (table look up VLC part) which is located on the top of table, the regularly ordered part by simple arithmetic operation which is located on the bottom of the table.  We modify the regularly ordered part only in the table.

- By eliminating the (Level Run) events of Run > Nuncoded, values of Run appear in the table from 0 Nuncoded -1. However we need multiplication or division operation to calculate the regularly part if Nuncoded is not powers of 2. Therefore we eliminate the (Level Run) events of Run > MaxRun only, where MaxRun+1 means the minimum integer of powers of 2 not less than Nuncoded.

Figure 2 shows the position adaptive 2D-VLC table, that is converted from the original Tcoeff_chrom_DC table of TML9.0.

MaxRun = 3                 MaxRun = 1             MaxRun = 0

Figure 2: Position adaptive 2D-VLC table for Tcoeff_chrom_DC.

### Inverse order zigzag scanning

By introducing position adaptive 2D-VLC, we can encode (Level, Run) event with shorter codes if Nuncoded is large, and its reduction performance is more efficient for the event that has large code number. Usually the event having large code number occurs at the quantized coefficients corresponding to low frequency. Therefore, we can reduce bits to encode (Level, Run) events by reordering from high frequency to low frequency order before 2D-VLC.

Encoding step becomes as follows.

- List up (Level, Run) events by using trivial zigzag scanning order from low frequency to high frequency. Then, get (Level1, Run1), (Level2, Run2), (Level3, Run3), (Level4, Run4),…, (Level n-2, Run n-2), (Level n-1, Run n-1), (EOB, 0).

- Reordering events from high frequency to low frequency order. The, get (Level n-1, Run n-1), (Level n-2, Run n-2),…, (Level4, Run4), (Level3, Run3), (Level2, Run2), (Level1, Run1), (EOB, 0).

- Encode the reordered events using the position adaptive 2D-VLC table.

Figure 3 shows examples of the original and the inverse order zigzag scanning for encoding luminance block. In this example, event is represented as (Level, Run, UVLC) and both scanning method use position adaptive 2D-VLC.

Figure 3: Example of inverse order zigzag scanning.

### Syntax Changes to TML9.0

Generally speaking, the proposed level scaling scheme works better with small QP size and worse with large QP than TML9.0. However, a decision whether the proposed scheme works better or worse than TML9.0 deeply depends on its sequence.

Therefore, we introduce a new flag is_level_not_scaled just after picture type syntax element, which is encoded using UVLC.

if (is_level_not_scaled == ‘1’)

level scaling is not used for this picture

else

level scaling is used for this picture

Figure 4: Syntax diagram for all the elements in the video bitstream.

## Simulation

### Conditions of Simulation

In order to confirm the effectiveness of the proposed method, we have implemented the proposed methods into TML9.0 software and compared it with TML9.0.

Simulation conditions are the same as VCEG-N81 (recommended simulation common condition) except QP as follows:

- Software version: TML9.0.

- Motion estimation and mode selection: High complexity mode.

- QP: 2, 4, 6, 8, and fixed for all over sequence.

- Picture coding type:  IPPPPP for P-picture simulations, IIIIII for I-picture simulations.

- Number of reference frames: 5 frames.

- The motion vector search range: 32 pels.

- Motion compensation: 1/4-pel accuracy for QCIF, 1/8-pel accuracy for CIF.

- Other coding options: using Hadamard, no CABAC, and no slice structure.

- Test sequences: see Table 1.

Table 1: Test seaquances.

| Frame Skip | 2 | 1 | 0 |
| --- | --- | --- | --- |
| Sequences | Container, QCIF,300 News, QCIF,300 Foreman, QCIF,300 | Paris, CIF,300 Silent Voice, QCIF,300 | Mobile CIF 300 Tempete CIF 260 |

For convenience, we define the following abbreviations.

| Original: | Original TML9.0. |
| --- | --- |
| Scl: | Level scaling is used. |
| Pos: | Position adaptive 2D-VLC is used. |
| Pos_Inv: | Position adaptive 2D-VLC and inverse scanning are used. |
| Scl_Pos_Inv: | Level scaling, position adaptive and inverse scanning are used. |

### Simulation Results

As performance of the proposed methods are depends QP, we calculate avsnr using each successive two points on the RD-curves. That is, we get 3 avsnr values for each RD-curves by using points of QP = 2 and 4, using points of QP = 4 and 6, and using points of QP = 6 and 8, respectively

From Figure 5 to Figure 11 shows bit savings for I-picture and P-picture compared to TML9.0, which are calculated using avsnr software. From these result, it is obvious that the proposed scheme improves more coding performance at small QP ( high bitrate ) and the rate of reducing bits depends on test sequence. For I-picture, about 7% bits can be reduced for ‘Mobile’, and about 5% bits can be reduced for ‘News’ and ‘Paris’. For P-picture, about 2% bits can be reduced for ‘News’ and ‘Tempete’.

For I-picture, Scl_Pos_Inv shows the best performance for all sequence. For P-picture, Scl and Scl_Pos_Inv show the worst performance for some sequences; however, Pos_Inv always shows better performance than TML9.0.

### Study of Results

Level scaling works better if there are many large Level and short Run quantized coefficients. Position adaptive 2D-VLC and inverse order scanning work better if there are many non-zero quantized coefficients. Both situations occur by using small QP. Further more, bits for encoding quantized coefficients dominate for small QP while bits for encoding motion vector dominate for large QP. This is the reason why our scheme shows better performance for small QP.

Level scaling contributes much than the other proposed method, however its performance deeply depends on QP and sequence. Sometimes, as shown in Figure 5(b), it may increase many bit rates. Position adaptive 2D-VLC and inverse scanning always work well. Therefore we introduce is_level_not_scaled flag to switch enabling or disabling only level scaling.

## Conclusion and Remarks

A new 2D-VLC coding scheme is proposed. It consists of level scaling method, position adaptive VLC method and inverse order zigzag scanning method. The proposed scheme needs only a slightly computational overhead compared to original TML 9.0. By simulation results, the proposed scheme reduce bits about 4-7% at QP = 2 for I-pictures, and about 1-2% bit at QP = 2 for P-pictures. Therefore we proposed to this scheme is incorporated into the next TML.

## References

- T.Wiegand, “H.26L Test Model Long-Term Number 9 (TML-9) draft 0”, Dec. 2001.

- G.Sullivan, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material”, VCEG-N81, Sep. 2001.

- S.Kadono and M.Schlockermann, “Improved 2D-VLC coding for High-Bitrate”, VCEG-O27, Nov. 2001.

(a) I-pictuer	(b) P-picture

Figure 5: Bit savings for ‘Continaer’.

(a) I-pictuer	(b) P-picture

Figure 6: Bit savings for ‘News’.

(a) I-pictuer	(b) P-picture

Figure 7: Bit savings for ‘Foreman’.

(a) I-pictuer	(b) P-picture

Figure 8: Bit savings for ‘Silent’.

(a) I-pictuer	(b) P-picture

Figure 9: Bit savings for ‘Paris’.

(a) I-pictuer	(b) P-picture

Figure 10: Bit savings for ‘Mobile’.

(a) I-pictuer	(b) P-picture

Figure 11: Bit savings for ‘Tempete’.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma-City, Osaka, 571-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | Geneva, CH, Jan. 29 - Feb. 1, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ## JVT |  |
| Title |  | ## Proposal of Improved 2D-VLC coding for High-Bitrate |  |
| Contribution number |  | ## JVT-B056 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

We modify table look up VLC part for MaxRun = 0 case, because the table is very simple that has Run = 0 code only.