| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B062 Filename: JVT-B062.doc Generated: 2002-01-20 |
| --- | --- |

| Title: | Improved Entropy Coding with Codeword Re-Association |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeungwoo Jeon School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea Woongil Choi SungKyunKwan Univ./ Serome Technology, Inc 300 Chunchun-dong, Suwon, Korea Kyeong Joong Kim Serome Technology, Inc. 6th Fl., Asem Tower 159-1 Samsung-dong, Seoul, Korea | Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: | +82-31-290-7144 +82-31-290-5819 bjeon@yurim.skku.ac.kr +82-31-290-7186 +82-31-290-7181 creata@ece.skku.ac.kr +82-2-550-2333 +82-2-550-2453 kjkim@serome.co.kr |
| Source: | SungKyunKwan Univ and Serome Technology, Inc. |  |  |

_____________________________

## 1. Introduction

The basic entropy coding tool in the current TML9.0 [1] is the Huffman coding with a fixed single VLC table for all syntax elements. The VLC table is based on the universal VLC(UVLC) which constructs codewords by interleaving symmetric VLC code into fixed-length code(FLC) whose length is regulated with respect to the symmetric code. The UVLC has several advantages such as easy extensibility of the VLC table using a straightforward construction rule, no look-up-table necessary at encode and decoder, easy decoding, and robustness to error, etc. In spite of these outstanding features, its performance in terms of entropy coding efficiency has some space for further improvement [2-5] since the codewords are not designed according to the symbol probabilities, but simply assigned according to the fixed construction rule. The actual symbol probabilities can change according to coding conditions such as different QP values, characteristics of video sequences, and also from one syntax element to another.

The purpose of this document is to propose one way of improving the current UVLC approach by using fixed re-association table (FRAT) that assigns existing codewords to different symbols according to QP values. We propose to use this approach for the high bit rate applications whose statistics are different from the cases with medium and low bit rates.

## 2. Probability Distribution of Symbols

We have examined the actual statistics of the symbols taking major portions in bitstream in high bit rate including luminance TCOEFF. Since QP is an important factor governing statistical behavior of symbols, we have checked the statistics with different QPs and also for different image sequences. This experiment is done using TML4.0.

Figure 1. Comparison of probability distributions between UVLC and actual symbols

Figure 1 shows the actual probability distribution of the symbols “MB_type” that indicates mode of the macroblock and transform coefficients denoted by “Tcoeff_Sluma (denoted as Sluma in the figure).” It also shows the probabilities represented by the given UVLC table. The current UVLC represents the geometric probabilities modeled as,

where is the probability of a symbol corresponding to a codeword specified by whose length increases 1, 3, 5, and so on. To check dependency of probability{Sluma} on coding condition, we also checked for other QPs (QP=4, 16, 28) in Figure 2. The probability of symbol “Sluma” is flatter at a high bit-rate than at a low bit-rate. The probability of the first codeword indicates that the End of Block (i.e. EOB) is over 50% at low bit-rate due to high QP, therefore current assignment of one bit UVLC codeword to the EOB is efficient. However the probability at a high bit-rate is quite different from the UVLC distribution. Therefore it can be remarked that the actual probabilities are not necessarily the same to those of the UVLC codewords especially for low QPs. Furthermore some symbols appearing later in the UVLC table have larger probability than those located upper position in the table.

Figure 2. Probability distribution of Sluma vs. QPs

Therefore, in this proposal, we intend to re-map the codeword-symbol relationship according to QP values when QP is low. Since the probability of actual symbol can be dependent to other factors as well, we have also investigated the variability of probabilities with other factors such as the image sequence and the macroblock type, but just have found out that the quantization parameter (i.e. QP) is a main factor that mostly governs the probability distribution as fairly well expected.

## 3. Universal Variable Length Coding with FRAT

In the previous section, it is shown that the actual symbols in H.26L depend on QP. This analysis on the actual symbol probabilities suggests us one method to improve the coding performance of UVLC by re-associating the UVLC codeword based on the coding condition of QP. We propose using simple fixed re-association table (FRAT) between symbols and codewords in order to increase the coding efficiency of UVLC while still maintaining the structure of UVLC and all the benefits associated with it as well. The FRAT is a re-mapping table that accounts for the statistical variability of symbols according to QP.

In its simplest implementation, encoder and decoder both use the same pre-defined FRAT, therefore eliminating additional overhead of sending the re-mapping table information. Its very simple nature allows simple implementation. One way of implementing codeword-symbol re-association is using re-mapper from code number A to code number B using FRAT based on QP. For example, for coding symbol S and given QP,

The modified code number B refers to actual codeword to use in making bitstream.

## 4. Estimation of FRATs

Since QP can be 1 ~ 31, ideally there are 31 FRATs to consider. But as a first step, we test dividing the 31 QPs into groups. We have tested two cases: 6QP and 3QP approaches as below. Although we intent to use FRAT only for low QPs (i.e., for high bit rates applications), we have tested and report here the test results for whole range of QPs.

3QP_FRAT:

- FRAT 1: QP= 1~10

- FRAT 2: QP=11~20

- FRAT 3: QP=21~31

6QP_FRAT:

- FRAT 1: QP =  1  ~  5

- FRAT 2: QP =  6  ~ 10

- FRAT 3: QP = 11 ~ 15

- FRAT 4: QP = 16 ~ 20

- FRAT 5: QP = 21 ~ 26

- FRAT 6: QP = 27 ~ 31

The fixed re-association table,  is estimated for a given symbol S and for the QP ranges specified above using the statistics of training data. In this experiment, we re-associate codewords of following target symbols only.

- Intraframe: CBP, Tcoeff_Chroma_AC, Tcoeff_Chroma_DC, Tcoeff_luma

- Interframe : MB_type, CBP, Tcoeff_luma.

In this simulation, we used H.26L TML-9 coder at various bit-rates with test sequences, ‘silent’ (QCIF, 10fps), ‘foreman’ (QCIF, 10fps), ‘container’(QCIF, 10fps), ‘news’(QCIF, 10fps), and ‘mobile’ (CIF, 30fps). For each sequence, 100 frames are encoded. Figure 3 shows the bit-reduction ratio of the proposed FRAT scheme in inter and intra frames. The bit-reduction ratio R is defined as,

where S1 is the number of bits generated by the TML-9 encoder, and S2 refers to the number of bits generated by applying FRAT as described above.

## 5. Experiment 1: FRATs For Intra Frames

Figure 3 shows experimental result of intra frames. Here, we perform the leave-one-out experiment (the test sequences except selected one are used for FRAT generation, and the left-out sequence is used for test). Figure 3 also shows the result of MOC(Maximally Optimized Case) that is obtained by using 31 FRATs obtained based on actual statistics (test set = training set) – this is not interesting in practical sense, but it is shown here to know how close the pre-fixed FRAT can attain the limit set by the MOC).

As shown in Figure 3, the proposed FRAT method has improvement (up to 9%) in high and low bit rates. Major improvement comes from the symbol, “Tcoeff_luma and Tcoeff_chroma.”.

Figure 3. Bit Reduction Ratio of FRAT Approach over TML 9.0 UVLC: Intraframe

Following table shows the amount of contribution to bit reduction by each symbol. Note that although CBP_intra does have bit reduction by employing FRAT, its relatively low portion in entire bit counts makes its effect trivial in Table 1.

Table 1. Bit Reduction by Each Symbol (QP=1; Intra frame; 3QP)

| Sequence | CBP | T_luma | T_chroma_DC | T_chroma_AC | TOTAL |
| --- | --- | --- | --- | --- | --- |
| Mobile | 0.00 | 5.06 | 0.63 | 2.97 | 8.67 |
| News | 0.00 | 3.53 | 0.72 | 2.08 | 6.32 |
| Foreman | 0.00 | 1.85 | 0.43 | 0.79 | 3.07 |
| Silent | 0.00 | 2.42 | 0.64 | 1.68 | 4.73 |
| Container | 0.00 | 3.06 | 0.53 | 1.43 | 5.03 |

From Figure 3 and Table 1, we can conclude that,

- there is space for further improvement of entropy coding especially in high bit rate;

- and the room is mostly associated with TCOEFF;

- the proposed 3QP, or 6QP approaches are workable tool to realize such improvement.

## 6. Experiment 2: FRATs For Inter Frames

Same experiment is done for inter frames and the result is shown in Figure 4 where the proposed FRAT method has a marginal improvement at middle QP range (which suggests the UVLC is optimized at this bit-rate in TML-9), but 5~7% of bit rate reduction for QP < 10. In QP<10, Tcoeff_luma contributes most in bit reduction (more than 80%) like the previous intra case. By the way, for QP>20, CBP and MB_type also share the role due to their non-trivial bit-portion. In Figure 4, the container sequence is an exception. It is because the statistical property of container is different from the others.

Figure 4. Bit Reduction Ratio of FRAT Approach over TML 9.0 UVLC : Interframes

Table 2. Bit Reduction by Each Symbol (QP=1; Inter frame; 3QP)

| sequence | MB_type | CBP | T_Luma | TOTAL |
| --- | --- | --- | --- | --- |
| mobile | 0.08 | 0.54 | 5.91 | 6.62 |
| news | 0.18 | 0.47 | 4.34 | 5.11 |
| foreman | 0.16 | 0.82 | 2.59 | 3.58 |
| silent | 0.25 | 0.63 | 2.39 | 3.23 |
| container | 0.21 | 0.98 | -1.02 | 0.03 |

Also in the inter frame case, we can make similar conclusion as the previous intra case.

## 7. Experiment 3: FRATs as a Function of QP and Image Class

In this experiment, we have used different FRATs in encoding inter frames for low motion and high motion images sequences as group below:

Class A(Low Motion): Container(QCIF, 10fps), Akiyo(QCIF, 10fps), Suzie(QCIF, 15fps), Mother and Daughter(QCIF, 10fps), Clair(QCIF, 15fps)

Class B(High Motion): News(QCIF, 10fps), Foreman(QCIF, 10fps), Silent(QCIF, 15fps), Mobile(CIF, 30fps), Paris(CIF, 15fps), Tempete(CIF, 30fps)

Data set belonging only to the same class is used to estimate the FRATs for that class under the leave-one-out test scheme. Experiment 3 gives slightly better result than the previous Exp 2, and the improvement is larger in the class B. It is because Exp3 can better accommodate motion characteristics in FRAT. The proposed FRAT can attain up to 10% bit reduction in Mobile sequence.

This observation suggests further study for setting FRAT as a function of QP and image class as well. It will be further analyzed.

Figure 5. Bit Reduction Ratio of FRAT Approach over TML 9.0 UVLC in Exp 3 (Class A)

Figure 6. Bit Reduction Ratio of FRAT Approach over TML 9.0 UVLC in Exp 3 (Class B)

Table 3. Bit Reduction by Each Symbol (QP=1; Inter frame; 3QP & Class)

| CLASS | sequence | MB_type | CBP | T_Luma | TOTAL |
| --- | --- | --- | --- | --- | --- |
|  | container | 0.24 | 0.74 | 0.05 | 0.86 |
| Class | akiyo | 0.22 | 0.65 | 0.67 | 1.50 |
| A | clair | 0.11 | 0.21 | 0.46 | 0.75 |
|  | mad | 0.21 | 0.82 | 0.40 | 1.29 |
|  | suzie | 0.21 | 0.70 | 0.68 | 1.38 |
|  | silent | 0.20 | 0.62 | 0.45 | 1.16 |
|  | foreman | 0.16 | 0.82 | 2.89 | 3.92 |
| Class | news | 0.18 | 0.50 | 4.53 | 5.35 |
| B | mobile | 0.09 | 0.47 | 8.98 | 9.59 |
|  | paris | 0.01 | 0.77 | 3.85 | 4.90 |
|  | tempete | 0.13 | 0.60 | 6.99 | 7.57 |

## 8. Conclusion

In this document, we propose a very simple extension of current UVLC method using FRAT. It is based on fixed re-association tables (FRAT) which are used to change the mapping between codewords and symbols according to such factors as QP or possibly, image class (reflecting motion activity). The proposed method can attain 3 ~ 9% (intra frame) and 0 ~ 7% (inter frame) of bit reduction in high bit rate (QP=1). If the motion property is also reflected in FRAT estimation, it can save 1 ~ 10% of bits in high motion sequences (Class B).

Therefore, we suggest to consider this proposal for high bit rate application which have different statistical characteristics from the others. The proposed method can accommodate different probability distribution characteristics of symbols while still retaining the nice features of the UVLC.

## 9. References

[1]	G. Bjontegaard, “H.26L Test Model Long Term Number 9 (TML-9),” ITU-T Q.15/16, Document VCEG-N83, Dec. 2001.

[2] 	B. Jeon, “Entropy Coding for H.26L”, ITU-T Doc. Q15-J-57, May 2000.

[3] 	L. Kerofsky, “Entropy Coding of Transform Coefficients”, ITU-T Doc. Q15-K-45, August 2000.

[4] 	Kadono, “Improved 2D VLC Coding for High Bitrate,” JVT Document VCEG-O27, Dec 2001.

Appendix A. New Table with FRAT for Tcoeff_luma_Double_scan at High Bit Rate

(QP=1, intraframe)

| code - number | Tcoeff_luma_Double_scan |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | UVLC |  | FRAT1_3QP |  | FRAT1_6QP |  |
|  | level | run | level | run | level | run |
| 0 | EOB | - | EOB | - | EOB | - |
| 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 2 | -1 | 0 | -1 | 0 | -1 | 0 |
| 3 | 1 | 1 | -2 | 0 | -2 | 0 |
| 4 | -1 | 1 | 2 | 0 | 2 | 0 |
| 5 | 2 | 0 | 1 | 1 | -3 | 0 |
| 6 | -2 | 0 | -1 | 1 | 3 | 0 |
| 7 | 1 | 2 | 3 | 0 | 1 | 1 |
| 8 | -1 | 2 | -3 | 0 | -1 | 1 |
| 9 | 3 | 0 | 4 | 0 | -4 | 0 |
| 10 | -3 | 0 | -4 | 0 | 4 | 0 |
| 11 | 4 | 0 | 5 | 0 | 5 | 0 |
| 12 | -4 | 0 | -5 | 0 | -5 | 0 |
| 13 | 5 | 0 | 1 | 2 | 6 | 0 |
| 14 | -5 | 0 | -1 | 2 | -6 | 0 |
| 15 | 1 | 3 | 6 | 0 | 1 | 2 |
| 16 | -1 | 3 | -6 | 0 | -1 | 2 |
| 17 | 1 | 4 | -2 | 1 | -2 | 1 |
| 18 | -1 | 4 | 2 | 1 | 2 | 1 |
| 19 | 2 | 1 | 7 | 0 | 7 | 0 |
| 20 | -2 | 1 | -7 | 0 | -7 | 0 |
| 21 | 3 | 1 | 8 | 0 | 8 | 0 |
| 22 | -3 | 1 | -8 | 0 | -8 | 0 |
| 23 | 6 | 0 | 9 | 0 | 9 | 0 |
| 24 | -6 | 0 | -9 | 0 | -9 | 0 |
| 25 | 7 | 0 | 3 | 1 | 10 | 0 |
| 26 | -7 | 0 | -3 | 1 | -10 | 0 |
| 27 | 8 | 0 | 1 | 3 | -3 | 1 |
| 28 | -8 | 0 | -1 | 3 | 3 | 1 |
| 29 | 9 | 0 | 10 | 0 | 11 | 0 |
| 30 | -9 | 0 | -10 | 0 | -11 | 0 |
| 31 | 10 | 0 | 11 | 0 | 1 | 3 |
| 32 | -10 | 0 | -11 | 0 | -1 | 3 |
| 33 | 4 | 1 | 12 | 0 | -12 | 0 |
| 34 | -4 | 1 | -12 | 0 | 12 | 0 |
| 35 | 2 | 2 | -4 | 1 | 13 | 0 |
| 36 | -2 | 2 | 4 | 1 | -13 | 0 |
| 37 | 2 | 3 | 2 | 2 | 4 | 1 |
| 38 | -2 | 3 | -2 | 2 | -4 | 1 |
| 39 | 2 | 4 | 13 | 0 | 14 | 0 |
| 40 | -2 | 4 | -13 | 0 | -14 | 0 |
| 41 | 2 | 5 | 14 | 0 | 2 | 2 |
| 42 | -2 | 5 | -14 | 0 | -2 | 2 |
| 43 | 2 | 6 | 15 | 0 | 15 | 0 |
| 44 | -2 | 6 | -15 | 0 | -15 | 0 |
| 45 | 2 | 7 | -5 | 1 | 16 | 0 |
| 46 | -2 | 7 | 5 | 1 | -16 | 0 |
| 47 | 11 | 0 | 1 | 4 | -5 | 1 |
| 48 | .. | .. | .. | .. | .. | .. |

Appendix B. New Table with FRAT for Tcoeff_luma_Single_scan at High Bit Rate

(QP=1, interframe)

| code -number | Tcoeff_luma_Single_scan |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UVLC |  | FRAT1_3QP |  | FRAT1_3QP_HighMotion |  | FRAT1_3QP_LowMotion |  |
|  | level | run | level | run | level | run | level | run |
| 0 | EOB | - | EOB | - | EOB | - | EOB | - |
| 1 | 1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 |
| 2 | -1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 3 | 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 |
| 4 | -1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 5 | 1 | 2 | -2 | 0 | -2 | 0 | 1 | 2 |
| 6 | -1 | 2 | 2 | 0 | 2 | 0 | -1 | 2 |
| 7 | 2 | 0 | 1 | 2 | -1 | 2 | -1 | 3 |
| 8 | -2 | 0 | -1 | 2 | 1 | 2 | 1 | 3 |
| 9 | 1 | 3 | -1 | 3 | -3 | 0 | -2 | 0 |
| 10 | -1 | 3 | 1 | 3 | 3 | 0 | 2 | 0 |
| 11 | 1 | 4 | 3 | 0 | 1 | 3 | 1 | 4 |
| 12 | -1 | 4 | -3 | 0 | -1 | 3 | -1 | 4 |
| 13 | 1 | 5 | -1 | 4 | -2 | 1 | -1 | 5 |
| 14 | -1 | 5 | 1 | 4 | 2 | 1 | 1 | 5 |
| 15 | 3 | 0 | -2 | 1 | -1 | 4 | 1 | 6 |
| 16 | -3 | 0 | 2 | 1 | 1 | 4 | -1 | 6 |
| 17 | 2 | 1 | 1 | 5 | -4 | 0 | -2 | 1 |
| 18 | -2 | 1 | -1 | 5 | 4 | 0 | 2 | 1 |
| 19 | 2 | 2 | -4 | 0 | 1 | 5 | -3 | 0 |
| 20 | -2 | 2 | 4 | 0 | -1 | 5 | 3 | 0 |
| 21 | 1 | 6 | 1 | 6 | -2 | 2 | -1 | 7 |
| 22 | -1 | 6 | -1 | 6 | 2 | 2 | 1 | 7 |
| 23 | 1 | 7 | -2 | 2 | -5 | 0 | 2 | 2 |
| 24 | -1 | 7 | 2 | 2 | 5 | 0 | -2 | 2 |
| 25 | 1 | 8 | -5 | 0 | -3 | 1 | -4 | 0 |
| 26 | -1 | 8 | 5 | 0 | 3 | 1 | 4 | 0 |
| 27 | 1 | 9 | -3 | 1 | 1 | 6 | -1 | 8 |
| 28 | -1 | 9 | 3 | 1 | -1 | 6 | 1 | 8 |
| 29 | 4 | 0 | -1 | 7 | -6 | 0 | 2 | 3 |
| 30 | -4 | 0 | 1 | 7 | 6 | 0 | -2 | 3 |
| 31 | 5 | 0 | -6 | 0 | 2 | 3 | -3 | 1 |
| 32 | -5 | 0 | 6 | 0 | -2 | 3 | 3 | 1 |
| 33 | 3 | 1 | 2 | 3 | -1 | 7 | -1 | 9 |
| 34 | -3 | 1 | -2 | 3 | 1 | 7 | 1 | 9 |
| 35 | 3 | 2 | 1 | 8 | -4 | 1 | 1 | 10 |
| 36 | -3 | 2 | -1 | 8 | 4 | 1 | -1 | 10 |
| 37 | 2 | 3 | -4 | 1 | -7 | 0 | -2 | 4 |
| 38 | -2 | 3 | 4 | 1 | 7 | 0 | 2 | 4 |
| 39 | 2 | 4 | 7 | 0 | -3 | 2 | -5 | 0 |
| 40 | -2 | 4 | -7 | 0 | 3 | 2 | 5 | 0 |
| 41 | 2 | 5 | 2 | 4 | 1 | 8 | -1 | 11 |
| 42 | -2 | 5 | -3 | 2 | -1 | 8 | 1 | 11 |
| 43 | 2 | 6 | -2 | 4 | 2 | 4 | -2 | 5 |
| 44 | -2 | 6 | 3 | 2 | -2 | 4 | 2 | 5 |
| 45 | 2 | 7 | -1 | 9 | -8 | 0 | 3 | 2 |
| 46 | -2 | 7 | 1 | 9 | 8 | 0 | 6 | 0 |
| 47 | 2 | 8 | -8 | 0 | -5 | 1 | -3 | 2 |
| 48 | .. | .. | .. | .. | .. | .. | .. | .. |

JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Byeungwoo Jeon |  |  |
| Mailing address | School of ECE, SungKyunKwan Univ. 300 Chunchun-dong, Suwon, Korea |  |  |
| Country | Korea |  |  |
| Contact person | ###### Byeungwoo Jeon |  |  |
| Telephone | +82-31-290-7144 |  |  |
| Fax | +82-31-290-5819 |  |  |
| Email | bjeon@yurim.skku.ac.kr |  |  |
| Place and date of submission | JVT meeting Geneva, January 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT/H.26L |  |
| Title |  | Improved Entropy Coding with Codeword Re-Association |  |
| Contribution number |  | JVT-B062 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| #### X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Patent filed, Fall 2000 |  |
| Inventor(s)/Assignee(s) |  | Byeungwoo Jeon / Samsung Electronics |  |
| Relevance to JVT |  | H.26L Entropy Coding (proposal) |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| #### X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

Since the size of FRAT can become arbitrary large in case of TCOEFF, we consider only the most probable  symbols comprising upper 99% of probability. This corresponds to upper 255 (interframes) and 500 (Intraframes) codewords.