| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B064 Filename: JVT-B064.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Additional Results related to the CABAC AHG |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gero Bäse; Norbert Oertel CT IC 2 Otto Hahn Ring 6 81371 München Germany | Tel: +49 89 636 53193 Email: Gero.Baese@mchp.siemens.de Norbert.Oertel@mchp.siemens.de |  |
| Source: | Siemens AG |  |  |

_____________________________

# Summary

Additional results are presented for the CABAC related CE according to the proposed method in VCEG-O34. The number of coefficients is used to enhance the Level coding further. The proposed method always outperforms any combination of the other methods for smaller QP values.  Up to 1% bitrate savings on top of the combination of the HHI and Nokia proposals can be achieved without additional complexity.

# Results

All results were produced using the extended software provided by HHI and Nokia (TML 8.5_hhi_nokia). The recommended simulation common conditions for H.26L coding efficiency (VCEG-N81) were followed.

Test A comprises the VCEG-O24 coding method from Nokia. Test E comprises the complete VCEG-O18 proposal of HHI and the Nokia proposal as well. Detailed results can be found in the JVT-B064.xls document.

For all sequences additional gain can be achieved by using the number of coefficients to be coded for Level coding. Up to 1% additional gain can be observed. Mostly the bitrate savings are obtained for smaller QP values. 
On decoder side the transmitted number of coefficients can furthermore be utilized for error recovery purposes.

There is no additional complexity needed.

An adapted initialization routine for the applied context models would increase bitrate savings further, especially for the low bitrate range.

_____________________________