| ### Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B074r1 Filename: JVT-B074r1.doc Generated: 2002-01-28 |
| --- | --- |

| Title: | An improved variable length coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Yoshihiro Kikuchi | Tel: Email: | +81 44 549 2288 takeshi.chujoh@toshiba.co.jp |
| Source: | Toshiba |  |  |

_____________________________

# Introduction

There were several contributions claiming that UVLC in the current TML serves moderate coding performance for low bitrate case (when QP is large) but remains room for improvement for high bitrate case (lower QP). In this contribution, we propose a more efficient variable length code than UVLC in TML9. Similar to UVLC, the codewords can be encoded and decoded only by calculations without having a codeword table. It rather simplifies the decoding operation because no interleaving is necessary. Simulations were conducted, which show a coding gain up to about 6% for high bitrate cases compared to TML9.

# Proposed variable length code

Table 1 shows the codeword table of the proposed VLC.

#### Table 1 Codeword table

Proposed VLC	UVLC

| index | codeowrd |  | index | codeword |
| --- | --- | --- | --- | --- |
| 0 | 00 |  | 0 | 1 |
| 1 | 010 |  | 1 | 001 |
| 2 | 111 |  | 2 | 011 |
| 3 | 0110 |  | 3 | 00001 |
| 4 | 1011 |  | 4 | 00011 |
| 5 | 1101 |  | 5 | 01001 |
| 6 | 01110 |  | 6 | 01011 |
| 7 | 10011 |  | 7 | 0000001 |
| 8 | 10101 |  | 8 | 0000011 |
| 9 | 11001 |  | 9 | 0001001 |
| 10 | 011110 |  | 10 | 0001011 |
| 11 | 100011 |  | 11 | 0100001 |
| … | … |  | … | … |

The codeword is generation of the following equations.

For index n, find m that satisfies the following equation:

12m(m−1)≤n<12m(m+1)

Generate code C(n) as follows:

C(n)=011⋯11m−10,n=12m(m−1)

where d=n−12m(m−1)−1.

# Decoding operation

VLC decoding operation may differ depend on the architecture of the decoder implementation. Hard-wired logic with bit-by-bit operations may be used in custom chips. Word oriented (byte, 16-bit, 32-bit, etc.) operations may be suited for general-purpose processors (CPU) or DSPs.

The codewords of both the proposed VLC and UVLC in TML9 have regular nature. Nevertheless, because of the interleaved codeword of UVLC, it may force much burden for decoding operation on word-oriented architecture such as DSP. The proposed VLC requires no interleaving operation and decodable with simple calculations for both bit-by-bit and word oriented architectures.

# Error-resilience characteristics

For some networks with bit errors, such as mobile circuit-switched based networks (e.g. 3G-324M), the coded streams are injured by both bit errors and burst errors with rather high BER (10-4-10-2 for some cases). Bit-error resiliency serves a key role for such networks. It was claimed that UVLC provides a self-resynchronization characteristic. However, it is doubtful whether the self-resynchronization nature is useful for error resiliency. The correspondence between the coded information and the codeword itself is lost by error; a DCT coefficient may be decoded in an incorrect block, or may be decoded as wrong information such as mode or MV information. These will cause much degradation on the decoded image in most cases.

The proposed VLC provides some characteristics for bit and burst error resiliency as follows:

- The codewords are decodable for both forward and backward directions. It enables bi-directional decoding in combination with the data partitioned syntax. It is useful for bit and burst error resiliency as promised in MPEG-4 and H.263 Annex V.

- Hamming distance between codewords with the same length is always more than 2. This serves as bit-error detection capability.

# Simulation results

We conducted a computer simulation to evaluate the coding performance of the proposed method following the common test condition (VCEG-N81). Table 1 summarizes the simulation conditions.

##### Table 1 Simulation conditions

| Software version | TML 9.4 |
| --- | --- |
| R-D optimization | Used |
| MV search range | 32 |
| B picture | Not used (P-pictures only) |
| CABAC | Not used (UVLC and new VLC) |
| 1/8 pel MC | Not used (up to 1/4 pel accuracy) |
| Number of reference pictures | 1 (no multi-frame reference) |
| QP | 0 to 31 |

| Video sequences | Size | Frame rate (Hz) |
| --- | --- | --- |
| Container | QCIF | 10 |
| News | QCIF | 10 |
| Foreman | QCIF | 10 |
| Silent | QCIF | 15 |
| Paris | CIF | 15 |
| Mobile | CIF | 30 |
| Tempete | CIF | 30 |

For preliminary evaluation purpose, whole variable length codes (including DCT coefficient, MV, mode, etc.) are replaced by the proposed VLC in the simulation.

Figure 1 and Figure 2 show the R-D curves and the coding gain (percentage of bit-saving) compared to TML9 for various QP values, respectively. TML9 UVLC is better when QP is large (low-bitrate cases) whereas there is a coding gain by using the proposed VLC when QP is small (high-bitrate cases). Percentage of bit saving varies depend on the sequences, but the proposed VLC outperforms UVLC with QP values at around 10-15 or less for all sequences except for coastguard. The coding gain is up to 6.2 %. The propose VLC is better for all video sequences when QP is smaller than 6.

# Conclusion

A new variable length code to improve the coding efficiency is proposed. Simulation results show that the proposed VLC outperforms UVLC for high bitrate. The Proposed VLC is better for all sequences when QP is small. The coding gain is up to 6.2%. The proposed VLC has a low-complexity nature since the codewords are regular and without interleaving, thus decodable with only simple operations. It also provides bit-error resilient characteristics that may be useful for some mobile networks.

Since TML9 UVLC is better suited for low-bitrate cases, combination of UVLC and the proposed VLC would perform the best. Overhead of switching between these two VLCs is marginal since both UVLC and the proposed VLC are decodable with operations. Combination with other schemes for the improvement, such as efficient mapping method to VLC codewords, should be considered, while we should be care not to introduce a complicated scheme to strike for a single VLC.

- QCIF (Container, News, Foreman, Silent)

- CIF 15Hz (Paris)

(c) CIF 30Hz (Mobile, Tempte)

#### Figure 1 R-D curves

Figure 2 Bit saving against TML9

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | TOSHIBA CORPORATION |  |  |
| Mailing address | 1, Komukai Toshiba-Cho, Saiwai-Ku, Kawasaki 212-8582, Japan |  |  |
| Country | Japan |  |  |
| Contact person | Takeshi Chujoh |  |  |
| Telephone | +81-44-549-2288 |  |  |
| Fax | +81-44-520-1267 |  |  |
| Email | takeshi.chujoh@toshiba.co.jp |  |  |
| Place and date of submission | 2nd JVT Meeting Jan.29-Feb.1, Geneva |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | An improved variable length coding |  |
| Contribution number |  | JVT-B074 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| ## X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ## X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: