| ### Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B076 Filename: JVT-B076.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | New picture type for error recovery |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshihiro Kikuchi Takeshi Chujoh Takeshi Nagai | Tel: Email: | +81 44 549 2288 kiku@eel.rdc.toshiba.co.jp |
| Source: | Toshiba |  |  |

_____________________________

- Introduction

The purpose of this contribution is to propose a quick error recovery mechanism by introducing a new picture type “Reference Picture (R-picture/R-frame)”. Periodical Intra refresh or demand refresh are used for error recovery in conventional schemes. However, intra refresh causes degradation on coding efficiency for error free case. Demand refresh requires real-time encoding and thus not applicable for streaming services. Recently, some feedback mechanisms such as RTCP feedback have been provided. The proposed method utilizes such feedback mechanism and sends intra pictures for refresh only when the error happened. Also, the method is applicable to streaming services, since the R-picture is embedded with other ordinary frames. In contrast to the conventional intra refresh frame, signals after motion compensation is intra coded in the R-picture. This enables a perfect reconstruction without drift when used with the SP-frame.

- Proposed method

The basic idea here is to provide a new picture type, called “Reference Picture (R-picture/R-frame)” for error recovery purpose. R-picture is generated by intra encoding the motion compensated frame instead of the input frame. The R-pictures are embedded in a stream. The server does not send the R-picture for normal error-free case, but sends it when error happens on the network. The decoder replaces the frame memory with decoded R-picture.

Figure 1 shows the block diagram of the overall system. The server has a switcher that parses the picture type and decides whether or not to send R-picture. Figure 2 shows the encoder diagram. The motion compensated reference frame is intra encoded to produce an R picture and the coded R-picture stream is multiplexed with normal code-streams. Intra coding itself, except for its input signal, is identical to that of TML9. Figure 3 shows the decoder diagram. When the decoder receives an R picture, the next P picture is decoded without motion compensation since R picture is the motion compensated image. And the decoder does not output the decoded R picture.

Figure 4 shows the frame relationship at the sender. And Figure 5 shows the frame relationship at the decoder. The R-picture is not used for normal decoding (error free case) since it is just additional data. Therefore, the server would not send R-pictures (R5 in Figure 4). When the server receives a report indicating that the client encounters an error, the server switches to the next R-picture (R5) instead of P-pictures (P4) and sends it. The client decodes the R-picture (R5) and replaces the frame memory with it.

The decision method for R-picture insertion in the encoder is outside the scope of the standard. Decoder that does not support R-pictures does not need to decode them but just to recognize R-picture’s picture type and skip them.

Moreover, the combination with SP-picture enables drift-free reconstruction of the reference picture.

Figure 1 system overview

Figure 2 encoder overview

Figure 3 decoder overview

Figure 4 frame relationship in sender

Figure 5 frame relationship in decoder

- Advantages

The followings are the advantages of the proposal.

  - No change on the core coding algorithm

  - Applicable also for streaming services (non real-time encoding)

  - Required operation for the server is switching the stream by parsing the picture type

  - The code-stream sent to the decoder under an error free condition is identical to the normal stream.

  - The whole size of code-stream stored in the server is less than that of other methods such as multiple stream switching, since only R-pictures are required.

  - Perfect reconstruction is achieved when used in combination with SP-picture.

- Conclusion

We proposed a new frame type called “R-picture” in order for quickly recovery from error (e.g. packet loss). This scheme is applicable for streaming service. R-pictures are sent only when error happens, the code-stream sent to the client for error free case stays same as the normal case. Therefore, the size of the sent code-stream does not increase unlike periodical intra refresh. In addition, combination with SP pictures enables perfect drift-free reconstruction.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | TOSHIBA CORPORATION |  |  |
| Mailing address | 1, Komukai Toshiba-Cho, Saiwai-Ku, Kawasaki 212-8582, Japan |  |  |
| Country | Japan |  |  |
| Contact person | Yoshihiro Kikuchi |  |  |
| Telephone | +81-44-549-2288 |  |  |
| Fax | +81-44-520-1267 |  |  |
| Email | kiku@eel.rdc.toshiba.co.jp |  |  |
| Place and date of submission | 2nd JVT Meeting Jan.29-Feb.1, Geneva |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | New picture type for error recovery |  |
| Contribution number |  | JVT-B076 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| ## X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ## X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: