| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B077 Filename: JVT-B077.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | # Short tap filter for high resolution sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei-ichi Chono and Yoshihiro Miyamoto NEC Multimedia Laboratories, 4-1-1 Miyazaki Miyamae Kawasaki, 216-8555 JAPAN | Tel: Fax: Email: | +81 44 856 8141 +81 44 856 2232 chono@dsp.cl.nec.co.jp miyamoto@dsp.cl.nec.co.jp |
| Source: | NEC Corporation Kawasaki, 216-8555 JAPAN |  |  |

_____________________________

# 1. Background

In VCEG-O16 of the Pattaya meeting adaptive interpolation scheme, estimating and transmitting 6-tap filter coefficients per picture, was proposed [1] To consider non-stationary statistical properties of video signals, this adaptive interpolation scheme seems to be beneficial [1][2]. However, there are some issues: complexity and so on. AHG was formed to study these issues for the in-corporation of this feature into TML.

We agree to the adaptive interpolation scheme, and have compared coding quality gains of some interpolation filters, for a given set selection of filter coefficients. Experiment results show us two facts. First fact is that TML9 6-tap filter coding gain to Telenor 4-tap filter depends on sequences. Second fact is that 4-tap filter implementation reduces the amount of fractional-pel interpolation calculation to 50% at decoder. We propose that encoder/decoder may use shorter tap filter for high-resolution sequences, in order to reduce the complexity.

# 2. Experiments

# 2.1 Interpolation

In order to produce 2:1 resolution, Telenor 4-tap filter, with impulse response (-4, 20, 20, -4)/32 [3], is used. For 4:1 resolution, Bilinear filter is used as second filter. Hence, the difference between 4-tap filter interpolation and TML9 is first filter coefficients.

# 2.2 Test sequences and coding conditions

Test sequences are shown in Table 1.

Table 1: Test sequence

The coding conditions are: Quantization parameter for I-picture(QI) set to 24, 16 pixel search range(32 pixel is only used for Football CCIR), RestrictSearchRange set to two, UseHadamard set to on, quarter-pel motion, one reference frame, UVLC in entropy coding, RD optimization set to on, no B-picture.

CCIR sequences are interlace-formant, not supported by TML9 yet. The encoder treated 2 fields as 1 frame described Fig 1.

Fig 1	Input YUV format at CCIR encoding

# 2.3 Results

TML9 filter coding gain compared to Telenor 4-tap filters is the following:

QCIF results:

TML9 filter leads to average gain 0.19[dB] with avsnr4, up to 0.48[dB

CIF and SIF results:

TML9 filter leads to average gain 0.17[dB] with avsnr4, up to 0.60[dB]

CCIR results:

TML9 filter leads to average gain 0.24[dB] with avsnr4, up to 0.54[dB]

Avsnr4 is used to calculate objective differences for common measurement method [4]. The avsnr4-results are shown in Table 1-3, and rate-distortion curves are attached.

# 3. Proposal : adaptive filter taps for reducing coding calculation

Experiment results show that TML9’s coding gain to Telenor 4-tap filter depends on sequences. The total amount of fractional-pel calculation increases in proportion to the spatial resolution. The current TML9 uses direct-interpolation for reducing multiplications and memory requirement. Telenor 4-tap direct-interpolation can reduce multiplication to half of 6-tap’s [5]. We have evaluated the decoding time improvement by the multiplication reduction. Evaluation condition is: Pentium4 1.7GHz, 512MB memory, Microsoft Windows2000, and Intel VTune5 with 1msec sampling rate. The multiplication reduction cuts half of interpolation calculation, and 10 percent of total decoding calculation (See Chart 1-3).

From view of complexity reduction, we think that encoder/decoder may use shorter tap filter for high-resolution sequences, because TML9’s coding gain depends on the sequence.

# 4. Conclusion

In this report, two facts are shown:

- TML 6-tap filter coding gain to Telenor 4-tap filter depends on sequences

- 4-tap direct interpolation cuts half of interpolation calculation, and 10 percent of total decoding calculation

We propose that encoder/decoder may use shorter tap filter for high-resolution sequences, in order to reduce the complexity. The additional study on higher resolution sequences, as HDTV, may be valuable for confirming this interpolation filter issue.

# Reference

- Thomas Wedi, “H.26L Interpolation Filtering Ad Hoc Report”, ITU-T SG16, doc. VCEG-O11, Pattaya, Thailand, 4-6 Dec., 2001

- Thomas Wedi, “Adaptive Interpolation Filter for H.26L”, ITU-T SG16, doc. VCEG-N28, Santa Barbara, CA, USA, Sep. 2001

- Thomas Wedi, “Interpolation Filter for motion compensated prediction with 1/4 and 1/8-pel accuracy”, ITU-T SG16, doc.Q15-J-14, Osaka, Japan, May. 2000.

- Gary Sullivan and Gisle Bjontegaard, “Recommended Simulation Conditions for H.26L Coding efficiency Experiment on Low-Resolution Progressive-Scan Source Material”, ITU-T SG16, doc. VCEG-N81, Santa Barbara, CA, USA, Sep. 2001

- Thomas Wedi,” Results on complexity and coding performance investigations: Displacement vector resolution and interpolation filter tap size”, ITU-T SG16, doc. VCEG-M46, Austin, Texas, USA, April, 2001.

# Avsnr4-results

Table1: QCIF results

Table2: CIF and SIF results

Table3: CCIR results

Decoding samples

Chart 1	Decoding samples (Sequence: Foreman with QCIF, 10fps, QI=24, QP=20, and 133 frames)

Chart 2	Decoding samples (Sequence: Paris with CIF, 15fps, QI=24, QP=20, and 150 frames)

Chart 3	Decoding samples (Sequence: Flower with CCIR, 30fps, QI=24, QP=20, and 150 frames)

# Rate-distortion curves

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | NEC CORPORATION |  |  |
| Mailing address | 4-1-1 Miyazaki, Miyamae-Ku, Kawasaki 216-8555, Japan |  |  |
| Country | Japan |  |  |
| Contact person | Yoshihiro Miyamoto |  |  |
| Telephone | +81-44-856-8141 |  |  |
| Fax | +81-44-856-2232 |  |  |
| Email | miyamoto@dsp.cl.nec.co.jp |  |  |
| Place and date of submission | 2nd JVT Meeting Jan.29-Feb.1, Geneva |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Short tap filter for high res sequences |  |
| Contribution number |  | JVT-B077 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ## X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: